3GPP TSG RAN WG1 #104e			R1- 2100799

e-Meeting, January 25th – February 05th, 2021

Agenda Item:	7.2.4

Source:	 Spreadtrum Communications

Title:	 Remaining issues in NR sidelink mode 2 resource allocation

Document for:	 Discussion and decision

- Introduction

In this contribution, we discuss some remaining issues of resource allocation mode 2 for NR V2X communication and provide some TPs.

- Discussion

  - Sensing window size parameter T0 in periodic reservation

The following agreements have been achieved for sensing window size parameter T0 and reservation periodicity.

Agreements: - Confirm that sensing window size parameter T0 is (pre)-configured between two values: 1100 ms and 100 ms Agreements: - On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled: - A period is additionally signalled in SCI and the same reservation is applied with respect to resources indicated within NMAX within window W at subsequent periods - A set of possible period values is the following: 0, [1:99], 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000 ms - <= 4 bits are used in SCI to indicate a period - An actual set of values is (pre-)configured Agreements: - On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled: - A set of possible period values additionally includes all integer values from 1 to 99 ms

According to the above agreements, T0 is (pre)-configured between 1100 ms and 100 ms. When periodic reservation is enabled in a resource pool, T0 is (pre)-configured as 1100 ms. Considering an actual set of period values is (pre-)configured on a per resource pool basis, with the maximum period value denoted as Pm, T0=1100 ms is not required since sensing results on slots earlier than t-Pm-100 ms is not referred for resource (re-selection) at time instance t. Therefore, it’s better to (pre)-configured T0 as Pm+100 ms.

Proposal 1: When periodic reservation is enabled in a resource pool, T0 should be (pre)-configured as Pm+100 ms, where Pm is the maximum period value (pre)-configured in the resource pool.

  - UE procedure for determining the subset of resources to be reported to higher layers

According to section 8.1.4 of TR 38.214, since the number of candidate single-slot resources remaining in the set SA may be greater than X⋅Mtotal, X⋅Mtotal candidates in the set SA should be selected and reported to higher layers. Then how to select is up to UE implementation, for example, it can be selected randomly or based on RSRP measurement.

Proposal 2: X⋅Mtotal candidates should be selected and reported to higher layers, and adopt TP #1 in subsection 2.4.

TP #1:

----------------------------------------------------Begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources. The set SB is initialized to an empty set.

<Unchanged parts omitted>

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4. Otherwise, the UE selects the candidate single-slot resource Rx,y from the set SA to SB. This step is repeated until the number of candidate single-slot resources in the set SB becomes greater than or equal to X⋅Mtotal.

The UE shall report set SB to higher layers.

<Unchanged parts omitted>

-----------------------------------------------------End text proposal for 38.214-----------------------------------------------------

- Conclusions

In this contribution, we discussed the resource allocation mode 2 for NR V2X sidelink transmission and following conclusions are proposed:

Proposal 1: When periodic reservation is enabled in a resource pool, T0 should be (pre)-configured as Pm+100 ms, where Pm is the maximum period value (pre)-configured in the resource pool.

Proposal 2: X⋅Mtotal candidates should be selected and reported to higher layers, and adopt TP #1 in subsection 2.4.

- References

- 3rd Generation Partnership Project;Technical Specification Group Radio Access Network;NR;Physical layer procedures for data(Release 16), 3GPP TS 38.214