3GPP TSG RAN WG1 #104-e		R1-2100937

e-Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Remaining issues on mode1

Source: 	ZTE, Sanechips

Agenda item:	7.2.4

Document for:	Discussion and decision

Introduction

In the past meetings, some agreements about SL HARQ have been achieved[1][2]:

RAN1#102e: Agreements: For Mode 1 when applicable: - For the same TB, the minimum time between PSFCH reception and next scheduled PSCCH/PSSCH retransmission is Tprep +delta (ms) - To conclude the value of delta>=0 during the e-Meeting - A UE is not expected to be scheduled consecutive SL transmisions for the same TB such that the minimum time between PSFCH reception and next PSCCH/PSSCH retransmission can not be guaranteed - FFS the detailed conditions of the applicability RAN1#103e: Agreements: - The agreement from RAN1#102 applies when the grant is provided for a pool configured with PSFCH resources and the UE uses it for a TB with SL HARQ-feedback enabled. Agreements: - If the time between PSFCH reception and next scheduled PSCCH/PSSCH retransmission is less than Tprep + delta, the UE is allowed to drop the PSCCH/PSSCH retransmission with SL HARQ feedback enabled. - Note: it is RAN1 understanding that the UE is allowed to drop the PSCCH/PSSCH retransmission only if the UE can not complete the PSFCH processing and the preparation of the next PSCCH/PSSCH retransmission in the time between PSFCH reception and the next scheduled PSCCH/PSSCH retransmission

In this contribution, some Mode-1 issues are discussed including general corrections, SL HARQ report procedure and type-1 codebook generations.

Discussion

General Corrections in TS 38.214

In section 8.1.2 of TS 38.214[6]，for description of mode 1 “PDCCH validation for SL grant Type 2”, it is captured as :

In sidelink resource allocation mode 1: - for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.3 of [6, TS 38.213].

While Clause 10.3 of [6, TS 38.213] is just for describing “PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells”, not for SL, so it should be corrected to keep accordance.

In TS 38.212[4], filed “Configuration index” in DCI 3_0 is described as:

- Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

But we could not find corresponding reference information in clause 8.1.2 of [6, TS 38.214]. so it is suggested to add the missing part in clause 8.1.2 of [6, TS 38.214].

A text proposal in clause 8.1.2 of [6, TS 38.214] is provided below:

- 8.1.2 Resource allocation In sidelink resource allocation mode 1: - for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.2A 10.3 of [6, TS 38.213]. - 8.1.2.1 Resource allocation in time domain ... In sidelink resource allocation mode 1: - For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0. - For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0. - For sidelink dynamic grant and sidelink configured grant type 2: - The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter timeGapFirstSidelinkTransmission and the table value at index m + 1 will be referred to as slot offset KSL. - The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslott is the SL slot duration. - The "Configuration index" field , if provided, in the DCI format 3_0 indicates an index of a configured SL grant given by high layer. ...

SL HARQ report procedure

For SL HARQ report procedure which is described in section 16.3 of TR38.213[5] as follows:

... For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers ...

There is also some misunderstandings on “receives a PSFCH”, is it means receive a PSFCH resource or a PSFCH time domain occasion with multiple PSFCH resources corresponding to the same PSSCH transmission?

Besides, it has been agreed that when SL HARQ is enabled, both gNB and UE will guarantee the time gap between PSFCH reception and next scheduled PSCCH/PSSCH retransmission, so after a PSSCH transmission occasion, only one corresponding PSFCH reception occasion need to be considered when the UE determine to report SL HARQ feedback to high layers.

For group-cast transmission with SL HARQ feedback enabled, the UE can not find “at least one PSFCH reception occasion from the number of PSFCH reception occasions”, the UE just report ACK if it determine ACK value from PSFCH resources associated with every identity MID of UEs in the PSFCH reception occasion.

So some corrections are needed for SL HARQ report procedure.

For corrections for SL HARQ report procedure, a corresponding 38.213 TP in section 16.3.1 is given below:

For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH resource associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH resources in a PSFCH reception occasion associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE receives a PSFCH resources in a PSFCH reception occasion associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers

Corrections for Type-1 codebook

In this section two points for corrections for type-1 codebook are given respectively, and lastly a final TP is provided.

- Point 1:

In sidelink physical procedure , it has been agreed that the number of logical slots in a resource pool within 10240ms period is not limited to be an integer multiple of the PSFCH resource period, and for the location of PSFCH slots where the PSFCH resource period is N, Logical slot index #0, #N, #2N, …, within 10240 ms period is used [1]. When the number of logical slots in a resource pool within 10240ms happen to be not an integer multiple of the PSFCH resource period, the PSSCH transmission occasions corresponding to the first PSFCH slot with 10240ms is no longer N. In this case if we still generate type-1 codebook according to PSFCH resource period N, there will be some confusion because we cannot find N PSSCH transmission occasions corresponding to the first PSFCH slot with 10240ms. So type-1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

Type 1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

- Point 2:

In current TR38.213[5], multiple resource pools are allowed to be configured, but the set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions only consider one resource pool but the procedure in the specification does not distinguish which resource pool is used.

- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool …… The cardinality of the set MA defines a total number M of PSFCH reception occasions corresponding to the HARQ-ACK information bits. ……

From the yellow marked text above, it seems sub-bullet c) defines multiple resource pools with a set of sidelink resource pool bitmaps, but just only one but unknown which one resource pool is used to generate the set MA in the following pseudo-code and the HARQ-ACK information bits, this is not correct when multiple resource pools with different PSFCH period are configured.

In current 38.213 specification when multiple resource pools are configured, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits

All the configured resource pools should be considered for Type-1 codebook generation

- Final TP:

From all the analysis in this section above, a text proposal can be provided for type-1 codebook correction.

For corrections for Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0-index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

Conclusion

This paper concludes with the following proposals and observations:

Observation 1: In current 38.213 specification when multiple resource pools are configured, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits

Proposal 1: A text proposal in clause 8.1.2 of [6, TS 38.214] is provided below:

- 8.1.2 Resource allocation In sidelink resource allocation mode 1: - for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.2A 10.3 of [6, TS 38.213]. - 8.1.2.1 Resource allocation in time domain ... In sidelink resource allocation mode 1: - For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0. - For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0. - For sidelink dynamic grant and sidelink configured grant type 2: - The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter timeGapFirstSidelinkTransmission and the table value at index m + 1 will be referred to as slot offset KSL. - The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslott is the SL slot duration. - The "Configuration index" field , if provided, in the DCI format 3_0 indicates an index of a configured SL grant given by high layer. ...

Proposal 2: For corrections for SL HARQ report procedure, a corresponding 38.213 TP in section 16.3.1 is given below:

For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH resource associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH resources in a PSFCH reception occasion associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE receives a PSFCH resources in a PSFCH reception occasion associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers

Proposal 3: Type 1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

Proposal 4: All the configured resource pools should be considered for Type-1 codebook generation

Proposal 5: For corrections for Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0-index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

References

- Chairman note RAN1#102-e

- Chairman note RAN1#103-e

- R1-2006961 Feature lead summary#2 on Resource allocation for NR sidelink Mode 1

- TS 38.212 g30

- TS 38.213 g30

- TS 38.214 g30