3GPP TSG RAN WG1 #104-e	                                       	  R1- 2100992

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.13.1

Source:	Lenovo, Motorola Mobility

Title:	Cross-carrier scheduling (from Scell to Pcell)

Document for:	Discussion

# Introduction

This contribution expresses our views on SCell scheduling PCell pertaining to the DSS WID objective and conditions highlighted:

DSS WID objectives:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS):

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

During last RAN1 meeting, the following was agreed:

Agreements:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Conclusion

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Conclusion

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCellUSS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set configured for scheduling of in PCell/PSCell)

# Discussion

## DCI format 0_2/1_2

One question to answer is whether the UE can monitor (1) a UESS search space associated with a CORESET in PCell and (2) a UESS search space associated with a CORESET in sSCell for PCell scheduling. In our view, there could be advantages in allowing such monitoring: for instance,

- DCI formats 0_2 and 1_2 might be configured to be monitored with 0 bits for carrier indicator (saving bits in URLLC-DCI);

- in case of PCell and sSCell having different SCS, the resulting UE PDSCH reception preparation time may not be desirable for URLLC operation (e.g., DCI format(s) x_2)

- if CA deactivation is supported for sSCell, DCI format x_2 (similarly x_1) cannot be monitored.

and hence to schedule PCell with DCI formats 0_2 and 1_2, it would be good to monitor them on the PCell while the PCell could be scheduled with DCI formats 0_1/1_1 from sSCell.

Proposal 1:  UESS for PCell scheduling can be monitored on both PCell and sSCell.

## DCI format 0_0/1_0

Currently, DCI format 0_0/1_0 does not have a carrier indicator field. If dci-FormatsExt field is configured dci-Formats field is ignored, and UESS can monitor either formats0-2-And-1-2 or formats0-1-And-1-1And-0-2-And-1-2. If dci-FormatsExt field is not configured, UESS can monitor formats0-0-And-1-0 or formats0-1-And-1-1.

One question is whether DCI format 0_0/1_0 for PCell can be monitored in sSCell. One advantage of allowing this is in case a UESS contains PDCCH candidates with DCI format 0_0/1_0 (e.g., when dci-FormatsExt field is not configured), the UESS on sSCell can be used for both sSCell and PCell scheduling. However, to avoid additional specification impact, we prefer to keep the legacy setting to not allow cross-carrier scheduling with DCI format 0_0/1_0. Also, if the SCell scheduling PCell feature is disabled (e.g., via RRC reconfiguration), having a carrier indicator field in DCI format 0_0/1_0 becomes useless, unless it is made configurable. Even if DCI format 0_0/1_0 in UESS of sSCell for PCell scheduling is motivated to be specified, we prefer to defer this part till decision is made on whether to support PDCCH of SCell scheduling PDSCH on multiple cells using a single DCI (second objective of the WID) as it could impact the way cross-carrier scheduling with DCI format 0_0/1_0 gets specified.

Proposal 2:  DCI format 0_0/1_0 for PCell is only monitored in PCell.

## Unlicensed Cells

It would be good to clarify whether the feature of sSCell scheduling PCell can be applied to unlicensed cells as well.

Proposal 3:  Clarify whether PCell and/or scheduling SCell can be unlicensed cell(s).

## Maximum number of DL and UL unicast DCI formats in a span

If PCell can be simultaneously scheduled via PCell and sSCell, it would be good to clarify the maximum number of DL and UL unicast DCI formats in a span (one span of PCell and one span of sSCell).

For UE feature 3-5b, we have:

For the set of monitoring occasions which are within the same span:

- Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD

- Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD

- Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD

Proposal 4:  Clarify whether for FDD, the UE can process:

- one unicast DCI scheduling DL and one unicast DCI scheduling UL for PCell across set of monitoring occasions of PCell and scheduling SCell; or

- one unicast DCI scheduling DL and one unicast DCI scheduling UL for PCell across set of monitoring occasions in PCell and one unicast DCI scheduling DL and one unicast DCI scheduling UL for PCell across set of monitoring occasions in scheduling SCell

## Dormancy behavior of sSCell

For power saving purpose, an SCell may need to get CA deactivated/become dormant. If the sSCell becomes dormant/deactivated, sSCell scheduling PCell functionality is affected. There are two high-level options:

- option 1: the sSCell is not allowed to become dormant/deactivated,

  - Option 1a: if the sSCell belongs to a dormancy group, the dormancy indication is not applicable to the sSCell

- option 2: the sSCell can and is indicated to become dormant/deactivated

  - option 2a: another SCell becomes the new sSCell

  - option 2b: PCell becomes self-scheduled only

  - option 2c: sSCell dormancy/deactivation supported if DCI format 2_x, to be monitored for PCell, is at least monitored on PCell

Proposal 5: If dormancy/CA deactivation is supported for sSCell, further discuss the following behaviors:

- option a: another SCell becomes the new sSCell

- option b: PCell becomes self-scheduled only

- option c: sSCell dormancy/deactivation supported if DCI format 2_x, to be monitored for PCell, is at least monitored on PCell

# Conclusions

This contribution provided our views regarding SCell scheduling PCell as follows:

Proposal 1:  UESS for PCell scheduling can be monitored on both PCell and SCell.

Proposal 2:  DCI format 0_0/1_0 for PCell is only monitored in PCell.

Proposal 3:  Clarify whether PCell and/or scheduling SCell can be unlicensed cell(s).

Proposal 4:  Clarify whether for FDD, the UE can process:

- one unicast DCI scheduling DL and one unicast DCI scheduling UL for PCell across set of monitoring occasions of PCell and scheduling SCell; or

- one unicast DCI scheduling DL and one unicast DCI scheduling UL for PCell across set of monitoring occasions in PCell and one unicast DCI scheduling DL and one unicast DCI scheduling UL for PCell across set of monitoring occasions in scheduling SCell

Proposal 5: If dormancy/CA deactivation is supported for sSCell, further discuss the following behaviors:

- option a: another SCell becomes the new sSCell

- option b: PCell becomes self-scheduled only

- option c: sSCell dormancy/deactivation supported if DCI format 2_x, to be monitored for PCell, is at least monitored on PCell