3GPP TSG RAN WG1 #104-e																																																				R1-2101069

e-Meeting, January 25th – February 5th, 2021

Source:	CMCC

Title:	Discussion on scenarios for IoT NTN

Agenda Item:	8.15.1

Document for:	Discussion and Decision

Introduction

In the last RAN1 #103-e meeting, several agreements were achieved on IoT NTN scenarios and parameters [1].

Agreement: IoT NTN scenarios A, B, and C are included in the study as shown below: NTN Configurations; Transparent satellite. GEO based non-terrestrial access network; Scenario A. LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km); Scenario B. LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km); Scenario C. Agreement: The following IoT NTN reference scenario parameters are agreed: Scenarios; GEO based non-terrestrial access network - scenario A; LEO based non-terrestrial access network -Scenario B & C. Orbit type; station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point; circular orbiting at low altitude around the earth. Altitude; 35,786 km; 600 km 1,200 km. Frequency Range (service link); < 6 GHz (e.g. 2 GHz in S band). Device channel Bandwidth (service link) (NOTE 7); - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL). Payload; Transparent type; Transparent Type. Earth-fixed beams; Yes; Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite). Max beam foot print size (edge to edge) regardless of the elevation angle; 3500 km (NOTE 3); 1000 km (NOTE 2). Min Elevation angle for both sat-gateway and C-IoT device; 10° for service link and 10° for feeder link; 10° for service link and 10° for feeder link. Max distance between satellite and C-IoT device at min elevation angle; 40,581 km; 1,932 km (600 km altitude) 3,131 km (1,200 km altitude). Max Round Trip Delay (propagation delay only); 541.46ms (service and feeder links); 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links). Max differential delay within a cell; 10.3 ms; 3.12 ms and 3.18 ms for respectively 600km and 1200km. Max Doppler shift (earth fixed user equipment) (NOTE 6); 0.93 ppm; 24 ppm (600km) 21ppm(1200km). Max Doppler shift variation (earth fixed user equipment) (NOTE 6); 0.000 045 ppm/s; 0.27 ppm/s (600km) 0.13 ppm/s (1200km). C-IoT device motion on the earth; Min 0 km/s (stationary device), max 120 km/h; Min 0 km/s (stationary device), max 120 km/h. C-IoT device antenna types; Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4). C-IoT device max Tx power; UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm). C-IoT device Noise Figure; Omnidirectional antenna: 7 dB or 9 dB (NOTE 5). Service link; 3GPP defined Narrow Band IoT and eMTC. NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam. NOTE 4: The use of a Circular polarized antenna is optional. NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888 for device can be assumed for link budget. The noise figure is device vendor implementation specific. NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 7: System bandwidth is FFS

In this contribution, we will discuss on scenarios and evaluation methodology to verify the feasibility for NB-IoT/eMTC over NTN.

Discussion

Coverage performance

A link budget analysis is needed to evaluate the coverage performance of IoT NTN.

As shown in Table 1 and Table 2, link budget is evaluated in line with the latest satellite parameters (Set-1, Set-2, Set-3) and UE characteristics.

Table 1: Satellite parameters.

|  | Set-1 |  |  | Set 2 |  |  | Set 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 |
| Satellite altitude (km) | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 |
| Satellite antenna pattern | Section 6.4.1 in TR 38.811 |  |  |  |  |  |  |  |  |
| Payload characteristics for DL transmissions (S-band, 2 GHz) |  |  |  |  |  |  |  |  |  |
| Equivalent satellite antenna aperture (m) | 22 | 2 | 2 | 12 | 1 | 1 | 12 | 0.4 | 0.4 |
| Satellite EIRP density (dBW/MHz) | 59 | 40 | 34 | 53.5 | 34 | 28 | 59.8 | 33.7 | 28.3 |
| Satellite Tx max Gain (dBi) | 51 | 30 | 30 | 45.5 | 24 | 24 | 45.7 | 16.2 | 16.2 |
| 3dB beamwidth (deg) | 0.4011 | 4.4127 | 4.4127 | 0.7353 | 8.832 | 8.832 | 0.7 | 22.1 | 22.1 |
| Satellite beam diameter (km) | 250 | 90 | 50 | 450 | 190 | 90 | 459 | 470 | 234 |
| Payload characteristics for UL transmissions (S-band, 2 GHz) |  |  |  |  |  |  |  |  |  |
| Equivalent satellite antenna aperture (m) | 22 | 2 | 2 | 12 | 1 | 1 | 12 | 0.4 | 0.4 |
| G/T （dB/K) | 19 | 1.1 | 1.1 | 14 | -4.9 | -4.9 | 16.7 | -12.8 | -12.8 |
| Satellite Rx max Gain (dBi) | 51 | 30 | 30 | 45.5 | 24 | 24 | 45.7 | 16.2 | 16.2 |

Table 2: UE characteristics.

| Characteristics |  | C-IoT device |
| --- | --- | --- |
| Basic | Frequency band | S band (2 GHz) |
|  | Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
|  | Polarisation | Linear: +/-45°X-pol |
| DL | Channel Bandwidth | NB-IoT: 180 kHz eMTC: 1080 kHz |
|  | Rx Antenna gain | 0 dBi |
|  | Antenna temperature | 290 K |
| UL | Noise figure | 7 dB or 9dB |
|  | Tx transmit power | UE power class 3: 23dBm UE power class 5: 20dBm |
|  | Tx antenna gain | 0 dBi |
|  | Channel Bandwidth | - NB-IoT: 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080kHz, 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz |

Table 3 summarizes the path loss for IoT NTN scenarios.

Table 3: Path loss for link budget.

|  | GEO 35786 km | LEO 1200 km | LEO 600 km |
| --- | --- | --- | --- |
| Elevation angle (deg) | 10 | 10 | 10 |
| FSPL (dB) | 190.64 | 168.38 | 164.19 |
| Scintillation losses (dB) | 2.2 | 2.2 | 2.2 |
| Atmospheric losses (dB) | 0.2 | 0.2 | 0.2 |
| Polarization loss (dB) | 3 | 3 | 3 |
| Shadow margin (dB) | 3 | 3 | 3 |
| Additional losses [dB] | 9dB for carriage and container penetration loss or vegetation loss. |  |  |

Regarding the additional losses, carriage and container penetration loss and vegetation loss are considered.

For logistics application, carriage and container penetration loss is non-negligible. O2I car penetration loss modeling in [2] may be reused to calculate the carriage and container penetration loss, i.e.,

PLcar=Nμ,σP2

μ=9, σP=5. Optionally, for metallized car windows, μ=20 can be used.

For outdoor application, a UE may be surrounded by the vegetation, e.g., woods. In this case, the vegetation loss is non-negligible. The vegetation loss (PLv) can be derived as [3]

PLv=l∙γ

where, l is the length of path within the tree canopy (m), and γ is specific attenuation for very short vegetative paths (dB/m).

Figure 1: Specific attenuation due to woodland.

Figure 1 shows typical values for specific attenuation derived from various measurements over the frequency range 30 MHz to about 30 GHz in woodland [3]. According to Figure 1, at 2 GHz, γ≈0.4 dB/m.

Figure 2 gives an example of alee-trees. In this case, the length of path within the tree canopy (m) l can be calculated as,

l=hsinα∙Wd

where, α is the elevation angle (rad), h is the height of tree canopy, W is the width of tree canopy, d is the interval distance between two trees. When the elevation angle is 10 deg, h=6.5m, W=3m, d=5m, l is about 22.5 m.

Then the vegetation loss (PLv) can be calculated as

PLv=l∙γ=0.4dB / m*22.5m=9dB

Figure 2: Example of alee-trees.

Proposal 1: For link budget, the following additional pathloss needs to be considered.

- Carriage and container penetration loss (9~20dB) for logistics application.

- Vegetation loss (e.g., 9dB) for outdoor application.

In Appendix A1-A4, we provide the preliminary link budget results for both LEO and GEO satellites.

Connection density

The connection density was defined as the number of devices per unit area that fulfil a target quality of service (QoS) requirement to ensure a packet drop rate of 1% for a certain packet size and packet arrival rate [4]. The terrestrial IoT is able to fulfill the requirement of IMT-2020, i.e., 1 million device/km2, by assuming the BS deployment with ISD equaling to 500 m or 1732 m in different evaluation configurations.

The cell sizes in NTN are significantly larger than those considered while evaluating eMTC/NB-IoT connection density requirement in terrestrial cellular networks. For example, as listed in Table 1, the beam diameter is up to 40~470 km even in case of nadir point.

Therefore, it is more challengeable to fulfill the connection density requirement of IMT-2020. Instead, we may need to revisit the target connection density requirement for IoT NTN.

Proposal 2: Regarding connection density for IoT NTN, revisit the target requirement is needed.

Complexity

A low device complexity was one of the key design targets for NB-IoT and eMTC devices. In NTN, GNSS is assumed as default capability of terminal to enable the pre-compensation. Therefore, the complexity and cost of IoT NTN devices need to be estimated to account for GNSS.

Proposal 3: Regarding complexity of IoT device, investigate the impact of GNSS-assisted operation is needed.

Conclusions

In this contribution, we share our views on scenarios and evaluation methodology for IoT NTN. The observations and proposals are summarized as follows:

Proposal 1: For link budget, the following additional pathloss needs to be considered.

- Carriage and container penetration loss (9~20dB) for logistics application.

- Vegetation loss (e.g., 9dB) for outdoor application.

Proposal 2: Regarding connection density for IoT NTN, revisit the target requirement is needed.

Proposal 3: Regarding complexity of IoT device, investigate the impact of GNSS-assisted operation is needed.

References

- Chairman's Notes RAN1#103-e, October 26th – November 13th, 2020.

- TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

- ITU-R, P.833-8, “Attenuation in vegetation”.

- TR 38.913, “Study on scenarios and requirements for next generation access technologies”.

- RP-191524, “3GPP ITU-R Contribution - Final IMT-2020 Submission Overview”.

- Appendix

- A1: Link budgets for DL transmissions (2 GHz) for eMTC

Table 4-1: Link budget for DL transmissions (2 GHz) for eMTC without additional loss.

| Satellite orbit | Satellite parameters | UL/DL | TX: EIRP [dBW] | Noise figure [dB] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | DL | 59.3 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -3.1 |
|  |  |  | 59.3 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -5.1 |
| GEO | Set-2 | DL | 53.8 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -8.6 |
|  |  |  | 53.8 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -10.6 |
| GEO | Set-3 | DL | 60.1 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -2.3 |
|  |  |  | 60.1 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -4.3 |
| LEO-1200 | Set-1 | DL | 40.3 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | 0.2 |
|  |  |  | 40.3 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -1.8 |
| LEO-1200 | Set-2 | DL | 34.3 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -5.8 |
|  |  |  | 34.3 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -7.8 |
| LEO-1200 | Set-3 | DL | 34.0 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -6.1 |
|  |  |  | 34.0 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -8.1 |
| LEO-600 | Set-1 | DL | 34.3 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -1.6 |
|  |  |  | 34.3 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -3.6 |
| LEO-600 | Set-2 | DL | 28.3 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -7.6 |
|  |  |  | 28.3 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -9.6 |
| LEO-600 | Set-3 | DL | 28.6 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -7.3 |
|  |  |  | 28.6 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -9.3 |

Table 4-2: Link budget for DL transmissions (2 GHz) for eMTC with additional loss (e.g., carriage penetration or vegetation loss).

| Satellite orbit | Satellite parameters | UL/DL | TX: EIRP [dBW] | Noise figure [dB] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | DL | 59.3 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -12.1 |
|  |  |  | 59.3 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -14.1 |
| GEO | Set-2 | DL | 53.8 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -17.6 |
|  |  |  | 53.8 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -19.6 |
| GEO | Set-3 | DL | 60.1 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -11.3 |
|  |  |  | 60.1 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -13.3 |
| LEO-1200 | Set-1 | DL | 40.3 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -8.8 |
|  |  |  | 40.3 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -10.8 |
| LEO-1200 | Set-2 | DL | 34.3 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -14.8 |
|  |  |  | 34.3 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -16.8 |
| LEO-1200 | Set-3 | DL | 34.0 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -15.1 |
|  |  |  | 34.0 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -17.1 |
| LEO-600 | Set-1 | DL | 34.3 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -10.6 |
|  |  |  | 34.3 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -12.6 |
| LEO-600 | Set-2 | DL | 28.3 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -16.6 |
|  |  |  | 28.3 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -18.6 |
| LEO-600 | Set-3 | DL | 28.6 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -16.3 |
|  |  |  | 28.6 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -18.3 |

- A2: Link budgets for DL transmissions (2 GHz) for NB-IoT

Table 5-1: Link budget for DL transmissions (2 GHz) for NB-IoT without additional loss.

| Satellite orbit | Satellite parameters | UL/DL | TX: EIRP [dBW] | Noise figure [dB] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | DL | 51.6 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -3.1 |
|  |  |  | 51.6 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -5.1 |
| GEO | Set-2 | DL | 46.1 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -8.6 |
|  |  |  | 46.1 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -10.6 |
| GEO | Set-3 | DL | 52.4 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -2.3 |
|  |  |  | 52.4 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -4.3 |
| LEO-1200 | Set-1 | DL | 32.6 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | 0.2 |
|  |  |  | 32.6 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -1.8 |
| LEO-1200 | Set-2 | DL | 26.6 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -5.8 |
|  |  |  | 26.6 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -7.8 |
| LEO-1200 | Set-3 | DL | 26.3 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -6.1 |
|  |  |  | 26.3 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -8.1 |
| LEO-600 | Set-1 | DL | 26.6 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -1.6 |
|  |  |  | 26.6 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -3.6 |
| LEO-600 | Set-2 | DL | 20.6 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -7.6 |
|  |  |  | 20.6 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -9.6 |
| LEO-600 | Set-3 | DL | 20.9 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -7.3 |
|  |  |  | 20.9 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -9.3 |

Table 5-2: Link budget for DL transmissions (2 GHz) for NB-IoT with additional loss (e.g., carriage penetration or vegetation loss).

| Satellite orbit | Satellite parameters | UL/DL | TX: EIRP [dBW] | Noise figure [dB] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | DL | 51.6 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -12.1 |
|  |  |  | 51.6 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -14.1 |
| GEO | Set-2 | DL | 46.1 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -17.6 |
|  |  |  | 46.1 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -19.6 |
| GEO | Set-3 | DL | 52.4 | 7.0 | -31.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -11.3 |
|  |  |  | 52.4 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -13.3 |
| LEO-1200 | Set-1 | DL | 32.6 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -8.8 |
|  |  |  | 32.6 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -10.8 |
| LEO-1200 | Set-2 | DL | 26.6 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -14.8 |
|  |  |  | 26.6 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -16.8 |
| LEO-1200 | Set-3 | DL | 26.3 | 7.0 | -31.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -15.1 |
|  |  |  | 26.3 | 9.0 | -33.6 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -17.1 |
| LEO-600 | Set-1 | DL | 26.6 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -10.6 |
|  |  |  | 26.6 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -12.6 |
| LEO-600 | Set-2 | DL | 20.6 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -16.6 |
|  |  |  | 20.6 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -18.6 |
| LEO-600 | Set-3 | DL | 20.9 | 7.0 | -31.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -16.3 |
|  |  |  | 20.9 | 9.0 | -33.6 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -18.3 |

- A3: Link budgets for UL transmissions (2 GHz) for eMTC

Table 6-1: Link budget for UL transmissions (2 GHz) for eMTC without additional loss.

| Satellite orbit | Satellite parameters | UL/DL | UE power class | TX: EIRP [dBm] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | UL | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -18.8 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -11.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -8.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -5.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -3.2 |
| GEO | Set-1 | UL | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -21.8 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -14.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -11.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -8.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -6.2 |
| GEO | Set-2 | UL | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -23.8 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -16.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -13.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -10.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -8.2 |
| GEO | Set-2 | UL | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -26.8 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -19.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -16.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -13.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -11.2 |
| GEO | Set-3 | UL | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -21.1 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -13.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -10.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -7.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -5.5 |
| GEO | Set-3 | UL | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -24.1 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -16.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -13.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -10.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -8.5 |
| LEO-1200 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -14.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -6.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -3.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -0.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | 1.1 |
| LEO-1200 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -17.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -9.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -6.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -3.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -1.9 |
| LEO-1200 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -20.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -12.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -9.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -6.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -4.9 |
| LEO-1200 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -23.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -15.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -12.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -9.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -7.9 |
| LEO-1200 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -28.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -20.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -17.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -14.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -12.8 |
| LEO-1200 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -31.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -23.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -20.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -17.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -15.8 |
| LEO-600 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -10.2 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -2.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | 0.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | 3.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | 5.3 |
| LEO-600 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -13.2 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -5.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -2.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | 0.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | 2.3 |
| LEO-600 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -16.2 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -8.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -5.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -2.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -0.7 |
| LEO-600 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -19.2 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -11.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -8.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -5.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -3.7 |
| LEO-600 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -24.1 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -16.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -13.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -10.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -8.6 |
| LEO-600 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 1080.0 | -27.1 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.0 | -19.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.0 | -16.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.0 | -13.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 30.0 | -11.6 |

Table 6-2: Link budget for UL transmissions (2 GHz) for eMTC with additional loss (e.g., carriage penetration or vegetation loss).

| Satellite orbit | Satellite parameters | UL/DL | UE power class | TX: EIRP [dBm] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | UL | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -27.8 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -20.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -17.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -14.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -12.2 |
| GEO | Set-1 | UL | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -30.8 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -23.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -20.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -17.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -15.2 |
| GEO | Set-2 | UL | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -32.8 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -25.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -22.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -19.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -17.2 |
| GEO | Set-2 | UL | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -35.8 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -28.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -25.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -22.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -20.2 |
| GEO | Set-3 | UL | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -30.1 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -22.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -19.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -16.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -14.5 |
| GEO | Set-3 | UL | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -33.1 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -25.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -22.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -19.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -17.5 |
| LEO-1200 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -23.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -15.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -12.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -9.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -7.9 |
| LEO-1200 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -26.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -18.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -15.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -12.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -10.9 |
| LEO-1200 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -29.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -21.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -18.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -15.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -13.9 |
| LEO-1200 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -32.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -24.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -21.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -18.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -16.9 |
| LEO-1200 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -37.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -29.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -26.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -23.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -21.8 |
| LEO-1200 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -40.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -32.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -29.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -26.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -24.8 |
| LEO-600 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -19.2 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -11.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -8.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -5.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -3.7 |
| LEO-600 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -22.2 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -14.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -11.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -8.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -6.7 |
| LEO-600 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -25.2 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -17.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -14.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -11.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -9.7 |
| LEO-600 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -28.2 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -20.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -17.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -14.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -12.7 |
| LEO-600 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -33.1 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -25.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -22.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -19.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -17.6 |
| LEO-600 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 1080.0 | -36.1 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.0 | -28.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.0 | -25.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.0 | -22.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 30.0 | -20.6 |

- A4: Link budgets for UL transmissions (2 GHz) for NB-IoT

Table 7-1: Link budget for UL transmissions (2 GHz) for NB-IoT without additional loss.

| Satellite orbit | Satellite parameters | UL/DL | UE power class | TX: EIRP [dBm] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | UL | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -11.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -8.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -5.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -0.2 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 5.8 |
| GEO | Set-1 | UL | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -14.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -11.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -8.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -3.2 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 2.8 |
| GEO | Set-2 | UL | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -16.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -13.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -10.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -5.2 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 0.8 |
| GEO | Set-2 | UL | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -19.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -16.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -13.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -8.2 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | -2.2 |
| GEO | Set-3 | UL | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -13.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -10.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -7.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -2.5 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 3.5 |
| GEO | Set-3 | UL | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -16.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -13.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -10.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -5.5 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 0.5 |
| LEO-1200 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -6.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -3.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -0.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | 4.2 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 10.2 |
| LEO-1200 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -9.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -6.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -3.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | 1.2 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 7.2 |
| LEO-1200 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -12.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -9.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -6.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -1.8 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 4.2 |
| LEO-1200 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -15.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -12.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -9.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -4.8 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 1.2 |
| LEO-1200 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -20.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -17.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -14.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -9.7 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | -3.7 |
| LEO-1200 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -23.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -20.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -17.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -12.7 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | -6.7 |
| LEO-600 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -2.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | 0.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | 3.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | 8.3 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 14.4 |
| LEO-600 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -5.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -2.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | 0.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | 5.3 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 11.4 |
| LEO-600 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -8.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -5.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -2.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | 2.3 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 8.4 |
| LEO-600 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -11.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -8.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -5.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -0.7 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 5.4 |
| LEO-600 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -16.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -13.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -10.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -5.6 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | 0.5 |
| LEO-600 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 180.00 | -19.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 90.00 | -16.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 45.00 | -13.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 15.00 | -8.6 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 0.0 | 3.75 | -2.5 |

Table 7-2: Link budget for UL transmissions (2 GHz) for NB-IoT with additional loss (e.g., carriage penetration or vegetation loss).

| Satellite orbit | Satellite parameters | UL/DL | UE power class | TX: EIRP [dBm] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | BW (kHZ) | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | Set-1 | UL | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -20.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -17.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -14.0 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -9.2 |
|  |  |  | Class 3 | 23.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -3.2 |
| GEO | Set-1 | UL | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -23.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -20.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -17.0 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -12.2 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -6.2 |
| GEO | Set-2 | UL | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -25.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -22.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -19.0 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -14.2 |
|  |  |  | Class 3 | 23.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -8.2 |
| GEO | Set-2 | UL | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -28.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -25.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -22.0 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -17.2 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -11.2 |
| GEO | Set-3 | UL | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -22.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -19.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -16.3 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -11.5 |
|  |  |  | Class 3 | 23.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -5.5 |
| GEO | Set-3 | UL | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -25.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -22.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -19.3 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -14.5 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -8.5 |
| LEO-1200 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -15.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -12.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -9.6 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -4.8 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | 1.2 |
| LEO-1200 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -18.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -15.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -12.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -7.8 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -1.8 |
| LEO-1200 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -21.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -18.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -15.6 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -10.8 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -4.8 |
| LEO-1200 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -24.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -21.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -18.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -13.8 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -7.8 |
| LEO-1200 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -29.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -26.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -23.5 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -18.7 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -12.7 |
| LEO-1200 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -32.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -29.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -26.5 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -21.7 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 168.4 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -15.7 |
| LEO-600 | Set-1 | UL | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -11.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -8.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -5.4 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -0.7 |
|  |  |  | Class 3 | 23.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | 5.4 |
| LEO-600 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -14.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -11.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -8.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -3.7 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | 2.4 |
| LEO-600 | Set-2 | UL | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -17.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -14.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -11.4 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -6.7 |
|  |  |  | Class 3 | 23.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -0.6 |
| LEO-600 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -20.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -17.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -14.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -9.7 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -3.6 |
| LEO-600 | Set-3 | UL | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -25.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -22.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -19.3 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -14.6 |
|  |  |  | Class 3 | 23.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -8.5 |
| LEO-600 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 180.00 | -28.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 90.00 | -25.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 45.00 | -22.3 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 15.00 | -17.6 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.2 | 164.2 | 3.0 | 2.2 | 3.0 | 9.0 | 3.75 | -11.5 |