3GPP TSG RAN WG1 Meeting #104-e	R1-2101175

# e-Meeting, January 25th – February 5th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Sidelink Physical Duration to Logical Slot Conversion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 12-Jan-2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Slots used for SL (sidelink) reception and transmissions of a resource pool are a subset of the slots available for sidelink reception and transmission. This subset is determined by a bitmap of length Lbitmap configured by higher layers. In the specifications SL slots are denoted by tiSL, whereas, SL slots that belong a resource pool are denoted as t'iSL. The index of the slots of a SL resource pool t'iSL are numbered number successively 0, 1, … , wherein, i is the logical slot index. In section 8.1.7 of TS 38.214, when converting a physical duration (i.e. resource reservation period Prsvp) to logical slots, the length of the bitmap and the number of bits in the bitmap that belong to the resource pool is not taken into account. Furthermore, slots used for synchronization (S-SSB slots) and reserved slots are excluded from being candidate slots for a SL resource pool. This leads to the duration of the logical resource reservation period being longer than the physical resource reservation. This is corrected applying additional scaling to the physical duration of the resource reservation period to account for slots excluded from the resource pool such S-SSB slots and reserved slots as well as accounting for slots that are not part of the SL resource pool based on the bitmap configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 8.1.7, apply appropriate scaling to the logical resource reservation period so that the duration of the logical resource reservation period is approximately equal to the physical duration of the resource reservation period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE might overestimate the logical resource reservation period. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR impacts the conversion of physical duration to logical slots, by correcly acccouting for slots SL slots not configured for a resource pool. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### UE procedure for determining the number of logical slots for a reservation period

A given resource reservation period Prsvp in milliseconds is converted to a period Prsvp' in logical slots as:

Prsvp'=NSL-RPN20 ms×Prsvp

where NSL-RP is the average number of slots that can be used for SL transmission for a SL resource pool within 20 ms of the configured UL-DL configuration. Where,

NSL-RP=N-NS-SSB+Nreserved512×Lbitmap,oneLbitmap

- N is the number of slots that can be used for SL in 20ms interval.

- NS-SSB is the number of S-SSB slots across 10240×2μ slots .

- Nreservedis the number of reserved slots across 10240×2μ slots.

- Lbitmap is the length of the bitmap of the SL resource pool.

- Lbitmap,one is the number of bits in the bitmap of the SL resource pool with logical 1.