3GPP TSG RAN WG1 #104-e							    R1-2101809

e-Meeting, January 26th – February 5th, 2021

Source:	Moderator (Intel Corporation)

Title:	Feature lead summary on Mode-2 resource allocation issues in Rel.16 5G V2X

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution provides a summary of proposed correction to Mode-2 sidelink resource allocation for NR-V2X communication, based on review of the submitted contributions [1]-[50].

Identification of email discussions

It is proposed to organize the following email discussion:

- M2 thread 1

  - M2-14: Correction to step 6) to include slots within Tproc0 (R1-2100945)

  - M2-17: Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions (R1-2101533)

- Separate thread for uncaptured agreements

  - M2-3: Capture RAN1#103-e agreement on pre-emption

- Separate thread for LS reply

  - M2-10: MCS range for Mode-2 per MCS table (reply to R1-2009644)

Finally agreed threads:

[104-e-NR-5G_V2X-04]: UE behaviour regarding non-monitored slots in mode 2, till 1/28, with potential CRs till 2/2 – Sergey (Intel)

- M2-17: Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions

- Changes for the uncaptured agreement (M2-3: Capture RAN1#103-e agreement on pre-emption) can be discussed during the CR preparation.

Issue list

Open issue list:

- M2-1: Infinite loop or candidate resource set starvation due to step 5)

- M2-2: Clarification on UE procedure for determining the number of logical slots for a reservation period

- M2-9: HARQ RTT time gap capturing issue in MAC

- M2-14: Correction to step 6) to include slots within Tproc0 (R1-2100945)

- M2-15: Resource reservation period setting capturing issue in MAC

- M2-16: Handling of multiple TBs during resource selection

- M2-17: Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions (R1-2101533)

- M2-18: Exclude the slots with PSFCH when sl-LengthSymbols≤9 in the identification of candidate resources in the sensing procedure (R1-2101533)

Uncaptured RAN1#103-e agreements

- M2-3: Capture RAN1#103-e agreement on pre-emption

LS replies

- M2-10: MCS range for Mode-2 per MCS table (reply to R1-2009644)

Analysis of Draft Corrections

| Tdoc# | Issue within the tdoc | Issue index | FL comment |
| --- | --- | --- | --- |
| R1-2100137, OPPO | Infinite loop or candidate resource set starvation due to step 5) | 1 | Can be discussed |
| R1-2100204, Huawei, HiSilicon | Infinite loop or candidate resource set starvation due to step 5) | 1 | Can be discussed |
|  | Clarification on UE procedure for determining the number of logical slots for a reservation period | 2 | Open issue. Decide whether to handle as Mode-2 or Procedures |
|  | Capture RAN1#103-e agreement on pre-emption | 3 | Include as a separate thread |
| R1-2100334, CATT, GOHIGH | Do not increment RSRP threshold in some cases for pre-emption | 4 | Optimization |
|  | Infinite loop or candidate resource set starvation due to step 5) | 1 | Can be discussed |
|  | Backward indication | 5 | Optimization |
|  | Name “sl-ThresPSSCH-RSRP-List” should be modified to “sl-Thres-RSRP-List” | 6 | Optimization |
|  | "remaining packet budget" should be replaced by "remaining packet delay budget" | 7 | Editorial |
| R1-2100411, vivo | Define dropping when HARQ RTT violated | 8 | Optimization |
|  | HARQ RTT time gap capturing issue in MAC | 9 | MAC capturing issue |
| R1-2100515, LG Electronics | MCS range for Mode-2 to be configured per MCS table | 10 | LS reply to R1-2009644 |
|  | Setting reservation period in non-initial resource selection after pre-emption | 11 | Optimization |
| R1-2100630, Intel Corporation | Capture RAN1#103-e agreement on pre-emption | 3 | Include as a separate thread |
| R1-2100799, Spreadtrum Communications | Sensing window to be Pm + 100 ms where Pm is the maximum of pre-configured periods | 12 | Optimization |
|  | X⋅Mtotal candidates should be selected and reported to higher layers | 13 | Optimization |
| R1-2100938, ZTE, Sanechips | Clarification on UE procedure for determining the number of logical slots for a reservation period | 2 | Open issue. Decide whether to handle as Mode-2 or Procedures |
| R1-2100945, NEC | Correction to step 6) to include slots within Tproc0 | 14 | Can be discussed |
| R1-2101073, ETRI | Capture resource reservation period setting either in L1 or MAC | 15 | MAC capturing issue |
| R1-2101175, Samsung | Clarification on UE procedure for determining the number of logical slots for a reservation period | 2 | Open issue. Decide whether to handle as Mode-2 or Procedures |
| R1-2101176, Samsung | MCS range for Mode-2 to be configured per MCS table | 10 | LS reply to R1-2009644 |
| R1-2101346, Apple | Handling of multiple TBs during resource selection | 16 | Can be discussed |
|  | Infinite loop or candidate resource set starvation due to step 5) | 1 | Can be discussed |
| R1-2101437, Qualcomm Incorporated | Infinite loop or candidate resource set starvation due to step 5) | 1 | Can be discussed |
| R1-2101533, Sharp | Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions | 17 | Can be discussed |
|  | Exclude the slots with PSFCH when sl-LengthSymbols≤9 in the identification of candidate resources in the sensing procedure | 18 | Can be discussed |
|  | Clarification on timing relation between re-evaluation moment and initial selection moment | 19 | Optimization |
| R1-2101571, ASUSTeK | Handling of multiple TBs during resource selection | 16 | Can be discussed |
| R1-2101582, NTT DOCOMO, INC. | MCS range for Mode-2 to be configured per MCS table | 10 | LS reply to R1-2009644 |
| R1-2101759, Nokia, Nokia Shanghai Bell | Handling of multiple TBs during resource selection | 16 | Can be discussed |

- References

Contributions identified by FL to contain Mode-2 related issues:

- R1-2100137	Remaining open issues and corrections for mode 1 and mode 2 RA	OPPO

- R1-2100204	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2100334	Discussion and TPs on resource allocation in NR V2X	CATT, GOHIGH

- R1-2100411	Maintenance on resource allocation mechanisms for NR sidelink	vivo

- R1-2100515	Discussion on essential corrections in resource allocation for Mode 1 and 2	LG Electronics

- R1-2100630	Corrections to Mode-2 resource allocation	Intel Corporation

- R1-2100799	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2100938	The slot set for SL resource allocation procedure	ZTE, Sanechips

- R1-2100945	Remaining issues on resource allocation mode 2	NEC

- R1-2101073	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

- R1-2101175	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2101176	Maintenance for NR Sidelink Mode 2 Operation	Samsung

- R1-2101346	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2101437	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2101533	Remaining issues on resource allocation for NR sidelink	Sharp

- R1-2101571	Remaining issues on sidelink mode 2	ASUSTeK

- R1-2101582	Maintenance for sidelink synchronization and mode 2	NTT DOCOMO, INC.

- R1-2101759	Remaining details for Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

Other Rel.16 NR V2X contributions

- R1-2100135	Draft TP on physical strucutre for NR sidelink	OPPO

- R1-2100136	Remaining open issues and corrections for physical layer procedure	OPPO

- R1-2100333	Discussion and TPs on sidelink synchronization mechanism and physical layer structure in NR V2X	CATT, GOHIGH

- R1-2100335	Discussion and TPs on physical layer procedures in NR V2X	CATT, GOHIGH

- R1-2100410	Maintenance on physical layer structure for NR sidelink	vivo

- R1-2100412	Maintenance on NR sidelink synchronization and procedures	vivo

- R1-2100514	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2100516	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2100629	Correction to FD-OCC for PSCCH	Intel Corporation

- R1-2100631	Corrections to sidelink procedures	Intel Corporation

- R1-2100734	A remaining issue on Mode-1 resource allocation for NR sidelink	Fujitsu

- R1-2100735	Remaining issues on physical layer procedures for NR sidelink	Fujitsu

- R1-2100800	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2100936	Remaining issues on sidelink synchronization	ZTE, Sanechips

- R1-2100937	Remaining issues on mode1	ZTE, Sanechips

- R1-2101174	Maintenance for NR Sidelink Physical Layer Structure	Samsung

- R1-2101344	Remaining Issues of Sidelink Physical Layer Procedures	Apple

- R1-2101345	Remaining Issue of Mode 1 Resource Allocation	Apple

- R1-2101436	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

- R1-2101438	Remaining Issues in Physical Layer Procedure	Qualcomm Incorporated

- R1-2101532	Remaining issues on physical layer structure and procedures for NR sidelink	Sharp

- R1-2101534	Remaining issues on synchronization mechanism for NR sidelink	Sharp

- R1-2101581	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2101583	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2101649	Remaining issues on type-1 HARQ-ACK codebook considering multiple sidelink reosurce pools	ASUSTeK

- R1-2101650	Remaining issues on sidelink procedure	ASUSTeK

- R1-2101707	Draft_CR_TS38.212	Ericsson

- R1-2101708	Draft_CR_TS38.213	Ericsson

- R1-2101709	Draft_CR_TS38.306	Ericsson

- R1-2101732	Correction on PSBCH payload generation	Huawei, HiSilicon

- R1-2101733	Correction on determination of PSFCH resources based on a set of configured PRBs	Huawei, HiSilicon

- R1-2101760	Remaining details for Physical layer structure for sidelink	Nokia, Nokia Shanghai Bell