3GPP TSG-RAN WG1 Meeting #104-e	R1-2101820

e-Meeting, January 25th - February 5th, 2021

Agenda Item:	8.2.7

Source:	ZTE, Sanechips

Title:	Evaluation results for 52.6 to 71GHz

Document for:	Discussion

- Introduction

At the RAN #90 meeting, a new WID on extending current NR operation to 71 GHz [1] was approved. And in the WID, the following objectives may need further evaluation.

  - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

- Link level simulation results

  - PTRS

In above 52.6 GHz SI phase, it has been discussed that for high modulation order(64QAM) and lower subcarrier spacing(e.g. 120kHz, 480kHz), ICI compensation method, which is probably more complex and efficient compared with CPE compensation, could be utilized to mitigate the phase noise.

There are 2 kinds of ICI compensation approach with  taps introduced in SI phase:

- Direct De-ICI approach: This approach could be performed with Rel-15 legacy PTRS pattern, the ICI estimation equations are formulated by the distributed PTRS REs and the adjacent data REs, then the received signals will be directly filtered by the estimated filter coefficients.

- ICI filter approximation approach: This approach relies on the consecutive PTRS structure of block PTRS, each  PTRS REs can be circularly used to formulate one ICI estimation equation, then the received signals will be filtered by the conjugate inverse of the estimated filter coefficients.

- Rel-15 legacy PTRS        (b)Block PTRS          (c)Hybrid PTRS

Figure 2.1-1. Different PTRS patterns

As shown in Figure 2.1-1, (a) is the Rel-15 legacy PTRS pattern, there will be one PTRS RE for each 2 RBs if frequency density K=2; (b) is block PTRS, the total number of PTRS REs is the same as the legacy distributed PTRS pattern, and these PTRS REs are consecutively placed in frequency domain; (c) is the hybrid PTRS, which combines the features of legacy PTRS and block PTRS, multiple blocks are distributed uniformly in frequency domain.

For block PTRS, the PTRS REs are concentrated in a small part of the whole bandwidth, therefore the frequency diversity couldn’t be reflected well. For hybrid PTRS, each block requires at least  consecutive PTRS REs to formulate enough equations for ICI estimation, it might be difficult to balance the ICI estimation accuracy and the frequency diversity for each PTRS density configuration.

In order to study whether PTRS enhancement is beneficial, evaluations are performed with the legacy PTRS, block PTRS and hybrid PTRS. To keep the comparison in a relatively fair condition, it is assumed that for each PTRS pattern, 3-tap ICI compensation method are used, the total PTRS RE number is the same as Rel-15 legacy PTRS with (K=2, L=1).

Figure 2.1-2 Performance of different PTRS patterns

As illustrated in Figure 2.1-2, for 120kHz TDL-A 5ns with 64QAM modulation,

- ICI compensation can achieve 10% BLER while CPE compensation can not.

- ICI compensation based on legacy PTRS, block PTRS and hybrid PTRS shows similar performance.

And for 480kHz TDL-A 5ns with 64QAM modulation,

- The performance of ICI compensation based on block PTRS is similar as CPE compensation.

- The performance of ICI compensation based on hybrid PTRS and legacy PTRS are similar, both provide about 1dB gain @10% BLER and more than 2dB gain @1% BLER compared with CPE compensation.

Observation 1: ICI compensation based on legacy PTRS can achieve similar or better performance compared with ICI compensation based on block PTRS and hybrid PTRS.

To investigate the complexity of each ICI compensation approach based on different PTRS patterns, the following main formulas in [4] are analyzed:

Direct De-ICI approach:

(1)

ICI filter approximation approach:

(2)

Where and  are respectively ICI filter taps of Direct De-ICI approach and ICI filter approximation approach, it can be seen that the calculation procedures are similar for the 2 approaches, the factors impact the complexity would be the dimension of the matrices.

Assume the number of PTRS REs is M, then for legacy PTRS with Direct De-ICI approach, is a  matrix,  is a  matrix, so ; for block PTRS with  ICI filter approximation approach, is a matrix, is a  matrix, there is also one matrix inverse calculation of dimension ; for hybrid PTRS with N blocks, X is a  matrix, r is a  matrix.

It can be seen from the analysis above, there is one matrix inverse calculation of dimension  for these three kinds of PTRS patterns, the matrix inverse complexity of the 3 patterns is the same. The number of complex multiplication and complex addition is slightly larger for legacy PTRS with Direct De-ICI approach[5], for example, for 400MHz bandwidth 120kHz SCS, assume K=2 , the complexity comparison of legacy PTRS, block PTRS and hybrid PTRS can be shown as:

Table 2.1-1. Complexity comparison of legacy PTRS, block PTRS and hybrid PTRS

|  | Complex multiplication | Complex addition | Matrix inverse |
| --- | --- | --- | --- |
| Legacy PTRS | 12032 | 8436 | 1 |
| Block PTRS | 11990 | 8400 | 1 |
| Hybrid PTRS | 11856 | 8292 | 1 |

Observation 2: The calculation complexity of ICI compensation based on legacy PTRS, block PTRS and hybrid PTRS is similar.

  - DMRS

In above 52.6GHz SI phase, some companies studied and evaluated DMRS enhancement in frequency domain, i.e. extend the DMRS in every RE in order to perform channel estimation properly. The reason to raise this thought is that, in 60GHz with larger subcarrier spacing (e.g.960kHz), Rel-15 DMRS density may not be sufficient to reflect the frequency selection characteristics. In this chapter, it is investigated and evaluated whether enhancement on Rel-15 DMRS is needed or not.

Figure 2.2-1 Rel-15 DMRS Type 1 and new DMRS pattern

Consider 400MHz bandwidth with 480kHz and 960kHz SCS, 64QAM modulation order, the performance of Rel-15 DMRS Type 1 and new DMRS pattern with low and high delay spread are shown in Figure 2.2-4. In order to get rid of the impact of phase noise in high frequency, it is assumed ideal PN estimation and compensation in the evaluation of DMRS patterns.

(a) TDL-A 5ns 480kHz                                                     (b) TDL-A 20ns 480kHz

(c) TDL-A 5ns 960kHz                                                        (d) TDL-A 20ns 960kHz

Figure 2.2-4 performance of Rel-15 DMRS Type 1 and new DMRS pattern

As illustrated in Figure 2.2-4, for 480kHz and 960kHz, new DMRS pattern with higher DMRS density is slightly better(less than 1dB gain) than Rel-15 DMRS Type 1, both in low delay spread and high delay spread, so there is no need to further enhance DMRS patterns.

Observation 3: Rel-15 DMRS Type 1 and the new DMRS pattern that fully occupied in frequency domain show comparable performance.

  - PUCCH

- CM calculation

In order to minimize the spec impact, it is better to reuse the current design. So we firstly calculate the CM of sequence using the Rel-15 CGS of length 12 with X times repetition directly. And the CCDF of CM is shown in Figure 2.3-1.

Figure 2.3-1 CM of direct-CGS repetition with different length

As shown in Figure 2.3-1, the CM value increases rapidly with the PRB number. And the CM at 95th percentile is about 3.6 dB even for 2 PRBs. It’s obvious that direct-CGS repetition sequence is not appropriate due to the high CM values for multiple PRBs, therefore this sequence is not considered in the analysis in this section..

Next in this section, the CM of ZC sequence and CGS extended sequence with different sequence length in granularity of PRBs are calculated and analyzed.

Figure 2.3-2 CM of CGS and ZC for sequences with different length

In Figure 2.3-2, the legend of “CGS extended,RBX” means the sequence is CGS extended sequence , and the cyclic shift increased by a step of 5 from the first PRB to the last PRB, the legend of “ZC,RBX” denotes ZC sequence of low PAPR sequence generation in 38.211. The “RBX” in each legends is the number of PRB.

The CM at 95th percentile is listed in Table 2.3-1.

Table 2.3-1  CM values at 95th percentile of different number of PRBs for CGS and ZC sequence

| # RB | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 20 | 24 | 28 | 32 | 36 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CGS | 2.88 | 3.79 | 3.2 | 2.38 | 1.846 | 1.49 | 0.99 | 1.50 | 2.10 | 2.34 | 3.05 | 3.63 | 3.96 | 4.57 |
| ZC | 2.01 | 2.16 | 2.24 | 1.58 | 1.85 | 2.07 | 2.30 | 2.30 | 2.30 | 2.19 | 2.25 | 2.37 | 2.27 | 2.30 |

Note: In Table 2.3-1, “CGS” denotes CGS extended.

From Figure 2.3-2 and Table 2.3-1, it can be seen that, for PRB number 10~20, the 95th percentile CM of “CGS extended” with the solid curve is lower than that of “ZC” with the dash curve. When the number of PRB larger than 20, the 95th percentile CM of CGS extended is increased with the number of PRB, while the CM of ZC sequence is stable.

Observation 4: The CGS extended sequence has lower CM than ZC sequence when the PRB number is 10~20, while the performance gets worse for other numbers of PRBs.

Observation 5: The CM of ZC sequence is always kept under 2.5dB with different PRB number.

- Detection performance

The detection performance is shown in Figure 2.3-3, and the FAR is kept under 1%. The key simulation parameters are listed in Table 2.3-2.

Table 2.3-2 LLS simulation parameters

| Parameters | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency |  | 60GHz |
| Sub-carrier spacing |  | 120kHz |
| Channel Model |  | TDL-C 100ns |
| Antenna configuration |  | 1T2R |
| UE speed |  | 3km/h |

In 52.6GHz~71GHz frequency band, phase noise is also a factor different from low frequency band. PUCCH is sequence or QPSK modulated symbols, which are not sensitive to phase noise. Therefore phase noise is not considered in this simulation.

Figure 2.3-3 PMD and error rate for different case

The legend “direct-CGS repetition,RBX” denotes repeating the rel-15 CGS with length 12 directly X times, here X is the number of PRB.

From Figure 2.3-3, we can see that the detection performance is similar for different cases under the same sequence length. The longer sequence length, the lower value of the PMD (P(ACK to DTX))and the error rate(P(NACK to ACK)).

Observation 6: The detection performance is proportional to the sequence length while independent of sequence types.

- Coverage analysis

Based on the detection performance, the MCL calculation is provided in the following table.

Table 2.3-3. Reported metrics for enhanced PUCCH format 0 and 1 performance evaluation(1 bit HARQ ACK detection)

| Number of PRB | 1 | 10 | 20 |
| --- | --- | --- | --- |
| SCS [kHz] | 120 | 120 | 120 |
| Bandwidth [MHz] | 1.44 | 14.4 | 28.8 |
| Noise level, Np (dBm) | -102.4 | -92.4 | -89.4 |
| Required SNR [dB](Measured for 1% missed detection with 1% false alarm) | 6.47 | -5.2 | -8.2 |
| Max Tx power (according to regulations) [dBm](max PSD=13dBm/Hz) | 8.6 | 18.6 | 21.6 |
| Max Tx power (according to regulations) [dBm](max PSD=23dBm/Hz) | 24.6 | 34.6 | 37.6 |
| Maximum coupling loss (MCL) [dB](max PSD=13dBm/Hz) | 104.5 | 116.2 | 119.2 |
| Maximum coupling loss (MCL) [dB](max PSD=23dBm/Hz) | 120.5 | 132.2 | 135.2 |

The MCL calculation is relative to the required SNR under the same Tx power and noise level assumption, since the detection performance of different sequence types with the same sequence length is similar, MCL is also independent of sequence types. Therefore, in Table 3, the parameters and values are collected from ZC sequence. It is illustrated that with the increasing number of PRBs, the MCL value is also increasing.

Observation 7: The coverage becomes better with sequence length increasing while independent of sequence types.

- System level simulation results

3.1 Deployment scenarios

Figure 3.1-1 Indoor scenario A

In this section, the simulation is performed to study channelization schemes. The evaluation scenario is Indoor scenario A. There are two operators in the scenario (operator A and operator B), no LBT is used in both operators for the DL only case.

- Aligned case                                                  (b) Misaligned case

Figure 3.1-2 Aligned and misaligned cases for two operators

The channel of aligned case is shown in Figure 3.1-2(a), two operators have the same channel bandwidth 2GHz, and the channels are aligned with each other. In Figure 3.1-2(b), operator B uses three 800MHz channels misaligned with 2GHz channel used by operator A, the UEs of operator B are divided into three groups, each group can be scheduled in one sub-channel. Furthermore, in misaligned case, to achieve a relatively fair comparison, operator B has the same power per channel as operator A.

3.2 Channelization simulation

In this section, the simulation is performed to study the influence of the NR channelization aligned or misaligned with 802.11ad channelization in 60 GHz frequency.

Figure 3.2-1 compares the throughput of Operator A in downlink at both low load and medium load. Operator A in aligned case is represented by the blue bar and Operator A in misaligned case is represented by the orange bar.

Figure 3.2-1 DL mean user throughput

It can be shown that, in terms of user throughput with different traffic loads, the performance of Operator A in misaligned case is comparable to the aligned case.

Observation 8: Aligned and misaligned channelization show similar performance in coexistence scenario.

Proposal 1: It is not necessary to align NR channelization with IEEE 802.11ad channelization from coexistence perspective.

Conclusion

Observation 1: ICI compensation based on legacy PTRS can achieve similar or better performance compared with ICI compensation based on block PTRS and hybrid PTRS.

Observation 2: The calculation complexity of ICI compensation based on legacy PTRS, block PTRS and hybrid PTRS is similar.

Observation 3: Rel-15 DMRS Type 1 and the new DMRS pattern that fully occupied in frequency domain show comparable performance.

Observation 4: The CGS extended sequence has lower CM than ZC sequence when the PRB number is 10~20, while the performance gets worse for other numbers of PRBs.

Observation 5: The CM of ZC sequence is always kept under 2.5dB with different PRB number.

Observation 6: The detection performance is proportional to the sequence length while independent of sequence types.

Observation 7: The coverage becomes better with sequence length increasing while independent of sequence types.

Observation 8: Aligned and misaligned channelization show similar performance in coexistence scenario.

Proposal 1: It is not necessary to align NR channelization with IEEE 802.11ad channelization from coexistence perspective.

.

Reference

- RP-202925, WID on extending current NR operation to 71 GHz

- 38.213 g30, NR; Physical layer procedures for control(Release 16)

- 38.331 g20, NR; Radio Resource Control (RRC) protocol specification(Release 16)

- R1-2005922 On Phase Noise Compensation for OFDM, Ericsson.

- R1-2007652 Discussions on required changes to NR using existing NR waveform, vivo

Appendix

  - A1. Link level simulation assumptions

Table A1-1 Link level simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Waveform | CP-OFDM |
| System Bandwidth | 400MHz |
| Subcarrier spacing | 120kHz/480kHz/960kHz |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols per slot at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| PN compensation | CPE & ICI |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | 2 |
| UE antenna Array configuration | 2 |

  - A2. System level simulation assumptions

Table A2-1 SLS simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Channel Bandwidth | 2GHz/800M |
| Subcarrier spacing | 960kHz |
| Scenario | Indoor A |
| LBT schemes | NoLBT |
| Channel Model | The channel model for UE-to-UE links：InH open office: InH – office channel model with LOS probability for indoor - mixed office from TR38.901 The channel model for gNB-to-UE and gNB-gNB links：InH open office: InH – office channel model with LOS probability for indoor - open office from TR38.901 |
| Max. allowed BS Tx power | 40 dBm EIRP |
| Max. allowed UE Tx Power | 25 dBm EIRP |
| BS Antenna gain | 5dBi |
| UE Antenna gain | 5dBi |
| BS Noise Figure | 7dB |
| UE Receiver Noise Figure | 10dB |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), dH = dV = 0.5 λ |
| Traffic model | FTP Model 3 (27Mbyte file) |

A3. System level simulation results

Table A3-1  performance of aligned cases with low traffic load

| Simulator Case |  | 2G & 2G |  | 2G & 4*500M |  |
| --- | --- | --- | --- | --- | --- |
| Operator Metrics |  | Operator A | Operator B | Operator A | Operator B |
| DL UPT (Mbps) | 5%ile | 3792.6211 | 2644.4617 | 4167.1111 | 1207.5562 |
|  | 50%ile | 13847.2783 | 12334.5322 | 14256.0244 | 6883.7781 |
|  | 95%ile | 20711.5645 | 19690.7852 | 21600.0000 | 13213.9731 |
|  | mean | 13063.9492 | 12585.8379 | 12919.0938 | 6474.5538 |
| DL delay (s) | 5%ile | 0.010 | 0.010 | 0.010 | 0.036 |
|  | 50%ile | 0.017 | 0.017 | 0.017 | 0.077 |
|  | 95%ile | 0.072 | 0.086 | 0.069 | 0.821 |
|  | mean | 0.025 | 0.029 | 0.025 | 0.179 |
| Arrival rate (files/s) |  | 2 |  |  |  |
| BO |  | 23.295 | 28.051 | 19.703 | 71.944 |

Table A3-2  performance of aligned cases with medium traffic load

| Simulator Case |  | 2G & 2G |  | 2G & 4*500M |  |
| --- | --- | --- | --- | --- | --- |
| Operator Metrics |  | Operator A | Operator B | Operator A | Operator B |
| DL UPT (Mbps) | 5%ile | 659.2416 | 493.5231 | 1517.9063 | 730.4522 |
|  | 50%ile | 8828.8125 | 9626.2949 | 9725.1035 | 3426.3316 |
|  | 95%ile | 19264.3535 | 18512.9844 | 19322.4707 | 6470.0743 |
|  | mean | 10142.6377 | 10208.5811 | 10950.2646 | 3213.3989 |
| DL delay (s) | 5%ile | 0.010 | 0.010 | 0.010 | 0.038 |
|  | 50%ile | 0.023 | 0.023 | 0.020 | 0.113 |
|  | 95%ile | 0.400 | 0.386 | 0.203 | 0.456 |
|  | mean | 0.145 | 0.110 | 0.087 | 0.224 |
| Arrival rate (files/s) |  | 4 |  |  |  |
| BO |  | 54.048 | 55.161 | 42.425 | 91.496 |

Table A3-3  performance of misaligned case with low traffic load

| Simulator Case |  | 2G & 2G |  | 2G & 3*800M |  |
| --- | --- | --- | --- | --- | --- |
| Operator Metrics |  | Operator A | Operator B | Operator A | Operator B |
| DL UPT (Mbps) | 5%ile | 3792.6211 | 2644.4617 | 3734.2539 | 1221.7268 |
|  | 50%ile | 13847.2783 | 12334.5322 | 13770.3828 | 5215.4150 |
|  | 95%ile | 20711.5645 | 19690.7852 | 20861.8145 | 12570.0743 |
|  | mean | 13063.9492 | 12585.8379 | 13088.0127 | 8462.4682 |
| DL delay (s) | 5%ile | 0.010 | 0.010 | 0.010 | 0.017 |
|  | 50%ile | 0.017 | 0.017 | 0.017 | 0.035 |
|  | 95%ile | 0.072 | 0.086 | 0.068 | 0.259 |
|  | mean | 0.025 | 0.029 | 0.025 | 0.078 |
| Arrival rate (files/s) |  | 2 |  |  |  |
| BO |  | 23.295 | 28.051 | 19.992 | 51.748 |

Table A3-4  performance of misaligned case with medium traffic load

| Simulator Case |  | 2G & 2G |  | 2G & 3*800M |  |
| --- | --- | --- | --- | --- | --- |
| Operator Metrics |  | Operator A | Operator B | Operator A | Operator B |
| DL UPT (Mbps) | 5%ile | 659.2416 | 493.5231 | 1856.9265 | 512.9541 |
|  | 50%ile | 8828.8125 | 9626.2949 | 11077.0381 | 4861.8345 |
|  | 95%ile | 19264.3535 | 18512.9844 | 19340.3203 | 8215.4150 |
|  | mean | 10142.6377 | 10208.5811 | 11114.6162 | 4867.4146 |
| DL delay (s) | 5%ile | 0.010 | 0.010 | 0.010 | 0.023 |
|  | 50%ile | 0.023 | 0.023 | 0.023 | 0.045 |
|  | 95%ile | 0.400 | 0.386 | 0.105 | 0.459 |
|  | mean | 0.145 | 0.110 | 0.078 | 0.248 |
| Arrival rate (files/s) |  | 4 |  |  |  |
| BO |  | 54.048 | 55.161 | 44.776 | 81.097 |