3GPP TSG RAN WG1 #104-e			R1-2101832

e-Meeting, January 25th – February 5th, 2021

Agenda item:	7.2.11

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on [104-e-NR-UEFeature-NRU-01]

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion.

NR-U

[104-e-NR-UEFeature-NRU-01] Email discussion/approval on UE features for NR-U (25th Jan – 29th Jan) – Hiroki (DCM)

- Whether or not to ask RAN2 to update the optionality of FG10-2f

- Whether or not to clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-7/1-8/1-9 are supported by the UE in unlicensed bands

- Whether or not to clarify that Rel-15 FGs 1-4 applies to licensed band operation only

- How to reply to RAN4 on RAN1’s understanding on RB set of NR-U

This email discussion/approval covers following potential reply LSs.

LSs:

[104-e-NR-UEFeature-LS-01] Email discussion/approval of the reply LS to R1-2100003, to be handled under 7.2.11 (name TBD, CMCC)

[104-e-NR-UEFeature-LS-02] Email discussion regarding LS in R1-2100019, including a possible reply LS if necessary, to be handled under 7.2.11 (name TBD, Qualcomm)

- Discussion on Rel-16 NR UE features for NR-U

  - Optionality of FG10-2f

Following proposals are made in contributions.

| [4] | RAN2#112-e has further discussed this and agreed that this capability should be optional only for the UEs which do not support non-CA deployment scenarios, corresponding to scenarios B, C, D, or E in Annex B.3 of TS 38.300. The reason for this decision was that, to configure ra-ResponseWindow larger than 10ms, the network needs to know this capability before initial channel access. RAN2 assumes that this is in line with the RAN1 justification for making the capability optional. RAN2 has sent an LS to RAN1 [2] regarding the optionality of the UE capability on extended RAR window (FG 10-2f) informing RAN1 of the following: RAN1 discussed the RAN2 decision conveyed in LS R1-2005204 (R2-2005865) not to define a capability bit for FG10-2f. It is RAN1’s understanding that FG10-2f should be optional because some UEs may not require this capability, e.g. UEs supporting only CA/LAA scenario (scenario A in the NR-U WID). Therefore, RAN1 would like to ask RAN2 to introduce a capability bit for FG10-2f. The RAN2 decision to make 10-2f mandatory for Scenarios B, C, D, or E was based on the RAN1 reply LS [3] in which only one example was provided in which the capability should be optional, namely that it does not make sense that UEs supporting only CA/LAA scenarios (Scenario A) should be required to implement extended RAR window (FG 10-2f) since the RAR is sent on the PCell in licensed spectrum, where LBT is not used. We observe that in the RAN1 reply LS, the CA/LAA scenario was identified only as an example. It also does not make sense for UEs supporting only FBE (ChannelAccessMode-r16 ='semistatic') in any of the deployment scenarios to also support extended RAR window. This is because the likelihood of LBT failure preventing the gNB from sending the RAR within 10 ms is extremely low in a controlled environment, thus making extension of the RAR window not useful in such deployments. Based on this, we propose that RAN2 updates TS 38.306 to specify that for UEs only supporting FBE in any of Scenarios A, B, C, D, or E the capability parameter extRA-ResponseWindow-r16 is optional. - The capability parameter extRA-ResponseWindow-r16 in TS 38.306 (corresponding to FG 10-2f) should be optional for UEs supporting FBE only (ChannelAccessMode-r16 ='semistatic'), i.e., UEs that do not also support LBE (ChannelAccessMode-r16 ='dynamic'). For UEs supporting only FBE, the capability should be optional for all of Scenarios A, B, C, D, and E listed in Annex B.3 of TS 38.300. - Send an LS reply to RAN2 informing RAN2 of the recommendation in Proposal 1 |
| --- | --- |
| [16] | In [1], RAN2 informed RAN1 of the agreement that the capability extRA-ResponseWindow-r16 (i.e., FG 10-2f in [2]) is mandatory for the UE supporting any of scenarios B, C, D, and E in TS 38.300 Annex B.3, and also informed of TS 38.306 CR capturing that agreement. However, RAN2 agreement is not aligned with RAN1 agreement as follows that was captured in [2] and was made in RAN1#103-e before RAN2 agreement. Agreements: - Following classification of scenarios is used to define basic FGs for NR-U according to TS38.300 B.3 - Scenario A: Carrier aggregation between NR in licensed spectrum (PCell) and NR in shared spectrum (SCell); - Scenario A.1: SCell is not configured with uplink (DL only). - Scenario A.2: SCell is configured with uplink (DL+UL). - Scenario B: Dual connectivity between LTE in licensed spectrum and NR in shared spectrum (PSCell); - Scenario C: NR in shared spectrum (PCell); - Scenario D: NR cell in shared spectrum and uplink in licensed spectrum; - Scenario E: Dual connectivity between NR in licensed spectrum (PCell) and NR in shared spectrum (PSCell). - Ask RAN2 to consider following TP for TS38.300 B.3 TS38.300, Annex B (informative): Deployment Scenarios B.3 NR Operation with Shared Spectrum NR Radio Access operating with shared spectrum channel access can support the following deployment scenarios: - Scenario A: Carrier aggregation between NR in licensed spectrum (PCell) and NR in shared spectrum (SCell); - Scenario A.1: SCell is not configured with uplink (DL only). - Scenario A.2: SCell is configured with uplink (DL+UL). - Scenario B: Dual connectivity between LTE in licensed spectrum and NR in shared spectrum (PSCell); - Scenario C: NR in shared spectrum (PCell); - Scenario D: NR cell in shared spectrum and uplink in licensed spectrum; - Scenario E: Dual connectivity between NR in licensed spectrum (PCell) and NR in shared spectrum (PSCell). Carrier aggregation of cells in shared spectrum is applicable to all deployment scenarios.. - Following FGs are defined as basic FGs for corresponding scenario(s) for NR-U, and associated scenario(s) for the basic FG is clarified in the note column of UE features list - 10-1: A2, B, C, D and E with dynamic channel access mode - 10-1a: A2, B, C, D and E with semi-static channel access mode - 10-2: A1, A2, B, C, D and E with dynamic channel access mode - 10-2a: A1, A2, B, C, D and E with semi-static channel access mode - 10-2b: B, C, D and E - 10-2c: B, C, D and E with dynamic channel access mode - 10-2d: B, C, D and E with semi-static channel access mode - 10-2e: C and D - The note "This FG may be part of basic operation for a particular scenario" is removed from following FGs. - 10-2f, 10-3, 10-3a, 10-27, 10-29, 10-30, 10-31 - Note: There will be no more discussion on whether/how to capture the classification of scenarios in TR/TS. As highlighted above, FG 10-2f is clearly optional for all scenarios (different from FG 10-1/1a/2/2a/2b/2c/2d/2e for which UE should support for some scenarios). In addition, it seems that RAN2 misunderstood RAN1’s intention of the following statements in [3] that was sent to RAN2 in RAN1#102-e. - Regarding FG10-2f in UE features list for NR-U To RAN2: RAN1 discussed the RAN2 decision conveyed in LS R1-2005204(R2-2005865) not to define a capability bit for FG10-2f. It is RAN1’s understanding that FG10-2f should be optional because some UEs may not require this capability, e.g. UEs supporting only CA/LAA scenario (scenario A in the NR-U WID). Therefore, RAN1 would like to ask RAN2 to introduce a capability bit for FG10-2f.. The highlighted part is just one of examples for RAN1 to request RAN2 to make FG 10-2f optional, and RAN1 was discussing another aspects (but not captured in the LS [3]) on whether FG 10-2f needs to be also optional for stand-alone scenarios (e.g., scenario C in TS 38.300 Annex B.3) or not. Furthermore, the rationale behind RAN1 agreement that FG 10-2f is optional for all scenarios was that the UE feature related to RAR window extension is not critical for UE to support NR-U. In other words, if gNB configures RAR window larger than 10 ms by SIB1 but a UE does not support FG 10-2f, the UE cannot access the cell and shall search another cell for which gNB configures RAR window no larger than 10 ms, which is also applicable to FG 10-3/3a (interlaced PUSCH/PUCCH) that is optional for all scenarios even though it can be configured by SIB1. Therefore, we recommend RAN2 to reflect RAN1 agreement in TS 38.306, and suggest the following proposal. Proposal: Send a reply LS to RAN2 to inform RAN1’s agreement in RAN1#103-e that the capability extRA-ResponseWindow-r16 (i.e., FG 10-2f) is optional for all scenarios and recommend the following TS 38.306 CR. Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. extRA-ResponseWindow-r16 Indicates whether the UE supports the configuration of maximum length of RAR window with a value larger than 10ms and up to 40ms by decoding of the 2 LSBs of SFN in the DCI format 1_0 for 4-step RA type.; Band; No; N/A; N/A. |

Based on the above proposals, following point can be discussed in RAN1#104-e meeting.

Discussion point #1

- Whether or not to ask RAN2 to update the optionality of FG10-2f for UE supporting only FBE in any of scenarios

In AI 5, following conclusion was made in the preparation phase of RAN1#104-e meeting with following companies’ views.

Conclusion:

- Email discussion regarding LS in R1-2100019, including a possible reply LS if necessary, to be handled under 7.2.11 (name TBD, Qualcomm)

| Company | Views |
| --- | --- |
| Samsung | Agree with the initial assessment. |
| Intel | We don’t think it is necessary to be discussed. We just need to note RAN2 agreement and RAN2’s CR. |
| vivo | Agree with Chairman’s initial assessment |
| NTT DOCOMO | Agree with Chairman’s initial assessment, and this issue is captured as candidate discussion topic for NRU UE features in 7.2.11 summary. Same as here, there are different views among companies in NRU UE feature summary in 7.2.11 whether RAN1 need to discuss this issue or just need to note the RAN2 LS. |
| ZTE | Agree with the intitial assessment |
| Nokia | Agree with Intel, Should suffice that RAN1 just notes the RAN2 LS, but this can be of course confirmed in the NR-U UE features if there is a desire to do so. |
| MediaTek | Agree with the initial assessment. |
| LG Electronics | Agree with the initial assessment. |
| Huawei, HiSilicon | Agree with the initial assessment. |
| FUTUREWEI | Agree with Intel and Nokia |

Companies’ views in the contributions and above table can be summarized as below.

- Just note RAN2 LS and confirm RAN2 agreement that FG10-2f is optional only for the UEs which do not support non-CA deployment scenarios, corresponding to scenarios B, C, D, or E in Annex B.3 of TS 38.300

  - Support: Intel, Nokia, FUTUREWEI

- Send LS reply to RAN2 to update the optionality of FG10-2f that FG10-2f is optional for all scenarios (as already agreed in RAN1)

  - Support: LG

- Send LS reply to RAN2 to update the optionality of FG10-2f that FG10-2f is optional for UE supporting only FBE in any of scenarios

  - Support: Ericsson

Based on above, following FL proposal can be made.

FL proposal 1:

- Adopt one of following alternatives

  - Alt.1: Note RAN2 LS and confirm RAN2 agreement that FG10-2f is optional only for the UEs which do not support non-CA deployment scenarios, corresponding to scenarios B, C, D, or E in Annex B.3 of TS 38.300

  - Alt.2: Send LS reply to RAN2 to update the optionality of FG10-2f that FG10-2f is optional for all scenarios

  - Alt.3: Send LS reply to RAN2 to update the optionality of FG10-2f that FG10-2f is optional for UE supporting only FBE in any of scenarios

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| DOCOMO | We support Alt.1. While RAN1 didn’t see the necessity of mandatory support of FG10-2f in any deployment scenarios, it is also true that “to configure ra-ResponseWindow larger than 10ms, the network needs to know this capability before initial channel access” mentioned in RAN2 LS. As RAN2 is proper WG to handle RA procedure and to avoid ping-pong among WGs, we prefer to follow RAN2 agreement. |
| Nokia, NSB | We support Alt.1. We also agree with DOCOMO that RAN2 is the proper group to handle this issue, and they have made their decision already, and there is no clear RAN1 reason to motivate a request for them to revert it. |
| LG Electronics | We support Alt.2 which is in line with previous RAN1 agreement. The RAN2 LS states that “The reason for this decision was that, to configure ra-ResponseWindow larger than 10ms, the network needs to know this capability before initial channel access. RAN2 assumes that this is in line with the RAN1 justification for making the capability optional.” When RAN1 discussed this FG 10-2f in previoius meetings, we also recognized that the length of RAR window is configured by SIB1 (i.e., before the network knows whether th UE is capable of FG 10-2f or not). However, RAN1 concluded that FG 10-2f is optional for all scenarios since this FG is not an essential feature for UE to support NR-U. The same rationale is applied to UE capabilities corresponding to interlaced PUCCH/PUSCH. Therefore, SIB1 signaling cannot justify the corresponding FG to be mandatory and RAN2’s understanding/assumption is not in line with RAN1’s principle for making FG 10-2f optional. In other words, if gNB configures RAR window larger than 10 ms by SIB1 but a UE does not support FG 10-2f, the UE cannot access the cell and shall search another cell for which gNB configures RAR window no larger than 10 ms, which is also applicable to FG 10-3/3a (interlaced PUSCH/PUCCH) that is optional for all scenarios even though it can be configured by SIB1. |
| vivo | We support Alt.1. Agree with Docomo and Nokia that RAN2 should handle this since extended RAR window is a developed feature mainly in RAN2. RAN1 just follow the agreement made in RAN2. |
| ZTE | We support Alt.1. Agree with DOCOMO, Nokia and vivo that RAN2 should be the proper group to handle this issue. |
| Ericsson | We support Alt-3. We note that RAN2’s decision on making 10-2f mandatory for UEs supporting Scenarios B,C,D,E was based on incomplete information from RAN1. When RAN1 sent the LS to RAN2, scenario A (LAA CA operation) was given as an example of a scenario where 10-2f should be optional, since extended RAR window is not needed in such a scenario. At the time the LS was sent, RAN1 had not concluded the discussion on basic feature groups considering FBE/LBE variants of Scenarios A,B,C,D,E. Furthermore, the reply LS from RAN2 stated "RAN2 assumes that this is in line with the RAN1 justification for making the capability optional," which is correct regarding Scenario A, but RAN1 did not provide information on FBE vs. LBE. Clearly, for a UE that supports only FBE (i.e., does not support LBE), then extended RAR window is not needed (or even useful). Based on this, we believe RAN1 should inform RAN2 that there are two UE types for which FG 10-2f should be optional: (1) UEs supporting only Scenario A and (2) UEs supporting only FBE (in any Scenario A,B,C,D,E). In our view extended RAR window is not needed in either of these situations, so it does not make sense to force a UE vendor implementing an FBE-only device to implement this functionality. We also agree with some of the points raised by LGE. LGE is correct in pointing out that interlaced PUCCH/PUSCH is another example of a feature for which a capability is defined and for which the feature is enabled/disabled by SIB signalling. As LGE points out, if a UE is not capable of such a feature, it will not be able to access the cell. This is one situation. However, there is another important use for UE capability signalling that needs to be highlighted. A network vendor can collect statistics on various UE capabilities prior to deploying a feature. Once a significant population of UEs support the feature, the feature can be enabled in the network. Since 10-2f is not essential for operation in unlicensed spectrum, there is no reason to mandate it on the UE side from Day 1. If a certain deployment can benefit from extended RAR, then the feature can be enabled at a later point once a significant fraction of UEs in the network indicate capability. Based on this reasoning, we would also be okay with Alt-2. |
| Qualcomm | We support Alt-1. Consider the feature is mostly addressed in RAN2, since they already made an agreement, we can respect their decision. |
| Moderator (NTT DOCOMO) | Thanks for feedbacks. Based on the feedbacks so far, following is the situation. - Alt.1: Note RAN2 LS and confirm RAN2 agreement that FG10-2f is optional only for the UEs which do not support non-CA deployment scenarios, corresponding to scenarios B, C, D, or E in Annex B.3 of TS 38.300 - DOCOMO, Nokia/NSB, vivo, ZTE, Qualcomm - Alt.2: Send LS reply to RAN2 to update the optionality of FG10-2f that FG10-2f is optional for all scenarios - LG, Ericsson - Alt.3: Send LS reply to RAN2 to update the optionality of FG10-2f that FG10-2f is optional for UE supporting only FBE in any of scenarios - Ericsson Although Alt.1 is supported by clearly larger number of companies, companies supporting Alt.2/3 seem to have a strong opinion. If Alt.1 is not acceptable for some companies, one possible way is to share this situation in RAN1 with RAN2 and ask RAN2 to decide whether/how to update the optionality of FG10-2f. But since RAN2 already discussed and made the agreements, it may not be so meaningful at this stage. So, my suggestion as the moderator is to go to Alt.1 unless RAN2 agreement on the optionality of FG10-2f causes an essential issue. |

Updated FL proposal 1:

- Adopt one of following alternatives

  - Alt.1: Note RAN2 LS and confirm RAN2 agreement that FG10-2f is optional only for the UEs which do not support non-CA deployment scenarios, corresponding to scenarios B, C, D, or E in Annex B.3 of TS 38.300

  - Alt.2’: Send LS reply to RAN2 to share the following RAN1 situation so that RAN2 can decide whether/how to update the optionality of FG10-2f

    - In past RAN1 meeting, RAN1 agreed that FG10-2f is optional for all scenarios since this FG is not considered as an essential feature for UE to support NR-U.

    - During the discussion in RAN1#104-e, there are companies prefer to update the optionality of FG10-2f according to the above previous RAN1 agreement or to make FG10-2f optional also for UE supporting only FBE in any scenario, while other majority companies prefer to confirm RAN2 agreement that FG10-2f is optional only for the UEs which do not support non-CA deployment scenarios.

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| Ericsson | We still have a concern on Alt-1, and we think Alt-3 should be adopted. We would like to emphasize that the scenario we have in mind is UEs that support FBE only (i.e., UEs that do not support LBE). Why should extended RAR be mandatory given that it would not be used in an FBE deployment? We think that the issue could benefit from some on-line discussion. |
| LG Electronics | We share the view from Ericsson in that online discussion seems necessary. If we go with Alt.2’, it should be pointed out that SIB1 signaling cannot justify the corresponding FG to be mandatory and RAN2’s understanding/assumption is not in line with RAN1’s principle for making FG 10-2f optional. |
| Huawei, HiSilicon | We support Alt 1. It can avoid the situation in initial access that gNB transmits RAR in extended RAR window and UE does not have chance to feedback its incapability. In FBE, it is still possible that LBT fails at beginning of a FFP. The extended RAR window allows gNB to transmit RAR in the next several FFP considering maximum extended RAR window can be 40ms. As for Alt 2’, we are not sure what the LS can help. The feature of extended RAR window is RAN2 feature and RAN2 already made agreement on it. |
| CATT | We prefer Alt.1. Because RAN2 makes the agreement and inform RAN1, RAN2 is the proper group to handle this issue if there is any issue on FG10-2f. RAN1 isn’t necessary to change RAN2’s agreement on FG10-2f. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks and discussion in GTW session. Based on the discussion, it seems difficult to do anything further for this issue in RAN1. But sharing your views and further consideration on discussed aspects e.g., points mentioned by Ericsson and LG, will be appreciated. |

  - Clarification on NR-U UE features with Rel-15 FG 1-4/5/5a/6/7/8/9

Following proposals are made in contributions.

| [11] | Proposal 5: Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-7/1-8/1-9 are supported by the UE in unlicensed bands. 10. NR-unlicensed; 10-26a; CSI-RS based RRM for operation with shared spectrum channel access; CSI-RS based RRM for operation with shared spectrum channel access; Yes; N/A; Per band; N/A; N/A; N/A; the signaling is per band but is only expected for a band where shared spectrum channel access must be used; Optional with capability signalling. 1-5; CSI-RS based RRM measurement with associated SS-block; 1) CSI-RSRP measurement 2) CSI-RSRQ measurement; 1-1, CSI-RS; Yes; Not support CSI-RSRP and CSI-RSRQ measurement; Type 4; No need; Yes; RAN1; Optional with capability signaling Note: This does not discourage RAN4 to complete their work Note: there is expectation that RAN4 will complete the corresponding RRM measurement. 1-5a; CSI-RS based RRM measurement without associated SS-block; 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed; 1-1, CSI-RS; Yes; Type 4; No need; Yes; RAN1; Optional with capability signaling Note: This does not discourage RAN4 to complete their work Note: there is expectation that RAN4 will complete the corresponding RRM measurement. 1-6; CSI-RS based RS-SINR measurement; 1) CSI-SINR measurement; 1-1 1-5; Yes; Not support CSI-SINR measurement; Type 4; No need; Yes; RAN1; Optional with capability signaling; Optional with capability signaling. 1-7; CSI-RS based RLM; 1) CSI-RS based RLM; 1-1, CSI-RS; Yes; Not support CSI-RS based RLM; Type 4; No need; Yes; RAN1; Mandatory with capability signaling. 1-8; RLM based on a mix of SS block and CSI-RS signals within active BWP; 1-4 and 1-7; Yes; UE does not support RLM based on a mix of SS block and CSI-RS signals; Type 4; No need; No need; RAN1; [Mandatory /optional with capability signaling]; Optional with capability signaling. 1-9; CSI-RS based contention free RA for HO; 1-1 CSI-RS 1-5 or 1-5a; Yes; UE does not support CSI-RS based contention free RA for HO; Type 4; No need; No need; RAN1; Optional with capability signaling; Optional with capability signaling. Similar to the above, we suggest clarifying the relationship between Rel-16 FG 10-2c/10-2d “SSB-based RLM” and Rel-15 FG 1-4 “SSB-based RLM”, each of which is copied below. Proposal 6: Clarify that Rel-15 FG 1-4 applies to licensed band operation only. 10. NR-unlicensed; 10-2c; SSB-based RLM for dynamic channel access mode; 1. SSB-based RLM with Q for dynamic channel access mode; Yes; N/A; Per band; N/A; N/A; N/A; Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2d; SSB-based RLM for semi-static channel access mode; 1. SSB-based RLM with Q for semi-static channel access mode, when DRS window is no longer than the fixed frame period; Yes; N/A; Per band; N/A; N/A; N/A; Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 1-4; SS block based RLM; 1) SS block based RLM; 1-1; Yes; Not support SS block based RLM; Type 4; No need; No need; RAN1; Mandatory with capability signaling which shall be set to ‘1’. |
| --- | --- |

Based on the above proposals, following point can be discussed in RAN1#104-e meeting.

Discussion point #2

- Whether or not to clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-7/1-8/1-9 are supported by the UE in unlicensed bands

- Whether or not to clarify that Rel-15 FGs 1-4 applies to licensed band operation only

Since there are only one company proposals, those proposals can be discussed.

FL proposal 2:

- Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-7/1-8/1-9 are supported by the UE in unlicensed bands

- Clarify that Rel-15 FG 1-4 applies to licensed band operation only

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| DOCOMO | For the 1st bullet, we agree with the proposal in principle, but FG10-26: CSI-RS based RLM for operation with shared spectrum channel access is proper FG corresponding to FG 1-7/1-8 For the 2nd bullet, we agree with the proposal. |
| Nokia, NSB | The proposal is first bullet is not clear, as not all FGs in the list are directly related to RRM. Second bullet is OK, as there are NR-U-specific FGs defined already, which are basic FGs for the required scenarios. |
| LG Electronics | As we have separate FG that corresponds to CSI-RS based RLM as follows, FG 1-7/1-8 needs to be excluded in the first bullet. 10. NR-unlicensed; 10-26; CSI-RS based RLM for operation with shared spectrum channel access; CSI-RS based RLM for operation with shared spectrum channel access; Yes; N/A; Per band; N/A; N/A; N/A; the signaling is per band but is only expected for a band where shared spectrum channel access must be used; Optional with capability signalling. For FG 1-7, it seems that the handling similar to FG 1-4 is necessary, i.e., Rel-15 FG 1-7 applies to licensed band operation only. This is because Rel-15 FG 1-7 is a manadatory feature. For FG 1-8, it seems that the handling similar to FG Rel-15 FG 1-5/1-5a/1-6/1-9 is necessary, i.e., a UE indicating no support of FG 10-26, also indicates Rel-15 FG 1-8 is not supported by the UE in unlicensed bands Therefore, we suggest the followings. - Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-7/1-8/1-9 are supported by the UE in unlicensed bands - Clarify that Rel-15 FG 1-4 applies to licensed band operation only - Clarify that a UE indicating no support of FG 10-26, also indicates that Rel-15 FG 1-8 is not supported by the UE in unlicensed bands - Clarify that Rel-15 FG 1-7 applies to licensed band operation only |
| vivo | Agree with the proposal and LG’s modification |
| ZTE | Agree with LG’s modifications. |
| Qualcomm | Agree with LG’s modifications. |
| Ericsson | We are in principle fine with LG modifications and it is indeed an improvement. But I would like to check if it works considering the pre-requisite. Let’s take 3rd bullet: - If the UE doesn’t support 1-26 for unlic, 1-8 is also not supported for unlic. (OK). - If Ue support 1-26 for unlic, it may or may not support 1-8 for unlic. In case, it supports 1-8, don’t we have a conflict here since 1-4 and 1-7 are pre-requiste for 1-8 but they are only for lic as LG proposal states. - So, I think the solution is not complete. Or maybe you can clarify how it works. |
| Moderator (NTT DOCOMO) | Thanks for feedbacks. Based on the feedbacks so far, the majority companies are fine with LG’s modified version of the proposal. Regarding the question from Ericsson, I think that if the UE supports 1-8, it would also support FG10-2c/2d and 10-26 for unlicensed similar to FG1-4 and 1-7 for licensed. Some companies can clarify that point. |

Updated FL proposal 2:

- Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-9 are supported by the UE in unlicensed bands

- Clarify that Rel-15 FG 1-4 applies to licensed band operation only

- Clarify that a UE indicating no support of FG 10-26, also indicates that Rel-15 FG 1-8 is not supported by the UE in unlicensed bands

- Clarify that Rel-15 FG 1-7 applies to licensed band operation only

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| LG Elctronics | To address the issue brought up by Ericsson, another approach could be to make separate feature groups for FG 1-5/1-5a/1-6/1-8/1-9 in unlicensed band for licensed/unlicensed differentiation, similar to discussion in Others agenda. Then, the alternative proposal can be as follows: - Clarify that Rel-15 FG 1-4 applies to licensed band operation only - Clarify that Rel-15 FG 1-7 applies to licensed band operation only - Add new FGs to indicate the support of each of FG 1-5/1-5a/1-6/1-8/1-9 in unlicensed band - The pre-requisite for FG 1-5/1-5a/1-6/1-9 in unlicensed band includes FG 10-26a - The pre-requisite for FG 1-8 in unlicensed band includes FG 10-26. |
| Huawei, HiSilicon | Agree with FL’s updated proposal. In order to solve the Ericsson’s issue, 10-2c/10-2d and 10-26 can be added as pre-requisite for FG1-8 for unlicensed band which corresponding to 1-4 and 1-7 for licensed band. We do not think it necessary to introduce new FG which duplicate the FG in licensed band. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks and discussion in GTW session. Based on the discussion, following updated FL proposal 2 can be checked by companies. |

Updated FL proposal 2:

- Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-9 are supported by the UE in unlicensed bands

- Clarify that Rel-15 FG 1-4 applies to licensed band operation only

- Clarify that a UE indicating no support of FG 10-26, also indicates that Rel-15 FG 1-8 is not supported by the UE in unlicensed bands

- Clarify that Rel-15 FG 1-7 applies to licensed band operation only

- Clarify that if the UE supporting unlicensed operation supports 1-8, it would also support FG10-2c/2d and 10-26 for unlicensed similar to FG1-4 and 1-7 for licensed

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| DOCOMO | We agree with the proposal with following revision. - Correct numbering of above “FG1-4/1-5/1-5a” to “FG1-3/1-4/1-5” according to TR38.822. - Modify the last bullet as “…and 10-26 for unlicensed bands similar to FG1-4 and 1-7 for licensed bands” |
| LG Electronics | Agree with DOCOMO’s modification. One question on the next step after this proposal is agreed: In which specification those clarification will be captured? |
| Nokia, NSB | We are not OK with all the proposals, details below, using FG numbers from 38.822: - FG 1-3 is about RLM and SS-block, while 10-26a is about RRM and CSI-RS, so better not to mix those. FGs 1-4 and 1-5 are optional, and there are FGs specifically for RRM in unlicensed bands. The cleanest solution here would be to clarify in 306 that FGs 1-4 and 1-5 do not apply to unlicensed bands, without making conditions on the reporting of 10-26a. - It is OK to clarify that FG1-3 applies to licensed bands only, as there are corresponding FGs for unlicensed already. - FG 1-8: The discussion here should be on the applicability of 1-8 to unlicensed in general. Assuming 1-3 and 1-7 are not applied to unlicensed, then it follows naturally that 1-8 should not apply either. However, in that case it seems we are missing an FG that replicates FG 1-8 functionality with the proper FGs for unlicensed as pre-requisites. - It is OK to clarify that FG1-7 applies to licensed bands only, as there are corresponding FGs for unlicensed already. - 10-2c/2d are basic for the corresponding scenarios already, hence no further clarification is needed. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks. Based on the feedbacks we can continue discussing on following two alternatives. Alt.1: - Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-4/1-5/1-6/1-9 are supported by the UE in unlicensed bands - Clarify that Rel-15 FG 1-3/1-7 applies to licensed band operation only - Clarify that a UE indicating no support of FG 10-26, also indicates that Rel-15 FG 1-8 is not supported by the UE in unlicensed bands - Clarify that if the UE supporting unlicensed operation supports 1-8, it would also support FG10-2c/2d and 10-26 for unlicensed bands similar to FG1-4 and 1-7 for licensed bands Alt.2: - Clarify that Rel-15 FG 1-3/1-4/1-5 applies to licensed band operation only - Clarify that Rel-15 FG 1-3/1-7 applies to licensed band operation only - Clarify that Rel-15 FG 1-8 applies to licensed band operation only - Add new Rel-16 FG based on 1-8 for unlicensed band operation - Prerequisite FGs of this new FG are 10-26 and one of {10-2c, 10-2d} |

Updated FL proposal 2:

Alt.1:

- Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-4/1-5/1-6/1-9 are supported by the UE in unlicensed bands

- Clarify that Rel-15 FG 1-3/1-7 applies to licensed band operation only

- Clarify that a UE indicating no support of FG 10-26, also indicates that Rel-15 FG 1-8 is not supported by the UE in unlicensed bands

- Clarify that if the UE supporting unlicensed operation supports 1-8, it would also support FG10-2c/2d and 10-26 for unlicensed bands similar to FG1-4 and 1-7 for licensed bands

Alt.2:

- Clarify that Rel-15 FG 1-3/1-4/1-5 applies to licensed band operation only

- Clarify that Rel-15 FG 1-3/1-7 applies to licensed band operation only

- Clarify that Rel-15 FG 1-8 applies to licensed band operation only

- Add new Rel-16 FG based on 1-8 for unlicensed band operation

  - Prerequisite FGs of this new FG are 10-26 and one of {10-2c, 10-2d}

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| DOCOMO | Regarding Alt.2, - 1st and 2nd bullets can be marged as “Clarify that Rel-15 FGs 1-3/1-4/1-5/1-7 apply to licensed band operation only” - We think the same clarification as Alt.1 is necessary for FG1-6/1-9, i.e., “Clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-6/1-9 are supported by the UE in unlicensed bands” With that, we are fine with either alternative, while slightly prefer Alt.1 as adding new FG in this late stage should be avoided as much as possible |
| LG Electronics | For Alt 2, - The first bullet should be for FG 1-4/1-5/1-6, instead of FG 1-3/1-4/1-5. - If FG 1-4/1-5/1-6 is applied to licensed band only, don’t we need new Rel-16 FGs similar to 1-8 in the last bullet? - For FG 1-9, do we need similar handling with FG 1-8? It would be good to hear Nokias’ view on above questions to make a complete solution. At least the common thing between two alternatives is for FG 1-3 and 1-7, so we could first agree on that. On the other hand, how to deal with FG 1-4/1-5/1-6/1-8/1-9 can be further discussed in this meeting or next meeting. |
| Nokia, NSB | Regarding LGE questions to us above, our views are as follows: - The assumption in our comments above is that 1-4/1-5/1-6 are covered by 10-26a, at least in principle. Indeed this is not a perfect association, as 10-26a simply says “CSI-RS based RRM for operation with shared spectrum channel access”. In lack of detailed, our understanding is that it covers all the possible aspects related to CSI-RS based RRM, but we are open for further discussion on how to make it more clear. That could be achieved either by expanding the description of 10-26a or adding FGs as suggested by LGE. - On FG 1-9, indeed it might require similar handling as with FG 1-8. The reason we did not include it in our previous comments is that in our understanding the functionality itsef has not been much debated in Rel-16, and hence it is not that obvious to us what would be the best way forward. But we are open to creating a Rel-16 version of the FG specific for unlicensed, if that is the common view among companies. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks. Based on the discussion in GTW session, following updated FL proposal 2 was agreed. |

Updated FL proposal 2:

- Clarify that Rel-15 FG 1-4/1-5/1-6 applies to licensed band operation only

  - Add new Rel-16 FG based on 1-4 for unlicensed band operation

  - Add new Rel-16 FG based on 1-5 for unlicensed band operation

  - Add new Rel-16 FG based on 1-6 for unlicensed band operation

  - Ask RAN2 to dummify FG10-26a

- Clarify that Rel-15 FG 1-3/1-7 applies to licensed band operation only

  - Functionalities of 1-3 is covered by FG10-2c/2d in unlicensed band operation

  - Functionalities of 1-7 is covered by FG10-26 in unlicensed band operation

- Clarify that Rel-15 FG 1-8 applies to licensed band operation only

  - Add new Rel-16 FG based on 1-8 for unlicensed band operation

    - Prerequisite FGs of this new FG are 10-26 and one of {10-2c, 10-2d}

- Clarify that Rel-15 FG 1-9 applies to licensed band operation only

  - Add new Rel-16 FG based on 1-9 for unlicensed band operation

    - Prerequisite FGs of this new FG are one of {new FGs based on 1-4, 1-5}

  - Reply LS on Rel-16 updated RAN4 UE features

Following proposals are made in contributions.

| [15] | 1. Overall Description: RAN1 thanks RAN4 for the LS, and would like to provide following feedback: For FG4-1 and FG4-2 of NR-U, RAN4’s understanding is a RB set corresponds to 20MHz bandwidth on which a channel access procedure is performed in shared spectrum. RAN4 kindly would like to ask RAN1 to check whether RAN4 understanding is correct. RAN1’s understanding is that a RB set approximately corresponds to 20MHz bandwidth on which a channel access procedure is performed, and the bandwidth corresponding to the RB set can be less than or slightly greater than 20 MHz based on configuration. Actions: To RAN WG4 ACTION: RAN1 respectfully requests RAN4 to take above response into account. |
| --- | --- |

Based on the above proposals, following point can be discussed in RAN1#104-e meeting.

Discussion point #3

- How to reply to RAN4 on RAN1’s understanding on RB set of NR-U

In AI 5, following conclusion was made in the preparation phase of RAN1#104-e meeting.

Conclusion:

- Email discussion/approval of the reply LS to R1-2100003, to be handled under 7.2.11 (name TBD, CMCC)

Since there is one draft LS, it can be starting point for the discussion.

FL proposal 3:

- Reply LS to RAN4 with following contents

  - RAN1’s understanding is that a RB set approximately corresponds to 20MHz bandwidth on which a channel access procedure is performed, and the bandwidth corresponding to the RB set can be less than or slightly greater than 20 MHz based on configuration.

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| DOCOMO | We agree with the proposal |
| Nokia, NSB | We agree with the proposal |
| LG Electronics | We support the proposal, as a proposing company. |
| Vivo | We agree with the proposal |
| ZTE | We agree with the proposal |
| Ericsson | We agree with the proposal |
| Qualcomm | We agree with the proposal |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks. Based on the feedbacks so far, all companies are fine with FL proposal 3, i.e., draft reply LS in R1-2100887. So, we can ask original proponent (LG) or LS rapporteur (CMCC) to prepare final version of the reply LS once the draft reply LS is agreed (hopefully in Tuesday UE feature session). |
| Huawei, HiSilicon | Sorry for the late feedback. We are also fine with the proposal. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks and discussion in GTW session. Based on the discussion, we can check whether we need to mention also on 10 MHz bandwidth case or not. |
| LG Electronics | From offline discussion with Huawei, it is confirmed that a RB set is not defined for 10 MHz bandwidth carrier. So, the current wording might be sufficient. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks and offline discussion. Based on the discussion in GTW session, following agreement was made. |

Agreements:

- Reply LS to RAN4 with following contents – final LS in R1-2101939 (Seonwook, LG)

  - RAN1’s understanding is that a RB set approximately corresponds to 20MHz bandwidth on which a channel access procedure is performed, and the bandwidth corresponding to the RB set can be less than or slightly greater than 20 MHz based on configuration.

- Conclusion

Agreements:

- Clarify that Rel-15 FG 1-4/1-5/1-6 applies to licensed band operation only

  - Add new Rel-16 FG based on 1-4 for unlicensed band operation

  - Add new Rel-16 FG based on 1-5 for unlicensed band operation

  - Add new Rel-16 FG based on 1-6 for unlicensed band operation

  - Ask RAN2 to dummify FG10-26a

- Clarify that Rel-15 FG 1-3/1-7 applies to licensed band operation only

  - Functionalities of 1-3 is covered by FG10-2c/2d in unlicensed band operation

  - Functionalities of 1-7 is covered by FG10-26 in unlicensed band operation

- Clarify that Rel-15 FG 1-8 applies to licensed band operation only

  - Add new Rel-16 FG based on 1-8 for unlicensed band operation

    - Prerequisite FGs of this new FG are 10-26 and one of {10-2c, 10-2d}

- Clarify that Rel-15 FG 1-9 applies to licensed band operation only

  - Add new Rel-16 FG based on 1-9 for unlicensed band operation

    - Prerequisite FGs of this new FG are one of {new FGs based on 1-4, 1-5}

Agreements:

- Reply LS to RAN4 with following contents – final LS in R1-2101939 (Seonwook, LG)

  - RAN1’s understanding is that a RB set approximately corresponds to 20MHz bandwidth on which a channel access procedure is performed, and the bandwidth corresponding to the RB set can be less than or slightly greater than 20 MHz based on configuration.

# Reference

[1]	R1-2009585	Updated RAN1 UE features list for Rel-16 NR	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2100094	Discussion on NR Rel-16 UE Features	ZTE

[3]	R1-2100140	Correction for V2X UE feature list	OPPO

[4]	R1-2100522	Remaining details of Rel-16 NR UE features	Ericsson

[5]	R1-2100554	Discussion on NR Rel-16 UE features	LG Electronics

[6]	R1-2100635	Remaining issue on UE features	Intel Corporation

[7]	R1-2101184	On NR Rel.16 UE features	Samsung

[8]	R1-2101249	Updates on NR UE Features	Nokia, Nokia Shanghai Bell

[9]	R1-2101273	Remaining details of Rel-16 NR UE features	Huawei, HiSilicon

[10]	R1-2101342	Discussions on NR Rel-16 UE features	Apple

[11]	R1-2101444	Discussion on NR Rel-16 UE features	Qualcomm Incorporated

[12]	R1-2101517	Correction on half-DuplexTDD-CA-SameSCS-r16	CATT

[13]	R1-2101587	Remaining issues on Rel-16 NR UE features	NTT DOCOMO, INC.

[14]	R1-2101685	Remaining issues on Rel-16 eMIMO UE features	vivo

[15]	R1-2100887	Draft reply LS on Rel-16 updated RAN4 UE features lists for LTE and NR	LG Electronics

[16]	R1-2100889	Discussion on RAN2 LS on capability for extended RAR window monitoring	LG Electronics

# Appendix: NR UE features list for NR-U in [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access and contention window size adjustment 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario A2, B, C, D and E with dynamic channel access mode |
| 10. NR-unlicensed | 10-1a | UL channel access for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. CP extension up to 1 symbol for PUSCH/PUCCH transmission |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario A2, B, C, D and E with semi-static channel access mode |
| 10. NR-unlicensed | 10-2 | SSB-based RRM for dynamic channel access mode | 1. SSB-based RRM with Q for dynamic channel access mode |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario A1, A2, B, C, D and E with dynamic channel access mode |
| 10. NR-unlicensed | 10-2a | SSB-based RRM for semi-static channel access mode | 1. SSB-based RRM with Q for semi-static channel access mode, when SMTC window is no longer than the fixed frame period |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario A1, A2, B, C, D and E with semi-static channel access mode |
| 10. NR-unlicensed | 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell for PCell and PSCell |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario B, C, D and E |
| 10. NR-unlicensed | 10-2c | SSB-based RLM for dynamic channel access mode | 1. SSB-based RLM with Q for dynamic channel access mode |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario B, C, D and E with dynamic channel access mode |
| 10. NR-unlicensed | 10-2d | SSB-based RLM for semi-static channel access mode | 1. SSB-based RLM with Q for semi-static channel access mode, when DRS window is no longer than the fixed frame period |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario B, C, D and E with semi-static channel access mode |
| 10. NR-unlicensed | 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell for PCell |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling This FG is a part of basic operation for following scenarios defined in TS38.300 - Scenario C and D |
| 10. NR-unlicensed | 10-2f | Support monitoring of extended RAR window | 1. Support of RAR extension from 10ms to 40ms by decoding of the 2-bit SFN indication in DCI 1_0 |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-2g | SSB-based BFD/CBD for dynamic channel access mode | SSB-based BFD/CBD with Q for dynamic channel access mode |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-2h | SSB-based BFD/CBD for semi-static channel access mode | SSB-based BFD/CBD with Q for semi-static channel access mode |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-2i | CSI-RS-based BFD/CBD for operation with shared spectrum channel access | CSI-RS-based BFD/CBD for operation with shared spectrum channel access |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-7 | UL channel access for 10 MHz SCell | - 10 MHz LBT bandwidth | one of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-10 | RSSI and channel occupancy measurement and reporting | - RSSI measurement - Channel occupancy reporting |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-11 | SRS starting position at any OFDM symbol in a slot | - Support transmitting SRS starting in all symbols (0,…,13) of a slot |  | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 10. NR-unlicensed | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Maximum number of frequency domain locations for a search space set configuration with freqMonitorLocations-r16 |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Candidate values of component 1: {1, 2, ,3, 4, 5} the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset |  | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 10. NR-unlicensed | 10-23 | CGI reading on unlicensed cell for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Support reading RMSI from an unlicensed cell for ANR the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-25 | Enable configured UL transmissions when SFI field in DCI 2_0 is configured but DCI 2_0 is not detected | 1. Support configuration of enableConfiguredUL-r16 and enable transmission of higher-layer configured UL *SRS, PUCCH, CG-PUSCH etc) when SFI field in DCI 2_0 is configured but DCI 2_0 is not detected |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-27 | Wideband PRACH | - Enhanced PRACH design for operation with shared spectrum channel access by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-29 | Support available RB set indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read availableRB-Sets-r16 |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read COT duration |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-8 | Type B PDSCH length {3, 5, 6, 8, 9, 10, 11, 12, 13} without DMRS shift due to CRS collision | - Type B PDSCH length {3, 5, 6, 8, 9, 10, 11, 12, 13} without DMRS shift due to CRS collision | 5-6a | Yes | N/A |  | Per band | N/A | N/A | N/A | Note length 9/10 with DMRS shift due to CRS collision are already covered by 14-2 | Optional with capability signalling |
| 10. NR-unlicensed | 10-9 | Search space set group switching with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support switching the search space set group with PDCCH decoding in group 1 4. Support a timer to switch back to original search space set group 5. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-9c | Joint search space group switching across multiple cells | - Configured with a group of cells and switch search space set group jointly over these cells | one of {10-9, 10-9b} | Yes | N/A |  | Per BC | N/A | N/A | N/A | Without this capability, the UE will switch search space set groups for different cells independently the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-9d | Support Search space set group switching capability 2 | - Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS | one of {10-9, 10-9b} | Yes | N/A |  | Per band | N/A | N/A | N/A | Without this capability, the UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2 the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | - Support configuration of a value for dl-DataToUL-ACK indicating an inapplicable time to report HARQ ACK |  | Yes | N/A |  | Per band | N/A | N/A | N/A | If non-numerical K1 value is supported the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 (configuration of nfi-TotalDAI-Included) 2. Support of bit field in DCI 0_1 for other group total DAI if configured. (configuration of ul-TotalDAI-Included) 3. Support the retransmission of HARQ ACK (pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16) |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each group | Optional with capability signalling |
| 10. NR-unlicensed | 10-16 | One-shot HARQ ACK feedback | - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH - Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
| 10. NR-unlicensed | 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 |  | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-19a] |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. NR-unlicensed | [10-19b] |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. NR-unlicensed | [10-19c] |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. NR-unlicensed | [10-19d] |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. NR-unlicensed | [10-19e] |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. NR-unlicensed | [10-19f] |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. NR-unlicensed | 10-26 | CSI-RS based RLM for operation with shared spectrum channel access | CSI-RS based RLM for operation with shared spectrum channel access |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-26a | CSI-RS based RRM for operation with shared spectrum channel access | CSI-RS based RRM for operation with shared spectrum channel access |  | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-31 | Support of P/SP-CSI-RS reception with CSI-RS-ValidationWith-DCI-r16 configured | 1. Validate P/SP-CSI-RS reception when receiving a DCI granting a PDSCH over the same set of symbols 2. Validate P/SP-CSI-RS reception when receiving a DCI triggering a A-CSI-RS over the same set of symbols |  | Yes | N/A |  | Per band | N/A | N/A |  | If UE does not signal capability for FG 10-31, the UE cannot be configured with CSI-RS-ValidationWith-DCI-r16. If none of the RRC parameters CO-DurationPerCell-r16, SlotFormatIndicator, and CSI-RS-ValidationWith-DCI-r16 is configured on a cell with shared spectrum access, and P/SP CSI-RS is configured, for reception/cancellation of SP/P CSI-RS the behavior in 11.1 of TS38.213 applies as per agreement. the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUSCH the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-3a | PRB interlace mapping for PUCCH | - PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 - PRB interlace frequency domain resource allocation for PUCCH format 2 - PRB interlace frequency domain resource allocation for PUCCH format 3 |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUCCH format 0/1 the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 | 1. OCC2 2. OCC4 | 10-3a | Yes | N/A |  | Per band | N/A | N/A | N/A | UE OCC capability for EPF2/EFP3 the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-13a | Extended CP range of more than one symbol for CG-PUSCH | - UE supports generating a CP extension of length longer than 1 symbol for Configured Grant PUSCH transmission | One or both of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-18 | Configured grant with retransmission in CG resources | 1. Support retransmission in CG resources 2. Support configured grant retransmission timer 3. Support DFI monitoring 4. Support CG-UCI in CG-PUSCH | One or both of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support configured grant with retransmission in configured grant resource the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-21a | Support using ED threshold given by gNB for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for Type 1 channel access for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for Type 1 channel access for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information | 10-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-21b | Support UL to DL COT sharing | 1. Support Type 1 LBT for scheduled UL to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16 2. Support Type 1 LBT for CG-PUSCH to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16 3. Indicate in CG-UCI the COT sharing information | 10-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 10. NR-unlicensed | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | 10-18 | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| 10. NR-unlicensed | 10-28 | Configured grant with Rel-16 enhanced resource configuration | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | One or both of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |