3GPP TSG RAN WG1 Meeting #104-e	R1-2101982

e-Meeting, January 25 – February 05, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1378 | rev | - | Current version: | V16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on power control for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1 #84bis, it was agreed that: MNPUSCH,c(i): {1/4, 1, 3, 6,12} (reflecting UL transmission resource bandwidth normalized by 15 kHz). However, in 16.2.1.1.1 of 36.213, the way for a NB-IoT UE to derive the value of from {1, 3, 6, 12} is not clearly defined for the case of 15 kHz subcarrier spacing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the definition of thatis the UL transmission resource bandwidth normalized by 15 kHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NB-IoT UE may not correctly derive the value of |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

16.2.1.1.1	UE behaviour

The setting of the UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission is defined as follows. For FDD, if the UE is capable of enhanced random access power control [12], and it is configured by higher layers, and for TDD,  enhanced random access power control shall be applied for a UE which started the random access procedure in the first or second configured NPRACH repetition level.

The UE transmit power  for NPUSCH transmission in NB-IoT UL slot i for the serving cell is given by:

For NPUSCH (re)transmissions corresponding to the random access response grant if enhanced random access power control is not applied, and for all other NPUSCH transmissions except for NPUSCH (re)transmission corresponding to preconfigured uplink resource, when the number of repetitions of the allocated NPUSCH RUs is greater than 2:

[dBm]

otherwise

[dBm]

where,

-	is the configured UE transmit power defined in [6] in NB-IoT UL slot i for serving cell .

-	MNPUSCH,c(i) is the NPUSCH transmission resource bandwidth normalized by 15 kHz, where {1/4} is used for 3.75 kHz subcarrier spacing and {1, 3, 6, 12} are used for 15kHz subcarrier spacing

-	PO_NPUSCH,cj is a parameter composed of the sum of a component  provided from higher layers and a component  provided by higher layers for j=1, 3 and for serving cell where . For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2 and for NPUSCH transmission using preconfigured uplink resource then j=3. PO_UE_NPUSCH,c2=0. If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied,

PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3

-	For j=1, for NPUSCH format 2, =1; for NPUSCH format 1, is provided by higher layers for serving cell . For j=2,  For j=3,  is the parameter alpha in PUR-Config-NB provided by higher layers for serving cell .

-	 is the downlink path loss estimate calculated in the UE for serving cell  in dB and  = nrs-Power + nrs-PowerOffsetNonAnchor – NRSRP, where nrs-Power is provided by higher layers and Clause 16.2.2, and nrs-PowerOffsetNonAnchor is set to zero if it is not provided by higher layers and NRSRP is defined in [5] for serving cell .

<Unchanged parts are omitted>