3GPP TSG RAN WG1 Meeting #104-e

e-Meeting, January 25th – February 5th, 2021

Title:	RAN1 Chairman’s Notes

1	Opening of the meeting (Day 1: UTC 9:00 AM)	3

1.1	Call for IPR	3

1.2	Competition Law Statement	3

1.3	Network Usage Conditions	3

1.4	Check-in for Registered Delegates	4

1.5	Aspects related to RAN1 Meeting Management	4

2	Approval of Agenda	4

3	Highlights from RAN plenary	4

4	Approval of Minutes from previous meetings	4

5	Incoming Liaison Statements (4)	4

5.1	RAN1 Aspects for RF requirements for NR frequency range 1 (FR1)	8

6	E-UTRA (10)	8

6.1	Maintenance of E-UTRA Releases 8 – 15 (4)	8

6.2	Maintenance of E-UTRA Release 16 (6)	8

6.2.1	Maintenance of Additional MTC Enhancements (2)	9

6.2.2	Maintenance of Additional Enhancements for NB-IoT (2)	9

6.2.3	Maintenance of DL MIMO efficiency enhancements for LTE (0)	9

6.2.4	Maintenance of LTE-based 5G Terrestrial Broadcast	9

6.2.5	LTE Rel-16 UE Features	9

6.2.6	Others	9

7	NR Maintenance (13+48=61)	9

7.1	Maintenance of Release 15 NR (18)	9

7.2	Maintenance of Release 16 NR (1+5+8+6+5+1+3+3+9+2=43)	11

7.2.1	Maintenance of Two step RACH for NR (1)	11

7.2.2	Maintenance of NR-based Access to Unlicensed Spectrum (5)	11

7.2.3	Maintenance of Integrated Access and Backhaul for NR (0)	13

7.2.4	Maintenance for 5G V2X with NR sidelink (8)	13

7.2.5	Maintenance of Physical Layer Enhancements for NR URLLC (6)	14

7.2.6	Maintenance of Enhancements on MIMO for NR (5)	16

7.2.7	Maintenance of UE Power Saving for NR (1)	16

7.2.8	Maintenance of NR positioning support (3)	17

7.2.9	Maintenance of NR Mobility Enhancements (0)	17

7.2.10	Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (3)	17

7.2.11	NR Rel-16 UE Features (9)	18

7.2.12	Others (2)	19

8	Release 17	20

8.1	Further enhancements on MIMO for NR (8)	20

8.1.1	Enhancements on Multi-beam Operation	20

8.1.2	Enhancements for Multi-TRP Deployment	21

8.1.2.1	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	21

8.1.2.2	Enhancements on Multi-TRP inter-cell operation	21

8.1.2.3	Enhancements on beam management for multi-TRP	22

8.1.2.4	Enhancements on HST-SFN deployment	23

8.1.3	Enhancements on SRS flexibility, coverage and capacity	23

8.1.4	CSI enhancements: MTRP and FR1 FDD reciprocity	23

8.1.5	Others	24

8.2	Supporting NR from 52.6GHz to 71 GHz	24

8.2.1	Initial access aspects	24

8.2.2	PDCCH monitoring enhancements	25

8.2.3	Enhancements for PUCCH formats 0/1/4	25

8.2.4	Beam management for new SCSs	26

8.2.5	PDSCH/PUSCH enhancements	26

8.2.6	Channel access mechanism	27

8.2.7	Others	27

8.3	Enhanced Industrial Internet of Things (IoT) and URLLC (6)	27

8.3.1	Necessity and Support of Physical Layer feedback enhancements	27

8.3.1.1	UE feedback enhancements for HARQ-ACK	27

8.3.1.2	CSI feedback enhancements	28

8.3.2	Enhancements for unlicensed band URLLC/IIoT	28

8.3.3	Intra-UE Multiplexing/Prioritization	29

8.3.4	Others	30

8.4	Solutions for NR to support non-terrestrial networks (NTN) (5)	30

8.4.1	Timing relationship enhancements	30

8.4.2	Enhancements on UL time and frequency synchronization	31

8.4.3	Enhancements on HARQ	31

8.4.4	Others	32

8.5	NR Positioning Enhancements	32

8.5.1	Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	32

8.5.2	Accuracy improvements for UL-AoA positioning solutions	33

8.5.3	Accuracy improvements for DL-AoD positioning solutions	33

8.5.4	Others	34

8.6	Support of Reduced Capability NR Devices	34

8.6.1	UE complexity reduction	34

8.6.2	Higher layer support of Reduced Capability NR Devices	34

8.6.3	Others	35

8.7	UE Power Saving Enhancements (4)	35

8.7.1	Enhancements for idle/inactive-mode UE power saving	35

8.7.1.1	Potential paging enhancements	35

8.7.1.2	TRS/CSI-RS occasion(s) for idle/inactive UEs	36

8.7.1.3	Others	36

8.7.2	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime	37

8.7.3	Others	37

8.8	NR coverage enhancement	37

8.8.1	PUSCH enhancements	37

8.8.1.1	Enhancements on PUSCH repetition type A	37

8.8.1.2	TB processing over multi-slot PUSCH	38

8.8.1.3	Joint channel estimation for PUSCH	38

8.8.2	PUCCH enhancements	39

8.8.3	Type A PUSCH repetitions for Msg3	39

8.8.4	Others	40

8.9	Rel-17 enhancements for NB-IoT and LTE-MTC (3)	40

8.9.1	Support of 16-QAM for unicast in UL and DL for NB-IoT	40

8.9.2	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC	40

8.9.3	Support a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability	40

8.9.4	Others	41

8.10	Enhancements to Integrated Access and Backhaul (2)	41

8.10.1	Enhancements to resource multiplexing between child and parent links of an IAB node	41

8.10.2	Other enhancements for simultaneous operation of IAB-node’s child and parent links	41

8.10.3	Others	42

8.11	NR Sidelink enhancement (4)	42

8.11.1	Resource allocation enhancement	42

8.11.1.1	Resource allocation for power saving	42

8.11.1.2	Feasibility and benefits for mode 2 enhancements	43

8.11.2	Others	43

8.12	NR Multicast and Broadcast Services (3)	44

8.12.1	Mechanisms to support group scheduling for RRC_CONNECTED UEs	44

8.12.2	Mechanisms to improve reliability for RRC_CONNECTED UEs	44

8.12.3	Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	45

8.12.4	Others	45

8.13	NR Dynamic spectrum sharing (DSS) (3)	45

8.13.1	Cross-carrier scheduling (from Scell to Pcell)	46

8.13.2	Multi-cell PDSCH scheduling via a single DCI	46

8.13.3	Support efficient activation/de-activation mechanism for SCells in NR CA	46

8.14	Study on XR Evaluations for NR (RAN1) (2)	47

8.14.1	Traffic Model	47

8.14.2	Evaluation Methodology	47

8.14.3	Initial Performance Evaluation Results	48

8.14.4	Others	48

8.15	Study on NB-IoT/eMTC support for Non-Terrestrial Network	48

8.15.1	Scenarios applicable to NB-IoT/eMTC	48

8.15.2	Enhancements to time and frequency synchronization	49

8.15.3	Timing relationship enhancements	49

8.15.4	Enhancements on HARQ	50

8.15.5	Others	50

8.16	Introduction of DL 1024QAM for NR FR1 (1)	50

8.17	Others	50

9	Closing of the meeting (Day 10: UTC 17:00 at the latest)	51

# Opening of the meeting (Day 1: UTC 9:00 AM)

## Call for IPR

I draw your attention to your obligations under the 3GPP Partner Organizations' IPR policies.  Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

The attention of the delegates to this meeting was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were thereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (http://www.etsi.org/WebSite/document/Legal/IPRForms.doc).

## Competition Law Statement

The attention of the delegates to this meeting was drawn to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel. The present meeting will be conducted with strict impartiality and in the interests of 3GPP. Furthermore, delegates were reminded that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network Usage Conditions

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1721392 for some thoughts on RAN1 meeting management.

# Approval of Agenda

R1-2100000	Draft Agenda of RAN1#104-e meeting	RAN1 Chair

Email approval till 1/26 – no dedicated email thread. Please express any comments under the general management email thread. Automatically approved if no comments.

R1-2101668	RAN1#104-e Meeting Invitation, Timelines, Scope, Process	RAN1 Chair, ETSI MCC

R1-2101669	Additional Guidelines for RAN1#104 e-Meeting Management	RAN1 Chair

# Highlights from RAN plenary

R1-2100002	Highlights from RAN#90-e	RAN1 Chair

# Approval of Minutes from previous meetings

R1-2100001	Report of RAN1#103-e meeting	ETSI MCC

Email approval till 1/26 - approved

# Incoming Liaison Statements (4)

R1-2101821	RAN1#104-e preparation phase on LSs	RAN1 Chair

To be handled in other AIs

- Email discussion/approval for the reply LS to R1-2100005 till 1/28, to be handled under 7.2.8  (Su Huang, Huawei)

- Email discussion/approval of the reply LS to R1-2100008 till 1/29, to be handled under 7.2.2 (Alex, Lenovo)

- Email discussion/approval of the reply LS to R1-2100009 till 1/29, to be handled under 7.2.4 (Qun, Xiaomi)

- Email discussion/approval of a potential reply LS to R1-2100013 till 1/29, to be handled under 7.2.10 (James, MediaTek)

- Email discussion/approval of a potential reply LS to R1-2100018 till 1/27, to be handled under 7.2.6 (Karri, Nokia)

- Email discussion/approval of the reply LS to R1-2100026 till 1/29, to be handled under 7.2.5 (name TBD, Huawei)

- Email discussion/approval of the reply LS to R1-2100003, to be handled under 7.2.11 (name TBD, CMCC)

- Email discussion regarding LS in R1-2100019, including a possible reply LS if necessary, to be handled under 7.2.11 (name TBD, Qualcomm)

- Email discussion/approval for the reply LS to R1-2100004 till 2/1, to be managed under 8.10 (name TBD, Samsung)

- Email discussion/approval of the reply LS to R1-2100014 till 2/1, to be handled under 8.4 (Xiaofeng, Qualcomm)

- Email discussion regarding LS in R1-2100020, including a possible reply LS till 2/1, to be handled under 8.7 (Weide, MediaTek)

- Email discussion regarding LS in R1-2100021, including a possible reply LS till 2/1, to be handled under 8.11 (Boyuan Zhang, ZTE)

- Email discussion/approval for the reply LS to LS in R1-2009644 till 1/28, to be handled under 7.2.4 (name Kevin, OPPO)

To be handled under AI5

[104-e-AI5-LS-01] Email discussion/approval for the reply LS to R1-2100022 till 1/28 (Stephen, Ericsson)

Update on 1/28: Latest draft LS (R1-2101953) is approved. Final LS in R1-2101962.

[104-e-AI5-LS-02] Email discussion/approval for the reply LS to R1-200023 till 1/28 (Karri, Nokia)

Possible CRs will be handled under 7.2.12

Update on 1/28: to check on 1/29 – to check on 2/1  2/2  2/5

[104-e-AI5-LS-03] Email discussion regarding LS in R1-2100025 including a potential reply LS till 2/3 – (Tian Li, ZTE)

[104-e-AI5-LS-04] Email discussion regarding LS in R1-2100015, including potentially a reply LS till 1/29 – (Kevin, OPPO)

Update on 1/29: latest text from the moderator is endorsed. To check draft LS for approval on 2/1  2/2

Update on 2/2: final LS is approved in R1-2101976

//NR

Rel-15

R1-2100022	LS on the use of simultaneous CSI-RS resources and ports	RAN2, Ericsson

R1-2100403	Draft reply LS on use of simultaneous CSI-RS resources and ports	CATT

R1-2101170	Draft reply LS on the use of simultaneous CSI-RS resources and ports	Samsung

R1-2101303	Draft Reply LS on the use of simultaneous CSI-RS resources and ports	Ericsson

R1-2101742	Discussion on use of simultaneous CSI-RS resources and ports	Huawei, HiSilicon

Rel-16 Pos.

R1-2100005	LS on Rel-16 NR Positioning Correction	RAN3, Huawei

R1-2100312	Discussion on RAN3 LS on Rel-16 NR Positioning Correction	CATT

R1-2100313	Draft Reply LS on Rel-16 NR Positioning Correction	CATT

R1-2101153	Draft reply LS on Rel-16 NR Positioning Correction	vivo

R1-2101250	Discussion on SRS carrier information	Huawei, HiSilicon

R1-2101251	Reply LS on Rel-16 NR Positioning Correction	Huawei, HiSilicon

Rel-16 Unlic

R1-2100008	LS on measuring CSI-RS during SCell activation.	RAN4, Ericsson

R1-2100888	Discussion on RAN4 LS on measuring CSI-RS during SCell activation	LG Electronics

R1-2101164	Discussion on measuring CSI-RS during Scell activation	Samsung

R1-2101156	Draft reply LS on measuring CSI-RS during SCell activation	vivo

R1-2101287	Discussion on RAN4 LS on measuring CSI-RS during SCell activation	Nokia, Nokia Shanghai Bell

R1-2101335	Discussion on measuring CSI-RS during SCell activation	Apple

R1-2101336	[Draft] LS reply on measuring CSI-RS during SCell activation	Apple

R1-2101747	Discussion on measuring CSI-RS during SCell activation	Huawei, HiSilicon

Rel-16 SL

R1-2100009	LS on SL switching priority	RAN4, Xiaomi

R1-2100139	Draft reply LS on SL switching priority	OPPO

R1-2100316	Draft Reply LS on SL switching priority	CATT, GOHIGH

R1-2100935	[Draft] Reply LS on SL switching priority	ZTE, Sanechips

R1-2101090	[Draft] Reply LS on SL switching priority	Xiaomi

R1-2101148	Draft relay LS on SL switching priority	vivo

R1-2101149	Discussion on switching between LTE SL and NR SL	vivo

R1-2101169	Draft reply LS on SL switching priority	Samsung

R1-2101427	Reply to RAN4 LS on SL Switching Priority	Qualcomm Incorporated

R1-2101706	[Draft] LS on SL switching priority	Ericsson

R1-2101735	Discussion on RAN4 LS on SL switching priority	Huawei, HiSilicon

Rel-16 MR-DC/CA

R1-2100013	LS on TCI state indication at Direct SCell activation	RAN4, MediaTek

R1-2100502	On TCI state indication for Scells	Nokia, Nokia Shanghai Bell

R1-2101147	Draft relay LS on TCI state indication at Direct SCell activation	vivo

R1-2101741	Discussion on TCI state indication at Direct SCell activation	Huawei, HiSilicon

Rel-16 eMIMO

R1-2100018	LS on multi-TRP description in Stage-2	RAN2, Nokia

R1-2100125	Discussion on LS on multi-TRP description in Stage-2	OPPO

R1-2101750	On changes to multi-TRP description in Stage 2	Huawei, HiSilicon

R1-2100031	Reply LS on multi-CC simultaneous TCI activation with multi-TRP/panel transmission	RAN2, Ericsson

Rel-16 URLLC/IIoT

R1-2100026	LS on overlapped data and SR are of equal L1 priority	RAN2, Huawei

R1-2100088	[DRAFT] Reply LS on overlapped data and SR of equal L1 priority	ZTE

R1-2100089	Discussion on overlapped data and SR of equal L1 priority	ZTE

R1-2100318	Discussion on overlapped data and SR of equal L1 priority	CATT

R1-2100727	[Draft] Reply LS on overlapped data and SR are of equal L1 priority	Nokia

R1-2101154	Reply LS on overlapped data and SR with equal L1 priority	vivo

R1-2101166	Draft reply LS on overlapped data and SR are of equal L1 priority	Samsung

R1-2101276	Draft reply LS on overlapped data and SR with equal L1 priority for Rel-16 URLLC	Huawei, HiSilicon

R1-2101277	Discussion on overlapped data and SR with equal L1 priority for Rel-16 URLLC	Huawei, HiSilicon

R1-2101502	Discussion of Response LS on Overlapped Data and SR of Equal L1 Priority	Ericsson Inc.

R1-2101528	Discussion on LS on overlapping data and SR with same L1 priority	Intel Corporation

UE Features

R1-2100003	LS on Rel-16 updated RAN4 UE features lists for LTE and NR	RAN4, CMCC

R1-2100887	Draft reply LS on Rel-16 updated RAN4 UE features lists for LTE and NR	LG Electronics

R1-2100019	LS on capability for extended RAR window monitoring	RAN2, Qualcomm

R1-2100889	Discussion on RAN2 LS on capability for extended RAR window monitoring	LG Electronics

Rel-16 TEIs

R1-2100023	LS on half-duplex operation	RAN2, Nokia

R1-2100085	[DRAFT] Reply LS on half-duplex operation	ZTE

R1-2100319	Draft reply LS on half-duplex operation	CATT

R1-2100504	On half-duplex operation	Nokia, Nokia Shanghai Bell

R1-2101158	Discussion on Configuration for CA Half duplex operation	vivo

R1-2101163	Draft reply LS on half-duplex operation	Samsung

R1-2101745	Discussion on LS on half-duplex operation	Huawei, HiSilicon

Rel-17 IAB

R1-2100004	LS on inter-donor topology redundancy	RAN3, Samsung

R1-2100961	About reply LS on inter-donor topology redundancy in IAB	ZTE, Sanechips

R1-2101167	Draft reply LS on inter-donor topology redundancy	Samsung

R1-2101155	Draft reply LS on on inter-donor topology redundancy	vivo

R1-2101749	Discussion on inter-donor topology redundancy	Huawei, HiSilicon

R1-2101697	Response to RAN3 LS on inter-donor topology redundancy	Ericsson

Rel-17 60GHz

R1-2100007	LS on Phase noise and other RF Impairment modelling	RAN4, Ericsson

Rel-17 NTN

R1-2100014	AN-PDB and PER targets for satellite access	SA2, Qualcomm

R1-2101719	Discussion on LS on AN-PDB and PER targets for satellite access	Intel Corporation

R1-2101748	Discussion on the AN-PDB and PER targets for satellite access	Huawei, HiSilicon

Rel-17 Power Saving:

R1-2100020	LS on Paging Enhancement	RAN2, MediaTek

R1-2101152	Discussion on Paging enhancement	vivo

R1-2101645	Discussion on the feasibility and limitations of paging subgrouping	ZTE , Sanechips

R1-2101667	[Draft] Reply LS on Paging Enhancement	Nokia, Nokia Shanghai Bell

R1-2101746	Discussion on LS on paging enhancement	Huawei, HiSilicon

Rel-17 SL

R1-2100021	LS to RAN1 on SL DRX design	RAN2, ZTE

R1-2100311	Discussion on LS from RAN2 on SL DRX design	CATT, GOHIGH

R1-2100513	Draft reply LS on SL DRX design	LG Electronics

R1-2100923	[draft]Reply LS to RAN2 on sidelink DRX	ZTE, Sanechips

R1-2101151	Draft relay LS on SL DRX design	vivo

R1-2101429	Reply to RAN2 LS on SL DRX design	Qualcomm Incorporated

R1-2101705	[Draft] LS on SL DRX design	Ericsson

R1-2101736	Discussion on RAN2 LS on DRX impact	Huawei, HiSilicon

Rel-17 IIoT/URLLC

R1-2100024	Reply LS on propagation delay compensation enhancements	RAN2, Nokia

Rel-17 small data

R1-2100025	LS on physical layer aspects of small data transmission	RAN2, ZTE

R1-2100080	Discussion on the physical layer aspects of small data transmission	ZTE, Sanechips

R1-2100317	Discussion on physical layer aspects of small data transmission	CATT

R1-2100501	On physical layer aspects of small data transmission	Nokia, Nokia Shanghai Bell

R1-2100627	Discussion on LS on small data transmission	Intel Corporation

R1-2100910	Discussion on physical layer aspects of small data transmission	LG Electronics

R1-2101159	Discussion on RAN1 impacts for small data transmisison	vivo

R1-2101165	Discussion on physical layer aspects of small data transmission	Samsung

R1-2101267	RA and CG based small data transmission	Huawei, HiSilicon

R1-2101338	Discussion on physical layer aspects of small data transmission	Apple

R1-2101405	Physical layer aspects of small data transmission	InterDigital, Inc.

R1-2101430	Reply to RAN2 LS on SDT resource configuration	Qualcomm Incorporated

R1-2101518	[DRAFT] LS reply on physical layer aspects of small data transmission	Ericsson

Rel-17 XR

R1-2101765	LS on XR-Traffic Models	SA4, Qualcomm

Others

R1-2100015	LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)	SA2, Tencent

R1-2101157	Draft reply LS on New Standardized 5QIs for 5G-AIS	vivo

R1-2101319	Draft LS response on New Standardized 5QIs for AIS	Ericsson

R1-2101337	Draft reply LS on newly proposed 5QIs for 5G-AIS	Apple

R1-2101410	Discussion on new standardized 5QIs for 5G-AIS from SA2	OPPO, Tencent, China Mobile, China Telecom, China Unicom

R1-2101411	[Draft] Reply LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)	OPPO

R1-2101753	Discussion on LS on new standardized 5QIs for 5G-AIS	Huawei, HiSilicon

R1-2100016	LS on Use of Inclusive Language in 3GPP	SA, Intel

//The following LSs are noted (RAN1 is cc-ed) with no email/GTW discussion, unless specifically requested

R1-2100006	Reply LS on LS on signalling of satellite backhaul connection	RAN3, Huawei

R1-2100010	LS response on cell-grouping UE capability for synchronous NR-DC	RAN4, Apple

R1-2100011	LS on DC location reporting for intra-band UL CA	RAN4, Nokia

R1-2100012	LS on removing restriction on configuring UL MIMO for SUL band	RAN4, CMCC

R1-2100017	LS on Physical layer assisted lightweight AKA (PL-AKA) protocol for the Internet of things			SA3, Ericsson

R1-2100027	Reply LS on power control for NR-DC	RAN2, vivo

R1-2100028	Reply LS on Direct Discovery and Relay	RAN2, OPPO

R1-2100029	Reply LS on defined values for V field in the Release 14 specification of MAC header	RAN2, Huawei

R1-2100030	Reply LS on Full slot formats support in TDD UL-DL configuration	RAN2, Qualcomm

R1-2100032	IEEE 1609 WG Liaison Message to 3GPP regarding defined values for V field in the Release 14 specification of MAC header	IEEE Vehicular Technology/Intelligent Transportation System 1609 WG

//Related to LS in R1-2009664

R1-2100138	Draft reply LS on per-table MCS range for mode-2	OPPO

R1-2100315	Draft Reply LS on per table MCS range for mode-2	CATT, GOHIGH

R1-2101150	Draft relay LS on  per-table MCS range for mode-2	vivo

R1-2101168	Draft reply LS on per-table MCS range for mode-2	Samsung

R1-2101428	Reply to RAN2 LS on per-table MCS range for mode-2	Qualcomm Incorporated

R1-2101704	[Draft] LS on per-table MCS range for mode-2	Ericsson

R1-2101734	Discussion on RAN2 LS on per-table MCS range for mode-2	Huawei, HiSilicon

//Related to LS in R1-2009508

R1-2100314	Draft Reply LS on RAN1 agreement on pre-emption	CATT, GOHIGH

//Related to LS in R1-2007504

R1-2100503	On DC location reporting	Nokia, Nokia Shanghai Bell

//Related to UE capbability

R1-2101737	Discussion on UE capability xDD differentiation for SUL/SDL bands	Huawei, HiSilicon

//LSs received during the e-meeting

R1-2102071	LS to capture Text Proposal for TR 38.857	RAN2, Ericsson

## RAN1 Aspects for RF requirements for NR frequency range 1 (FR1)

Refer to RP-202799 for details regarding RAN4-led WID on RF requirements for NR frequency range 1 (FR1)

Limit to maximum 1 contribution per company/organization/university.

Note: This is for Rel-17 only. For any Rel-16 related maintenance, please submit under 7.2.12

R1-2100086	Discussion on Rel-17 UL Tx Switching	ZTE

R1-2100126	Dsicussion on Rel-17 Tx switching enhancement	OPPO

Withdrawn

R1-2101270	Discussions on enhancements for UL Tx switching	Huawei, HiSilicon

# E-UTRA (10)

## Maintenance of E-UTRA Releases 8 – 15 (4)

R1-2100081	Draft CR on clarification of TPC command for PUCCH	ZTE

[104-e-LTE-6.1CRs-01] Email discussion/approval on R1-2100081 (MBMS) by Jan-28 – Xingguang (ZTE)

R1-2100559	Clarification on power control for NB-IoT	ZTE

[104-e-LTE-6.1CRs-02] Email discussion/approval on R1-2100559 (NB-IoT) by Jan-28 – Huiying (ZTE)

R1-2101281	Correction on spectral efficiency of 1024QAM	Huawei, HiSilicon

R1-2101508	Correction for support of 1024QAM for PDSCH	Qualcomm Incorporated

[104-e-LTE-6.1CRs-03] Email discussion/approval on R1-2101281  and R1-2101508  (1024QAM) by Jan-27 – Yubo (Huawei)

R1-2101577	Discussion on UCI multiplexing for partial PUSCH mode 1	Samsung, Huawei, HiSilicon

R1-2101578	Draft 36.212 CR on UCI multiplexing for partial PUSCH mode 1	Samsung, Huawei, HiSilicon

[104-e-LTE-6.1CRs-04] Email discussion/approval R1-2101577 and R1-2101578 (LTE-unlic) by Jan-29 – Jinyoung (Samsung)

## Maintenance of E-UTRA Release 16 (6)

Only essential corrections

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”, unless otherwise noted

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including one or more TPs in the contribution(s), where each TP is provided necessary justification

### Maintenance of Additional MTC Enhancements (2)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2100560	Clarification on DCI size alignment for MTC	ZTE

R1-2100561	Corrections on scheduling enhancement for MTC	ZTE

R1-2100761	Corrections on multicast gap in Multiple TB	Lenovo, Motorola Mobility

R1-2101279	Corrections on multi-TB scheduling for eMTC	Huawei, HiSilicon

R1-2101702	PUR maintenance issues for Rel-16 LTE-MTC	Ericsson

[104-e-LTE-eMTC5-01] PUR Issues – Gus (Sierra Wireless)

- Issue #1: Unclear when zeros shall be appended to DCI (R1-2100560)

- Issue #2: Unclear meaning of ‘PUSCH repetition adjustment’ field (R1-2101702)

- Discussion and decision by Jan 29, TPs by Feb 5

[104-e-LTE-eMTC5-02] Multi-TB issues – Johan (Ericsson)

- Issue #1: Clarification of DCI definition for SPS validation (R1-2100561)

- Issue #2: Clarification of multicast scheduling gap definition (R1-2100761, R1-2101279)

- Discussion and decision by Jan 29, TPs by Feb 5

### Maintenance of Additional Enhancements for NB-IoT (2)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2100562	Clarification on DCI size alignment for NB-IoT	ZTE

R1-2100563	Corrections on scheduling enhancement for NB-IoT	ZTE

R1-2101703	PUR maintenance issues for Rel-16 NB-IoT	Ericsson

R1-2101282	Correction on transmission scheme for NPDSCH configured by PUR-RNTI	Huawei, HiSilicon

[104-e-LTE-NB_IoTenh3-01] PUR issues – Mixiang (Huawei)

- Issue#1: Transmission scheme for NPDSCH configured by PUR-RNTI (R1-2101282)

- Issue#2: DCI size alignment (R1-2100562)

- Issue#3: Adding parameter name (R1-2101703)

- Discussion and decision by Jan 29, TPs by Feb 5

[104-e-LTE-NB_IoTenh3-02] Multi-TB issues – Huiying (ZTE)

- Issue#1: Mapping between DCI field and the number of scheduled TB (R1-2100563)

- Issue#2: Determination of number of scheduled TB for SC-MTCH (R1-2100563)

- Discussion and decision by Jan 29, TPs by Feb 5

### Maintenance of DL MIMO efficiency enhancements for LTE (0)

### Maintenance of LTE-based 5G Terrestrial Broadcast

### LTE Rel-16 UE Features

### Others

R1-2100320	Draft CR to 36.214 on corrections for Reference Point for eNB Rx – Tx time difference	CATT

# NR Maintenance (18+44=62)

## Maintenance of Release 15 NR (18)

Only essential corrections – a rejected draft CR will be marked in red

Including CRs rejected for Rel-15, but can be considered for Rel-16. For each of these contributions, please clearly indicate that it’s originated from Rel-15 maintenance discussion (but not adopted), and is for Rel-16 maintenance consideration only

R1-2100082	Discussion on UL skipping for PUSCH	ZTE

R1-2100321	Discussion on PUSCH skipping	CATT

R1-2100405	Discussion on PUSCH skipping with overlapping UCI on PUCCH in Rel-16	vivo

R1-2100754	PUSCH skipping with UCI (without LCH and PHY prioritization) (Rel-16)	Nokia, Nokia Shanghai Bell

R1-2100796	Discussion on UL skipping for CG PUSCH	Spreadtrum Communications

R1-2101145	Discussion on CG collision with UCI and DG	Ericsson

R1-2101341	Discussions on PUSCH skipping	Apple

R1-2101580	Discussion on skipping vs UCI multiplexing on configured PUSCH	NTT DOCOMO, INC.

R1-2101739	Discussion on remaining issues of UL skipping for CG PUSCH	Huawei, HiSilicon

[104-e-NR-7.1CRs-01] Discussion on UL skipping for PUSCH – Xiaohang (vivo) by Feb 5

- For Rel-16 only

R1-2100084	Discussion on Ambiguity Issues for UE Features with Cross-Carrier Operation	ZTE

R1-2100404	Discussion on UE FG ambiguity in case of cross-carrier operation	vivo

R1-2101339	Clarification of FG3-5b with Cross Carrier Operation	Apple

R1-2101433	Clarification on UE Features with Cross-Carrier Operation	Qualcomm Incorporated

[104-e-NR-7.1CRs-02] Discussion on Ambiguity Issues for UE Features with Cross-Carrier Operation – Xingguang (ZTE) by Jan 29

R1-2100580	Clarification on back-to-back PUSCHs scheduling restriction in Rel-15	MediaTek Inc.

R1-2101340	Clarification on the PUSCH scheduling constraint in Rel-15	Apple

[104-e-NR-7.1CRs-03] Clarification on back-to-back PUSCHs scheduling restriction in Rel-15 – Yushu (Apple) by Jan 29

R1-2101275	Correction on the search space configuration of PUCCH-SCell	Huawei, HiSilicon

[104-e-NR-7.1CRs-04] Correction on the search space configuration of PUCCH-Scell – Xinghua (Huawei) by Jan 29

R1-2101434	Clarification on Cancellation due to SFI, PDSCH, or CSI-RS	Qualcomm Incorporated

[104-e-NR-7.1CRs-05] Clarification on Cancellation due to SFI , PDSCH , or CSI–RS – Kianoush (Qualcomm) by Jan 29

R1-2101529	Correction on MCS values for PT-RS time density determination in TS 38.214	Sharp

[104-e-NR-7.1CRs-06] Correction on MCS values for PT-RS time density determination in TS 38.214 – TBD (Sharp) by Jan 29

R1-2100283	Discussion on closed-loop power control with an absolute TPC command	ZTE

R1-2100284	Draft CR on closed-loop power control with an absolute TPC command	ZTE

[104-e-NR-7.1CRs-07] Draft CR on closed-loop power control with an absolute TPC command – Bo (ZTE) by Jan 29

- For Rel-16 only

R1-2100326	Correction on UCI multiplexing with PUCCH overriding	CATT

[104-e-NR-7.1CRs-08] Correction on UCI multiplexing with PUCCH overriding – Yanping (CATT) by Jan 29

R1-2100329	Correction on UE sounding procedure	CATT

[104-e-NR-7.1CRs-09] Correction on UE sounding procedure – Runhua (CATT) by Jan 29

R1-2100610	Clarification on Numerology for Active DL and UL BWPs	MediaTek Inc.

[104-e-NR-7.1CRs-10] Clarification on Numerology for Active DL and UL BWPs – Yi-Ju (MediaTek) by Jan 29

R1-2101136	Draft 38.214 CR on CSI request constraint per slot	MediaTek Inc.

[104-e-NR-7.1CRs-11] Draft 38.214 CR on CSI request constraint per slot – Yi-Ju (MediaTek) by Jan 29

R1-2101431	Discussion on CSI-RS ambiguities	Qualcomm Incorporated

[104-e-NR-7.1CRs-12] Discussion on CSI-RS ambiguities – Chenxi (Qualcomm) by Jan 29

R1-2101432	Discussion on SRS carrier switching	Qualcomm Incorporated

[104-e-NR-7.1CRs-13] Discussion on SRS carrier switching – Alberto (Qualcomm) by Jan 29

- For Rel-16 only

R1-2101722	Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different SCSs in DL and UL			Ericsson

R1-2101723	Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different SCSs in DL and UL			Ericsson

[104-e-NR-7.1CRs-14] Draft CR on Type-1 HARQ -ACK for PDSCH repetition with different SCSs in DL and UL – Sorour (Ericsson) by Jan 29

R1-2100083	Draft CR on Type-2 HARQ-ACK codebook in physical uplink shared channel	ZTE

R1-2100324	Correction on HARQ-ACK transmission in PUSCH	CATT

R1-2100325	Correction on HARQ-ACK transmission in PUSCH	CATT

[104-e-NR-7.1CRs-15] Draft CR on Type-2 HARQ -ACK codebook in physical uplink shared channel – Yanping (CATT) by Jan 29

R1-2100406	Draft CR for Msg3 retransmission power control	vivo

[104-e-NR-7.1CRs-16] Draft CR for Msg3 retransmission power control – Siqi (vivo) by Jan 29

R1-2101134	38.213 CR on DCI ordering in a search space set	Nokia, Nokia Shanghai Bell

[104-e-NR-7.1CRs-17] 38.213 CR on DCI ordering in a search space set – Karri (Nokia) by Jan 29

R1-2100322	Correction on HARQ-ACK transmission in PUCCH	CATT

R1-2100323	Correction on HARQ-ACK transmission in PUCCH	CATT

R1-2100327	Correction on HARQ-ACK/SR/CSI multiplexing in a PUCCH	CATT

R1-2100328	Correction to UE CSI computation time	CATT

R1-2100407	Draft CR on PUCCH closed loop power control	vivo

R1-2100616	Draft CR on prioritization between SRS and PUCCH	LG Electronics

R1-2101135	Draft 38.212 CR on NNZC indicator field for type-II CSI reporting	MediaTek Inc.

R1-2101160	Discussion on Rel-15 beamswitchtiming capability reporting and QCL assumption for aperiodic TRS		vivo

R1-2101161	Correction on QCL assumption for aperiodic TRS	vivo

R1-2101171	Draft CR on configured grant based PUSCH	Samsung

R1-2101743	Correction on PUSCH frequency hopping in 38.214	Huawei, HiSilicon

R1-2101744	Correction on out-of-order HARQ operation in 38.214	Huawei, HiSilicon

[104-e-NR-7.1CRs-18] Review of draft CRs not discussed under any of the emails threads [104-e-NR-7.1CRs-01] ~ [104-e-NR-7.1CRs-17] – Youngbum (Samsung) by Jan 29

## Maintenance of Release 16 NR (1+5+8+6+5+1+4+3+9+2=44)

Only essential corrections

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”, unless otherwise noted

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including one or more TPs in the contribution(s), where each TP is provided necessary justification

R1-2100256	Alignment of notation	Ericsson

[104-e-NR-AlignmentCRs-xxx] Email discussion/approval regarding Rel-16 alignment CRs till 2/2 (editors)

- Where xxx is the spec #.

- Including inputs in R1-2100256 for 38.211

Update on 2/3 – extend to 2/4

### Maintenance of Two step RACH for NR (1)

R1-2101408	FL summary on the maintenance of 2-step RACH	Moderator (ZTE)

[104-e-NR-2step-RACH-01] Corrections of 2-step RACH related issues till 1/28 – Li (ZTE)

- CR in R1-20087852100243, CR in R1-2101526, and TP in R1-2101573

Update on 1/28: final summary in R1-2101897, draft CRs in R1-2101898 (38.214) and R1-2101899 (38.213).

The draft CRs are endorsed. Final CRs in R1-2101965 (38.213, CR0184) and R1-2101964 (38.214, CR0157)

Conclusion:

- It’s up to gNB scheduling to handle the case that unicast PDSCH(s) TDMed multiplexing with MsgB in a slot per CC based on UE capability.

R1-2100243	Correction on DMRS configuration for MsgA	Huawei, HiSilicon

R1-2101526	Draft CR to 38.213 on corrections for 2-step RACH	Ericsson

R1-2101573	Discussion on remaining issues on 2-step RACH	Spreadtrum Communications

### Maintenance of NR-based Access to Unlicensed Spectrum (5)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2101247	Feature lead summary for NR-U DL Signals and Channels	Moderator (Lenovo)

R1-2101283	Feature Lead Summary on Channel Access Procedures for NR-U	Moderator (Nokia)

R1-2101332	Feature lead summary for enhancements to initial access procedures	Moderator (Charter Communications)

R1-2100071	Text proposals on type-3 HARQ-ACK codebook and multi-PUSCH scheduling	ZTE, Sanechips

R1-2100072	Remaining issue on the channel access for FBE	ZTE, Sanechips

R1-2100147	Discussion on the remaining issues of channel access procedure	OPPO

R1-2100148	Text proposals on type-3 HARQ-ACK codebook	OPPO

R1-2100199	Maintenance on channel access procedures for NR Unlicensed	Huawei, HiSilicon

R1-2100240	Maintenance on DL signals and channels for NR Unlicensed	Huawei, HiSilicon

R1-2100331	Correction on Type-3 HARQ-ACK codebook	CATT

R1-2100332	Correction on power control for HARQ-ACK transmission	CATT

R1-2100408	Maintenance on HARQ operation for NR-U	vivo

R1-2100409	TP on frequency hopping for NR-U configured grant	vivo

R1-2100628	Remaining issues on NR-U	Intel Corporation

R1-2100818	Maintenance of DL signals and channels for NR-U	Spreadtrum Communications

R1-2100890	Remaining issues of channel access procedure and DL signals and channels for NR-U	LG Electronics

R1-2100891	Remaining issues of HARQ procedure for NR-U	LG Electronics

R1-2101072	Remaining issues on UL transmissions	ETRI

R1-2101172	Correction on the condition to use Type 1 DL channel access	Samsung

R1-2101173	Correction on the use of subCarrierSpacingCommon in NR-U	Samsung

R1-2101284	Corrections on Channel Access Procedures for NR-U 	Nokia, Nokia Shanghai Bell

R1-2101304	Corrections related to DL, UL, and channel access	Ericsson

R1-2101305	Discussion on LS from RAN4 on measuring CSI-RS during SCell activation	Ericsson

R1-2101435	Invalid SSB in FBE	Qualcomm Incorporated

R1-2101530	Correction to FDRA field description	Sharp

R1-2101531	Correction on FBE COT definition	Sharp

R1-2101651	Remaining issues for multi PUSCHs in NR-U	ASUSTeK

R1-2101652	Remaining issues for CG PUSCH in NR-U	ASUSTeK

R1-2101671	Correction on DL/UL channel access procedure for NR-U	WILUS Inc.

[104-e-NR-NRU-01] Email discussion/approval on DL signals and channels until Jan-29 – Alex (Lenovo)

High priority on

- DL-C1: Front-loaded DMRS collision with CORESET

- DL-C2: PDSCH mapping type B with durations larger than 7 symbols

- DL-C3: Processing time

Low priority on DL-A2: Search space set group switching

[104-e-NR-NRU-02] Email discussion/approval on channel access until Jan-29 – Timo (Nokia)

High priority on

- CA 2.3: Clarifications to channel access for semi-static channel occupancy

- CA 2.2: Clarifications to LBT with consecutive UL transmissions

Low priority on

- CA 2.1: LBT type for non-contiguous SRS and PUSCH/PUCCH

- CA 2.4: Clarifications to restrictions for Type 1 DL channel access / DRS

- CA 2.5: Clarifications to UL CWS adjustment

- CA 2.6: Multi-channel Channel Access

- CA 2.7: LBT type indication in DCI 0_2 and 1_2

[104-e-NR-NRU-03] Email discussion/approval on HARQ enhancements in R1-2101770 until Feb-03 – David (Huawei)

High priority on HARQ3-Issue1, HARQ3-issue4, HARQ3-issue5, HARQ4, Multi-PUSCH issue2, Multi-PUSCH Issue3. Low priority on HARQ1, HARQ2, Multi-PUSCH issue1

[104-e-NR-NRU-04] Email discussion/approval on editorial issues in initial access signal, UL signals and channels and configured grant until Jan-29 – Jing (Qualcomm)

High priority on

- Init-2: Clarification on usage of subCarrierSpacingCommon for unlicensed

- UL-01: Correction to description of FDRA field description in DCI 0_0 and 0_1 to ensure that it is defined both for the case when interlacing is configured and the case when interlacing is not configured

- CG-TP2: RRC parameter name alignment in 38.213

Low priority on

- Init-1: Invalid SSB by SSB positions in burst for FBE

- CG-TP1: The intra-slot frequency hopping is supported while the inter-slot frequency hopping is not supported for NR-U configured grant PUSCH repetition

- CG-TP3: for K=1 and UE provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, the case of whether UE transmits in repK earliest transmission occasion candidate is missing in 38.214

[104-e-NR-NRU-05] Email discussion/approval of the reply LS to R1-2100008 until Jan-29 - Alex (Lenovo)

### Maintenance of Integrated Access and Backhaul for NR (0)

R1-2101259	Discussion on inter-band DC for Rel-16 IAB	Huawei, HiSilicon

### Maintenance for 5G V2X with NR sidelink (8)

For this agenda, no more than 3 contribution per 1 company/organization/university

[104-e-NR-5G_V2X-01]: PS-1: SL max data rate – till 1/28, with potential CRs till 2/2– Jeongho (Samsung)

- Editorial changes for FD-OCC, CSI-RS resources, reference in SCI fields, MCS threshold for SL PT-RS can be discussed in the CR preparation.

Update on 1/28:

Agreements:

- The following value, OH, are used for the calculation of SL max data rate.

  - 0.25 for FR2 in SL

- RAN1 sends an LS to RAN2 to inform the agreed overhead value for SL max data rate and also to fix the typo as below.

  - Qmis the maximum supported modulation order between 6 or 8 given by higher layer parameter sl-Tx-256QAM and sl-Rx-256QAM,

OH for FR1? Check on 1/29  2/1

Agreements:

- The following value, OH, are used for the calculation of SL max data rate.

  - 0.217 for FR1 in SL

Draft LS? 2/2

Update 2/3:

- The draft LS is approved. Final LS in R1-210xxxx.

- The TPs for issues #2/#3 are endorsed. To be included in the alignment CRs (38.212, 38.214)

[104-e-NR-5G_V2X-02]: Clarification on the S-SSB slot – till 1/28, with potential CRs till 2/2 – Teng (CATT)

- SY-4: Determination of slots including S-SSB transmission

- SY-6: Restriction of S-SSB slot

- Corrections for SY-1 Correction on SL-BCH, SY-2 Corrections/clarifications on S-SSB and SL-SSID can be discussed in the CR preparation.

Update on 1/28: to check on 1/29   to check on 2/1

Update on 2/1: latest two TPs are endorsed. To check the draft CRs 2/2

Update on 2/3:

- The four draft CRs (38.211, 38.212, 38.213 (two)) are approved.

- Final CRs in

  - R1-210xxxx (38.211, CRxxxx)

  - R1-210xxxx (38.212, CRxxxx)

  - R1-210xxxx (38.213, CRxxxx)

  - R1-210xxxx (38.213, CRxxxx)

[104-e-NR-5G_V2X-03]: Issues related to SL HARQ-ACK reporting to gNB, till 1/28, with potential CRs till 2/2 – Ricardo (Ericsson)

- M1-1-1: 	How to multiplex SL HARQ-ACK on a PUSCH scheduled by DCI 0-2

- M1-1-2: Codebook configuration

- M1-1-4: 	Clarifications on PUCCH slot/resource determination

- “Simpler corrections” in M1-1-5-4 (Exceptional reports: Due to intra-prioritization: Correction because it applies to DG and CG), 7-2 (Clarification about PUCCH TX power), 7-3 (Clarification that higher layer parameter N1PUCCH-AN-r16 is used only for SL CG type 1) can be discussed during the CR preparation.

Update on 1/28: to check on 1/29

Agreements:

Clarify in Clause 16.5.1.2 and 16.5.2.2 in TS 38.213 that an UL transmission resulting in DL/SL HARQ-ACK information multiplexed in PUSCH may be scheduled by DCI format 0_2.

Agreements:

The parameter pdsch-HARQ-ACK-Codebook is always used for reporting SL HARQ-ACK information.

Agreements:

Clarify that when DCI format 3_0 does not include the PSFCH-to-HARQ_feedback timing indicator field, the feedback slot is determined by sl-PSFCH-ToPUCCH-CG-Type1-r16 for CG type-1 and sl-PSFCH-ToPUCCH-r16 otherwise.

Last proposal? 2/1

Agreements:

- Send an LS to RAN2 describing that:

  - Per RAN1 agreements, the parameter sl-N1PUCCH-AN-r16 should be used for SL CG Type 2 (only for PSCCH/PSSCH transmissions without a corresponding PDCCH).

  - In RAN1’s understanding, the parameter cannot be configured for SL CG type-2 and asking RAN2 whether they can provide a solution that would be compatible with the existing RAN1 agreements.

LS & CR? 2/2

Agreement:

- Send an LS to RAN2 with the following agreement and asking them to make the appropriate changes to their specifications.

Agreement: - The parameter pdsch-HARQ-ACK-Codebook is always used for reporting SL HARQ-ACK information.

- Draft LS is approved. To combine with the above agreement into an updated LS  2/3

- Draft CR?  2/2 late today?

[104-e-NR-5G_V2X-04]: UE behavior regarding non-monitored slots in mode 2, till 1/28, with ponteital CRs till 2/2 – Sergey (Intel)

- M2-17: Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions

- Changes for the uncaptured agreement (M2-3: Capture RAN1#103-e agreement on pre-emption) can be discussed during the CR preparation.

Update on 1/28: to check on 1/29 – for the issue discussed in Section 3.1 of the latest summary, conclude that no change is necessary; latest TP + one update is endorsed. CR till 2/2

Update on 2/3: the 38.214 draft CR is approved. Final CR in R1-210xxxx (38.214, CRxxxx).

[104-e-NR-5G_V2X-05]: Remaining UL/SL prioritization rule, till 1/28, with potential CRs till 2/2 – Hanbyul (LGE)

- PP-1-1: How SL HARQ-ACK report is piggybacked on PUSCH

- PP-1-3: Further consideration on prioritization rule between PUCCH for the response of MsgB and SL TX

- PP-1-4: Further consideration on prioritization rule between SL reception and PUCCH carrying SL HARQ-ACK report

Update on 1/28:

Agreements:

- UE does not expect the case when a PUSCH with no UCI overlaps with two non-overlapping PUCCHs each of which contains SL HARQ-ACK and Uu UCI.

  - No spec change is needed.

Agreements:

- PUCCH transmission for the response of MsgB and Msg4 is prioritized over SL transmission(s).

Agreements:

- For prioritization between SL PSFCH or S-SSB reception and PUCCH carrying SL HARQ reporting,

  - The PUCCH transmission has higher priority than a SL PSFCH or S-SSB reception if a priority value of the PUCCH is smaller than a priority value of the SL PSFCH or S-SSB reception.

  - If the priority value of the PUCCH transmission is larger than the priority value of the SL PSFCH or S-SSB reception, the SL PSFCH or S-SSB reception has higher priority.

Proposal 1? 1/29  2/1  2/2

Update on 2/2:

Agreement: In terms of prioritization between PUCCH with SL HARQ-ACK and PUCCH with UCI, the principle specified in 9.2.5.0 are used only.

CRs? 2/2

[104-e-NR-5G_V2X-06]: PP-4: Interpretation of resource reservation period field in a SCI format 1-A (the same issue in M2-2: Clarification on UE procedure for determining the number of logical slots for a reservation period), till 1/28, with potential CRs till 2/2 – Hanbyul (LGE)

Update on 1/28: to check on 1/29

Update on 1/31:

Agreement:

- Alt 1 shown below is agreed.

Latest draft LS (+two updates) is endorsed. Final LS in R1-2101922.

Any other issues? Draft CRs to be prepared, conditioned on RAN2’s reply LS for potential approval  2/4?

[104-e-NR-5G_V2X-07]: Email discussion/approval for the reply LS to LS in R1-2009644 (MCS range for Mode-2 per MCS table) till 1/28, to be handled under 7.2.4 (Kevin, OPPO)

Update on 1/28: a proposal for final check, 1/29  -> 2/1

Update on 2/1:

Conclusion:

RAN1 answer: There is no consensus in RAN1 for this issue, although  there is a majority view that the MCS ranges for mode-2 operation should be defined per MCS table. RAN1 would like to leave the decision to RAN2 to make any update or not

Draft reply LS? 2/2 – final LS is approved in R1-2102017.

[104-e-NR-5G_V2X-08]: Email discussion/approval of the reply LS to R1-2100009 (sidelink switching priority) till 1/29, to be handled under 7.2.4 (Qun, Xiaomi)

Update on 1/31 – to check on 2/1

Update on 2/1 – latest draft LS is approved. Final LS in R1-2102034.

R1-2100135	Draft TP on physical strucutre for NR sidelink	OPPO

R1-2100136	Remaining open issues and corrections for physical layer procedure	OPPO

R1-2100137	Remaining open issues and corrections for mode 1 and mode 2 RA	OPPO

R1-2100204	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

R1-2100333	Discussion and TPs on sidelink synchronization mechanism and physical layer structure in NR V2X		CATT, GOHIGH

R1-2100334	Discussion and TPs on resource allocation in NR V2X	CATT, GOHIGH

R1-2100335	Discussion and TPs on physical layer procedures in NR V2X	CATT, GOHIGH

R1-2100410	Maintenance on physical layer structure for NR sidelink	vivo

R1-2100411	Maintenance on resource allocation mechanisms for NR sidelink	vivo

R1-2100412	Maintenance on NR sidelink synchronization and procedures	vivo

R1-2100514	Discussion on essential corrections in physical layer structure	LG Electronics

R1-2100515	Discussion on essential corrections in resource allocation for Mode 1 and 2	LG Electronics

R1-2100516	Discussion on essential corrections in physical layer procedure	LG Electronics

R1-2100629	Correction to FD-OCC for PSCCH	Intel Corporation

R1-2100630	Corrections to Mode-2 resource allocation	Intel Corporation

R1-2100631	Corrections to sidelink procedures	Intel Corporation

R1-2100734	A remaining issue on Mode-1 resource allocation for NR sidelink	Fujitsu

R1-2100735	Remaining issues on physical layer procedures for NR sidelink	Fujitsu

R1-2100799	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

R1-2100800	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

R1-2100936	Remaining issues on sidelink synchronization	ZTE, Sanechips

R1-2100937	Remaining issues on mode1	ZTE, Sanechips

R1-2100938	The slot set for SL resource allocation procedure	ZTE, Sanechips

R1-2100945	Remaining issues on resource allocation mode 2	NEC

R1-2101073	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

R1-2101174	Maintenance for NR Sidelink Physical Layer Structure	Samsung

R1-2101175	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

R1-2101176	Maintenance for NR Sidelink Mode 2 Operation	Samsung

R1-2101344	Remaining Issues of Sidelink Physical Layer Procedures	Apple

R1-2101345	Remaining Issue of Mode 1 Resource Allocation	Apple

R1-2101346	Remaining Issues of Mode 2 Resource Allocation	Apple

R1-2101436	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

R1-2101437	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

R1-2101438	Remaining Issues in Physical Layer Procedure	Qualcomm Incorporated

R1-2101532	Remaining issues on physical layer structure and procedures for NR sidelink	Sharp

R1-2101533	Remaining issues on resource allocation for NR sidelink	Sharp

R1-2101534	Remaining issues on synchronization mechanism for NR sidelink	Sharp

R1-2101571	Remaining issues on sidelink mode 2	ASUSTeK

R1-2101581	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

R1-2101582	Maintenance for sidelink synchronization and mode 2	NTT DOCOMO, INC.

R1-2101583	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

R1-2101649	Remaining issues on type-1 HARQ-ACK codebook considering multiple sidelink reosurce pools			ASUSTeK

R1-2101650	Remaining issues on sidelink procedure	ASUSTeK

R1-2101707	Draft_CR_TS38.212	Ericsson

R1-2101708	Draft_CR_TS38.213	Ericsson

R1-2101709	Draft_CR_TS38.306	Ericsson

R1-2101732	Correction on PSBCH payload generation	Huawei, HiSilicon

R1-2101733	Correction on determination of PSFCH resources based on a set of configured PRBs	Huawei, HiSilicon

R1-2101759	Remaining details for Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

R1-2101760	Remaining details for Physical layer structure for sidelink	Nokia, Nokia Shanghai Bell

### Maintenance of Physical Layer Enhancements for NR URLLC (6)

Including maintenance for IIoT

For this agenda, no more than 3 contribution per 1 company/organization/university

R1-2101670	Summary#1 on UCI enhancements for R16 URLLC	OPPO

R1-2100090	Draft CR on resource allocation for uplink transmission with configured grant	ZTE

R1-2100091	Remaining issues on intra-UE multiplexing in Rel-16 URLLC	ZTE

R1-2100092	Discussion on the timing for secondary cell activation and deactivation	ZTE

R1-2100178	Text proposal for intra UE prioritization timeline	OPPO

R1-2100179	Remaining issues on  scheduling and HARQ	OPPO

R1-2100180	Remaining issues on DL SPS enhancement	OPPO

R1-2100265	Intra-UE Multiplexing and Prioritization for Rel-16 URLLC	Ericsson

R1-2100266	Maintenance of UCI for NR URLLC	Ericsson

R1-2100267	Maintenance of Scheduling/HARQ for NR URLLC	Ericsson

R1-2100336	Discussion on overlapping between CG PUSCH and DG PUSCH	CATT

R1-2100337	Remaining issues on UCI enhancements	CATT

R1-2100338	Remaining issues on intra-UE prioritization	CATT

R1-2100413	Maintenance on UCI enhancements	vivo

R1-2100414	Maintenance on Scheduling/HARQ	vivo

R1-2100415	Maintenance on intra-UE prioritization	vivo

R1-2100632	On remaining details for handling CG-DG PUSCH overlaps	Intel Corporation

R1-2100736	A remaining issue on timing for applying SCell activation or deactivation	Fujitsu

R1-2100756	URLLC/IIoT PUSCH skipping (with LCH and/or PHY prioritization configured)	Nokia, Nokia Shanghai Bell

R1-2100792	Remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

R1-2100793	Remaining issues of CG-PUSCH enhancements for URLLC	Spreadtrum Communications

R1-2100826	Maintenance of Rel-16 URLLC intra-UE and inter-UE prioritization/ multiplexing enhancements 			Nokia, Nokia Shanghai Bell

R1-2100827	Draft CRs on timing of SCell activation/deactivation for sub-slot PUCCH and number of PUCCHs with HARQ-ACK in a slot/sub-slot	Nokia, Nokia Shanghai Bell

R1-2100829	PUSCH skipping	InterDigital, Inc.

R1-2100898	PHR issues related to URLLC/IIOT WI	LG Electronics

R1-2100899	Remaining issues of other aspects for URLLC/IIOT	LG Electronics

R1-2101091	Correction on UE procedure for reporting HARQ-ACK	Xiaomi

R1-2101177	Maintanence on PDCCH as PDSCH SLIV reference	Samsung

R1-2101178	Maintanence on SPS PDSCH	Samsung

R1-2101179	Draft CR on SPS release for PDSCH with aggregation	Samsung

R1-2101262	Corrections on PDCCH enhancements	Huawei, HiSilicon

R1-2101263	Remaining issues on UCI enhancements	Huawei, HiSilicon

R1-2101264	PHY prioritization/multiplexing with consideration of UL skipping	Huawei, BUPT, China Southern Power Grid, HiSilicon

R1-2101347	Remaining issues on UCI and PUSCH enhancements for eURLLC	Apple

R1-2101348	PUSCH skipping considering physical layer priority & LCH based prioritization	Apple

R1-2101439	Remaining issue on HARQ and scheduling for URLLC	Qualcomm Incorporated

R1-2101440	Remaining issues on eCG enhancements for URLLC	Qualcomm Incorporated

R1-2101535	Correction on inconsistence between TS 38.213 and TS 38.331 in terms of the dci-FormatsExt for NR URLLC	Sharp

R1-2101536	Remaining issue on PDSCH rate matching for DCI format 1_0 for NR URLLC	Sharp

R1-2101584	Corrections on UCI enhancement for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2101585	Corrections on scheduling/HARQ for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2101586	Corrections on enhanced configured grant transmission for Rel-16 URLLC	NTT DOCOMO, INC.

[104-e-NR-L1enh-URLLC-01] Email discussion/approval on remaining issues on PDCCH enhancements – Chengyan (Huawei) by Feb 3

- Issue A-1: Correction on dci-FormatsExt in section 10.1 in TS 38.213

- Issue A-5: PDSCH resource mapping with RE symbol level granularity

[104-e-NR-L1enh-URLLC-02] Email discussion/approval on remaining issues on UCI enhancements – Jia (OPPO) by Feb 3

- Issue 1: Timing for secondary cell activation / deactivation

- Issue 2: Limitation on the number of PUCCHs carrying HARQ-ACK in a slot/subslot

- Issue 3: Conflict between the first PUCCH repetition and semi-static configuration

- Issue 5: Correction for sub-slot based PUCCH

[104-e-NR-L1enh-URLLC-03] Email discussion/approval on remaining issues on PUSCH enhancements – Sigen (Apple) by Feb 3

- Issue 1: New RRC parameter for TDRA indication to support up to 64 entries in a TDRA table for Type 1 configured grant with PUSCH repetition Type B

- Issue 2: Part 2 CSI dropping for UCI multiplexing on PUSCH repetition Type B

[104-e-NR-L1enh-URLLC-04] Email discussion/approval on remaining issues on Scheduling & HARQ enhancements – Kianoush (Qualcomm) by Feb 3

Issue 1: Correction on intra-UE prioritization timeline by replacing “before the first overlapping symbol” with “no later than the first overlapping symbol”

- Issue 2: Prioritization due to collision with semi-static DL and SSB symbols

- Issue 4: Active duration of CSI-RS resources in case of cancellation

- Issue 5: Including the agreement that any HP DCI can cancel a LP transmission

[104-e-NR-L1enh-URLLC-05] Email discussion/approval on remaining issues on SPS enhancements – Duckhyun (LG) by Feb 3

- Issue 3: SPS PDSCH release and SPS receptions with slot aggregation

- Issue 4: PUCCH resource for SPS PDSCH HARQ-ACK and SR

[104-e-NR-L1enh-URLLC-06] Email discussion/approval on remaining issues on intra-UE prioritization  – Lihui (Vivo) by Feb 3

- Issue 1: PHY behavior for collision between CG and DG with same/different PHY-priority index

- Reply LS to R1-2100026 on overlapped data and SR with equal L1 priority for Rel-16 URLLC

### Maintenance of Enhancements on MIMO for NR (5)

For this agenda, no more than 3 contribution per company/organization/university

R1-2101181	Summary for Rel.16 NR eMIMO maintenance	Moderator (Samsung)

R1-2100113	Text proposals for default close loop index for different TRPs	OPPO

R1-2100114	Text proposals for overlapping between uplink signals	OPPO

R1-2100115	Text Proposals for Multi-beam Operation Enhancement	OPPO

R1-2100279	Maintenance of multi-beam operation	ZTE

R1-2100280	Draft CR on UL full power transmission Mode 1	ZTE

R1-2100281	Maintenance of Multi-TRP enhancements	ZTE

R1-2100298	Maintenance on multi-TRP transmission	Lenovo, Motorola Mobility

R1-2100339	Corrections to multi-beam operation	CATT

R1-2100340	On remaining issues of multi-TRP/panel transmission	CATT

R1-2100416	Maintenance on multi beam related issues	vivo

R1-2100417	Maintenance on multi TRP	vivo

R1-2100418	Maintenance on default QCL issues and multi TRP	vivo

R1-2100617	Draft CR on multi-TRP/panel transmission	LG Electronics

R1-2100633	Correction for multi TRP PDCCH prioritization	Intel Corporation

R1-2100634	Draft CR for clarification of scheme 2a/2b	Intel Corporation

R1-2101180	On maintenance of Rel.16 multi-beam operation	Samsung

R1-2101182	On Rel.16 multi-TRP/panel transmission	Samsung

R1-2101183	On UL full power transmission	Samsung

R1-2101256	Remaining issues for multi-TRP in Rel-16	Huawei, HiSilicon

R1-2101271	Corrections on Type II CSI reporting in Rel-16	Huawei, HiSilicon

R1-2101272	Corrections on Scell BFR in Rel-16	Huawei, HiSilicon

R1-2101349	Remaining issues on Rel-16 Multi-TRP enhancement	Apple

R1-2101441	Remaining Issues on Multi-TRP Enhancements	Qualcomm Incorporated

R1-2101565	Interoperation between cross-carrier scheduling and multiple TRPs	ASUSTeK

R1-2101688	Draft CR on DL SPS based PDSCH repetitions	Ericsson

R1-2101690	Maintenance for multi-DCI based multi-TRP in Rel-16	Ericsson

R1-2101691	Maintenance for single-DCI based multi-TRP in Rel-16	Ericsson

[104-e-NR-eMIMO-01] Maintenance and TPs for Multi-beam – Yushu (Apple) by Jan 29

- Addressing E-rated issues MB.9, MB.10, and H-rated issue MB.7

[104-e-NR-eMIMO-02] Maintenance for Multi-TRP 1 – Li (OPPO) by Jan 29

- Addressing H-rated issue MT.1

[104-e-NR-eMIMO-03] Maintenance for Multi-TRP 2 – Li (OPPO) by Jan 29

- Addressing H-rated issue MT.4 (only issue 2)

[104-e-NR-eMIMO-04] Maintenance for Multi-TRP 3 – Li (OPPO) by Jan 29

- Addressing H-rated issue MT.12

[104-e-NR-eMIMO-05] Email discussion/approval of a potential reply LS to R1-2100018 – Karri (Nokia) by Jan 27

### Maintenance of UE Power Saving for NR (1)

[104-e-NR_UE_Pow_Sav-01] Corrections for the following issues (as identified in the summary) till 1/29 – Fangchen (CATT)

- Issue 1 and Issue 2-1, 2-2, 2-3 and 2-4.

Update on 1/29:

Conclusion:  There is no consensus on change in Issue 1: Correction on only one DRX group configured when DCI format 2_6 and/or SCell dormancy is configured.

Conclusion:  Editorial changes and associated TPs of Issue 2-1, 2-2, 2-3, 2-4 are agreed (to be included in the alignment CRs)

- Issue 2-1:  Suggest Editor to change the ‘The minimum scheduling restriction’ to ‘minimum scheduling offset restriction’ in section 6.1.2.1.1 of TS 38.214.[2]

- Issue 2-2: Remove the postfix ‘-r16’ of parameter minimumSchedulingOffsetK0 in section 5.2.1.5.1a  of TS38.214.[2]

- Issue 2-3:  Add “DRX-config” in Clause 5.2.2.5 CSI reference resource definition of TS38.214 [3]

- Issue 2-4: Change “Size-DCI-2-6” to “SizeDCI-2-6” in Clause 10.3 of TS38.213 [3]

R1-2100341	Draft TS38.213 CR on UE behavior of DRX group configuration	CATT

R1-2101258	Remaining issues for Rel-16 UE power saving	Huawei, HiSilicon

R1-2101552	Maintenance for Rel-16 UE power savings	Ericsson

### Maintenance of NR positioning support (4)

R1-2100127	Text Proposals on NR Positioning	OPPO

R1-2100282	Maintenance of NR positioning support	ZTE

R1-2100342	Discussion and TP on remaining issues in NR positioning	CATT

R1-2100419	Maintenance on Rel-16 NR positioning	vivo

R1-2100552	Draft CR on the usage of the term cell	Nokia, Nokia Shanghai Bell

R1-2100707	Editorial CR on Rel-16 NR positioning	LG Electronics

R1-2101731	Corrections to positioning SRS and higher layer parameters	Huawei, HiSilicon

R1-2101758	Maintenance of NR positioning support	Ericsson

[104-e-NR-Pos-01] Email discussion/approval on DL PRS until Jan-29 – Alexey (Intel)

- Aspect 2.4 – Change of Cell on DL PRS ID

- Aspect 2.7 – Misalignment of ‘nr-TimeStamp’ with TS 37.355

- Aspect 2.8 – Ambiguity for Measurement Gap Request

- Aspect 2.9 – DL PRS Resource / Resource Set IDs reporting for DL-AOD

[104-e-NR-Pos-02] Email discussion/approval on SRS for positioning until Jan-29 – Florent (Ericsson)

- Aspect 2.2 – Semi-persistent SRS for Positioning Activation

- Aspect 2.10 – Editorial Corrections for the TS 38.214

- Aspect 2.11 – Misalignment of dl-PRS-r16 in spatialRelationInfoPos

- Aspect 2.12 – Corrections to Positioning SRS and Higher Layer Parameters

[104-e-NR-Pos-03] Email discussion/approval for the reply LS to R1-2100005 until Jan-28 - Su Huang (Huawei)

[104-e-NR-Pos-04] Email discussion/approval for R1-2100320 until Jan-28 – Xiaotao Ren (CATT)

### Maintenance of NR Mobility Enhancements (0)

### Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (3)

R1-2100093	Maintenance of Rel-16 MR-DC and CA	ZTE

R1-2100420	Maintenance on MR-DC and CA enhancements	vivo

R1-2100584	Remaining issues on Rel-16 uplink power control for supporting NR-NR dual-connectivity			MediaTek Inc.

R1-2100585	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

R1-2101442	Remaining issue on NR-DC power-control	Qualcomm Incorporated

R1-2101443	Remaining issues on SCell dormancy and cross-carrier scheduling	Qualcomm Incorporated

R1-2101553	Maintenance for Rel-16 MR-DC and CA enhancements	Ericsson

R1-2101751	Corrections on SCell dormancy in TS 38.212	Huawei, HiSilicon

[104-e-NR-MRDC-CA-01] Email discussion/approval on power control for dual connectivity and SCell Dormancy until 10/29 – Ravi (Ericsson)

- PC-1: Proposal 1 in R1-2100420

- PC-2: R1-2100584

- Dorm-1: Proposal 1 in R1-2100093

- Dorm-4: R1-2101751

[104-e-NR-MRDC-CA-02] Email discussion/approval on cross-carrier triggering scheduling and A-CSI RS triggering, and unaligned CA until 10/29 – Karri (Nokia)

- XCC-1: Proposal 1 in R1-2100585

- XCC-2: Proposal 3 in R1-2100585

- XCC-3 (could be discussed with XCC-1): Proposal 2 in R1-2101443

- CA-1: Proposal (for conclusion) in R1-2101553 and Proposals 2 and 3 in R1-2100420

- CA-2: Proposals 4 and 5 in R1-2100420

[104-e-NR-MRDC-CA-03] Email discussion/approval of a potential reply LS to R1-2100013 until Jan-29 - James (MediaTek)

### NR Rel-16 UE Features (9)

R1-2101588	Summary on UE features for two-step RACH	Moderator (NTT DOCOMO, INC.)

R1-2101589	Summary on UE features for NR-U	Moderator (NTT DOCOMO, INC.)

R1-2101590	Summary on UE features for URLLC/IIoT	Moderator (NTT DOCOMO, INC.)

R1-2101591	Summary on UE features for NR positioning	Moderator (NTT DOCOMO, INC.)

R1-2101592	Summary on UE features for MR-DC/CA	Moderator (NTT DOCOMO, INC.)

R1-2101593	Summary on UE features for CLI/RIM	Moderator (NTT DOCOMO, INC.)

R1-2101594	Summary on UE features for TEIs	Moderator (NTT DOCOMO, INC.)

R1-2101595	Summary on NR UE features for others	Moderator (NTT DOCOMO, INC.)

[104-e-NR-UEFeatures-LS] Email approval of updated Rel-16 NR UE features by 2/4 – Hiroki (DCM)

Update on 2/3: the latest updated feature list (R1-2102006) is agreed. The draft LS is approved, with final LS in R1-2102007.

Notes:

- There will not be any dedicated email discussions/approvals for 5G V2X Rel. 16 NR UE features during RAN1 #104-e. Editorial changes such as corrections of typos can be handled in email discussion/approval [104-e-NR-UEFeatures-LS]

Mobility Enhancements:

[104-e-NR-UEFeatures-MobEnh-01] Email discussion/approval of whether to change the prerequisites of FGs 21-2, 21-2a, 21-2b to include FG 21-1b and update the description with “for inter-frequency DAPS HO” , till 1/29 (Ralf, AT&T)

- Any necessary alignments between RAN1 and RAN2 (e.g., changing the type of FG 21-1a to “Per Band/per BC”) can be handled in email discussion/approval [104-e-NR-UEFeatures-LS]

Update on 1/28:

Agreement:

| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO - Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO - Support of PDCCH blind decoding capability in the first MCG and second MCG. - Support of cancelling UL transmission to the source cell for intra-frequency DAPS-HO | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with intra-frequency DAPS HO | Per Band per FS | No | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with inter-frequency DAPS HO | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| 21-2 | Semi-static UL power sharing mode 1 for DAPS HO | Support of semi-static power sharing mode1 between source and target cells of same FR for inter-frequency DAPS HO | DAPS, 21-1b (Note: RAN2 feature) | Yes | N/A | UE is not expected to simultaneously transmit PRACH/PUSCH/PUCCH/SRS to source and target cell that overlap in time domain | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 between source and target cells of same FR for inter-frequency DAPS HO | 21-2, 21-1b | Yes | N/A |  | Per BC | No | N/A | N/A | only applicable to DAPS HO in synchronous scenarios | Optional with capability signalling |
| 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing between source and target cells of same FR for inter-frequency DAPS HO 1) T_offset | 21-2, 21-1b | Yes | N/A |  | Per BC | No | N/A | N/A | Candidate values for (1) are {short, long} | Optional with capability signalling |
| 21-2d | UL transmission cancellation | Indicates support of cancelling UL transmission to the source cell for inter-frequency DAPS-HO | 21-1b | Yes | N/A | UE does not support scheduling of overlapping PUSCH/PUCCH/SRS transmissions to source and target cells for inter-frequency DAPS-HO | per band combination | No | N/A | N/A |  | Optional with capability signalling |

eMIMO:

[104-e-NR-UEFeatures-eMIMO-01] Email discussion/approval of whether/how to address that UEs that support multi-DCI multi-TRP are not mandated to support PDSCH retransmission scheduled by different TRP, till 1/29 (Ralf, AT&T)

Update on 1/28:

Agreement: The following new feature group is introduced

| 16-2a-8 | Indicates that retransmission scheduled by a different CORESETPoolIndex for multi-DCI multi-TRP is not supported. | For multi-DCI multi-TRP operation, if this FG is indicated, UE does not support retransmission scheduled by PDCCH received in a different CORESETPoolIndex compared to the CORESETPoolIndex of the initial transmission, i.e., the UE is not expected to receive, for the same HARQ process ID, DCI from a different CORESETPoolIndex that schedules the retransmission, i.e., NDI not flipped. This applies to both PDSCH and PUSCH retransmissions. | 16-2a | Yes | N/A |  | Per UE | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[104-e-NR-UEFeatures-eMIMO-02] Email discussion/approval of whether/how to capture that the maximum number of configured PUCCH spatial relations and candidate beams in BFR can be up to 64, till 1/29 (Ralf, AT&T)

Update on 2/1:

Agreement: The following new feature groups are introduced

| 16-1h | Support of 64 configured PUCCH spatial relations | 1. Support of configuring maximum 64 PUCCH spatial relations per BWP per CC 2. Maximum number of configured spatial relations per CC for PUCCH and SRS | 2-59 | Yes | N/A |  | Per band | No | FR2 only |  | Component 12: Candidate value set {96, 128, 160, 192, 224, 256, 288, 320} Note: if component 12 is reported, UE shall report 96 in FG 2-59 and the UE may assume that the value reported in FG 2-59 is used by Rel-15 gNB and ignored by Rel-16 gNB. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1I | Support of 64 configured candidate beam RSs for PCell/PSCell BFR | 1. Support of configuring maximum 64 candidate beam RSs per BWP per CC | 2-31 | Yes | N/A |  | Per band | No | No |  |  | Optional with capability signaling |

[104-e-NR-UEFeatures-eMIMO-03] Email discussion/approval of how to interpret FG 16-2c “Simultaneous reception with different Type-D” and whether changes are needed, till 1/29 (Ralf, AT&T)

Update on 1/28: email thread is closed without further agreements.

[104-e-NR-UEFeatures-eMIMO-04] Email discussion/approval of whether/how Section 5.2.1.6 in 38.214 applies to FGs 16-1a-1 and 16-1g, till 1/29 (Ralf, AT&T)

Agreement: The following changes highlighted in red are adopted

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of SSB/CSI-RS (1Tx) for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of CSI-IM/NZP IMR resources Other limitations: - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements | 2-21, 2-22 or 2-23, 2-23a | Yes | N/A |  | Per band | No | No |  | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {8, 16, 32, 64} Component 3: Candidate values {0, 4, 8, 16, 32, 64} Component 4: Candidate values {8, 16, 32, 64 , 128} Component 5: Candidate values {8, 16, 32, 64 , 128} Component 6: Candidate values {‘1 only’, ‘3 only’, ‘1 and 3’} Component 7: Candidate values {2, 4, 8, 16, 32, 64} Component 8: Candidate values: bitmap with entries {SSB as CMR with dedicated CSI-IM, SSB as CMR with dedicated NZP IMR, CSI-RS as CMR with dedicated NZP IMR configured, CSI-RS as CMR without dedicated IMR configured} If a UE supports FG 16-1a-1 it must support CMR(CSI-RS) + dedicated CSI-IM Note1: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note2: For component 4 and 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note3: For components 1, 2 and 3, CSI-RS resources configured as CMR without dedicated IMR are counted both as CMR and IMR Note4: For components 1, 2, 3, 7, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: The following changes highlighted in red are adopted

| 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31 | Yes | N/A |  | Per UE | No | Yes |  | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: For RS configured for new beam identification, they are always counted regardless of beam failure event Note: The “configure to measure” RS (component1) only counts those in active BWP but the configured RS (component2) counts all configured including both active and inactive BWP Note: the reference slot duration is the shortest slot duration defined for the reported FR supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: The following changes highlighted in red are adopted

| 16-1g-1 | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31, 16-1g | Yes | N/A |  | Per UE | No | No |  | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: This FG indicates the maximum number of resources across all FR(s) that are supported by the UE Note: The signalled values apply to the shortest slot duration defined in any FR(s) that are supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

NR-U

[104-e-NR-UEFeature-NRU-01] Email discussion/approval on UE features for NR-U (25th Jan – 29th Jan) – Hiroki (DCM)

- Whether or not to ask RAN2 to update the optionality of FG10-2f

- Whether or not to clarify that a UE indicating no support of FG 10-26a, also indicates that none of Rel-15 FG 1-5/1-5a/1-6/1-7/1-8/1-9 are supported by the UE in unlicensed bands

- Whether or not to clarify that Rel-15 FGs 1-4 applies to licensed band operation only

- How to reply to RAN4 on RAN1’s understanding on RB set of NR-U

Update on 1/31 – email thread is closed. Agreements are captured in the updated spreadsheet. Summary of the discussion in R1-2101832.

URLLC/IIoT

[104-e-NR-UEFeature-URLLCIIoT-01] Email discussion/approval on UE features for NR URLLC and IIoT (25th Jan – 29th Jan) – Hiroki (DCM)

- Whether or not to confirm working assumption to add the replicated FGs of 11-2a/c with restriction for non-aligned span case

- Whether or not to add the replicated FGs of 11-2d/e with restriction for non-aligned span case

- Whether/how to clarify the interpretation of FG11-7b/9/9a and FG12-2/2a in case of cross-carrier operation (interpretation 1 or 3)

- Whether/how to clarify the relationship between FG11-4/4a and FG12-1

Update on 2/2 - email thread is closed. Agreements are captured in the updated spreadsheet.

Others

[104-e-NR-UEFeature-Others-01] Email discussion/approval on NR UE features for others (25th Jan – 29th Jan) – Hiroki (DCM)

- Regarding FG22-6/6a/7

  - Whether or not to add replicated FGs 6-[8/]9/9a to be reported with FG22-7

  - Whether or not to update how to handle SDL/SUL

  - Whether or not to confirm the working assumption on how to count SUL

  - Whether/how to cover the case where the NUL and the SUL are in different FRs or in different licensed/unlicensed types

- Whether/how to update the prerequisite of FG22-8a/b/c/d

- Regarding licensed/unlicensed differentiation for Rel-15 FGs

  - Confirm the FG descriptions of new FGs to indicate the support of following FG in unlicensed band (as agreed in RAN1#103-e)

    - FG 1-2 (SS block based SINR measurement (SS-SINR))

    - FG 2-32a/2-32b (Semi-persistent CSI report on PUCCH/PUSCH)

    - FG 3-6 (Dynamic SFI monitoring)

    - FG 4-19a/4-19b/4-19c/4-28 (HARQ-ACK multiplexing)

    - FG 4-23 (Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8)

    - FG 5-14/5-16/5-17/5-17a (PDSCH and PUSCH repetitions)

  - Whether or not to add new FG(s) to indicate the support of following FG in unlicensed band

    - [FG 4-19]

    - [FG 5-18/5-19/5-20/5-21 (SPS and configured grant)]

  - Whether each of Rel-16 versions of 4-19/4-23/4-28/5-17 is part of basic operation for corresponding scenarios of NR-U

  - Whether/how to clarify the interpretation of support of FG in case of cross-carrier operation between licensed and unlicensed carriers

Update on 2/3 - email thread is closed. Agreements are captured in the updated spreadsheet.

LSs:

[104-e-NR-UEFeature-LS-01] Email discussion/approval of the reply LS to R1-2100003, to be handled under 7.2.11 (name TBD, CMCC)

Handled by [104-e-NR-UEFeature-NRU-01]

[104-e-NR-UEFeature-LS-02] Email discussion regarding LS in R1-2100019, including a possible reply LS if necessary, to be handled under 7.2.11 (name TBD, Qualcomm)

Handled by [104-e-NR-UEFeature-NRU-01]

R1-2100094	Discussion on NR Rel-16 UE Features	ZTE

R1-2100140	Correction for V2X UE feature list	OPPO

R1-2100522	Remaining details of Rel-16 NR UE features	Ericsson

R1-2100554	Discussion on NR Rel-16 UE features	LG Electronics

R1-2100635	Remaining issue on UE features	Intel Corporation

R1-2101184	On NR Rel.16 UE features	Samsung

R1-2101249	Updates on NR UE Features	Nokia, Nokia Shanghai Bell

R1-2101273	Remaining details of Rel-16 NR UE features	Huawei, HiSilicon

R1-2101342	Discussions on NR Rel-16 UE features	Apple

R1-2101343	Discussion on Rel-16 UE features for mobility enhancement	Apple

Withdrawn

R1-2101444	Discussion on NR Rel-16 UE features	Qualcomm Incorporated

R1-2101517	Correction on half-DuplexTDD-CA-SameSCS-r16	CATT

R1-2101587	Remaining issues on Rel-16 NR UE features	NTT DOCOMO, INC.

R1-2101685	Remaining issues on Rel-16 eMIMO UE features	vivo

### Others (2)

Including any essential TPs related to the endorsed Rel-16 NR TEIs, Rel-16 RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) (used to be under 5.1)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2101596	Summary on Rel-16 NR TEI related discussion	Moderator (NTT DOCOMO, INC.)

[104-e-NR-TEI-01] Email discussion/approval on Rel-16 NR TEI related issues (29th Jan – 3rd Feb)

- Whether/how to adopt CR on half-duplex operation in CA with unpaired spectrum with considering R1-2100023

Summary of TxSwitching preparation phase in R1-2001782

[104-e-NR-TxSwitching-01] Email discussion/approval on corrections regarding – till 1/29, Jianchi (China Telecom)

- Issue#1: Clarification on the ambiguity issue on SCS in TS 38.214

- Issue#2: Align the RRC parameters in TS 38.214 with TS 38.331

- Issue#3: Clarification on the Maximum data rate in TS 38.306

- Issue#4: Clarification of Tswitch in TS 38.213

- Issue#6: Clarification on the state of Tx chains for SRS antenna switching

and how to handle the following proposal (as a conclusion in Chairman’s notes?)

- For Rel-16 inter-band UL CA, SUL and EN-DC, if uplink Tx switching is configured on two uplinks, the case where SRS carrier switching is configured on a third uplink is not supported.

  - No spec impact.

Update on 1/31:

Agreements:

Other

Other proposals? 2/1

Update on 2/1:

Agreements:

- Adopt the following TP to TS 38.306

4.1.2 Supported max data rate < unchanged text omitted> NOTE 2: For UL Tx switching between carriers in cell(s), only the supported MIMO layer combination across carriers that results in the highest combined data rate is counted for the carriers cell(s) in the supported maximum UL data rate. < unchanged text omitted>

Other proposals? 2/2

Update on 2/2

Agreements:

Adopt the following TP to TS 38.214

6.1.6 Uplink switching < unchanged text omitted> The UE does not expect to perform more than one uplink switching in a slot with µUL = max (µUL,1, µUL,2), where the µUL,1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier transmitted before the switching gap and the µUL,2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier transmitted after the switching gap. < unchanged text omitted>

Draft CRs and LS? 2/3

- The two draft CRs are approved. Final CRs

  - R1-210xxxx (38.213, CRxxxx)

  - R1-210xxxx (38.214, CRxxxx)

- The draft LS is approved. Final LS in R1-210xxxx

Other issues? 2/4

R1-2100087	Remaining issues of Rel-16 UL Tx Switching	ZTE

R1-2100116	Enhancement on uplink power control for M-TRP	OPPO

R1-2100117	Text Proposals for Tx Switching between Two Uplink Carriers	OPPO

R1-2100330	Correction on half-duplex operation in CA with unpaired spectrum	CATT

R1-2100505	DRAFT38.213CR, Correction to the configuration of half duplex operation for unpaired spectrum in CA	Nokia, Nokia Shanghai Bell

R1-2100700	Draft TP for half-duplex operation	ZTE

R1-2101162	Draft CR on correction of RRC parameter name	vivo

R1-2101445	Remaining issues for 1Tx-2Tx switching	Qualcomm Incorporated

R1-2101554	Maintenance for Rel-16 UL Tx Switching	Ericsson

R1-2101738	Discussion on the remaining problems of supporting Tx switching between two uplink carriers			Huawei, HiSilicon

R1-2101752	Corrections on CCS with different subcarrier spacings for PDCCH and PDSCH in TS 38.214			Huawei, HiSilicon

# Release 17

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”.

## Further enhancements on MIMO for NR (8)

Please refer to RP-202024 for detailed scope of the WI

### Enhancements on Multi-beam Operation

Mainly targeting FR2 while also applicable to FR1

R1-2101185	Moderator summary for multi-beam enhancement	Moderator (Samsung)

R1-2100044	Enhancement on multi-beam operation	FUTUREWEI

R1-2100063	Discussions on Rel-17 Beam Management	InterDigital, Inc.

R1-2100118	Enhancements on Multi-beam Operation	OPPO

R1-2100208	Enhancements on multi-beam operation	Huawei, HiSilicon

R1-2100273	Enhancements on Multi-beam Operation	Lenovo, Motorola Mobility

R1-2100285	Enhancements on Multi-beam Operation	ZTE

R1-2100343	Enhancements on multi-beam operation	CATT

R1-2100421	Further discussion on multi beam enhancement	vivo

R1-2100534	Enhancements on multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

R1-2100588	Enhancement on multi-beam operation	MediaTek Inc.

R1-2100618	Enhancements on Multi-beam Operation	LG Electronics

R1-2100636	Enhancements to Multi-Beam Operations	Intel Corporation

R1-2100737	Enhancements on Multi-beam Operation	Fujitsu

R1-2100779	Enhancements on multi-beam operations	AT&T

R1-2100783	Enhancements on Multi-beam Operation	Spreadtrum Communications

R1-2100844	Further enhancement on multi-beam operation	Sony

R1-2100949	Discussion on multi-beam operation	NEC

R1-2100964	Discussion on Enhancements for Multi-beam Operation	Asia Pacific Telecom, FGI

R1-2101005	Enhancements on Multi-beam Operation	Nokia, Nokia Shanghai Bell

R1-2101023	Discussion on multi-beam operation	ASUSTeK

R1-2101032	Enhancements on multi-beam operation	CMCC

R1-2101092	Enhancements on multi-beam operation	Xiaomi

R1-2101186	Multi-Beam Enhancements	Samsung

R1-2101313	Enhancements on Multi-beam Operation	Ericsson

R1-2101350	Views on Rel-17 Beam Management enhancement	Apple

R1-2101414	Multi-beam Enhancements	Convida Wireless

R1-2101446	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2101597	Discussion on multi-beam operation	NTT DOCOMO, INC.

R1-2101644	Enhancements on Multi-Beam Operation	TCL Communication Ltd

[104-e-NR-feMIMO-01] Email discussion on enhancements on multi-beam operation – Eko (Samsung)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Enhancements for Multi-TRP Deployment

#### Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

R1-2100038	Multi-TRP/panel for non-PDSCH	FUTUREWEI

R1-2100064	Discussion on M-TRP Enhancements for PDCCH, PUCCH, and PUSCH	InterDigital, Inc.

R1-2100119	Enhancements on Multi-TRP based enhancement for PDCCH, PUCCH and PUSCH	OPPO

R1-2100209	Enhancements on multi-TRP for reliability and robustness in Rel-17	Huawei, HiSilicon

R1-2100274	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Lenovo, Motorola Mobility

R1-2100286	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	ZTE

R1-2100344	Enhancements on multi-TRP/panel for PDCCH, PUCCH and PUSCH	CATT

R1-2100422	Further discussion on enhancement of MTRP operation	vivo

R1-2100535	On multi-TRP enhancements for PDCCH and PUSCH	Fraunhofer IIS, Fraunhofer HHI

R1-2100582	Enhancements on Multi-TRP for PDCCH, PUSCH and PUCCH	MediaTek Inc.

R1-2100619	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	LG Electronics

R1-2100637	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	Intel Corporation

R1-2100738	Enhancements on Multi-TRP for PDCCH PUCCH and PUSCH	Fujitsu

R1-2100784	Discussion on enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Spreadtrum Communications

R1-2100845	Considerations on Multi-TRP for PDCCH, PUCCH, PUSCH	Sony

R1-2100950	Discussion on multi-TRP for PDCCH, PUCCH and PUSCH	NEC

R1-2100965	Discussion on Enhancements on Multi-TRP for Uplink Channels	Asia Pacific Telecom, FGI

R1-2101006	Enhancements for Multi-TRP URLLC schemes	Nokia, Nokia Shanghai Bell

R1-2101033	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	CMCC

R1-2101093	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Xiaomi

R1-2101187	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Samsung

R1-2101351	Views on Rel-17 multi-TRP reliability enhancement	Apple

R1-2101415	Multi-TRP Enhancements for PDCCH, PUCCH and PUSCH	Convida Wireless

R1-2101447	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

R1-2101537	Enhancements on Multi-TRP PUSCH	Sharp

R1-2101598	Discussion on MTRP for reliability	NTT DOCOMO, INC.

R1-2101653	Discussion on enhancement on Multi-TRP PDCCH	ASUSTeK

R1-2101654	On PDCCH, PUCCH and PUSCH enhancements	Ericsson

R1-2101662	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	TCL Communication Ltd.

[104-e-NR-feMIMO-02] Email discussion on enhancements on multi-TRP for PDCCH – Mostafa (Qualcomm)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

[104-e-NR-feMIMO-03] Email discussion on enhancements on multi-TRP for PUSCH, PUCCH – Keeth (Nokia)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

#### Enhancements on Multi-TRP inter-cell operation

R1-2100039	Clarification on network synchronization for inter-cell multi-TRP operation	FUTUREWEI, InterDigital

R1-2100065	Synchronization Analysis for M-TRP Inter-cell Operation and RRC Configurations	InterDigital, Inc.

R1-2100120	Enhancement on inter-cell multi-TRP operation	OPPO

R1-2100210	Enhancements on inter-cell multi-TRP operations in Rel-17	Huawei, HiSilicon

R1-2100275	Enhancements on Multi-TRP inter-cell operation	Lenovo, Motorola Mobility

R1-2100287	Discussion on Multi-TRP inter-cell operation	ZTE

R1-2100345	Inter-cell operation for multi-TRP/panel	CATT

R1-2100423	Further discussion on inter-cell MTRP operation	vivo

R1-2100620	Enhancements on Multi-TRP inter-cell operation	LG Electronics

R1-2100638	Multi-TRP enhancements for inter-cell operation	Intel Corporation

R1-2100785	Discussion on enhancement multi-TRP inter-cell operation	Spreadtrum Communications

R1-2100846	Considerations on inter-cell operation	Sony

R1-2100966	Discussion of enhancements on Multi-TRP inter-cell operation	Asia Pacific Telecom, FGI

R1-2101007	Enhancements to enable inter-cell multi-TRP operations	Nokia, Nokia Shanghai Bell

R1-2101034	Enhancements on Multi-TRP inter-cell operation	CMCC

R1-2101094	Enhancement on Inter-cell Multi-TRP operations	Xiaomi

R1-2101144	Enhancement on Multi-TRP inter-cell operation	Ericsson

R1-2101188	Enhancements on Multi-TRP inter-cell operation	Samsung

R1-2101352	Views on Rel-17 Inter-cell multi-TRP operation	Apple

R1-2101448	Enhancements on Multi-TRP inter-cell operation	Qualcomm Incorporated

R1-2101599	Discussion on inter-cell multi-TRP operations	NTT DOCOMO, INC.

[104-e-NR-feMIMO-04] Email discussion on enhancements on multi-TRP intercell operation – Rakesh (vivo)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

#### Enhancements on beam management for multi-TRP

R1-2100040	Beam management for simultaneous multi-TRP transmission with multi-panel reception				FUTUREWEI

R1-2100066	Discussion on M-TRP Beam Management Enhancements	InterDigital, Inc.

R1-2100121	Enhancements on beam management for multi-TRP	OPPO

R1-2100211	Multi-panel reception for multi-TRP in Rel-17	Huawei, HiSilicon

R1-2100276	Enhancements on beam management for multi-TRP	Lenovo, Motorola Mobility

R1-2100288	Enhancements on beam management for Multi-TRP	ZTE

R1-2100346	Enhancements on beam management for multi-TRP	CATT

R1-2100424	Further discussion on MTRP multibeam enhancement	vivo

R1-2100589	Enhancements on beam management for multi-TRP	MediaTek Inc.

R1-2100621	Enhancements on beam management for multi-TRP	LG Electronics

R1-2100639	Multi-TRP enhancements for beam management	Intel Corporation

R1-2100739	Enhancements on beam management for multi-TRP	Fujitsu

R1-2100780	Enhancements on beam management for multi-TRP	AT&T

R1-2100786	Discussion on enhancements on beam management for multi-TRP	Spreadtrum Communications

R1-2100847	Considerations on beam management for multi-TRP	Sony

R1-2100951	Discussion on beam management for multi-TRP	NEC

R1-2100967	Discussion of enhancements on beam management for Multi-TRP	Asia Pacific Telecom, FGI

R1-2101008	Enhancements on Beam Management for Multi-TRP/Panel Transmission	Nokia, Nokia Shanghai Bell

R1-2101026	Discussion on beam management for multi-TRP	ASUSTeK

R1-2101035	Enhancements on beam management for multi-TRP	CMCC

R1-2101074	Enhancements on beam management for multi-TRP	ETRI

R1-2101095	Enhancement on beam management for Multi-TRP	Xiaomi

R1-2101189	Enhancements on beam management for multi-TRP	Samsung

R1-2101353	Views on Rel-17 multi-TRP BM enhancement	Apple

R1-2101416	On Per-TRP Beam Failure Recovery	Convida Wireless

R1-2101449	Enhancements on beam management for multi-TRP	Qualcomm Incorporated

R1-2101568	Discussion on beam management for multi-TRP	ITRI

R1-2101600	Discussion on beam management for MTRP	NTT DOCOMO, INC.

R1-2101686	On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception	Ericsson

[104-e-NR-feMIMO-05] Email discussion on enhancements on beam management for multi-TRP – Runhua (CATT)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

#### Enhancements on HST-SFN deployment

R1-2100041	Enhancement to support HST-SFN deployment scenario	FUTUREWEI

R1-2100067	Enhancements for M-TRP Transmission to Support HST-SFN in Rel-17	InterDigital, Inc.

R1-2100122	Enhancements on HST-SFN deployment	OPPO

R1-2100212	Enhancements for high speed train for multi-TRP in Rel-17	Huawei, HiSilicon

R1-2100289	Discussion on Multi-TRP HST enhancements	ZTE

R1-2100347	Discussion on enhancements for HST-SFN deployment	CATT

R1-2100425	Further discussion and evaluation on HST-SFN schemes	vivo

R1-2100622	Enhancements on HST-SFN deployment	LG Electronics

R1-2100640	Enhancements to HST-SFN deployments	Intel Corporation

R1-2100787	Discussion on enhancements on HST-SFN deployment	Spreadtrum Communications

R1-2100848	Considerations on HST-SFN operation for multi-TRP	Sony

R1-2100952	Discussion on HST-SFN deployment	NEC

R1-2100988	Enhancements for HST-SFN deployment	Lenovo, Motorola Mobility

R1-2101009	Enhancements for HST-SFN deployment	Nokia, Nokia Shanghai Bell

R1-2101036	Enhancements on HST-SFN deployment	CMCC

R1-2101143	Enhancement on HST-SFN deployment	Ericsson

R1-2101190	Enhancements on HST-SFN	Samsung

R1-2101354	Views on Rel-17 HST enhancement	Apple

R1-2101450	Enhancements on HST-SFN deployment	Qualcomm Incorporated

R1-2101601	Discussion on HST-SFN deployment	NTT DOCOMO, INC.

[104-e-NR-feMIMO-06] Email discussion on enhancements on HST-SFN deployment – Alexei (Intel)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Enhancements on SRS flexibility, coverage and capacity

R1-2100042	Enhancements on SRS flexibility, coverage and capacity	FUTUREWEI

R1-2100068	Flexible SRS Transmission and Antenna Switching	InterDigital, Inc.

R1-2100123	Enhancements on SRS flexibility, coverage and capacity	OPPO

R1-2100213	Enhancements on SRS for Rel-17	Huawei, HiSilicon

R1-2100277	Enhancements on SRS	Lenovo, Motorola Mobility

R1-2100290	Enhancements on SRS flexibility, coverage and capacity	ZTE

R1-2100348	Discussion on SRS enhancement for Rel-17	CATT

R1-2100426	Further discussion on SRS enhancement	vivo

R1-2100590	Enhancements on SRS flexibility, coverage and capacity	MediaTek Inc.

R1-2100623	Enhancements on SRS flexibility, coverage and capacity	LG Electronics

R1-2100641	Discussion on SRS enhancements	Intel Corporation

R1-2100788	Considerations on SRS enhancement	Spreadtrum Communications

R1-2100849	Considerations on SRS flexibility, coverage and capacity	Sony

R1-2100953	Discussion on SRS enhancement	NEC

R1-2101010	Enhancements on SRS flexibility, coverage and capacity	Nokia, Nokia Shanghai Bell

R1-2101037	Enhancements on SRS flexibility, coverage and capacity	CMCC

R1-2101096	Discussion on SRS enhancements	Xiaomi

R1-2101191	Enhancements on SRS	Samsung

R1-2101355	Views on Rel-17 SRS enhancement	Apple

R1-2101451	Enhancements on SRS flexibility, coverage and capacity	Qualcomm Incorporated

R1-2101519	SRS Performance and Potential Enhancements	Ericsson

R1-2101538	Enhancements on SRS flexibility, coverage and capacity	Sharp

R1-2101602	Discussion on SRS enhancement	NTT DOCOMO, INC.

R1-2101684	Enhancements on SRS for coverage and capacity 	Fraunhofer IIS, Fraunhofer HHI

[104-e-NR-feMIMO-07] Email discussion on SRS flexibility, coverage, and capacity – Hao (ZTE)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### CSI enhancements: MTRP and FR1 FDD reciprocity

R1-2100043	CSI enhancement for multi-TRP and FDD	FUTUREWEI

R1-2100069	CSI Enhancements for the Support of NCJT MTRP	InterDigital, Inc.

R1-2100124	CSI enhancement for M-TRP and FDD reciprocity	OPPO

R1-2100214	CSI Enhancements for Rel-17	Huawei, HiSilicon, China Unicom

R1-2100291	CSI enhancements for Multi-TRP and FR1 FDD reciprocity	ZTE

R1-2100349	Further discussion on CSI enhancements for Rel-17	CATT

R1-2100427	Further discussion and evaluation on MTRP CSI and Partial reciprocity	vivo

R1-2100536	CSI enhancements on Type II PS codebook and multi-TRP 	Fraunhofer IIS, Fraunhofer HHI

R1-2100583	CSI enhancement for NCJT and FR1 FDD reciprocity	MediaTek Inc.

R1-2100624	CSI enhancements for Rel-17	LG Electronics

R1-2100642	On CSI enhancements for MTRP and FDD	Intel Corporation

R1-2100789	Discussion on CSI enhancement for multi-TRP and FR1 FDD reciprocity	Spreadtrum Communications

R1-2100850	Further considerations on CSI enhancements	Sony

R1-2100954	Discussion on CSI enhancement for multi-TRP	NEC

R1-2100989	CSI enhancements for mTRP and FDD reciprocity	Lenovo, Motorola Mobility

R1-2101011	Enhancement on CSI measurement and reporting	Nokia, Nokia Shanghai Bell

R1-2101038	Enhancements on CSI reporting for Multi-TRP	CMCC

R1-2101192	Views on Rel. 17 CSI enhancements	Samsung

R1-2101356	Views on Rel-17 CSI enhancement	Apple

R1-2101452	CSI enhancements: MTRP and FR1 FDD reciprocity	Qualcomm Incorporated

R1-2101603	Discussion on CSI enhancements	NTT DOCOMO, INC.

R1-2101687	CSI enhancements for Multi-TRP and FR1 FDD reciprocity	Ericsson

[104-e-NR-feMIMO-08] Email discussion on CSI enhancements: MTRP and FR1 FDD reciprocity – Min (Huawei)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Others

R1-2100070	Further Results on Multi-Panel UE Performance in a Multi-TRP Deployment	InterDigital, Inc.

R1-2100278	HARQ feedback of SPS PDSCH reception in multi-DCI based multiple TRPs	Lenovo, Motorola Mobility

R1-2100292	Further details on Multi-beam and Multi-TRP operation	ZTE

R1-2100350	Evaluation of enhancements for multi-TRP	CATT

R1-2100428	Discussion on IRC receiver performance	vivo

R1-2100537	On FDD channel reciprocity in real-world scenarios	Fraunhofer IIS, Fraunhofer HHI

R1-2101193	Additional enhancements for multi-beam	Samsung

R1-2101274	Further details on CSI Enhancements for Rel-17	Huawei, HiSilicon, China Unicom

R1-2101318	Additional simulation results on multi-beam operation	Ericsson

R1-2101497	Further considerations for MIMO	Qualcomm Incorporated

Withdrawn

R1-2101604	Discussion on UL dense deployment	NTT DOCOMO, INC.

## Supporting NR from 52.6GHz to 71 GHz

Please refer to RP-202925  for detailed scope of the WI

### Initial access aspects

Including SSB, initial BWP, PRACH, etc.

R1-2100051	Considerations on initial access for additional SCS in Beyond 52.6GHz	FUTUREWEI

R1-2100057	Initial access enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2100073	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

R1-2100149	Discusson on initial access aspects	OPPO

R1-2100200	Initial access signals and channels for 52-71GHz band	Huawei, HiSilicon

R1-2100257	Initial access aspects	Nokia, Nokia Shanghai Bell

R1-2100299	Some views on initial access aspects for 52.6-71GHz	CAICT

R1-2100370	Initial access aspects for up to 71GHz operation	CATT

R1-2100429	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

R1-2100541	Initial access aspects	TCL Communication Ltd.

R1-2100607	Initial access aspects for NR operations in 52.6-71 GHz	MediaTek Inc.

R1-2100643	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

R1-2100740	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2100781	Further Discussion of Initial Access Aspects	AT&T

R1-2100825	Discussion on initial access aspects for NR from 52.6GHz to 71GHz	Spreadtrum Communications

R1-2100836	Discussions on initial access aspects	InterDigital, Inc.

R1-2100892	Initial access aspects to support NR above 52.6 GHz	LG Electronics

R1-2100939	Discussion on initial access aspects supporting NR from 52.6 to 71GHz	NEC

R1-2101109	On initial access aspects for NR from 52.6GHz to 71GHz	Xiaomi

R1-2101194	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101286	Discussion on Initial access aspects for NR beyond 52.6 GHz	CEWiT

R1-2101306	Initial Access Aspects	Ericsson

R1-2101372	On Initial access signals and channels	Apple

R1-2101417	Consideration for NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

R1-2101453	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2101605	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2101672	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

### PDCCH monitoring enhancements

R1-2100058	PDCCH monitoring enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2100074	Discussion on the PDCCH monitoring enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2100150	Discussion on PDCCH monitoring	OPPO

R1-2100241	Enhancement on PDCCH monitoring	Huawei, HiSilicon

R1-2100258	PDCCH monitoring enhancements	Nokia, Nokia Shanghai Bell

R1-2100371	PDCCH monitoring enhancements for up to 71GHz operation	CATT

R1-2100430	Discussions on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz			vivo

R1-2100608	PDCCH monitoring enhancement  for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2100644	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2100817	Discussion on PDCCH monitoring enhancement for NR beyond 52.6 GHz	Spreadtrum Communications

R1-2100837	Discussions on PDCCH monitoring enhancements	InterDigital, Inc.

R1-2100851	PDCCH enhancement for NR from 52.6GHz to 71GHz	Sony

R1-2100893	PDCCH monitoring enhancements to support NR above 52.6 GHz	LG Electronics

R1-2101110	PDCCH monitoring enhancement for NR 52.6-71GHz	Xiaomi

R1-2101195	PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101307	PDCCH Monitoring Enhancements	Ericsson

R1-2101321	Discussion on PDCCH monitoring enhancements for NR above 52.6GHz	CEWiT

R1-2101373	PDCCH monitoring enhancements for NR between 52.6GHz and 71 GHz	Apple

R1-2101418	Consideration for PDCCH Monitoring for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2101454	PDCCH monitoring enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2101606	PDCCH monitoring enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

### Enhancements for PUCCH formats 0/1/4

R1-2100059	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2100075	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2100151	Discussion on enhancements for PUCCH format 0/1/4	OPPO

R1-2100239	Enhancement on PUCCH formats	Huawei, HiSilicon

R1-2100259	Enhancements for PUCCH formats	Nokia, Nokia Shanghai Bell

R1-2100372	Enhancements for PUCCH formats for up to 71GHz operation	CATT

R1-2100431	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2100487	Discussion on PUCCH Channel enhancements	FUTUREWEI

R1-2100609	PUCCH formats 0/1/4 enhancement for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2100645	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2100819	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

R1-2100838	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

R1-2100894	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

R1-2101196	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101308	PUCCH enhancements	Ericsson

R1-2101374	Enhancements for PUCCH formats 0/1/4 for NR between 52.6GHz and 71 GHz	Apple

R1-2101455	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2101607	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2101673	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

### Beam management for new SCSs

Including timing associated with beam-based operation

R1-2100052	Beam management for shared spectrum access in Beyond 52.6GHz	FUTUREWEI

R1-2100060	Beam-management enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2100076	Discussion on the beam management for 52.6 to 71GHz	ZTE, Sanechips

R1-2100152	Discussion on beam management	OPPO

R1-2100203	Discussion on the beam management procedures for 52-71GHz band	Huawei, HiSilicon

R1-2100260	Beam Management Aspects	Nokia, Nokia Shanghai Bell

R1-2100373	Beam management for new SCSs for up to 71GHz operation	CATT

R1-2100432	Discussions on beam management for new SCSs for NR operation from 52.6GHz to 71GHz			vivo

R1-2100646	Discussion on Beam management aspects for extending NR up to 71 GHz	Intel Corporation

R1-2100839	Discussions on beam management for new SCSs	InterDigital, Inc.

R1-2100852	Beam management enhancement for NR from 52.6GHz to 71GHz	Sony

R1-2100895	Enhancements for beam management to support NR above 52.6 GHz	LG Electronics

R1-2101111	Discussion on beam management in NR from 52.6 GHz to 71GHz	Xiaomi

R1-2101197	Beam management for new SCSs for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101309	Beam Management for New SCSs	Ericsson

R1-2101375	On beam management for new SCSs	Apple

R1-2101419	On Beam Management for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2101456	Beam management for new SCS for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2101608	Beam based operation for new SCSs for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

### PDSCH/PUSCH enhancements

Including defining maximum bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz, reference signals, scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, etc.

R1-2100050	Considerations for higher SCS in Beyond 52.6 GHz	FUTUREWEI

R1-2100061	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2100077	Discussion on the data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2100153	Discussion on PDSCH/PUSCH enhancements	OPPO

R1-2100201	PDSCH/PUSCH enhancments for 52-71GHz band	Huawei, HiSilicon

R1-2100261	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

R1-2100300	Discussions on PDSCH and PUSCH enhancements for 52.6-71GHz	CAICT

R1-2100374	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

R1-2100433	Discussions on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz			vivo

R1-2100553	PT-RS enhancements for NR from 52.6GHz to 71GHz	Mitsubishi Electric RCE

R1-2100605	On Enhancements of PDSCH Reference Signals	MediaTek Inc.

R1-2100647	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2100741	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2100820	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

R1-2100840	Discussions on PDSCH/PUSCH enhancements	InterDigital, Inc.

R1-2100853	PDSCH/PUSCH enhancements for NR from 52.6GHz to 71GHz	Sony

R1-2100896	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

R1-2100940	PDSCH enhancements on supporting NR from 52.6GHz to 71 GHz	NEC

R1-2101112	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

R1-2101198	PDSCH/PUSCH enhancements  for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101310	PDSCH-PUSCH Enhancements	Ericsson

R1-2101320	Enhancements on Reference Signals for PDSCH/PUSCH for NR beyond 52.6 GHz	CEWiT

R1-2101330	PDSCH-PUSCH Enhancement Aspects for NR beyond 52.6 GHz	Charter Communications

R1-2101376	PDSCH/PUSCH enhancements for NR between 52.6GHz and 71 GHz	Apple

R1-2101457	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2101609	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2101674	Discussion on PDSCH/PUSCH enhancements for NR beyond 52.6GHz	WILUS Inc.

Withdrawn

### Channel access mechanism

R1-2100053	Considerations on channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

R1-2100062	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2100078	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

R1-2100154	Discussion on channel access mechanism	OPPO

R1-2100202	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2100262	Channel access mechanism	Nokia, Nokia Shanghai Bell

R1-2100301	Discussions on channel access mechanism for 52.6G-71 GHz	CAICT

R1-2100375	Channel access mechanism for up to 71GHz operation	CATT

R1-2100434	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

R1-2100542	Channel access mechanism	TCL Communication Ltd.

R1-2100648	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

R1-2100742	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

R1-2100755	Channel access for multi-beam operation	PANASONIC

R1-2100782	Further Discussion of Channel Access Mechanisms	AT&T

R1-2100821	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2100841	Discussion on channel access mechanisms	InterDigital, Inc.

R1-2100854	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2100897	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2100941	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

R1-2101113	Channel access mechanism for NR on 52.6-71 GHz	Xiaomi

R1-2101199	Channel access mechanism  for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101311	Channel Access Mechanisms	Ericsson

R1-2101331	Channel access mechanisms	Charter Communications

R1-2101377	Channel access mechanisms for unlicensed access above 52.6GHz	Apple

R1-2101420	On Channel Access Mechanism for Extending Current NR to 71 GHz	Convida Wireless

R1-2101458	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2101569	Discussion on LBT mode	ITRI

R1-2101610	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

### Others

R1-2100079	Evaluation results for 52.6 to 71GHz	ZTE, Sanechips

R1-2100155	Discussion on other aspects	OPPO

R1-2100263	Simulation Results	Nokia, Nokia Shanghai Bell

R1-2100435	Discussions on frequency range definition for 52.6-71GHz	vivo

R1-2100842	Efficient beam management based on FR2	InterDigital, Inc.

R1-2101200	Discussion on the synchronization raster for NR from 52.6 GHz to 71 GHz	Samsung

R1-2101268	Link level and system evaluation results for 52-71 GHz	Huawei, HiSilicon

R1-2101312	NR channelization for 52.6 - 71 GHz band	Ericsson

R1-2101498	Other aspects of NR in 52.6 to 71GHz band	Qualcomm Incorporated

Withdrawn

R1-2101611	Potential enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

## Enhanced Industrial Internet of Things (IoT) and URLLC (6)

Please refer to RP-201310 for detailed scope of the WI

Please also refer to RP-202872 for additional guidance for this e-meeting

R1-2101689	Revised Rel-17 NR IIoT/URLLC Work Plan	Nokia (Rapporteur)

### Necessity and Support of Physical Layer feedback enhancements

#### UE feedback enhancements for HARQ-ACK

[104-e-NR-R17-IIoT_URLLC-01] Email discussion on UE feedback enhancements for HARQ-ACK – Klaus (Nokia)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Agreements:

- Support deferring SPS HARQ-ACK dropped due to TDD specific collisions until a next available PUCCH in Rel-17 based on semi-static configuration of slot format

  - FFS: Details (including possible conditions for such a deferring, whether or not to consider semi-statically configured flexible symbols for PUCCH availability, etc.)

  - Aim for minimal standardization efforts and UE complexity in implementation

Update on 1/28 email:

Agreements:

Further down-select between the following two options for SPS HARQ-ACK deferral:

- Option 1: Joint RRC configuration of the SPS HARQ-ACK deferral per PUCCH cell group

  - Note: any SPS HARQ-ACK within a PUCCH cell group in principle is subject to deferral

- Option 2: The SPS HARQ-ACK deferral is configured per SPS configuration

  - Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations configured for deferral is in principle subject to deferral

Agreements: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH

- Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary

- FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK

- Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17

  - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed

Agreements: Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition.

- FFS: Support for slot-based PUCCH repetition

R1-2100101	Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2100181	HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2100226	UE feedback enhancements for HARQ-ACK	Huawei, BUPT, China Southern Power Grid, HiSilicon

R1-2100268	HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2100302	UE feedback enhancements for HARQ-ACK	CAICT

R1-2100376	UE feedback enhancements for HARQ-ACK	CATT

R1-2100436	HARQ-ACK enahncements for Rel-17 URLLC	vivo

R1-2100574	On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2100649	UE HARQ feedback enhancements for URLLC/IIoT	Intel Corporation

R1-2100728	HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2100803	Discussion on physical Layer feedback enhancements	Spreadtrum Communications

R1-2100855	Considerations on HARQ-ACK enhancements for URLLC	Sony

R1-2100880	Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2100911	Discussion on UE feedback enhancements for HARQ-ACK	China Telecom

R1-2100920	UE feedback enhancements for HARQ-ACK	TCL Communication Ltd.

R1-2100948	UE feedback enhancements for HARQ-ACK	NEC

R1-2100968	Discussion on UE feedback enhancements for HARQ-ACK	Asia Pacific Telecom, FGI

R1-2100993	HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2101013	Discussion on UE feedback enhancements for HARQ-ACK	Panasonic Corporation

R1-2101039	Discussion on UE feedback enhancements for HARQ-ACK	CMCC

R1-2101075	UE feedback enhancements for HARQ-ACK	ETRI

R1-2101114	UE feedback enhancement for HARQ-ACK	Xiaomi

R1-2101201	On HARQ-ACK reporting enhancements	Samsung

R1-2101290	HARQ-ACK enhancements for IIoT and URLLC	InterDigital, Inc.

R1-2101378	Views on UE feedback enhancements for HARQ-ACK	Apple

R1-2101459	HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2101539	UE feedback enhancements for HARQ-ACK	Sharp

R1-2101612	Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2101675	Discussion on HARQ-ACK enhancement for URLLC/IIoT	WILUS Inc.

#### CSI feedback enhancements

[104-e-NR-R17-IIoT_URLLC-02] Email discussion on CSI feedback enhancements– Paul (InterDigital)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

R1-2101811

R1-2100037	CSI feedback enhancements for URLLC	FUTUREWEI

R1-2100102	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2100182	CSI feedback enhancements for URLLC	OPPO

R1-2100227	CSI feedback enhancements	Huawei, HiSilicon

R1-2100269	CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2100377	CSI feedback enhancements	CATT

R1-2100437	CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2100575	CSI feedback enhancements for URLLC	MediaTek Inc.

R1-2100650	CSI feedback enhancements for URLLC/IIoT	Intel Corporation

R1-2100790	Discussion on CSI feedback enhancements	Spreadtrum Communications

R1-2100830	CSI feedback enhancements	InterDigital, Inc.

R1-2100835	CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2100856	Considerations on CSI feedback enhancements	Sony

R1-2100881	Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2100994	CSI feedback enhancements for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2101014	Discussion on CSI feedback enhancements	Panasonic Corporation

R1-2101040	Discussion on CSI feedback enhancements for URLLC	CMCC

R1-2101202	Improving MCS Selection for URLLC	Samsung

R1-2101379	Views on CSI feedback enhancements	Apple

R1-2101460	CSI enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2101613	Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

### Enhancements for unlicensed band URLLC/IIoT

[104-e-NR-R17-IIoT_URLLC-03] Email discussion on enhancements for unlicensed band URLLC/IIoT – Sorour (Ericsson)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

R1-2101830

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

- An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

- FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

- FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

  - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

    - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

R1-2100054	Further considerations on UE initiated COT for FFP	FUTUREWEI

R1-2100103	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2100183	Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2100229	Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, BUPT, China Southern Power Grid, HiSilicon

R1-2100270	Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2100378	Enhancements for unlicensed band URLLC/IIoT	CATT

R1-2100438	Enhancements for unlicensed band URLLC/IIoT	vivo

R1-2100543	Enhancements for unlicensed band URLLC/IIoT	TCL Communication Ltd.

R1-2100576	On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2100626	UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2100651	Discussion on enhancements to URLLC/IIoT in unlicensed band	Intel Corporation

R1-2100691	Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2100791	Discussion on enhancements for unlicensed band URLLC/IIoT	Spreadtrum Communications

R1-2100857	Considerations on unlicensed URLLC	Sony

R1-2100882	Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2100995	Enhancements for unlicensed band IIoT/URLLC	Lenovo, Motorola Mobility

R1-2101015	Enhancements for unlicensed band URLLC/IIoT	Panasonic Corporation

R1-2101076	Enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2101115	Enhancement for unlicensed band URLLC/IIoT	Xiaomi

R1-2101203	Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2101291	Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2101333	Unlicensed spectrum operation for URLLC/IIoT	Charter Communications

R1-2101380	URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2101461	Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

R1-2101540	Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2101614	Discussion on enhancements for unlicensed band URLLC	NTT DOCOMO, INC.

R1-2101676	Discussion on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

### Intra-UE Multiplexing/Prioritization

[104-e-NR-R17-IIoT_URLLC-04] Email discussion on intra-UE multiplexing/prioritization – Jia (OPPO)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

R1-2101842

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2.

- FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- FFS in case the total number of LP and HP HARQ-ACK bits is 2.

- FFS details

Working assumption:

Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities

- FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met

Agreements:

For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1.

- FFS value(s)

- FFS to additionally support beta-offset =0 or a value disabling the multiplexing

- Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

Agreements:

Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group

- FFS: dynamic indication

R1-2100104	Discussion on enhanced intra-UE multiplexing	ZTE

R1-2100184	Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2100228	Intra-UE multiplexing enhancements	Huawei, BUPT, China Southern Power Grid, HiSilicon

R1-2100271	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2100303	Considerations of intra UE multiplexing	CAICT

R1-2100379	Intra-UE multiplexing and prioritization	CATT

R1-2100439	Intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2100577	Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

R1-2100652	Considerations on intra-UE multiplexing and prioritization	Intel Corporation

R1-2100692	Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2100729	On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2100804	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2100831	Intra-UE Multiplexing/Prioritization	InterDigital, Inc.

R1-2100858	Considerations on intra-UE UL multiplexing	Sony

R1-2100883	Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2100921	Intra-UE Multiplexing and Prioritization	TCL Communication Ltd.

R1-2100970	Discussion on Intra-UE multiplexing/prioritization	Asia Pacific Telecom, FGI

R1-2100996	Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2101016	Discussion on Intra-UE multiplexing and prioritization of different priority	Panasonic Corporation

R1-2101041	Discussion on intra-UE multiplexing or prioritization	CMCC

R1-2101077	Intra-UE Multiplexing/Prioritization	ETRI

R1-2101116	Intra-UE multiplexing prioritization for URLLC/IIoT	Xiaomi

R1-2101204	Uplink intra-UE multiplexing and prioritization	Samsung

R1-2101381	Views on Intra-UE Multiplexing/Prioritization	Apple

R1-2101462	Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

R1-2101541	Enhancements on intra-UE UCI multiplexing and PUSCH prioritization	Sharp

R1-2101570	Discussion on intra-UE multiplexing	ITRI

R1-2101615	Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2101677	Discussion on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

### Others

Including any RAN1 involvement in propagation delay compensation enhancements (including mobility issues, if any).

[104-e-NR-R17-IIoT_URLLC-05] Email discussion on propagation delay compensation enhancements – Chengyan (Huawei)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

R1-2101816

Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately.

- Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not

  - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background

- FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

R1-2100105	Discussion on propagation delay compensation enhancements	ZTE

R1-2100185	Enhancements for Propagation Delay Compensation	OPPO

R1-2100272	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R1-2100380	Discussion on propagation delay compensation enhancements	CATT

R1-2100440	Discussion on propagation delay compensation enhancements	vivo

R1-2100578	Discussion on propagation delay compensation for time synchronization	MediaTek Inc.

R1-2100653	Propagation delay compensation analysis and design considerations	Intel Corporation

R1-2100730	Discussion on enhancements for propagation delay compensation	Nokia, Nokia Shanghai Bell

R1-2100884	Discussion on propagation delay compensation enhancements	LG Electronics

R1-2101078	Propagation delay compensation enhancements	ETRI

R1-2101205	Discussion for propagation delay compensation enhancements	Samsung

R1-2101265	Enhancements for support of time synchronization	Huawei, BUPT, China Southern Power Grid, HiSilicon

R1-2101382	Orphan symbol treatment in unlicensed spectrum access	Apple

R1-2101463	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

## Solutions for NR to support non-terrestrial networks (NTN) (5)

Please refer to RP-202908 for detailed scope of the WI

R1-2100832	Reply LS on SA WG2 assumptions AN-PDB and PER targets for satellite access	THALES

### Timing relationship enhancements

R1-2100156	Discusson on timing relationship enhancement	OPPO

R1-2100222	Discussion on timing relationship enhancements for NTN	Huawei, HiSilicon

R1-2100244	Discussion on timing relationship for NR-NTN	ZTE

R1-2100304	Timing relationship enhancements to support NTN	CAICT

R1-2100381	Timing relationship enhancement for NTN	CATT

R1-2100441	Discussion on timing relationship enhancements for NR-NTN	vivo

R1-2100496	Timing relationship enhancements for NTN	Zhejiang Lab

R1-2100594	Timing relationship enhancements for NR-NTN	MediaTek Inc.

R1-2100654	On timing relationship enhancements for NR NTN	Intel Corporation

R1-2100703	Discussions on timing relationship enhancements in NTN	LG Electronics

R1-2100757	Discussion on NTN timing relationship	Lenovo, Motorola Mobility

R1-2100807	Consideration on timing relationship enhancements	Spreadtrum Communications

R1-2100859	Calculation of timing relationship offsets	Sony

R1-2100912	Discussion on Timing Relationship Enhancements in NR-NTN	China Telecom

R1-2100922	On timing relationship enhancements for NTN	Ericsson

R1-2100971	Timing relationship enhancements in NTN	Asia Pacific Telecom, FGI, III, ITRI

R1-2100984	On timing relationship for NTN	InterDigital, Inc.

R1-2101024	NTN Timing relationship enhancement	Panasonic Corporation

R1-2101042	Discussion on timing relationship enhancements for NTN	CMCC

R1-2101117	Discussion on the timing relationship enhancement for NTN	Xiaomi

R1-2101206	Timing relationship enhancements for NTN	Samsung

R1-2101296	Further views on DL-UL timing relationship for NTN operation	Nokia, Nokia Shanghai Bell

R1-2101383	Enhancement on Timing Relationship in NTN	Apple

R1-2101464	Enhancements on Timing Relationship for NTN	Qualcomm Incorporated

R1-2101616	Discussion on timing relationship enhancements for NTN	NTT DOCOMO, INC.

R1-2101694	Discussion on Timing Relationship Enhancements for NTN	Fraunhofer IIS, Fraunhofer HHI

[104-e-NR-NTN-01] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Xingqin (Ericsson)

### Enhancements on UL time and frequency synchronization

R1-2100157	Discussion on UL time and frequency synchronization	OPPO

R1-2100223	Discussion on UL time and frequency synchronization enhancement for NTN	Huawei, HiSilicon

R1-2100245	Discussion on UL synchronization for NR-NTN	ZTE

R1-2100305	Considerations on Enhancements on UL Time Synchronization in NTN	CAICT

R1-2100382	UL time and frequency compensation for NTN	CATT

R1-2100442	Discussion on UL time and frequency synchronization  enhancements for NR-NTN	vivo

R1-2100520	Considerations on UL timing and frequency synchronization in NR NTN	THALES

R1-2100540	UL time synchronization acquisition for NTN	Mitsubishi Electric RCE

R1-2100595	UE Time and frequency Synchronisation for NR-NTN	MediaTek Inc.

R1-2100655	On UL synchronization for NR NTN	Intel Corporation

R1-2100704	Discussions on UL time and frequency synchronization enhancements in NTN	LG Electronics

R1-2100758	Discussion on NTN uplink time synchronization	Lenovo, Motorola Mobility

R1-2100808	Consideration on enhancements on UL time and frequency synchronization	Spreadtrum Communications

R1-2100860	Enhancement for UL time synchronization	Sony

R1-2100927	On UL time and frequency synchronization enhancements for NTN	Ericsson

R1-2100972	UL time and frequency synchronization in NTN	Asia Pacific Telecom, FGI

R1-2100985	On UL time/frequency synchronization for NTN	InterDigital, Inc.

R1-2101043	Enhancements on UL time and frequency synchronization for NTN	CMCC

R1-2101079	Discussion on UL timing synchronization for NTN	ETRI

R1-2101118	Discussion on UL time and frequency synchronization for NTN	Xiaomi

R1-2101207	Enhancements on UL time and frequency synchronization for NTN	Samsung

R1-2101297	Time and frequency synchronization for NTN systems	Nokia, Nokia Shanghai Bell

R1-2101384	Discussion on Uplink Time and Frequency Synchronization for NTN	Apple

R1-2101465	UL time and frequency synchronization for NTN	Qualcomm Incorporated

R1-2101648	Discussion on UL time and frequency synchronization for NTN	PANASONIC R&D Center Germany

R1-2101716	UL time synchronization methods for NTN systems	CEWiT,IITM,IITH,Tejas Networks,

R1-2101717	UL time synchronization methods for NTN systems	CEWiT,IITM,IITH,Tejas Networks,Reliance Jio

Withdrawn

[104-e-NR-NTN-02] Email discussion/approval on UL time and frequency synchronization with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Mohamed (Thales)

### Enhancements on HARQ

R1-2100158	Discussion on HARQ enhancement	OPPO

R1-2100224	Discussion on HARQ enhancement for NTN	Huawei, HiSilicon

R1-2100246	Discussion on HARQ for NR-NTN	ZTE

R1-2100306	Enhancements on HARQ to support NTN	CAICT

R1-2100383	HARQ operation enhancement for NTN	CATT

R1-2100443	Discussion on HARQ enhancements for NR-NTN	vivo

R1-2100485	INTELLIGENT PACKET REPETITION IN MOBILE SATELLITE SERVICE (MSS) LINKS TO OVERCOME CHANNEL BLOCKAGES	Ligado Networks

Late submission

R1-2100491	Discussion on HARQ enhancement for NTN	BUPT

R1-2100596	HARQ in NR-NTN	MediaTek Inc.

R1-2100656	On HARQ enhancements for NR NTN	Intel Corporation

R1-2100705	Discussions on HARQ enhancements in NTN	LG Electronics

R1-2100759	Enhancements on HARQ for NTN	Lenovo, Motorola Mobility

R1-2100809	Consideration on enhancements on HARQ	Spreadtrum Communications

R1-2100861	Enhancements on HARQ for NTN	Sony

R1-2100913	Discussion on HARQ Enhancements in NR-NTN	China Telecom

R1-2100928	On HARQ enhancements for NTN	Ericsson

R1-2100973	Enhancements on HARQ in NTN	Asia Pacific Telecom, FGI

R1-2100986	On HARQ enhancement for NTN	InterDigital, Inc.

R1-2101025	HARQ enhancement for NTN 	Panasonic Corporation

R1-2101044	Enhancements on HARQ for NTN	CMCC

R1-2101080	Discussion on HARQ Enhancements for NTN	ETRI

R1-2101119	Discussion on the HARQ enhancement for NTN	Xiaomi

R1-2101208	Enhancements on HARQ for NTN	Samsung

R1-2101289	Discussion on HARQ for NTN	THALES

R1-2101298	On HARQ operation for NTN systems	Nokia, Nokia Shanghai Bell

R1-2101385	Discussion on HARQ Enhancements for NTN	Apple

R1-2101466	Enhancements on HARQ for NTN	Qualcomm Incorporated

[104-e-NR-NTN-03] Email discussion/approval on HARQ enhancements with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Nan (ZTE)

### Others

Including the list of topics to specify if beneficial and needed

R1-2100598	Summary #1 of 8.4.4 Other Aspects of NR-NTN	MediaTek Inc.

R1-2100159	Discussion on other aspects	OPPO

R1-2100247	Discussion on additional enhancement for NR-NTN	ZTE

R1-2100264	IoT NTN Observations and Proposals	Lockheed Martin

R1-2100307	Discusson on beam management of NTN	CAICT

R1-2100384	Beam management and other aspects for NTN	CATT

R1-2100444	Discussion on other aspects for NR-NTN	vivo

R1-2100597	Other Aspects of  NR-NTN	MediaTek Inc.

R1-2100706	Discussions on other aspects of NTN	LG Electronics

R1-2100723	Discussion on channel models for marine and inland river scenes	GDCNI

Late submission

R1-2100760	Discussion on other aspects for NTN	Lenovo, Motorola Mobility

R1-2100862	Discussion on beam management and polarization for NTN	Sony

R1-2100929	On other enhancements for NTN	Ericsson

R1-2100974	Scrambling sequence enhancement in NTN	Asia Pacific Telecom, FGI, ITRI

R1-2100987	On feeder link switch and beam management for NTN	InterDigital, Inc.

R1-2101045	Other Aspects for NTN	CMCC

R1-2101120	Discussion on other design aspects for NTN	Xiaomi

R1-2101209	Remaining issues for NTN	Samsung

R1-2101260	Discussion on other design aspects for NTN	Huawei, HiSilicon

R1-2101288	Beam management and BWP operation in NTN	THALES

R1-2101295	SSB, beam management and polarization signaling for NTN 	PANASONIC

R1-2101299	Additional aspects for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2101386	Other Aspects of NR NTN	Apple

R1-2101467	BWP operation and other issues for NTN	Qualcomm Incorporated

R1-2101693	Discussion on Beam Management for NTN	Fraunhofer IIS, Fraunhofer HHI

[104-e-NR-NTN-04] Email discussion/approval on other enhancements including list of topics to specify if beneficial and needed with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Hao (Oppo)

[104-e-NR-NTN-05] Email discussion/approval of the reply LS to R1-2100014 until Feb-01 – Xiaofeng (Qualcomm)

## NR Positioning Enhancements

Please refer to RP-202900 for detailed scope of the WI

### Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Including DL, UL and DL+UL positioning methods, and UE-based and UE-assisted positioning solutions.

R1-2100128	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays		OPPO

R1-2100195	Enhancement to mitigate gNB and UE Rx/Tx timing error	Huawei, HiSilicon

R1-2100293	Positioning accuracy improvement by mitigating timing delay	ZTE

R1-2100308	Disussion on accuracy improvements of NR positioning enhancements	CAICT

R1-2100385	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays			CATT

R1-2100445	Discussion on methods for RX/TX timing delay mitigating	vivo

R1-2100548	Initial views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

R1-2100657	Mitigation of UE and gNB Tx/Rx timing errors	Intel Corporation

R1-2100697	Positioning enhancement by UE Assistance	TCL Communication Ltd.

R1-2100708	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

R1-2100752	Techniques to improve accuracy in the presence of UE Rx/Tx and/or gNB Rx/Tx timing delays			InterDigital, Inc.

R1-2101046	Discussion on gNB/UE Rx/Tx timing delay mitigation solutions	CMCC

R1-2101131	On methods for Rx/Tx timing delays mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2101140	The mitigation of  RX/TX timing delays for higher accuracy	MediaTek Inc.

R1-2101210	Discussion on accuracy improvements on timing based positioning solutions	Samsung

R1-2101387	Positioning accuracy enhancements under UE and/or gNB Tx/Rx timing errors	Apple

R1-2101468	Enhancements on Timing Error Mitigations for improved Accuracy	Qualcomm Incorporated

R1-2101527	NR positioning enhancements by mitigating timing delays	China Telecom

R1-2101754	Techniques mitigating UE Rx/Tx timing delays	Ericsson

### Accuracy improvements for UL-AoA positioning solutions

R1-2100129	Enhancements for UL AoA Positioning	OPPO

R1-2100237	Enhancement for UL AoA positioning	Huawei, HiSilicon

R1-2100294	Accuracy improvement for UL-AoA positioning solutions	ZTE

R1-2100386	Discussion on accuracy improvements for UL-AoA positioning solutions	CATT

R1-2100446	Discussion on potential enhancements for UL-AoA method	vivo

R1-2100488	Discussion on improving the accuracy of UL AoA positioning solutions	FUTUREWEI

R1-2100497	Accuracy improvements for UL-AoA positioning solutions	BUPT

R1-2100549	Initial views on enhancing UL AoA	Nokia, Nokia Shanghai Bell

R1-2100658	NR positioning enhancements for UL-AoA method	Intel Corporation

R1-2100659	NR positioning enhancements for DL-AoD method	Intel Corporation

R1-2100709	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

R1-2100753	Discussions on techniques to improve accuracy for UL-AoA positioning solutions	InterDigital, Inc.

R1-2100863	Discussion on accuracy improvements for UL-AoA positioning method	Sony

R1-2101047	Discussion on UL-AoA enhancement	CMCC

R1-2101132	UL-AoA positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2101211	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

R1-2101388	Accuracy enhancements for DL-AoD positioning technique	Apple

R1-2101469	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

R1-2101617	Discussion on UL-AoA positioning enhancements	NTT DOCOMO, INC.

R1-2101755	Enhancements of UL-AoA positioning solutions	Ericsson

### Accuracy improvements for DL-AoD positioning solutions

Including UE-based and network-based (including UE-assisted) positioning solutions

R1-2100130	Enhancements for DL-AoD positioning	OPPO

R1-2100238	Enhancement for DL AoD positioning	Huawei, HiSilicon

R1-2100295	Accuracy improvements for DL-AoD positioning solutions	ZTE

R1-2100387	Discussion on accuracy improvements for DL-AoD positioning solutions	CATT

R1-2100447	Discussion on potential enhancements for DL-AoD method	vivo

R1-2100489	Discussion on improving the accuracy of DL AoD positioning solutions	FUTUREWEI

R1-2100550	Initial views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2100710	Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2100750	Accuracy improvements for DL-AoD positioning solutions	InterDigital, Inc.

R1-2100864	Discussion on accuracy improvements for DL-AoD positioning method	Sony

R1-2101048	Discussion on DL-AoD enhancement	CMCC

R1-2101121	Accuracy improvements for DL-AoD positioning solutions	Xiaomi

R1-2101133	DL-AoD positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2101141	Accuracy enhancement for DL-AOD technique	MediaTek Inc.

R1-2101212	Accuracy improvements for DL-AoD positioning solutions	Samsung

R1-2101389	Accuracy enhancements for UL-AoA positioning technique	Apple

R1-2101470	Potential Enhancements on DL-AoD positioning	Qualcomm Incorporated

R1-2101501	Potential DL-AoD Positioning Enhancements	Lenovo, Motorola Mobility

R1-2101618	Discussion on DL-AoD positioning enhancements	NTT DOCOMO, INC.

R1-2101756	Enhancements of DL-AoD positioning solutions	Ericsson

### Others

R1-2100131	Discussion on NR Positioning Enhancements	OPPO

R1-2100296	Other accuracy enhancements for NR positioning	ZTE

R1-2100388	Potential techniques for Rel-17 positioning enhancements	CATT

R1-2100448	Discussion on idle and inactive states positioning	vivo

R1-2100551	Additional views on positioning enhancements	Nokia, Nokia Shanghai Bell

R1-2100711	Discussion on latency and efficiency enhancement for NR positioning	LG Electronics

R1-2100751	Discussions on LOS/NLOS detection	InterDigital, Inc.

R1-2101142	Solving the synchronization error by the combination of measurements for DL-TDOA and UL-TDOA		MediaTek Inc.

R1-2101213	Discussion on differential beamforming for positioning enhancement	Samsung

R1-2101252	Considerations on other enhancements for positioning accuracy	Huawei, HiSilicon

R1-2101499	Other asprects of positioning enhancements	Qualcomm Incorporated

Withdrawn

R1-2101757	PRS with Cyclic shifts	Ericsson

## Support of Reduced Capability NR Devices

Please refer to RP-202933 for detailed scope of the WI

R1-2100033	WI work plan for RedCap	Rapporteur (Ericsson)

### UE complexity reduction

Including reduced maximum UE bandwidth, reduced minimum number of Rx branches, maximum number of DL MIMO layers, relaxed maximum modulation order and duplex operation

R1-2100034	UE complexity reduction for RedCap	Ericsson

R1-2100230	Potential solutions for UE complexity reduction	Huawei, HiSilicon

R1-2101049	Discussion on UE complexity reduction	CMCC

R1-2101214	UE complexity reduction	Samsung

R1-2101390	On UE complexity reduction features for RedCap	Apple

R1-2101766	Complexity Reduction for RedCap Devices	Qualcomm Incorporated

R1-2101471

R1-2101619	Discussion on UE complexity reduction for RedCap	NTT DOCOMO, INC.

[104-e-NR-RedCap-01] Email discussion on UE complexity reduction – Johan (Ericsson)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

R1-2101849

Agreements:

- Sharing of the same SSB and CORESET#0 between RedCap and non-RedCap UEs is supported when the bandwidth is no wider than the RedCap UE bandwidth

- The initial DL BWP (derived based on MIB/SIB) for RedCap UEs can be the same as the initial DL BWP for non-RedCap UEs at least when the initial DL BWP is no wider than the RedCap UE bandwidth.

  - FFS: after initial access, whether a RedCap UE is allowed to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth

    - Discuss further whether or not it is also applicable during initial access

- The initial UL BWP (derived based on SIB) for RedCap UEs can be the same as the initial UL BWP for non-RedCap UEs at least when the initial UL BWP is no wider than the RedCap UE bandwidth.

  - FFS: during and after initial access, whether a RedCap UE is allowed to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth

- FFS whether or not to further introduce the following (e.g., for offloading purpose, for differentiation of RedCap vs. non RedCap UEs, for different BWP#0 configuration options, etc.)

- Whether an additional CORESET can be configured for scheduling of RACH (msg2 & msg4)/Paging/SI messages for RedCap UEs

- Whether the SIB-configured initial DL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial DL BWP for non-RedCap UEs.

- Whether the SIB-configured initial UL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial UL BWP for non-RedCap UEs.

Agreements:

- For relaxed maximum number of DL MIMO layers:

  - FFS: need for modification of DCI fields/formats

  - FFS: need for modification of CSI measurement/reporting

R1-2101850

Conclusion: RAN1 does not consider acquisition time improvements for FR2 RedCap UEs with SSB and CORESET#0 multiplexing patterns 2 and 3 as part of this WI.

Agreements:

- For HD-FDD, for cases (if any) where collision handling needs to be specified, then the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum are used as a starting point if deemed applicable.

Agreements:

- (Working assumption) For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

- Sending an LS to RAN4 to inform the above working assumption, and to ask for feedback if any

  - The LS will not include the two FFS bullets

Draft LS in R1-2102094 is approved. Final LS to be uploaded/updated depending on whether or not there are additional agreements for RedCap related to RAN4

Agreements:

- Study further how to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, with the following options:

  - Option 1: Proper RF-retuning for RedCap

- Option 2: Separate initial UL BWP(s) for RedCap UEs

- Option 3: gNB configuration (e.g., restrictions on existing PRACH configurations, or FDM-ed ROs, or always restricting the initial UL BWP to within RedCap UE bandwidth)

- Option 4: Dedicated PRACH configurations (e.g., ROs) for RedCap UEs

- Other options are not precluded

Agreements:

- For reduced minimum number of Rx branches in FR1 and FR2 frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports:

  - FFS: need for solutions to reduced PDCCH blocking

  - FFS: need for reporting of UE antenna related information to gNB (e.g., # of panels, polarization, etc.)

  - Information related to the reduction of the number of antenna branches is assumed to be known at the gNB (either implicitly or explicitly, to be FFS)

Agreements:

- The MCS tables currently defined are re-used for RedCap UEs

  - FFS which MCS table is the default one for RedCap (i.e., the default one for non-RedCap UEs or the one with low SE entries)

  - FFS mandatory/optional of the MCS tables

  - Note: there is no new MCS table to be introduced for RedCap UEs

Agreements:

- For HD-FDD operation for RedCap UEs, consider at least the following DL/UL collision cases collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

  - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

    - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH, or RO

  - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

    - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

  - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

  - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

    - e.g., PUSCH, PUCCH, PRACH, SRS

  - Case 6: Monitoring for UL cancellation indication (if supported) while transmitting in UL

  - Case 7: Collision due to BWP switching (if supported)

  - Case 8: Dynamic or semi-static DL vs. valid RO

  - Case 9: Collision due to direction switching

R1-2100046	Complexity reduction features for RedCap UEs	FUTUREWEI

R1-2100165	Discussion on UE complexity reduction	OPPO

R1-2100389	Discussion on UE complexity reduction features	CATT

R1-2100449	Discussion on UE Complexity reduction	vivo, Guangdong Genius

R1-2100499	UE complexity reduction	Nokia, Nokia Shanghai Bell

R1-2100564	UE complexity reduction for Reduced Capability NR devices	ZTE

R1-2100579	On complexity reduction features for NR RedCap UEs	MediaTek Inc.

R1-2100625	Discussion on RedCap features	NEC

R1-2100660	On UE complexity reduction for RedCap devices	Intel Corporation

R1-2100772	UE complexity reduction features for RedCap	Lenovo, Motorola Mobility

R1-2100823	Discussion on UE complexity reduction features	Spreadtrum Communications

R1-2100843	UE complexity reduction	Panasonic Corporation

R1-2100865	UE complexity reduction for Redcap devices	Sony

R1-2100900	Discussion on complexity reduction of reduced capability NR devices	LG Electronics

R1-2100969	Discussion on UE complexity reduction	Asia Pacific Telecom, FGI

R1-2101122	Discussion on the complexity reduction for Redcap	Xiaomi

R1-2101507	Discussion on UE complexity reduction features	InterDigital, Inc.

R1-2101542	Discussion on UE complexity reduction	Sharp

R1-2101640	Potential enhancement for UE complexity reduction	TCL Communication Ltd.

R1-2101659	Discussion on UE complexity reduction	ASUSTeK

R1-2101718	Discussion on UE complexity reduction	China Unicom

### Higher layer support of Reduced Capability NR Devices

A placeholder only for this e-meeting – no intention to have any email or GTW discussion

R1-2100035	Higher layer support for RedCap	Ericsson

R1-2100166	Mechanism in higher&PHY layer for Reduced Capability NR Devices	OPPO

R1-2100231	RAN1 aspects of RedCap UE type and identification	Huawei, HiSilicon

R1-2100390	Discussion on high layer support for RedCap UE	CATT

R1-2100450	Higher layer support for RedCap NR devices	vivo, Guangdong Genius

R1-2100500	Higher layer support of Reduced Capability NR Devices	Nokia, Nokia Shanghai Bell

R1-2100565	Higher layer support of Reduced Capability NR devices	ZTE

R1-2100661	On higher layer support for RedCap devices	Intel Corporation

R1-2100773	Higher layer support of Reduced Capability NR devices	Lenovo, Motorola Mobility

R1-2100822	Discussion on Higher layer support of RedCap UE	Spreadtrum Communications

R1-2100901	Higher layer support for reduced capability NR devices	LG Electronics

R1-2101050	Discussion on early identification of RedCap UEs	CMCC

R1-2101123	Discussion on the higher layer support for Redcap	Xiaomi

R1-2101215	UE capability report and access barring for Redcap UE	Samsung

R1-2101322	Design consideration for Higher layer support of RedCap	Sierra Wireless, S.A.

R1-2101334	UE identification for redcap devices	Sony

R1-2101391	On Higher Layer Support of Redcap Devices	Apple

R1-2101472	Higher layer support for RedCap devices	Qualcomm Incorporated

R1-2101506	Higher layer support for Reduced Capability NR devices	InterDigital, Inc.

R1-2101620	Discussion on higher layer support of RedCap	NTT DOCOMO, INC.

R1-2101660	Higher layer support of reduced capability NR devices	ASUSTeK

R1-2101678	Discussion on higher layer support of Redcap UE	WILUS Inc.

R1-2101724	UE type definition for RedCap UEs	FUTUREWEI

### Others

R1-2100036	Other aspects of RedCap	Ericsson

R1-2100167	Other considerations for reduced UE capability	OPPO

R1-2100310	Consideration on reduced capability bandwidth reduction	TCL Communication Ltd.

R1-2100391	Views on potential UE complexity reduction features	CATT

R1-2100451	Discussion on reduced capability signalling	vivo, Guangdong Genius

R1-2100506	Coverage recovery aspects for RedCap UEs	Nokia, Nokia Shanghai Bell

R1-2100566	NR UE features for RedCap	ZTE

R1-2100693	Reduced PDCCH monitoring for REDCAP NR devices	NEC

R1-2100902	Other aspects of reduced capability NR devices	LG Electronics

R1-2100997	PDCCH monitoring at reduced capability UE	Lenovo, Motorola Mobility

R1-2101051	Discussion on network access control of RedCap devices	CMCC

R1-2101124	Discussion on the coverage recovery for Redcap	Xiaomi

R1-2101216	Remaining issues for Redcap	Samsung

R1-2101248	Discussions on PDCCH monitoring for reduced capability devices	CEWiT

R1-2101266	Potential solutions for UL coverage recovery	Huawei, HiSilicon

R1-2101473	Design principles for RedCap devices	Qualcomm Incorporated

R1-2101543	Discussion on other issues for reduced capability UEs	Sharp

R1-2101575	PDCCH monitoring at reduced capability UE	Lenovo, Motorola Mobility

Withdrawn

R1-2101621	Discussion on coverage recovery for RedCap	NTT DOCOMO, INC.

R1-2101661	Discussion on other consideration for reduced capability UE	ASUSTeK

R1-2101725	UE capability framework for RedCap UEs	FUTUREWEI

## UE Power Saving Enhancements (4)

Please refer to RP-200938 for detailed scope of the WI

From AI5

[104-e-NR-R17-PowSav-LS-01] Email discussion regarding LS in R1-2100020, including a possible reply LS till 2/1, to be handled under 8.7 (Weide, MediaTek)

Update on 2/1 – to check on 2/2

### Enhancements for idle/inactive-mode UE power saving

#### Potential paging enhancements

[104-e-NR-R17-PowSav-01] Email discussion on potential paging enhancements – Weide (MediaTek)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Agreements:

- Carrying UE subgroups information is considered in physical layer design for paging enhancement

Agreements:

For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed:

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

- Behv-B:

  - PEI indicates whether or not UE should monitor a PO

  - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

Agreements:

For the evaluation and comparison of PEI candidate designs, companies to report

- Description of how PEI design can co-exist with existing channels/signals, and impact to legacy UEs.

  - Rel-15 designs for multiplexing PEI with legacy channels/signals are assumed as baseline

o   Other multiplexing method with legacy channels/signals can be additionally reported with justification

Agreement:

- Take Alt 1 as mandatory, and Alt 2 as optional

Alt 1

For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS,

- The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging PDSCH is 1%,

  - When Behv-A is assumed:

    - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%:

MDR_Joint_A = MDR_PEI + (1 – MDR_PEI) MDR_PagingPDCCH

    - The False-Alarm Rate (FAR) of PEI should be no larger than [1%]

  - When Behv-B is assumed:

    - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%:

MDR_Joint_B = FAR_PEI + (1 – FAR_PEI) MDR_PagingPDCCH

    - The MDR of PEI should be no larger than [1%]

  - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO.

- Companies to provide:

  - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection)

  - The required #REs to comply with the performance assumptions

  - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions

Alt 2

For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS,

- The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging DCI is 1%,

  - When Behv-A is assumed:

    - The MDR of PEI should be no larger than 0.1%

    - The False-Alarm Rate (FAR) of PEI should be no larger than 1%

  - When Behv-B is assumed:

    - The FAR of PEI should be no larger than 0.1%

    - The MDR of PEI should be no larger than 1%

  - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO.

- Companies to provide:

  - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection)

  - The required #REs to comply with the performance assumptions

  - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions

R1-2100168	Further discussion on Paging enhancements for power saving	OPPO

R1-2100216	Paging enhancement(s) for UE power saving in IDLE/inactive mode	Huawei, HiSilicon

R1-2100392	Paging enhancement for UE power saving	CATT

R1-2100452	Paging enhancements for idle/inactive mode UE power saving	vivo

R1-2100523	Discussion on power saving enhancements for paging	ZTE , Sanechips

R1-2100544	Potential paging enhancements	TCL Communication Ltd.

R1-2100591	Design of paging early indication for idle/inactive-mode UE power saving	MediaTek Inc.

R1-2100662	On paging enhancements for UE power saving	Intel Corporation

R1-2100824	Discussion on potential paging enhancements	Spreadtrum Communications

R1-2100866	Paging enhancement in Idle/Inactive state	Sony

R1-2100903	Discussion on potential paging enhancements	LG Electronics

R1-2100998	Paging enhancement for UE power saving	Lenovo, Motorola Mobility

R1-2101052	Discussion on paging early indication design	CMCC

R1-2101125	Paging enhancement for power saving	Xiaomi

R1-2101217	Discussion on paging enhancements	Samsung

R1-2101300	On paging enhancement	Panasonic

R1-2101392	Paging early indication for idle/inactive-mode UE	Apple

R1-2101474	Paging enhancements for idle/inactive UE power saving	Qualcomm Incorporated

R1-2101503	Paging enhancements for UE power saving	InterDigital, Inc.

R1-2101555	Design of Paging Enhancements	Ericsson

R1-2101622	Discussion on paging enhancements	NTT DOCOMO, INC.

R1-2101664	On paging enhancements for UE power saving	Nokia, Nokia Shanghai Bell

#### TRS/CSI-RS occasion(s) for idle/inactive UEs

[104-e-NR-R17-PowSav-02] Email discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs– Qiongjie (Samsung)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Update on 1/28 email:

Agreements:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least:

- powerControlOffsetSS,

- scramblingID

- firstOFDMSymbolInTimeDomain,

- startingRB.

- nrofRBs,

- FFS other parameters

- FFS applicable values

Agreements:

The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b-eRAN1#105-e:

- Alt1: same as initial BWP

- Alt2: configurable parameter

Agreements:

Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details (including whether or not to restrict the RS to be TRS only)

Update on 1/31:

Agreements:

For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication.

- FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.)

- There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

Conclusion

From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs.

Agreements:

The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e:

- Alt-1: within initial DL BWP

- Alt-2: is not restricted by initial BWP

  - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

R1-2100169	Further discussion on RS occasion for idle/inactive UEs	OPPO

R1-2100217	Assistance RS occasions for IDLE/inactive mode	Huawei, HiSilicon

R1-2100393	Configuration of TRS/CSI-RS for paging enhancement	CATT

R1-2100453	TRS/CSI-RS occasion(s) for idle/inactive UEs	vivo

R1-2100524	TRS for RRC idle and inactive UEs	ZTE , Sanechips

R1-2100545	TRS/CSI-RS occasion(s) for idle/inactive UEs	TCL Communication Ltd.

R1-2100592	On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving	MediaTek Inc.

R1-2100663	TRS/CSI-RS functionality during idle/inactive mode	Intel Corporation

R1-2100814	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs	Spreadtrum Communications

R1-2100867	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Sony

R1-2100904	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	LG Electronics

R1-2100999	Provision of TRS/CSI-RS for idle/inactive UEs	Lenovo, Motorola Mobility

R1-2101053	Discussion on TRS/CSI-RS occasion(s) for IDLE/INACTIVE-mode UEs	CMCC

R1-2101126	On TRS/CSI-RS occasion(s) for idle/inactive UEs	Xiaomi

R1-2101218	Moderator summary for TRS/CSI-RS occasion(s) for idle/inactive UEs	Moderator (Samsung)

R1-2101219	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Samsung

R1-2101301	Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs	Panasonic

R1-2101393	Indication of TRS/CSI-RS for idle/inactive-mode UE power saving	Apple

R1-2101475	TRS/CSI-RS for idle/inactive UE power saving	Qualcomm Incorporated

R1-2101504	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	InterDigital, Inc.

R1-2101544	On TRS/CSI-RS occasions for idle/inactive UEs	Sharp

R1-2101556	Provisioning of TRS occasions to Idle/Inactive UEs	Ericsson

R1-2101623	Discussion on TRS/CSI-RS occasion for idle/inactive UEs	NTT DOCOMO, INC.

R1-2101665	On RS information to IDLE/Inactive mode Ues	Nokia, Nokia Shanghai Bell

#### Others

R1-2100394	Details of PEI configuration	CATT

R1-2100454	Discussion on paging grouping	vivo

R1-2100525	Additional simulation results of UE power consumption in RRC idle and inactive state	ZTE , Sanechips

R1-2101557	Modeling of Network Power Consumption	Ericsson

R1-2101740	Analysis on power consumption for IDLE mode UE	Huawei, HiSilicon

### Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime

[104-e-NR-R17-PowSav-03] Email discussion on potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime– Xiaodong (vivo)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Agreements:

- Strive for a common design for DCI based PDCCH monitoring adaptation in active time for an active BWP to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration.

  - Details FFS

R1-2100170	DCI-based power saving adaptation solutions	OPPO

R1-2100218	Extension(s) to Rel-16 DCI-based power saving adaptation for an active BWP	Huawei, HiSilicon

R1-2100395	PDCCH monitoring adaptation	CATT

R1-2100455	Discussion on DCI-based power saving adaptation in connected mode	vivo

R1-2100498	Extension to Rel-16 DCI-based power sabing adaptation during DRX Active Time	GDCNI

R1-2100526	Extension to Rel-16 DCI-based power saving adaptation during DRX Active Time	ZTE , Sanechips

R1-2100593	On enhancements to DCI-based UE power saving during DRX active time	MediaTek Inc.

R1-2100664	Discussion on PDCCH monitoring reduction techniques during active time	Intel Corporation

R1-2100815	Discussion on power saving techniques for connected-mode UEs	Spreadtrum Communications

R1-2100905	Discussion on DCI-based power saving adaptation during DRX ActiveTime	LG Electronics

R1-2100980	Discussion on extension(s) to Rel-16 DCI-based power saving adaptation	Asia Pacific Telecom, FGI

R1-2101000	Enhanced DCI based power saving adaptation	Lenovo, Motorola Mobility

R1-2101054	Discussion on PDCCH monitoring reduction during DRX active time	CMCC

R1-2101220	Discussion on DCI-based power saving techniques	Samsung

R1-2101285	DCI-based Power Saving Enhancements	Fraunhofer HHI, Fraunhofer IIS

R1-2101302	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime			Panasonic

R1-2101394	Enhanced DCI-based power saving adaptation	Apple

R1-2101476	DCI-based power saving adaptation during DRX ActiveTime	Qualcomm Incorporated

R1-2101505	PDCCH monitoring reduction in Active Time	InterDigital, Inc.

R1-2101558	Design of active time power savings mechanisms	Ericsson

R1-2101567	Power saving adaptation during Active Time	ASUSTeK

R1-2101624	Discussion on extension to DCI-based power saving adaptation	NTT DOCOMO, INC.

R1-2101666	UE power saving enhancements for Active Time	Nokia, Nokia Shanghai Bell

### Others

R1-2100171	Discussion on RLM relaxation	OPPO

R1-2100396	System overhead analysis of PEI and TRS/CSI-RS for IDLE mode UE	CATT

R1-2100456	Discussion on RLM/BFD/RRM relaxation	vivo

R1-2100527	Further discussion on potential power saving schemes for RRC connected UEs	ZTE , Sanechips

R1-2100816	Discussion on other power saving techniques	Spreadtrum Communications

R1-2101257	Other considerations on power saving in Rel-17	Huawei, HiSilicon

R1-2101500	Other power saving techniques	Qualcomm Incorporated

Withdrawn

R1-2101559	Evaluation results for UE power saving schemes	Ericsson

R1-2101720	Paging indication based on sub-time units	InterDigital, Inc.

## NR coverage enhancement

Please refer to RP-202928 for detailed scope of the WI

R1-2100914	Work plan for Rel-17 WI on NR coverage enhancements	China Telecom

### PUSCH enhancements

R1-2101625	PUSCH enhancements for coverage enhancements	NTT DOCOMO, INC.

Withdrawn

#### Enhancements on PUSCH repetition type A

R1-2100915	Enhancements on PUSCH repetition type A	China Telecom

[104-e-NR-CovEnh-01] Email discussion on enhancements on PUSCH repetition type A – Toshi (Sharp)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

R1-2100095	Discussion on enhanced PUSCH repetition type A	ZTE

R1-2100172	Enhancements on PUSCH repetition type A	OPPO

R1-2100196	Coverage enhancements for PUSCH repetition typeA	Huawei, HiSilicon

R1-2100397	Discussion on enhancements on PUSCH repetition type A	CATT

R1-2100457	Discussion on enhancement for PUSCH repetition type A	vivo

R1-2100665	Enhancements on PUSCH repetition type A	Intel Corporation

R1-2100712	Discussions on PUSCH repetition type A enhancements	LG Electronics

R1-2100731	PUSCH repetition for coverage enhancements	InterDigital, Inc.

R1-2100942	Discussion on  enhancements on PUSCH repetition type A	NEC

R1-2101001	Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

R1-2101017	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

R1-2101055	Discussion on enhancements on PUSCH repetition type A	CMCC

R1-2101127	Enhancements on PUSCH repetiton type A	Xiaomi

R1-2101221	Enhancements on PUSCH repetition type A	Samsung

R1-2101327	Design Considerations for Enhancements on PUSCH repetition	Sierra Wireless, S.A.

R1-2101395	Discussion on PUSCH repetition type A enhancement	Apple

R1-2101407	PUSCH Repetitions for Coverage Enhancement	Indian Institute of Tech (H)

R1-2101477	Enhancements on PUSCH repetition type A	Qualcomm Incorporated

R1-2101520	PUSCH Repetition Type A Enhancement	Ericsson

R1-2101545	Enhancements on PUSCH repetition type A	Sharp

R1-2101641	Enhancements on PUSCH repetition type A	NTT DOCOMO, INC.

R1-2101656	Enhancements on PUSCH repetiton type A	Xiaomi

R1-2101679	Discussion on enhancements on PUSCH repetition type A	WILUS Inc.

R1-2101710	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

#### TB processing over multi-slot PUSCH

R1-2101018	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

[104-e-NR-CovEnh-02] Email discussion on TB processing over multi-slot PUSCH– Marco (Nokia)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Update on 2/1:

Agreement:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

Agreement:

- The same number of PRBs per symbol is allocated across slots for TBoMS transmission.

Proposal:

- Both consecutive and (as a working assumption) non-consecutive physical slots for UL transmission can be used for TBoMS for unpaired spectrum

  - FFS the details particularly related to how to handle non-consecutive physical slots, aiming for minimal/low additional specification impact/complexity w.r.t. the consecutive case

- Consecutive physical slots for UL transmission can be used for TBoMS for paired spectrum

  - FFS if non-consecutive physical slots for UL transmission are also supported for paired spectrum

  - FFS the case of SUL, e.g., in the SUL case.

Proposal 6

One or two of the following options will be considered to calculate NohPRB for TBoMS:

- Option 1: NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is performed and can be configured by xOverhead as in Rel-15/16.

- Option 2: NohPRB is calculated depending on both xOverhead and the number of symbols/slots over which the TBoMS transmission is performed.

FFS: if either the number of symbols or the number of slots is used.

FFS: if xOverhead is separately configured from the one in Rel-15/16.

Proposal 7

For TBoMS, the maximum supported TBS per HARQ process should not exceed legacy maximum supported TBS in Rel-15/16.

FFS: Details and further constraints.

R1-2100096	Discussion on TB processing over multi-slot PUSCH	ZTE

R1-2100173	Supporting TB  over multi-slot PUSCH	OPPO

R1-2100232	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

R1-2100398	Discussion on TB processing over multi-slot PUSCH	CATT

R1-2100458	Discussion on PUSCH TB processing over multiple slots	vivo

R1-2100666	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

R1-2100713	Discussions on TB processing over multi-slot PUSCH	LG Electronics

R1-2100732	TB processing over multi-slot PUSCH	InterDigital, Inc.

R1-2100743	Views on TB processing over multi-slot PUSCH	Fujitsu

R1-2100916	Discussion on TB processing over multi-slot PUSCH	China Telecom

R1-2100943	Discussion on TB processing over multi-slot PUSCH	NEC

R1-2101002	Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

R1-2101056	Discussion on TB processing over multi-slot PUSCH	CMCC

R1-2101128	Joint channel estimation for PUSCH	Xiaomi

R1-2101222	TB processing over multi-slot PUSCH	Samsung

R1-2101328	Design Considerations for TB processing over multi-slot PUSCH	Sierra Wireless, S.A.

R1-2101396	Discussion on TB processing over multi-slot PUSCH	Apple

R1-2101406	On TB processing over multiple slots for PUSCH	Indian Institute of Tech (H)

R1-2101478	TB processing over multi-slot PUSCH	Qualcomm Incorporated

R1-2101521	TB Processing over Multi-Slot PUSCH	Ericsson

R1-2101546	TB processing over multi-slot PUSCH	Sharp

R1-2101642	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

R1-2101646	Discussion on TB processing over multi-slot PUSCH	MediaTek Inc.

R1-2101680	Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

R1-2101711	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

#### Joint channel estimation for PUSCH

R1-2101522	Joint Channel Estimation for PUSCH	Ericsson

[104-e-NR-CovEnh-03] Email discussion on joint channel estimation for PUSCH– Jianchi (China Telecom)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Agreements:

- Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Agreements:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

Agreements:

- For joint channel estimation, define a time domain window is introduced to facilitate further discussion, during which UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS: whether the window should be specified

  - FFS: the length of the time domain window is defined by a set of repetitions/slots/symbols

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: the time domain window may or may not be configured or specified.

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS: Whether the window is determined by the power consistency and phase continuity requirements and/or by other factors is to be decided.

Agreements:

- Companies are encouraged to study optimization of DMRS granularity in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - Different DMRS density for different PUSCH transmissions

    - No DMRS for some PUSCH transmissions

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Companies are encouraged to study optimization of DMRS location in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - DMRS equally spaced among PUSCH transmissions

    - DMRS located in special slots

    - Orphan symbol used for DMRS

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Note: the simulation assumptions for DM-RS in TR 38.830 are used as baseline for performance evaluation on optimization of DMRS location/granularity in time domain.

  - Take into account impairments such as frequency offset, and report corresponding parametrization together with the results. Further discuss impairment details.

Working assumption:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for TB processing one TB processed over multiple slots

    - It’s subject to UE capability

R1-2100097	Discussion on joint channel estimation for PUSCH	ZTE

R1-2100174	Joint channel estimation for PUSCH	OPPO

R1-2100233	Discussion on Joint channel estimation for PUSCH	Huawei, HiSilicon

R1-2100399	Discussion on joint channel estimation for PUSCH	CATT

R1-2100459	Discussion on Joint channel estimation for PUSCH	vivo

R1-2100558	Potential techniques for PUSCH coverage enhancement	Potevio Company Limited

R1-2100667	Discussion on joint channel estimation for PUSCH	Intel Corporation

R1-2100714	Discussions on joint channel estimation for PUSCH	LG Electronics

R1-2100733	Discussions on joint channel estimation for PUSCH	InterDigital, Inc.

R1-2100797	Considerations on joint channel estimation over multi-PUSCH	Spreadtrum Communications

R1-2100917	Discussion on joint channel estimation for PUSCH	China Telecom

R1-2101003	Enhancements for DM-RS bundling for multiple PUSCH	Lenovo, Motorola Mobility

R1-2101020	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

R1-2101057	Discussion on joint channel estimation for PUSCH	CMCC

R1-2101223	Joint channel estimation for PUSCH	Samsung

R1-2101329	Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless, S.A.

R1-2101397	Discussion on joint channel estimation for PUSCH	Apple

R1-2101479	Joint channel estimation for PUSCH	Qualcomm Incorporated

R1-2101547	Joint channel estimation for PUSCH	Sharp

R1-2101643	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

R1-2101681	Discussion on joint channel estimation for PUSCH	WILUS Inc.

R1-2101712	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

### PUCCH enhancements

Including dynamic PUCCH repetition factor indication and DMRS bundling across PUCCH repetitions

R1-2100460	Discussion on PUCCH enhancements	vivo

R1-2101224	PUCCH enhancements	Samsung

[104-e-NR-CovEnh-04] Email discussion on PUCCH enhancements – Yi (Qualcomm)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Agreements: Down select from the following two options to support dynamic PUCCH repetition factor indication.

- Option 1 (without DCI enhancement): Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.

  - FFS details, e.g., via reusing the “PUCCH resource indicator” field (without increase # bits of it), starting CCE index (when applicable) of DCI,  by PDCCH aggregation level, etc.

  - FFS: RRC signaling enhancement details

- Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI

  - e.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication

  - FFS whether there is a need for RRC update

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling.

- FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16.

- Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible

Update on 1/31:

Agreements:

Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

- FFS: the configuration is per UE or per PUCCH resource.

- FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling

- FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

Conclusion: In Rel-17, deprioritize the study of DMRS pattern/location/granularity optimization for PUCCH coverage enhancement in AI 8.8.2. This conclusion could be revisited after the progress on the study of  DMRS pattern/location/granularity optimization for PUSCH coverage enhancement in AI 8.8.1.3.

R1-2100098	Discussion on coverage enhancements for PUCCH	ZTE

R1-2100175	PUCCH enhancements for coverage	OPPO

R1-2100198	PUCCH coverage enhancement	Huawei, HiSilicon

R1-2100400	Discussion on PUCCH enhancements	CATT

R1-2100668	Discussion on PUCCH enhancements	Intel Corporation

R1-2100715	Discussions on coverage enhancement for PUCCH	LG Electronics

R1-2100747	Discussions on PUCCH enhancements	InterDigital, Inc.

R1-2100798	Considerations on PUCCH coverage enhancement	Spreadtrum Communications

R1-2100918	Discussion on PUCCH enhancements	China Telecom

R1-2101021	Discussion on PUCCH enhancement for NR coverage enhancement	Panasonic Corporation

R1-2101058	Discussion on PUCCH enhancements	CMCC

R1-2101081	PUCCH enhancements	ETRI

R1-2101129	PUCCH enhancement	Xiaomi

R1-2101398	PUCCH coverage enhancement	Apple

R1-2101480	PUCCH coverage enhancements	Qualcomm Incorporated

R1-2101523	PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

R1-2101548	Dynamic PUCCH repetition factor indication	Sharp

R1-2101576	Enhancements for PUCCH repetition	Lenovo, Motorola Mobility

R1-2101626	PUCCH enhancements for coverage enhancements	NTT DOCOMO, INC.

R1-2101682	Discussion on PUCCH enhancements for coverage enhancement	WILUS Inc.

R1-2101713	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

### Type A PUSCH repetitions for Msg3

R1-2100099	Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

[104-e-NR-CovEnh-05] Email discussion on type A PUSCH repetitions for Msg 3 – Xianghui (ZTE)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Update on 1/31:

Agreements:

- For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below.

  - Option1: UL grant scheduling Msg3.

    - FFS details.

    - FFS fallbackRAR UL grant.

    - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported.

  - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI

    - FFS details.

  - Option3: SIB1 only

- Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively

Agreements:

- For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below.

lOption1: DCI format 0_0 with CRC scrambled by TC-RNTI.

nFFS details.

nAny modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI.

lOption2: Can be determined based on the repetition number  for  Msg3 initial transmission

Agreements:

Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission.

lFFS details, e.g., signaling etc.

Agreements:

For Msg3 PUSCH repetition,  the following options are considered, aiming for down-selection in RAN1#104b-e:

- Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request,

- A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- FFS details if any.

- Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request,

- gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition

- For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc.

- gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition.

- FFS details if any.

- Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS details if any.

- Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

  - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

  - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

  - FFS details if any.

- Other options are not precluded.

R1-2100176	Type A PUSCH repetitions for Msg3 coverage	OPPO

R1-2100197	Msg3 repetition for coverage enhancement	Huawei, HiSilicon

R1-2100401	Discussion on Type A PUSCH repetitions for Msg3	CATT

R1-2100461	Discussion on Type A PUSCH repetitions for Msg3	vivo

R1-2100490	Target of PUSCH Msg.3 coverage enhancements	SoftBank Corp.

R1-2100669	On Msg3 PUSCH repetition	Intel Corporation

R1-2100716	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

R1-2100748	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

R1-2100919	Discussion on type A PUSCH repetitions for Msg3	China Telecom

R1-2100944	Discussion on  PUSCH repetitions for Msg3	NEC

R1-2101022	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

R1-2101059	Discussion on type A PUSCH repetitions for Msg3	CMCC

R1-2101082	PUSCH coverage enhancement	ETRI

R1-2101130	Type A PUSCH repetitions for Msg3	Xiaomi

R1-2101225	Type A PUSCH repetitions for Msg3	Samsung

R1-2101399	Discussion on msg3 PUSCH repetition	Apple

R1-2101481	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

R1-2101524	Type A PUSCH Repetition for Msg3	Ericsson

R1-2101549	Type A PUSCH repetitions for Msg3	Sharp

R1-2101627	Type A PUSCH repetitions for Msg3 for coverage enhancements	NTT DOCOMO, INC.

R1-2101683	Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

R1-2101714	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

### Others

R1-2100100	Discussion on PUSCH and Msg3 enhancements for Redcap UEs	ZTE

R1-2100177	Other considerations for coverage enhancement	OPPO

R1-2100402	Views on reusing PUSCH enhancements for Msg3	CATT

R1-2100462	Enhanced Contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition			vivo

R1-2100749	Discussion on the number of HARQ processes for VoIP 	InterDigital, Inc.

R1-2100868	Association of dual polarized SSBs	Sony

R1-2101226	Discussion on PRACH enhancements for msg3 improvement	Samsung

R1-2101253	Views on handling potential overlaps with other WIs	Huawei, HiSilicon

R1-2101525	Other Coverage Enhancements for PUSCH	Ericsson

R1-2101715	On the scope of RAN1 LS on phase and power requirements for joint channel estimation/DM-RS bundling of PUSCH and PUCCH	Nokia, Nokia Shanghai Bell

## Rel-17 enhancements for NB-IoT and LTE-MTC (3)

Please refer to RP-201306 for detailed scope of the WI

R1-2101280	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC	Huawei, Ericsson

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2100253	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2100507	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2100567	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2100581	Consideration on CQI report and Repetition applicability for 16QAM in R17	MediaTek Inc.

R1-2100762	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2101324	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

R1-2101509	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

R1-2101698	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

[104-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2100254	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2100508	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2100568	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC		ZTE

R1-2101325	Design considerations to support 14-HARQ Feature for LTE-M	Sierra Wireless, S.A.

R1-2101510	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

R1-2101699	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson, AT&T, SoftBank, Telefónica, Verizon

[104-e-LTE-Rel17_NB_IoT_eMTC-02] Email discussion on support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC – Gerardo (Ericsson)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Support a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability

For HD-FDD Cat. M1 UEs in CE mode A only

R1-2100255	Support of a max DL TBS of 1736 bits in LTE-MTC	Huawei, HiSilicon

R1-2100509	Support of a maximum DL TBS of 1736 bits for eMTC	Nokia, Nokia Shanghai Bell

R1-2100569	DL TBS increase for eMTC	ZTE

R1-2100869	Support of 1736 bit maximum DL TBS for eMTC	Sony

R1-2101326	Design considerations to support DL TBS of 1736 bits for LTE-M	Sierra Wireless, S.A.

R1-2101511	Support of larger TBS for eMTC	Qualcomm Incorporated

R1-2101700	Support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson

[104-e-LTE-Rel17_NB_IoT_eMTC-03] Email discussion on support of a max TBS of 1763 bits – Martin (Sony)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Others

R1-2100570	Channel quality report for 16QAM in NB-IoT	ZTE

R1-2101278	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon

R1-2101701	Compendium of 16-QAM simulation results in UL and DL for NB-IoT	Ericsson

## Enhancements to Integrated Access and Backhaul (2)

Please refer to RP-201293 for detailed scope of the WI

R1-2101482	Workplan for Rel-17 IAB	Qualcomm Incorporated

### Enhancements to resource multiplexing between child and parent links of an IAB node

R1-2100219	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2100463	Enhancement to resource multiplexing between child and parent links	vivo

R1-2100670	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2100690	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

R1-2100717	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2100777	Enhancements for IAB resource multiplexing	AT&T

R1-2100833	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2100958	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2100990	Resource multiplexing in enhanced IAB systems	Lenovo, Motorola Mobility

R1-2101083	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2101227	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2101483	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2101628	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2101695	Resource multiplexing and DC in enhanced IAB	Ericsson

[104-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

R1-2100220	Enhancements for simultaneous operation of MT and DU	Huawei, HiSilicon

R1-2100464	Other enhancements for simultaneous operation of child and parent links	vivo

R1-2100671	Other Enhancements for Simultaneous Operations	Intel Corporation

R1-2100718	Discussions on other enhancement for simultaneous operation	LG Electronics

R1-2100744	Other enhancements for simultaneous operation of IAB-node’s child and parent links	Fujitsu

R1-2100778	Enhancements for IAB interference management	AT&T

R1-2100834	Other enhancements for simultaneous operation of IAB Parent and Child	Nokia, Nokia Shanghai Bell

R1-2100955	Discussion on simultaneous operation of IAB-node’s child and parent links	CEWiT

R1-2100959	Enhancements for simultaneous operation of child and parent links	ZTE, Sanechips

R1-2100991	Timing, interference, and power control in enhanced IAB systems	Lenovo, Motorola Mobility

R1-2101084	Discussion on simultaneous operation enhancements for eIAB	ETRI

R1-2101228	Enhancements to Timing, Power Control and CLI for NR IAB	Samsung

R1-2101484	On enhancements for simultaneous operation of IAB-node's child and parent links	Qualcomm Incorporated

R1-2101629	Other enhancements for simultaneous operation of IAB-node’s child and parent links	NTT DOCOMO, INC.

R1-2101696	Timing, CLI and power control in enhanced IAB	Ericsson

[104-e-NR-eIAB-02] Email discussion non other enhancements for simultaneous operation of IAB-node’s child and parent links – Luca (Qualcomm)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Others

R1-2100221	Other enhancements for IAB	Huawei, HiSilicon

R1-2100465	Other enhancements to Rel-17 IAB nodes	vivo

R1-2100960	Discussion on multiplexing capability for IAB	ZTE, Sanechips

R1-2101085	Other enhancements to eIAB	ETRI

## NR Sidelink enhancement (4)

Please refer to RP-202846 for detailed scope of the WI

R1-2101229	On Sidelink Enhacement Work Item	Samsung

From AI5

[104-e-NR-R17-SL-LS-01] Email discussion regarding LS in R1-2100021, including a possible reply LS till 2/1, to be handled under 8.11 (Boyuan Zhang, ZTE)

Update on 2/1: postpone to next meeting. Email thread is closed. Summary in R1-2102024.

### Resource allocation enhancement

R1-2100722	V2X channel model and scenario updates	GDCNI

R1-2101230	On Resource Allocation Enhacements	Samsung

#### Resource allocation for power saving

Including consideration of the impact of sidelink DRX, if any

[104-e-NR-R17-SL-01] Email discussion on resource allocation for power saving– Kevin (OPPO)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

R1-2101412	FL summary for AI 8.11.1.1 – resource allocation for power saving	Moderator (OPPO)

Agreements:

- Random resource selection is applicable to both periodic and aperiodic transmissions

  - FFS conditions for random resource selection

Conclusion:

- PSFCH reception is not included for Type A UE

- S-SSB reception is not included for Type A UE

- SL reception Type B is additionally added

  - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception

- Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- The resource selection window is [n+T1, n+T2]

  - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that

    - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- A minimum value for Y is (pre-)configured from a range of values, FFS details

- FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX)

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

R1-2100141	Power saving mechanism in NR sidelink	OPPO

R1-2100205	Sidelink resource allocation to reduce power consumption	Huawei, HiSilicon

R1-2100309	Considerations on partial sensing in NR V2X	CAICT

R1-2100351	Discussion on resource allocation for power saving	CATT, GOHIGH

R1-2100466	Resource allocation for sidelink power saving	vivo

R1-2100486	Power consumption reduction for sidelink resource allocation	FUTUREWEI

R1-2100492	Discussion on resource allocation for power saving	Zhejiang Lab

R1-2100517	Discussion on resource allocation for power saving	LG Electronics

R1-2100538	Sidelink resource allocation for power saving	Nokia, Nokia Shanghai Bell

R1-2100546	Resource allocation for power saving	TCL Communication Ltd.

R1-2100612	Resource allocation for sidelink power saving	MediaTek Inc.

R1-2100672	Design of sidelink power saving solutions	Intel Corporation

R1-2100687	Resource allocation mechanisms for power saving	Ericsson

R1-2100696	Discussion on Sidelink Resource Allocation for Power Saving	Panasonic Corporation

R1-2100701	NR Sidelink Resource Allocation for UE Power Saving	Fraunhofer HHI, Fraunhofer IIS

R1-2100745	Considerations on partial sensing and DRX in NR V2X	Fujitsu

R1-2100766	Sidelink resource allocation for Power saving	Lenovo, Motorola Mobility

R1-2100801	Discussion on sidelink resource allocation for power saving	Spreadtrum Communications

R1-2100870	Discussion on sidelink resource allocation for power saving	Sony

R1-2100924	Discussion on sidelink power saving	ZTE, Sanechips

R1-2100946	Discussion on resource allocation for power saving	NEC

R1-2100962	Discussion on resource allocation for power saving	Hyundai Motors

R1-2100981	Resource allocation for power saving	InterDigital, Inc.

R1-2101060	Discussion on resource allocation for power saving	CMCC

R1-2101086	Discussion on resource allocation for power saving	ETRI

R1-2101097	Discussion on sidelink resource allocation for power saving	Xiaomi

R1-2101231	On Resource Allocation for Power Saving	Samsung

R1-2101357	Sidelink Resource Allocation for Power Saving	Apple

R1-2101400	Discussion on Reduce Power Consumption for Sidelink	ROBERT BOSCH GmbH

R1-2101422	On NR Sidelink Resource Allocation for Power Saving	Convida Wireless

R1-2101485	Power Savings for Sidelink	Qualcomm Incorporated

R1-2101550	Discussion on resource allocation for power saving	Sharp

R1-2101572	Discussion on partial sensing and SL DRX impact	ASUSTeK

R1-2101630	Discussion on sidelink resource allocation for power saving	NTT DOCOMO, INC.

R1-2101663	Resource allocation for power saving with partial sensing in NR sidelink enhancement	ITL

#### Feasibility and benefits for mode 2 enhancements

[104-e-NR-R17-SL-02] Email discussion on feasibility and benefits for mode 2 enhancements– Seungmin (LGE)

- 1st check point: Jan 28

- 2nd check point: Feb 2

- 3rd check point: Feb 4

Conclusion:

- RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g.,  reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

  - The detailed observations can be found in the attachment of the LS

Further discuss the detailed observations (starting from the FL’s summary)

Agreements: Enclose following contents as an attachment of LS

======================================================================================

RAN1 has studied and evaluated schemes of inter-UE coordination in the following categories:

- Type A: UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

  - e.g., based on its sensing result

- Type B: UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

  - e.g., based on its sensing result and/or expected/potential resource conflict

- Type C: UE-A sends to UE-B the set of resources where the resource conflict is detected

Observations from evaluation results are summarized below. Note that the detailed evaulations for coordination schemes may not be fully aligned among companies. The details of the above schemes may also be different among companies in the evaluation. As a result, the observations drawn may be specific to the corrpresonding evaluated schemes with the assumed evaluation assumptions.

- Observations from evaluation results w/ signaling overhead and latency for the inter-UE coordination scheme:

- Type A

  - Periodic traffic

    - When UE-A sends to UE-B the set of resources used for UE-B’s transmission,

        - Source 1 (R1-2101941) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

    - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

        - Source 1 (R1-2101941) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

        - Source 5 (R1-2100925) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for broadcast

        - Source 6 (R1-2101911) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast under the scenario where UL transmission can overlap with SL transmission/reception

        - Source 2 (R1-2100673) observed that their coordination scheme using the Type A-like resource is not beneficial compared to Rel-16 Mode 2 RA for unicast

        - Source 3 (R1-2100746) observed that their coordination scheme using the Type A-like resource is not beneficial compared to Rel-16 Mode 2 RA for groupcast with SL HARQ-ACK feedback Option 1

  - Aperiodic traffic

    - When UE-A sends to UE-B the set of resources used for UE-B’s transmission,

        - Source 1 (R1-2101941) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

        - Source 2 (R1-2100673) observed that PRR loss of their coordination scheme using the Type A-like resource is shown compared to Rel-16 Mode 2 RA for unicast

    - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

        - Source 1 (R1-2101941) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

        - Source 6 (R1-2101911) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast under the scenario where UL transmission can overlap with SL transmission/reception

- Type B

  - Periodic traffic

    - Source 9 (R1-2101926) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

  - Aperiodic traffic

    - Source 12 (R1-2101804) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast

- Type C

  - Periodic traffic

    - Source 3 (R1-2100746) and Source 13 (R1-2101910) observed that their coordination scheme using the Type C-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast with SL HARQ-ACK feedback Option 1

- Type C

  - Aperiodic traffic

    - Source 2 (R1-2100673), Source 3 (R1-2100746), Source 12 (R1-2101804), and Source 13 (R1-2101910) observed that their coordination scheme using the Type C-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast with SL HARQ-ACK feedback Option 1

- Combination of Type B and C

  - Aperiodic traffic

    - Source 12 (R1-2101804) observed that their coordination scheme using a combination of Type B-like and C-like resources is beneficial compared to Rel-16 Mode 2 RA, Type B-like resource, and Type C-like resource, respectively for groupcast

    - Source 13 (R1-2101910) observed that their coordination scheme using a combination of Type B-like and C-like resources is beneficial compared to Rel-16 Mode 2 RA, Type B-like resource, and Type C-like resource, respectively for groupcast with SL HARQ-ACK feedback Option 1

      - Both signaling overhead and latency are considered for Type C-like resource, but only latency is considered for Type B-like resource

- Observations from evaluation results w/o signaling overhead and/or latency for the inter-UE coordination scheme:

- W/o signaling overhead, w/ latency

  - Type A

    - Periodic traffic

        - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

          - Source 7 (R1-2100352) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

    - Aperiodic traffic

        - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

          - Source 7 (R1-2100352) observed that their coordination scheme using the Type A-like resource is not beneficial compared to Rel-16 Mode 2 RA for unicast

  - Type B

    - Periodic traffic

        - Source 7 (R1-2100352) and Source 10 (R1-2100142) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

          - Source 10 (R1-2100142) observed that PIR gain of their coordination scheme using the Type B-like resource is shown for unicast

        - Source 11 (R1-2100828) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast

    - Aperiodic traffic

        - Source 13 (R1-2101910) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast with SL HARQ-ACK feedback Option 1

        - Source 7 (R1-2100352) observed that their coordination scheme using the Type B-like resource is not beneficial compared to Rel-16 Mode 2 RA for unicast

  - Combination of Type A and B

    - Periodic traffic

        - Source 7 (R1-2100352) observed that their coordination scheme using a combination of Type A-like and B-like resources is beneficial compared to Rel-16 Mode 2 RA, Type A-like resource, and Type B-like resource, respectively for unicast

  - Combination of Type A and B

    - Aperiodic traffic

        - Source 7 (R1-2100352) observed that their coordination scheme using a combination of Type A-like and B-like resources is not beneficial compared to Rel-16 Mode 2 RA for unicast

- W/ signaling overhead, w/o latency

  - Type A

    - Periodic traffic

        - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

          - Source 6 (R1-2101911) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

    - Aperiodic traffic

        - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

          - Source 6 (R1-2101911) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

  - Type B

    - Periodic traffic

        - Source 6 (R1-2101911) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

        - Source 6 (R1-2101911) observed that the gain of their coordination scheme using Type B-like resource becomes larger under the scenario where UL transmission can overlap with SL transmission/reception for unicast

        - Source 11 (R1-2100828) observed that their coordination scheme using the Type B-like resource with enhanced mechanism of UE-A selection is beneficial for groupcast

    - Aperiodic traffic

        - Source 6 (R1-2101911) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

        - Source 6 (R1-2101911) observed that their coordination scheme using the gain of Type B-like resource becomes larger under the scenario where UL transmission can overlap with SL transmission/reception for unicast

- W/o signaling overhead, w/o latency

  - Type A

    - Periodic traffic

        - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

          - Source 8 (R1-2101232) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

          - Source 3 (R1-2100746) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast with SL HARQ-ACK feedback Option 1

          - Source 4 (R1-2101786) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for broadcast

    - Aperiodic traffic

        - When UE-A sends to UE-B the set of resources used for UE-B’s transmission,

          - Source 2 (R1-2100673) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

        - When UE-A sends to UE-B the set of resources preferred for UE-B’s transmission,

          - Source 8 (R1-2101232) observed that their coordination scheme using the Type A-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

          - Source 2 (R1-2100673) observed that their coordination scheme using Type A-like resource is not beneficial compared to Rel-16 Mode 2 RA for unicast

  - Type B

    - Periodic traffic

      - Source 11 (R1-2100828) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

      - Source 11 (R1-2100828) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for groupcast

      - Source 9 (R1-2101926) observed that their coordination scheme using the Type B-like resource is beneficial compared to Rel-16 Mode 2 RA for unicast

R1-2100047	Views on resource allocation enhancements for sidelink communication	FUTUREWEI

R1-2100142	Inter-UE coordination in mode 2 of NR sidelink	OPPO

R1-2100206	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

R1-2100352	Discussion on feasibility and benefits for mode 2 enhancements	CATT, GOHIGH

R1-2100467	Discussion on mode-2 enhancements	vivo

R1-2100493	Inter-UE coordination for mode 2	Zhejiang Lab

R1-2100518	Discussion on feasibility and benefits for mode 2 enhancements	LG Electronics

R1-2100539	Inter-UE coordination in mode 2 sidelink resource allocation	Nokia, Nokia Shanghai Bell

R1-2100547	Feasibility and benefits for mode 2 enhancements	TCL Communication Ltd.

R1-2100606	Discussion on Mode 2 enhancements	MediaTek Inc.

R1-2100673	On feasibility and benefits of inter-UE coordination for sidelink mode-2 design	Intel Corporation

R1-2100688	Feasibility and benefits of mode 2 enhancements for inter-UE coordination	Ericsson

R1-2100702	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

R1-2100746	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

R1-2100767	Sidelink resource allocation for Reliability enhancement	Lenovo, Motorola Mobility

R1-2100802	Discussion on feasibility and benefit of mode 2 enhancements	Spreadtrum Communications

R1-2100828	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

R1-2100871	Discussion on reliability and latency enhancements for mode 2	Sony

R1-2100925	Discussion on inter-UE coordination	ZTE, Sanechips

R1-2100947	Discussion on feasibility and benefits for mode 2 enhancements	NEC

R1-2100963	Discussion on feasibility and benefits for mode 2 enhancements	Hyundai Motors

R1-2100982	On inter-UE coordination for Mode 2 enhancement	InterDigital, Inc.

R1-2101004	Mode 2 enhancements in sidelink	Panasonic Corporation

R1-2101061	Discussion on reliability and latency enhancements for mode-2 resource  allocation	CMCC

R1-2101087	Discussion on feasibility and benefits for mode 2 enhancements	ETRI

R1-2101098	Feasibility and benefits for mode2 enhancements	Xiaomi

R1-2101232	On Feasibility and Benefits for Mode2 Enhancements	Samsung

R1-2101358	Inter-UE Coordination for Mode 2 Resource Allocation	Apple

R1-2101401	Discussion on Sidelink Mode-2 Resource Allocation Enhancements	ROBERT BOSCH GmbH

R1-2101409	Inter-UE coordination for mode 2 enhancement	ITL

R1-2101423	On NR Sidelink Resource Allocation Mode 2 Enhancement	Convida Wireless

R1-2101486	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

R1-2101551	Discussion on feasibility and benefits for mode 2 enhancements	Sharp

R1-2101574	Discussion on V2X mode 2 enhancements	ASUSTeK

R1-2101631	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

R1-2101647	Feasibility and benefits for NR Sidelink mode 2 enhancements	CEWiT

### Others

Including remaining issues for sidelink evaluation methodology update for power saving, if any

[104-e-NR-R17-SL-03] Email discussion on 8.11.2 (remaining issues for sidelink evaluation methodology update for power saving) – Teng (CATT)

- 1st check point: Jan 28

- 2nd check point: Feb 3

Agreements:

- For commercial use case, at least following layout options are supported:

  - Option 3 of TR 36.843: Urban macro (500m ISD) (all UEs outdoor)

    - UE dropping as in Table A.2.1.1-1

      - All UEs are outdoors UEs

  - Option 1: Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell for optional

    - UE dropping as in Table A.2.1.1-1

      - Mix of outdoor and indoor UEs

  - Option 5 of TR 36.843: Urban macro (1732m ISD) for optional

    - UE dropping as in Table A.2.1.1-1

      - All UEs are outdoors UEs

      - Mix of outdoor and indoor UEs

Agreements:

- For public safety use case, at least the following options are supported for traffic model:

  - Option 2: VoIP model specified in TR 36.843

  - Option 4: FTP model 3 in TR 38.840 with packet size of 0.5Mbytes and mean inter-arrival time of 200ms

  - Option 7: Periodic traffic model 3 specified in TR 37.885

  - Option 9: VoIP model specified in TR36.843 with change of the value of outage definition into 0.01 and with packet delay budget of 75 ms

- Companies are encouraged to provide results for more than one traffic model including option 7

R1-2100143	Remaining issues in sidelink evaluation methdology for power saving	OPPO

R1-2100353	Remaining issues on sidelink evaluation methodology	CATT, GOHIGH

R1-2100468	Other aspects on SL enhancements	vivo

R1-2100519	Discussion on remaining aspects of sidelink evaluation methodology update for power saving	LG Electronics

R1-2100689	Remaining evaluation assumptions and methodology for power saving	Ericsson

R1-2100926	Discussion on remaining issues for sidelink evaluation methodology	ZTE, Sanechips

R1-2100983	On SL multi-carrier operation and remaining issues for simulation methodology update	InterDigital, Inc.

R1-2101099	Discussion on remaining issues of sidelink evaluation methodology	Xiaomi

R1-2101233	On Sidelink Issues and RAN1 Impacts	Samsung

R1-2101254	Physical layer impacts of sidelink DRX	Huawei, HiSilicon

## NR Multicast and Broadcast Services (3)

Please refer to RP-201038 for detailed scope of the WI

R1-2101062	Updated NR MBS work plan	CMCC

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

R1-2100048	Discussion on PDCCH aspects for group scheduling	FUTUREWEI

R1-2100106	Discussion on mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2100144	Group scheduling for NR Multicast and Broadcast Services	OPPO

R1-2100189	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2100354	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2100469	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

R1-2100510	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2100613	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2100674	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2100698	Views on group scheduling for NR MBS	Google Inc.

R1-2100768	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

R1-2100805	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2100872	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

R1-2100906	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2100956	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	ETRI

R1-2101063	Discussion on group scheduling mechanisms	CMCC

R1-2101234	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

R1-2101359	Discussion on group scheduling mechanism for RRC_connected UEs	Apple

R1-2101424	On group scheduling mechanism for NR multicast and broadcast	Convida Wireless

R1-2101487	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2101579	Discussion on group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2101658	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2101726	Mechanisms to support group scheduling for RRC_CONNECTED Ues	Ericsson

[104-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Fei (CMCC)

### Mechanisms to improve reliability for RRC_CONNECTED UEs

R1-2100049	Discussion on improving reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2100107	Discussion on mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2100145	UL feedback for RRC-CONNECTED UEs in MBMS	OPPO

R1-2100190	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2100355	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2100470	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2100511	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2100557	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio Company Limited

R1-2100614	Discussion on HARQ operation for NR MBS reliable transmission	MediaTek Inc.

R1-2100675	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2100699	Views on retransmission procedure for NR MBS	Google Inc.

R1-2100769	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2100806	Mechanisms to improve reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2100907	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2100957	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	ETRI

R1-2101064	Discussion on reliability improvement	CMCC

R1-2101235	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2101360	Discussion on MBS reliability improvement for RRC_connected UEs	Apple

R1-2101425	On reliability enhancement for NR multicast and broadcast	Convida Wireless

R1-2101488	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2101637	Study on the reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2101727	Discussion on reliability mechanisms for NR MBS	Ericsson

[104-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Jinhuan (Huawei)

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

R1-2100108	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs		ZTE

R1-2100146	Discussion on support for IDLE and INACTIVE state UEs	OPPO

R1-2100191	Discussion on multicast support for IDLE/INACTIVE UEs	Huawei, HiSilicon

R1-2100356	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			CATT, CBN

R1-2100471	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues			vivo

R1-2100512	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2100615	Common frequency resource for NR PTM transmission	MediaTek Inc.

R1-2100676	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2100770	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2100873	Considerations on MBS functions for RRC_IDLE UEs	Sony

R1-2100908	Basic function for broadcast/multicast	LG Electronics

R1-2101065	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2101236	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2101361	Discussion on MBS for RRC_IDLE/RRC_INACTIVE  UEs	Apple

R1-2101426	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2101489	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2101638	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

R1-2101721	Feature lead summary on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

R1-2101728	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

[104-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – David (BBC)

### Others

R1-2100109	Consideration on performance enhancement for RRC_IDLE/INACTIVE UEs	ZTE

R1-2100357	Discussion on other issues for MBS	CATT

R1-2100472	Other issues for Rel-17 MBS	vivo

R1-2100909	Other aspects for MBS	LG Electronics

R1-2101639	Effects of NR MBS on PDSCH and PDCCH	CHENGDU TD TECH LTD.

R1-2101729	Assumptions for Performance Evaluations of NR-MBS	Ericsson

R1-2101730	LTE SC-PTM design as a reference for NR MBS in IDLE/INACTIVE	Huawei, HiSilicon

## NR Dynamic spectrum sharing (DSS) (3)

Please refer to RP-193260 for detailed scope of the WI

Also include RAN1 impact from RP-201040 (LTE_NR_DC_enh2)

R1-2101560	Work plan for Rel17 WI on NR Dynamic spectrum sharing (DSS)	Ericsson

### Cross-carrier scheduling (from Scell to Pcell)

R1-2100110	Discussion on Cross-Carrier Scheduling from SCell to PCell	ZTE

R1-2100186	Discussion on cross-carrier scheduling from Scell to Pcell	OPPO

R1-2100193	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH	Huawei, HiSilicon

R1-2100358	Discussion on cross-carrier scheduling from Scell to Pcell	CATT

R1-2100473	Discussion on Scell scheduling Pcell	vivo

R1-2100677	On SCell scheduling PCell transmissions	Intel Corporation

R1-2100694	Discussion on cross carrier scheduling for NR DSS	NEC

R1-2100719	Om cross-carrier scheduling from SCell to Pcell	Nokia, Nokia Shanghai Bell

R1-2100794	Discussion on cross-carrier scheduling from SCell to PCell	Spreadtrum Communications

R1-2100885	Discussion on cross-carrier scheduling from SCell to Pcell	LG Electronics

R1-2100992	Cross-carrier scheduling (from Scell to Pcell)	Lenovo, Motorola Mobility

R1-2101066	Discussion on cross-carrier scheduling from SCell to Pcell	CMCC

R1-2101088	Cross-carrier scheduling from SCell to Pcell	ETRI

R1-2101100	Discussion on Cross-carrier scheduling from SCell to PCell	Xiaomi

R1-2101237	Cross-carrier scheduling from SCell to PCell	Samsung

R1-2101292	USS monitoring in sSCell and PCell	InterDigital, Inc.

R1-2101362	Views on Rel-17 DSS SCell scheduling PCell	Apple

R1-2101490	Cross-carrier scheduling from an SCell to the PCell/PSCell	Qualcomm Incorporated

R1-2101561	Enhanced cross-carrier scheduling for DSS	Ericsson

R1-2101632	Discussion on cross-carrier scheduling enhancements for NR DSS	NTT DOCOMO, INC.

R1-2101655	Discussion on cross-carrier scheduling from Scell to Pcell	ASUSTeK

[104-e-NR-DSS-01] Email discussion/approval for CCS – Ravi (Ericsson)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Multi-cell PDSCH scheduling via a single DCI

Focusing on study whether or not to support the feature first

R1-2100111	Discussion on Multi-cell PDSCH Scheduling via a Single DCI	ZTE

R1-2100187	Discussion on multi-cell PDSCH scheduling via a single DCI	OPPO

R1-2100194	Discussion on multi-carrier scheduling using single PDCCH	Huawei, HiSilicon

R1-2100359	Discussion on multi-cell PDSCH scheduling via a single DCI	CATT

R1-2100474	Discussion on joint scheduling	vivo

R1-2100611	On Multi-cell PDSCH Scheduling via Single DCI	MediaTek Inc.

R1-2100678	On 2-cell scheduling via single DCI	Intel Corporation

R1-2100720	On support of Single DCI scheduling two cells	Nokia, Nokia Shanghai Bell

R1-2100771	Discussion on multi-cell PDSCH scheduling via a single DCI	Lenovo, Motorola Mobility

R1-2100886	Discussion on multi-cell PDSCH scheduling via a single DCI	LG Electronics

R1-2101089	Discussion on multi-cell PDSCH scheduling via a single DCI	ETRI

R1-2101238	Considerations for scheduling on two cells using a single DCI format	Samsung

R1-2101293	On the support of single DCI scheduling multi-cell	InterDigital, Inc.

R1-2101363	Views on Rel-17 DSS Multi-cell PDSCH scheduling via a single DCI	Apple

R1-2101491	Multi-cell PDSCH scheduling via a single DCI	Qualcomm Incorporated

R1-2101562	Study on single DCI scheduling PDSCH on multiple cells	Ericsson

R1-2101633	Discussion on multi-cell PDSCH scheduling via a single DCI for NR DSS	NTT DOCOMO, INC.

R1-2101657	Discussion on multi-cell PDSCH scheduling via a single DCI	ASUSTeK

[104-e-NR-DSS-02] Email discussion/approval for multi-cell PDSCH scheduling via a single DCI – Haipeng (Lenovo)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

### Support efficient activation/de-activation mechanism for SCells in NR CA

R1-2100045	Support efficient activation/de-activation mechanism for Scells	FUTUREWEI

R1-2100112	Discussion on Support Efficient Activation De-activation Mechanism for SCells in NR CA			ZTE

R1-2100188	Discussion on efficient activation/de-activation for Scell	OPPO

R1-2100192	Discussion on efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

R1-2100360	Discussion on efficient activation and de-activation mechanism for SCell in NR CA	CATT

R1-2100475	Discussion on efficient activation/de-activation mechanism for Scells	vivo

R1-2100679	On efficient activation/de-activation for SCells	Intel Corporation

R1-2100695	Discussion on efficient activation mechanism for SCells	NEC

R1-2100721	On low latency Scell activation	Nokia, Nokia Shanghai Bell

R1-2100795	Discussion on efficient activation/de-activation mechanism for SCells in NR CA	Spreadtrum Communications

R1-2101067	Discussion on efficient activation/de-activation mechanism for SCells	CMCC

R1-2101239	On efficient activation/de-activation mechanism for Scells	Samsung

R1-2101294	Fast SCell Activation	InterDigital, Inc.

R1-2101364	On Efficiency Activation/De-activation for SCells in CA	Apple

R1-2101492	Efficient activation/de-activation mechanism for SCells in NR CA	Qualcomm Incorporated

R1-2101563	Reduced Latency SCell Activation	Ericsson

R1-2101566	Efficient activation/deactivation of SCell	ASUSTeK

R1-2101634	Discussion on efficient activation/deactivation mechanism for SCells	NTT DOCOMO, INC.

[104-e-NR-DSS-03] Email discussion/approval for efficient activation/de-activation mechanism for SCells in NR CA – Frank (Huawei)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

## Study on XR Evaluations for NR (RAN1) (2)

Please refer to RP-201145 for detailed scope of the WI

[104-e-NR-XR-01] Email discussion/approval for traffic model and capacity KPI – Eddy (Qualcomm)

- 1st check point: 1/28

- 2nd check point: 2/2

- 3rd check point: 2/4

Agreements: RAN1 adopts a parameterized statistical traffic model for evaluation of XR and CG, and KPI with details as shown below (RAN1 strives to agree on the remaining details during RAN1 #104e, based on SA4 input):

- There are M1 and M2 streams in DL and UL respectively

  - At least adopt the case where M1=1 & M2=1

  - FFS the values of M1 and M2, including the possibility of being application-dependent

- DL

  - Bitrate for video streaming

    - VR/AR: [60 Mbps (mandatory), 30 Mbps (optional)]

    - CG: [30 Mbps (mandatory), 45 Mbps (optional)]

      - FFS: other optional values

  - Air interface Packet Delay budget (PDB)

    - Air interface delay is measured from the point when a packet arrives at gNB to the point when it is successfully delivered to UE

    - Air interface PDB for video streaming

      - VR/AR: [10ms (mandatory), 20ms (optional)]

      - CG: [15ms (mandatory), 30ms (optional)]

        - FFS: other optional values

  - FFS: Frame-level/IP packet-level modeling for packet arrival, latency measure, etc.

  - FFS: Packet size, including the possibility of varying packet sizes

  - FFS: Packet Inter arrival time including the possibility of modeling jitter

- UL

  - FFS: Bitrate

  - FFS: Air interface Packet Delay budget (PDB)

  - FFS: Frame-level/IP packet-level modeling for packet arrival, latency measure, etc.

  - FFS: Packet size

- Per UE KPI

  - Baseline: A UE is declared a satisfied UE if more than X (%) of packets are successfully transmitted within a given air interface PDB. The exact value of X is FFS.

  - FFS: In addition to the baseline, the following additional method is FFS

    - When determining a XR/CG user is satisfied or not, the following factors are considered. FFS how to use those factors.

      - Packet loss information

      - Packet delay information

      - Some XR/CG source related information if they can be available within RAN, e.g. the mapping between packet and slices or frames and the packet importance

      - Multiple data streams traffic model

  - FFS if there are multiple streams (if adopted)

- FFS additional aspects not addressed above.

- Note 1: Companies are encouraged to provide details such as parameters (e.g., mean, STD, etc.), distributions, etc., by analyzing SA4 input, e.g., V/S/P traces

- Note 2: All FFS points above are to be further discussed in RAN1 #104e

Agreements

- Statistical traffic model for a single DL video stream for a single UE

  - The statistical traffic model for a single UE for a single DL video stream in Figure 1 is adopted, where a packet is assumed to represent multiple IP packets corresponding to a single video frame for modelling/evaluation purposes, e.g., traffic arrival, packet size, evaluation of latency and reliability.

- Frame per second (fps) for DL video stream for a single UE

  - 60 fps (baseline)

  - 120 fps (optional)

  - Other values, e.g., 30, 90 fps can be also optionally evaluated.

- Average data rate for DL video stream:

  - VR/AR: 30, 45 Mbps @60fps (baseline)

    - 30, 60 Mbps @60fps (optional)

    - Note: this is the aggregated data rate when applicable

  - CG: 8, 30 Mbps @60fps (baseline)

    - 8, 45 Mbps @60fps (optional)

  - Other values (in combination with fps) can be also optionally evaluated.

- Truncated Gaussian distribution is used for the packet size distribution of video stream for AR/VR/CG.

  - Other distribution is not precluded.

- (Working assumption) Parameters of Truncated Gaussian distribution for Packet size (note: these parameter values are those before the truncation)

  - Mean: Derived from average data rate and fps as follows.

    - (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes]

  - STD

    - TBD

  - Max packet size

    - TBD

  - Min packet size

    - TBD

    - FFS whether or not to use this parameter

[104-e-NR-XR-02] Email discussion/approval for other evaluation methodology and assumptions – Xiaohang (vivo)

- 1st check point: 1/28

- 2nd check point: 2/2

- 3rd check point: 2/4

Agreement: adopt following update for TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

  - Option 2: DDDUU

Detailed S slot format is 10D:2F:2U. Other S slot format(s) can also be optionally evaluated.

Further clarify that for option 2 for FR1/FR2, there is [2]-symbol gap at the end of third “D” slot of  DDDUU.

FFS whether or not to differentiate the two options (e.g., mandatory vs. optional)

Agreements: For XR evaluation, ideal channel estimation can be optionally evaluated.

Agreements:System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz, 2*100 MHz with CA

- FR2

  - Option 1: 100 MHz

  - Option 2: 400 MHz

Companies should report the CA setting if CA is adopted.

Other system bandwidth can also be optionally evaluated.

Agreements:For outdoor scenarios, the BS antenna parameters are as

- Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

- Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

Company to report the BS antenna parameters for XR/CG evaluation.

Other BS antenna parameters can also be optionally evaluated.

Agreements:For FR2, UE antenna parameters for XR/CG evaluations are as follows.

- Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

  - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

- Option 2 (from TR 38.802 – developed in Rel-14)

  - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Company to report the UE antenna parameters for XR/CG evaluation.

Other UE antenna parameters can also be optionally evaluated.

Agreements: For XR/CG evaluation, adopt following assumptions for BS height for Urban Macro

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| BS height | 25m |

Agreements:For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873.

|  |  | Urban Micro/Macro cell with high UE density (3D-UMi) /(3D-UMa) |
| --- | --- | --- |
| UE height (hUT) in meters | general equation | hUT=3(nfl – 1) + 1.5 |
|  | nfl for outdoor UEs | 1 |
|  | nfl for indoor UEs | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |

Agreements:At least for XR/CG capacity evaluation, for DL and UL

- Baseline: DL and UL performances are evaluated independently

- Optional: DL and UL performance are evaluated together

- FFS details both the baseline and the optional evaluations

### Traffic Model

R1-2100207	Discussion on applications and traffic model for XR and Cloud Gaming	Huawei, HiSilicon

R1-2101137	Traffic Model for XR and CG	MediaTek Inc.

R1-2100055	XR traffic model	FUTUREWEI

R1-2100132	Discussion on the XR traffic models for evaluation	OPPO

R1-2100361	XR traffic model	CATT

R1-2100476	Discussion on traffic models of XR	vivo

R1-2100528	Discussion on Traffic Model for XR evaluations	ZTE , Sanechips

R1-2100555	Discussion on traffic model for XR study	LG Electronics

R1-2100571	Discussion on XR applications and traffic models	InterDigital, Inc.

R1-2100680	On traffic model for XR	Intel Corporation

R1-2100724	On Traffic Model for XR study	Nokia, Nokia Shanghai Bell

R1-2100775	XR Traffic Model Considerations	AT&T

R1-2100879	Discussion on XR Applications and Evaluation Assumptions	Sony

R1-2101101	Discussion on Traffic model for XR evaluation	Xiaomi

R1-2101240	XR Applications and Traffic Models	Samsung

R1-2101314	Traffic model for XR	Ericsson

R1-2101365	Views on XR traffic models	Apple

R1-2101493	XR Traffic Models	Qualcomm Incorporated

R1-2101635	Discussion on traffic model for XR	NTT DOCOMO, INC.

### Evaluation Methodology

Including identification of KPIs of interest for relevant deployment scenarios

R1-2100477	Discussion on evaluation methodologies of XR	vivo

R1-2100056	XR evaluation methodology	FUTUREWEI

R1-2100133	Discussion on the XR evaluation methodology	OPPO

R1-2100242	Discussion on evaluation methodology for XR and Cloud Gaming	Huawei, HiSilicon

R1-2100362	Evaluation methodology and performance index for XR	CATT

R1-2100529	On XR Evaluation Methodology	ZTE , Sanechips

R1-2100556	Discussion on evaluation assumption for XR study	LG Electronics

R1-2100572	Discussion on Evaluation Methodology for XR	InterDigital, Inc.

R1-2100586	On Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2100681	On evaluation methodology for XR	Intel Corporation

R1-2100725	Development of the Evaluation Methodology for XR Study	Nokia, Nokia Shanghai Bell

R1-2100776	XR Evaluation Assumptions	AT&T

R1-2101102	Discussion on evaluation methodology for XR services	Xiaomi

R1-2101241	XR Evaluation Methodology and KPIs	Samsung

R1-2101315	Evaluation methodology for XR	Ericsson

R1-2101366	Views on XR evaluation methodology	Apple

R1-2101494	Evaluation Methodology for XR	Qualcomm Incorporated

R1-2101636	Discussion on evaluation methodology for XR	NTT DOCOMO, INC.

### Initial Performance Evaluation Results

Placeholder only – no email/GTW discussion

R1-2100363	Evaluation results of XR performance	CATT

R1-2100478	Initial performance evaluation results of XR	vivo

R1-2100530	Preliminary Performance Evaluation Results for XR	ZTE , Sanechips

R1-2100587	Initial Performance and Evaluation Results for XR and CG	MediaTek Inc.

R1-2100682	Initial results for XR	Intel Corporation

R1-2100726	Initial Performance Evaluation Results	Nokia, Nokia Shanghai Bell

R1-2101068	Discussion on XR evaluation	CMCC

R1-2101255	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2101316	Initial XR performance evaluation results	Ericsson

R1-2101495	Initial XR Performance Evaluation Results	Qualcomm Incorporated

### Others

R1-2100134	Discussion on the  support of XR/CG service in sidelink-unlicensed	OPPO

R1-2100364	Potential area of NR enhancement for the support of XR services	CATT

R1-2100479	Challenges and potential enhancements of XR	vivo

R1-2100531	Considerations on XR specific enhancements	ZTE , Sanechips

R1-2100573	Discussion on potential enhancements for supporting XR	InterDigital, Inc.

R1-2101103	Discussion on potential enhancement for supporting XR	Xiaomi

R1-2101269	Other aspects for XR and Cloud Gaming	Huawei, HiSilicon

R1-2101317	Mobility and XR applications	Ericsson

R1-2101367	Views on potential enhancements for XR	Apple

## Study on NB-IoT/eMTC support for Non-Terrestrial Network

Please refer to RP-202689 for detailed scope of the WI

R1-2100599	Rel-17 IoT NTN Work Plan	MediaTek Inc.

R1-2100774	Market expectations for IoT over NTN	NOVAMINT

R1-2101138	Skeleton TR 36.763 Study on NB-IoT / eMTC for NTN  (Release 17)	MediaTek Inc.

R1-2101139	Text Proposal for TR 36.673 chapter related to RAN1	MediaTek Inc.

### Scenarios applicable to NB-IoT/eMTC

R1-2100160	Discussion on scenarios applicable to NB-IoT/eMTC	OPPO

R1-2100225	Application scenarios of IoT in NTN	Huawei, HiSilicon

R1-2100248	Discussion on the scenarios and assumption for IoT-NTN	ZTE

R1-2100365	Applicable scenarios to NB-IoT/eMTC	CATT

R1-2100480	Discussion on scenarios applicable to NB-IoT/eMTC for NTN	vivo

R1-2100494	Scenarios applicable to NB-IoT/eMTC	Zhejiang Lab

R1-2100521	Discussion on NB-IoT NTN scenarios with small satellites / CubeSats	Sateliot, Gatehouse, Thales, Kepler

R1-2100533	Discussion on NB-IoT NTN scenarios with small satellites	Sateliot, Gatehouse

Withdrawn

R1-2100600	IoT NTN scenarios	MediaTek Inc.

R1-2100874	Scenarios for IoT-NTN	Sony

R1-2100930	On scenarios and evaluations for NB-IoT and eMTC based NTN	Ericsson

R1-2100975	Scenarios applicable to NB-IoT in NTN	Asia Pacific Telecom, FGI

R1-2101012	Feasibility of the large, single-beam small sats / CubeSats scenario	THALES

Withdrawn

R1-2101019	Feasibility of the large, single-beam small sats / CubeSats scenario	Thales, Sateliot, GateHouse

R1-2101027	Scenarios and evalutions for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2101069	Discussion on scenarios for IoT NTN	CMCC

R1-2101104	Discussion on the scenarios of NBIoT and eMTC over NTN	Xiaomi

Withdrawn

R1-2101146	IoT NTN Link Budget	Eutelsat S.A.

R1-2101242	On Scenarios applicable to NB-IoT/eMTC	Samsung

R1-2101368	On Link Budget of IoT NTN	Apple

R1-2101413	Discussion on scenarios for NB-IoT and eMTC over NTN	CAICT

R1-2101512	Scenarios applicable to NB-IoT/eMTC	Qualcomm Incorporated

### Enhancements to time and frequency synchronization

Including random access procedure/signals

R1-2100161	Discussion on enhancements to time and frequency synchronization	OPPO

R1-2100234	Discussion on time and frequency synchronization enhancement for IoT in NTN	Huawei, HiSilicon

R1-2100249	Discussion on the synchronization for IoT-NTN	ZTE

R1-2100366	Time and frequency synchronization for NB-IoT/eMTC	CATT

R1-2100481	Discussion on time and frequency synchronization enhancements on NB-IoT/eMTC for NTN			vivo

R1-2100601	UE Time and frequency Synchronisation for IoT NTN	MediaTek Inc.

R1-2100683	On synchronization for NB-IoT and eMTC NTN	Intel Corporation

R1-2100763	Time and frequency synchronization for IoT NTN	Lenovo, Motorola Mobility

R1-2100810	Consideration on enhancements to time and frequency synchronization	Spreadtrum Communications

R1-2100875	Time and frequency synchronisation enhancements for IoT-NTN	Sony

R1-2100931	On time and frequency synchronization enhancements for IoT NTN	Ericsson

R1-2100976	Time and frequency synchronization to NB-IoT in NTN	Asia Pacific Telecom, FGI

R1-2101028	Enhancements to time and frequency synchronization for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2101070	Discussion on enhancements for IoT NTN	CMCC

R1-2101105	Discussion on time and frequency synchronization for IoT NTN	Xiaomi

R1-2101243	On enhancements to time and frequency synchronization	Samsung

R1-2101369	Discussion on Time and Frequency Synchronization in IoT NTN	Apple

R1-2101402	Time/Frequency Synchronization for IoT NTN	InterDigital, Inc.

R1-2101513	Enhancements to time and frequency synchronization	Qualcomm Incorporated

R1-2101692	Discussion on enhancement of time and frequency synchronization for NB-IoT over satellite			Fraunhofer IIS, Fraunhofer HHI

### Timing relationship enhancements

R1-2100162	Discussion on timing relationship enhancements	OPPO

R1-2100235	Discussion on timing relationship enhancement for IoT in NTN	Huawei, HiSilicon

R1-2100250	Discussion on timing relationship for IoT-NTN	ZTE

R1-2100367	Timing relationship enhancement for NB-IoT/eMTC	CATT

R1-2100482	Discussion on timing relationship enhancements on NB-IoT/eMTC for NTN	vivo

R1-2100495	Timing relationship enhancements to support NB-IoT/eMTC in Non-Terrestrial Network	Zhejiang Lab

R1-2100602	Timing relationship enhancements	MediaTek Inc.

R1-2100684	On timing relationship for NB-IoT and eMTC NTN	Intel Corporation

R1-2100764	Timing relationship enhancements for IoT NTN	Lenovo, Motorola Mobility

R1-2100811	Consideration on timing relationship enhancements	Spreadtrum Communications

R1-2100876	Timing relationship for IoT-NTN	Sony

R1-2100932	On timing relationship enhancements for IoT NTN	Ericsson

R1-2100977	Timing relationship enhancements to NB-IoT in NTN	Asia Pacific Telecom, FGI

R1-2101029	Timing relationship enhancements for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2101106	Discussion on the timing relationship enhancement for IoT NTN	Xiaomi

R1-2101244	On timing relationship enhancements	Samsung

R1-2101370	Discussion on Timing Relationship Enhancement in IoT NTN	Apple

R1-2101403	On timing relationship enhancement for IoT NTN	InterDigital, Inc.

R1-2101514	Timing relationship enhancements	Qualcomm Incorporated

### Enhancements on HARQ

R1-2100163	Discussion on HARQ enhancements	OPPO

R1-2100236	Discussion on HARQ enhancement for IoT in NTN	Huawei, HiSilicon

R1-2100251	Discussion on HARQ for IoT-NTN	ZTE

R1-2100368	HARQ operation enhancement for NB-IoT/eMTC	CATT

R1-2100483	Discussion on HARQ enhancements on NB-IoT/eMTC for NTN	vivo

R1-2100603	Enhancement on HRQ	MediaTek Inc.

R1-2100685	On HARQ enhancements for NB-IoT and eMTC NTN	Intel Corporation

R1-2100765	HARQ enhancement for IoT NTN	Lenovo, Motorola Mobility

R1-2100812	Consideration on enhancements on HARQ	Spreadtrum Communications

R1-2100877	HARQ issues for IoT-NTN	Sony

R1-2100933	On HARQ enhancements for IoT NTN	Ericsson

R1-2100978	Enhancements on HARQ to NB-IoT in NTN	Asia Pacific Telecom, FGI

R1-2101030	HARQ for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2101107	Discussion on the HARQ enhancement for IoT NTN	Xiaomi

R1-2101245	On enhancements on HARQ	Samsung

R1-2101323	NTN IoT HARQ Considerations 	Sierra Wireless, S.A.

R1-2101371	Discussion on HARQ Enhancement in IoT NTN	Apple

R1-2101404	HARQ enhancement for IoT NTN	InterDigital, Inc.

R1-2101515	Enhancements on HARQ	Qualcomm Incorporated

### Others

R1-2100164	Discussion on other aspects	OPPO

R1-2100252	Discussion on additional enhancement for IoT-NTN	ZTE

R1-2100604	Others	MediaTek Inc.

R1-2100813	Consideration on other design aspects for IOT NTN	Spreadtrum Communications

R1-2100878	Power consumption of IoT-NTN	Sony

R1-2100934	On evaluation assumptions for NB-IoT and eMTC based NTN	Ericsson

R1-2100979	NB-IoT modifications to support the NTN deployment	Asia Pacific Telecom, FGI

R1-2101031	Link budget analysis for UE power class 6 for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2101108	Discussion on the other design aspects for IoT NTN	Xiaomi

R1-2101261	Other aspects to support IoT in NTN	Huawei, HiSilicon

R1-2101516	Other aspects for NTN IOT	Qualcomm Incorporated

## Introduction of DL 1024QAM for NR FR1 (1)

Please refer to RP-202886 for detailed scope of the RAN4-led WI, particularly the details to specify high order modulation for PDSCH.

R1-2100215	On support of DL 1024QAM for NR FR1	Huawei, HiSilicon

R1-2100369	DL 1024QAM for NR FR1	CATT

R1-2100484	On supporting DL 1024QAM for NR FR1	vivo

R1-2100532	Discussion on DL 1024QAM for NR FR1	ZTE , Sanechips

R1-2100686	Support of 1024QAM	Intel Corporation

R1-2101071	Discussion on DL 1024QAM for NR FR1	CMCC

R1-2101246	On remaining issues of DL 1024QAM for NR FR1	Samsung

R1-2101421	Support for NR DL 1024 QAM in FR1	Nokia, Nokia Shanghai Bell

R1-2101496	1024-QAM for NR PDSCH	Qualcomm Incorporated

R1-2101564	1024QAM for NR DL	Ericsson

[104-e-NR-1024QAM-01] Email discussion/approval for DL 1024QAM – Ajit (Ericsson)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

## Others

R1-2100297	Discussion on issues of NR codeword mapping	ZTE

# Closing of the meeting (Day 10: UTC 17:00 at the latest)