3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102339

E-meeting, April 12th – 20th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon, China Unicom

Title:	Discussion on CSI Enhancements for Rel-17

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1 #104e meeting, the following agreements are mode for CSI enhancements [1].

- FDD CSI enhancement

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where

- W1 is a free selection matrix, with identity matrix as special configuration

  - FFS polarization-common/specific selection

- Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1

  - At least one value of Mv>1 is supported

    - Decide on the value(s) of Mv, e.g. Mv=2,  in RAN1# 104bis-e

  - FFS candidate value(s)  of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf

- Wf can be turned off by gNB. When turned off, Wf  is an all-one vector (FFS; the length of all-one vector)

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay,

- W1 ∈ N^{P×K1} (K1≤P) is a port selection matrix in order to freely select K1 ports out of P CSI-RS ports or K1/2 ports out of P /2 CSI-RS ports

- Note that P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study).

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, study following options (or combinations) for CSI-RS configurations associated with Rel-17 PS codebook for supporting low CSI-RS overhead and/or CSI-RS processing complexity considering the impact on UPT performance under realistic CSI-RS measurement:

- Option 0: No further CSI-RS enhancement as the baseline

- Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25

- Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook

- Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

- CSI enhancements for MTRP

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis

- Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s).

  - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2.

  - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses

  - N CMR pairs are higher-layer configured by selecting from all possible pairs

    - signalling mechanism can be discussed further, e.g. using a bitmap

    - FFS: Whether MAC-CE or RRC+MAC CE indication is needed

    - FFS: how to support NCJT measurement hypotheses in FR2

- Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2

- Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- two RIs, two PMIs, two LIs and one CQI per codeword, for single-DCI based NCJT when the maximal transmission layers is less than or equal to 4

  - FFS: Maximal transmission layers larger than 4

  - FFS: Whether/how a subset of above reporting quantities are allowed to be configured to the UE

- FFS: whether/how to support two RIs, two PMIs, two LIs and two CQIs, for multi-DCI based NCJT

- FFS: whether/how to support CRI(s) to be reported in a CSI

- FFS: restrictions among reported CSI quantities, e.g. among reported RIs and PMIs

- FFS: whether/how to support non-PMI based port-selection

- FFS: whether/how to support single value of reported LI

Note that other NCJT CSI measurement/reporting enhancement for other scenarios is not precluded, e.g. for HST-SFN

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options:

- Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis

  - X = 0, 1, 2

    - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups

    - Support of X=1,2 is UE optional for the UE supporting option 1

  - FFS omission of CSI associated with NCJT measurement hypothesis

- Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses

  - FFS how to report recommended measurement hypothesis associated with that CSI report

In this contribution, we provide our views on CSI enhancement based on FDD angle and delay reciprocity including remaining issues of codebook structure, quantization of linear combination coefficients and mechanism to improve utilization of CSI-RS. Besides CSI enhancement for multi-TRP is also discussed in this contribution.

Rel-17 port selection codebook enhancement based on angle and/or delay reciprocity

Based on the agreement in RAN1#104e, W=W1W2WfH is supported as the codebook structure for R17 PS CB enhancement and some remaining issues are listed to be further studied. In this section, remaining issues of codebook structure and quantization are discussed in section 2.1. In section 2.2, mechanisms to improve utilization of CSI-RS is discussed.

Remaining issues of Rel-17 PS CB enhancement for Rank 1

W=W1W2WfH , where W1∈NP×K1, Wf∈CN3×Mv and W2∈CK1×Mv, is supported as the codebook structure for R17 PS CB enhancement, in which

- W1∈NP×K1 is a port selection matrix whereas each column of has only one elements of “1”. In addition, P is the CSI-RS ports number and K1 is the number of ports selected by UE.

  - Following legacy design principle of Rel-15/16 PS codebook, the UE may select dominant ports up to K1 ports based on UL channel. As we discussed later in Table 1, it is found that it is more efficient to use binominal coefficient to select K1 ports among total P ports so that  W2 can compress further as Rel-16, if need.

    - If further compression is conducted by W1, a bitmap with P (Polarization-specific) or P/2  (Polarization-common) bits may be needed for reporting selected coefficients so that such a bitmap may give rise to large CSI feedback overhead. At the mean time Mv coefficients must have to be selected jointly and may give rise to worse performance. So it is generally preferred to do further compression by W2 across spatial and frequency domains, if needed.

  - To avoid consistently reconfiguring CSI-RS resources for channel measurement and associated Rel-17 PS CB parameters, the value of K1 is configured by gNB based FDD channel reciprocity, either short/long term statistics or another implementation-based gNB consideration.

- Wf∈CN3×Mv is a DFT based compression matrix, in which N3=NCQISubband*R and Wfi,n=ej2πniN3, n=0,1,⋯,N3-1.

  - Wfi,n is limited within a DFT set with a size of N, which is configured/indicated by gNB based on delay reciprocity.

  - Similar to design principle of Rel-16 PS codebook, the UE can select and report dominant Mv vectors within the set configured/indicated by gNB. For N = 1, Mv equals to 1 so that there is no need to select and report DFT vectors in this case.

- W2∈CK1×Mv is a linear combination coefficients matrix.

  - Similar to Rel-16, a bitmap can be used for further compression, if need, by removing negligible coefficients across spatial and frequency domains based on selected K1 ports and Mv DFT vectors

  - By controlling compression ratios for reported non-zero coefficients, flexible balance between CSI feedback overhead and performance can be achieved at difference level since each non-zero coefficient will consume 7 bits.

In this section, we will focus on Rank 1 for remaining issues of codebook components W1, Wf and W2 for R17 PS CB.

- Remain issues of codebook structure for W1

For port selection matrix W1∈NP×K1, main remaining issues may include:

- Values of K1;

- Polarization-common or polarization-specific selection of W1;

- Indication mechanism by bitmap or binominal number for W1.

These first two issues can be discussed together as following:

- K1 ≤ P with polarization-common free selection: In addition K1 = P, K1 can be smaller than P. And UE considers two polarizations jointly so that selected ports have the same index for two polarizations and UE can freely select K1/2 ports out of P/2 ports.

- K1 ≤ P with polarization-specific free selection: In addition K1 = P, K1 can be smaller than P. And UE can freely select K1 ports out of P ports.

In order to make comparisons more straightforward, the third issue is discussed at first. UE can use bitmap or binomial co-efficient to report the selected ports. Some examples are calculated in Table 1 for polarization-specific and polarization-common. If considering a bitmap, the feedback overhead is always equal to P for polarization-specific and P /2 for polarization-common. For binomial coefficient, the feedback overhead is as follows:

L1=log2CPK1,    polarization-specificlog2(CP/2K1/2), polarization-common (1)

For binomial co-efficient, the feedback overhead is the largest when K1 = P/2 for polarization-specific and K1 /2 = P/4 for polarization-common.

| P | 32 | 24 | 16 | 12 | 8 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bitmap | 32 | 24 | 16 | 12 | 8 | 4 |
| binomial coefficient (CPP/2) | 30 | 22 | 14 | 10 | 7 | 3 |

(1)  Polarization-specific

| P | 32 | 24 | 16 | 12 | 8 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bitmap | 16 | 12 | 8 | 6 | 4 | 2 |
| binomial coefficient (CP/2P/4) | 14 | 10 | 7 | 5 | 3 | 1 |

(2) Polarization-common

Table 1 Feedback overhead of W1 with different method

Observation 1: Compared to use a bitmap, using binomial co-efficient can save at least a few bits of feedback overhead.

Based on above analysis, we have the following proposal for selection of W1.

Proposal 1: Binomial coefficient can be used for port-selection for W1 in R17 PS CB.

For the first two issues, “Relative UPT gain and CSI feedback overhead” is simulated and shown in Figure 1. For simplicity, we assume N=Mv=1 for Wf. There is no compression for W2 with 3bits amplitude and 4 bits phase for the quantization of linear combination coefficients. For W1, two schemes are simulated under 32 CSI-RS ports. More details about simulation assumption can be found in Table A-1 of Appendix A.

For the CSI feedback overhead, binomial co-efficient is used for both “Blue line” and “Purple line”, which is calculated as following:

CSI feedback overhead=L1+7*K1

Whereas L1 equals to the overhead for selected ports indexes shown above and 7*K1 means the overhead for linear combination coefficients, K1 equals to selected ports number, and “7” means 3bits for amplitude and 4bits for phase. The performance of Type I codebook is assumed as 100%

To compare the relative gain performance with same selected ports number (K1), if taking into account CSI feedback overhead, both polarization-common free selection and polarization-specific free selection have almost same performance and overhead trade-off.

Figure 1. Performance of different selection schemes for W1

Observation 2: If taking into account CSI feedback overhead, polarization-common free selection and polarization-specific free selection have almost identical performance and overhead trade-off.

Though many different values of K1 were simulated in Figure 1, e.g. K1 = {2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32}, the relative mean UPT gain corresponding to some values of K1 differ slightly. To distribute mean UPT gain equally as much as possible, values of 4, 8, 12, 16, 24 and 32 should be supported for K1, which is marked by red circle in the figure. Therefore we have following proposal:

Proposal 2: Polarization-common based free-selection should be supported for W1. Moreover, 4, 8, 12, 16, 24 and 32 can be considered as candidate values of K1.

- Remain issues of codebook structure for Wf

- Values of Mv and N

For the DFT basis, based the delay reciprocity for TDD/FDD, gNB can limit the FD bases used for Wf within a DFT set with a smaller size as N. As shown in Figure 2，by applying reciprocal delay information over beamformed CSI-RS, it is equivalent that gNB may shift the delay of each angle-delay pair to a specific delay position. Although multiple taps of channel may be continuous or discrete in delay domain, gNB can shift discretely distributed delays to the same location in delay domain, which corresponds to same FD base window across CSI-RS ports. Therefore a single FD base window with a small size of N,  configured/indicated by gNB, can be sufficient to counteract the effect of non-ideal delay reciprocity, if needed.

Figure 2. Illustration of applying delay reciprocity

Proposal 3: FD bases used for Wf quantitation are limited within a DFT window with size of N, which can be configured/indicated by gNB.

For the value of Mv and N, according to the RAN1#104e’s agreement, Mv = 1 has been supported and when turned off, Wf can be an all-one vector, which implicitly means that N=1 has also been supported. Furthermore, at least one value of Mv>1 should be supported and FFS value(s) of Mv>1. In order to decide the value(s) of Mv>1, the size of window, namely N, should be determined at first.

Figure 3 compares the performance with different values of N and we assume that N=Mv . Only DFT basis case is considered here. In the simulation, we assume K1 = P = 32 for W1, amplitude with 3bits and phase with 4bits for W2. Since K1 = P, there is no reporting bit for W1. Besides, a polarization-common bitmap (As discussed later, polarization-common bitmap is more straightforward) with length K1Mv/2 is used to indicate non-zero coefficients in W2 and log2NMv bits are required to report Wf. Note that FD bases used for Wfquantitation are limited within the adjacent N DFT vectors staring from i=0 for Wfi,n=ej2πniN3 in frequency domain, i.e. i=0:1:N   for simplicity.

It is shown in Figure 3 that although more candidate coefficients for selection are available by using larger values of Mv, larger reporting overhead will be incurred due to large bitmap and leads to worse trade-off between reporting payload and performance gain, especially at low overhead regime. Therefore, as shown in simulation results, Mv=N=2 can provide the best performance assuming the same overhead.

Figure 3. System level simulation between different values of N and Mv (1<Mv≤N) for R17 PS CB

Observation 3: For the case of N>1, N=2 can provide the better performance than other values assuming the same CSI feedback overhead.

Assuming that N=2, it is possible to select Mv=1 DFT. Thus further evaluation is needed to determine whether Mv=1 should be supported. It can be observed in Figure 4 that Mv=1 has better performance at low overhead.  For Mv=2, a large bitmap dominates the overhead at low overhead without improving the performance sufficiently enough. At the high overhead, Mv=2 has slightly better performance since larger Mv can offer more candidate coefficients for W2 to be selected and the overhead incurred by the bitmap for W2 contributes only a small portion of total reporting payload.

Figure 4. System level simulation between different values of Mv under N=2 for R17 PS CB [Update]

Observation 4: If N=2, Mv=1 can provide the best performance/overhead trade-off at low overhead whilst Mv=2 can provide the best performance/overhead trade-off at high overhead.

Based on above simulation results and analysis, we have the following proposal:

Proposal 4: N=2, Mv=1 and 2 should be supported for R17 PS CB. When N=2 and Mv=1, UE can report selected FD components within the set configured by gNB.

- Value(s) R and N3

According to agreements in #104e, Wf∈CN3× Mv and N3=NCQISuband*R, where  NCQISuband is the number of CQI sub-bands. R is the number of PMIs per CQI sub-band. In general, finer PMI can achieve better performance so that with higher resolution of PMI quantization, the benefit of larger R is more profound, especially for larger system bandwidth.

In Figure 5, different values of R are simulated based on R17 PS CB where we assume K1 = P = 32 and Mv=N=1. In our simulation, values of R correspond to the number of PMIs per CQI sub-band but also indicates the CSI-RS precoding granularity in frequency domain at gNB as  NPRBSBR implicitly, where NPRBSBis the PRB number in a sub-band which is determined in the Table5.2.1.4-2 in TS38.214 [2]. The performance gains of R = 4, 8, 16 compared with R = 2 for different bandwidth and overhead under both DFT and non-DFT basis are listed in Table2.

From Figure 5 and Table 2, it can be found that larger R have a better performance, in particular for large bandwidth. In addition, R=NPRBSB and R=NPRBSB2 have the same performance (e.g. 20MHz R=4 has the same performance with R=8), this is because the PDSCH precoding is same in the PRG size which equals to 2RBs.

| Bandwidth | DFT basis |  |  |
| --- | --- | --- | --- |
|  | R=4 | R=8 | R=16 |
| 20MHz | 2.2% | 2.2% | N.A. |
| 50MHz | 2.7% | 4.2% | 4.2% |
| 100MHz | 2.9% | 4.6% | 5.4% |

Table 2 Performance gains of larger R values compared with R=2

- 20MHz

- 50MHz

（3）100MHz

Figure 5. System level simulation between different values of R for R17 PS CB

Observation 5: Compared with R=2, larger R (R=4, 8, 16) can provide a better performance, especially in the case of large bandwidth for which the benefit of finer PMI quantization than CQI subband size is more profound.

Proposal 5: Larger R should be supported for R17 PS CB, with R= 2, 4, 8, 16.

- Remaining issues of codebook structure for  W2

Based on above selection of W1 and Wf, a bitmap can be used to further compress the CSI feedback overhead, so that UE can only report non-zero linear combination coefficients based on measurement results, which is similar with design principle of W2 in R16.

Considering following bitmap design:

- Polarization-common bitmap: UE can freely selects K0/2 non-zero coefficients out of  K1Mv/2  coefficients

- Polarization-specific bitmap: UE can freely selects K0 non-zero coefficients out of  K1Mv  coefficients

“Relative UPT gain and CSI feedback overhead” is simulated and shown as Figure 6. In the simulation, we assume P=32 and K1=16/32 for W1 and Mv=N=1 for  Wf.

(1) DFT basis，P=K1=32                                   (2) DFT basis，P=K1=16

Figure 6. Performance with different compression mode of bitmap

It can be found in Figure 6 that even though polarization-specific bitmap can provide better performance with the same number of selected non-zero linear combination coefficients, polarization-specific bitmap gives rise to larger CSI feedback overhead. Polarization-specific bitmap for  W2 has worse performance at low overhead especially, assuming the same feedback overhead.

Observation 6: Polarization-common bitmap for W2 can provide better performance gain assuming the same reporting overhead, e.g.  4.9% gain can be observed at low overhead for 32 ports.

For compression ratios of reported non-zero coefficients (like parameter β in R16), since gNB can implement compression implicitly based on FDD angle/delay reciprocity, larger β (e.g. β = 1) should be supported for R17. In addition to β = 1, some other values (e.g. 1/8, 1/2, 1/4, 3/4) can be considered as well to achieve different levels of performance/overhead/UE complexity trade-off, which are circled by the red rings in Figure 6.

Proposal 6: Polarization-common based bitmap for W2 should be supported for R17 PS CB. Considering that gNB can implement compression implicitly, β = 1 should be supported for R17 and other candidate values can be considered as well.

Analyzing mechanisms to improve utilization of CSI-RS

For PS codebook enhancements improving utilization of CSI-RS, the four options for CSI-RS configurations associated with Rel-17 PS codebook to support low CSI-RS overhead and CSI-RS  processing complexity are provided:

- Option 0: No further CSI-RS enhancement as the baseline

- Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25

- Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook

- Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

Between the above options, Option 1can be used to reduce CSI-RS overhead, and Option 2 and Option 3 can be used to provide higher and more flexible CSI-RS resource configurations. In order to compare CSI-RS overheads, we evaluate and calculate the CSI-RS overhead (Average CSI-RS ports number per 5ms per cell during the simulation time) of Option 0 (density 1 and density 0.5) and Option1 by system level simulation based on CSI-RS overhead calculation method agreed in RAN1#102e. And then the CSI-RS overhead is deducted from the performance (Relative Mean UPT gain) by the following formula, we assume that Y equals to 5 and 132 REs are used for PDSCH transmission.

MeanUPTwith CSI-RS overhead=MeanUPTwithout CSI-RS overhead(1-CSI-RS port Numberper Y ms per cell Y*RE NumberPDSCH per slot )

First,  three CSI-RS utilizing solutions are simulated and summarized in Table 3 assuming the same 32 DFT-based SD-FD beamforming pairs per CSI-RS resource, in which the first and the second solutions are supported in option 0, and the third solution has lower frequency domain density - 0.25.

| Solution | CSI-RS port number | Frequency density |
| --- | --- | --- |
| Option 0 | 32 | 1 |
| Option 0 | 32 | 0.5 |
| Option 1 | 32 | 0.25 |

Table 3. Details of different CSI-RS utilizing solutions for low CSI-RS overhead

In the simulation, we assume P=32 and K1=8, 12, 16, 24, 32 linear combination coefficients are choose with polarization-common for W1, and 4, 8, 12, 16, 24, 32 linear combination coefficients are choose with polarization-common by W2 and amplitude with 3bits, phase with 4bits for the quantization of linear combination coefficients for W2, and N=Mv=1 for Wf. Figure 7 gives the performance of relative gain taking account into CSI-RS overhead of different solutions with taking the gain of Type I as 100%.

Figure 7. Relative gain with CSI-RS overhead of different frequency domain density

From the simulation, we found that if taking account into CSI-RS overhead, Option 1 can provide 6% gain at low UCI payload and 5% gain at high UCI payload compared with Option 0 with density as 1. Compared to Option 0 with density as 0.5, about 2% gain can be observed at low UCI payload and about 1.5% gain can be observed at high UCI payload.

Observation 7: Lower CSI-RS density as 0.25 can provide better mean UPT gain if taking into account CSI-RS overhead:

- Compared to Option 0 with density as 1, about 6% gain can be observed at low UCI payload and about 5% gain can be observed at high UCI payload

- Compared to Option 0 with density as 0.5, about 2% gain can be observed at low UCI payload and about 1.5% gain can be observed at high UCI payload

Then in order to compared flexible configuration of CSI-RS resource between different solutions in Table 4. We take 32 SD-FD beamforming bases as an example, for Option 0 and 1, there are always 32 REs per RB. For Option 1 + Option2/3, we can have 8 REs, 16REs and 32 REs per RB, shown as Figure 8-a. Benefiting from the flexible REs number per RB, gNB have more flexibility to CSI-RS configuration.

| Solution | CSI-RS port number | Frequency density |
| --- | --- | --- |
| Option 0 | 32 | 1 |
| Option 0 | 32 | 0.5 |
| Option 1 | 32 | 0.25 |
| Option 1 + Option2/3 | 32 | 0.25 |

Table 4. Details of different CSI-RS utilizing solutions for flexible configuration of CSI-RS resource

32 port CSI-RS pattern 1             32 port CSI-RS pattern 2

8 port CSI-RS pattern 1             8 port CSI-RS pattern 2

- 32 ports and 8 ports CSI-RS pattern

- PDCCH + Front loaded DMRS           (2) PDCCH + Front loaded DMRS + TRS

- RE allocation for different scenarios

Figure 8. Examples of CSI-RS pattern and RE allocation for different scenarios

Taking four solutions in Table 4 as examples, configurable CSI-RS resources per slot is calculated in Table 5 for different scenarios.

- PDCCH + Front loaded DMRS: except 2 OFDM symbols for PDCCH and one OFDM symbols for Font loaded DMRS, the remaining REs can be used for CSI-RS, shown as Figure 8-b (1).

- PDCCH + Front loaded DMRS + TRS: except 2 OFDM symbols for PDCCH, one OFDM symbols for Font loaded DMRS and 6REs/RB for TRS, the remaining REs can be used for CSI-RS, shown as Figure 8-b (2).

Table 5. Number of configurable CSI-RS resources per slot for different scenarios

| Solution |  | PDCCH + Front loaded DMRS | PDCCH + Front loaded DMRS + TRS |
| --- | --- | --- | --- |
| Option 0 with density as 1 | Pattern 1 | 2*1 = 2 | 0*1 = 0 |
|  | Pattern 2 | 2*1 = 2 | 1*1 = 1 |
| Option 0 with density as 0.5 | Pattern 1 | 2*2 = 4 | 0*2 = 0 |
|  | Pattern 2 | 2*2 = 4 | 1*2 = 2 |
| Option 1 | Pattern 1 | 2*4 = 8 | 0*4 = 0 |
|  | Pattern 2 | 2*4 = 8 | 1*4 = 4 |
| Option 1 + Option2/3 | Pattern 1 | 11*1 = 11 | 9*1 = 9 |
|  | Pattern 2 | 15*1 = 15 | 9*1 = 9 |

From Table 5, it can be observed that compared with Option 0 and Option 1, Option 1 + Option2 or Option 1 + Option 3 can provide more configurable CSI-RS resources per slot.

Observation 8: Option2 or Option 3 can provide more configurable CSI-RS resources per slot due to more efficient usage of CSI-RS ports.

Based on the above simulation and analysis, in order to improve the utilization of CSI-RS, we have the following proposal.

Proposal 7: Option 1 + Option 2 or Option 1 + Option 3 should be supported to improve utilization of CSI-RS.

CSI Enhancement for Multi-TRP

In the following, we focus on CSI enhancement for NC-JT where CSI measurement is associated to one single reporting setting.

CSI Measurement Enhancement

Determination of CMR pairs

It was agreed in last meeting that N CMR pairs are high layer configured by selecting from all possible pairs. Assuming that two CMR groups with K1 CMRs and K2 CMRs respectively are configured for UE, how to differentiate these two groups of CMRs is questionable. A simple method is that the first K1  CMRs in the set belong to the first CMR group, and the remaining CMRs belong to the second CMR group. The value of K1  is configured to the UE.

As for CMR pair, the total number of possible pairs is K1K2. A straightforward approach is to directly select N CMR pairs by a bitmap with a length of K1K2 bits so that the bit (i-1)K2+j in the bitmap corresponds to a CMR pair determined by the i-th CMR in the first group and the j-th CMR in the second group implicitly, whereas 1≤i≤K1 and 1≤j≤K2. The total number of “1” in such a bitmap is N.

For candidate values of K1 and K2, in our view, they can be different since the most complexity of UE processing is determined by the value of N preconfigured by gNB. Therefore for the simplicity of specification and signaling design, it may be assumed that K1≤Ks and K1=Ks-K2.

For the maximal value of N, the value of 2 is preferred for two reasons.

- Firstly, in NCJT scheduling, at most two TRPs can be involved for NCJT based PDSCH transmission. The gNB implementation shall strive to narrow-down possible candidate coordination TRPs for NCJT, e.g.  TRP1 from serving cell + TRP2 from one coordination TRP or TRP1 from serving cell + TRP3 from another coordinate TRP, based on some long term channel measurement or TDD/FDD channel reciprocity. Supporting a larger value of N is not necessary for NCJT CSI feedback, even in a deployment scenario with more than 3 coordination TRPs. From gNB perspective, configuring more Rel-15/16/17 CSI reports with different CSI report mechanism to spread evenly CPU occupancy at time is more efficient to use limited CSI processing engine at the UE side.

- Secondly, a larger value of N leads to higher UE implementation complexity. Note that it was agreed to support 32 ports per CSI-RS resource for NCJT.  It is generally acknowledged that a joint CSI calculation based on a CMR pair under NCJT hypothesis is even more complicated than two separate CSI calculations under sTRP hypothesis. Further optimization at the UE side may be needed to exploit more accurate and suitable joint PMI for NCJT. Therefore unless there are compelling reasons and simulation results to show benefits of N > 2, we prefer to keep the value of N as small as possible.

Hence we have the following proposal:

Proposal 8: The maximal value of N is 2 for NCJT measurement hypothesis, when the UE is configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR.

Association between CMR pairs and IMR

Based on the agreement that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses, and there are N CMR pairs are higher-layer configured by selecting from all possible pairs for NCJT measurement hypothesis, it can be relatively straightforward to support one-to-one mapping between IMR and CMR/CMR pair by considering additional N dedicated CSI-IM resources for N CMR pairs (NCJT), from Rel-15/16 design, i.e. for a NZP CSI-RS resource set with Ks CMRs for single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, it is associated with Ks+N CSI-IM resources.

Based on above simple design principle, we have following proposal:

Proposal 9: A NZP CSI-RS resource set configured with Ks CMRs for single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses is associated to a CSI-IM resource set configured with Ks+N CSI-IM resources,

- Whereas the first Ks CSI-IM resources are associated with Ks CMRs one-by-one and remaining N CSI-IM resources are associated with N selected CMR pairs one-by-one.

In FR2, QCL-typeD assumption of CSI-IM resources should be determined. According to legacy L1-SINR and CQI measurement, QCL assumption of a CSI-IM resource is determined according the QCL assumption of the associated CMR. For example, if a CSI-IM resource is associated with a CMR, UE shall assume that the CSI-IM resource is QCLed with the CMR with respect to ‘QCL-typeD’. However, for the CSI-IM resource associated with a CMR pair, how to determine its QCL assumption needs to be studied and clarified further.

As shown in Figure 9, a CSI-IM resource is used to measure the inter-cell interference towards two CMRs in the CMR pair. Interference towards CMR1 is measured with spatial Rx parameter (Type D) of CMR1, i.e., with the same QCL-typeD assumption as CMR1. Interference towards CMR2 is measured with spatial Rx parameter of CMR2, i.e., with the same QCL-typeD assumption as CMR2. In other words, the same CSI-IM resource is measure with both QCL-typeD assumptions configured for CMR1 and CMR2. As the CSI-IM resource is only transmitted once in one measurement instance, the CSI-IM resource should be measured/received simultaneously with two QCL-typeD assumptions. Note that simultaneous reception with two spatial Rx parameters is under discussion in 8.1.2.3 and will be supported in R17.

Proposal 10: If a CSI-IM resource is associated with a CMR pair, UE shall assume that the CSI-IM resource is QCLed with two CMRs in that pair with respect to ‘QCL-typeD’.

Figure 9. Interference measurement of CMR pair

CSI Reporting Enhancement

CSI report quantity configurations

In Rel-15, the reportQuantity in a CSI-ReportConfig can be ‘none’, ‘cri-RI-PMI-CQI’, ‘cri-RI-i1’, ‘cri-RI-i1-CQI’, ‘cri-RI-CQI’, ‘cri-RSRP’, ‘ssb-Index-RSRP’, or ‘cri-RI-LI-PMI-CQI’. In Rel-16, two additional report quantities, i.e., ‘cri-SINR-r16’ and ‘ssb-Index-SINR-r16’ can be configured to support beam measurement.

It was agreed that for a CSI report associated with a NCJT measurement hypothesis, the UE is expected to report 2 RIs, 2 PMIs, 2 LIs, and 1 CQI per codeword. The performance gain achieved by NCJT relies on complete and accurate CSI report. However when reportQuantity is set to be ‘cri-RI-i1-CQI’, only wideband PMI is reported based on Type I codebook. It is unclear to how to use wideband-like CSI report for NCJT transmission. Therefore to simplify specification changes for NCJT CSI, we have following proposal:

Proposal 11: The report quantity is either ‘cri-RI-PMI-CQI’ or ‘cri-RI-LI-PMI-CQI’ if a CSI-ReportConfig is associated with NCJT measurement hypothesis in Rel-17.

UCI design

In Rel-15/16, each CRI codepoint can be used to uniquely determine a CSI measurement hypothesis. This principle can be reused in the mapping rule between the CRI codepoint and CMR/CMR pair in Rel-17. For a NZP CSI-RS resource set with Ks CMRs for single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, Ks+N  CRI codepoints can be assumed with corresponding bitwidth log2(Ks+N) in UCI.  To simplify the design, the CRI codepoint mapping rule should be unified in both reporting options, i.e. Option 1 and Option 2 agreed in RAN1 104e.

Therefore we have following proposal:

Proposal 12: For a CSI-RS resource set with Ks NZP CSI-RS resources for sTRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, each of Ks+N  CRI codepoints corresponds to one measurement hypothesis and total bitwidth for CRI in UCI is log2(Ks+N).

In Rel-15/16, the CSI can be carried by a 1-part or a 2-part UCI. Specifically, 1-part UCI is used to convey wideband CSI. In UCI, the bitwidth of PMI and LI depend on the reported RI. For example, assuming that 4Rx UE, allowed RI values {1,2,3,4}, 16-port CSI-RS, and Ks=2,  the bitwidth of 1-part CSI in Rel-15/16 is shown in Table 6. We can see that total bits under Rank = 1 can be different from total bits under other rank values, up to 2 bits. Consequently, the payload ambiguity of 1-part UCI leads to the unnecessary complexity at gNB. To address the payload ambiguity of UCI, zero padding bits are applied to a 1-part UCI.

Table 6: The bitwidth of CSI report in Rel-15/16 for 1-part UCI

|  | Rank = 1 | Rank = 2 | Rank = 3 | Rank = 4 |
| --- | --- | --- | --- | --- |
| CRI | 1 | 1 | 1 | 1 |
| RI | 2 | 2 | 2 | 2 |
| LI | 0 | 1 | 2 | 2 |
| PMI | 9 | 10 | 9 | 9 |
| CQI | 4 | 4 | 4 | 4 |
| Total bits of 1-part UCI | 16 | 18 | 18 | 18 |

Assuming that the same padding mechanism is reused to address the issue of payload variation, more padding bits are required but at the expense of much lower reporting efficiency of PUCCH transmission. For example, assuming that 4Rx UE, 16-port CSI-RS, Ks=2 and N = 1 in Table 6, it can be shown that the payload size variation for different ranks can be enormous, i.e., from the minimal payload with 17 bits to the maximal payload with 30bits, for Option 2.

Table 7: The bitwidth of CSI report in Rel-17 assuming 1-part UCI and Option 2.

|  | sTRP measurement hypothesis |  |  |  | NCJT measurement hypothesis |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rank = 1 | Rank = 2 | Rank = 3 | Rank = 4 | 1+1 | 1+2/2+1 | 2+2 |
| CRI | log2(Ks+N)=2 | 2 | 2 | 2 | 2 | 2 | 2 |
| RI | 2 | 2 | 2 | 2 | 1+1=2 | 1+1=2 | 1+1=2 |
| LI | 0 | 1 | 2 | 2 | 0 | 0+1/1+0=1 | 1+1=2 |
| PMI | 9 | 10 | 9 | 9 | 9+9=18 | 9+10=19 | 10+10=20 |
| CQI | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Total payload | 17 | 19 | 19 | 19 | 26 | 28 | 30 |

In addition, 1-part UCI is limited to wideband CSI which may not be useful at all for NCJT transmission. Hence we have the following proposal:

Proposal 13: 1-part CSI report is not supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis.

For a 2-part CSI report in Rel-15/16, CSI part 1 consists of CRI, RI, WB CQI and SB CQIs for the first TB and CSI part 2 consists of WB CQI and SB CQIs for the second TB, LI and PMI. For the sake of simplicity of gNB implementation, CSI part 1 is with the fixed payload determined by pre-configured CSI parameters. The payload of CSI part 2 can vary depending on reported content of CSI part 1, for example whether the UE reports CQI for the second TB in CSI part 2 or the bitwidth of PMI/LI conditioned on reported RI in CSI part 1.

Generally, above design principles can be reused in Rel-17 with minor clarifications for NCJT CSI reports. For Option 1, UCI reporting content for each of X sTRP hypotheses can be same as the counterparts in Rel-15/16 for CSI part 1. Additionally, UCI reporting content for NCJT hypothesis is composed of 1 CRI, 2 RIs, 1 WB CQI and SB CQIs for CSI part 1. Note that two RIs reported for NCJT hypothesis and X RI reported for X sTRP hypotheses are to determine the payload of CSI part 2.

For Option 2, UCI reporting content is the same as Rel-15/16 in CSI part 1 when reported CSI is associated with sTRP hypothesis. UCI reporting content is composed of 1 CRI, 2 RIs, 1 WB CQI, SB CQIs in CSI part 1 when reported CSI is associated with NCJT hypothesis. The variation of one or two RIs depending on hypotheses will not change payload (or marginally) in CSI part 1 based on our calculation in Table 7.

Therefore we have following proposal:

Proposal 14: For a 2-part CSI report associated with NCJT measurement hypothesis:

- For Option 1, CSI report part 1 comprises

  - One CRI, two RIs, one WB CQI, and SB CQIs for NCJT hypothesis and

  - X CRI, X RI, X WB CQI, and SB CQIs corresponding to each WB CQI, for X sTRP hypotheses respectively (if applicable)

- For Option 2, CSI report part 1 comprises

  - One CRI, one or two RIs, one WB CQI, SB CQIs

  - The remaining CSI are in CSI report part 2.

CSI processing criteria

From UE implementation perspective, the calculation of a CSI assuming NCJT measurement hypothesis may occupy 2 CPUs. Therefore the total number of CPUs required for given CSI report associating with a CSI-RS resource set with Ks NZP CSI-RS resources for sTRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, OCPU(n) , is given by:

- 2N, when reporting Option 1 with X=0 is configured

- Ks+2N, when reporting Option 1 with X=1 or 2 or Option 2 is configured.

Table 8: The number of occupied CPUs

| Ks | N | OCPU(n) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rel-15/16 | Opt. 1 with X=0 | Opt. 1 with X=1/2 or Opt.2 |
| 2 | 1 | 2 | 2 | 4 |
| 3 | 1 | 3 | 2 | 5 |
| 3 | 2 | 3 | 4 | 7 |
| 4 | 1 | 4 | 2 | 6 |
| 4 | 2 | 4 | 4 | 8 |

As shown in above table, the minimal required CPU for a NCJT report in Rel-17 is 2 when the UE is configured with reporting Option 1 with X=0, or 4 for other cases. However, typical chipset can support very limited CPUs per CC, e.g. 2 or 4 CPUs per CC for 16 ports CSI-RS resources for Rel-15 commercial UEs, so that Option 1 with X=1/2 or Option 2 becomes even more challenging in UE implementation, or unfriendly network limitation vice versa. Note that gNB normally has to configure periodic CSI reporting for basic sTRP transmission, which will occupy and reserve a certain amount of CPUs for given UE.

Observation 9: The CPU occupation or CSI dropping/updating rules for given NCJT CSI report should be enhanced and improved to mitigate burden of CPU occupancy for practical NW/UE implementation.

Conclusions

This contribution provides our views on CSI enhancement based on angle and delay reciprocity and CSI enhancement for multi-TRP. In summary, the following proposals and observations are made.

Proposal 1: Binomial coefficient can be used for port-selection for W1 in R17 PS CB.

Proposal 2: Polarization-common based free-selection should be supported for W1. Moreover, 4, 8, 12, 16, 24 and 32 can be considered as candidate values of K1.

Proposal 3: FD bases used for Wf quantitation are limited within a DFT window with size of N, which can be configured/indicated by gNB.

Proposal 4: N=2, Mv=1 and 2 should be supported for R17 PS CB. When N=2 and Mv=1, UE can report selected FD components within the set configured by gNB.

Proposal 5: Larger R should be supported for R17 PS CB, with R= 2, 4, 8, 16.

Proposal 6: Polarization-common based bitmap for W2 should be supported for R17 PS CB. Considering that gNB can implement compression implicitly, β = 1 should be supported for R17 and other candidate values can be considered as well.

Proposal 7: Option 1 + Option 2 or Option 1 + Option 3 should be supported to improve utilization of CSI-RS.

Proposal 8: The maximal value of N is 2 for NCJT measurement hypothesis, when the UE is configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR.

Proposal 9: A NZP CSI-RS resource set configured with Ks CMRs for single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses is associated to a CSI-IM resource set configured with Ks+N CSI-IM resources,

- Whereas the first Ks CSI-IM resources are associated with Ks CMRs one-by-one and remaining N CSI-IM resources are associated with N selected CMR pairs one-by-one.

Proposal 10: If a CSI-IM resource is associated with a CMR pair, UE shall assume that the CSI-IM resource is QCLed with two CMRs in that pair with respect to ‘QCL-typeD’.

Proposal 11: The report quantity is either ‘cri-RI-PMI-CQI’ or ‘cri-RI-LI-PMI-CQI’ if a CSI-ReportConfig is associated with NCJT measurement hypothesis in Rel-17.

Proposal 12: For a CSI-RS resource set with Ks NZP CSI-RS resources for sTRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, each of Ks+N  CRI codepoints corresponds to one measurement hypothesis and total bitwidth for CRI in UCI is log2(Ks+N).

Proposal 13: 1-part CSI report is not supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis.

Proposal 14: For a 2-part CSI report associated with NCJT measurement hypothesis:

- For Option 1, CSI report part 1 comprises

  - One CRI, two RIs, one WB CQI, and SB CQIs for NCJT hypothesis and

  - X CRI, X RI, X WB CQI, and SB CQIs corresponding to each WB CQI, for X sTRP hypotheses respectively (if applicable)

- For Option 2, CSI report part 1 comprises

  - One CRI, one or two RIs, one WB CQI, SB CQIs

  - The remaining CSI are in CSI report part 2.

Observation 1: Compared to use a bitmap, using binomial co-efficient can save at least a few bits of feedback overhead.

Observation 2: If taking into account CSI feedback overhead, polarization-common free selection and polarization-specific free selection have almost identical performance and overhead trade-off.

Observation 3: For the case of N>1, N=2 can provide better performance than other values assuming the same CSI feedback overhead.

Observation 4: If N=2, Mv=1 can provide the best performance/overhead trade-off at low overhead whilst Mv=2 can provide the best performance/overhead trade-off at high overhead.

Observation 5: Compared with R=2, larger R (R=4, 8, 16) can provide a better performance, especially in the case of large bandwidth for which the benefit of finer PMI quantization than CQI subband size is more profound.

Observation 6: Polarization-common bitmap for W2 can provide better performance gain assuming the same reporting overhead, e.g.  4.9% gain can be observed at low overhead for 32 ports.

Observation 7: Lower CSI-RS density as 0.25 can provide better mean UPT gain if taking into account CSI-RS overhead:

- Compared to Option 0 with density as 1, about 6% gain can be observed at low UCI payload and about 5% gain can be observed at high UCI payload

- Compared to Option 0 with density as 0.5, about 2% gain can be observed at low UCI payload and about 1.5% gain can be observed at high UCI payload

Observation 8: Option2 or Option 3 can provide more configurable CSI-RS resources per slot due to more efficient usage of CSI-RS ports.

Observation 9: The CPU occupation or CSI dropping/updating rules for given NCJT CSI report should be enhanced and improved to mitigate burden of CPU occupancy for practical NW/UE implementation.

- References

“RAN1 #104e Chairman’s Notes”, E-meeting, January 25th – February 5th, 2021.

3GPP TS 38.214 version 16.4.0 Release 16, Physical layer procedures for data.

- Appendix A.

Table A-1 SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro |
| Frequency Range |  | FR1 only, 2.1GHz, with duplexing gap of 200MHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ Other configuration is not precluded. |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 [21] |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank adaptation |
| CSI feedback |  | Feedback assumption: - CSI feedback periodicity (full CSI feedback) : 5ms - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| SRS Configuration |  | - SRS periodicity with 10ms - Comb: 2 - Number of OFDM symbols: 2 |
| SRS Error Model |  | SRS error Modelling in Table A.1-2 in 36.897. Δ=9dB and detailed derivation of Δ can be found in R1-144943[20]. |
| Calibration error model at gNB |  | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN amplitude error (expressed in decibels) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation for simulation bandwidth , respectively |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI reporting overhead |