3GPP TSG RAN WG1 #104b-e	                                                                              R1-2102557

e-Meeting, April 12th – 20th, 2021

Source:	OPPO

Title:	Discussion on NR Rel-16 UE features

Agenda Item:	7.2.11

Document for:	Discussion and Decision

- Introduction

In this contribution, we will present our views one a few remaining issues of NR Rel-16 NR URLLC, IIOT and eMIMO UE features.

# UE features of NR_L1enh_URLLC & NR_IIOT

FG 11-3c, FG 11-3d, FG 11-4d and FG 11-4e

It did not mention in what time granularity where the two PUCCH should be supported.

Proposal 1: For FG 11-3c, FG 11-3d, FG 11-4d and FG 11-4e, add “in the same subslot” to restrict the time granularity where the two PUCCH should be supported.

FG 11-3d and FG 11-4e:

It did not mention the “consecutive symbols” for supporting the two PUCCH. Without this restriction, it is logically wrong since there would be no missing case left for FG 11-3f and FG 11-4i which are supposed to support the non-consecutive case on top of FG 11-3d and FG 11-4e.

Proposal 2: For FG 11-3d and FG 11-4e, add the restriction of “consecutive symbols” for supporting the two PUCCH.

FG 11-3e and FG 11-3f

It is wrong to use the plural, since it is for a single codebook.

Proposal 3: For FG 11-3e and FG 11-3f, change the plural to singular.

FG 11-4c, FG11-4d, FG 11-4f and FG 11-4h:

It is for two codebooks where one of the two is sub-slot based codebook, but did not mention the other codebook is slot or sub-slot based codebook. Considering that they are all dependent on 11-4, which is for “Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based + sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE”, it can be derived that they are for slot-based + sub-slot-based case.

Proposal 4: For FG 11-4c, FG11-4d, FG 11-4f and FG 11-4h, add the restriction that they are for two codebooks where one of the two is sub-slot based codebook, and the other is slot-based codebook.

FG 11-4f: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for HARQ-ACK codebooks with one 2*7-symbol subslot based HARQ-ACK codebook.

It is for two codebooks, but the number “two” is missing.

Proposal 5:  For 11-4f, clarify it is for “two” codebooks.

FG 11-4h: 2 PUCCH transmissions in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot which are not covered by 11-4c and 11-4e.

It is for the others cases not covered by FG 11-4c and 11-4e, but FG 11-4h is for 2*7-symbol + 1 sub-slot based case or  2*7-symbol + 1 slot-based codebook case, while FG 11-4c is for 7*2-symbol case, and FG 11-4e is for two sub-slot based case, so there are no overlapping case. The case in FG 11-4h should be not cover by 11-4d and 11-4f.

Proposal 6:  Correct that FG 11-4h is to cover the missing case in 1-4d and 11-4f.

FG 11-4i: 2 PUCCH transmissions in the same subslot for two subslot based HARQ-ACK codebooks which are not covered by 11-4d and 11-4f.

It is for the others cases not covered by FG 11-4d and 11-4f, but FG 11-4i is for 2 sub-slot based codebook case, while FG 11-4d and 11-4f are for 1 sub-slot based and 1 slot-based codebook, so no overlapping. The case in FG 11-4i should be not cover by 11-4e and 11-4g.

Proposal 7:  Correct that FG 11-4i is to cover the missing case in 1-4e and 11-4g.

| 11. NR_L1enh_URLLC | 11-3c | 2 PUCCH of format 0 or 2 in the same subslot for a single 7*2-symbol subslot based HARQ-ACK codebook | 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-3 |
| --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-3d | 2 PUCCH of format 0 or 2 in consecutive symbols in the same subslot for a single 2*7-symbol subslot based HARQ-ACK codebook | 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-3 |
| 11. NR_L1enh_URLLC | 11-3e | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for a single 2*7-symbol HARQ-ACK codebooks | If the UE supports a 2*7-symbol subslot HARQ-ACK codebook, the UE also supports: 1) 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 and 4 in the same subslot | 11-3 |
| 11. NR_L1enh_URLLC | 11-3f | 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and 11-3e | If the UE supports a 2*7-symbol subslot HARQ-ACK codebook, the UE also supports: 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and 11-3e | 11-3 |

| 11. NR_L1enh_URLLC | 11-4c | 2 PUCCH of format 0 or 2 in the same subslot for two HARQ-ACK codebooks with one 7*2-symbol sub-slot based HARQ-ACK codebook and one slot based HARQ-ACK codebook | If the UE supports a 7*2-symbol subslot HARQ codebook, the UE also supports: 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-4 |
| --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-4d | 2 PUCCH of format 0 or 2 in consecutive symbols in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol sub-slot based HARQ-ACK codebook and one slot based HARQ-ACK codebook | If the UE supports a 2*7-symbol subslot HARQ codebook, the UE also supports: 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-4 |
| 11. NR_L1enh_URLLC | 11-4e | 2 PUCCH of format 0 or 2 in consecutive symbols in the same subslot for two subslot based HARQ-ACK codebooks | If the UE supports two subslot HARQ codebooks, the UE also supports: 1) 2 PUCCH format 0/2 in different symbols and once per subslot per codebook for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot per priority for SR | 11-4a |
| 11. NR_L1enh_URLLC | 11-4f | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot based HARQ-ACK codebook and one slot based HARQ-ACK codebook | If the UE supports a 2*7-symbol subslot HARQ-ACK codebook, the UE also supports: 1) 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 and 4 in the same subslot of the codebook | 11-4 |
| 11. NR_L1enh_URLLC | 11-4g | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for two subslot based HARQ-ACK codebooks | If the UE supports two subslot HARQ-ACK codebooks both configured with 2*7-symbols, the UE also supports: 1) 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 and 4 in the same subslot of a codebook | 11-4a |
| 11. NR_L1enh_URLLC | 11-4h | 2 PUCCH transmissions in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot and one slot based HARQ-ACK codebook which are not covered by 11-4cd and 11-4ef | If the UE supports two HARQ-ACK codebooks with one subslot based codebook with 2*7-symbol configuration, the UE also supports: 1) 2PUCCH transmissions in the same subslot of the codebook which are not covered by 11-4cd and 11-4ef | 11-4 |
| 11. NR_L1enh_URLLC | 11-4i | 2 PUCCH transmissions in the same subslot for two subslot based HARQ-ACK codebooks which are not covered by 11-4de and 11-4fg | If the UE supports two HARQ-ACK codebooks both with 2*7-symbol configuration, the UE also supports: 1) 2PUCCH transmissions in the same subslot of a codebook which are not covered by 11-4de and 11-4fg | 11-4a |

- UE features of eMIMO

In RAN1#104 meeting, the following conclusion on UE capability of RS used for beam management were agreed [1]:

Conclusion: - If one resource is used for one or multiple of BFD/RLM, it is counted as one (basic usage1) - If one resource is used for one or multiple of NBI/PL-RS/RSRP, add 1 (basic usage 2) - If one resource is used for L1-SINR in addition to basic usage 1 & 2, add N if referred N times by one or more CSI Reporting Settings

However, those conclusions are not captured in the Rel.16 NR UE features yet.  For the completeness of specification of Rel-16 UE features, we shall update the related FG to capture those conclusions so that the method of counting each resource used in beam management are clarified.

Proposal 8: Update the notes of FG 16-1a-1 to capture the agreed conclusion:

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of SSB/CSI-RS (1Tx) for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of CSI-IM/NZP IMR resources Other limitations: - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements | … | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {8, 16, 32, 64} Component 3: Candidate values {0, 4, 8, 16, 32, 64} Component 4: Candidate values {8, 16, 32, 64 , 128} Component 5: Candidate values {8, 16, 32, 64 , 128} Component 6: Candidate values {‘1 only’, ‘3 only’, ‘1 and 3’} Component 7: Candidate values {2, 4, 8, 16, 32, 64} Component 8: Candidate values: bitmap with entries {SSB as CMR with dedicated CSI-IM, SSB as CMR with dedicated NZP IMR, CSI-RS as CMR with dedicated NZP IMR configured, CSI-RS as CMR without dedicated IMR configured} If a UE supports FG 16-1a-1 it must support CMR(CSI-RS) + dedicated CSI-IM Note1: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note2: For component 4 and 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note3: For components 1, 2 and 3, CSI-RS resources configured as CMR without dedicated IMR are counted both as CMR and IMR Note4: For components 1, 2, 3, 7, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note5: For components 1, 2, 3, 7, if one SSB or CSI-RS resource used for L1-SINR measurement is referred N times in one or more CSI reporting settings with reportQuantity-r16= ssb-Index-SINR-r16 or cri-SINR-r16, that resource is counted as N times. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- |

Proposal 9: Update the notes of FG 16-1g and FG16-1g-1 to capture the agreed conclusion:

| 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | … | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: For RS configured for new beam identification, they are always counted regardless of beam failure event Note: The “configure to measure” RS (component1) only counts those in active BWP but the configured RS (component2) counts all configured including both active and inactive BWP Note: the reference slot duration is the shortest slot duration defined for the reported FR supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note: If one SSB or CSI-RS is used in one or more of BFD/RLM, it is counted as one (basic usage 1). Note: If one SSB or CSI-RS is used in one or more of NBI, PLRS or L1-RSRP, the counting for that resource is added by one (basic usage 2). Here L1-RSRP measurement includes the use cases of reportQuantity set to ‘ssb-Index-RSRP’ or ‘cri-RSRP’ or 'none'. Note: If one resource is used for L1-SINR measurement, the counting for this resource is added by N if this resource is referred N times in one or more CSI Settings with reportQuantity-r16= ‘ssb-Index-SINR-r16’ or ‘cri-SINR-r16’. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- |

| 16-1g-1 | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | … | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: This FG indicates the maximum number of resources across all FR(s) that are supported by the UE Note: The signalled values apply to the shortest slot duration defined in any FR(s) that are supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note: If one SSB or CSI-RS is used in one or more of BFD/RLM, it is counted as one (basic usage 1). Note: If one SSB or CSI-RS is used in one or more of NBI, PLRS or L1-RSRP, the counting for that resource is added by one (basic usage 2). Here L1-RSRP measurement includes the use cases of reportQuantity set to ‘ssb-Index-RSRP’ or ‘cri-RSRP’ or 'none'. Note: If one resource is used for L1-SINR measurement, the counting for this resource is added by N if this resource is referred N times in one or more CSI Settings with reportQuantity-r16= ‘ssb-Index-SINR-r16’ or ‘cri-SINR-r16’. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we presented our views on a few remaining issues of Rel-16 UE feature and the following proposals are provided:

Proposal 1: For FG 11-3c, FG 11-3d, FG 11-4d and FG 11-4e, add “in the same subslot” to restrict the time granularity where the two PUCCH should be supported.

Proposal 2: For FG 11-3d and FG 11-4e, add the restriction of “consecutive symbols” for supporting the two PUCCH.

Proposal 3: For FG 11-3e and FG 11-3f, change the plural to singular.

Proposal 4: For FG 11-4c, FG11-4d, FG 11-4f and FG 11-4h, add the restriction that they are for two codebooks where one of the two is sub-slot based codebook, and the other is slot-based codebook.

Proposal 5:  For 11-4f, clarify it is for “two” codebooks.

Proposal 6:  Correct that FG 11-4h is to cover the missing case in 1-4d and 11-4f.

Proposal 7:  Correct that FG 11-4i is to cover the missing case in 1-4e and 11-4g.

Proposal 8: Update the notes of FG 16-1a-1 to capture the agreed conclusion:

Proposal 9: Update the notes of FG 16-1g and FG16-1g-1 to capture the agreed conclusion:

- Reference

R1-2102051 Summary of email discussion/approval [104-e-NR-UEFeatures-eMIMO-04] AT&T