3GPP TSG RAN WG1 #104b-e		                                                      R1-2102585

e-Meeting, April 12th – 20th, 2021

Source:	CATT

Title:	Correction on half-duplex operation in CA with unpaired spectrum

Agenda Item:	7.2.12

Document for:	Discussion and Decision

Introduction

At the RAN1#104-e meeting, several issues on half-duplex operation were discussed and no conclusion was reached. In this contribution, we discuss our understandings on half-duplex operation in CA with unpaired spectrum.

Discussion

In 38.306, the UE capability parameter for half-duplex operation is defined as below. As highlighted in yellow, the UE can include half-DuplexTDD-CA-SameSCS-r16 only if  simultaneousRxTxInterBandCA is not present.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| half-DuplexTDD-CA-SameSCS-r16 Indicates whether the UE supports directional collision handling between reference and other cell(s) for half-duplex operation in TDD CA with same SCS. The UE can include this field, only if simultaneousRxTxInterBandCA is not present. | BC | No | TDD only | N/A |

It is not clear whether half-duplex operation is applicable to intra-band CA and the following alternatives were discussed in last meeting:

  - Alt.1: Directional collision handling is also applicable to CCs within a frequency band

    - Alt.1-1: “Not supporting simultaneousRxTxInterBandCA” should not be prerequisite of half-DuplexTDD-CA-SameSCS-r16

    - Alt.1-2: “Not supporting simultaneousRxTxInterBandCA” should be kept as prerequisite of half-DuplexTDD-CA-SameSCS-r16 only for the case with inter-band pair of cells

  - Alt.2: Directional collision handling is not applicable to CCs within a frequency band

During the discussion of this feature, intra-band CA was supported as shown in Appendix. In the last meeting, it was discussed that the prerequisite is also fine for intra-band CA since UE cannot include simultaneousRxTxInterBandCA for a band combination that is intra-band only so that UE can indicate half-DuplexTDD-CA-SameSCS-r16 for this band combination and directionalCollisionHandling-r16 can be configured. In case of a mix of intra- and inter-band CA, if simultaneousRxTxInterBandCA is included, the UE cannot indicate half-DuplexTDD-CA-SameSCS-r16 so that collision handling cannot be applied to intra-band CCs. Therefore, we have the following proposals.

Proposal 1: Rel-16 collision handling is applicable to TDD intra-band CA

- UE can report half-DuplexTDD-CA-SameSCS-r16 for a band combination that is intra-band only;

- UE can report half-DuplexTDD-CA-SameSCS-r16 in case of mix of intra- and inter-band CA if simultaneousRxTxInterBandCA is not included.

Another issue for half-duplex operation is that whether the parameter directionalCollisionHandling-r16 should be configured per serving cell or per cell group. According to the discussion in last meeting, companies prefer that the parameter directionalCollisionHandling-r16 is configured per serving cell. A text proposal is provided below for half-duplex operation in CA with unpaired spectrum.

Proposal 2: Adopt the following TP for half-duplex operation in CA with unpaired spectrum.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

- 11.1	Slot configuration

<Unchanged text omitted>

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells.

For a set of symbols of a slot corresponding to a valid PRACH occasion and  symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

If a UE is scheduled by a DCI format to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot.

If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE determines a reference cell for a symbol as an active cell with the smallest cell index among serving cells that are configured with directionalCollisionHandling-r16 where the symbol is configured as

-	downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

-	uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

-	downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol

If a UE

-	is configured with multiple serving cells in a frequency band and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE does not expect

-	a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated,

-	tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and

-	to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell.

If the reference cell and another cell for a UE operate in different frequency bands and if the UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE

-	UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell among the cells configured with directionalCollisionHandling and as uplink or downlink for the reference cell, respectively,

-	transmits a signal/channel scheduled by a DCI format on a symbol of the other cell among the cells configured with directionalCollisionHandling when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell,

-	is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell among the cells configured with directionalCollisionHandling.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any cell from the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

the UE

-	does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell

-	does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell

-	does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell

-	does not transmit a SRS that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell

-	does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell

-	assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell among the cells configured with directionalCollisionHandling to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell

-	does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively

After the UE applies the procedures described above for directional collision handling within the set of cells that have been configured with directionalCollisionHandling-r16, the UE does not expect any directional collision (with including the effects of timing advance) across any pair of cells, irrespective of whether either or both of those cells were configured with directionalCollisionHandling-r16 if the UE is not capable of simultaneous transmission and reception on the pair of cells.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Conclusion

In this contribution, we discuss our understanding on half-duplex operation and give the following proposals:

Proposal 1: Rel-16 collision handling is applicable to TDD intra-band CA

- UE can report half-DuplexTDD-CA-SameSCS-r16 for a band combination that is intra-band only;

- UE can report half-DuplexTDD-CA-SameSCS-r16 in case of mix of intra- and inter-band CA if simultaneousRxTxInterBandCA is not included.

Proposal 2: Adopt the following TP for half-duplex operation in CA with unpaired spectrum.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

- 11.1	Slot configuration

<Unchanged text omitted>

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells.

For a set of symbols of a slot corresponding to a valid PRACH occasion and  symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

If a UE is scheduled by a DCI format to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot.

If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE determines a reference cell for a symbol as an active cell with the smallest cell index among serving cells that are configured with directionalCollisionHandling-r16 where the symbol is configured as

-	downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

-	uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

-	downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol

If a UE

-	is configured with multiple serving cells in a frequency band and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE does not expect

-	a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated,

-	tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and

-	to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell.

If the reference cell and another cell for a UE operate in different frequency bands and if the UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE

-	UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell among the cells configured with directionalCollisionHandling and as uplink or downlink for the reference cell, respectively,

-	transmits a signal/channel scheduled by a DCI format on a symbol of the other cell among the cells configured with directionalCollisionHandling when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell,

-	is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell among the cells configured with directionalCollisionHandling.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable'with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and

-	is not capable of simultaneous transmission and reception on any cell from the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability for the multiple serving cells, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

the UE

-	does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell

-	does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell

-	does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell

-	does not transmit a SRS that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell

-	does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell

-	assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell among the cells configured with directionalCollisionHandling to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell

-	does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively

After the UE applies the procedures described above for directional collision handling within the set of cells that have been configured with directionalCollisionHandling-r16, the UE does not expect any directional collision (with including the effects of timing advance) across any pair of cells, irrespective of whether either or both of those cells were configured with directionalCollisionHandling-r16 if the UE is not capable of simultaneous transmission and reception on the pair of cells.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Appendix

The following are agreements for half-duplex in CA agreed in email discussion after RAN1#95, it can be seen that some cases are defined as error cases in intra-band CA, which means the other cases which does not distinguish intra-band and inter-band can be applied for both intra-band and inter-band CA.

Agreements: For a UE configured with multiple serving cells in a band or band combination, where the UE is not capable of simultaneous transmission and reception, support a behavior in the following table:

- FFS: other combinations of Semi SFI, RRC and Dynamic

- FFS: behaviour for SSB in RRM measurements

- FFS: down-select further between Alt1 and Alt2. Down-selection may be performed independently for case 3 and 6.

- For the agreed behaviour introduce a new feature as “mandatory with capability”

  - Note: Sep./Dec. UEs are still able to operate HD TDD CA if network ensures same transmission direction across all the serving cells.

- Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell.

| No | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 1 | Semi SFI D | Semi SFI U | Allowed to drop U for inter-band Error case in intra-band | Dropping U on other cell Error case in intra-band |
| 2 | Semi SFI D | RRC U | Allowed to drop U | Dropping on other cell |
| 3 | Semi SFI D | Dynamic U | Alt 1: Allowed to drop D for inter-band Error case in intra-band | Overriding semi SFI D to F on reference cell for the UE |
|  |  |  | Alt 2: Error | Error |
| 4 | Semi SFI U | Semi SFI D | Allowed to drop D for inter-band Error case in intra-band | Dropping D on other cell Error case in intra-band |
| 5 | Semi SFI U | RRC D | Allowed to drop D | Dropping on other cell |
| 6 | Semi SFI U | Dynamic D | Alt 1: Allowed to drop U for inter-band Error case in intra-band | Overriding semi SFI U to F on reference cell for the UE |
|  |  |  | Alt 2: Error | Error |
| 7 | RRC D | RRC U | Allowed to drop U | Dropping on other cell |
| 8 | RRC U | RRC D | Allowed to drop D | Dropping on other cell |
| 9 | Dynamic D | Dynamic U | Error | Error |
| 10 | Dynamic U | Dynamic D | Error | Error |

Note, in above tables, the terminology is as follows:

- Semi SFI D and U:  D and U symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated

- Semi SFI F: flexible symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, when provided to a UE, or when TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated are not provided to the UE

- RRC D: symbols corresponding to a higher-layer configured PDCCH, or a PDSCH, or a CSI-RS on semi SFI F of the same cell

- RRC U: symbols corresponding to a higher-layer configured SRS, or PUCCH, or PUSCH, or PRACH on semi SFI F of the same cell

- Dynamic D and U: symbols scheduled as D and U by DCI formats other than DCI format 2_0 on semi SFI F of the same cell

- Note: there is no intention to change the existing behavior w.r.t. how to update the reference cell

Agreement:

- For case 3, down-select Alt 1

- For case 6, down-select Alt 2

- Note: above down-selection applies to NR CA only

For the remaining cases, adopt the following behavior:

| No | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 11 | RRC U | Semi SFI D | Allowed to drop D | Dropping on other cell |
| 12 | Dynamic U | Semi SFI D | Allowed to drop D | Dropping on other cell |
| 13 | RRC D | Semi SFI U | Allowed to drop U | Dropping on other cell |
| 14 | Dynamic D | Semi SFI U | Allowed to drop U | Dropping on other cell |
| 15 | RRC U | Dynamic D | Alt 2: Error |  |
| 16 | RRC D | Dynamic U | Alt 1: Allowed to drop D for inter-band Error case in intra-band |  |
| 17 | Dynamic U | RRC D | Allowed to drop D | Dropping on other cell |
| 18 | Dynamic D | RRC U | Allowed to drop U | Dropping on other cell |

Agreement:

In symbols indicated in ssb-PositionsInBurst on reference and other cell(s), half-duplex CA UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot.

Agreement:

In R15, behavior for mobility CSI-RS related to HD CA is not specified.

Agreement:

Half-duplex CA UE determines reference cell per symbol as a cell with the lowest cell ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U

- Note: this overrides earlier agreements ” Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell. “

- Note: Agreed cases 12, 14, 17 and 18 are not needed anymore

Note: Agreed cases 9 and 10 should apply to collisions between two cells irrespective of a cell being reference or other