3GPP TSG RAN WG1 Meeting #104b-e 	R1-2102694

e-Meeting, April 12th – 20th, 2021

Agenda Item: 8.3.1.1

Source: MediaTek Inc.

Title: On UE feedback enhancements for HARQ-ACK

Document for: Discussion and Decision

# Introduction

During RAN#88-e plenary [1], it was agreed to specify in Rel-17 the required UE feedback enhancements for HARQ-ACK.

In this paper, we discuss some HARQ-ACK feedback enhancements like cross-carrier PUCCH switching.

# Dynamic cross-carrier PUCCH scheduling for TDD carriers

Dynamic selection of the CC used for the PUCCH transmission will help reducing the latency for Carrier Aggregation operation with two or multiple inter-band carriers having different TDD patterns. Utilizing the nearest UL transmission opportunity on different CCs for PUCCH transmission will help reducing the HARQ feedback delay.

Figure 1: “Dynamic switching of CC carrying PUCCH” not currently allowed.

Evaluation of the dynamic cross-carrier PUCCH technique in terms of latency and capacity was carried out using the SLS assumptions in Appendix-B and simulation results are shown in Table 1, Figure 2 and Figure 3.

Also, further evaluation is now available in our contribution under the Performance and evaluation for the XR and Cloud Gaming service in [6] under section 5 (shown also in Appendix E).  With the additional CA enhancements, the capacity can further increase by 66% (6 to 10 users per cell) for Dense Urban and by 33% (6 to 8 users per cell) compared to the CA baseline. The additional enhancements for CA, including PUCCH on both carriers and cross-carrier HARQ retransmission where the PUCCH on both carriers provides the largest enhancement.

Table 1: Latency enhancement using the dynamic PUCCH on 2CCs with Carrier Aggregation

|  | Latency (ms) (90th percentile) |  |  |
| --- | --- | --- | --- |
| # of UEs/Cell | CA Baseline | CA Baseline TDD pattern DU | CA w/ dynamic PUCCH on 2CCs |
| 2 | 3.9996 | 2.5371 | 2.7571 (-31.07%) |
| 4 | 3.7496 | 2.5371 | 2.7596 (-26.40%) |
| 6 | 3.7496 | 2.5371 | 2.7571 (-26.47%) |
| 8 | 3.9971 | 2.5371 | 2.7596 (-30.96%) |
| 10 | 3.9971 | 2.5371 | 2.7571 (-31.02%) |

Table 1 shows the gain in terms of latency of using the dynamic cross-carrier PUCCH compared to the carrier aggregation baseline operation. The obtained latency is very close to using a small periodicity TDD pattern like DU. The 3ms latency requirement of the power distribution use case is not met using the CA baseline but it is met with the dynamic PUCCH enhancement.

Observation 1: Dynamic cross-carrier PUCCH allows for up to 30% latency reduction.

| Figure 2: Capacity in terms of #UEs meeting the latency and reliability requirements | Figure 3: Resource Utilization |
| --- | --- |

Also, Figure 2 shows the capacity gain where a UE is considered successful if 99.9% of the files are delivered within the PDB. Figure 2 shows that, at 95-percentile, the capacity is doubled from 7 UEs with the baseline Carrier Aggregation to 14 UEs using the dynamic cross-carrier PUCCH. Also with the dynamic cross-carrier PUCCH enhancement, the obtained capacity is very close to the capacity achieved using the DU TDD pattern (#UEs = 16 at 95-percentile).

Figure 3 shows the resource utilization where a substantial enhancement could also be observed using the dynamic cross-carrier PUCCH compared to the CA baseline.

Observation 2: Dynamic cross-carrier PUCCH doubles the network capacity and reduces the resource utilization compared to the Carrier Aggregation baseline operation.

- Support dynamic cross-carrier PUCCH for Carrier Aggregation.

From specification perspective, the impact is minor. The dynamic indication of the carrier carrying PUCCH could be supported with a new DCI field PUCCH-carrier-Indicator.

Also, for the HARQ-ACK codebook construction and to keep the mechanism simple, the indication in the first DCI could be used to select the PUCCH carrier and the carrier couldn’t be overridden afterwards. TDD patterns are static and no need to change the carrier in the middle of the codebook construction.

## Dynamic vs Semi-static PUCCH carrier switching

In RAN1#104e, the following agreement has been reached regarding the PUCCH carrier switching.

Agreements: For further study on whether and how to support PUCCH carrier switching in a PUCCH group, focus on the following three alternatives:

- Alt. 1: PUCCH carrier switching is based dynamic indication in DCI

- Alt. 2B: PUCCH carrier switching is based on certain (semi-static) rules

- Alt. 2C: PUCCH carrier switching is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells

- Note: In above alternatives, it is assumed that HARQ-ACK corresponding to PDSCH received on a Pcell/PScell or an Scell in a PUCCH group, can be sent on a PUCCH on an Scell also instead of only on Pcell/PScell/PUCCH-SCell in the same PUCCH group, as opposed to Rel-16 where HARQ-ACK corresponding to PDSCH received on a Pcell/PScell or an Scell in a PUCCH group can only be sent on Pcell/PScell/PUCCH-SCell in the same PUCCH group.

- Note: Realistic deployment scenarios including TDD configurations should be considered for the study

We support Alt-1 and it should be included as one possible option for PUCCH carrier switching. The dynamic indication of the PUCCH carrier allows to anticipate the change in the UL/DL direction for aggregated carriers in TDD bands hence reducing the latency. Any set of aggregated carriers with different TDD patterns will benefit from a dynamic PUCCH carrier switching in terms of latency. The reduced latency allows for more HARQ-ACK transmission opportunities, hence better reliability and also better system capacity.

FDD + TDD aggregated carriers will also benefit from the dynamic PUCCH carrier switching. FDD UL is always available which is ideal for latency. However, if the FDD UL carrier is experiencing bad channel conditions or sever interference, then there will be the option to switch to another TDD UL carrier on the available UL slots.

Also, TDD carriers usually have wider bandwidth than FDD carriers and the gNB can allocate more UL resources on the TDD carriers, hence can guarantee better reliability by switching to the TDD carriers when PUCCH reliability on FDD is compromised.

Hence, the dynamic PUCCH carrier switching allows to counter the change in the UL channel conditions by selecting the best carrier for the UL PUCCH reliability.

In terms of UE complexity, the dynamic PUCCH carrier switching doesn’t incur any additional complexity cost at the UE side. The UE needs to parse the PUCCH carrier index from the DCI and switch to another carrier when required for the PUCCH transmission, which doesn’t require any extra processing.

Alt-2B and Alt-2C on the other hand have unclear benefits in terms of latency, reliability and capacity enhancement compared to the existing design in Rel-15 and Rel-16. Adding more flexibility in semi-static configuration, by allowing a semi-static switch to another PUCCH carrier with the cell group doesn’t bring any latency enhancement or any other benefits.

## Support of the Dynamic PUCCH carrier switching

Dynamic PUCCH carrier switching could be supported by a simple DCI indication and will only require an extra bit field in the DCI. First DCI in the HARQ codebook construction could be used to select the PUCCH carrier and doesn’t change during the codebook construction, hence no overriding. Also the same DCI bit-field will remain the same during the codebook construction, hence there is no ambiguity if one DCI is missed as all the DCIs carry the same information about the PUCCH carrier to be selected.

- All DCIs pointing to the same PUCCH carry the same PUCCH carrier index, hence no overriding and no risk if one DCI is missed.

Regarding the configuration, two possible options could be explored:

- Option 1: Define a PUCCH configuration per PUCCH carrier. It is a simple approach where each PUCCH carrier will have its own PUCCH configuration.

- Option 2: Define two levels of PUCCH configuration, “per PUCCH group” and “per PUCCH carrier”. The “per PUCCH group” configuration carry the PUCCH configuration common to the PUCCH carriers per PUCCH group. And the “per PUCCH carrier” carry the PUCCH configuration specific to a carrier (e.g. K1 list, power control, PUCCH formats, PUCCH resources,…)

- Selection between Option-1 and Option-2 for the PUCCH configuration:

- Option 1: A PUCCH configuration per PUCCH carrier.

- Option 2: Define two levels of PUCCH configuration, “per PUCCH group” and “per PUCCH carrier”.

Regarding the PUCCH power control for dynamically switched PUCCH resources. TPC commands for PUCCH are provided using DCI format 2_2 (TPC-PUCCH-RNTI). In mmWave operation, the MAC CE (PUCCH spatial relation activation/deactivation) signals to the UE to change the beam used for the PUCCH transmission. While changing the beam, the UE also changes the power control parameter sets at the same time. A similar approach could be adopted for the dynamic PUCCH carrier switch. Each cell carrying PUCCH has its own TPC configuration (PUCCH-PowerControl) and has its own TPC loop and when changing the PUCCH carrier the UE changes the power control parameters to use the ones associated to the new PUCCH carrier.

- Each cell carrying PUCCH has its own TPC configuration (PUCCH-PowerControl) and has its own TPC loop. When switching the PUCCH carrier, UE changes the power control parameters to use the ones associated to the new PUCCH carrier.

# PUCCH reliability enhancements

## Discussion

HARQ based transmission is essential to achieve the strict reliability requirements for URLLC with efficient use of radio resources. For HARQ based DL transmission, the probability for successful DL transmission will heavily depend on the reliability of the uplink control channel (PUCCH) that carries ACK/NACK feedback. Considering a HARQ based DL transmission with one retransmission, the probability (P) of successfully delivering a packet is given by

P=PPDCCHPPDSCH1Successful receptionfrom 1st trans.+1-PPDCCH1-PDTX→ACKPPDCCH,Re-TxPPDSCH,  Re-Tx1Successful reception after 1st PDCCH failure+PPDCCH1-PPDSCH11-PNACK→ACKPPDCCH,Re-TxPPDSCH,Re-Tx2Successful reception after 1st PDSCH failure,                   (1)

where PPDCCH is the probability of successfully decoding the PDCCH, PPDSCH1 is the probability of successfully decoding the PDSCH transmission without soft combining, PPDSCH2 is the probability of successfully decoding the PDSCH transmission with soft combining. PDTX→ACK (resp. PNACK→ACK) are the probabilities of falsely detecting DTX (resp. NACK) as ACK at the gNB. As it can be seen from (1), the successful detection of uplink DTX and NACK at the gNB is essential for the reliability and latency of HARQ based DL transmission. Thus, design of PUCCH should ensure very low impact of DTX-to-ACK and NACK-to-ACK errors. Missed ACK error (i.e. ACK-to-DTX) results in unnecessary retransmission, but does not affect the reliability of HARQ based DL transmission. However, missed ACK errors need to be kept low to preserve the spectral efficiency. To avoid unnecessary retransmissions, missed ACK target similar to LTE should be considered, which is 10-2.

Comparing the contributions of the DTX-to-ACK and NACK-to-ACK errors, we can see that DTX-to-ACK errors can only occur when PDCCH detection fails, while the NACK-to-ACK errors occur when the PDSCH decoding fails. For LTE PDCCH BLER target was ~ 10-2 and PDSCH BLER target was ~ 10-1. For URLLC similar asymmetry between PDCCH BLER and PDSCH BBLER can be expected with a PDCCH BLER target between 10-6 and 10-3 and a larger PDSCH BLER target potentially between 10-1 and 10-3, we can therefore expect that for URLLC the NACK-to-ACK to be more critical than the DTX-to-ACK errors.

In this contribution, we focus on methods to enhance the PUCCH performance in terms of missed ACK and NACK-to-ACK errors. For the analysis we consider a fixed DTX-to-ACK error probability of 10-2, a missed ACK target of 10-2 and a NACK-to-ACK target ≤10-4. Given the low latency requirements for URLLC, PUCCH format_0 can be considered the most relevant for URLLC scenario. The main approaches to enhance the reliability of PUCCH are receive diversity, time repetition and frequency hopping.

## Evaluation of missed-ACK and NACK-to-ACK errors

Here we show the performance of PUCCH format_0 with different number of receive antennas. Simulation parameters are provided in Table 3in Appendix D. Figure 4 shows the missed ACK and NACK-to-ACK errors with number of receive antennas 1, 2 and 4. Increasing the number of receive antennas enhances the PUCCH reliability. Further enhancements can be achieved with repetition and frequency hopping.

Figure 4: Missed ACK and NACK-to-ACK error rates for PUCCH format_0, 2PRBs

Table 2 lists the required SNR for achieving a given PUCCH reliability target. It can be seen that one receive antenna at the gNB is not enough to achieve the required performance targets when PUSCH is used. Furthermore, the required SNR for achieving the target NACK-to-ACK error rate is generally higher than the required SNR for achieving the target missed ACK rate. This gap gets even larger with lower NACK-to-ACK error rate target. In addition, the gap between the required SNRs for missed ACK and NACK-to-ACK varies based on the system settings (here it is the number of PRBs and number of receive antennas).

Table 2: Required SNR (dB) for PUCCH format_0 reliability targets

|  | 1PRB |  | 2PRBs |  |
| --- | --- | --- | --- | --- |
|  | nRX = 2 | nRX = 4 | nRX = 2 | nRX = 4 |
| Missed ACK (1%) | 5.8 | 0.02 | 3 | -2.82 |
| NACK-to-ACK (10-4) | 6.8 | 0.7 | 3.65 | -2 |
| NACK-to-ACK (10-5) | 12.2 | 3.73 | 10 | 0.2 |

Observation 3: Receive diversity is essential for enhancing the reliability of PUCCH.

Observation 4: The required SNR for achieving the target NACK-to-ACK error rate is generally higher than the required SNR for achieving the target missed ACK rate.

Observation 5: The difference between the required SNR for achieving the target missed ACK and NACK-to-ACK error rates depends on the system setting (e.g. number of PRBs and number of receive antennas).

## Asymmetric NACK and ACK transmissions

One option to achieve the reliability targets (missed ACK and NACK-to-ACK errors) for PUCCH is by transmitting the PUCCH with higher power (PPUCCH) to achieve the stricter of the two targets. In addition to other system parameters, the PUCCH power can be specified as follows

PPUCCH=f(maxRequired_PMissed ACK, Required_PNACK-to-ACK),

where Required_PMissed ACK is the required transmission power to achieve the target missed ACK rate, and Required_PNACK-to-ACK is the required transmission power to achieve the target NACK-to-ACK error rate. Considering that ACK transmission are expected to happen more often compared to NACK transmission, this approach will lead to inefficient use of the transmission power. Another approach is to adjust the transmission power of PUCCH based on the UCI content, i.e. depending on whether the ACK or NACK is being transmitted. The PUCCH transmission power PPUCCH will be based on Required_PMissed ACK if there is an ACK to be transmitted, and based on Required_PNACK-to-ACK if there is a NACK to be transmitted. From the ACK/NACK perspectives, two PUCCH transmission powers can be defined as: PPUCCHACK is the PUCCH power when there is an ACK to be transmitted, PPUCCHNACK is the PUCCH power when there is a NACK to be transmitted. For two bits UCI, the PPUCCHACK can be used when both bits are ACK, and PPUCCHNACK when at least one of the bits is NACK. Figure 5 shows the PUCCH performance with different transmission powers for ACK and NACK. The NACK-to-ACK target of 10-4 can be achieved with about 1 dB and 2 dB increase (for nRX= 2 and 1, respectively) in the PUCCH power in the case on NACK transmission. Higher power increase for NACK transmission will be needed to achieve the 10-5 NACK-to-ACK target.

Figure 5: Missed ACK and NACK-to-ACK error rates for PUCCH format_0 with asymmetrical transmission power.

The main advantage of using different power levels for ACK and NACK transmissions is that the average consumed power by the UE is greatly reduced compared to the case when the same missed ACK and NACK-to-ACK performance is targeted by using identical ACK and NACK power levels. For example, assuming 10% NACK probability (as a worst case scenario) then using a PPUCCHNACK with 3dB boost compared to the PPUCCHACK results in 0.4dB average power increase only, this is to be compared with the 3dB average power increase if both the PPUCCHNACK and PPUCCHACK are boosted by 3dB. Adopting this solution will reduce the power consumption and the inter-cell interference as well.

- Support different PUCCH transmission power levels depending on whether ACK or NACK is transmitted.

# Other Enhancements

## SPS HARQ skipping for “skipped” SPS PDSCH

In RAN1#103e [3], the agreement below has been reached for the skipped SPS PDSCH:

Agreements: For the studies on SPS HARQ skipping for skipped SPS PDSCH, the further discussions should focus on the following reduced sets methods:

- ‘NACK skipping’ for (skipped) SPS PDSCH (Alt. 1)

  - FFS: details including at least when to skip the HARQ-ACK as well as NACK skipping configuration details (per SPS or group of SPS configurations etc.)

  - Note: this alternative assumes inherently no identification of a skipped SPS PDSCH by the UE

- Dynamic indication of skipped SPS PDSCH occasions (Alt. 3)

  - FFS: details including dynamic indication methods such as e.g. DCI, MAC CE, specific DM-RS instead of SPS DM-RS, …

We oppose both alternatives in the agreement and we think “SPS HARQ skipping for skipped SPS PDSCH” is not beneficial and doesn’t provide good system gain. Also, the specification effort is very high and a lot of optimization is required for a minor potential gain. The following reasons show that the potential enhancement is very marginal:

- For periodic traffic, the SPS PDSCH is transmitted in all the SPS occasions. Thus, the probability of having skipped SPS PDSCH is very low limiting the advantage of skipping the HARQ feedback for a “skipped” SPS-PDSCH.

- For aperiodic traffic, DG-PDSCH is more spectrally efficient way compared to SPS-PDSCH. For DL, as the PDCCH and PDSCH can be FDMed or have no gap at all, the scheduling DCI doesn’t cause delay compared to UL transmission. Thus, using SPS-PDSCH for aperiodic traffic in not a typical scenario.

- Skipping the SPS HARQ could save some of the PUCCH resources, but it is not expected that the PUCCH to be the bottleneck in this case, as the PUCCH resources will be very small compared to the PDSCH resources.

- The PUCCH resource will be reserved to the UE, even if the UE skipped the HARQ feedback. So, there is no gain in terms of reducing the UL overhead.

- Skipping SPS HARQ when multiplexed with HARQ of DG PDSCH will complicate the HARQ cookbook construction, and offers no advantage as the PUCCH transmission can’t be skipped in that case.

- UE is not be able to differentiate whether SPS PDSCH is skipped or it is not correctly decoded and it needs a mechanism in place to identify the ‘skipped’ SPS PDSCH (E.g. DMRS detection) which adds to the UE implementation complexity.

- SPS HARQ skipping for “skipped” SPS PDSCH shouldn’t be supported.

## PUCCH repetition enhancements

The following two agreements have been made in RAN1#104e [5]

Agreements: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH

- Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary

- FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK

- Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17

  - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed

Agreements: Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition.

FFS: Support for slot-based PUCCH repetition

There is still FFS about supporting slot-based PUCCH repetitions. There is no clear the justifications to support slot based PUCCH repetition for short-PUCCHs. If it is beneficial from CovEnh perspective then the CovEnh WI should introduce this feature.

- Don’t support slot based PUCCH repetition for short-PUCCHs

## Retransmission of cancelled HARQ

Aborting PUCCH carrying eMBB HARQ is not always the best strategy as it tends to trigger multiple superfluous retransmissions of large chunks of data in the downlink. For instance, if 10% of eMBB HARQ transmissions is aborted, the eMBB DL BLER degrades by 10%. Therefore, this should be accounted for unless it occurs infrequently.

In some cases the gNB might decide to prioritize eMBB actually. For instance, the cost of single-shot URLLC transmission is lower than the cost of triggering superfluous eMBB retransmissions.

Also, in most cases the URLLC PUCCH would be carrying an ACK and in these circumstances it is not very crucial for the URLLC service. Obviously, it is still required (e.g. for statistics) but the latency and (possibly) reliability are no longer a concern.

Hence, the LP-HARQ case should be supported to alleviate the negative impact on (eMBB) PDSCH performance. Also, HP-HARQ should also be supported as dropped (URLLC) HARQ feedback is also very important to be received by gNB.

The same mechanism to study and specify can be used for low priority and high priority HARQs. Although we think it is more relevant for low priority HARQ, it is also very useful for high priority HARQ and we don’t see a need to optimize the mechanism separately for high priority HARQ.

However, overlapping with AI 8.3.3 should be clarified as it seems to be tackling similar issue by multiplexing the HP-HARQ and LP-HARQ.

- Support retransmission of cancelled low priority and high priority HARQ.

However, to minimize specification effort, we support reusing the existing Rel-16 Type 3 HARQ-ACK codebook. If it is not suitable for URLLC operation, we support adopting the “UL grant scheduling PUSCH to carry dropped HARQ” option.

- Support reusing the existing Rel-16 Type 3 HARQ-ACK codebook.

## SPS HARQ payload size reduction and / or skipping for ‘non-skipped’SPS PDSCH

In RAN1#103e [3], the agreement below has been reached for the SPS HARQ payload size reduction:

Agreements: For the studies on SPS HARQ payload size reduction (of non-skipped SPS PDSCH), the further discussions should focus on the following reduced sets of methods:

- ACK skipping (NACK-only) (Alt. 1)

  - FFS: Details

- NACK skipping (ACK-only) (Alt. 2)

  - FFS: Details

- HARQ bundling / compression (Alt. 3)

  - FFS: Details including HARQ bundling / compression window, bundling / compression technique

- HARQ-ACK disabling /skipping for certain SPS configurations (Alt. 4)

  - The skipping / disabling is higher-layer configured per SPS configuration

  - FFS: HARQ-ACK skipping behaviour for Type 1 CB

We oppose the four alternatives above in the agreement above. Skipping PUCCH if only SPS ACK is to be transmitted is not needed and will impact the performance of the system by impacting the reliability of the HARQ feedback. The gNB couldn’t distinguish between skipped SPS ACK and NACK-to-DTX error. Hence, NACK-to-DTX error could be interpreted as ACK and creates a bottleneck for the system reliability.

Also, there is no resource efficiency gain as PUCCH resources for a possible HARQ feedback needs to be configured and reserved by the gNB anyway.

Also, if this proposal is adopted it should equally apply to any HARQ and not necessarily SPS HARQ which means a lot of specification impact.

Regarding Alt-2, for NACK skipping, NACKs happen very rarely and usually multiplexed with ACKs in the same PUCCH, which makes this alternative very inefficient. Also the resource saving is very minor as NACKs happen with very low probability.

Alt- 3, for HARQ bundling, could be explored further as a potential option for the payload size reduction considering that most the HARQ feedback will be ACKs. Also Alt-3 is very useful for SPS traffic with jitter. But Alt-3 comes with some scheduling restriction. Also it requires some specification efforts to be supported.

If the SPS HARQ payload size reduction to be supported in RAN1, then we are in favor of Alt-4 (HARQ-ACK disabling /skipping) as it is a simple solution and it can resolve the payload size issue and it also comes with low UE complexity and reduced specification effort.

- Don’t proceed with SPS HARQ payload size reduction study in RAN1

## Type 1 HARQ codebook based on sub-slot PUCCH config

There is no strong benefit in supporting sub-slot based type 1 HARQ-ACK codebook for URLLC since this will further increase the PUCCH payload which is already large with type 1 HARQ-ACK codebook and it will further compromise its reliability with the increased overhead. Thus, it will only add to the UE implementation complexity without much benefits.

The only benefit of Type 1 HARQ-Ack codebook is its robustness against PDCCH detection failure which is not very likely for URLLC traffic with the very reliable PDCCH. Hence, overcoming this small risk of PDCCH failure by using the large Type 1 overhead is compromising the reliability of PUCCH and doesn’t seem to bring any overall gain.

Also, multiple issues will raise especially when the UL and DL operate in different numerologies and when sub-slots on the UL carrier are crossing slot or sub-slot boundaries on the DL carrier and this requires a lot of specification effort.

Also, the concept of PDSCH TDRA grouping based on UL sub-slots mentioned as one possible option has even large standards impact and will lead to redundant feedback further increasing the overhead.

But apart from the specification effort, what matters most is the absence of any substantial gain of using Type 1 HARQ-Ack codebook on sub-slots for URLLC applications.

- Don’t proceed with sub-slot based type 1 HARQ-ACK codebook for URLLC in RAN1 Rel-17

# Conclusion

In this contribution, we have made the following observations and proposals:

Observation 1: Dynamic cross-carrier PUCCH allows for up to 30% latency reduction.

Observation 2: Dynamic cross-carrier PUCCH doubles the network capacity and reduces the resource utilization compared to the Carrier Aggregation baseline operation.

Observation 3: Receive diversity is essential for enhancing the reliability of PUCCH.

Observation 4: The required SNR for achieving the target NACK-to-ACK error rate is generally higher than the required SNR for achieving the target missed ACK rate.

Observation 5: The difference between the required SNR for achieving the target missed ACK and NACK-to-ACK error rates depends on the system setting (e.g. number of PRBs and number of receive antennas).

- Support dynamic cross-carrier PUCCH for Carrier Aggregation.

- All DCIs pointing to the same PUCCH carry the same PUCCH carrier index, hence no overriding and no risk if one DCI is missed.

- Selection between Option-1 and Option-2 for the PUCCH configuration:

- Option 1: A PUCCH configuration per PUCCH carrier.

- Option 2: Define two levels of PUCCH configuration, “per PUCCH group” and “per PUCCH carrier”.

- Each cell carrying PUCCH has its own TPC configuration (PUCCH-PowerControl) and has its own TPC loop. When switching the PUCCH carrier, UE changes the power control parameters to use the ones associated to the new PUCCH carrier.

- Support different PUCCH transmission power levels depending on whether ACK or NACK is transmitted.

- SPS HARQ skipping for “skipped” SPS PDSCH shouldn’t be supported.

- Don’t support slot based PUCCH repetition for short-PUCCHs

- Support retransmission of cancelled low priority and high priority HARQ.

- Support reusing the existing Rel-16 Type 3 HARQ-ACK codebook.

- Don’t proceed with SPS HARQ payload size reduction study in RAN1

- Don’t proceed with sub-slot based type 1 HARQ-ACK codebook for URLLC in RAN1 Rel-17

# References

- RP-201310 “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” Nokia, Nokia Shanghai Bell, 3GPP TSG RAN Meeting #88e.

- R1-2007448 “Summary#3 of email thread [102-e-NR-L1enh_URLLC-UCI_Enh-02]”, Oppo, 3GPP TSG RAN WG1 #102-e.

- RAN1#103e chairman notes.

- RP-202872 “Moderator's summary for email discussion [90E][26][IIOT_scope]”, Nokia, Nokia Shanghai Bell, RP#90e, December 2020.

- RAN1#104e chairman notes.

- R1-2102707 “Initial Performance and Evaluation Results for XR and CG” Mediatek, 3GPP TSG RAN WG1 #104b-e.

- GTI URLLC Evaluation White Paper (PhaseⅡ) (04/2020), http://www.gtigroup.org/Resources/rep/2020-05-08/15029.html

# Appendix-A

|  | Use case | Reliability | Latency | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- | --- |
| Rel-15 | generic | 99.999 (BLER 10-5) | 1 ms | 32 bytes |  |
| Rel-16 | Factory automation | 99.9999 (BLER 10-6) | 1 ms | 32 bytes Periodic and deterministic, arrival interval 2 ms | Motion control |
|  | Power distribution | 99.9999 (BLER 10-6) | 2-3 ms | 100 bytes FTP model 3, arrival interval 100 ms | Power distribution grid fault and outage management |
|  |  | 99.999 (BLER 10-5) | 6-7 ms | 250 bytes Periodic and deterministic, arrival interval 0.833 ms | Differential protection |
|  | Transport Industry | 99.999 (BLER 10-5) | 3 ms | UL: 2.5 Mpbs; Packet size 5220 bytes DL: 1Mbps; Packet size 2083 bytes | Remote driving |
|  |  | 99.999 (BLER 10-5) | 7 ms | 1.1 Mbps; Packet size 1370 bytes Arrival rate 100 packets/sec for periodic traffic | Intelligent transport system (ITS) |

# Appendix-B

TDD pattern for PUCCH enhancement evaluation (CMCC TDD patterns used in GTI URLLC paper 04/2020) [7]:

- Baseline (same TDD patterns on both CCs):

  - TDD pattern CC1= [D D D S U D D S U U]

  - TDD pattern CC2= [D D D S U D D S U U]

- Test case:

  - TDD pattern CC1 = [D D D S U D D S U U]

  - TDD pattern CC2 = [S U U U D S U U U D]  which is [D S U U U D U U U] with offset = 1

SLS Assumptions

| Use case |  | Electric Power Distribution (22.804:5.6.6) |
| --- | --- | --- |
| Description |  | Power distribution grid fault and outage management |
| Criteria for each transmission | Reliability | 99.999 % |
|  | Latency | - 15 ms (end to end latency) - 2-3 ms air interface latency |
| Requirement |  | 95% UEs satisfy the criteria |
| Data packet size and traffic model |  | - DL & UL: 100 bytes - Periodic and deterministic with arrival interval 100 ms - Random offset between UEs |

| Parameters | Value |
| --- | --- |
|  | Electric Power Distribution |
| Channel model | UMa in TR 38.901 |
| Duplexing | TDD |
| Carrier frequency (GHz) | 2.6GHz |
| Simulation bandwidth (MHz) | 20MHz |
| SCS (kHz) | 30KHz |
| BS antenna configurations | 4 Tx/ 4 Rx: (8,4,2,1,1; 1,2) dH = 0.5λ; dV = 0.8λ |
| UE antenna configuration | 2 Tx: (1,1,2,1,1; 1,1), 4 Rx: (1,2,2,1,1; 1,2), dH = 0.5λ |
| BS antenna gain + connector loss (dBi) | 8 |
| UE antenna gain (dBi) | 0 |
| BS Tx power(dBm) | 49 |
| UE Tx power(dBm) | 23 |
| BS receiver noise figure (dB) | 5 |
| UE receiver noise figure (dB) | 9 |
| Layout | Single layer - Macro layer: Hex. Grid |
| Transmission rank | 1 |
| Overall target BLER | 10-6 |
| UE distribution | 100% of users are outdoors : 3 km/h UE speed |
| BS antenna height (m) | 25 |
| UE antenna height (m) | 1.5 |
| Inter-BS distance (m) | 500 |
| BS receiver | MMSE-IRC |
| UE power control | P0 = -90, alpha = 1 |
| Channel estimation | Realistic |

# Appendix-C: Dynamic PUCCH Enhancement: another TDD scenario

Scenario:

TDD pattern for UL/DL enhancement

- Baseline:

  - TDD pattern 1 = [D D D S U D D S U U] (for CC1)

  - TDD pattern 2 = [S U U D D S U U D D] (for CC2)

- UL enhancement:

  - TDD pattern same as baseline

  - ACK/NACK can be reported in either on CC1 or CC2

SLS settings are the same as in Appendix B.

Latency Enhancement

|  | Latency (ms) (90th percentile) |  |
| --- | --- | --- |
| # of UEs/Cell | CA Baseline | CA w/ dynamic PUCCH on 2CCs |
| 1 | 3.01 | 1.76 |
| 2 | 3.50 | 1.76 |
| 3 | N/A | 1.76 |
| 4 | N/A | 1.76 |
| 5 | N/A | 2.50 |
| 6 | N/A | 2.70 |
| 7 | N/A | 2.75 |
| 8 | N/A | 2.75 |
| 9 | N/A | 2.75 |
| 10 | N/A | 2.77 |

Capacity Enhancement

# Appendix D

Table 3: Simulation parameters for link-level simulation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Bandwidth | 20 MHz |
| Subcarrier Spacing | 15 kHz |
| Antenna Configuration | 1Tx, and (1, 2, 4) Rx antennas |
| Number of PRBs | 1 and 2 |
| Number of Symbols | 1 Symbol |
| Channel | TDL-C with 300ns RMS delay, @ 3 km/h |
| Noise estimation | Ideal |
| Performance metrics | DTX-to-ACK probability of 0.01 |

# Appendix E

Figure 6: System capacity curves for the downlink traffic of AR/VR in FR1 with inter-band CA