3GPP TSG RAN WG1 #104-e	                       		                         R1-2102750

e-Meeting, April 12th – 20th, 2021

Title: 	Discussion on NTN-IoT scenarios with MEO

Source: 	Hughes/EchoStar

Agenda Item:	8.15.1

Document for:	Discussion and Decision

# 1. Introduction

Referring to IoT NTN SI work [1], the main motivation of this document is to include MEO in the adaptation/enhancement of an NTN-IoT protocol that is also suitable for deployment in GEO and NGSO scenarios.

The purpose of this contribution is to ensure study results that apply to GEO and NGSO with circular orbit at altitude greater than 600 km also apply to MEO. MEO scenarios feature characteristics that are less challenging than GEO scenarios in terms of latency, and less than LEO scenarios for the doppler effects. Therefore, if the normative work considers GEO and LEO scenarios, the NTN-IoT systems will also be able to support MEO scenarios.

There is a wide range of delay tolerant IoT/M2M applications which require global coverage and continuous service. Such services are ideally suited for support by a satellite network offering larger and adequate performance such as MEO. Examples are maritime use cases for non-critical asset tracking where a data logger is currently used, livestock monitoring across dispersed rural areas, and other critical and non-critical asset tracking in landline, environmental monitoring, wildlife monitoring, environmental monitoring, climate change and so on.

# 2. MEO Characteristics

The following observations and proposal on MEO characteristics in R2-1914721 [2] were approved:

Observation 1: Propagation round trip delay (RTD) for bent pipe GEO satellite at 35786 km is about 545 ms, while a typical LEO system at 600 km has RTD of 28.4 ms. The RTD for MEO or any NGSO satellite with altitude above 600 km falls in the range from 28.4 ms to 545 ms.

Observation 2: The Differential One-Way Delay between nadir and EOC paths for GEO satellite at 35786 km is 16 ms and for LEO satellite at 600 km is 4.44ms. For MEO or any NGSO satellite with altitude between LEO and GEO, the Differential One-Way Delay between nadir and EOC is in the range from 4.44ms to 16ms, therefore it is within the range of LEO and GEO.

Observation 3: The Doppler shift and variation of MEO constellation or any NGSO satellite with altitude between LEO and GEO is smaller than that of for LEO constellation.

Observation 4: The RTT for MEO is less than for GEO but greater than for LEO and hence it is FFS if HARQ can be used

Proposal 1: Add a sentence in the TR 38.821:

“The study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km”.

Studies in TR 38.811 and solutions developed for GEO and LEO scenarios in TR 38.821 demonstrate that the doppler effect and latency for NGSO constellation (with altitude greater than 600 km) are less challenging than GEO scenarios in terms of latency and less challenging than LEO at 600 km scenarios for the doppler effects.

In TR 38.821 it was noted “The study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km”.

# 3. MEO Characteristics and Link Budget Analysis

This section introduces the MEO characteristics for the IoT NTN reference scenarios. Based on conclusion in section 2, proposal is to add reference scenario D: MEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 10000 km).

Proposal 1: To add MEO scenario D in Table 4.2-1 in TR 36.763.

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |
| MEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 10000 km) | Scenario D |

Table 4.2-1: IoT NTN reference scenarios

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C | MEO based non-terrestrial access network -Scenario D |
| --- | --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth | circular orbiting at low altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km | 10,000 km |
| Frequency Range | < 6 GHz (e.g. 2 GHz in S band) |  |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz (UL) - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |  |
| Payload | Transparent type | Transparent Type | Transparent type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) | Scenario D: The beams move with the satellite |
| Max beam footprint size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) | 4018 km |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link | 10° for service link and 5° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) | 14018 km |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) | 95.19 ms (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km | 13.4 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) | 7.5 ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) | 0.003 ppm/s |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |  |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: The Maximum beam footprint size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam. NOTE 4: The use of a Circular polarized antenna is optional. NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888 for device can be assumed for link budget. The noise figure is device vendor implementation specific. NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 7: System bandwidth is FFS |  |  |  |

Table 6.1-1: IoT NTN reference scenario parameters

Proposal 2: To add MEO IoT NTN reference scenario parameters in Table 6.1-1 in TR 36.763.

# 4. Scenario parameters for MEO

Table 1 below shows the two sets of parameters that were defined for NR NTN studies (i.e., “set 1” and “set 2” specified in TR 38.821) and “set 3” and “set 4” that were presented in RAN1#103-e meeting. Proposal is to add Set 5 for MEO characteristics.

|  | “Set-1” in TR 38.821 | “Set-2” in TR 38.821 | “Set 3” in R1-2008868 | “Set-4” in R1-2008868 | Proposed MEO Scenarios (Set 5) |
| --- | --- | --- | --- | --- | --- |
| Satellite orbit | LEO-600 | LEO-600 | LEO-600 | LEO-600 | MEO |
| Satellite altitude | 600 km | 600 km | 600 km | 600 km | 10,000 km |
| Payload characteristics for DL transmission |  |  |  |  |  |
| Frequency band | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 2 m | 1 m | 0.5 m | NOTE 2 | 1.5 m |
| Satellite EIRP density | 34 dBW / MHz | 28 dBW / MHz | 28.3 dBW / MHz | 21.45 dBW/MHz | 45.4 dBW/MHz |
| Satellite Tx max Gain | 30 dBi | 24 dBi | 16.2 dBi | 11 dBi | 28.1 dBi |
| 3dB beamwidth | 4.4127 deg | 8.8320 deg | 22.1 deg | 60 deg | 6.5 degrees |
| Satellite beam diameter (at nadir pointing) | 20 km | 90 km | 234 km | 700 km | 1140 km |
| Payload characteristics for UL reception |  |  |  |  |  |
| Frequency band | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 2 m | 1 m | 0.5 m | NOTE 2 | 1.5 m |
| G/T | 1.1 dB·K-1 | -4.9 dB·K-1 | -12.8 dB·K-1 | - 17.9 dB·K-1 | 3.8 dB/K |
| Satellite Rx max Gain | 30 dBi | 24 dBi | 16.2 dBi | 11 dBi | 28.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter for the parabolic reflector modelled in Sec. 6.4.1 of TR 38.811. NOTE 2: Antenna models different from the parabolic reflector described in TR 38.811 should be used. |  |  |  |  |  |

Table 1: Sets of satellite parameters for link budget and system level evaluations

For link budget analysis the following parameter could be utilized:

| Set 5 | MEO |
| --- | --- |
| 3 dB Beam width (HPBW) | 6.5 degrees |
| Central beam center elevation | 90 degrees |
| Central beam edge elevation | 86.1 degrees |
| Central beam edge satellite-UE distance | 10042 km |

Table 6.2-2: Set-5 parameters for link budget analysis

Proposal # 3: To include MEO Set-5 parameters for link budget analysis in a new Table 6.2-8 in TR 36.763, as a representative characterization of NTN-IoT scenarios with MEO altitude and characteristics.

Proposal 4: To add MEO Set-5 satellite parameters for system level simulator calibration in a new Table 6.2-9 in TR 36.763.

# 4.0 Link budgets

The link budget calculation results are as follows:

Using “Set 5” NTN-IoT scenarios with MEO altitude and characteristics.

# 5. Summary and conclusions

This document presents information of relevance for 3GPP to consider the adaptation of NTN-IoT to also allow for deployment of MEO. The document describes the main characteristics of MEO and its related parameters.

Proposal 1: To add MEO scenario D in Table 4.2-1 in TR 36.763.

Proposal 2: To add MEO IoT NTN reference scenario parameters in Table 6.1-1 in TR 36.763.

Proposal 3: To include MEO Set-5 parameters for link budget analysis in a new Table 6.2-8 in TR 36.763, as a representative characterization of NTN-IoT scenarios with MEO altitude and characteristics.

Proposal 4: To add MEO Set-5 satellite parameters for system level simulator calibration in a new Table 6.2-9 in TR 36.763.

Observation: The doppler shift/variation and the delay variation for MEO are smaller than for LEO. The maximum delay for MEO is smaller than for GEO. The IoT-NTN enhancements for LEO and GEO should be sufficient to support MEO.

# References

- RP-193235, “Study on NB-Io/eMTC support for Non-Terrestrial Network”, MediaTek Inc, 3GPP TSG RAN Meeting #86 Sitges, Spain, December 9-31, 2019

[2] R2-1914721 “Inputs to TR 38.821 on NGSO constellation greater than 600 km