3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102759

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.3.1.2

Source:	Futurewei

Title:	CSI feedback enhancements for URLLC

Document for:	Discussion and decision

# Introduction

In RAN1#104-e meeting, the Rel. 17 NR NR_IIOT_URLLC_enh WID [1] was discussed. The group arrived at the following conclusion [2] on the topic of CSI enhancement for URLLC:

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131. - Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B. - Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e. - Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

In this contribution, we present performance evaluation results for several schemes.  We also discuss our views on Case 1-3 new reporting and Case 2 new reporting, and proposals for moving forward.

# Performance Evaluation of Schemes

## Schemes under evaluation

We evaluate performance of the following three schemes through system level simulations:

Scheme 1: Baseline scheme.  In this scheme, the UE measures and reports CQI to gNB every K TTIs.  In the simulations, K is set to 5.  The eNB, after receiving the CQI, converts the CQI to SINR, and derives a predicted SINR for MCS selection:

SINRpredict=SINRmeasured-∆SINR, where SINRmeasured is the SINR converted from the received CQI, SINRpredict is the predicted SINR for MCS selection, and ∆SINR is a fixed backoff factor and is set to 20 dB [4].

The CQI delay, e.g., the gap between the time CQI is reported from the UE and the time PDSCH is transmitted to the UE based on the MCS derived from the CQI, is set to 4 TTIs in the simulations.

Scheme 2: Enhanced scheme utilizing interference statistics, e.g., based on Case 1-3 as described in [3].  In this scheme, the UE measures and reports CQI to gNB every K TTIs.  Similar to Scheme 1, K is set to 5 in the simulations.  The UE also reports to gNB the interference statistics, e.g., the variance/standard deviation of the measured interference, every L TTIs.  In the simulations, L is set to 100 to achieve a good balance between the feedback overhead of the interference statistics and performance.  The eNB, after receiving the CQI, converts the CQI to SINR, and derives a predicted SINR for MCS selection utilizing the UE-reported interference standard deviation:

  - SINRpredict=SINRmeasured-∆SINR, where SINRmeasured is the SINR converted from the received CQI, SINRpredict is the predicted SINR for MCS selection, and ∆SINR is a backoff factor set according to the UE-reported interference standard deviation and is set to ∆SINR=Aσinterference dB, where σinterference is the UE-reported interference standard deviation, and A is a scaling factor and is set to 4.26 in the simulations to take into account the targeted reliability of 99.999%, e.g., A = Q-110-5, where Qx=12πx∞exp⁡(-t22)dt.

The CQI delay is set to 4 TTIs in the simulations.

Scheme 3: Enhanced scheme utilizing SINR statistics, e.g., based on Case 1-1 as described in [4][5].  In this scheme, the UE measures and reports CQI to gNB every K TTIs.  Similar to Schemes 1 and 2, K is set to 5 in the simulations.  The UE also reports to gNB the SINR statistics, e.g., the variance/standard deviation of the measured SINR, every L TTIs.  Similar to Scheme 2, in the simulations, L is set to 100 to achieve a good balance between the feedback overhead of the SINR statistics and performance.  The eNB, after receiving the CQI, converts the CQI to SINR, and derives a predicted SINR for MCS selection utilizing the UE-reported SINR standard deviation:

SINRpredict=SINRmeasured-∆SINR, where SINRmeasured is the SINR converted from the received CQI, SINRpredict is the predicted SINR for MCS selection, and ∆SINR is a backoff factor set according to the UE-reported SINR standard deviation and is set to ∆SINR=AσSINR dB, where σSINR is the UE-reported SINR standard deviation, and A is a scaling factor and is set to 4.26 as in Scheme 2 to take into account the targeted reliability of 99.999%.

The CQI delay is set to 4 TTIs in the simulations.

- Please note again that Scheme 2 corresponds to Case 1-3 and Scheme 3 corresponds to Case 1-1 as described in [6].

## Simulation results

The simulation assumptions follow the ones defined in RAN1#102-e and can be found in the Appendix.

Figure 1 illustrates the percentage of UEs satisfying reliability and latency requirements (Reliability: 99.999%, Latency: 4 ms).  As shown in Figure 1, the percentage of satisfied UEs for the baseline scheme, the enhanced scheme with interference statistics, and the enhanced scheme with SINR statistics, are 68%, 92%, and 81%, respectively.  The enhanced scheme with interference statistics has the highest percentage of satisfied UEs among the three schemes, with a gain about 35% over the baseline scheme, and a gain about 14% over the enhanced scheme with SINR statistics.  Table 1 tabulates the percentage of satisfied UEs for the three schemes and their relative gain over the baseline scheme.

Figure 1. Percentage of UEs satisfying reliability and latency requirements

Table 1: Percentage of Satisfied UEs

| Schemes | Percentage of Satisfied UEs | Gain over Baseline |
| --- | --- | --- |
| Baseline | 68% | - |
| Enhanced with Interference Statistics | 92% | 35% |
| Enhanced with SINR Statistics | 81% | 19% |

The reason why the enhanced scheme with interference statistics (Scheme 2) performs better than the baseline scheme is as follows.  In Scheme 2, the SINR backoff factor is adjusted according to the interference standard deviation, instead of being fixed as in the baseline scheme.  Therefore, the effective SINR used for MCS selection does not have to be that conservative as in baseline with a fixed backoff factor, and it can be adapted to better fit to different interference variation, resulting in performance gain.

Comparing Scheme 2 to Scheme 3 (the enhanced scheme with SINR statistics), we look at the reason why Scheme 2 performs better than Scheme 3.  The reason is as follows.  In Scheme 2, the SINR backoff factor is adjusted according to the interference standard deviation, whereas in Scheme 3, the SINR backoff factor is adjusted according to the SINR standard deviation.  However, since SINR include both the signal part and the interference part, its variance captures both the fluctuation of the signal part and the burstiness/variation of the interference part.  On the other hand, with the frequently reported CQI (e.g., reported every 5 TTIs in the simulations), the latest CQI already comprises information of the latest signal part.  And since the signal part changes slowly, especially for the low mobility UEs, it will be stable from the time the CQI report is received till the time the related PDSCH is transmitted.  So the uncertainty in the CQI report for MCS selection is related to the variation of the interference only.  However, due to the relatively long time window (e.g., 100 TTIs in the simulations) used to derive the SINR variance/standard deviation, the signal part fluctuates in the time window even for low mobility UEs, therefore the SINR variance include effect of the signal variance and the interference variance, exaggerating the uncertainty in the CQI report for MCS selection.  Scheme 2, on the other hand, only uses the interference standard deviation to adjust the SINR backoff factor.  The interference standard deviation/variance captures the burstiness/variation of the interference part only, targeting at the right uncertainty in the CQI report for MCS selection without exaggeration.  Therefore Scheme 2 achieves better performance than Scheme 3.

Figure 2(a) illustrates the CDF of interference standard deviation and SINR standard deviation over wideband while Figure 2(b) shows the CDF of interference standard deviation and SINR standard deviation over subband.  It is observed from Figure 2 that interference standard deviation is smaller than the SINR standard deviation.  For example, at 50% cumulative probability, the interference standard deviation is about 1 dB and 4 dB lower than the SINR standard deviation over the wideband and subband, respectively, confirming our analysis above.

| (a) | (b) |
| --- | --- |

Figure 2.  CDF of interference standard deviation and SINR standard deviation (a) wideband and (b) subband

Table 2 lists the resource utilization (RU) levels of the three schemes.  It is observed that both Scheme 2 (the enhanced scheme with interference statistics) and Scheme 3 (the enhanced scheme with SINR statistics) have lower RU level than the baseline scheme, while the RU levels of Scheme 2 and Scheme 3 are very close.

Table 2: Resource Utilization Level

| Schemes | RU Level |
| --- | --- |
| Baseline | 9.5% |
| Enhanced with Interference Statistics | 7.2% |
| Enhanced with SINR Statistics | 6.5% |

Base on the simulation results and the above analysis, we have the following observation:

  - Observation 1: The CSI enhanced scheme with new reporting quantity of interference statistics (e.g., variance/standard deviation) performs better than the baseline scheme and better than the enhanced scheme with new reporting quantity of SINR statistics.

# Views on New Reporting for CSI Enhancements

## Case 1-3 new reporting

After RAN1#104-e meeting, there were additional email discussions on CSI feedback enhancements for URLLC/IIoT and companies’ inputs were collected in [6].  We recapture our views on Case 1-3 below.

- Benefits:  As shown in the performance evaluation results presented in the previous section, with the new reporting quantity of interference statistics (e.g., variance/standard deviation), Case 1-3 can achieve a 35% gain over the baseline scheme and a 14% gain over the Case 1-1 scheme.  There is no existing R16 solution available to provide the interference statistics to achieve the performance gain.

- Implementation complexity:  For Case 1-3, the impact on UE/gNB implementation complexity is low.  The interference part already needs to be measured anyway by UE based on assigned CSI-IM or NZP CSI-RS.  The UE just needs to derive the interference statistics, e.g., the variance/standard deviation of the interference, and reports it back to the gNB. The gNB can use, e.g., the variance/standard deviation of the interference, to set a backoff factor for SINR used in MCS selection.

- Specification impact:  The impact on specifications is low.  It only needs to add new reporting quantity for interference statistics, for example, interference variance/standard deviation. Legacy CSI framework can still be used.

- Testability:  The new report is testable such that inter-operability is achieved.  With controllable interference sources, e.g., in a lab environment, the variance of the interference can be set and testing can be conducted.

- Maturity:  The concept is simple and easy to be understood.  It is mature.

- Continue study: Yes, RAN1 should continue study of this scheme for R17 IIoT/URLLC.

Based on the above analysis, we propose the following:

  - Proposal 1: NR_IIOT_URLLC_enh supports new reporting quantity of interference statistics (e.g., variance/standard deviation).

## Case 2 new reporting

Regarding Case 2 new reporting, it was proposed to report additional information such as PDCCH/PDSCH decoding together with HARQ-ACK to improve the Open Loop Link Adaptation (OLLA) performance.  In our opinion, since the additional information such as PDCCH/PDSCH decoding only represent a snapshot of the channel and interference status at the PDCCH/PDSCH reception time, it gives little information about the interference at future PDCCH/PDSCH reception time due to the dynamic and unpredictable nature of interference.  Therefore, it is unclear how this additional information can help gNB improve MCS selection for the future PDCCH/PDSCH transmission considering the low latency requirements in URLLC.

Based on the above analysis, we propose the following:

Proposal 2: NR_IIOT_URLLC_enh should have a better understanding on how the other measurement such as PDCCH/PDSCH decoding can help gNB improve MCS selection for future PDCCH/PDSCH transmission before making a decision on Case 2 new reporting.

# Conclusions

In this contribution, we present our views on channel/interference measurement for new CSI reporting.  Based on the discussions in the previous sections, we have the following observation and proposals:

  - Observation 1: The CSI enhanced scheme with new reporting quantity of interference statistics (e.g., variance/standard deviation) performs better than the baseline scheme and better than the enhanced scheme with new reporting quantity of SINR statistics.

  - Proposal 1: NR_IIOT_URLLC_enh supports new reporting quantity of interference statistics (e.g., variance/standard deviation).

Proposal 2: NR_IIOT_URLLC_enh should have a better understanding on how the other measurement such as PDCCH/PDSCH decoding can help gNB improve MCS selection for future PDCCH/PDSCH transmission before making a decision on Case 2 new reporting.

# References

RP-201310, “Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, RAN#88e.

3GPP RAN1, RAN1#104-e, Chairman Notes.

R1-2100037, “CSI feedback enhancements for URLLC”, Futurewei, RAN1#104-e

R1-2100269, “CSI Feedback Enhancements for IIoT/URLLC”, Ericsson, RAN1#104-e

R1-2100835, “CSI feedback enhancements for URLLC/IIoT use cases”, Nokia, Nokia Shanghai Bell, RAN1#104-e

R1-2102749, “Summary of additional discussions on CSI feedback enhancements for enhanced URLLC/IIoT after RAN1#104-e”, Moderator (InterDigital, Inc.)

# Appendix

Table A - 1: Simulation assumptions

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8 Tx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4) dH = 0.5λ, dV = 0.8λ; |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 4 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2); dH = 0.5λ |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Simulation bandwidth | FDD 40 MHz |
| SCS | 30 kHz |
| Channel estimation | Realistic |
| Number of UEs per cell | 7 |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Indoor penetration loss is modelled according to low loss model |
| Traffic mode | FTP model 3 (100 packets/s), 200bytes/packet |