3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102763

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.1.2.3

Source:	Futurewei

Title:	Beam management for simultaneous multi-TRP transmission with multi-panel reception

Document for:	Discussion and decision

# Introduction

In RAN1#104-e meeting, the Rel. 17 NR FeMIMO WID [1] was discussed. The following was agreed [2] on the topic of enhancement on multi-beam operation:

R1-2101862 Moderator summary on M-TRP simultaneous transmission with multiple Rx panels (round 0) Moderator (CATT) Agreement For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) R1-2101973 Moderator summary on M-TRP simultaneous transmission with multiple Rx panels (round 1) Moderator (CATT) Agreement For M-TRP BFR Support 1-to-1 association between each BFD-RS set and an NBI-RS set - FFS: Association details Agreement BFRQ response - Support at least the same gNB response as in Rel.16 SCell BFR (i.e. DCI with toggled NDI scheduling a same HARQ process ID as the PUSCH carrying BFRQ MAC-CE) Agreement For BFRQ of M-TRP BFR - Option 3: Up to two dedicated PUCCH-SR resources in a cell group - FFS: Whether PUCCH-SR for SCell can be reused for M-TRP - Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found - Support at least indication of a single TRP failure - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE - FFS: whether/how to support indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell) - FFS: UE behavior when TRP failure status is different across cells - FFS: Whether PUCCH SR resource can be configured with 2 spatial relations R1-2102153 Moderator summary on M-TRP simultaneous transmission with multiple Rx panels (round 2) Moderator (CATT) Agreement For beam measurement in support of M-TRP simultaneous transmission - Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously - Support M = 2 - Support extending the maximum value of N > 1, exact value FFS - N=1 and N=2 - FFS: Other values larger than 2 - FFS: Whether the UE could report beams are received with different RX beams - Further study the support of option 1 and option 3 - The above applies at least for L1-RSRP - FFS: L1-SINR

In this contribution, we present our views on beam measurement/reporting enhancement and beam failure recovery for multi-TRP, and proposals for moving forward.

# Beam Measurement/Reporting Enhancement for Inter-TRP Beam Pairing

In RAN1#104-e meeting, the following was agreed regarding beam measurement/reporting enhancement to facilitate inter-TRP beam pairing.

Agreement For beam measurement in support of M-TRP simultaneous transmission - Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously - Support M = 2 - Support extending the maximum value of N > 1, exact value FFS - N=1 and N=2 - FFS: Other values larger than 2 - FFS: Whether the UE could report beams are received with different RX beams - Further study the support of option 1 and option 3 - The above applies at least for L1-RSRP - FFS: L1-SINR

With the above agreement, another FFS item is whether different beams reported by the UE that can be received simultaneously should be associated with different TRPs or different UE panels.  Since the goal of the beam measurement/reporting enhancement is to facilitate inter-TRP beam pairing, the different beams reported by the UE that can be received simultaneously should be from different TRP.  Otherwise, if the different beams reported are from the same TRP, those beams cannot be transmitted from the TRP simultaneously since it is not feasible for a single TRP to transmit multiple beams simultaneously in FR2, defeating the purpose of facilitating inter-TRP beam pairing.  Therefore, the different beams should be associated with different TRPs.  On the contrast, if the different beams are associated with different UE panels, although the UE can receive these beams simultaneously by using different UE panel for different beam, it is possible that those different beams are from the same TRP on different time.  And as mentioned earlier, it is not feasible for the TRP to transmit those beams simultaneously.  Therefore, the CSI report cannot facilitate inter-TRP beam pairing.

Based on the above analysis, we have the following proposal:

Proposal 1: Different beams reported by the UE that can be received simultaneously should be associated with different TRPs.

Regarding the number of beam pairs/groups N, we prefer that N can be 1, 2, or 4 to provide more flexibility to the gNB for scheduling.

Proposal 2: For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, in a single CSI-report, UE can report N(N=1, 2, and 4) pairs/groups and M (M=2) beams per pair/group,  and different beams within a pair/group can be received simultaneously and are associated with different TRPs.

In Rel. 15/16, group-based reporting is supported when the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled' [3].  In this case, the UE reports in a single reporting instance two different CRI or SSBRI for each report setting, where the CSI-RS and/or SSB resources can be received simultaneously by the UE.  The supported report quantities for group-based reporting are 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', and 'ssb-Index-SINR'.

However, in Rel. 15/16, there is no TRP identification associated with different CSI-RS and/or SSB resources.   In Rel. 17, to facilitate inter-TRP beam pairing and association of different beams to different TRPs, a groupID field can be added in the TCI-state information element, which is also discussed in our companion contribution [4].  The groupID field serves as a TRP identification.  Table 1 shows an example of how to add the groupID field to the TCI-state information element.  In Table 1, the red parts are the proposed additions/modifications to the TCI-state information element.  Below we focus on the introduction of groupID and physCellId which are more relevant to the topic.  Details on other parts can be found in [4].

- physCellId: this is the PCI of the serving cell or non-serving cell.  With the PCI info added to the TCI state, the TCI states of non-serving cell can also be configured for the UE, which could enable the UE with faster candidate beam identification and BFR utilizing the non-serving cell’s beams.

- groupID: in mTRP case, this ID could serve as the TRP identification.  With this ID, the UE can, for example, identify the TRP source of the CSI-RS.  The UE can then avoid reporting multiple beams from the same TRP to facilitate inter-TRP beam pairing, considering the fact that it is not feasible for one TRP to transmit multiple beams simultaneously in FR2.

Table 1: Example of enhancements on content of TCI state

TCI-State information element -- ASN1START -- TAG-TCI-STATE-START TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ... } QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R physCellId PhysCellId OPTIONAL, groupId GroupId OPTIONAL, bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index, srs SRS-ResourceId }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } -- TAG-TCI-STATE-STOP -- ASN1STOP

Based on the above analysis, we have the following proposal:

Proposal 3: FeMIMO supports adding TRP identification (e.g., groupID) to TCI state for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing.

# Beam Failure Recovery

In RAN1#103-e and #104-e meeting, the following was agreed regarding beam failure recovery.

Agreement - For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set. - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP - Support at least one of explicit and implicit BFD-RS configuration - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured - FFS: Further study QCL relationship between BFD-RS and CORESET - FFS: How to determine implicit BFD-RS configuration, if supported - For M-TRP new beam identification - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured - FFS: detail on association of BFD-RS and NBI-RS - Support the same new beam identification and configuration criteria as Rel.16, including L1-RSRP, threshold Agreement For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) Agreement For M-TRP BFR Support 1-to-1 association between each BFD-RS set and an NBI-RS set - FFS: Association details

Regarding explicit and implicit BFD-RS configuration, it is noted that this topic is related to item on enhancements on multi-beam operation that is under discussion in Agenda Item (AI) 8.1.1.  In  AI 8.1.1, it was agreed that on Rel. 17 unified TCI framework, for the separate DL TCI, the source reference signal(s) in M TCIs provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC.  However, the value of M (e.g., whether M > 1 is supported or only M = 1 is supported) is not decided.  If the Rel-17 unified TCI framework support M>1 TCIs such that each of the TCI provide QCL information for UE-dedicated reception on a subset of CORESETs in a CC, then for the explicit BFD-RS configuration, the QCL relationship between BFD-RS and CORESET can be defined such that the BFD-RS are QCLed with PDCCH DMRS with respect to 'QCL-TypeD', e.g., similar to those defined in Rel. 15/16.  However, if the Rel-17 unified TCI framework support only M=1 TCI and the TCI provide QCL information for UE-dedicated reception on all of CORESETs in a CC, then the BFD-RS should not be QCLed with PDCCH DMRS with respect to ‘QCL-TypeD’ since there is only one beam available for all the CORESETs.  If the implicit BFD-RS configuration is defined similar to those defined in Rel. 15/16, e.g., the BFD-RS is determined based on the QCL-TypeD RS of the activated TCI states for PDCCH reception, the above consideration also applies.

Based on the above analysis, we have the following proposal:

Proposal 4: FeMIMO supports BFD-RS are QCLed with PDCCH DMRS with respect to 'QCL-TypeD' assuming that the Rel-17 unified TCI framework supports M>1 DL TCIs such that each of the TCI provides QCL information for UE-dedicated reception on a subset of CORESETs in a CC.

# Conclusions

In this contribution, we present our views on enhancement on beam management for multi-TRP.  Based on the discussions in the previous sections we propose the following:

Proposal 1: Different beams reported by the UE that can be received simultaneously should be associated with different TRPs.

Proposal 2: For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, in a single CSI-report, UE can report N(N=1, 2, and 4) pairs/groups and M (M=2) beams per pair/group,  and different beams within a pair/group can be received simultaneously and are associated with different TRPs.

Proposal 3: FeMIMO supports adding TRP identification (e.g., groupID) to TCI state for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing.

Proposal 4: FeMIMO supports BFD-RS are QCLed with PDCCH DMRS with respect to 'QCL-TypeD' assuming that the Rel-17 unified TCI framework supports M>1 DL TCIs such that each of the TCI provides QCL information for UE-dedicated reception on a subset of CORESETs in a CC.

# References

RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung, RAN#86.

3GPP RAN1, RAN1#104-e, Chairman Notes

TS 38.214, “NR: Physical layer procedures for data (Release 16)”.

R1-2102767, “Enhancement on multi-beam operation”, Futurewei, RAN1#104-bis-e.