3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102850

e-Meeting, April 12– April 20, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev | - | Current version: | V16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on multi-TB gap for multicast in NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB-IoTenh3-Core |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In subclause 16.4.1 of TS36.213, when the scheduling gap or the processing gap is inserted between the TBs for multicast, the subframes for multiple TBs transmission is not N consecutive NB-IoT DL subframe(s) any more. However, in current spec, after the gap inserted, the N NB-IoT DL subframe(s) for multicast transmission with multi-TB scheduling is still consecutive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For DL NB-IoT subframes for multi-TBs transmission, the subframes of scheduling gap and processing gap for multicast are not counted in N NB-IoT DL subframe(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The N NB-IoT DL subframes counting is not correct when the scheduling gap or processing gap is inserted for multicast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

<Unchanged parts are omitted>

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in

-	n+5 DL subframe for FDD,

-	n+5 subframe for TDD,

the corresponding NPDSCH transmission in N NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	, where the value of  is determined by the repetition number field in the corresponding DCI (see Clause 16.4.1.3), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.4.1.3), and the value of is determined by the Number of scheduled TB for Unicast field or Number of scheduled TB for SC-MTCH field, if present, in the corresponding DCI,  otherwise,

-	k0 is the number of NB-IoT DL subframe(s) starting in DL subframe n+5 for FDD or subframe n+5 for TDD, until DL subframe n0, where k0 is determined by the scheduling delay field () for DCI format N1, and k0 = 0 for DCI format N2. For DCI CRC scrambled by G-RNTI, k0 is determined by the scheduling delay field () according to Table 16.4.1-1a, otherwise k0 is determined by the scheduling delay field () according to Table 16.4.1-1. The value of is according to Clause 16.6 for the corresponding DCI format N1,

-  if  and NPDSCH corresponding to an NPDCCH with DCI CRC scrambled by G-RNTI,

-  if multiTB-Gap is not configured and ,  subframe(s) ni is the first NB-IoT DL subframe after subframe , where  is processing gap with 20 subframes, δd=1, d=00,d≠0, and   is the number of overlapped subframes if the jth processing gap overlaps with the NPDSCH transmission gap defined in [3] and .

- otherwise, subframe(s) ni is the first NB-IoT DL subframe after subframe , where is given by higher layer parameter multiTB-Gap,  δd=1, d=00,d≠0, and  is the number of overlapped subframes if the jth scheduling gap overlaps with the NPDSCH transmission gap defined in [3] and .

-	otherwise, subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages where, n0<n1<…,nN-1 ,

-	for ,

-	if the UE is configured with higher layer parameter multiTB-Config in npdsch-MultiTB-Config set to 'interleaved', and NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

<Unchanged parts are omitted>