3GPP TSG RAN WG1 #104b-e		R1-2102947

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Maintenance on multi TRP

Agenda Item:	7.2.6

Document for:	Discussion and Decision

Introduction

This paper discusses some remaining issues and suggests text proposals on Rel-16 M-TRP.

Discussions and corrections on TCI States

In previous meetings [1][2], there was agreement on default QCL for AP CSI-RS when the scheduling offset of AP CSI-RS is less than beamSwitchTiming for both single-DCI-based and multi-DCI-based M-TRP, and the default QCLs of PDSCH for Scheme 3 and Scheme 4 for single-PDCCH based multi-TRP system when the scheduling offset is less than timeDurationForQCL were agreed. Similarly, there are some other cases where more details of the default QCL(s) for PDSCH need to be specified depending on DCI format and the transmission schemes. Many companies have recognized these issues as important ones and the specification would not be complete if there were no further agreements.

In this section we would like to elaborate our views and provide our proposals on these issues.

Default QCL for PDSCH

As depicted in [3], “Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-		…

-	If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers)”

The common understanding is that when at least one TCI codepoint indicates two TCI states, the UE may buffer signals with two default TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. For Scheme 3 and Scheme 4, the mapping of the two default TCI states applied to the PDSCH reception by the UE shall depend on the decoding of the scheduling DCI because only after the successful decoding the UE is able to identify the transmission scheme. However, default QCL for Scheme 2a/2b and cross-carrier scheduling is left unsolved.

Default QCL for Scheme 2a/2b

In Scheme 2a/2b, comb-like frequency resource allocation between two TRPs. For wideband PRG, first ⌈N_RB/2⌉ RBs are assigned to TCI state 1 and the remaining ⌊N_RB/2⌋ RBs are assigned to TCI state 2. For PRG size=2 or 4, even PRGs within the allocated FDRA are assigned to TCI state 1 and odd PRGs within the allocated FDRA are assigned to TCI state 2. The abovementioned TCI state 1 and 2 are the two TCI states indicated by DCI. If the offset between the reception of the DL DCI and scheduled PDSCH of Scheme 2a/2b are less than the threshold timeDurationForQCL, although the UE has buffered signals with two default TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, it is desired to specify the mapping between the two default TCI states and the allocated frequency resources to different TRPs for the sake of low UE implementation complexity and aligned network and UE assumption on the default TCI state, just like Scheme 3 and Scheme 4. Therefore, we have following proposal:

For default TCI-states for PDSCH of Scheme 2a/2b, when the time offset between the DCI and the PDSCH is less than the threshold, the mapping between two default TCI states and frequency resources of PDSCH follows the mapping specified for indicated TCI states in Section 5.1.2.3 in TS 38.214.

PDSCH default QCL for cross-carrier scheduling

For non-CA case, the default QCL(s) of a scheduled PDSCH depends on the maximum number of TCI states mapped to a codepoint in the MAC CE activation command, or the configured number of values of CORESETPoolIndex, supporting both single-TRP and multi-TRP cases. However, for cross-carrier scheduling, the default QCL for a PDSCH follows the activated TCI state with the lowest ID in the active BWP which was obviously specified for single TRP. To align with non-CA case, following default QCL determination can be applied to the cross-carrier scheduling for both single-PDCCH-based and multi-PDCCH-based M-TRP cases for cross carrier scheduling.

For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

For M-TRP in cross carrier scheduling, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

In general, we have following text proposal on clause 5.1.5 of TS38.214.

Text proposal on clause 5.1.5 of TS38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', - the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets, - the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same coresetPoolIndex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA', 'FDMSchemeA', 'FDMSchemeB' or is configured with higher layer parameter repetitionNumber and two TCI states are indicated in a DCI with 'Transmission Configuration Indication' field, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 or 5.1.2.3 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers) - In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH is configured with qcl-Type set to 'typeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with enableDefaultBeam-ForCCS: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell; if a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Miscellaneous

PUCCH for HARQ-ACK feedback and other UCIs

In RAN1 #98bis [1], PUCCH/PUSCH collision between different TRPs is avoided by implementation when separate HARQ-ACK feedback is configured.

Agreement For multi-PDCCH based multi-TRP/panel transmission, when separated ACK/NACK feedback is enabled, - PUCCH/PUSCH collision between different TRPs can be avoided by implementation and UE doesn’t expect overlapping PUCCHs/PUSCHs transmission toward different TRPs. For PUCCH/PUSCH transmission toward the same TRP, Rel-15 multiplexing rules apply. - Note that PUCCH resources can be associated with values of higher layer index per CORESET so that indices may be used to differentiate TRP to determine whether there is overlapping among TRPs. PUSCH can be differentiated by scheduling CORESET in terms of targeted TRP.

As stated in [2], “HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE”. And a UE configured with separate HARQ-ACK feedback does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs.

For M-TRP operation, it may be required to transmit PUCCH for CSI, SR or LRR. However, it is not clear the association between the CORESETPoolIndex and a PUCCH for P/SP-CSI, SR or LRR. Thus, we prefer to clarify the corresponding behavior when a PUCCH for CSI, SR or LRR collides with PUCCH for HARQ-ACK. Considering the above agreements, some restrictions can be addressed as follows:

A UE doesn’t expect a PUCCH for CSI/SR/LRR overlap with more than one PUCCH for HARQ-ACK.

If a PUCCH with HARQ-ACK overlaps with a PUCCH with CSI/SR/LRR information, the UE shall apply the procedures specified in Rel-15.

Adopt following text proposal in section 9 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ackNackFeedbackMode = separate the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a PUCCH with HARQ-ACK overlaps with a PUCCH with CSI/SR/LRR, the UE shall apply the procedures described in Clause 9.2. A UE doesn’t expect a PUCCH with CSI/SR/LRR overlap with more than one PUCCH with HARQ-ACK information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

PDSCH overlapping with PDCCH from different TRPs

In current spec[3], UE behaviour is clearly specified when the PDSCH overlaps with its scheduling PDCCH: “If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH.”

However, “When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, the following operations are allowed:

-	For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of coresetPoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of coresetPoolIndex that ends later than symbol i.”

It implies that the PDSCH scheduled by a PDCCH from one TRP may overlap with another PDCCH from a different TRP. Meanwhile, “a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH”, which includes the case that the PDCCH associated with any CORESETPoolIndex in our understanding. To avoid the unclear UE behavior to deal with such cases, we propose to have the follow restriction.

Conclude that UE does not expect to be scheduled a PDSCH overlapping with a PDCCH associated to CORESET having different CORESETPoolIndex from the scheduling PDCCH.

Conclusion

We have following proposals regarding to Rel-16 M-TRP.

- For default TCI-states for PDSCH of Scheme 2a/2b, when the time offset between the DCI and the PDSCH is less than the threshold, the mapping between two default TCI states and frequency resources of PDSCH follows the mapping specified for indicated TCI states in Section 5.1.2.3 in TS 38.214.

For M-TRP in cross carrier scheduling, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

Text proposal on clause 5.1.5 of TS38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', - the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets, - the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same coresetPoolIndex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA', 'FDMSchemeA', 'FDMSchemeB' or is configured with higher layer parameter repetitionNumber and two TCI states are indicated in a DCI with 'Transmission Configuration Indication' field, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 or 5.1.2.3 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers) - In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH is configured with qcl-Type set to 'typeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with enableDefaultBeam-ForCCS: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell; if a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt following text proposal in section 9 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ackNackFeedbackMode = separate the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a PUCCH with HARQ-ACK overlaps with a PUCCH with CSI/SR/LRR, the UE shall apply the procedures described in Clause 9.2. A UE doesn’t expect a PUCCH with CSI/SR/LRR overlap with more than one PUCCH with HARQ-ACK information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Conclude that UE does not expect to be scheduled a PDSCH overlapping with a PDCCH associated to CORESET having different CORESETPoolIndex from the scheduling PDCCH.

References

- Chairman's Notes, RAN1 #98bis.

- 3GPP TS 38.213 V16.5.0.

- 3GPP TS 38.214 V16.5.0.