3GPP TSG RAN WG1 #104b-e																				R1-2102973

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.15.2

Source:		Xiaomi

Title:	Discussion on time and frequency synchronization for IoT NTN

Document for:	Discussion

Introduction

In RAN#86 e-meeting, the SI of “New Study WID on NB-IoT/eTMC support for NTN” was approved and the SID was updated in [1]. This Study will evaluate and confirm solutions to address the minimum necessary specifications for NB-IoT and eMTC according to the following objectives.

The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items:

-	Aspects related to random access procedure/signals [RAN1, RAN2]

-	Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

-	Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

-    Aspects related to HARQ operation [RAN2, RAN1]

-	General aspects related to timers (e.g. SR, DRX, etc.) [RAN2]

-	RAN2 aspects related to idle mode and connected mode mobility [RAN2]

-	RLF-based for NB-IoT

-	Handover-based for eMTC

-	System information enhancements [RAN2]

-	Tracking area enhancements [RAN2]

NOTE 3: 	GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

Recommendations for NB-IoT and recommendations for eMTC will be documented in the conclusions.

In RAN1#104-e meeting, the following agreements were achieved for UL time and frequency synchronization for IoT NTN [2].

Agreement:

Study potential impact of GNSS Position fix on UE power consumption using battery life methodology in Rel-13 TR 45.820 (Section 5.4)

FFS: Details of the study

Agreement:

Discuss whether GNSS measurement window is needed and beneficial for initial access.

Agreement:

For the study of potential impact of GNSS Position fix on UE power consumption consider at least the following parameters

- GNSS power consumption value

- GNSS position Time To First Fix

Agreement:

Study potential impact of NTN SIB carrying the satellite ephemeris on

- UE power consumption in NB-IoT and eMTC

- Accuracy of satellite location tracking

- PRACH congestion

Agreement:

Study the UE pre-compensation of satellite delay during long UL transmission on (N)PUSCH in NB-IoT and eMTC.

Agreement:

Study the UE pre-compensation of satellite delay and Doppler during long UL transmission on PRACH in NB-IoT and eMTC.

Agreement:

Study the UE pre-compensation of satellite Doppler shift during long UL transmission on (N)PUSCH in NB-IoT and eMTC.

DL synchronization

DL initial synchronization performance based NB-IoT/eMTC synchronization signals should be evaluated. For the LEO without pre-compensation of the frequency offset, additional complexity is needed at UE receiver to achieve DL initial synchronization based on NB-IoT/eMTC synchronization signals. Beam specific pre-compensation of common frequency shift, e.g., conducted with respect to the spot beam centre at network side, should be considered.

Proposal 1: Pre-compensation on the Doppler shift for DL transmission should be supported.

Random access

In IoT NTN, UE with GNSS capability is assumed [1]. Through detecting UE position and satellite position, UE can acquire the UL frequency/timing offset of service link. Based on the results summarized in [3], it can be predicted that with assumption on pre-compensation of timing and frequency offset at UE side for UL transmission, existing NB-IoT/eMTC PRACH formats and preamble sequences can be reused. The necessity of additional enhancements, e.g., repetitions and/or larger CP, to ensure UL coverage can be further discussed.

However, in case pre-compensation of timing and frequency offset is not performed at UE side for UL transmission, enhanced PRACH formats and/or preamble sequences should be studied.

Observation 1: Existing NB-IoT/eMTC PRACH formats and preamble sequences can be reused with the assumption UE having GNSS capability.

Maintenance for UL timing advance and frequency synchronization

In IoT NTN, UE with GNSS capability is assumed [1]. Through detecting DL reference signals, UE position and satellite position, UE can acquire the UL frequency/timing offset of service link. Therefore, both the estimation and pre-compensation of UE-specific UL frequency/timing offset of service link can be conducted at the UE side.

In general, we should reuse the output of NTN WI on timing and frequency cocmpensation mechanism. One exception point is how to reduce the computation complexity of IoT UE to reduce the power cosumption.

Proposal 2: Reuse the UL time and frequency synchronization mechanism for IoT NTN in short UL transmission while taking into account the UE power consumption.

NPUSCH transmission can be with up to 128 repetitions, where a TBS can be transmitted over up to 10 RUs. With sub-carrier spacing of 15 kHz, the transmission time can be up to 1 second; with single tone transmission with sub-carrier spacing 3.75 kHz, the transmission time can be up to 4 second. Assuming a transmission time of 1 second, the delay drift can be 26 us. This would result in loss of OFDM orthogonality with significantly impact on NPUSCH and PRACH detection performance. In order to track the delay drift, it is necessary that the UE adjusts uplink timing in every transmission interval.

There are three options for UE pre-compensation during long UL transmission on NPUSCH and PRACH:

- Option 1: Use UE-specific TA calculation based on GNSS-acquired UE position and serving satellite ephemeris.

- Option 2: Use UE-specific TA calculation based on the timing drift rate.

Since simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed, it seems reasonable to support option 2 for that GNSS measurement is switched off when IoT module is working.

Proposal 3: Use UE-specific TA calculation based on the timing drift rate for UE pre-compensation during long UL transmission.

Observation 2: Segmented UE pre-compensation of satellite delay is needed during long UL transmission.

The radial speed of NGSO satellite as seen by the UE on the service link can reach significant values. As a consequence, the Doppler shifts experienced on the service DL and UL link become critical in the NGSO scenarios [3] (cf. Table ). DL and UL Doppler shift can be observed:

- DL RX signal at UE side is going to be affected by the un-compensated Doppler shift due to satellite mobility.

- UL RX signal at gNB side is going to be affected by the Doppler shift due to satellite mobility.

Table 1 : Max Doppler shift/drift to be supported in NTN scenarios

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) |

Assuming a transmission time of 1 second, max Doppler shift variation can be 0.54Hz. Doppler shift variation is little even in the long UL transmission.

Observation 3: Segmented UE pre-compensation of satellite Doppler shift is not needed.

Conclusions

In this contribution, we discuss time and frequency synchronization for NB-IoT/eMTC over satellite, the following observations and proposals have been made:

Observation 1: Existing NB-IoT/eMTC PRACH formats and preamble sequences can be reused with the assumption UE having GNSS capability.

Observation 2: Segmented UE pre-compensation of satellite delay is needed during long UL transmission.

Observation 3: Segmented UE pre-compensation of satellite Doppler shift is not needed.

Proposal 1: Pre-compensation on the Doppler shift for DL transmission should be supported.

Proposal 2: Reuse the UL time and frequency synchronization mechanism for IoT NTN in short UL transmission while taking into account the UE power consumption.

Proposal 3: Use UE-specific TA calculation based on the timing drift rate for UE pre-compensation during long UL transmission.

References

- RP-210868	Revised SID: Study on NB-IoT/eMTC support for Non-Terrestrial 	MediaTek Inc., Eutelsat Network    March 16th - 26th, 2021

- Chairman’s Notes RAN1#104-e

- TR 38.821 v16.0.0	Solutions for NR to support non-terrestrial networks (NTN), v16.0.0, December, 2019