3GPP TSG RAN WG1 #104b-e	R1-2103120

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.10.1

Source:	Apple Inc.

Title:	Enhanced resource multiplexing within an IAB node

Document for:	Discussion, Decision

# Introduction

Rel-17 WI to enhance integrated access and backhaul (IAB) aims the following RAN1-led objectives, related to duplexing enhancements [1]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we present our views on potential techniques and required signalling to enhance resource multiplexing between IAB node’s child and parent links.

# SDM Resource Multiplexing within an IAB-node

IAB specification in Rel-16 is mainly limited to TDM operation within DU and MT components of an IAB node. In Rel-17, and as part of enhanced resource multiplexing, SDM and FDM multiplexing within an IAB node is being discussed. In that regard, it was agreed in RAN1#104-e that [2]:

Agreement: Support indication/reporting of information between an IAB node and its parent node to assist in the determination of the applicability of a given multiplexing capability in case of simultaneous operation. The following solutions are considered (other solutions not precluded):

- Temporal applicability of a given multiplexing capability

- Time/frequency resource restrictions (e.g. access vs. backhaul links, DL vs. UL resources)

- Indications of conditions/reporting information required to realize the given multiplexing capability, (e.g. timing mode, power control, guard symbols, etc.)

FFS: channels/signals used for indicating/reporting information

Agreement: Further study whether/how to manage resources in the spatial domain. Candidate solutions are:

- Dynamic signaling between parent and child nodes for using/restricting/sharing antenna panels/beams

- Beam management / multi-panel enhancements for simultaneous operations

- Extension of H/S/NA resource indication to the spatial domain

For a successful simultaneous operation in SDM multiplexing mode within IAB-DU and IAB-MT, parent node IAB-DU and IAB-DU should be aware of the desired (or used) parameters in the other side like beam/panel, SLIV, transmit power, DMRS ports. For example, if parent IAB-DU wishes to schedule a PDSCH to IAB-MT, ideally it should have some indication from IAB node about which panel/beam for PDSCH transmission is desired once the IAB-DU is simultaneously receiving from child IAB-MT (or UE), as shown in Fig. 1. Alternatively, parent IAB-DU will indicate to the IAB node about the panel/beam to receive Lp,DL. Next, IAB-DU in IAB-node will choose the reception beam/panel for LA,UL / Lc,UL links, with minimum impact on Lp,DL and will consequently indicate to its child IAB and/or access UE about their transmit beam/panel, e.g. through SRI for PUSCH. The choice between the above options can be based on the communication link and/or data priority, e.g. if IAB-DU schedules a UE (on access link LA,UL) with URLLC traffic, it will be IAB-DU that controls the flow. Alternatively, if traffic on Lp,DL is higher priority than traffic on Lc,UL, it will be the parent IAB-DU that controls the flow. Based on the above discussions, we have the following proposals:

Figure 1: Simultaneous reception within an IAB-node.

Proposal 1: For simultaneous SDM operation, either parent IAB-DU or IAB-DU can initiate and control the flow.

Proposal 2: The entity with higher data priority will control the flow by initiating the scheduling on high priority link and/or indicating the desired parameters to the other link

Regardless of whether IAB-node or IAB-parent controls resource coordination for SDM multiplexing, it should be discussed how to indicate the beam management/coordination parameters. Whether the indication is dynamic or not also depends on the status of communication links, e.g.  for semi-static/periodic transmissions from parent IAB-DU, like periodic CSI-RS, SSB, SPS-PDSCH, a semi-static beam indication from parent IAB-node is sufficient. While for a dynamic PUSCH scheduling on access link, a quick dynamic beam indication is preferred.

Proposal 3: For the case of parent IAB-DU indicating IAB node about the receiving beams/panels for Lp,DL and LA,UL / Lc,UL links, the indication can be dynamic and/or semi-static:

- Dynamic e.g. through TCI index in DCI for scheduling PDSCH Lp,DL

- Semi-static, e.g. through MAC-CE/RRC to IAB-MT or through F1 interface to IAB-DU

Proposal 4: For the case of IAB indicating desired reception beams/panels on Lp,DL and LA,UL / Lc,UL links, the indication to parent IAB-DU can be dynamic and/or semi-static:

- Dynamic e.g. through UCI transmission from IAB-MT to parent DU

- Semi-static, e.g. through MAC-CE from IAB-MT or through F1 interface from IAB-DU

- Alternatively, IAB-MT performs SR or initiates PRACH to indicate desired receiving beam on Lp,DL

To achieve a better resource efficiency under SDM multiplexing with simultaneous reception within an IAB node, a conditional indication from parent IAB-DU may be deployed, where IAB-node has the chance to provide feedback on allocation by parent IAB-DU, and possibly provide desired set of parameters. Figure 2 shows an example of the procedure. Here we note that, at least in some scenarios, better utilization of SDM resource multiplexing is achieved if PDSCH allocation crosses the slot border, since otherwise tighter slot/symbol alignment is required, which is not always possible/desirable. Based on this discussion, we make the following proposals:

Proposal 5: To achieve a simultaneous reception within an IAB node, i.e. DU-Rx & MT-Rx, a two-step indication is deployed

- Step1: Parent IAB-DU indicates initial beam/panel, K0, SLIV, etc, and a grace time for IAB node in which IAB-MT can indicate its desired parameters like beam/panel, K0, SLIV, etc

- Step2: IAB indicates its desired parameters (beam/panel, K0, SLIV, Tx power, etc), for example, on PUCCH indicated by DL DCI from parent IAB-DU

Proposal 6: To utilize SDM resource multiplexing, support PDSCH allocation for IAB-MT with crossing the slot border.

Figure 2: Two-step scheduling for SDM simultaneous Rx.

# Conclusions

In this contribution, we provided our thoughts on potential techniques to enhance resource multiplexing between IAB node’s parent and child links. Based on views shared in this paper, we make the following proposals:

Proposal 1: For simultaneous SDM operation, either parent IAB-DU or IAB-DU can initiate and control the flow.

Proposal 2: The entity with higher data priority will control the flow by initiating the scheduling on high priority link and/or indicating the desired parameters to the other link

Proposal 3: For the case of parent IAB-DU indicating IAB node about the receiving beams/panels for Lp,DL and LA,UL / Lc,UL links, the indication can be dynamic and/or semi-static:

- Dynamic e.g. through TCI index in DCI for scheduling PDSCH Lp,DL

- Semi-static, e.g. through MAC-CE/RRC to IAB-MT or through F1 interface to IAB-DU

Proposal 4: For the case of IAB indicating desired reception beams/panels on Lp,DL and LA,UL / Lc,UL links, the indication to parent IAB-DU can be dynamic and/or semi-static:

- Dynamic e.g. through UCI transmission from IAB-MT to parent DU

- Semi-static, e.g. through MAC-CE from IAB-MT or through F1 interface from IAB-DU

- Alternatively, IAB-MT performs SR or initiates PRACH to indicate desired receiving beam on Lp,DL

Proposal 5: To achieve a simultaneous reception within an IAB node, i.e. DU-Rx & MT-Rx, a two-step indication is deployed

- Step1: Parent IAB-DU indicates initial beam/panel, K0, SLIV, etc, and a grace time for IAB node in which IAB-MT can indicate its desired parameters like beam/panel, K0, SLIV, etc

- Step2: IAB indicates its desired parameters (beam/panel, K0, SLIV, Tx power, etc), for example, on PUCCH indicated by DL DCI from parent IAB-DU

Proposal 6: To utilize SDM resource multiplexing, support PDSCH allocation for IAB-MT with crossing the slot border.

# References

- RP-210758, “New WID on Enhancements to Integrated Access and Backhaul”, March 2021

- Chairman’s Notes RAN1#104-e final