| 3GPP TSG-RAN WG1 Meeting #104bis-e e-Meeting, April 12th –20th, 2021 | R1-2103154 |
| --- | --- |

Agenda item:	8.1.2.4

Source: 	Qualcomm Incorporated

Title: 	Enhancements on HST-SFN deployment

Document for:	Discussion/Decision

# Introduction

Release-17 RAN1 work-item on further enhanced MIMO (FeMIMO) in [1] scoped the following for the enhancement of high-speed train – single frequency network deployment for both FR1 and FR2.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we discuss the enhancement for HST SFN deployment including transmission schemes for PDSCH/PDCCH and adaptive DMRS configuration while addressing aspects related to signaling and configuration.

# Enhanced SFN DL transmission

In the last RAN1 meeting #104-e, it was agreed to support enhanced SFN scheme 1 in Rel-17 where DMRS and PDCCH/PDSCH are transmitted from both TRPs in SFN manner whereas the DMRS ports are associated with at most two TCI states for both FR1 and FR2. The DMRS port(s) of PDCCH/PDSCH are associated with two TCI states containing TRS as source reference signal. Variant E is supported where both TCI states are associated with QCL-TypeA and each TCI state may be additionally associated with QCL-TypeD.

Agreement Scheme 1 is supported in Rel-17 - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - FFS other details For scheme 1 and SFN transmission of PDCCH support Variant E for QCL assumption in TCI state when TRS is used as source RS Agreement Two TCI states are supported for scheme 1 in FR2

## SFN DL Transmission schemes

The different approaches for enhancing DL transmission of HST-SFN have been categorized into two different transmission schemes depending on whether DMRS is transmitted in SFN or nonSFN manner across the two TRPS.

### SFN Scheme 1: non-SFN TRS and SFN DMRS

In this scheme, the TRS is transmitted in non-transparent manner from both TRPs to the UE. The two variation of the scheme is shown in Figure 2-1. The picture to the left is TRP specific TRS where each TRP sends a specific TRS. The picture to the right represents a backward compatible enhanced SFN scheme 1 where the first TRS (TRS0) is transmitted in SNF manner from both TRS while TRS1 is transmitted from single TRP. The latter scheme helps the network to serve mixture of Rel-17 UE that are capable of the reception of enhanced SFN scheme 1 and legacy UE that can support only legacy (Rel-15) SFN transmission. It is worth mentioning that the Rel-17 UE in the later scheme with SFN TRS (TRS0) is still capable of estimating the Doppler shift from each TRP transmission.

Figure 2-1 Scheme 1 with non-SFN TRS; left (TRP specific TRS) and right (BC SFN TRS)

In first scheme with TRP specific TRS, the DMRS port(s) are associated with two TCI states in which TRS1 and TRS2 are the source reference signals and each TCI state is associated with QCL Type A. And similarly, for SFN scheme 1 with BC SFN TRS, the DMRS port(s) are associated with two TCI states in which TRS0 and TRS1 are the source reference signals and each TCI state is associated with QCL Type A. This QCL/TCI configuration is referred as variant E based on the latest RAN1 agreement in RAN 103e meeting.

| SFN Scheme-1 | TCI state #1 | TCI state#2 | QCL Variant |
| --- | --- | --- | --- |
| TRP specific TRS | TRS1: QCL Type A | TRS2: QCL Type A | Variant E |
| BC SFN TRS | TRS1: QCL Type A | TRS0: QCL Type A | Variant E |

For SFN scheme-1 with TRP specific TRS, the UE estimates the Doppler shift of each TRP from the corresponding TRS (TRS1/2) and extracts the overall delay spread and average delay based on combining the channel impulse response from each TRS. However, for scheme-1 with SFN TRS, the UE extracts the delay spread and average delay from the SFN TRS (TRS0). The Doppler shift of the first TRP is obtained directly from the TRS1 and Doppler shift of 2nd TRP is obtained based on the differential channel from TRS0 and TRS1. The processing of each TRS to obtain large scale time frequency properties of the SFN transmission is shown in Figure 2-2 and Figure 2-3 for SFN schemes with TRP specific TRSP and BC SNF TRS respectively.

Figure 2-2: SFN scheme-1 with TRP specific TRS

Figure 2-3  SFN scheme-1 with BC SFN TRS

Link-level study to evaluate the performance of both flavours of TRS as described above is shown Figure 2-4 at the track points between two adjacent TRPs. We considered system BW of 20MHz with 50RB allocation. The DL precoder is random precoder cycling per each TRP with PRG = 2RB. The UE is assumed to have 4 Rx and with speeds of 500 km/hr. The complete list of simulation assumptions is listed in the appendix at Table 2. The simulation is done using link-adaptation with fixed rank-1 and rank-2 at fixed SNR = 12 dB and 2-taps channel model is used.

Figure 2-4: Scheme-1 evaluation using 2-tap channel model for Rank-1 (left figure) and Rank-2 (right figure)

In Figure 2-5, the evaluation results of both TRS configuration are compared using the extended CDL-D channel model at the mid-track. The simulation is based on fixed MCS (MCS = 4, 13 and 17) with fixed rank-1 and rank-2.

Figure 2-5 Scheme1 evaluation with Fixed MCS using the extended CDL-D channel model at mid-track point.

It is clear from the results that both flavours of SFN scheme-1 have almost the same performance. Meanwhile, SFN scheme-1 with SFN TRS has the advantage of backward compatibility. As both options of TRS configuration are associated with two TCI states and same QCL type, the differentiation between both TRS configuration can be achieved by the configuration of higher layer parameter.

Observation 1: SFN scheme-1 with backward compatible SFN TRS has almost the same performance of Scheme-1 with distributed TRS (TRP-specific) while enabling backward compatible to pure SFN scheme.

Proposal 1: Support the introduction of higher layer parameter to differentiate between SFN scheme-1 with BC SFN TRS and TRP-specific TRS.

### SFN Scheme 2: non-SFN TRS and DMRS

In this scheme, to improve the DM-RS channel estimation, the DM-RS is transmitted in non-SFN manner from each TRP. However, the expected improvement comes with the cost of extra DMRS overhead as the number of DMRS ports need be at least doubled (e.g. two TRPs scenario).  Two variation of the scheme is shown in Figure 2-6 with the assumption of Rank-1 DL transmission. The picture to the left is TRP specific TRS and DMRS where each TRP sends its own TRS and DMRS port(s). The picture to the right represents a backward compatible enhanced SFN scheme 2 where the first TRS (TRS1) and DMRS port 0 are transmitted in SNF manner from both TRPs while TRS2 and DMRS port 1 is transmitted from only one TRP.

Figure 2-6: Scheme 2 with non-SFN TRS and non-SFN DMRS

Compared to SFN scheme-1, scheme-2 requires extra DMRS overhead which is not favourable from both gNB and UE side. From network side, it results into resources overhead compared to scheme 1. From UE side, it doubles the complexity of the DM-RS channel estimation and may affect UE processing timeline.

Observation 2: SFN transmission scheme-2 requires double the number of DMRS ports as compared to SFN scheme-1 which results into network resource overhead, increase of UE complexity and timeline.

For SFN scheme 2, there are two possible configuration DMRS ports mapping and configuration. The first option is where the DMRS ports mapped to the same PDSCH layer are transmitted using different CDM groups, which is equivalent that having a separate DMRS CDM group to each TRP. The second scenario, where the DMRS ports mapped to the same PDSCH layer are transmitted using the same CDM group. The latter approach has the same DMRS overhead as compared to scheme 1 when only single PDSCH layer is scheduled as it allows for FDMing PDSCH at the non-used DMRS resources. For the scenario of two layers, scheme 2 has extra DMRS overhead (roughly 12.5%) as compared to scheme-1 which results into lower achieved throughput. The DMRS ports mapping are shown in Figure 2-7.

Figure 2-7: DMRS port mapping where left picture shows the FDM of PDSCH and DMRS and right picture shows full utilization of DMRS symbols of both CDM groups

The LLS evaluation comparing both SFN schemes are shown below using 2-taps channel model and extended CDL-D channel model. The TRS is assumed distributed: one TRS per each TRP. Figure 2-8 shows the DL throughput performance comparing SFN schemes 1 and 2 at each track points. The simulation is done using link-adaptation with fixed rank-1 and rank-2 at fixed SNR = 12 dB. Figure 2-9 shows the performance comparison of both candidate schemes at fixed track location (mid-point) with different SNR values in which different DMRS CDM groups is configured SFN scheme 2. Finally, using the extended CDL-D with 100ns channel model, a comparison using fixed MCS is shown in Figure 2-10.The simulation parameters are summarized in Table 2 in the appendix.

Figure 2-8: Rank-1 (left figure) and Rank-2 (right figure) comparing the two candidate SFN schemes at the different track point and SNR = 12 dB

Figure 2-9: Rank-1 (left) and rank-2(right) evaluation results for candidate schemes at mid-point with different SNR

Figure 2-10: Fixed MCS comparison between SFN schemes using the extended CDL-D channel model at mid-track point

From performance point of view, it is clear from the results that due to the DMRS overhead of SFN scheme 2 with different CMD group, its performance is inferior as compared SFN scheme-1 for both rank-1 and rank-2. Meanwhile, SFN scheme-2 with same DMRS overhead as scheme 1 (DMRS ports are on same CDM group) achieves similar performance to scheme 1, however, this is limited only to rank-1 transmission.

Observation 3: Scheme 1 outperforms scheme 2 (with different CDM group per TRP) at mid-track point for the different SNR.

Observation 4: Scheme 2 with DMRS ports mapped to different CDM groups suffer from extra DMRS overhead as compared to scheme 1 which results into lower DL throughput.

Observation 5: Scheme 2 with DMRS ports mapped to same CDM group has same DMRS overhead and achieves same performance as scheme 1 only rank 1.

Based on the evaluation results where SFN scheme 2 does not outperform SFN scheme 1 as well as the drawbacks of SFN scheme-2 of network resource overhead and increase UE complexity, we believe SFN scheme 1 is sufficient and there is no need to support SFN scheme 2.

Proposal 2: No support of SFN scheme 2 in Rel-17.

## Signalling and configuration for SFN PDSCH scheme 1

### Configuration of SFN scheme 1

The UE processing for the enhanced SFN scheme is different from single TRP and the other Rel-16 M-TRP transmission scheme (TDM/FDM and SDM). For the UE to be prepared for the DL reception of the enhanced SNF scheme, an RRC configuration should be introduced. This is similar to Rel-16 RAN4 discussion, where new RRC parameters (highSpeedDemodFlag-r16 and highSpeedMeasFlag-r16) are introduced for legacy SFN scheme for enhanced demodulation processing and enhanced RRM requirements.

Proposal 3: Support the introduction of higher layer parameter for the introduction of SFN scheme 1 (PDSCH).

### Switching between scheme-1 and other transmission schemes

In the last RAN1 meeting #104-e, the switching between SFN scheme1 and Rel-16 M-TRP schemes was discussed. It was agreed to support semi-static switching between scheme 1 and TDM/FDM M-TRP schemes.

Agreement For HST-SFN scenario: - Support semi-static (RRC based) switching of scheme 1 (PDSCH) with 2a, 2b, 3, 4 - FFS all other details including RRC signaling, possible RAN4 impact (if any), etc.

In a similar way, the switching of scheme-1 with scheme 1a (SDM) should be semi-static. These are two different transmission schemes with different use-cases: improved reliability versus increase capacity. In HST environment, there is no proper justification or use case that necessitates dynamic switching between these two schemes schemes.

Proposal 4: Support semi-static (RRC based) switching of scheme 1 (PDSCH) with scheme 1a.

In similar way, the switching between scheme-1 and single TRP should be semi-static. From the UE perspective, HST scheme1 requires advanced tracking and demodulation processing that is different from single TRP operation. Forcing the UE to dynamically switch between single TRP and scheme1 is not acceptable neither justified. To support dynamic switching, the UE is required to track three channels: one channel for each TRP as well as the combined SFN channel. Also, the UE is required to add extra complexity to support simultaneously two different channel estimation algorithm and demodulation processing schemes: one for single TRP and the other one for scheme1. From deployment perspective, we do not see the need for a UE in HST to switch in the order of 1ms between single TRP and scheme 1.

Proposal 5: Support semi-static (RRC based) switching of scheme 1 (PDSCH) and single TRP.

## SFN PDCCH channel

In the RAN1 meeting #103-e, it was decided that the SFN PDCCH enhancement should be discussed under HST agenda and build on top of the agreements made under PDCCH reliability enhancement agenda 8.1.2.1.

Agreement For PDCCH reliability enhancements, support SFN scheme + Alt 1-1. - FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR Agreement For Alt 1 (one CORESET with two active TCI states), study the following - Alt 1-1: One PDCCH candidate (in a given SS set) is associated with both TCI states of the CORESET.

And in the last RAN1 meeting #104-e, it was agreed to support enhanced MAC-CE activation command that indicates the two TCI sates for PDCCH.

Agreement - Support MAC CE activation of two TCI states for PDCCH - FFS other details

Similar to the discussion of SFN PDSCH scheme 1, SFN PDCCH should be semi-static configured such that UE performs the proper channel tracking for the reception for the SFN PDCCH. Also, this is needed to differentiate between SFN PDCCH and enhanced non-SFN PDCCH with repetition.

Proposal 6: Support the introduction of higher layer parameter for the indication of SFN PDCCH scheme 1.

### CORESET TCI states configuration

In Rel-15, a list of TCI states IDs is RRC configured as part of the CORESET configuration with a maximum of 64 TCI states. A single TCI state is activated through the MAC-CE if the list has more than one TCI state ID. For the scheme-1 SFN PDCCH, the MAC CE activation is needed when the SFN CORESET is RRC configured with more than two TCI states. However, if only two TCI states are RRC configured for the SFN CORESET, there is no need for MAC-CE activation command.

Observation 6: Rel-17 MAC-CE activation of two TCI states for SFN PDCCH is required only when more than two TCI states are RRC configured in the CORESET.

### Default TCI state for PDSCH

A default TCI is used when the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’. The PDSCH may assume the same QCL parameters of the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE.

Therefore, for the determination of the default TCI state, when the lowest CORESET ID in the latest slot hast two activate TCI states, then either both TCI states or single TCI state is assumed for PDSCH depending on whether the UE supports the enhanced SFN PDSCH with multiple TCI states or not.

Proposal 7: If the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’ and the lowest CORESET ID in the latest slot has two active TCI states, the same TCI states are assumed for the PDSCH.

### TCI states of PDSCH with absent TCI field

On the other hand, when the time offset between the reception of DL DCI and the corresponding PDSCH is equal or larger than the threshold ‘timeDurationForQCL, then TCI state and the corresponding QCL assumption of the PDSCH is determined form the TCI state of the scheduling CORESET when ‘tci-PresentInDCI’ is enabled but the TCI field in the DCI scheduling PDSCH is not present. Similar rule can be adopted when the TCI field is not present in a DCI received on CORESET with two activated TCI states and when time offset is larger the threshold such that the two activated TCI states of the CORESET are assumed for the PDSCH.

Proposal 8: The UE assumes the TCI states of the PDSCH is identical to the TCI states of the scheduling CORESET when the TCI filed is absent in the DCI and the time offset is equal to or larger than the threshold of ‘timeDurationForQCL’.

### BFD resource for BFR

A CORESET with two active TCI states can be configured with two RSs with QCL Type-D. Rel.15 support only singe active TCI state CORESET and equivalently single RS with QCL Type-D. Therefore, RAN1 should discuss how BFD resource for BFR should be selected. One solution is to consider only CORESETs with single active TCI states, when subset of CORESETs have two active TCI states, and apply the same rules for BFD resource selection in Rel-15. The other solution is to consider CORESETs with two active TCI states as well and define some rules for selecting one or both RSs for link quality assessment.

Proposal 9: For the selection of BFD resource for BFR, RAN1 to discuss both options:

- Alt-1: Consider only CORESETs with single active TCI states.

- Alt-2: Consider both CORESETs with single and dual TCI states and define some rules for BFD RS selection.

# Doppler shift pre-compensation

Doppler shift pre-compensation is a network-based solution in which network determines the UE downlink Doppler shift frequency experienced from each RRH (e.g., by using the UE uplink signals) and then compensates DL transmission from each RRH by the corresponding Doppler frequency shift. This mechanism should help improving the DL throughput as theoretically, the received DL signal from each RRH should have almost zero Doppler shift and small Doppler spread as compared to no compensation scheme.

In the last RAN1 meeting, TRP pre-compensation scheme was intensively discussed. There were few concerns from some companies on the accuracy of the pre-compensation as well as the benefits of the scheme compared to enhanced SFN scheme 1 and DPS transmission scheme. In the end, it was concluded to decide on whether the scheme is supported or not in RAN1#104-e-bis meeting. To that extent, we first review the procedure of TRP pre-compensation scheme, then discuss the factors that affect the accuracy of the procedure and finally provide evaluation results.

Conclusion The decision on support of specification based TRP pre-compensation scheme for HST-SFN scenario to be made in RAN1#104-e-bis meeting. To facilitate RAN1 decision, companies are encouraged to provide evaluation results according to the agreed evaluation assumptions. The evaluations not compliant with agreed assumptions will not be considered by RAN1 in the decision process.

## Procedure and indication DL Doppler shift pre-compensation

In RAN1 meeting #103-e, different variants of the TCI/QCL configurations were discussed depending on the network configuration of the TRS and whether the DL transmission is Doppler shift compensated or not.

Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. - This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation.

Similar to the discussion in section 2.1.1, there are two main approaches for network-based pre-compensation schemes based on the TRSs configuration. In the first scheme, The TRSs are transmitted in non-transparent manner from both TRPs to the UE (i.e. TRP-specific TRS). The second scheme is a backward compatible enhanced SFN scheme 1 where the first TRS (TRS1) is transmitted in SNF manner from both TRS while TRS2 is transmitted from single TRP. For each scheme, the two TCI states have different QCL configuration for the source TRS reference signal as summarized in the Table below. Based on RAN1-103e agreement of the QCL variant, the first scheme is referred as variant A or C and the second scheme is referred as variant B.

| SFN Transmission Scheme | TCI state #1 | TCI state#2 | QCL Variant |
| --- | --- | --- | --- |
| BC SFN TRS | QCL Type B | New QCL Type = {‘average delay’, ‘delay spread’} | Variant B |
| Distributed TRS (TRP specific TRS) | QCL Type A | New QCL type = {‘average delay’, ‘delay spread’} | Variant A |

Figure 3-1 Doppler Shift pre-compensation w/ BC SFN TRS (variant B)

Figure 3-1 shows the procedure of Doppler shift pre-compensation framework with the backward compatible (BC) SFN TRS. In this procedure, TRS0 is SFNed TRS transmitted from both TRPs and TRS1 is a non-transparent TRS transmitted from TRP1 which is the primary TRP (UE closer to TRP1 than TRP2). The UE utilizes the single TRP TRS to run the FTL loop and estimate the Doppler shift. Also, the UE changes the center frequency of the UL transmission based on TRS1. This enables the network to pre-compensate the PDSCH from the second TRP (TRP2) by the delta of the frequency shifts observed at both TRPs. Then, the PDSCH reception will be pre-compensated and frequency shift aligned with TRS1. In this case, the network will differentiate between the SFNed TRS and the transparent TRS based on the QCL type. More details are discussed at section 3.3.2

Figure 3-2: Doppler Shift pre-compensation w/ TRP-specific TRS(variant A)

## Accuray of SFN Pre-Compensation Schems

There are multiple factors that affects the accuary of the TRP pre-compensation schems whether Doppler shift is either explicitly indicated by the UE or implicitly measured by the network. For the implicit method based on the UL signal or UL channel, there are three main factors:

- Frequency estimation errors per each TRP.

- Doppler decorrelation between the time instance where Doppler shift is estimated by the basestaion to the time insance where the pre-compensated DL signal is transmitted

- Non-perfect frequency synchronization between the TRPs and the per-TRP carrier frequency offset which is according to RAN4 specs is up to 0.01 or 0.05 ppm for wide range and medium range basestation accordingly.

For the explicit method based on UE reporting, there are also three factors:

- The quatization error of Doppler shift measurements

- Doppler decorrelation between the time instance where Doppler shift is estimated by the UE to the time insance where the pre-compensated DL signal is transmitted

- Non-perfect frequency synchronization between the TRPs and per-TRP carrier frequency offset which is according to RAN4 specs is up to 0.01 or 0.05 ppm for wide range and medium range basestation accordingly.

Therefore, considering these non idealities, we can’t assume perfect pre-compensation and realstic assumptions should be considered for the evaluation of network based pre-compensation schemes. Also, for weak LOS channel, Doppler shift and Doppler spread cannot be decomposed with good accuracy which will impact the accuracy of Doppler shift estimation.

Observation 7: Perfect pre-compensation is a not a realistic assumption due to many factors that affects the accuracy of Doppler shift measurements/estimation or quantization error by reporting, non-prefect frequency synchronization between the TRPs as well as the temporal variation of CFO per TRP.

## Evaluation results

Given the earlier discussion on the accuarcy of pre-compensation schemes, we considered in our analysis non-perfect frequency pre-compensation where the pre-compensated PDSCH per each TRP has a frequency error (Freqerr). This frequency error is moded as a random uniform error with betwneen (-Freqerr  Freqerr) per each TRP. We evaluated the performance for different values of the freq. error = 0, 150, 250 and 350 Hz.

Using the CLD-D 100ns extended channel model, we compare the performance beween the variant (A/B) of  SFN scheme-1 with PDSCH pre-compensation. The results are shown in Figure 3-3. The first figure to the left shows the BC SFN (variant B) and the figure to the shows the results of TRP-specific (variant A) for rank-1. And Figure X shows similar results for rank-2.

Figure 3-3: Evaluation results for SFN pre-compensation schemes variant A/B for rank-1

Figure 3-4: Evaluation results for SFN pre-compensation schemes variant A/B for rank-2

As expected, the performance of both variants of DL pre-compensation schemes are similar given the similarity of the pre-compensation procedure. Also, it is very clear from the results that the performance sensitivity for the frequency error is larger for higher rank and higher MCS.

Using the same channel propagation model, CLD-D 100ns extended channel model, we then compare the performance  of the tow variant (A/B) of  SFN scheme-1 with PDSCH pre-compensation versus single TRP based on DPS. A per TRP CFO is assmued as a random uniform error with betwneen (-Freqerr  Freqerr). We evaluated the performance for different values of the freq. error = 0, 150, 250 and 350 Hz.

The results are shown in Figure 3-5 and Figure 3-6 for SNR = 12 dB and 20 dB equivantelyFigure 3-3. For CFO in the range of 0Hz to 150 Hz and SNR = 12 dB, we observe gain of TRP pre-compensation compared to DPS. However for higher range of CFO, the gain vanishes.  For 20 dB SNR, there is no gain observed for TRP pre-compensation at CFO larger than 150 Hz.

Figure 3-5 DPS vs variant A/B of TRP-pre-compensation at SNR = 12 dB

Figure 3-6: DPS vs variant A/B of TRP-pre-compensation at SNR = 20 dB

Observation 8: Pre-compensation SFN schemes with QCL variant B (BC-SFN) and variant A (TRP-specific TRS) have similar results.

Observation 9: The accuracy of pre-compensation impacts the performance at higher SNR, larger MCS and higher rank.

## Signalling and configuration aspects

Based on UE capability reporting, some UE vendors may not support the enhanced SFN scheme 1. Therefore, from network perspective to deploy enhanced SFN transmission, it is necessary to support in Rel-17 TPR pre-compensation in addition to scheme 1.

Proposal 10: Support TRP pre-compensation scheme to enable HST-SFN enhancement when scheme-1 is not supported by the UE.

From UE processing, variant A requires extra overhead where UE requires to time process TRS1 and TRS2 and then combine both channels to obtain the overall delay spread and average delay of the SFN channel as shown in Figure 3-8. However, variant B, UE needs to process TRS0 which has the combined Channel across both TRPs over the air. This is explained in Figure 3-7.

Figure 3-7: TRP Pre-compensation scheme with variant B

Figure 3-8: TRP Pre-compensation scheme with variant A

A summary between both schemes comparing different aspects included performance, UE complexity, backward compatibility and TRS overhead is presented in Table 1.

Table 1: Comparison summary between QCL variant A and variant B

|  | Variant B | Variant A |
| --- | --- | --- |
| Performance | Similar |  |
| UE complexity | Lower | Higher |
| Backward Compatibility | Yes | No |
| TRS overhead | Similar + |  |

+ variant A may require extra TRS overhead to support Rel-15/16 UE with legacy SFN scheme.

Proposal 11: Support QCL variant B where one of the TCI state can be associated with {average delay, delay spread} for the SFN TRS and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

### Configuration of TRP pre-compensation scheme

As illustrated in both section 2.1 and section 3.3, both scheme1 and TRP pre-compensation share similar UE processing where the UE needs to process two TRS to both get the average delay/delay spread as well as the Doppler shift and Doppler spread. Then, both schemes can be introduced in Rel-17 using the same higher layer parameter and they can be differentiated dynamically based on the QCL type variation.

Proposal 12: Support only single higher layer parameter for the introduction of SFN scheme 1 (PDSCH) and TRP pre-compensation.

Proposal 13: Support dynamic switching between scheme-1 and TRP pre-compensation (PDSCH) based on the indicated QCL variant of the two TCI states.

### UE indication of Doppler shift per-TRP

To enable Doppler shift pre-compensation, the two (or more) TRPs needs to estimate from the UE UL signal/channel transmission the DL doppler shifts experienced by the UE from each TRP. The other approach is based on UE explicit reporting of the Doppler shift as part of the CSI reporting to the gNB. The current framework of CSI report triggering, and transmission can be utilized with an added report quantity of Doppler shift that the UE can estimate from the transparent DL TRS or SSB. Both approaches have their pros and cons.

The first option which is based on estimation of the Doppler shift from the UL signal (e.g. SRS) will incur frequency estimation errors which will affect the quality of pre-compensation. The 2nd approach will have frequency error based on UE estimation and quantization. Also, to allow gNP proper tracking of the Doppler shift, it comes at the cost of resource overhead where frequent UL signal should be transmitted with small periodicity (e.g. 5 ms). Similarly, the second approach will require frequent reporting.

From UE perspective, implicit indication of Doppler shift is the preferred option. However, to improve the accuracy of estimation, some SRS configuration enhancement is required. The proposed enhancements are discussed at Section 5.

Observation 10: Both indication options of Doppler shift are impacted by some estimation and/or quantization errors.

- Implicit indication based on UL signal/channel is preferred from UE perspective.

- SRS enhancement is required to enable more accurate Doppler estimation.

Proposal 14: Support implicit indication of Doppler shift indication through the UL signal/channel.

- Support SRS enhancement to enable more accurate Doppler estimation.

### QCL between RSs with Doppler shift pre-compensation

One of the advantages of gNB Doppler shift pre-compensation is the reduction of the overall Doppler spread and also the almost symmetric profile of the Doppler spectrum as shown in Figure 3-9. This helps with improving the orthogonality between the subcarrier and reduces the ICI. Also, it improves the quality of the channel estimation. In some implementation with non-ideal backhaul between the two TRPs, differential Doppler pre-compensation as explained in section 3.1 is not possible. Then, each TRP should do pre-compensation independently. In this scenario, each TRP should do the Doppler shift pre-compensation separately. To keep the same QCL relationship between the TRS and the DMRS reference signals, the gNB may also Doppler shift pre-compensate the RS source (i.e. TRS).

Figure 3-9: Doppler spectrum with gNB Doppler shift pre-compensation

There are two issues with TRS pre-compensation. The first one is TRS overhead as two sets of TRS are needed. One set is pre-compensated and associated to DMRS, and the others is not. The UE needs non-compensated TRS to estimate the Doppler shift. The other issue, the pre-compensation breaks the Doppler shift QCL relationship between Doppler shift pre-compensated TRS and SSB. A similar scenario could happen between the DM-RS and CSI-RS/TRS if the NW only pre-compensated the PDSCH while keeping the CSI-RS/TRS uncompensated.

Observation 11: Doppler shift pre-compensation may create TRS overhead as two sets of TRS may be needed; one set with Doppler shift pre-compensation and another one without compensation

Observation 12: QCL violation may happen with Doppler shift pre-compensation framework where target and source RSs may not have the same Doppler shift.

A new QCL type can be introduced which has loose QCL Doppler shift/spread relationship between the source and the target RSs. The source RS and the reference RS may not have the same Doppler shift/spread. Given the procedure explained by Figure 3-1, the SFNed TRS should have a new QCL type (e.g. Type-E) to indicate to the UE that the Doppler shift/spread is not exactly the same as the PDSCH. The Doppler spectrum of the SFNed TRS and PDSCH have some common part of the spectrum (around fd1), that is why the UE should use the Doppler shift based on the transparent TRS1 which has QCL Type A.

Proposal 15: Introduce new QCL type-E with loose Doppler shift relationship between the target and source RS.

### QCL relationship between UL and DL signals

Network may trigger UL signal or schedule UL channel targeting different TRPs. Therefore, the UL transmission may use different UL beam based on the associated QCL Type-D of the associated source RS (e.g. TRS or SSB). On top of this, for the framework of Doppler shift pre-compensation where the gNB estimate the Doppler shift from the UL signal or channel, the UE should know which DL TRS and the corresponding Doppler shift to use for the UL signal/channel modulation of the centre frequency. This can be achieved by defining specific QCL type for one of the DL TRS such that UE can leverage the Doppler shift obtained from that TRS for the UL signal transmission.

There will be discussion of the unified TCI framework under the beam enhancement work item. Based on the outcome of the discussion, the framework for determining multiple QCL assumptions and the associated QCL types can be further discussed for HST enhancement.

Observation 13: The UL signal/channel with multiple QCL RS sources is beneficial for selecting UL beam targeting specific TRP. Further discussion based on the outcome of the unified TCI framework in agenda item 8.1.

Observation 14: For multiple DL TRSs, the UE and gNB should have a common understanding on which TRS and the corresponding Doppler the UE should use for the modulation the centre frequency of the UL signal.

Proposal 16: A specific QCL type for the source TRS should be used as an indication to the UE for the Doppler shift for modulating the UL signal.

# Adaptive PDSCH DMRS configuration

Current specification assumes usage of fixed pilot patterns which are defined by DMRS parameters that are RRC configured per UE. Any change in DMRS configuration parameters can be done only through RRC reconfiguration. RRC reconfiguration is a non-synchronous process that also typically involves a high latency till reconfiguration process is over. During the reconfiguration period there is an uncertainty on the NW side regarding the configuration that is currently used by a UE. For this reason, there is no practical way to convey RRC reconfiguration (DMRS adaptation) “on the fly” without introducing some link interruption.

Channel estimation accuracy depends on the level of correlation of the channel in time and frequency, working SNR point of a UE and on the used for chest 2D pilot grid option. Channel parameters and SNR conditions are different for different UEs and are also varying in time. Different channel and SNR conditions require different pilot configuration option to maximize spectral efficiency of a link per UE. Hence, using a fixed pilot configuration in the NW requires to trade off in advance between average pilots overhead and UEs performance. As a result, in some cases extra resources are wasted for unnecessary pilots while in other cases UE performance is floored because of non-appropriate pilot configuration.

On contrary, Adaptive DMRS approach will allow to dynamically follow an optimal tradeoff between an improved channel estimation ability versus reduced DMRS overhead (can be translated into increased redundancy of the channel code for a better reliability or to an increased TPUT/TB size alternatively).

Using adaptive DMRS configuration per UE per slot (or some number of slots) to keep it aligned with a varying channel (Doppler and delay spread) and UE reception conditions (SNR) can provide a significant link performance improvement for both DL and UL. DMRS configuration adaptation can improve both coverage and rate over range characteristics in the network and UE mobility.

## Adaptive DMRS in HST scenario

HST scenario brings with it an extremely high mobility with extreme range of applicable Doppler shifts and Doppler spread characteristics of the channel.  This in turn results in a critical need to use a convenient DMRS pattern in order to allow a reliable and efficient link. Correspondingly, the potential link efficiency improvement that can be achieved with DMRS adaptation is especially high for HST scenario (for both DL and UL).

HST SFN type of deployment involves multi TRP transmission in DL. Multi TRP transmission per layer or DMRS port results in even a higher delay spreads and Doppler spreads for the equivalent channel that degrade channel time and frequency coherency characteristics. Doppler pre-compensation for at least one of the TRPs is suggested as one of the possible approaches to mitigate the issue of Doppler spread deterioration for HST SFN scenario.

Assuming Doppler-shift pre-compensation is applied by the network, the resulting equivalent SFN channel will be ideally translated to a single TRP channel equivalent (that may still gain from DMRS adaptation depending on the channel modeling assumptions). Pre-compensation accuracy and tracking ability as well as its applicability limitations are yet to be studied. Non pre-compensated HST SFN scenario with an enhanced SFN scheme 1 will be associated with a multi TRP channel (SFN channel type) with a reduced time and frequency coherency per DMRS port. A non pre-compensated HST SFN channel and a pre-compensated HST SFN channel cases are eventually considered as a two different scenarios for DMRS adaptation and each one of them will be associated with a different most convenient DMRS configuration option for the same use case geometry and the same channel and reception conditions characteristics. Dynamic switching between pre-compensated HST SFN transmission mode and non pre-compensated HST SFN transmission mode is assumed to be applicable for HST SFN scenario and correspondingly a dynamic switching of the appropriate DMRS configurations should be also involved for improved link performance.

In addition to an option of a dynamic usage of Doppler shift pre-compensation in HST SFN scenario, there are also additional transmission modes like multi TRP SDM mode of  Rel16 or dynamic point switching (DPS) with a single TCI/TRP transmission (depending on UE location) that may be dynamically used in HST SFN  scenario. Backward compatible enhanced SFN scheme 1 and scheme 2 that rely on a transparent HST SFN transmission mode of Rel15 for at least some portion of DMRS ports is an additional transmission mode candidate that is expected to be required to support a legacy Rel15 UEs. Different transmission modes will be associated with a different channel types that can be classified into a two channel type categories: a single TRP channel or its equivalent and  a multi TRP (SFN) channel type. As will be shown in the next subsection, these two different channel type categories will require a different DMRS configuration for link performance optimization.

DMRS adaptation in DL may be suggested to be done with a UE assistance. The most convenient DMRS configuration option can be determined by a UE targeting maximization of the link efficiency for a given estimated channel and SNR characteristics (for example: doppler spread, doppler shift, delay spread, SNR) and reported to the NW as a part of or coupled to CSF reporting and evaluation. DMRS indication can be provided for a specific (or per) HST-SFN transmission mode option. In some cases, DMRS indication may be provided by a UE per TRP.

## Evaluation results

HST SFN scheme 1 scenario with and without Doppler shift pre-compensation can be considered as a representative case for two channel type categories from a channel estimation perspective that were mentioned before (a single TRP channel equivalent and a multi TRP (SFN) channel type per DMRS port). HST SFN scheme1 with and without Doppler shift  pre-compensation is addressed for results evaluations that are provided below.. Comparative throughput results for  different DMRS options and with the extended CDL-D channel model assumption are provided below for different UE speed options for HST SFN scheme 1 with and without Doppler shift pre-compensation.. The results are provided for the mid track point (D1=350m). An ideal Doppler shift pre-compensation is used for the pre-compensated scenario.

Upper bound for performance gain that can be achieved with DMRS adaptation for the mentioned above scenarios is shown using a relative comparison of a maximal achievable TPUT with different tested DMRS configurations per addressed scenario. The addressed DMRS configurations are based on configuration type 1 with 1 front loaded DMRS symbols plus a different number of additional DMRS symbols. Please note that the throughput per each SNR point is normalized by the max throughput achievable with the best DMRS option among the tested options.

Figure 4-1 Performance of different DMRS configurations at 200 kmh (HST SFN scenario with Doppler shift pre-compensation)

Figure 4-2 Performance of different DMRS configurations at 350 kmh (HST SFN scenario with Doppler shift pre-compensation)

Figure 4-3 Performance of different DMRS configurations at 500 kmh (HST SFN scenario with Doppler shift pre-compensation)

Figure 4-4 Performance of different DMRS configurations at 3 kmh (HST SFN scenario without Doppler shift pre-compensation)

Figure 4-5 Performance of different DMRS configurations at 60 kmh (HST SFN scenario without Doppler shift pre-

compensation)

Figure 4-6 Performance of different DMRS configurations at 200 kmh (HST SFN scenario without Doppler shift pre-compensation)

Figure 4-7 Performance of different DMRS configurations at 350 kmh (HST SFN scenario without Doppler shift pre-compensation)

Figure 4-8 Performance of different DMRS configurations at 500 kmh (HST SFN scenario without Doppler shift pre-compensation)

(*) - Jitter of the curves is related to the simulated SNR and MCS resolution and is also a result of a slow MCS change as a function of SNR for some of the scenarios without Doppler shift pre-compensation. Absolute TPUT curves are provided for the last scenario to show this behavior.

(**) Ideal rank and MCS adaptation per SNR point per DMRS option (RI=1,2 is addressed), random precoding per TRP is used

It can be seen that, for different scenarios and SNR ranges, a different DMRS configuration option (color) hits 100% of the max relative TPUT (becomes to be the best option among the 4 tested options). Relative degradations of different DMRS configuration options for every SNR point can be also observed. This clearly shows a significant value in DMRS adaptation.

In addition, it can be observed that for scenarios where Doppler shift pre-compensation is  employed (single TRP CDL channel equivalent with LOS is obtained), typically only 1 DMRS symbol would be the most appropriate choice for any UE speed scenario.  For scenarios without Doppler shift pre-compensation ( multi TRP/SFN channel), a higher number of DMRS symbols would be required (typically 3) depending on UE speed assumption.

Observation 15: The best DMRS configuration pattern depends on different channel characteristics including Doppler (coupled to a UE speed) and SNR conditions which can be estimated at the UE.

Observation 16: HST SFN scenarios with and without Doppler shift pre-compensation will typically have a different optimal DMRS configuration choice.

Observation 17:: The optimal number of the required DMRS symbols in case of a non pre-comepensated SFN channel  per DMRS port will depend on a UE speed and on a UE location.

Proposal 17: Introduce DMRS adaptation for HST SFN scenario to allow a dynamic selection of the most appropriate DMRS configuration option for a given HST SFN transmission mode and the estimated channel characteristics and SNR conditions.

Proposal 18: Consider  a UE assisted DMRS adaptation for DL, in which UE provides an indication of the most convenient DMRS configuration option per estimated channel and SNR conditions and per a given HST SFN transmission mode.

Proposal 19: Introduce a mechanism for dynamic DMRS configuration signaling to enable DMRS adaptation.

# SRS for Doppler estimation in UL

Knowledge of Doppler characteristics for both DL and UL channel is very important for several components and algorithms of a cellular network to allow an efficient link maintenance (for example for Doppler pre-compensation with multi TRP, DMRS adaptation, link adaptation, beam management and others).

Current specification does not allow Doppler reporting by a UE in general, but even if it would be allowed its applicability may be limited since in some scenarios there may be no Doppler reciprocity between UL and DL. An ability to estimate Doppler directly from UL signals that can guarantee Doppler knowledge on BS side would be beneficial for the NW.

Doppler estimation is based on correlation measured between two pilot symbols. The ability to differentiate between a Doppler shift and a Doppler spread characteristics depends on the channel type in general and requires obtaining correlation measurements for different time gaps between the pilot symbols. For scenarios where a significant Doppler shift is relevant like in case of HST, it would be beneficial to estimate it based on a relatively small time gap between two pilot symbols. This will allow also to better decorrelate Doppler shift estimation from the time coherency decay due to a Doppler spread. On contrary, a reliable Doppler spread or the corresponding time correlation estimation typically requires time gap of multiple symbol times which depends on a channel profile, UE speed scenario and other link parameters. Directional transmission with LOS (typical for FR2 case) will be associated with a channel having a low Doppler spread and correspondingly a high time coherency after Doppler shift removal. In case of a richer channel profile (FR1 scenario in general or FR1 with a less directional transmission or with a weaker LOS component) Doppler spread may be high and time coherency will be limited to a time duration of several symbols. The following figures assume Rayleigh channel and depict the channel time correlation behavior (per channel tap) for different carrier frequencies and SCS.

According to the above figures, different time gaps between two pilot symbols that are used for time correlation measurements are required for reliable Doppler spread estimation for different deployment and UE speed scenarios.

Observation 18: Different spacing between 2 pilot symbols would be required for reliable Doppler shift and Doppler spread measurements.

Observation 19: Different spacing between 2 pilot symbols would be required for reliable time correlation (Doppler spread) estimation for a different channel profile, different SCS and different carrier frequencies.

UL has only two pilot types that may have more than single symbol pattern in time domain and may be considered as a basis for Doppler estimation in UL – UL DMRS and SRS.

Time domain pattern of UL DMRS is based on a combination of explicitly RRC configured DMRS parameters (like a number of DMRS locations) and additional implicitly signaled by the allocating DCI parameters (like PUSCH allocation duration). A combination of explicit and implicit parameters defines time domain pattern of DMRS per allocation. At least 2 DMRS symbols should be configured to allow DMRS based Doppler estimation. Different configuration options and allocation scenarios may result in a different time domain pattern varying from allocation to allocation that may be not appropriate for Doppler estimation based on DMRS symbols (not convenient and not persistent time gap).

Observation 20: UL DMRS based Doppler estimation is very limited and may not fit for all the scenarios

Current Rel-16 spec enables up to 4 SRS resource/port repetitions on 4 consecutive SRS symbols and allows only too small-time gap between two SRS symbols that cannot guarantee a reliable Doppler correlation estimation for different scenarios. Addressing two symbol combination of SRS resources from different SFs allows only too high SRS symbols spacing in time (not appropriate for reliable Doppler estimation in most of the scenarios).

Observation 21: SRS signal according to R16 spec is merely convenient for Doppler estimation in some selected cases only.

In order to obtain a reliable Doppler shift and Doppler spread estimation for UL in any scenario, we suggest introducing a new option for SRS waveform, which includes two none-consecutive SRS symbols carrying an identical signal with configurable time gap between the symbols locations.

Proposal 20: Introduce new SRS pattern for UL Doppler estimation purpose, comprised of a two non-consecutive SRS symbol repetition with a configurable time gap between the symbols

For proper Doppler estimation, the transmitted signal properties should be maintained during the two non-consecutive SRS symbols, i.e., same antenna port, same QCL, same resources, same UL power and transmission phase continuity should be maintained.

Observation 22: UE transmission phase continuity might be influenced by UL-DL switching and by UL power modification between non-consecutive SRS symbols.

Proposal 21: SRS allocation for Doppler measurements should not be time or intra-band-CC frequency multiplexed with any UL or DL channel for the addressed UE.

# FR2 results

Link level simulations in the FR2 band are carried out using the parameters summarized in Table 2 of the Appendix. Comparisons are provided in terms of the achieved throughputs at various SNR values between Rel. 15 baseline SFN, Rel. 17 SFN Scheme 1 and Scheme 2 assuming both fixed MCS transmission and MCS adaptation. For the fixed MCS transmission, MCS 4, 13 and 17 of the 64 QAM Table are considered without any HARQ retransmissions, and for the MCS adaptation, the MCS is adapted based on the 6 4QAM table to achieve 10% initial BLER. In the simulation, no frequency pre-compensation at the transmitter is assumed. At both TRP and UE sides, directional transmission and reception is used based on DFT codebooks.

It is observed that at the HST speeds, Rel. 17 candidate schemes outperform the single TRP scheme. In the comparison, for Scheme 2, two different DMRS patterns are assumed; 1) same CDM group and 2) different CDM group. For relatively low MCSs at low SNRs, e.g., MCS 4 and MCS 13 in Figure 6-6-1 and Figure 6-6-2, it is observed that Scheme 2 with different DMRS CDM group outperforms other schemes, due to improved channel estimation performance. However, for high SNRs, the different DMRS CDM group suffers from the loss of peak throughput, by not being able to multiplexing DMRS and data on the same symbol. Further, with MCS adaptation in Figure 6-6-4, it is observed that Scheme 1 and Scheme 2 with the same DMRS CDM group perform almost identically and outperform Scheme 2 with different CDM group. For the performance comparison at different points on the track in Figure 6-6-5, it is observed that SFN Scheme 1 and 2 show quite stable performance regardless of the position of the train, while the single TRP scheme outperforms when the train is very close to one of the TRPs.

Figure 6-6-1: Fixed MCS = 4 comparison between SFN schemes at FR2 using the extended CDL-D and CDL-E channel models at mid-track point – rank1

Figure 6-6-2: Fixed MCS = 13 comparison between SFN schemes at FR2 using the extended CDL-D and CDL-E channel models at mid-track point – rank 1

Figure 6-6-3: Fixed MCS = 17 comparison between SFN schemes at FR2 using the extended CDL-D and CDL-E channel models at mid-track point – rank 1

Figure 6-6-4: Adaptive MCS comparison between SFN schemes at FR2 using the extended CDL-D and CDL-E channel models at mid-track point – rank 1

Figure 6-6-5: Adaptive MCS comparison between SFN schemes at FR2 using the extended CDL-D and CDL-E channel model at different points - rank1

Observation 23: For FR2, Rel. 17 candidate schemes outperform the single TRP scheme.

Observation 24: For FR2 with MCS adaptation, Scheme 1 and Scheme 2 with the same DMRS CDM group perform almost identically and outperform Scheme 2 with different CDM group.

Observation 25: For FR2, SFN Scheme 1 and 2 show quite stable performance regardless of the position of the train, while the single TRP scheme outperforms when the train is very close to one of the TRPs.

# Conclusion

Observation 1: SFN scheme-1 with backward compatible SFN TRS has almost the same performance of Scheme-1 with distributed TRS (TRP-specific) while enabling backward compatible to pure SFN scheme.

Observation 2: SFN transmission scheme-2 requires double the number of DMRS ports as compared to SFN scheme-1 which results into network resource overhead, increase of UE complexity and timeline.

Observation 3: Scheme 1 outperforms scheme 2 (with different CDM group per TRP) at mid-track point for the different SNR.

Observation 4: Scheme 2 with DMRS ports mapped to different CDM groups suffer from extra DMRS overhead as compared to scheme 1 which results into lower DL throughput.

Observation 5: Scheme 2 with DMRS ports mapped to same CDM group has same DMRS overhead and achieves same performance as scheme 1 only rank 1.

Observation 6: Rel-17 MAC-CE activation of two TCI states for SFN PDCCH is required only when more than two TCI states are RRC configured in the CORESET.

Observation 7: Perfect pre-compensation is a not a realistic assumption due to many factors that affects the accuracy of Doppler shift measurements/estimation or quantization error by reporting, non-prefect frequency synchronization between the TRPs as well as the temporal variation of CFO per TRP.

Observation 8: Pre-compensation SFN schemes with QCL variant B (BC-SFN) and variant A (TRP-specific TRS) have similar results.

Observation 9: The accuracy of pre-compensation impacts the performance at higher SNR, larger MCS and higher rank.

Observation 10: Both indication options of Doppler shift are impacted by some estimation and/or quantization errors.

- Implicit indication based on UL signal/channel is preferred from UE perspective.

- SRS enhancement is required to enable more accurate Doppler estimation.

Observation 11: Doppler shift pre-compensation may create TRS overhead as two sets of TRS may be needed; one set with Doppler shift pre-compensation and another one without compensation

Observation 12: QCL violation may happen with Doppler shift pre-compensation framework where target and source RSs may not have the same Doppler shift

Observation 13: The UL signal/channel with multiple QCL RS sources is beneficial for selecting UL beam targeting specific TRP. Further discussion based on the outcome of the unified TCI framework in agenda item 8.1.

Observation 14: For multiple DL TRSs, the UE and gNB should have a common understanding on which TRS and the corresponding Doppler the UE should use for the modulation the centre frequency of the UL signal.

Observation 15: The best DMRS configuration pattern depends on different channel characteristics including Doppler (coupled to a UE speed) and SNR conditions which can be estimated at the UE.

Observation 16: HST SFN scenarios with and without Doppler shift pre-compensation will typically have a different optimal DMRS configuration choice.

Observation 17: The optimal number of the required DMRS symbols in case of a non pre-comepensated SFN channel  per DMRS port will depend on a UE speed and on a UE location.

Observation 18: Different spacing between 2 pilot symbols would be required for reliable Doppler shift and Doppler spread measurements.

Observation 19: Different spacing between 2 pilot symbols would be required for reliable time correlation (Doppler spread) estimation for a different channel profile, different SCS and different carrier frequencies.

Observation 20: UL DMRS based Doppler estimation is very limited and may not fit for all the scenarios.

Observation 21: SRS signal according to R16 spec is merely convenient for Doppler estimation in some selected cases only.

Observation 22: UE transmission phase continuity might be influenced by UL-DL switching and by UL power modification between non-consecutive SRS symbols.

Observation 23: For FR2, Rel. 17 candidate schemes outperform the single TRP scheme.

Observation 24: For FR2 with MCS adaptation, Scheme 1 and Scheme 2 with the same DMRS CDM group perform almost identically and outperform Scheme 2 with different CDM group.

Observation 25: For FR2, SFN Scheme 1 and 2 show quite stable performance regardless of the position of the train, while the single TRP scheme outperforms when the train is very close to one of the TRPs.

Proposal 1: Support the introduction of higher layer parameter to differentiate between SFN scheme-1 with BC SFN TRS and TRP-specific TRS.

Proposal 2: No support of SFN scheme 2 in Rel-17.

Proposal 3: Support the introduction of higher layer parameter for the introduction of SFN scheme 1 (PDSCH).

Proposal 4: Support semi-static (RRC based) switching of scheme 1 (PDSCH) with scheme 1a.

Proposal 5: Support semi-static (RRC based) switching of scheme 1 (PDSCH) and single TRP.

Proposal 6: Support the introduction of higher layer parameter for the introduction of SFN scheme 1 (PDSCH).

Proposal 7: If the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’ and the lowest CORESET ID in the latest slot has two active TCI states, the same TCI states are assumed for the PDSCH.

Proposal 8: The UE assumes the TCI states of the PDSCH is identical to the TCI states of the scheduling CORESET when the TCI filed is absent in the DCI and the time offset is equal to or larger than the threshold of ‘timeDurationForQCL’.

Proposal 9: For the selection of BFD resource for BFR, RAN1 to discuss both options:

- Alt-1: Consider only CORESETs with single active TCI states.

- Alt-2: Consider both CORESETs with single and dual TCI states and define some rules for BFD RS selection.

Proposal 10: Support TRP pre-compensation scheme to enable HST-SFN enhancement when scheme-1 is not supported by the UE.

Proposal 11: Support QCL variant B where one of the TCI state can be associated with {average delay, delay spread} for the SFN TRS and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

Proposal 12: Support only single higher layer parameter for the introduction of SFN scheme 1 (PDSCH) and TRP pre-compensation.

Proposal 13: Support dynamic switching between scheme-1 and TRP pre-compensation (PDSCH) based on the indicated QCL variant of the two TCI states.

Proposal 14: Support implicit indication of Doppler shift indication through the UL signal/channel.

- Support SRS enhancement to enable more accurate Doppler estimation.

Proposal 15: Introduce new QCL type-E with loose Doppler shift relationship between the target and source RS.

Proposal 16: A specific QCL type for the source TRS should be used as an indication to the UE for the Doppler shift for modulating the UL signal.

Proposal 17: Introduce DMRS adaptation for HST SFN scenario to allow a dynamic selection of the most appropriate DMRS configuration option for a given HST SFN transmission mode and the estimated channel characteristics and SNR conditions.

Proposal 18: Consider a UE assisted DMRS adaptation for DL, in which UE provides an indication of the most convenient DMRS configuration option per estimated channel and SNR conditions and per a given HST SFN transmission mode.

Proposal 19: Introduce a mechanism for dynamic DMRS configuration signaling to enable DMRS adaptation.

Proposal 20: Introduce new SRS pattern for UL Doppler estimation purpose, comprised of a two non-consecutive SRS symbol repetition with a configurable time gap between the symbols.

Proposal 21: SRS allocation for Doppler measurements should not be time or intra-band-CC frequency multiplexed with any UL or DL channel for the addressed UE.

# Reference

- RP-193133, “WID proposal FeMIMO”, Samsung.

# Appendix

Table 2: Simulation setup and evaluation assumption

| Parameter | FR1 | FR2 |
| --- | --- | --- |
| Duplexing | TDD |  |
| TRP layout (Ds, Dmin, etc) | Ds=700m, Dmin=150m For CDL based model – TRP height: 35m, UE height: 1.5m | Ds=200m, Dmin=30m For CDL based model – TRP height: 20m, UE height: 1.5m |
| TRP antenna configuration including number of antennas, pattern, ports, orientation, etc | CDL based extension: 8 ports: [Mg, Ng, M, N, P]=[1, 1, 8, 4, 2], antenna model in Table 5 in [2], 8-to-1 mapping is used to virtualize the 8 antenna elements in a column with fixed weight to form an antenna port 4-tap channel model: 2 ports: omni-directional, MIMO matrix according to TS 38.101-4 (Annex B.1) | CDL based extension: 2 ports: [Mg, Ng, M, N, P]=[1, 1, 8, 4, 2], Directional antenna model in Table 5 in [2] |
| UE antenna configuration including number of antennas, pattern, ports, orientation, etc | 4 ports: [Mg, Ng, M, N, P]=[1, 1, 1, 2, 2], one-to-one mapping between antenna elements and TXRUs omni-directional antenna | 2 ports: [Mg, Ng, M, N, P]=[1, 1, 2, 4, 2], Directional antenna model in Table 6 in [2] |
| DMRS type | Mandatory: DM-RS type 1 |  |
| Number of DMRS symb. | 1+1+1 |  |
| TDD pattern | DDDDDDDSUU, S: 6D 4G 4U |  |
| MCS | MCS 4/MCS 13/MCS 17 based on 64QAM table, MCS adaptation |  |
| Number of scheduled RBs | 50 | 32 |
| Propagation condition | 4-tap channel model (TS 36.101 (Annex B.3A) / TR 36.878) CDL extension (CDL D/E, DS = 100ns) | CDL extension (CDL D/E, DS = 20ns/30ns) |
| TRS configuration, TRS periodicity | 10ms, 2-slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank 1, Rank 2 |  |
| BW | 20 MHz | 50 MHz |
| Carrier frequency or maximum Doppler shift | 3.5GHz, 500kmph | 30GHz, 200kmph, 350kmph |
| Performance metric | Throughput |  |
| Other assumptions or simulation parameters, e.g., correlation | 1) SCS: 30kHz 2) Random precoder cycling per PRG =2RB | 1) SCS: 120kHz 2) Random precoder cycling per PRG =2RB |