3GPP TSG RAN WG1 #104bis-e		R1-2103299

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.2.6

Source:	Sony

Title:	Channel access mechanism for 60 GHz unlicensed spectrum

Document for:	Discussion / Decision

Introduction

In the WID [1], the following part is included in the objective.

- Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

    - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

    - Study, and if needed specify, energy detection threshold enhancement

In this contribution, a channel access mechanism for unlicensed spectrum for frequencies around 60 GHz is discussed.

Discussion

Channel bandwidth

In the SI phase [2], RAN1 made the following interpretation of EN 302 567 [3].

The OCB requirement of draft version v2.1.20 of EN 302 567 [4] implies that - device supports one or multiple declared nominal channel bandwidths, - for each declared nominal channel bandwidth, RAN1 design should support at least one physical layer signal/channel transmission that occupies at least 70% of the nominal channel bandwidth.

Nominal channel bandwidth is used for co-existing with devices that use other RATs such as 802.11 ad/ay that can operate and perform LBT using 60 GHz unlicensed spectrum. In 802.11 ad/ay systems, it has been specified that a unit of LBT bandwidth is 2.16 GHz. If NR devices transmit signals using a channel bandwidth of less than 2.16 GHz, an 802.11 ad/ay device may not detect the existence of the NR device accurately thereby increasing the collision probability between NR and 802.11 ad/ay. As in NR-U using 5 GHz unlicensed spectrum, considering co-existence with NR and 802.11 ad/ay, channelization should be aligned in a co-existence environment. Therefore, at least support of about 2 GHz bandwidth in the 60GHz band should be considered for NR.

Proposal 1: NR devices support 2.16 GHz bandwidth in 60GHz spectrum as one of the nominal channel bandwidths.

LBT mode and No-LBT mode

RAN1 has reached the following observations for no-LBT mode during the SI phase [2].

It is recommended to support both channel access with LBT mechanism(s) and a channel access mechanism without LBT for gNB and UE to initiate a channel occupancy. Further investigation of the following issues may be needed: - LBT mechanisms such as omni-directional LBT, directional LBT, and receiver assisted LBT type of schemes when channel access with LBT is used, - whether operation restrictions for channel access without LBT are needed, e.g. compliance with regulations, and/or in presence of ATPC, DFS, long term sensing, or other interference mitigation mechanisms, and - the mechanism and condition(s) to switch between channel access with LBT and channel access without LBT (if local regulation allows). For operation where LBT is not required, the following can be further discussed when specifications are developed: - whether to introduce additional conditions/mechanisms for no-LBT to be used, or whether to leave it for gNB implementation, - when no-LBT mode is used, whether to introduce additional restrictions, such as DFS needs to be applied, ATPC needs to be applied, long term sensing needs to be applied, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc, or leave the restriction for gNB implementation, - when no-LBT mode is used, whether to introduce mechanism for the system to fallback to LBT mode, or whether to leave it for gNB implementation.

This section discusses the FFS part for the condition where no-LBT mode is applied.

From the regulatory requirement perspective, according to ECC recommendations for 57-71 GHz [5], 3 types of modes are categorized; ‘C1’, ‘C2’, and ‘C3’. ‘C1’ is associated with ETSI EN 302 567 [3] which has described that LBT is mandatory to facilitate spectrum sharing. ‘C2’ and ‘C3’ correspond to EN 302 567 and EN 303 722, respectively. These specifications have not been fixed yet. For countries other than those in the EU, since LBT is not mandated, no-LBT mode can be adopted.

Observation 1: In EU, no-LBT mode cannot be operated at least under the ‘C1’ mode for indoor and outdoor deployment.

The purpose of LBT is to avoid collision between transmissions from 2 or more devices in the congested environment. In the uncongested environment, since most CCA would tend to pass, it would be almost equivalent to when no-LBT mode is adopted.

Observation 2: No-LBT mode works in the uncongested environment.

In Rel-16 NR-U, average RSSI and channel occupancy have been introduced. Average RSSI represents the average level of interference and channel occupancy represents the ratio of interference occurrence in the time domain. If average RSSI and channel occupancy are low enough, the gNB could understand that the environment is not congested. Therefore, based on measurement results of average RSSI and channel occupancy, no-LBT mode could be configured.

Observation 3: Congestion could be measured by average RSSI and channel occupancy which have already been introduced in NR-U.

Proposal 2: No-LBT mode is configured by the network based on measurement results of RSSI and channel occupancy.

Short control signalling

Short control signalling, which LBT is not required for transmission if condition is met, is allowed by ETSI in 60 GHz unlicensed band. For the condition to adopt short control signalling, total duration of short control signalling transmission is less than 10% within 100 msec.

Although duration of short control signalling transmission is short, short control signalling is useful for essential transmission such as SSB transmission (especially for 480 kHz and 960 kHz SCS SSB) and PRACH transmission since transmission without LBT is allowed in European countries.

Proposal 3: Contention exempt short control signalling should be adopted at least for SSB and PRACH transmission.

Contention window size adjustment

The following conclusion for the LBT procedure was made during SI phase [2].

Use the CCA check procedure in EN 302 567 as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work: - whether CAPC and contention window adjustment mechanisms are introduced, - whether contention window range needs to be adjusted.

In Rel-16 NR-U, dynamic Contention Window Size adjustment based on HARQ-ACK feedback is introduced, which is aligned with the Wi-Fi LBT scheme. The dynamic Contention Window Size adjustment is beneficial to avoid transmission collision in the case that congestion cannot be estimated. However, an NR device can perform measurements and report on the channel environment. Optimized CWS could improve system performance because collision probability could be reduced and also channel access latency could be improved. CWS should be adjustable and configurable, based on measurement results.

Proposal 4: Support fixed Contention Window.

- gNB’s contention windows size is left to network implementation.

- UE’s contention window size is configured by network.

Cat 2 LBT

In RAN1#104-e meeting [6], the following agreements were made.

Agreement: For Cat 2 LBT, down-select from the following alternatives - Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation - Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation Agreement: If Cat 2 LBT is introduced, the following use cases can be further studied: - Resume transmission after a gap Y: Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y) - COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT - Multi-Beam LBT: Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z) - Rx-Assistance: Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling Other use cases not precluded. FFS if Cat 2 LBT is mandated for each use case or not.

Cat 2 LBT is not defined in EN 302 567. However, Cat 2 LBT at initiating device is useful to detect transmission from other devices when the other device starts to transmission during the transmission gap. Also, Cat 2 LBT at responding device is useful for detection of hidden node.

Instead of introducing Cat 2 LBT, eCCA could be considered. However, channel access latency would be increased if eCCA is used instead of Cat 2 LBT, which is not desirable.

Proposal 5: Introduce Cat 2 LBT for 60 GHz unlicensed band operation

Directional LBT

The following observation for directional LBT was made during the SI phase [2].

It can be further discussed when specifications are developed if 3GPP specifications should define the relationship between the LBT beam and the transmission beam or leave it as implementation. If such relationship is defined, it can also be further discussed when specifications are developed if ED threshold should be adjusted by the choice of LBT beam and transmission beam.

In 60 GHz, transmission with fine beamforming using massive antenna elements is necessary to combat high propagation loss. In this case, if omni-directional LBT is performed also in 60 GHz spectrum, direction mismatching between LBT and a transmission may happen. One of the issues caused by direction mismatching is increasing the “exposed node problem” shown in figure 1. In 60 GHz unlicensed spectrum, for example, a serving gNB performs LBT in all directions, including directions in which signals are not planned to be transmitted. If the serving gNB detects energy from the direction of a neighboring gNB, even though it does not interfere in that direction, the serving gNB must stop signal transmission. As a result, by inadvertently restricting transmission opportunity, system performance would be degraded.

To reduce the “exposed node problem”, directional LBT could be considered. For directional LBT, since the transmitting device does not perform energy detection in an unnecessary direction, the “exposed node problem” is expected to be reduced.

Figure 1. an example of exposed node problem in 60 GHz unlicensed spectrum

In the SI phase [2], directional sensing has been studied by simulation evaluation. Although the gain of directional sensing with no-LBT was marginal in an uncongested scenario, some companies showed gain of directional sensing over omni-directional sensing in a medium loaded scenario. Therefore, directional LBT could be considered to be supported for operation in some scenario.

Proposal 6: Directional LBT should be supported in 60 GHz unlicensed operation.

If directional LBT is supported, the relationship between the sensing beam and the transmission beam should be considered. [2].

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of "cover" - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

While Single LBT sensing has an advantage in terms of channel access latency since beam switching is not needed during LBT, per-beam LBT sensing is beneficial for improving channel access probability because applying narrower beam for LBT could reduce expose node issue. Since both LBT sensing would be useful, both should be supported. Whether wide or narrow beam is applied for LBT would be left to implementation.

Proposal 7: For a COT with MU-MIMO (SDM) transmission, both Alt 1 (Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold) and Alt 2 (Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT) should be supported.

Proposal 8: Within a COT with TDM of beams with beam switching, both Alt 1 (single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold) and Alt 2 (independent per-beam LBT sensing at the start of COT is performed for beams used in the COT) should be supported.

COT indication is beneficial to UE for support of COT sharing and relaxing UE receiver complexity. If per-beam LBT sensing is introduced, per-beam COT would be defined. In this case, per beam COT indication may need to be considered. How to indicate per-beam COT should be discussed further after decision whether per-beam LBT sensing is supported or not.

Observation 4: If per-beam LBT sensing is introduced, per beam COT indication may need to be needed.

Receiver-assisted LBT

The following observation regarding receiver-assisted LBT was made during the SI phase [2].

The following receiver assisted channel access and interference management schemes have been considered and can be further investigated when specifications are developed. - Class A) Receiver provides assistance information (signalling) to transmitter only. The following aspects of Class A can be further discussed when specifications are developed. - Applicability in the following potential channel access modes: - LBT is performed prior to transmission, - No LBT is performed prior to transmission. - Details of assistance information (e.g., type, timing, content, how the assistance information is obtained etc.). - Whether the assistance information can be obtained by LBT performed at the receiver prior to transmission. - Whether the assistance information can be obtained by existing layer 1 and layer 3 measurements with enhancements if needed. - If any specification changes are needed to support Class A.

In 60 GHz, transmission with fine beamforming using massive antenna elements is necessary to combat high propagation loss. It would also cause a mismatching between the sensing result of Tx and Rx, which is well-known as the “hidden node problem”. For example in figure 3, the serving gNB passes LBT then transmits signals to a UE 1. However, when the UE 1 is close to a UE 2, UE 1 may suffer from strong interference from transmissions from the neighboring gNB to UE 2 while the serving gNB may not be able to detect such interference due to high propagation loss. In this case, due to the strong interference, UE 1 would fail to decode the signal. This also causes system performance degradation.

In 60 GHz unlicensed spectrum, receiver assisted LBT, in which the transmitting device is allowed to transmit its signal only when LBTs in both the transmitting device and the receiving device pass (like the RTC/CTS mechanism introduced in Wi-Fi), could be considered. By using receiver assisted LBT, since the transmitting device doesn’t transmit a signal when strong interference happens in the receiving device, the “hidden node problem” can be alleviated.

Figure 3. an example of hidden node problem in 60 GHz unlicensed spectrum

In the SI phase [2], receiver assisted sensing has also been studied by simulation evaluation. Several sources show the gain for receiver assisted LBT compared to omni-directional LBT in a highly loaded environment. Therefore, receiver assisted LBT could be considered to support operation in some scenarios.

Proposal 9: Receiver assisted LBT should be supported in 60 GHz unlicensed operation.

Conclusions

In this contribution, based on the above discussion, we have the following observations and proposals:

Proposal 1: NR devices support 2.16 GHz bandwidth in 60GHz spectrum as one of the nominal channel bandwidths.

Observation 1: In EU, no-LBT mode cannot be operated at least under the ‘C1’ mode for indoor and outdoor deployment.

Observation 2: No-LBT mode works in the uncongested environment.

Observation 3: Congestion could be measured by average RSSI and channel occupancy which have already been introduced in NR-U.

Proposal 2: No-LBT mode is configured by the network based on measurement results of RSSI and channel occupancy.

Proposal 3: Contention exempt short control signalling should be adopted at least for SSB and PRACH transmission.

Proposal 4: Support fixed Contention Window.

- gNB’s contention windows size is left to network implementation.

- UE’s contention window size is configured by network.

Proposal 5: Introduce Cat 2 LBT for 60 GHz unlicensed band operation

Proposal 6: Directional LBT should be supported in 60 GHz unlicensed operation.

Proposal 7: For a COT with MU-MIMO (SDM) transmission, both Alt 1 (Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold) and Alt 2 (Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT) should be supported.

Proposal 8: Within a COT with TDM of beams with beam switching, both Alt 1 (single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold) and Alt 2 (independent per-beam LBT sensing at the start of COT is performed for beams used in the COT) should be supported.

Observation 4: If per-beam LBT sensing is introduced, per beam COT indication may need to be needed.

Proposal 9: Receiver assisted LBT should be supported in 60 GHz unlicensed operation.

References

- RP-202925, “Revised WID: Extending current NR operation to 71 GHz,” TSG RAN Meeting #90-e, December 2020.

- TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz”

- ETSI EN 302 567, V2.1.1, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU,” July 2017.

- ETSI EN 301 893, V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU,” May 2017.

- CEPT ECC, ERC Recommendation 70-03 for Short Range Devices, June 2019.

- RAN1#104-e chairman’s note