3GPP TSG RAN WG1 Meeting #104bis-e				           R1-2103376

e-Meeting, April 12th – 20th, 2021

______________________________________________________________________ Agenda item: 7.2.4

Source: LG Electronics

Title: Discussion on essential corrections in resource allocation procedure

Document for: Discussion and decision

______________________________________________________________________

- Introduction

This contribution addresses the essential corrections on NR sidelink resource allocation for Mode 1 and Mode 2.

- Discussion

  - Resource allocation for Mode 1

    - Search space for PDCCH monitoring

We have made the following agreement in RAN1#99 meeting.

Agreements:

- Use a separate PDCCH monitoring configuration (as configured in Rel-15) for DCI scheduling SL

  - When in the same slot, there is both PDCCH monitoring for Uu and PDCCH monitoring for SL for the same CC, the search space(s) for SL is configured to be the same or a subset of those for Uu for the same CC or vice versa

    - A UE does not expect to receive in the same PDCCH monitoring occasion and the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying a SL DG.

According to IE SearchSpace as described in TS 38.331, PDCCH monitoring for DCI format 3_0 is configured to be the same of those for Uu for the same carrier. To be specific, since dci-Formats is not optional, a UE will be configured with both dci-Formats and dci-FormatsSL-r16. In other words, the search space(s) for SL will be the same or a subset of those for Uu for the same CC. Meanwhile, if the scheduled cell is on ITS band, the UE does not need to monitor Uu DCI. Moreover, for the ITS-dedicate carrier, since there is no DL BWP or UL BWP, the payload size of the Uu DCI is unknown to the UE. In this case, it can be considered that if the scheduled cell of the search space configuration for cross-carrier scheduling is ITS-dedicated carrier, the UE ignores dci-Formats and uses only dci-FormatsSL-r16.

ue-Specific SEQUENCE { dci-Formats ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1}, ..., [[ dci-Formats-MT-r16 ENUMERATED {formats2-5} OPTIONAL, -- Need R dci-FormatsSL-r16 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1, formats3-0-And-3-1} OPTIONAL, -- Need R dci-FormatsExt-r16 ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2} OPTIONAL -- Need R ]] }

At least, within a slot, if the search space(s) for SL is not the same or a subset of those for Uu, the set of PDCCH candidates will not be shared between DL DCI and Uu DCI, so the number of PDCCH candidates to be monitored could be large, and then the UE may need to drop some portion of PDCCH candidates to fulfil the BD limit and CCE limit. At this moment, no spec change is needed in this stage.

Proposal 1: When Mode 1 SL TX is cross-carrier scheduled on ITS-dedicated carrier, the UE ignores dci-Formats and monitors only dci-FormatsSL-r16 in the relevant search space configuration on the scheduling licensed carrier.

    - SL HARQ-ACK reporting

      - Semi-static SL HARQ-ACK codebook

The number of PSSCH slots associated with the same PSFCH slot could be smaller than the PSFCH resource period. To be specific, when the total number of SL slots belonging a resource pool within 10240msec is not a multiple of PSFCH resource period, the number of PSSCH slots associated with the first PSFCH slot within a resource pool period is smaller than the PSFCH resource period. In this case, it is necessary to investigate how to modify the semi-static codebook for SL HARQ-ACK reporting as well. According to LTE TDD, the number of DL SF associated with the UL SF can be different, and the HARQ-ACK codebook size can be also changed depending on this HARQ-ACK association set size. Even for the NR Uu link, Type-I HARQ-ACK codebook size could be varying slot-by-slot depending on the time gap between DL BWP switching or PCell UL BWP switching and PUCCH TX timing. In addition, considering the definition of j is the index of occasions for candidate PSSCH transmission with corresponding PSFCH reception occasions,  the range of j also needs to consider the case when the number of PSSCH slots is smaller than the PSFCH resource period. In those points of views, similar approach could be employed. In other words, when the total codebook size is determined, each PSFCH occasions could have different number of SL HARQ-ACK bits.

Proposal 2: Adopt following TP for TS 38.213:

16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged part is omitted> For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period among the PSSCH slots associated with the PSFCH slot Set NPSFCH to the number of PSSCH slots associated with the PSFCH slot while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while

- Conclusion

In this contribution, the essential corrections on Mode 1 were discussed. Our proposals can be summarized as follows:

Proposal 1: When Mode 1 SL TX is cross-carrier scheduled on ITS-dedicated carrier, the UE ignores dci-Formats and monitors only dci-FormatsSL-r16 in the relevant search space configuration on the scheduling licensed carrier.

Proposal 2: Adopt following TP for TS 38.213:

16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged part is omitted> For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period among the PSSCH slots associated with the PSFCH slot Set NPSFCH to the number of PSSCH slots associated with the PSFCH slot while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while

- Reference