3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103384

E-meeting, April 12 – 20, 2021

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Discussion on PUSCH transmission power control corresponding to RAR UL grant

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the determination of  for PUSCH transmission corresponding to RAR UL grant.

# Discussion

## Determination of  for PUSCH transmission corresponding to RAR UL grant

The PUSCH transmission scheduled by RAR UL grant in random access procedure includes following two cases:

Case 1: CBRA. As specified in section 8.3 of TS 38.213 “Msg3 PUSCH retransmissions, if any, of the transport block, are scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message”, when Msg3 transmission is triggered by a CBRA preamble, the PUSCH retransmission is scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI.

Case 2: CFRA. As specified in section 8.3 of TS 38.213 “If a TC-RNTI is provided by higher layers, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by TC-RNTI. Otherwise, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by C-RNTI.”, when PUSCH transmission is scheduled in CFRA case, the PUSCH retransmission can be scheduled by DCI format 0_0 or DCI format 0_1 with CRC scrambled by C-RNTI.

According to following highlighted part in paragraph 1 in section 7.1.1 of TS 38.213,  for initial PUSCH transmission and PUSCH retransmission corresponding to a RAR UL grant are same (for both case 1 and case 2). However, according to following highlighted part in paragraph 2 in section 7.1.1 of TS 38.213,  for PUSCH retransmission scheduled by DCI format 0_1 corresponding to a RAR UL grant in case 2 can follow the SRI field indication in DCI format 0_1.

Paragraph 1:

- is a parameter composed of the sum of a component and a component where . - If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in Clause 8.3, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell

Paragraph 2:

- For , a value, applicable for all , is provided by p0-NominalWithGrant, or if p0-NominalWithGrant is not provided, for each carrier of serving cell and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the value of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

The determination of  for case 1 and case 2 are summarized in Table 1:

Table 1. The determination of  in current specification

| Cases |  |  |
| --- | --- | --- |
| Case 1 | Msg3 transmission |  |
|  | Msg3 retransmission scheduled by DCI format 0_0 |  |
| Case 2 | PUSCH transmission corresponding to RAR UL grant |  |
|  | PUSCH retransmission scheduled by DCI format 0_0 |  |
|  | PUSCH retransmission scheduled by DCI format 0_1 | UE behavior 1: same for PUSCH transmission and retransmission corresponding to a RAR UL grant - - |
|  |  | UE behavior 2: UE is provided by SRI-PUSCH-PowerControl with more than one values of p0-PUSCH-AlphaSetId, and DCI format 0_1 includes a SRI field. - is determined by the value indicated by the SRI field - |

As shown in Table 1, PUSCH retransmission scheduled by DCI format 0_1 corresponding to RAR UL grant has two specified UE behaviors to determine the value of . The first UE behavior is obtained according to paragraph 1, and same  is applied for initial PUSCH transmission and PUSCH retransmission corresponding to RAR UL grant. The second UE behavior is obtained according to paragraph 2, and  for PUSCH retransmission scheduled by DCI format 0_1 is indicated by SRI field in DCI format 0_1. Therefore, different UE behaviors lead to different values of  according to the current specification. In summary, the UE behavior on determining  is unclear when PUSCH retransmission corresponding to RAR UL grant is scheduled by DCI format 0_1.

Observation: There are two specified UE behaviors to determine the value of  for PUSCH retransmission that corresponds to RAR UL grant and is scheduled by DCI format 0_1.

## Solution

For the PUSCH retransmission scheduled by DCI format 0_1 corresponding to a RAR UL grant, UE can achieve dedicate RRC configuration and it is not necessary to restrict same value of  as initial PUSCH transmission corresponding to RAR UL grant. If SRI field is included in DCI format 0_1, it can be flexible to indicate  in DCI format 0_1. If SRI field is not included in DCI format 0_1, default value is applied. The UE behavior for PUSCH retransmission scheduled by DCI format 0_1 in case of CFRA is the same as any other PUSCH transmission scheduled by DCI 0_1. The above understanding can be applied to both Rel-15 and Rel-16 specification.

Therefore, we have following proposal:

Proposal: Confirm that if a PUSCH is scheduled by DCI 0_1 as a retransmission for an initial PUSCH transmission scheduled by RAR UL grant in case of CFRA,  determination is the same as any other PUSCH transmission scheduled by DCI 0_1. Adopt the draft CR [2] for section 7.1.1 of TS 38.213.

# Conclusions

Based on the discussion above, following observation and proposals are obtained:

Observation: There are two specified UE behaviors to determine the value of  for PUSCH retransmission that corresponds to RAR UL grant and is scheduled by DCI format 0_1.

Proposal: Confirm that if a PUSCH is scheduled by DCI 0_1 as a retransmission for an initial PUSCH transmission scheduled by RAR UL grant in case of CFRA,  determination is the same as any other PUSCH transmission scheduled by DCI 0_1. Adopt the draft CR [2] for section 7.1.1 of TS 38.213.

# References

R1-2102025, “Summary of [104-e-NR-7.1CRs-16]”, Vivo, RAN1#104-e, January 25 – February 5, 2021.

R1-2103742, “Correction on PUSCH transmission power control corresponding to RAR UL grant in Rel-15”, Huawei, HiSilicon, RAN1#104bis-e, April 12 – 20, 2021