3GPP TSG RAN WG1 #104b-e	                         		       R1-2103426

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.7.1.3

Source:	InterDigital, Inc.

Title:	Paging indication based on sub-time units

Document for:	Discussion

# Introduction

In RAN1 #103-e, evaluation of several paging enhancement techniques for idle/inactive UEs was performed and it was agreed to support paging early indication:

Agreements: For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective

The paging indication can be based on a sequence (e.g., SSS) and/or a PDCCH based channel. Both PDCCH and sequence detection may not be proper for very low power UEs due to the power spent on receiver procedures such as FFT, channel decoding, etc. If the paging indication needs to be repeated, for example to improve coverage, the power efficiency would degrade further. So, as an alternative, a paging indication that can be received using energy detection may be desirable due to its lower power requirements. Furthermore, the indication can use sub-time units created within an OFDM or a DFT-s-OFDM symbol to convey the information, reducing the resource overhead as it relies on sub-time units rather than the whole OFDM/DDT-s-OFDM symbol. In this contribution, such a scheme is proposed.

# Sub-time unit based paging indication

In the proposed scheme, the paging indication (PI) is carried in the set of sub-time units (STU) within one or more OFDM/DFT-s-OFDM symbols. An example is shown in Figure 2-1 in which each STU may be amplitude modulated to represent a 1 or a 0 to target a specific UE group.

Figure 2-1 An example of sub-time units in an OFDM symbol or DFT-s-OFDM symbol

To generate such an ON/OFF pattern (also knoan as ON/OFF keying, OOK) in time domain for OFDM can be achieved by masking the OFDM symbol, which would result in increased out-of-band emission. However, using DFT-s-OFDM, the masking can be done at the transmitter before the DFT operation, and orthogonality can be preserved. An illustration of such a scheme is shown in Figure 2.2. The input to the DFT block of the DFT-s-OFDM modulator is first divided into K parts and a sequence s∈CMs×1 is placed into the corresponding portion of the input depending on the bits, i.e., bi for i=1,2,…,K. For example, if bi=1, the corresponding input of DFT-s-OFDM is set to s, otherwise it is 0∈CMs×1. Since the time domain signal at the output of the IDFT is an oversampled version of the input sequence of the DFT block with a certain pulse shape (i.e., Dirichlet sinc function without frequency domain spectral shaping (FDSS)), the sub-time unit ON and OFF symbols can be generated at the output of the DFT-s-OFDM. This simple structure maintains the orthogonality in an OFDM symbol, enabling perfect compatibility with NR.

Figure 2.2 Orthogonal OOK generation

A sample DFT-s-OFDM signal generated using this approach, after the addition of the the cyclic prefix, is shown in Figure 2-3. In this example, the sub-time units represent the indication (1,1,0,0,0,1,0,0,0,1,1,0,1,1,1,0,1,1,1,0,0,1).

Figure 2-3 Time-domain illustration of a DFT-s-OFDM symbol consisting of amplitude modulated sub-time units

Observation 1: Paging indication using amplitude modulated sub-time units can be generated with DFT-s-OFDM while preserving orthogonality of the signal. Such a signal can be received very efficiently with energy detection and uses fewer resources.

OOK waveform can be used as a wake-up signal in connected mode as well due to the very low power required to monitor it. Therefore, we propose the following:

Proposal 1: OOK waveform based on DFT-s-OFDM should be studied for wake-up signal design for idle and connected mode UEs.

# Summary

In this contribution, we have presented a scheme for a paging indication signal using amplitude modulated sub-time units within a DFT-s-OFDM symbol. The following observation and proposal have been made:

Observation 1: Paging indication using amplitude modulated sub-time units can be generated with DFT-s-OFDM while preserving orthogonality of the signal. Such a signal can be received very efficiently with energy detection and uses fewer resources.

Proposal 1: OOK waveform based on DFT-s-OFDM should be studied for wake-up signal design for idle and connected mode UEs.

# References

- [1] Chairman’s Notes, 3GPP WG1 #103-e