3GPP TSG RAN WG1 #104bis-e		R1-2103499

e-Meeting, April 12th – 20th, 2021

Title: 	Remaining issues on mode1

Source: 	ZTE, Sanechips

Agenda item:	7.2.4

Document for:	Discussion and decision

Introduction

In this contribution, some remaining issues of mode 1 are discussed.

Corrections for Type-1 codebook

In this section two points for corrections for type-1 codebook are given respectively, and lastly a final TP is provided.

- Issue 1: Actual PSSCH transmission occasions corresponding to a PSFCH slot

In sidelink physical procedure , it has been agreed that the number of logical slots in a resource pool within 10240ms period is not limited to be an integer multiple of the PSFCH resource period, and for the location of PSFCH slots where the PSFCH resource period is N, they are on the logical slots with the index #0, #N, #2N, …, within 10240 ms period [1]. When the number of logical slots in a resource pool within 10240ms is not an integer multiple of the PSFCH resource period N, the PSSCH transmission occasions corresponding to the first PSFCH slot with 10240ms is no longer N, actually is smaller than N. In this case if to generate type-1 codebook according to PSFCH resource period N, there will be some confusion because we cannot find N PSSCH transmission occasions corresponding to the first PSFCH slot with 10240ms. So it is proposed that type-1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

Type 1 codebook should be generated according to the actual PSSCH transmission occasions corresponding to the PSFCH slot, rather than PSFCH resource period.

- Issue 2: Type-1 HARQ-ACK codebook generation for multiple resource pools

In current TR38.213[2], multiple resource pools are allowed to be configured, but the set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions only consider one resource pool but the procedure in the specification does not distinguish which resource pool is used.

- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool …… The cardinality of the set MA defines a total number M of PSFCH reception occasions corresponding to the HARQ-ACK information bits. ……

From the yellow marked text above, it seems sub-bullet c) defines multiple resource pools with a set of sidelink resource pool bitmaps, but just only one but unknown which one resource pool is used to generate the set MA in the following pseudo-code and the HARQ-ACK information bits, this is not correct when multiple resource pools with different PSFCH period are configured.

In current 38.213 specification when multiple resource pools are configured, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits

All the configured resource pools should be considered for Type-1 codebook generation

- Proposed TP:

From all the analysis in this section above, a text proposal can be provided for type-1 codebook correction.

For corrections for Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0-index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

Conclusion

This paper concludes with the following proposals and observations:

Observation 1:	In current 38.213 specification when multiple resource pools are configured, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits

Proposal 1:	Type 1 codebook should be generated according to the actual PSSCH transmission occasions corresponding to the PSFCH slot, rather than PSFCH resource period.

Proposal 2:	All the configured resource pools should be considered for Type-1 codebook generation

Proposal 3:	For corrections for Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0-index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

References

- Chairman note RAN1#102-e

- TS 38.213 g50