3GPP TSG RAN WG1 #104bis-e	                    	   R1-2103501
e-Meeting, April 12th – 20th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | draft | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on sidelink resource allocation mode 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-04-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Editorial corrections based on agreements made in RAN1 and RAN2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Modify quote to the correct clause. 2, Add the missing DCI field description. 3, Align the IE SL-Thres-RSRP-List defined in RRC(TS 38.331). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification for resource allocation mode 1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2 and 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

    - 8.1.2	Resource allocation

In sidelink resource allocation mode 1:

-	for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.32A of [6, TS 38.213].

<Unchanged parts omitted>

<Unchanged parts omitted>

    - 8.1.2.1	Resource allocation in time domain

...

In sidelink resource allocation mode 1:

-	For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0.

-	For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0.

-	For sidelink dynamic grant and sidelink configured grant type 2:

-	The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter timeGapFirstSidelinkTransmission and the table value at index m + 1 will be referred to as slot offset KSL.

-	The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than  TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslott is the SL slot duration.

-	The "Configuration index" field , if provided, in the DCI format 3_0 indicates an index of a configured SL grant provided by high layer.

-	For sidelink configured grant type 1:

-	The slot of the first sidelink transmissions follows the higher layer configuration according to [10, TS 38.321].

<Unchanged parts omitted>

<Unchanged parts omitted>

    - 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-ThresPSSCH-RSRP-List: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList

-	sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec

-	sl-TxPercentageList: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnable: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable

The resource reservation interval, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

Notation:

t'0SL, t'1SL, t'2SL,… denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL) where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-ThresPSSCH-RSRP-List, where i=pi+pj-1*8.

<Unchanged parts omitted>