3GPP TSG RAN WG1 Meeting #104b                                                               	     R1-2103531

e-Meeting, April 12th – April 23rd, 2021

Agenda Item:	8.9.1

Source:	Lenovo, Motorola Mobility

Title:	Support 16QAM for NBIoT

Document for:	Discussion and decision

Introduction

In the RAN#88 plenary meeting, an updated work item for Rel.17 eMTC/NBIoT was approved. The objective is to specify the following enhancements to NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

The following agreements on support 16QAM for NBIoT were achieved:

Working Assumption

The previous working assumption on the following TBS indices for downlink is updated with following modifications:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS for I_SF > 7

Agreement

I_SF>7 is not supported in Rel-17.

Agreement

Confirm the following working assumption:

- The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Agreement

DL 16-QAM is applicable for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI.

- At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS.

- FFS: Applicability of 16-QAM for PUR.

Agreement

Repetition is not used for 16-QAM in uplink.

Agreement

UL 16-QAM is applicable for NPUSCH scheduled from a DCI with CRC scrambled by C-RNTI.

- At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS.

- FFS: Applicability of 16-QAM for PUR or EDT.

Agreement

The soft buffer size for Cat. NB2 UEs supporting 16QAM for downlink is 12800 bits.

Agreement

The following working assumption is confirmed with following modifications:

- For inband deployments, the downlink TBS entries between 11 (TBS of 2024 for I_SF=7) and [17] are used for 16QAM.

Agreement

Repetition of 2 is NOT supported for 16-QAM in downlink.

Agreement

On the breaking point between QPSK and 16QAM for NPUSCH, the UL TBS entries only between 14 and 21 are used for 16QAM if 16QAM is configured.

Agreement

16-QAM can be used for 3 and 6 subcarriers NPUSCH format 1

Agreement

The NPDSCH EPRE in symbols with NRS can be different and can be the same with the NPDSCH EPRE in symbols without CRS and NRS.

- FFS on signaling details

- FFS for the handling on whether the PCI is different or the same

In this contribution, detailed considerations of 16QAM support for NPDSCH and NPUSCH, especially new TBS table and related DL power allocation design are presented.

Discussion

16QAM for NPDSCH

In Rel.13 NBIoT, up to QPSK is supported for DL PDSCH. In order to increase the data rate, higher modulation of 16QAM is proposed in Rel.17 NBIoT scope. If 16QAM for NPDSCH is configured, the maximum TBS is increased to 4968 for standalone and guardband and 3624 for inband case as agreed in last RAN1 meeting.

For the DCI format N1 optimization based on the existing agreement, the joint coding of MCS and repetition number field can be considered.

- For standalone and guardband case, if MCS field IMCS<14, QPSK is assumed, and ITBS = IMCS  as legacy, otherwise 16QAM is assumed, and the ITBS is determined by 3bit of “Repetition number field”  IRep  in DCI format N1 (one bit reserved).

- For inband case, if MCS field IMCS < 11, QPSK is assumed, and ITBS = IMCS  as legacy, otherwise 16QAM is assumed, and the ITBS is determined by 3bit of “Repetition number field” IRep in DCI format N1 (one bit reserved)

Proposal 1: For the DCI format N1 optimization, the joint coding of MCS and repetition number field can be considered.

In NBIoT, two kinds of UE category are defined in TS36.306 in Rel.13~16, at least one new UE category is needed, the maximum TBS is 4968 and soft buffer size is 12800 increased from 6400 accordingly.

# Table 1: Downlink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- |
| Category NB1 | 680 | 680 | 2112 |
| Category NB2 | 2536 | 2536 | 6400 |

CQI Reporting to support 16-QAM in DL

For NBIoT CQI reporting in Msg3, it is defined as the NPDCCH repetition level of hypothetical NPDCCH BLER of 1%. The reported values for NB-IoT are based on mapping table in TS 36.133 section 9.1.22.15 with the NPDCCH repetition levels (1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048). The CQI reporting framework is used as a baseline to introduce the channel quality reporting for 16-QAM in DL.

Currently, there are three unused fields utilized to incorporate the reporting for 16-QAM in DL in addition to the existing NPDCCH repetitions level. The three unused states of CQI reporting can consider metric or values based on the same target of hypothetical NPDCCH BLER of 1%, the detail relationship of these metrics or values and the NPDSCH TBS/MCS is up to eNB implementation.

Proposal 2: The three unused states of CQI reporting can consider metric or values based on the same target of hypothetical NPDCCH BLER of 1%.

DL power allocation

In Rel.13 NBIoT, the UE may assume NRS EPRE is constant across the downlink NB-IoT system bandwidth and constant across all subframes that contain NRS. A UE may assume the ratio of NPDSCH EPRE to NRS EPRE is 0 dB with one NRS antenna port and -3 dB with two NRS antenna ports.

In order to support 16QAM for NPDSCH, the ratio of NPDSCH EPRE to NRS EPRE should be configured due to need of 16QAM demodulation by receiver side. LTE DL power allocation solution is the baseline for NBIoT design.

Beside the two types of OFDM symbols in LTE downlink power allocation, there are three types of OFDM symbol for NBIoT (inband case) as shown in Figure 1.

- Type A OFDM: without NRS or CRS, symbol (1),2,4

- Type B OFDM: with NRS, symbol 5,6

- Type C OFDM: with CRS, symbol 0,(1),3

Figure 1 NBIoT downlink power allocation illustration (inband case)

The ratio of PDSCH EPRE (E_A, E_B, E_C) to NRS EPRE (E_NRS) among PDSCH REs for each OFDM symbol is denoted by ρA [dBm],ρB[dBm] and ρC[dBm] separately, e.g., ρA =E_A/E_NRS, ρB=E_B/E_NRS, ρC =E_C/E_NRS. Network should semi-statically configure three types of NPDSCH EPRE separately.

For NBIoT DL power allocation, in order to reduce the signalling overhead, it is reasonable to assume all the downlink OFDM symbols with same total transmission power, the detail configuration is listed as:

- ρA =E_A/E_NRS and ρA  is determined by PA, and PA[dB]=10lg ρA, PA is configured by parameter p-a from {-6, -4.77, -3, -1.77, 0, 1, 2, 3}.

- ρB =E_B/E_NRS and ρB  is derived from parameter ρA with assuming 10E_B+2E_NRS=12E_A (2 NRS ports).

- ρC =E_C/E_NRS and ρC is derived from parameter ρA  and ρCRS  with assuming 10E_C+2E_CRS=12E_A (2 CRS ports),  and ρCRS is the power ratio of CRS EPRE (E_CRS) to NRS EPRE (E_NRS) configured by higher layer.

Proposal 3: Network should semi-statically configure three types of NPDSCH EPRE with assuming all the OFDM symbols with same transmission power.

16QAM support for NPUSCH

For the DCI format N0 optimization, similar to 16QAM support for NPDSCH, the joint coding of MCS, Repetition number and/or resource assignment field can be considered.

Proposal 4: For the DCI format N0 optimization, the joint coding of MCS, repetition number and/or resource assignment field can be considered.

Conclusions

In this contribution, considerations of 16QAM support for NPDSCH and NPUSCH are provided. The following proposals are given.

Proposal 1: For the DCI optimization, the joint coding of MCS and repetition number field can be considered.

Proposal 2: The three unused states of CQI reporting can consider metric or values based on the same target of hypothetical NPDCCH BLER of 1%.

Proposal 3: Network should semi-statically configure three types of NPDSCH EPRE with assuming all the OFDM symbols with same transmission power.

Proposal 4: For the DCI format N0 optimization, the joint coding of MCS, repetition number and/or resource assignment field can be considered.

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e