3GPP TSG RAN WG1 #104b-e	R1-2103541
e-Meeting, April 12th – 20th, 2021

Source:				Panasonic

Title:		Aspects related to reduced number of Rx branches

Agenda item:	8.6.1.2

Document for:			Discussion and Decision

Introduction

This document discusses the aspects related to reduced number of Rx branches for Reduced Capability NR devices.

Discussion

The following agreement was made in RAN1 #104:

Agreements: - For reduced minimum number of Rx branches in FR1 and FR2 frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports: - FFS: need for solutions to reduced PDCCH blocking - FFS: need for reporting of UE antenna related information to gNB (e.g., # of panels, polarization, etc.) - Information related to the reduction of the number of antenna branches is assumed to be known at the gNB (either implicitly or explicitly, to be FFS)

The WID [1] is updated in RAN #90:

This WI has the following objectives: - Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - … - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. …

Regarding the PDCCH blocking for PDCCH USS, we think it is enough to use proper configuration with DCI format 0_2/1_2 (Rel-16) because DCI payload size can flexibly be controlled.

For PDCCH CSS, the enhancement of DCI format 0_0/1_0 might be considered as a solution, but it has the issue of DCI payload size mismatch between RedCap and non-RedCap UEs when initial BWP is shared with non-RedCap UEs. Besides, we don't think the specific optimization is necessary only when initial BWP is not shared with non-RedCap UEs because we expect the gain is limited and the standardization effort is large. Therefore, the enhancement on DCI format 0_0/1_0 is not required, either.

Regarding the antenna related information, the update of WID [1] was agreed that the number of the Rx branches is supposed to be reported by the RedCap UE. We don’t think more optimization using # of panels or polarization is very important, in order to reduce the amount of the specification work.

Proposal 1:	The reduction of PDCCH blocking is realized by utilizing DCI format 0_2/1_2. Further modification is not required.

Proposal 2:	Reporting of UE antenna related information to gNB beyond the number of Rx branches is not supported.

Conclusion

Proposal 1:	The reduction of PDCCH blocking is realized by utilizing DCI format 0_2/1_2. Further modification is not required.

Proposal 2:	Reporting of UE antenna related information to gNB beyond the number of Rx branches is not supported.

Reference

RP-210918, Revised WID on Support of reduced capability NR devices