3GPP TSG RAN WG1 #104bis-e	R1-2103613

e-Meeting, April 12th – April 20th, 2021

Agenda item:	8.3.3

Source: 	Lenovo, Motorola Mobility

Title: 	Intra-UE multiplexing enhancement for IIoT/URLLC

Document for:	Discussion

# Introduction

During RAN1 #104-e meeting, RAN1 made following agreements regarding intra-UE multiplexing enhancement for  Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) [1]:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2.

- FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- FFS in case the total number of LP and HP HARQ-ACK bits is 2.

- FFS details

Working assumption:

Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities

- FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met

Agreements:

For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1.

- FFS value(s)

- FFS to additionally support beta-offset =0 or a value disabling the multiplexing

- Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

Agreements:

Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group

- FFS: dynamic indication

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

- Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

- Opt.1a: The UE does not transmit negative SR.

- Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

- Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

- FFS: whether with power boost to transmit multiplexed payload or not.

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

- Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

- Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

- Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Opt.3: No enhancement over Rel-16.

- Other options not excluded.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):

- Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

- Opt.1a: The UE does not transmit negative SR.

- Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

- Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

- FFS: whether with power boost to transmit multiplexed payload or not.

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

- Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

- Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

- FFS on conditions of multiplexing.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.5: No enhancement over Rel-16.

- Other options not excluded.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Agreements:

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

- Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

- Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

- Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

- Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

- Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

- Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: No enhancement over Rel-16.

- Other options not excluded.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

In this document, we discuss further details for multiplexing UCI of different priorities on PUCCH and on PUSCH.

# Multiplexing UCI of different priorities in PUCCH

## Multiplexing UCI of mixed priorities in PUCCH formats 0/1

When a UE multiplexes UCI of mixed priorities including up to 2bit HARQ-ACK information with/without a positive HP SR in a PUCCH resource of PUCCH format 0 or 1 (PF0 or PF1) in a slot/sub-slot, the PUCCH resource can be determined based on

- a last DCI format among DCI formats that indicates the slot/sub-slot for the PUCCH transmission and a higher priority index (e.g. the priority indicator field of the DCI format set to 1), or

- a last DCI format among DCI formats indicating the slot/sub-slot for the PUCCH transmission if no DCI format indicating a higher priority index is detected, or

- a PUCCH resource configured for UCI of mixed priorities for up to 2 HARQ-ACK bits with a positive SR, when there is no corresponding DCI format.

In the determined PUCCH resource, LP/HP HARQ-ACK bits and additionally HP SR can be indicated in a same way that HARQ-ACK bits of the same priority and additionally SR of the same priority are indicated in PUCCH format 0 or 1, according to Subclause 9.2.3 and 9.2.5.1 of TS 38.213 [2]. For low-latency HARQ-ACK feedback, the PUCCH resource can be selected from a PUCCH resource set configured by the second PUCCH-Config associated with HARQ-ACK codebook of a higher priority index, even when no DCI format indicating the higher priority index is detected. Further, the UE may receive configuration of a list of PUCCH resources to be used for transmitting UCI of mixed priorities including HARQ-ACK when there is no corresponding DCI format, where the list of PUCCH resources are configured by the second PUCCH-Config.

Proposal 1: A PUCCH resource configured by the second PUCCH-Config for multiplexing UCI of mixed priorities including up to 2bit HARQ-ACK information with/without a positive HP SR is determined based on:

- a last DCI format indicating a higher priority index, or

- a last DCI format if no DCI format indicating a higher priority index is detected, or

- a PUCCH resource configured for UCI of mixed priorities for up to 2 HARQ-ACK bits with a positive SR, when there is no corresponding DCI format.

## Multiplexing UCI of mixed priorities in PUCCH formats 2/3/4

When a UE would transmit HP UCI (i.e. HP HARQ-ACK and/or HP SR) in a PUCCH of higher priority index that would overlap in time with a transmission of a PUCCH of lower priority index including LP HARQ-ACK and when the total number of HARQ-ACK bits is larger than 2, the UE may determine whether to multiplex the LP HARQ-ACK in a PUCCH resource of PUCCH format 2, 3, or 4 of higher priority index, based on

- whether the total payload consisting of LP HARQ-ACK and HP UCI (i.e. HP HARQ-ACK and/or HP SR) is less than a supportable payload based on configured parameters of the maximum code rate and the maximum number of PRBs of a given PUCCH format; and/or

- whether the PUCCH of lower priority index carrying LP HARQ-ACK is confined within a sub-slot where the transmission of the PUCCH of higher priority index would occur.

For example, if the payload size of UCI of mixed priorities is larger than the maximum payload size determined based on the configured nrofPRBs and maxCodeRate for multiplexing UCI of mixed priorities in a PUCCH transmission using PUCCH formats 2/3/4, the UE may not multiplex LP HARQ-ACK with HP HARQ-ACK/SR. Otherwise, the UE jointly encodes HP UCI with LP HARQ-ACK for transmission in a PUCCH.

For the case that LP HARQ-ACK is not multiplexed with HP UCI due to the large payload size, if the PUCCH of lower priority index carrying LP HARQ-ACK is not confined within the sub-slot (e.g. sub-slot n) where the transmission of the PUCCH of higher priority index carrying HP UCI would occur, the UE may further determine a separate PUCCH resource for LP HARQ-ACK that would not overlap (e.g., in time) with the PUCCH of higher priority index for HP UCI and performs a separate transmission on the separate PUCCH resource.

For example, the separate PUCCH resource for LP HARQ-ACK is in a sub-slot (e.g. sub-slot n+x, where x is a positive integer) following the sub-slot where the transmission of the PUCCH of higher priority index occurs. If the PUCCH of low priority index overlaps with a transmission of another PUCCH of higher priority index for another HP UCI in the next sub-slot (e.g. sub-slot n+1), decision of joint encoding and multiplexing (or separate encoding with separate transmission) is based on the total UCI payload size in sub-slot (n+1) and/or whether the PUCCH of lower priority index is confined within the sub-slot (n+1). Thus, the UE can handle the case that the PUCCH of lower priority index overlaps in time with more than one PUCCH of higher priority index.

Proposal 2: UE determines whether to multiplex LP HARQ-ACK with HP UCI in a PUCCH resource of PUCCH format 2, 3, or 4 of higher priority index, based on the total UCI payload size and configured max. code rate/max PRB parameters.

Proposal 3: Support joint encoding of HP UCI with LP HARQ-ACK, if multiplexed in PUCCH of PUCCH formats 2, 3, and 4.

Proposal 4: If LP HARQ-ACK not multiplexed due to payload size limitation, UE can further check possible multiplexing in the next sub-slot, unless a PUCCH of low priority index for LP HARQ-ACK is limited up to a current sub-slot.

# UCI multiplexing of different priorities in PUSCH

## Enhanced multiplexing timeline

In Rel-16 NR, a UE can be configured with a sub-slot based HARQ-ACK feedback procedure in order to support low latency HARQ-ACK feedback. However, the Rel-16 NR allow the UE to perform multiplexing of UCI including HARQ-ACK information into a PUSCH based on a slot-based HARQ-ACK feedback framework. For example, according to TS 38.213 [2], “a UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs.” Thus, if a PUSCH with longer duration in a slot is scheduled together with multiple short PUCCHs carrying sub-slot based HARQ-ACK information in the slot, the UE may drop most of the sub-slot based HARQ-ACK information and accordingly, the low-latency HARQ-ACK feedback may not be fully realized.

Observation 1: Rel-16 NR allow UE to perform multiplexing of UCI including HARQ-ACK into a PUSCH based on a slot-based multiplexing framework. Thus, low-latency HARQ-ACK feedback may not be fully realized.

To fully exploit the benefit of sub-slot based HARQ-ACK feedback without stringent PUSCH scheduling restrictions, RAN1 may have to consider enhancing UCI multiplexing rules including timeline requirements.

In an example shown in Figure 1 below, a UE is dynamically scheduled to transmit a PUSCH by a PDCCH including UL DCI and also receives another PDCCH including DL DCI that schedules a PDSCH and a corresponding PUCCH for sub-slot based HARQ-ACK feedback in a sub-slot 1 of a slot ‘n’, where the PUSCH overlaps in time with the PUCCH in the sub-slot 1 of the slot ‘n’. If the starting symbol of the PUSCH is not before a symbol with cyclic prefix (CP) starting after a PUSCH preparation time Tproc,2mux after a last symbol of the PDCCH including the DL DCI and if another symbol of the PUSCH is not before a symbol with CP starting after a PDSCH processing time , Tproc,1muxTproc,1mux after a last symbol of the PDSCH, where the another symbol is the earliest symbol of the PUSCH that overlaps with the PUCCH in the sub-slot 1 of the slot ‘n’, the UE multiplexes HARQ-ACK information bits scheduled to transmit on the PUCCH in the sub-slot into the part of the PUSCH within the sub-slot  and does not transmit the PUCCH. Note that the starting symbol of the PUSCH is not before a symbol with CP starting after Tproc,2mux after a last symbol of the PDCCH including the UL DCI. The UE may compute the number of coded modulation symbols per layer for HARQ-ACK transmission by including only symbols of the PUSCH (e.g., including DMRS symbols) that are within the sub-slot when counting the number of resource elements that can be used for transmission of UCI.

With enhanced multiplexing timeline requirements, if a UE is dynamically scheduled or configured to transmit a PUSCH and also to transmit a PUCCH for sub-slot based HARQ-ACK feedback in a given sub-slot, where the PUSCH overlaps in time partially or fully with the PUCCH in the sub-slot, the UE may determine whether to multiplex HARQ-ACK in the PUSCH or to cancel a transmission of the PUSCH and transmit HARQ-ACK in the PUCCH, based on an available resource of the PUSCH within the sub-slot, e.g. the number of available symbols of the PUSCH within the sub-slot (e.g., not including DMRS symbols).

Proposal 5: Support multiplexing of multiple sub-slot based HARQ-ACK transmissions in a PUSCH spanning more than one sub-slot.

Proposal 6: Specify modified multiplexing timeline requirements to enable sub-slot based HARQ-ACK multiplexing in PUSCH.

Figure 1 Example timing relations for sub-slot level HARQ-ACK multiplexing in PUSCH

## Low-priority UCI multiplexing in high-priority PUSCH

In RAN1#102-e, it was agreed to support multiplexing low-priority HARQ-ACK in a high-priority PUSCH conveying UL-SCH only. Further, RAN1 agreed to support multiplexing low-priority HARQ-ACK in a high-priority PUSCH conveying UL-SCH, high-priority HARQ-ACK, and/or CSI. We think a couple of additional cases for low-priority UCI multiplexing in a high-priority PUSCH need to be addressed. For example,

  - Collision of low-priority HARQ-ACK (with/without SR) with a high priority PUSCH without UL-SCH

  - Collision of low-priority SR with a high priority PUSCH without UL-SCH

If a UE is indicated to multiplex a semi-persistent or aperiodic CSI report into a high-priority PUSCH that overlaps with a low-priority PUCCH including HARQ-ACK with/without SR information and if the UE would transmit the high priority PUSCH without UL-SCH, the UE can multiplex the semi-persistent or aperiodic CSI report and the low-priority HARQ-ACK with/without SR information into the high-priority PUSCH without UL-SCH.

If a UE is indicated to multiplex a semi-persistent or aperiodic CSI report into a high-priority PUSCH that overlaps with a low-priority PUCCH including SR information and if the UE would transmit the high priority PUSCH without UL-SCH, the UE can multiplex the semi-persistent or aperiodic CSI report and the low-priority SR information into the high-priority PUSCH without UL-SCH.

Since UL-SCH is not included in the high priority PUSCH, decoding performance of the PUSCH payload may not be impacted significantly, as long as the number of the low priority SR or SR/HARQ bits is small. Similarly, low-priority HARQ-ACK without SR can be multiplexed into the high priority PUSCH without UL-SCH, if the overlapping occurs.

For example, if gNB explicitly indicates UE not to include UL-SCH in the high-priority PUSCH (e.g. an UL-SCH indicator field in DCI format 0_1 is set to 0), gNB may perform decoding of the semi-persistent or aperiodic CSI report assuming that eMBB SR information (i.e. indication of positive or negative SR) is multiplexed in the high priority PUSCH (i.e. when the PUCCH for eMBB SR overlaps with the high priority PUSCH). If the UE is also due for HARQ-ACK reporting with the overlapping low-priority PUCCH, gNB may take into account potential existence of eMBB HARQ-ACK in the high priority PUSCH. In this case, the UE may apply spatial bundling of eMBB HARQ-ACK in the high priority PUSCH irrespective of harq-ACK-SpatialBundlingPUSCH setting for the high priority PUSCH, in order to reduce the number of required resource elements for multiplexing the low priority SR and HARQ-ACK with or without SR in the high priority PUSCH.

Proposal 7: In Rel-17 NR, support multiplexing of low priority SR, SR/HARQ-ACK, or HARQ-ACK without SR into a high priority PUSCH without UL-SCH.

## High-priority UCI multiplexing in low-priority PUSCH

In RAN1#102-e, it was agreed to support multiplexing high-priority HARQ-ACK in a low-priority PUSCH conveying UL-SCH only and to support multiplexing high-priority HARQ-ACK in a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. Another high-level agreement has been made to support separate configurations of at least beta-offset values for multiplexing with different priority combinations. Furthermore, in RAN1#103-e, agreement related to signalling mechanism to enable/disable multiplexing different priority of HARQ-ACK and PUSCH is made However, specific configurations related to different priority combinations still need to be discussed and agreed. For the scenario of multiplexing high priority HARQ-ACK in low priority PUSCH conveying only UL-SCH and multiplexing high priority HARQ-ACK in a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI, both low-latency and high reliability requirements should be considered. Having an additional beta offset value for high priority HARQ-ACK for low code rate transmission could help satisfy the reliability requirement, but low-latency could be an issue.

Observation 2: For multiplexing high priority HARQ-ACK in low priority PUSCH, applying different beta offset values depending upon priority level might not be enough to satisfy low-latency requirement.

Another solution could be to support repetition of high priority HARQ-ACK (shorter length with relatively high code rate) in low priority PUSCH (longer length), as illustrated by an example in Figure 2. This could allow to use a smaller number of initial symbols for the first transmission and later symbols for repetitions.

Figure 2 Example multiplexing of (high priority) HARQ-ACK repetitions in (low-priority) PUSCH

Proposal 8: Consider supporting repetitions of high priority UCI such as HARQ-ACK in low priority PUSCH to ensure both the low-latency and high reliability requirements.

## Enhanced rate matching of UCI in PUSCH

If UCI is allowed to be multiplexed in a PUSCH with a different priority (e.g. low priority UCI multiplexed into a high priority PUSCH, high priority UCI multiplexed into a low priority PUSCH), it may be necessary to set the maximum number of resource elements assigned to the low priority UCI in the PUSCH to be smaller than the maximum number of resource elements assigned to the high priority UCI in the PUSCH. This would be even more beneficial when both the low priority UCI and the high priority UCI are multiplexed in the PUSCH.

In order to allocate different maximum numbers of resource elements to UCI of mixed priorities, a UE may be configured with multiple scaling values. For example, the UE may apply a smaller scaling value for the variable  in the equation shown below, to determine the maximum allowed number of coded modulation symbols per layer for low priority UCI.

As an example, for HARQ-ACK transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows [3]:

.

For a case of high priority UCI, the UE may apply a larger scaling value to determine a larger maximum allowed number of coded modulation symbols per layer for the high priority UCI and accordingly to improve reliability of the high priority UCI.

As described above, separate encoding and rate matching (with separate scaling values) for high priority UCI and low priority UCI, respectively, are desired to satisfy different reliability and latency requirements.

Proposal 9: Support configuring more than one scaling value for the variable , to allocate different maximum numbers of resource elements to UCI with different priorities.

Proposal 10: UCI with different priorities that is multiplexed in PUSCH are separately encoded and rate-matched.

# Conclusion

In summary, we observe and propose the followings for intra-UE multiplexing enhancement for Rel-17 IIoT/URLLC:

- Proposal 1: A PUCCH resource configured by the second PUCCH-Config for multiplexing UCI of mixed priorities including up to 2bit HARQ-ACK information with/without a positive HP SR is determined based on:

  - a last DCI format indicating a higher priority index, or

  - a last DCI format if no DCI format indicating a higher priority index is detected, or

  - a PUCCH resource configured for UCI of mixed priorities for up to 2 HARQ-ACK bits with a positive SR, when there is no corresponding DCI format.

- Proposal 2: UE determines whether to multiplex LP HARQ-ACK with HP UCI in a PUCCH resource of PUCCH format 2, 3, or 4 of higher priority index, based on the total UCI payload size and configured max. code rate/max PRB parameters.

- Proposal 3: Support joint encoding of HP UCI with LP HARQ-ACK, if multiplexed in PUCCH of PUCCH formats 2, 3, and 4.

- Proposal 4: If LP HARQ-ACK not multiplexed due to payload size limitation, UE can further check possible multiplexing in the next sub-slot, unless a PUCCH of low priority index for LP HARQ-ACK is limited up to a current sub-slot.

- Observation 1: Rel-16 NR allow UE to perform multiplexing of UCI including HARQ-ACK into a PUSCH based on a slot-based multiplexing framework. Thus, low-latency HARQ-ACK feedback may not be fully realized.

- Proposal 5: Support multiplexing of multiple sub-slot based HARQ-ACK transmissions in a PUSCH spanning more than one sub-slot.

- Proposal 6: Specify modified timeline requirements to enable sub-slot based HARQ-ACK multiplexing in PUSCH.

- Proposal 7: In Rel-17 NR, support multiplexing of low priority SR, SR/HARQ-ACK, or HARQ-ACK without SR into a high priority PUSCH without UL-SCH.

- Observation 2: For multiplexing high priority HARQ-ACK in low priority PUSCH, applying different beta offset values depending upon priority level might not be enough to satisfy low-latency requirement.

- Proposal 8: Consider supporting repetitions of high priority UCI such as HARQ-ACK in low priority PUSCH to ensure both the low-latency and high reliability requirements.

- Proposal 9: Support configuring more than one scaling value for the variable , to allocate different maximum numbers of resource elements to UCI with different priorities.

- Proposal 10: UCI with different priorities that is multiplexed in PUSCH are separately encoded and rate-matched.

# References

- “RAN1 Chairman’s Notes”, RAN1 104-e meeting, January 25th – February 5th, 2021.

- 3GPP TS 38.213 V16.4.0 (2020-12), “Physical layer procedures for control (Release 16)”.

- 3GPP TS 38.212 V16.2.0 (2020-06), “Multiplexing and channel coding (Release 16)”.