3GPP TSG RAN WG1 Meeting #104b-e	R1-2103628

e-Meeting, April 12th – 20th, 2021

Title:	Draft reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU

Response to:	R3-211359

Release:	Release 16

Work Item:	NR_IAB-Core

Source:	LG Electronics [RAN1]

To:	RAN3

Contact Person:

Name:

E-mail Address:

Attachments:	None

Overall description

RAN1 would like to provide response on the following question from RAN3 in R3-211359:

At the RAN3#111-e meeting, RAN3 discussed the interpretation of the following RAN1 agreements: - From RAN1#98: “The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies)” - From RAN1#98bis: “H/S/NA attributes for the per-cell DU resource configuration are explicitly indicated per-resource type (D/U/F) in each slot.” In TS 38.473, RAN3 previously specified the HSNA Slot Configuration List IE, which allows the CU to provide H/S/NA slot configurations per IAB-DU cell. Some companies believe that this specification does not correctly reflect the above RAN1 agreements, and that the CU should instead provide H/S/NA slot configurations per (IAB-DU cell, collocated IAB-MT’s serving cell) pair, rather than one configuration per IAB-DU cell. Since no consensus could be reached, RAN3 respectfully asks RAN1 to clarify the granularity of H/S/NA slot configurations for the IAB-DU.

Response:

It is RAN1’s understanding that the H/S/NA attributes are configured to per IAB-DU cell basis, i.e., one configuration per IAB-DU cell. In RAN1#97 meeting, RAN1 agreed that the DU resource configuration including H/S/NA attributes is per DU cell basis as follows:

RAN1#97 Agreements:

For the semi-static DU resource configuration, the following is supported:

- The resources are configured on a per DU (cell) basis

  - FFS: indication of additional supplemental per-link resource configurations of child DUs

To determine the availability of MT operation in the associated carrier frequency(ies) with the DU cell, the MT should take into account the H/S/NA attributes of the DU cell. For non-associated MT carrier frequency(ies), it does not need to consider the H/S/NA attributes of the DU cell. It is the intension of the agreement in RAN1#98, “The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies)”.

Consequently, RAN1 understands specified the HSNA Slot Configuration List IE correctly reflects RAN1’s agreement.

Actions

To RAN2

Actions: RAN1 respectfully asks RAN3 to take the above information into account in their further work.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#105-e,			10th May to 27th May 2021