3GPP TSG RAN WG1 #104bis-e	R1-2103769

e-Meeting, April 12th - April 20th, 2021

Title:	About reply LS on  granularity of the HSNA Slot Configurations for the IAB-DU

Source: 	ZTE, Sanechips

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

RAN3 sent to RAN1 an LS on granularity of the H/S/NA Slot Configurations for the IAB-DU [1], and the requested action for RAN1 is given below.

ACTION: RAN3 respectfully asks RAN1 to inform RAN3 on the granularity of the H/S/NA slot configurations for the IAB-DU, with respect to the RAN1#98 and RAN1#98bis agreements.

This contribution shares our views and suggestions on the reply LS.

Discussion

According to the follow text of current TS38.213 section 14, DU H/S/NA configuration would be provided per DU cell.

With reference to slots of an IAB-DU cell, a symbol in a slot of an IAB-DU cell can be configured to be of hard, soft, or unavailable type. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive in the symbol.

There would be more than one H/S/NA configuration on a symbol in a slot of an IAB-DU cell if  it were provided per (IAB-DU cell, collocated IAB-MT’s serving cell) pair, and this is not consistent with current RAN1 specification.

Observation 1: According to current RAN1 specification, H/S/NA slot configuration is provided per DU cell.

RAN3 has mentioned two agreements of RAN1 that may cause misunderstanding at RAN3, especially for the agreement at RAN1#98.

- From RAN1#98: “The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies)” - From RAN1#98bis: “H/S/NA attributes for the per-cell DU resource configuration are explicitly indicated per-resource type (D/U/F) in each slot.”

From RAN1’s perspective, it is just a implementation recommendation to consider the associated MT serving cells for CU to provide the the per-cell DU resource configuration, RAN1 does not achieve any agreement that the H/S/NA attributes should be provided per (IAB-DU cell, collocated IAB-MT’s serving cell) pair.

Observation 2:  RAN1 does not achieve any agreement that the H/S/NA attributes should be provided per (IAB-DU cell, collocated IAB-MT’s serving cell) pair.

RAN1 had provided the updated consolidated parameter list in R1-1913674 before, it is clearly state that the H/S/NA attributes should be provided per IAB DU cell, and current TS38.473 correctly reflect RAN1’s intention.

| WI code | Sub-feature group | Parameter name in the spec |  | Description |
| --- | --- | --- | --- | --- |
| NR_IAB-Core | Resource multiplexing (F1-AP) | IAB-DU-Resource-Configuration-H/S/NA-Config (final name in specification to be determined by RAN2/3) | .... | Per-cell H/S/NA attributes per-resource type |

Observation 3:  Current TS38.473 correctly reflect RAN1’s intention and agreements.

Based on the above analysis and observations, we propose the following:

Proposal 1: To clarify to RAN3 that the H/S/NA slot configurations for the IAB-DU is provided per IAB-DU cell.

Conclusion

In this contribution, we have discussed the RAN3 LS on ranularity of the HSNA Slot Configurations for the IAB-DU. Based on this, the following proposal is provided:

Proposal 1: To clarify to RAN3 that the H/S/NA slot configurations for the IAB-DU is provided per IAB-DU cell.

Reference

- R3-211359, LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU, RAN3, Ericssion