3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103870

E-meeting, April 12th – 20th, 2021

Agenda Item:	8.1.4

Source:	(Moderator) Huawei, HiSilicon

Title:	Summary of CSI enhancements for MTRP and FDD (Round 0)

Document for:	Discussion and Decision

# Introduction

Enhancement on CSI measurement and reporting:

- Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

- Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

In RAN1 102e, RAN1 have agreed a set of evaluation assumption for above enhancement on CSI measurement and reporting over FDD and NCJT.

In RAN 103e, based on agreed evaluation assumptions, RAN1 have confirmed the interest of enhancements based on evaluation results. Some high level agreement/basic CSI measurement/reporting framework for Multi-TRP CSI enhancement were agreed. Moreover a set of candidate codebook structures for Type II port selection codebook enhancement were agreed as well for further discussion and down-selection.

In RAN1 104e, basic codebook structure for Rel-17 Type II port selection codebook enhancement was agreed. For Multi-TRP CSI enhancement in Rel-17, further basic design targets were agreed for CSI measurement setting and also for two options for CSI reporting setting.

In RAN1 104e-bis, companies have shared their consideration/preference for further detailed design for both FDD CSI and Multi-TRP CSI, which can be found in Reference and Appendix:

- General targets in RAN1 104e-bis are:

  - For FDD CSI, we may strive to finalize codebook details for Rank 1 as much as possible. In case that companies prefer more time to study some details, we can start to list alternatives to facilitate comparisons/simulations by RAN1 at least, so that we can conclude remaining details of Rank 1 by RAN1 105.

  - For MTRP CSI, we may strive to discuss next level details of clarifications/restrictions for NCJT measurement and reporting settings, at least for FR1. RAN1 may start to work on more and more FR2 specific design from RAN1 105 which tends to be controversial later. From FL perspective, I am trying my best to identify specification impact exhaustively based on companies input.

- Assuming there are two or three GTW/check points during meeting weeks, the first GTW/check point is to prioritize topics/proposals having a clear majority view based on the first round feedback. After the first GTW, we will strive to identify potential agreements/compromise by further discussion for the second GTW.

  - Please note that RAN1 105 has only 2 TU so that even if RAN1 104e-bis may not be able to reach a consensus, we shall strive to provide a clear technical guidance for next meeting.

# Summary of CSI enhancement for FDD

## Remaining issues of codebook structure for Rel-17 PS for Rank 1

### 2.1.1 Remain issues of codebook structure for W1

For port selection matrix W1∈NP×K1, about 20 companies mention polarization-common or polarization-specific selection, which are shown as Table 1.

Table 1 Summary of Companies’ Views on polarization-common or polarization-specific selection for W1

| Views | Companies |
| --- | --- |
| Polarization-common based free selection (18) | Samsung, ZTE, OPPO, vivo, Fraunhofer IIS, Fraunhofer HHI, Huawei, HiSilicon, China Unicom, Apple, Spreadtrum, CATT, Nokia, Nokia Shanghai Bell, Sony, Lenovo, Motorola Mobility, Intel |
| Polarization-specific based free selection (3) | MediaTek, QC, Ericsson |

Many companies provide simulation results on this issue whilst companies’ simulation results (MTK and Ericsson) also show that polarization-specific provides better performance-overhead trade-off than polarization-common.

Companies preferring polarization-common based free selection have the following considerations.

- Simulation performance: most companies’ simulation results (OPPO, vivo, Fraunhofer IIS, Fraunhofer HHI, Huawei, HiSilicon, China Unicom, CATT, Nokia and Nokia Shanghai Bell) show that compared with polarization-specific, polarization-common provides better or similar performance-overhead trade-off.

- UE complexity and/or specification impact: companies (ZTE, Spreadtrum, Sony, Lenovo, Motorola Mobility, and Intel) support polarization-common due to lower UE complexity and/or minimum specification impact.

Companies preferring polarization-specific based free selection have the following considerations.

- Simulation performance: MTK shows that polarization-specific provides better performance-overhead trade-off than polarization-common. And Ericsson proposes that though polarization-specific increases the UL overhead, it provides better throughput gain compared with polarization-common

- QC thinks that angle and delay information obtained from each polarization may be different and proposes polarization-specific port-selection.

Based on most companies’ simulation result and the majority view, the following proposal is suggested:

Proposal 1: Polarization-common based free-selection should be supported for W1.

| Company | Comments |
| --- | --- |
| ZTE | Support FL proposal. |
| Lenovo/MotM | Support |
| Ericsson | We prefer polarization specific based on our evaluations. |
| Intel | Support the proposal |
| Apple | Support |
| CATT | Support the proposal. |
| Samsung | We are fine pol-common as baseline. In our view, pol-spec can be beneficial if CSI-RS beamforming is pol-spec. It is unclear whether companies have studied this aspect (pol-common vs pol-specific CSI-RS beamforming). So, we are also fine to study it. |
| OPPO | Support the proposal. |
| vivo | Support. |
| Fraunhofer IIS, Fraunhofer HHI | Support |
| MediaTek | Although we prefer polarization specific port selection, we support the proposal to make progress |
| Qualcomm | We think polarization specific (esp. for M=1) is more reasonable. At gNB side, when determining SD-FD pair for CSI-RS beamforming, gNB may project the UL channel to SD and FD bases, it is reasonable to see the distribution of dominant SD-FD pairs is polarization specific. This procedure is similar to NZC locations in Rel-16 eT2 codebook. On the other hand, forcing SD-FD pairs to be polarization common may introduce “bad” pairs, and it is detrimental to CSI-RS channel estimation. Even if network use polarization common CSI-RS beamforming, polarization-specific is also beneficial, because what left UE to do is NZC selection/quantization. Some evaluations can be found below, it has more gain when beta < 1 (e.g., 0.5 here). |
| LG | We slightly prefer polarization-specific design. |
| Sony | Support FL proposal. |
| Nokia/NSB | Support FL proposal |

For port selection matrix W1∈NP×K1, some companies also propose indication mechanism for the port selection, which are summarized as Table 2.

Table 2 Summary of Companies’ Views on the indication mechanism for the port selection

| Views | Companies |
| --- | --- |
| Combinatorial coefficients (6) | Huawei, HiSilicon, China Unicom, Nokia, Nokia Shanghai Bell, Intel |
| Other mechanism | N.A. |

Considering that a few bits of CSI feedback can be saved, companies propose that combinatorial coefficients can be used for port selection. Based in companies’ views on this issue, the following proposal is suggested:

Proposal 2: Combinatorial coefficient is used for port selection for W1 .

| Company | Comments |
| --- | --- |
| ZTE | Support FL proposal. |
| Lenovo/MotM | Support |
| Ericsson | Support |
| Intel | Support the proposal |
| Apple | In principle, we are fine. However, we need to first agree on - The maximum number of ports, e.g., 32 - The maximum L, 4? 6? For P=32 and L=6, we need 20 bits to actually indicate the port selection. |
| CATT | Support the proposal. |
| Samsung | We prefer to discuss this after we have some discussion about high rank (assuming we will have high rank R17 CB). For instance, we haven’t agreed to whether port selection is layer-common or layer-specific, or other alternatives, without which it is premature to agree to the combinatorial indexing. Also, the value of P is still FFS, since it depends on the CSI-RS related study. So, we don’t know what the max value of P is. |
| OPPO | support |
| vivo | Support. |
| MediaTek | Support |
| Qualcomm | Prefer bitmap. Looking for combinatorial number using table is a big cost for memory. Ok if there is a majority view. |
| LG | Fine with the proposal |
| Nokia/NSB | Support |
| DOCOMO | Support FL’s proposal in principle. However, as pointed out by some companies, a large memory footprint may be required based on what values are configured for P and L |

For the port selection matrix W1∈NP×K1, several companies propose candidate values of K1, which are summarized as following.

- Samsung: Study the value(s) of L from {1,2,3,4,6,8}.(Note: K1=L)

- Nokia, Nokia Shanghai Bell: Regarding the candidate values for K1, consider the same value set as for P, i.e., P,K1∈{4,8,12,16,24,32} withK1≤P.

- Huawei, HiSilicon, China Unicom: 4, 8, 12, 16, 24 and 32 can be considered as candidate values of K1.

Considering companies’ views on this issue, the following proposal is suggested:

Proposal 3: With regarding to the candidate values of K1 for port selection matrix W1∈NP×K1. K1∈{4,8,12,16,24,32} with K1 ≤ P

| Company | Comments |
| --- | --- |
| Lenovo/MotM | Support |
| Ericsson | Support |
| Intel | OK with the proposal |
| Apple | We need more discussion, so we use combinatorial coefficient to indicate (32, 12), for example? |
| CATT | Needs more discussion. One of the sources of the performance gain of enhanced PS codebook is free port selection. Limiting the number of select ports may degrade performance gain. |
| Samsung | We prefer to list candidate values for evaluation, and exact values can be decided from the agreed candidate list next meeting. Also, we would like to add K1=2 as a candidate value. |
| OPPO | Support |
| vivo | Support. |
| Fraunhofer IIS, Fraunhofer HHI | We didn’t observe much performance benefit when considering K1=32. Therefore, we prefer only {4,8,12,16,24} as the candidate values of K1. |
| MediaTek | Support |
| Qualcomm | More discussion and down-selection are needed. |
| LG | Fine with the proposal |
| Nokia/NSB | Support |
| DOCOMO | We think more discussion is necessary on this issue. Especially considering the memory footprint requirements associated with the combinatorial signalling |

### Remain issues of codebook structure for Wf

For Wf∈CN3×Mv, about 18 companies give proposal on value(s) of Mv>1, which are shown as Table 3.

Table 3 Summary of Companies’ Views on value of Mv

| Views | Companies |
| --- | --- |
| Mv=2 (17) | DOCOMO, Fraunhofer IIS, Fraunhofer HHI, Spreadtrum, MTK, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, China Unicom, vivo, QC, CATT, Intel, Lenovo，Motorola Mobility |
| Mv>2 (5) | Fraunhofer IIS, Fraunhofer HHI (in the range of [2, 4]), ZTE(Mv >= 3), MTK([2,4]), vivo |

Based on companies’ view, it can be found that Mv=2 is the majority view and companies preferring Mv>2 also supports Mv=2. Therefore for the value(s) of Mv>1, the following proposal is suggested:

Proposal 4-1: For Wf∈CN3×Mv, Mv = 2 is supported for R17 PS CB

| Company | Comments |
| --- | --- |
| Qualcomm | We think WA (M > 1 being option) made in last meeting should be confirmed as agreement before discussing details. |
| ZTE | We agree with QC on the need of confirming the WA first. |
| Lenovo/MotM | Support. There were two sub-bullets in agreement in RAN1#104-e: one related to supported M value for M>1, and another sub-bullet for whether M>1 is optional. Our understanding is that Proposal 4-1 only addresses the first sub-bullet. |
| Ericsson | Support |
| Intel | Support the proposal. Given that the UE capability for Mv > 1 is already agreed as WA we are OK to confirm it. In general, we prefer to define the detailed UE capability design after the codebook design details are clear. |
| Apple | No strong opinion. But we need to confirm that Mv>1 is optional. |
| CATT | Support the proposal. |
| Samsung | First, we have the same view that WA should be confirmed. Next, we are fine with Mv=2, but for small number of CSI-RS ports since there is no gain with Wf if number of CSI-RS ports is large (as shown by several companies). Then why adding Wf for large number of ports, when it has no performance benefits, has more UE complexity, and can have more CSI overhead (when compared with Wf being turned OFF)? |
| OPPO | Support to confirm the WA that Mv>1 is optional firstly. |
| vivo | Support. |
| Fraunhofer IIS, Fraunhofer HHI | Support FL proposal. It was agreed in the last meeting that Mv > 1 is supported regardless of the working assumption. Therefore, we prefer discussing the working assumption once the codebook design details are clear. |
| MediaTek | Confirm the WA that M>1 is optional to UE. Support the proposal after this confirmation |
| Qualcomm2 | Ok, but need to confirm M > 1 being option before discussing details for Wf. |
| LG | Fine with the proposal |
| Sony | Support. |
| Nokia/NSB | Support Regarding the WA on Mν>1 optional, before confirming the WA, we think we need to clarify what is the additional UE complexity in relation to other configuration parameters, as indicated in the RAN1#104e agreement. For example, the combination N≥Mν=1 in Proposal 6 seems to have similar complexity as Mν>1, but it would be mandated according to the WA. Also, in the case Mν=1 (and N=1), a UE implementation may still calculate multiple FD components, choose the strongest one with no need to report it (a shift of FD component, if common to all ports, does not need reporting, as in Rel16) |
| DOCOMO | Support FL’s proposal |

For Mv>2, different companies have different views, which are summarized as following.

- Simulation Performance

  - Many Companies (e.g. Ericsson, Huawei, HiSilicon, China Unicom, CATT, Nokia and Nokia Shanghai Bell) provide simulations result to show that compared with Mv=4, Mv=2 can provide better performance-overhead trade-off.

  - Some companies (e.g. MTK (7~8 %), Fraunhofer IIS and Fraunhofer HHI (0.5%), vivo (3-4%)) simulation result show that compared with Mv=2，Mv=4 can provide better performance gain.

- UE complexity: some companies (QC，Intel，DOCOMO and Spreadtrum) view that larger value of Mv will increase UE complexity, so Mv>2 is not preferred.

Based on companies’ views on  Mv>2, the following proposal is suggested:

Proposal 4-2: For Wf∈CN3×Mv,  whether Mv =4 is supported for R17 PS CB:

- Alt 1: Yes, it is supported

- Alt 2: No, it is not needed

| Company | Comments |
| --- | --- |
| Lenovo/MotM | Support Alt 2 |
| Ericsson | Alt.2 not needed |
| Intel | We don’t see performance gains with Mv = 4 over the codebook with Mv = 2 on SLS. Thus, we support Alt. 2. |
| Apple | No strong opinion. But we need to confirm that Mv>1 is optional |
| CATT | Support Alt 2. |
| Samsung | Support Alt2 |
| OPPO | Support Alt 2 |
| vivo | Support Alt 1 Firstly, for the case of abundant CSI-RS ports (e.g., 32 CSI-RS ports), larger Mv can increase the upper limit of performance even if the CSI feedback overhead is larger. Secondly, for the case of limited CSI-RS ports (e.g., 8 CSI-RS ports), larger Mv can report much more information, which is equivalent to larger K1 in the case of enough CSI-RS ports. According to our simulation, it can provide considerable performance improvement in the case of 8 CSI-RS ports. Also, if only SD information is precoded in the CSI-RS ports, larger Mv is needed to report the UE selected FD information. We also provide simulation results in our contribution. |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt 1. However, if majority of companies think Mv = 2 is sufficient we are fine with it. |
| MediaTek | Support Alt 2 to reduce feedback overhead |
| Qualcomm | Alt2 |
| Sony | We prefer Alt 1, but can agree to Alt 2 if that is the majorities’ view. |
| Nokia/NSB | Support Alt 2 |
| DOCOMO | Support Alt 2 |

There are more than 10 companies shared their views on the mechanism configuring/indicating Wf to the UE. The views are listed in the following tables.

Table 4 Summary of Companies’ Views on mechanism configuring/indicating Wf

| Views | Companies |
| --- | --- |
| The FD bases used for Wf quantitation are limited within a single window (12) | Fraunhofer IIS, Fraunhofer HHI, ZTE, Spreadtrum, OPPO, CATT, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, QC |
| The FD bases used for Wf quantitation are limited within multiple windows (1) | vivo |

Companies preferring single window have the following considerations:

- Companies (Huawei, HiSilicon, China Unicom, CATT, Nokia, Nokia Shanghai Bell) support the single window since the multiple windows is equivalent to gNB implementations, e.g. with delay shift or using more beamformed CSI-RS ports.

Company preferring multiple window have the following considerations:

- vivo proposes that multiple windows are used because it is a more general framework and it can be observed that indication of inconsecutive taps can provide performance gain (2%~4%) compared with indication of consecutive taps for limited CSI-RS ports scenarios

Based on companies’ views, the following proposal is suggested:

Proposal 5-1: The FD bases used for Wf quantitation are limited within a single window with size N

| Company | Comments |
| --- | --- |
| ZTE | Support FL proposal. |
| Lenovo/MotM | Support |
| Ericsson | Support |
| Intel | Support the proposal |
| Apple | No strong opinion |
| CATT | Support the proposal. |
| Samsung | We prefer to discuss this after Mv=2 agreed. Now, for Mv=2, one of the two vectors is all-one vector, so, we are left with only one vector to be selected for Wf. We are not sure we need to restrict to window-based since there is no significant benefits. We therefore prefer free selection. Proposal 5-1: The FD bases used for Wf quantitation are down-selected from - Alt1: limited within a single window with size N - Alt2: selected freely from the orthogonal DFT matrix One of the Mv columns of Wf is fixed to all-one vector (FD component 0) |
| OPPO | Support the proposal |
| vivo | We can support the FL’s proposal if the FD bases can be configured/indicated in a discrete way within the window. Actually, we are seeking to set up a general framework to support all the cases including abundant and limited CSI-RS ports, which is corresponding to the case of precoding all SD-FD information in the CSI-RS ports and precoding SD information only in CSI-RS ports. As the taps in time-domain is not consecutive, the taps of the window are not consecutive when partial SD-FD information is conveyed by limited CSI-RS ports. To show the benefits of inconsecutive tap indication within the window, we show the simulation results in the following figure. When only 8 CSI-RS ports are used for Rel-17 PS Type-II codebook enhancement, - For a window with size N=1 (N=1, 8 ports), the upper limit of the performance gain is limited to about 30%. - For a window with size N=4/8, inconsecutive tap indication obviously outperforms the consecutive one, even close to the case of 32 ports (N=1, 32 ports). Therefore, we prefer to update the proposal as: Proposal 5-1: The FD bases used for Wf quantitation are limited within a single window with size N - The taps in the window of size N can be inconsecutive. |
| Fraunhofer IIS, Fraunhofer HHI | Support |
| MediaTek | Support |
| Qualcomm | Support, but with M=N to reduce PMI searching complexity. Maybe it should be combined with P6. |
| LG | Fine with the proposal |
| Sony | Support the FL proposal. |
| Nokia/NSB | Support |
| DOCOMO | Support FL’s proposal |

Regarding the start point of the window, i.e., Minit, six companies have shared their view as follows:

Table 5 Summary of Companies’ Views on start point of the window

| Views | Companies |
| --- | --- |
| Minit can be configured by gNB (4) | ZTE, CATT, Lenovo/Motorola Mobility |
| Minit is fixed to be 0 (4) | Fraunhofer IIS, Fraunhofer HHI, Spreadtrum, QC |

Companies preferring configurable Minit  have the following considerations:

- ZTE and CATT propose that Minit can be configured by gNB to accommodate the possibility of multiplexing more than 1 delays or UEs in one CSI-RS ports.

Companies preferring fixed Minit  have the following considerations.

- QC proposes to fix the Minit because Minit simply provides an offset in delay domain and a phase change does not change the precoder matrix and corresponding value of CQI.

Based on the above observation, the following proposal is suggested:

Proposal 5-2: Whether Minit for the window shall be configurable:

- Alt 1: Minit can be configured by gNB

- Alt 2: Minit is fixed to be 0

| Company | Comments |
| --- | --- |
| ZTE | We support Alt 1. |
| Lenovo/MotM | Support Alt 1 |
| Ericsson | Support Alt 2. |
| Intel | In our view Alt. 2 provides simpler and working solution. Given that some companies see the benefit of flexible Minit configuration and that it requires small RRC overhead increase we are OK with Alt 1. One thing we can clarify is that if the Minit is not configured it is equal to 0 (i.e. default value of Minit is 0). |
| Apple | No strong opinion |
| CATT | Support Alt 1. |
| Samsung | We are fine the Wf being configured. Re window-based Wf configuration, we have the same comment as in previous proposal. |
| OPPO | Support Alt 2. |
| vivo | Support Alt1. When Minit is not configured, Minit = 0. |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt 2 |
| MediaTek | Support Alt 2 |
| Qualcomm | Support Alt2. Alt1 is only needed if UE has to shift strongest coefficient to first tap as in Rel-16 eT2. Otherwise, Mini provides no difference in PMI reporting. |
| LG | Support Alt2. |
| Sony | We support Alt 1. |
| DOCOMO | Support Alt 1 |

For the windows size of N, companies give proposal on the value(s) of N, which are shown as Table 6.

Table 6 Summary of Companies’ Views on value of N

| Views | Companies |
| --- | --- |
| N=2 (10) | Fraunhofer IIS, Fraunhofer HHI, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, vivo, QC, MTK |
| N>2 (7) | Fraunhofer IIS, Fraunhofer HHI, ZTE (N≥3), MTK, vivo, Nokia, Nokia Shanghai Bell |

Based on companies’ view, it can be found that N=2 is the majority view and most companies (6 out of 7) who support N>2 also support N=2. In addition to N=2,  ZTE proposes that N should be larger than 2 because Mv≤2 is restrictive from performance perspective which leads to that the size N≥3 is needed.

More than 10 companies have shared their views on the mechanism on reporting Wf to the gNB, which are listed in the following table.

Table 7 Summary of Companies’ Views on mechanism on reporting Wf

| Views | Companies |
| --- | --- |
| FD component(s) in Wf aren’t reported by UE (6) | Fraunhofer IIS, Fraunhofer HHI, Samsung, QC, Nokia, Nokia Shanghai Bell |
| FD component(s) in Wf are reported by UE (9) | ZTE, Spreadtrum, CATT, Intel, Huawei, HiSilicon, China Unicom, Lenovo/MotM |

Companies preferring UE to report Wf have the following considerations.

- Companies (Huawei, HiSilicon, China Unicom) observe that (N=2, Mv=1) has better performance than (N=2, Mv=2). In addition, companies (ZTE, Intel) propose to select and report the FD components in Wf due to non-perfect synchronization and UL/DL reciprocity impairments or different best FD component among layers.

- Companies (CATT) proposes to perform selection from all the configured FD components to reduce feedback overhead.

Companies preferring UE not to report Wf have the following considerations.

- Some companies (Fraunhofer IIS, Fraunhofer HHI and Samsung) have provided simulation results that both Wf being configured and Wf being reported achieve similar performance vs overhead trade-offs.

- In additional to simulation results, some companies (Qualcomm, Samsung) propose that FD component(s) in Wf aren’t reported by UE due to the high UE complexity and CSI overhead of reporting Wf.

Base on above view, following proposal is suggested:

Proposal 6: For relationship between N and Mv, down select from the following:

- Alt  1: N=Mv always

- Alt 2: N ≥Mv and N=2, 4

| Company | Comments |
| --- | --- |
| ZTE | We support Alt 2. Alt 1 is restrictive considering rank >1. Different layers should have different FD vectors to avoid performance loss considering experience in Rel-16, while to make N=Mv always means different layers have same FD vectors. |
| Lenovo/MotM | Support Alt 2. Agree with ZTE’s comments. We propose value of N is a function of configured M Proposal 6: For relationship between N and Mv, down select from the following: - Alt 1: N=Mv always - Alt 2: N ≥Mv - FFS value of N, and whether it is a function of Mv |
| Ericsson | Support Alt.1 |
| Intel | We support Alt 2 with layer-common selection of FD vectors at the UE to maintain low PMI search complexity. |
| Apple | No strong opinion. However, gNB cannot assume UE has perform DL/UL calibration. So whatever channel gNB observes on the UL cannot fully represent the DL channel. For Alt 1, there is no UE selection in our view, so it solely replies on the gNB which is strange. |
| CATT | Support Alt 2, but the value of N shall be a separate discussion. Proposal from Lenovo is fine to us. |
| Samsung | This can be discussed after we discuss last two proposals |
| OPPO | Support Alt 1 |
| vivo | We support Alt 2 with larger candidate values of N, e.g., 8. For rank = 1, UE can select the FD information in a wide range of window indicated by gNB, especially in the case of limited CSI-RS ports., e.g., 4 CSI-RS ports are configured for the UE. For rank > 1, UE can select different FD information for each layer in a wide range indicated by gNB. |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt 1 |
| MediaTek | Support Alt 2 |
| Qualcomm | Support Alt1. Alt2 requires too many PMI searching and its benefit is yet clear. |
| Sony | Support Alt 2. The value of N can be discussed separately. Whether selection of FDD vectors at the UE is layer-common or layer-specific can be discussed separately. |
| Nokia/NSB | Support Alt 2. We observe that in Alt 2, depending on the configuration of Mν, reporting of Wf may not be needed. For example, for N≥Mν=1, UE can select any component without need to report it, like in Rel-16. The benefit of this search is, for example, to compensate for a timing offset between UL and DL receiver, common for all CSI-RS ports, which is likely to occur even if delay reciprocity were perfect. |
| DOCOMO | Support Alt 2 |

For Wf∈CN3×Mv,  about 10 companies give proposal on value(s) of R. The main views can be summarized as follows：

Table 8 Summary of Companies’ Views on values of R

| Views | Companies |
| --- | --- |
| Only R =1 (2) | Samsung, Qualcomm |
| Only R={1,2} (2) | Fraunhofer IIS, Fraunhofer HHI |
| R={1,2,…, D×NPRBSB} (8) | Ericsson, OPPO, Intel, Huawei, HiSilicon, China Unicom, Nokia, Nokia Shanghai Bell |

Note：D is the CSI-RS density in frequency domain and NPRBSB is the PRB number in a CQI sub-band

Companies preferring only R =1 have the following considerations.

- Samsung simulation results show that compared with R=1, R =4 achieved small gain (2%). However, R=4 has more UE complexity and CSI overhead than R=1.

- Qualcomm consider that the benefit of R > 1 in Rel-17 FDD CSI is unclear considering increased UE complexity, and the FD basis used in CSI-RS beamforming can be in RB granularity, the network is able to obtain an RB-level precoder even with M=1 or {M > 1, R=1}. Therefore, supporting R > 1 is unnecessary for PMI resolution.

Companies preferring only R ={1,2} have the following considerations.

- Fraunhofer IIS and Fraunhofer HHI simulation results show that when using R=2, the performance of Rel. 17 PS CB improves for all parameter combinations. Further increasing the value of R to 4 results only in a slight improvement in performance.

Companies preferring R={1,2,…, D×NPRBSB} have the following considerations.

- OPPO simulation results show that R = 4 in M = 1 can provided ~2% gain.

- Some companies’ simulation results  ( Huawei, HiSilicon, China Unicom)  show that  compared with R=2, larger R (R=4, 8, 16) can provide a better performance, especially in the case of large bandwidth for which the benefit of finer PMI quantization than CQI subband size is more profound.

- Intel consider that for the codebook introduced for system with FDD reciprocity subband size can be further reduced since the complexity of PMI search is similar for different values of R for the new codebook. In fact, the PMI subband size can be reduced to the minimum possible value such that PMI subband contains one sample of CSI-RS, i.e. PMI subband size is equal to 1/D PRB, where D is CSI-RS density.

- Ericsson thinks value of R does not affect overhead for Rel-17 Type II PS codebook. Having R as large as the CQI subband size gives the best performance without significantly increase UE complexity. And R= {NPRBSB} can be supported.

Base on above views, following proposal is suggested:

Proposal 7: With regarding to the value of R for Rel-17 PS codebook enhancement:

- Alt 1: R=1

- Alt 2: R=1 and 2

- Alt 3: R=1,2, 4, and 8

- Alt 4: R= {1,2,…, D×NPRBSB} whereas D is the density of CSI-RS in frequency domain

| Company | Comments |
| --- | --- |
| ZTE | We support Alt 3. |
| Lenovo/MotM | Support Alt 2, similar to Rel. 16 codebook |
| Ericsson | Support Alt.4. as the complexity increase for large R is marginal, and there is no additional overhead. To elaborate on this: assume Mv=1 and D = 1. The number of PMI subbands is given by N3=R×NCQISB, where NCQISB is the number of CQI subbands. The filtered DL channel is given by H=f=1N3H(f), where H(f) is the DL channel for subband f. So, the complexity only scales linearly with R, there is no big difference for R=1 or R=8. In Rel-16, the value of R may also affect the number of subband SVD operations and the number of FD basis candidates, which increases UE complexity. However, in Rel-17, the FD basis candidates can either be confined in a small window irrespective of the R value or the UE can directly use configured FD basis, and the SVD can be wideband, so such complexity issue in Rel-16 does not carry over in the Rel-17 codebook. |
| Intel | Support Alt. 4 |
| Apple | Alt 1. We cannot accept Alt 3/4 |
| CATT | Support Alt. 4. |
| Samsung | Based on our study, we observe that performance of different R values depends on the assumption whether CSI-RS beamforming is also (changes) according to the R value(s) or is fixed. In case of former, there is some gain with increasing R values. However, with the later, we don’t see any gain with increasing R values, especially when the gNB performs CSI-RS beamforming at the RB level. This is shown below, which will be included in our updated Tdoc (will be sent on the reflector soon). We therefore prefer to study this aspect carefully before making any decision, and propose to include - Alt 0: R < 1 (e.g. ¼, ½ ). |
| OPPO | We are fine with Alt 4 for M = 1. Considering the benefit of M > 1 vs M = 1 is unclear in evaluation, we prefer no enhancement (Alt 1 or Alt 2) or Alt 0 for M > 1. |
| vivo | We support Alt 3/4. According to our simulation, larger R can provide performance improvement with minor increased CSI overhead because coefficients are reported in the window of time domain which is configured/indicated by the gNB. |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt 2 |
| Qualcomm | Alt1, R=1 only. Larger value of R can be obtained by network implementation, e.g., RB-level CSI-RS precoding or interpolation after obtaining the PMI. |
| LG | Support Alt2 |
| Nokia/NSB | Support Alt 4 |

### Remain issues of codebook structure for W2

For port selection matrix W2∈CK1×Mv, more than 10 companies mention the compression which are shown as Table 9.

Table 9 Summary of Companies’ Views on compression for non-zero coefficients

| Views | Companies |
| --- | --- |
| Compression with bitmap (10) | Samsung, Nokia, Nokia Shanghai Bell, ZTE, Huawei, HiSilicon, China Unicom, QC, Lenovo, Motorola Mobility |
| Compression with combination index (1) | Intel |
| No compression for W2 (If Mv is small) (1) | DOCOMO |

Note：β is compression ratio of reported non-zero coefficients

Based on the above companies view, it can be found that bitmap for W2 used for compression is the majority view. In addition, no compression can be taken as a special case withβ=1. Furthermore, some companies provide more details on the following issues for compression of W2.

- Polarization-common or polarization-specific based bitmap

  - Polarization-common: Huawei, HiSilicon, China Unicom

  - Polarization-specific: ZTE, Lenovo, Motorola Mobility

  - FSS between polarization-common and polarization-specific: Nokia, Nokia Shanghai Bell

- Candidate values of compression ratios  β

  - β = 1 : Samsung, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom

  - Other candidate values: Samsung – {3/4, 1/2}

Based on above companies view, the following proposal is suggested:

Proposal 8: A bitmap should be used for W2 with a compression coefficient β≤1 whereas

- β=1 is supported

- FFS additional compression coefficients, e.g. 3/4, 1/2

- FFS: whether/how such a bitmap can be absent for specific codebook configuration parameters

- FFS: whether a bitmap is polarization-common or polarization-specific

| Company | Comments |
| --- | --- |
| Lenovo/MotM | Support the proposal in principle. More details on FFS parts should be finalized in email discussion before finalizing in GTW session |
| Ericsson | Support |
| Intel | There is opportunity to save the overhead by using combinatorial index reporting. For example, for 2LM = 32 coefficients with beta = 0.75, 8 bits per layer can be saved which results in 32 bits reduction for rank 4. So, we prefer to leave bitmap in the square brackets for now until more details on the exact values of beta for different rank are clear. |
| Apple | In principle, we are okay. But we need to first discuss whether UE can report less number of NZ coef. like we allowed currently |
| CATT | The bitmap may not be necessary. For example, when Mv = 1, the bitmap is not needed as the port indication can already indicate the location of non-zero coefficients. |
| Samsung | Support, but with all beta values being FFS |
| OPPO | Support |
| vivo | Support. |
| Fraunhofer IIS, Fraunhofer HHI | Support the proposal in principle. β values should be FFS. |
| MediaTek | Support |
| Qualcomm | Support |
| Nokia/NSB | Support |
| DOCOMO | Support FL’s proposal. In addition, when β=1, UE is not expected to report a bitmap. This is particularly important when configured Mv is small |

Except for potential compression, three companies make proposal on the quantization approach for linear combination coefficients of W2, which are summarized as following.

- ZTE: Reuse the Rel-16 non-zero coefficient quantization approach for the amplitudes and phases in W2 of Rel-17 PS codebook.

- Intel: Support the same coefficient quantization design as for Rel. 16 Type II codebooks

- QC: Individual amplitude and phase quantization for non-zero each coefficient

Such a remaining issue shall be discussed. Based in companies’ views on this issue, the following proposal is suggested:

Proposal 9:  For the quantization of W2 coefficient, study following Alternatives:

- Alt1: Reusing Rel-16 quantization mechanism for Rank1 at least, which can be summarized as following:

  - An indicator for the strongest coefficient

  - Two polarization-specific reference amplitudes:

    - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

    - for the other polarization, reference amplitude is quantized to 4 bits

  - For coefficients other than the strongest coefficient

    - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - phase is quantized to 16PSK

- Alt2: Individual amplitude (3 bits) and phase (16PSK) quantization for each non-zero coefficient

| Company | Comments |
| --- | --- |
| ZTE | We support Alt 1, otherwise we should list and evaluate the alternatives discussed in Rel-16. To avoid this, it is natural to reuse what we agreed in Rel-16. |
| Lenovo/MotM | Support Alt 2 |
| Ericsson | If polarization-common W1 is agreed, then we support Alt-1. If not, the second sub bullet should be removed, since polarization-specific reference amplitude doesn’t make sense |
| Intel | Both Alts looks OK |
| Apple | Support |
| CATT | Ok with the proposal. |
| Samsung | We are fine with Alt1 as baseline |
| OPPO | Ok with both Alts |
| vivo | Alt1 is slightly preferred. |
| Fraunhofer IIS, Fraunhofer HHI | Support FL proposal |
| MediaTek | Support to study both the alternatives |
| Qualcomm | Support, with minor change on amplitude quantization in Alt2 Alt2: Individual amplitude (e.g., 3 bits) and phase (16PSK) quantization for each non-zero coefficient |
| Sony | Support FL proposal to study the alternatives. |
| Nokia/NSB | Support Alt 1 |
| DOCOMO | Support FL’s proposal |

### 2.1.4 Others

Remaining proposals on codebook structure for Rel-17 Port Selection Codebook Enhancements are also listed as follows for reference.

| Company | View |
| --- | --- |
| CATT | - For Mv=1, the location of non-zero coefficients are indicated by the port selection indication. |
| Lenovo | - Configure the UE with two frequency compression parameter values for both strong and weak uplink/downlink channel reciprocity, where the UE can select the appropriate parameter value based on the strength of the channel reciprocity. |
| Samsung | - The value of P (number of rows in W1) can be discussed together with the CSI-RS related study. |

| Company | Comments |
| --- | --- |
| Ericsson | Support CATT proposal. |
| Samsung | Re CATT proposal, when Wf is turned ON, then the min value of Mv is 2, right? This is according to agreement last meeting saying “at least one Mv > 1 will be supported” |

## Analysing mechanisms to improve utilization of CSI-RS

For mechanisms to improve utilization of CSI-RS, about 20 companies provide their views on the 4 options listed in the agreements of RAN1#104bis-e, which are shown as Table 10.

Table 10 Summary of Companies’ Views on mechanisms to improve utilization of CSI-RS

| Views | Companies |
| --- | --- |
| Option 0: No further CSI-RS enhancement as the baseline (9) | Fraunhofer IIS, Fraunhofer HHI, Samsung, Spreadtrum Communications, Lenovo, Motorola Mobility, Qualcomm Incorporated, Apple, Sony |
| Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25 (11) | OPPO, Apple, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, Intel Corporation, Sony, CATT |
| Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook (6) | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, Intel Corporation |
| Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook (8) | Huawei, HiSilicon, China Unicom, CATT, ZTE, Ericsson, Samsung, Sony |
| Either Option 1 or Option 3 (15) | OPPO, Apple, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, Intel Corporation, Sony, CATT, ZTE, Ericsson, Samsung, Sony |

- CSI-RS overhead:

  - Many companies (e.g. OPPO, Apple, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, Intel Corporation, Sony and CATT) think reduction in CSI-RS overhead is needed for R17 PS CB. Moreover some companies’ (e.g. Intel (5%), Huawei (1.5%~2%), HiSilicon (1.5%~2%), China Unicom (1.5%~2%)) simulation results show that performance gain can be observed if 0.25 density CSI-RS(Option 1) can be used to reduce the CSI-RS overhead.

  - Ericsson provide simulations to show performance loss (2%~3%) if introducing 0.25 density CSI-RS

- Chanel estimation

  - LG Electronics thinks the impact on measurement performance caused by low CSI-RS density in frequency domain may not be significant because that UE is expected to do wideband operation for beamformed CSI-RS port.

  - Qualcomm Incorporated shows around 1% performance loss will be introduced of 0.25 density CSI-RS(Option 1) without taking account into CSI-RS overhead

- Standard impact:

  - Some companies (Spreadtrum Communications, Lenovo, Motorola Mobility) think  CSI-RS enhancements is out of the scope, and some companies (Fraunhofer IIS, Fraunhofer HHI, Samsung) think Option 0 can achieve good performance with no specification impact.

- CSI-RS Configuration:

  - Some companies (Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Unicom, Intel Corporation, ZTE, Ericsson, Sony, Samsung and CATT) propose some solution(s) to support more flexibility CSI-RS configuration. Several of these companies (ZTE, Ericsson, Sony, CATT and Samsung) prefer Option 3 because Option 3 has lower specification impact compared with Option 2, and others of these companies (Nokia, Nokia Shanghai Bell, Intel Corporation) think new CSI-RS pattern (Option 2) can be considered.

- More than 32 SD-FD pairs:

  - CATT provides simulations to show that using 48 SD-FD pairs can obtain performance gain (2~6%) compared with using 32 SD-FD pairs based on Option 3.

Since companies preferring option 0 seem to be also flexible with either Option 1 or Option 3, following proposal is suggested as a compromise:

Proposal 10:  For Rel-17 PS CB enhancement, a UE can be configured with a lower CSI-RS density per CSI-RS resource (0.25) and/or multiple CSI-RS resources per CSI reporting configuration.

- Whether to support CSI-RS density as 0.25 or multiple CSI-RS resource per CSI reporting configuration is optional and are subject to UE capability

- FFS further restrictions # of CSI-RS resources and # of ports associated to resources, e.g. total # of ports are less than 32.

| Company | Comments |
| --- | --- |
| ZTE | Support FL proposal. |
| Lenovo/MotM | We have concerns regarding the proposal since it is out of the WID scope. Also, it is not clear why supporting a lower CSI-RS density would be an optional UE capability |
| Ericsson | We prefer to avoid UE capabilities as much as possible. It is already a big mess. |
| Intel | In principle we are OK with the proposal, however there may be problems with this approach related to counting the CSI-RS resources and ports as part of the UE capabilities (e.g. CPU occupancy, active CSI resources). Considering the above, it may be simpler to use multiple patterns like proposed by option 2. At least, we propose to add the following to the proposal. - For Rel-17 PS CB with multiple CSI-RS resource per CSI reporting configuration, CSI reporting configuration occupies one CPU and counted as one active CSI-RS resource |
| Apple | We do not support multiple CSI-RS resources per CSI-ReportConfig for CMR. We already allow PDSCH to rate match CSI-RS. For the reciprocity based MIMO to work meaningfully, overhead of CSI-RS cannot be that large We also have concern on the UE channel estimation quality for lower density We do not support this proposal. The default outcome is no CSI-RS enhancement |
| CATT | Support the proposal. |
| Samsung | We also have concerns with lower density. We prefer Option 0 (no enh), but can be open to Option 3 for further study. |
| OPPO | We prefer to have an agreement to reduce CSI-RS overhead |
| vivo | We think there is no need for CSI-RS enhancement. With the tap indication, UE can obtain enough information with the existing CSI-RS configurations. |
| Fraunhofer IIS, Fraunhofer HHI | Prefer option 0. |
| MediaTek | Prefer Option 0 – no CSI-RS enhancement |
| Qualcomm | Prefer option 0. Option 2/3 have performance loss and marginal benefit in overhead reduction. Besides, option 2/3 introduce too many new patterns for UE to handle, adding on CSI-RS processing complexity for UE. |
| LG | We do not support option 3. This option cannot help lowering CSI-RS overhead. |
| Nokia/NSB | We support Option 1 (lower density of 0.25) but not Option 3. The same advantages as Option 3 can be achieved by introducing a single new 4x8 pattern for X=32: the number of ports per symbol is reduced to 4 and the number of resources per slot is increased to 3/D, where D is the density We have concern with allowing multiple CSI-RS resources for this specific Type II CB only. We note that only 1 resource is allowed for all Type II/eType II CBs and reports with CRI is not configurable. Is CRI now configurable for Rel-17 PS? Besides, UE capabilities for maximum number of resources/ports may need to be handled differently and the rules for CPU occupation will need to change too (Ks=2 resources may now imply OCPU=1 w/o CRI?) We also share Ericsson’s view and prefer avoiding additional UE capability. We suggest a simpler formulation of the proposal as follows: Revised Proposal 10: For Rel-17 PS CB enhancement, a UE can be configured with a lower CSI-RS density per CSI-RS resource (0.25) and/or a single new 4x8 pattern for X=32 |

## Others

Remaining proposals for Rel-17 Port Selection Codebook Enhancements are also listed as follows for reference.

| Issues | Companies | Views |
| --- | --- | --- |
| SRSSpreadts s Higher Rank | Qualcomm | - Configuration of max ratio of non-zero coefficients per layer, β, max number of non-zero coefficients per layer is K1Mβmax number of non-zero coefficients across all layers is 2K1Mβ. - UE reporting of location of non-zero coefficients per layer via bitmap |
|  | Lenovo, Motorola Mobility | - A layer-common, window-based approach is used to configure the FD basis indices for all layers, where the set of FD basis indices is contiguous, and whose location and size are higher-layer configured. - Support UE-assisted FD basis indices selection for each layer from a layer-common network-configured window. |
|  | Huawei, HiSilicon, China Unicom | - In Rel-17 PS CB enhancement, up to Rank 4 should be supported |
| Others | vivo | - To reduce the complexity of UE to counteract timing mismatch, gNB can map CSI-RS ports with a predetermined order or provide information on which ports can be used for timing calibration. - Based on the order or the information, UE can process timing calibration with much less complexity without loss of timing calibration accuracy. |
|  | CATT | - The bandwidth and density of SRS are configured as same as that of CSI-RS to obtain accurate delay information of uplink channel. |
|  | Sony | - Based on UL CSI, further restrict the set of CSI-RS ports eligible by the UE to those compatible with UL signal angles. - Introduce an FD sampling size parameter d'. Based on UL CSI, further restrictions to d' can be applied in order to limit the set of FD DFT vectors eligible by the UE. |
|  | Intel | - Support the same partial CSI omission on PUSCH design as for Rel. 16 Type II codebooks |
|  | Apple | - For CSI enhancement utilizing partial reciprocity of DL/UL channels, more flexible wideband and subband CSI reporting configuration can be considered |
|  | LG Electronics | - Confirm the working assumption. - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters |
|  | DOCOMO | - Consider dynamic configuration of turning on/off Wf using DCI. |
|  | Samsung | - Confirm working assumption on Wf being UE optional - The length of the all-one vector (when Wf is turned OFF) depends on the reporting format (e.g. WB/SB, SB size etc.), and hence should be discussed with the reporting format discussion. |

| Company | Comments |
| --- | --- |

# Summary of CSI enhancement for Multi-TRP

## CS CSI Framework Enhancements for Multi-TRP

### Resource settings

- Further Details of Configuration Methodology

It was agreed in last meeting that the UE can configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis. The CMRs in each pair are determined from two CMR groups with  Ks=K1+K2 CMRs.

For the maximum values of N and Ks, i.e., Nmax and Ks_max, to avoid the high processing complexity at UE, InterDigital, Vivo and Spreadtrum propose that only Nmax=1 and Ks_max=2 is support in Rel-17. OPPO and MediaTek propose that Nmax=1 and Ks_max=2 is mandatory to support and the other values of 1<Nmax≤4 and 2<Ks_max≤4 should be up to UE capability.  Ericsson and Qualcomm propose that the maximum value of 8 CMRs in a CSI-RS resource set should remain the same as Rel.15/16. The companies’ view on the values of Nmax and Ks_max are summarized as Table 11.

Table 11 Summary of companies’ views on the values of Nmax and Ks_max

| Views |  | Companies |
| --- | --- | --- |
| Nmax | Nmax=1 | OPPO (1st preference), InterDigital, Vivo, MediaTek (1st preference), Spreadtrum (5) |
|  | Nmax=2 | Huawei, HiSilicon, China Unicom, OPPO (4) |
|  | Nmax=4 | MediaTek, Qualcomm, Ericsson (3) |
| Ks_max | Ks_max=2 | OPPO(1st preference), InterDigital, Vivo, MediaTek(1st preference), Spreadtrum (5) |
|  | Ks_max=4 | OPPO, MediaTek (2) |
|  | Ks_max=8 | Qualcomm, Ericsson, LGE (3) |

For the number of CMRs in two CMR groups, i.e., K1 and K2, 3 companies (OPPO, Qualcomm, Docomo) support K1= K2 while 5 companies (Huawei, HiSilcon, China Unicom, MediaTek, Ericsson) support no restriction on K1 and K2.

Based on above companies view, the following proposal is suggested:

Proposal 11: With regarding to the maximal values of Nmax for N,  Ks,max  for Ks and possible restriction betweenK1 and K2, down select from following Alternatives:

- Nmax=

  - 1

  - 2

  - 4

- Ks,max=

  - 2

  - 4

  - 8

- For K1 and K2

  - Alt 1: K1=K2=Ks/2

  - Alt 2: No restriction as long as K1+K2=Ks

| Company | Comments |
| --- | --- |
| QC | For max N, we are also fine with 2. For max Ks, we are also fine with 4. For K1 and K2, we prefer Alt1. |
| ZTE | Ks,max should be at least 8 which has been supported in Rel-15/16, we cannot have a backward design in Rel-17 We are flexible for Nmax value, maybe 1 is our first preference for simplicity |
| Lenovo/MotM | We suggest limits on Ks,max and Nmax depend on the frequency range and/or UE capability. We support Alt 2 with K1+K2=Ks, no need for Alt 1 restriction at least for FR1 |
| Intel | Nmax should be at least 2 and Ks,max should be at least 4. For K1 nad K2 we prefer Alt. 2. |
| Aple | We are fine with the proposal. The more important part of the CPU and active RS counting |
| MediaTek | We support Ks,max = 4. Either Nmax = 2 or Nmax = 4 is fine. We support Alt. 2 for K1 and K2. |
| OPPO | We prefer Nmax = 1 and Ks,max=2. We don’t think Nmax = 4 and Ks,max=8 is needed considering only N=2 and M=2 is agreed in 8.1.2.3. We don’t think a smaller value is a backward design since the value is applied for NC-JT transmission. |
| Ericsson | For Nmax, we prefer a value of at least 2. For Kmax, we prefer a value of at least 4. For K1 and K2, we do not see the need for introducing a restriction. We prefer Alt 2 (no restriction). |
| DOCOMO | Fine with the proposal. We support Nmax = 4, Ks,max = 8, and Alt.1 K1=K2=Ks/2. |
| vivo | We are OK with the proposal. For Nmax and Ks,max, Our first priority are 1 and 2, respectively. Our main concerns is on UE computation complexity, the number of CPUs and active RS counting, when Nmax and Ks,max are larger. For example, if K1=K2=4(i.e. Ks = 8) and N=4, it may require up to 16 CPUs (8 for NCJT and 8 for STRP), which seems impractical to UE. For K1 and K2, we prefer Alt1. |
| LG | We are fine with Nmax=2 or 4, and Ks,max=8. For K1 and K2, we prefer Alt.2. |
| Fraunhofer IIS, Fraunhofer HHI | Support FL’ proposal and fine with Alt1 and Alt2. |
| Spreadtrum | Fine with the proposal. We prefer Nmax = 1 and Ks,max=2. In Rel-15/16 UE capability, the simultaneousCSI-ReportsPerCC is [1,8] depending on UE capability. For Nmax > 1 and Ks,max>2, the occupied CPUs for this report is relatively large and it is easy to excess UE’s capability of supporting the number of simultaneously CPU, which will result in invalid CSI configuration and measurement. For example, if K1=K2=4(i.e. Ks = 8) and N=4, if following proposal 17, it may require up to 16 CPUs (8 for NCJT and 8 for STRP), and it is very larger than 8 (the strongest UE’s capability). |
| Nokia/NSB | For Nmax we prefer at least 2 For Ks,max we prefer at least 4 For K1 and K2 we prefer Alt1 for simplicity of configuration |
| Futurewei | We are ok with FL’s proposal. For Nmax, we are ok with Nmax = 2, For Ks,max, we are ok with Ks,max = 4, For K1 and K2, we are ok with Alt. 2. |
| NEC | Support the proposal. For Nmax, we prefer at least 2 For Ks,max, we prefer 8. And for K1 and K2, we are OK with Alt. 2. |
| Samsung | Support the proposal. We are fine with Ks,max = 8 as supported in Rel-15/16. For K1 and K2, we prefer Alt2. |
| InterDigital | Our preference is Nmax = 1 and Ks,max=2. |

For the determination of N CMR pairs, 10 companies (Huawei, HiSilicon, China Unicom[1], ZTE[7], Fraunhofer IIS, Fraunhofer HHI[10], Qualcomm[14], Docomo [21], Nokia, Nokia Shanghai Bell[17]) support to use a bitmap of size K1 K2 by RRC signalling. To allow network to adjust the number and type of measurement hypotheses to avoid CPU overbooking in semi-persistent and aperiodic multi-TRP CSI reporting, Nokia, Nokia Shanghai Bell [17] and Ericsson [20] propose that the gNB can dynamically update the NCJT pairs by MAC-CE. In some case, not all resources in 2 CMR groups always should be used for STRP measurements, especially when some transmit beams may only be suitable for MTRP transmission. In addition to a bitmap of size K1 K2 to indicate N CMR pairs for NCJT measurement hypothesis, a high layer parameter (Nokia, Nokia Shanghai Bell [17]) or additional bitmap (ZTE [7]) can be introduced in NR Rel-17 for the STRP CMRs indication.

It seems to be the majority view so that the following proposal is suggested:

Proposal 12: A bitmap of size K1K2 is used to configure N CMR pairs from given a set for NCJT hypotheses

- FFS: whether to support dynamic updating CMR pairs for NCJT measurement hypotheses, e.g. by MAC-CE

- FFS: whether additional high layer signalling, e.g., a bitmap, is needed to configure M (M≤ Ks) CMRs from the same set for Single-TRP measurement hypotheses

| Company | Comments |
| --- | --- |
| QC | For the second FFS, we think either a bit map or a flag is needed to enables / disable sTRP hypos. Just because a CMR pair is configured for NCJT hypothesis, it does not mean that by default they should also correspond to 2 additional sTRP hypotheses. We are not sure how the issue of reusing CMRs between sTRP and NCJT hypotheses in FR2 can be handled in the absence of this additional signalling. Also, we are not sure how CPU usage can be managed without this additional signalling (each pair automatically comes with 4 CPUs). |
| ZTE | We share the same view with QC. CMR pairs should not be automatically used for STRP as well especially in FR2. UE panel receive behaviour for STRP and MTRP may be different which may cause STRP CSI estimation error. Further, CPU/active resource and ports cost will be huge. We should give gNB flexibility to select whether a CMR is for STRP or not. In addition, this proposal relies on the proposal 11. If Nmaxi is 1, then there is no need to have bitmap for MTRP CMR pair selection. So we suggest to agree the second bullet. Proposal 12: A bitmap of size K1K2 is used to configure N CMR pairs from given a set for NCJT hypotheses - FFS: whether to support dynamic updating CMR pairs for NCJT measurement hypotheses, e.g. by MAC-CE - FFS: whether aAdditional high layer signalling, e.g., a bitmap, is needed to configure M (M≤ Ks) CMRs from the same set for Single-TRP measurement hypotheses |
| Lenovo/MotM | Support. We prefer discussing Proposal 12 after finalizing Proposal 11, as suggested by ZTE |
| Intel | There was a situation in 3GPP where RAN1 agreed to support bitmap with only one “1” in it; obviously that is not an optimal solution. To avoid such mistakes, we prefer to leave design of RRC signalling up to RAN2. Thus, we think that this agreement is not needed at all. |
| Apple | This is RAN2 signalling design. It depends on the K1, K2 and N, bitmap is not the most efficient way for indication. For example, for K1=K2=4, and N=1 Why do we need 16 bit to indicating how to select 1 pair out of the 16 pairs? |
| MediaTek | We share similar views as Intel and Apple. It can be up to RAN2 design based on the outcome of Proposal 11. |
| OPPO | Agree with Apple that the size of bitmap is depended on the outcome of proposal 11. The detailed design is up to RAN2, and we can send a LS once we have conclusion on the number of Ks and N. |
| Ericsson | We agree with Intel and Apple that RRC signalling details should be left to RAN2. In the example provided by Apple, 4 bits should be sufficient to select 1 pair out of 16. So, a bitmap of size K1*K2 is not efficient. We suggest not to discuss bitmap design details in RAN1 and leave this to RAN2. In the last meeting, we already agreed that N CMR pairs are selected from all possible pairs. So, the main bullet does not need to be discussed further. For the two sub-bullets, we are fine to study further. Please see below a some suggested revisions: Revised Proposal 12: For CSI measurement associated with a CSI-ReportConfig for NC-JT and/or single-TRP CSI A bitmap of size K1K2 is used to configure N CMR pairs from given a set for NCJT hypotheses - FFS: whether to support dynamic updating of CMR pairs for NCJT measurement hypotheses, e.g. by MAC-CE - FFS: whether additional high layer signalling, e.g., a bitmap, is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR same set for Single-TRP measurement hypotheses |
| DOCOMO | Support the proposal. Support additional high layer signalling to indicate the CMR resources for single-TRP measurement hypotheses at least for FR2. |
| vivo | We think this should be discussed after Proposal 11. |
| LG | We also think details of RRC signalling can be discussed in RAN2. |
| Fraunhofer IIS, Fraunhofer HHI | Support FL’ proposal. Support additional signalling to configure S-TRP hypotheses. |
| Nokia/NSB | We support the proposal for the NCJT bitmap part. It’s just a simple way to describe to RAN2 how the CMR pairing is done. It’s then up to RAN2 to “translate” the bitmap concept in appropriate RRC/MAC-CE signalling. As for the second FFS point, we support the need for additional RRC and MAC-CE signalling but we think the parameter M, with 0≤M≤Ks is sufficient, with M/2 CMRs taken from the first group and M/2 from the second group. In fact, the network already has the flexibility to list the CMRs corresponding to the “1”s in the S-TRP bitmap at the beginning of their respective group list, without restricting the choice of either S-TRP or NCJT hypotheses. Proposal 12: A bitmap of size K1K2 is used to configure N CMR pairs from given a set for NCJT hypotheses - FFS: whether to support dynamic updating CMR pairs for NCJT measurement hypotheses, e.g. by MAC-CE - FFS: whether additional high layer signalling is needed to configure M (M≤ Ks) CMRs from the same set for Single-TRP measurement hypotheses, e.g. - Alt 1: parameter M with 0≤M≤Ks, indicating the first M/2 CMRs from the first group and the first M/2 CMRs from the second group - Alt 2: a bitmap |
| Futurewei | We are ok with FL’s proposal in principal and as suggested by other companies, this proposal should be discussed after Proposal 11. |
| NEC | We are fine with the proposal. And fine to discuss the proposal after Proposal 11. |
| Samsung | We support the additional higher layer signalling for sTRP measurement for FR2, but this proposal seems detailed design for RAN2. Based on bitmap or something else can be up to RAN2, so we prefer Ericsson’s revised Proposal 12. |
| InterDigital | This can be discussed in RAN2 after RAN1 finalizes N, K1, K2 values as mentioned by several other companies. |

- Channel Measurement Resource

Another critical RAN1 decision is about whether a CMR used for a single-TRP measurement hypothesis can be reused for NCJT measurement hypothesis. CMCC[11],Nokia[17] propose that the CMR cannot be used for both NCJT and Single-TRP measurement hypotheses in FR2, because the CMRs transmitted by the single TRP used for single-TRP and NCJT measurement hypotheses might be received by different beams. Qualcomm [14] further propose that UE can indicate whether it supports reusing a CMR between a single-TRP hypothesis and a NCJT hypothesis or between two NCJT hypotheses in a reporting setting.On the other hand, simulation results provided by Vivo[5] show that there is almost no performance loss, which the CMRs for NCJT hypothesis are used for sTRP hypotheses for FR2.

Considering the companies’ views, the following proposal is suggested:

Proposal 13-1: Whether a NZP CSI-RS resource can be referred by two CMR pairs configured for NCJT measurement hypotheses (if supported):

- Alt 1: It is feasible for FR1 but not for FR2.

- Alt 2: It is feasible for both FR1 and FR2 but subject to further UE capability for FR2.

| Company | Comments |
| --- | --- |
| QC | We cannot accept Alt1. As we explained in the previous meeting, the feasibility depends on UE implementation. |
| ZTE | If one CMR m is referred by two CMR pair (m, a) and (m,b), another option is to ensure CMR a and CMR b correspond to the same UE receive beam, This is beneficial for the case when CMR a and b have different transmit beams, e.g. a narrow beam and a wide beam. Then a two panel UE can still support both CMR pair (m,a) and (m,b). For Alt 1, we think it is simple. For Alt 2, if UE supports one CMR referred by two CMR pairs, that means UE has to active at least three panels simultaneously which is not the typical case we discussed here. Further, what if one CMR referred by three CMR pairs? Can UE has four panels to support that subject UE capability? So we don’t prefer Alt2. Proposal 13-1: Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses (if supported): - Alt 1: It is feasible for FR1 but not for FR2. - Alt 2: It is feasible for both FR1 and FR2 but subject to further UE capability for FR2. - Alt 3: CMR a and CMR b should be QCLed in terms of QCL-TypeD |
| Lenovo/MotM | Support Alt 2 |
| Intel | In our view it is simpler to avoid this case in Rel. 17 for FR2. Thus, we prefer Alt. 1. |
| Apple | We are fine with the proposal |
| MediaTek | Support Alt. 2 with “It is feasible for both FR1 and FR2” only. If different Rx spatial filters are preferred, then two separate CSI reports can be configured, one for NCJT and one for single-TRP. We support that UE indicates whether different Rx spatial filters are applied. |
| OPPO | Support Alt 2. |
| Ericsson | This proposal depends on the value of Nmax which is to be agreed in Proposal 11. If Nmax = 1 is agreed, then there is no need for this proposal. Suggest to discuss this proposal after an agreement is reached in proposal 11. |
| DOCOMO | We prefer Alt.1 in Rel-17 which is simper. |
| vivo | This is not an issue when N=1. So we need to make an agreement on Proposal 11. |
| LG | Support Alt 1. |
| Spreadtrum | Generally we are fine with the proposal. But we are also fine with discussing it later, depending on other proposal progress, e.g., Proposal 11. |
| Nokia/NSB | We prefer Alt 1 and are ok with Alt 3 by ZTE |
| Futurewei | We support Alt. 2. |
| NEC | Prefer Alt 1, and also OK with Alt 3 from ZTE. |
| Samsung | Support Alt1 and prefer to discuss after finalizing the Proposal 11. |
| InterDigital | This can be discussed after Proposal 11 is concluded. |

Proposal 13-2: Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis:

- Alt 1: It is feasible for both FR1 and FR2.  Two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses.

- Alt 2: It is feasible for FR1 but it is not for FR2. For FR2, the UE is expected to have different NZP CSI-RS resources configured for all CMRs of Single-TRP and NCJT measurement hypotheses respectively.

- Alt 3: It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

| Company | Comments |
| --- | --- |
| QC | We support Alt2 or Alt3. We cannot accept Alt1 as the feasibility depends on UE implementation. |
| ZTE | We support Alt 2 for simplicity |
| Lenovo/MotM | Support. Prefer Alt3 |
| Intel | Same view as ZTE (Alt 2) |
| Apple | We are fine with FL proposal |
| MediaTek | We support Alt. 1. |
| OPPO | We support Alt 1 or Alt 3. It depends on UE implementation. |
| Ericsson | Given the results from some companies, we think further discussion is needed before we can do a down-selection among the alternatives. At this stage, we prefer not to introduce UE capabilities for this issue. So we do not support Alt 3. |
| DOCOMO | We prefer Alt.2 in Rel-17 which is simper. |
| vivo | We support Alt1 or Alt3. Alt2 forbids the use of NCJT's CMR for STRP measurement, which is too strict. Besides, on whether the CMRs for NCJT hypothesis measurement in 2 CMR groups can be used for STRP measurements for the FR2, we have done some simulations in R1-2102512, showing that there is little performance difference when UE has two panels with 90 and 180 degrees between each other. |
| LG | Support Alt3 and Alt2. |
| Spreadtrum | Support the proposal |
| Nokia/NSB | We prefer Alt 2 |
| Futurewei | We support Alt. 1. |
| NEC | We prefer Alt 2. |
| Samsung | Support Alt2 |
| InterDigital | We support the proposal. |

- Interference Measurement Resource

For CSI-IM, 6 companies (Huawei, HiSilicon, China Unicom [1], Qualcomm [14], LGE[18], Docomo [21]) support to follow the same principle as in Rel. 15 / 16. In particular, each CMR/CMR pair is one-by-one associated with a CSI-IM resource, and the total number of CMRs/CMR pairs is equal to the number of CSI-IM resources. For a NZP CSI-RS resource set with Ks CMRs for single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, it is associated with Ks+N CSI-IM resources.

Vivo [5] consider that one CSI-IM resource for NCJT CSI measurement and single-TRP CSI measurement hypothesis. Specifically, regarding interference measurement for NCJT hypothesis, one CSI-IM resource is enough to measure the outer-TRP interference. Regarding interference measurement for single-TRP CSI measurement hypothesis, inter-TRP and outer-TRP interference is needed to measure and multiple CSI-IM resources are required accordingly. However combining the out-of-dated CSI report, Vivo believe that the existence of the inter-TRP interference has little influence on the single-TRP performance. UE can acquire the inter-TRP interference, e.g., by historical measurement results or the CMR of another TRP. The simulation results provided by Vivo show that in both FR1 and FR2, configuring only one CSI-IM resource associated with a CMR set cause negligible performance difference, when N = 1 and Ks = 2.

FutureWei [9] proposes that a CSI-IM resource can be associated with a CMR pair for a NCJT measurement hypothesis and can also be associated with one CMR for single-TRP measurement hypothesis when the CMR is one CMR in the pair.

Considering the companies’ views, the following proposal is suggested:

Proposal 14-1: Whether a CSI-IM can be referred by both NCJT and Single-TRP measurement hypotheses:

- Alt 1: It is feasible in both FR1 and FR2. Only [one] CSI-IM resource is configured to be associated with a CMR set.

- Alt 2: A CSI-IM resource is configured to be associated with either a CMR for  Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis

| Company | Comments |
| --- | --- |
| QC | Support Alt2. It is not clear to us why CSI-IM should be shared. Not only the interference conditions are different (for single-TRP hypo, the other TRP can be configured as CSI-IM), but also the QCL-TypeD assumptions cannot be shared, which is similar to the issue of the previous proposal. |
| ZTE | We prefer Alt 2. But it is better to decide this after we get consensus of previous proposals. |
| Lenovo/MotM | We believe this should be discussed after down selection on Proposal 14-3 |
| Intel | Prefer Alt. 2. |
| Apple | We prefer the existing rule, i.e., one to one mapping |
| MediaTek | Support Alt. 2. |
| OPPO | Alt 2 is preferred. |
| Ericsson | Note that this proposal may depend on the outcome of Proposal 13-2. For instance, if we in Proposal 13-2, decide that a CMR in the CMR pair for NC-JT measurement hypothesis cannot be used for single TRP measurement hypothesis, then we don’t need to additional CSI-IM per CMR pair. Consider the following example: Resource set for CMR {CMR1, CMR2, CMR3} CSI-IM set {CSI-IM1, CSI-IM2, CSI-IM3} Let’s say the pair {CMR1, CMR2} is higher layer configured for NC-JT measurement hypothesis. If in Proposal 13-2, we conclude that CMR1 and CMR2 cannot be used for single TRP measurement hypothesis, then CSI-IM1 and CSI-IM2 can be reused for NC-JT measurement hypothesis. So, depending on the conclusion in Proposal 13-2, we may end up with the case that existing rule (one to one mapping) may be sufficient. Hence, we suggest to discuss this proposal after concluding Proposal 13-2. |
| DOCOMO | Support Alt.2. |
| vivo | Our focus is reducing the CSI-IM resource overhead. According to Alt2, KS + N CSI-IM resources may be needed, which will result in expensive signaling and resource overhead. As a matter of fact, due to the out-of-dated CSI report, we believe that the existence of the inter-TRP interference has little influence on the STRP performance in FR2. From the perspective of UE implementation, UE can also acquire the inter-TRP interference, e.g., by historical measurement results or the CMR of another TRP. The simulation results provided by R1-2102512 show that in FR2, configuring only one CSI-IM resource associated with a CMR set cause negligible performance difference, when N = 1 and Ks = 2. Proposal 14-1: Whether a CSI-IM can be referred by both NCJT and Single-TRP measurement hypotheses: - Alt 1: It is feasible in both FR1 and FR2. Only [one] CSI-IM resource is configured to be associated with a CMR set. CSI-IM can be shared by both NCJT and Single-TRP measurement hypotheses. - Alt 2: A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis |
| LG | Support Alt2. |
| Nokia/NSB | Prefer Alt 2 |
| Futurewei | We support vivo’s revised proposal. |
| NEC | Prefer Alt 2. |
| Samsung | Support Alt2 |
| InterDigital | We support Alt. 2. |

For QCL-TypeD assumption of CSI-IM resource, because the interference should be measured using the same receive beam as the one used for CMR measurement, 5 companies (Huawei, HiSilicon, China Unicom [1], Qualcomm [14], LGE[18]) propose that if a CSI-IM resource is associated with a CMR pair, the CSI-IM resource is QCLed with two CMRs in that pair with respect to ‘QCL-TypeD’.

Considering the companies’ views, the following proposal is suggested:

Proposal 14-2: The UE may assume that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource.

| Company | Comments |
| --- | --- |
| QC | Support the proposal. |
| ZTE | OK |
| Intel | We are fine with the proposal |
| Apple | This is ambiguous. When we have a pair of CMR, we do not need know how CSI-IM is configured. |
| MediaTek | Support. |
| OPPO | Does it mean that UE needs to measure the interference on CSI-IM resource based on two different beams and combine them together? The corresponding UE behaviour is unclear. |
| Ericsson | It seems this proposal assumes that there will be one additional CSI-IM introduced per CMR pair. But as we commented in Proposal 14-1, depending on the conclusion of Proposal 13-2, we may not need to introduce an addition CSI-IM per CMR pair (i.e., the existing one-to-one mapping may be sufficient). So we prefer to discuss this proposal after concluding on Proposal 13-2. |
| DOCOMO | OK. |
| vivo | Support. |
| LG | Support. |
| Spreadtrum | Support |
| Nokia/NSB | Support |
| Futurewei | We are ok with the proposal in principle. |
| NEC | Support. |
| Samsung | Support the proposal. |
| InterDigital | We have a similar question as OPPO. |

In NCJT, the layers from the different TRPs incur interference mutually. Such mutual interference cannot be measured by CSI-IM accurately. Three companies (InterDigital[3], Samsung [15], Ericsson [20]) consider that for NCJT measurement hypothesis, interference measurement based on NZP CSI-RS is supported.

Proposal 14-3: Whether to support interference measurement based on NZP CSI-RS for NCJT measurement hypothesis, in addition to CSI-IM,

- Alt 1: Yes, it is supported

- Alt 2: No, it is not supported

| Company | Comments |
| --- | --- |
| QC | Support Alt2. The condition is clear in current spec: “Except for L1-SINR, if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement.” |
| ZTE | This issue can be lower priority |
| Lenovo/MotM | Support Alt1 |
| Intel | Alt. 2 |
| Apple | Prefer Alt. 2 |
| MediaTek | Support Alt. 2. |
| OPPO | Prefer Alt 2. |
| Ericsson | We think it is beneficial to support interference measurement based on NZP CSI-RS. Let us consider an FR2 example where the UE measures with two panels. NZP CSI-RS #1 and NZP CSI-RS #2 are transmitted from TRP 1 and TRP 2, respectively. Then, the received signal vectors at two panels (denoted by y1 and y2) can be written as y1y2=H11H12H21H22W1s1W2s2+n1n2 H11 is the channel matrix corresponding to TRP1 – this is measured on NZP CSI-RS#1 on panel 1. H22 is the channel matrix corresponding to TRP2 – this is measured on NZP CSI-RS#2 on panel 2. To measure the cross term H12 from TRP1 to panel 2, the interference from NZP CSI-RS#1 to panel 2 needs to be measured. Similarly, to measure the cross term H21 from TRP2 to panel 1, the interference from NZP CSI-RS#2 to panel 1 needs to be measured. We do not think H12 and H21 can be accurately measured using CSI-IMs. Hence, we support Alt 1. |
| DOCOMO | Low priority. We can discuss the interference measurement based on CSI-IM first. |
| vivo | Prefer Alt. 2 |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt1. |
| Spreadtrum | We would like to clarify that here NZP CSI-RS is referred to be NZP CSI-RS configured as CMR, or NZP CSI-RS configured as IMR? |
| Nokia/NSB | Support Alt 2 |
| Futurewei | We support Alt. 1. |
| NEC | Prefer Alt 2. And we also think this can be low priority. |
| Samsung | Support Alt1. As in Ericsson’s example, that kinds of reusing CMR as NZP-IMR is desirable. |
| InterDigital | We support Alt. 1. The inter-layer interference cannot be accurately measured using CSI-IMs; the NZP CSI-RS can be used to emulate it. |

### 3.1.2 Reporting quantity configurations

- CRI Reporting

For the CSI measurement hypothesis indication, some companies provide the candidate designs summarized as following:

11 companies (Huawei, HiSilicon, China Unicom [1], OPPO [2] (Method 2), ZTE [7], Fraunhofer IIS, Fraunhofer HHI[10], Qualcomm [14], Nokia, Nokia Shanghai Bell [17], Docomo[21] (for CSI Option 1)) propose that CRI is used to indicate CSI hypothesis. Each codepoint corresponds to a Single-TRP measurement hypothesis or a NCJT measurement hypothesis. For the bitwidth of CRI, Huawei, HiSilicon, China Unicom [1], OPPO [2] (Method 2), Nokia, Nokia Shanghai Bell [17] propose to use a single CRI codebook and the corresponding bitwidth of CRI is log2(Ks+N), where Ks is the number of valid CMRs for Single-TRP measurement hypotheses and N is the number of valid CMR pairs for NCJT measurement hypotheses. For CSI Option 1, ZTE [7] propose that X+1 CRIs should be repored, where X CRIs are for STRP and 1 CRI is for MTRP CSI, the bitwidth of each CRI depends on the corresponding number of valid CMRs for STRP or valid CMR pairs for MTRP.

3 companies (OPPO [2] (Method 1), Vivo [5], Docomo [21](for CSI Option 2)) propose to introduce a separate field in a CSI report to report recommended hypothesis. Specifically, one bit is used to indicate the single-TRP measurement hypothesis or NCJT measurement hypothesis. The CRI, whose bitwidth is log2(max⁡(K,N)), is used to indicate one measurement hypothesis from N NCJT measurement hypotheses or from K single-TRP measurement hypotheses.

OPPO [2] (Method 3) and MediaTek [8] propose to indicate the measurement hypothesis by using the RI(s). Specifically, if two RIs are reported in CSI, when a specific value (e.g. RI=0) is reported in one RI, the CSI is measured based on single TRP measurement hypothesis, and only the other RI is applied. If both RIs indicate valid values, both Ris can be applied with NC-JT measurement hypothesis.

Considering companies’ views on this issue, following proposals for CSI report options 1 and 2 are suggested:

Proposal 15-1:  For the UE configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis (i.e. Option 1),

- Alt 1: X+1 CRIs are reported, whereas X CRIs are for single-TRP measurement hypotheses and one CRI is for NCJT measurement hypothesis.  Each CRI bit size depends on the corresponding number of either valid CMR pairs for NCJT measurement hypothesis or valid CMRs for single-TRP measurement hypotheses.

- Alt 2: if any

| Company | Comments |
| --- | --- |
| QC | Do not understand the proposal. Each CSI has its own CRI. The overall number of CRIs is not just X+1 but it depends on total number of CSIs n (across all report configs). The same framework should be reused. Otherwise, many of the tables in 38.212 need to changed: e.g., Table 6.3.1.1.2-7, Table 6.3.1.1.2-9, Table 6.3.1.1.2-10, Table 6.3.1.1.2-11. Therefore, how to generate UCI bits across different CSIs do not need to be discussed other than an order between 2 or 3 CSIs. Note that 38.212 relies on the priority order across the n CSIs “where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-13 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].” Hence, only 38.214 need to specify the order. The change should not be specifying new tables for X=1 and X=2 in 38.212. Instead, the new table is only needed for NCJT CSI. |
| ZTE | Support this straightforward proposal. @QC, we think this proposal just discuss the number of CRIs for one CSI reporting for Option 1. Definitely, X+1 CRIs should be reported by UE. For the mapping order, that is the second detailed issue. |
| Lenovo/MotM | We believe this should be discussed after finalizing Proposal 11 and Proposal 13-2 |
| Intel | Support the proposal |
| Apple | We do not fully understand the proposal. Why X CSI can be reported? |
| MediaTek | Support. |
| OPPO | This issue can be discussed later. |
| Ericsson | Support the proposal |
| DOCOMO | Support the proposal. @Apple, X=0,1,2 based on agreed Option 1 in last meeting. |
| vivo | We would like to reuse the CRI definition as Rel-15/16. So our preference is given in Alt.2 as follows. In addition, Alt 1 cannot tell the TRP order for NCJT which is needed to improve performance in our mind. This can be achieved by Alt 2 in terms of reporting a CRI of a good TRP for NCJT. For example, when X=0, Alt 2 can help the network with the fall-back scheduling to STRP transmission from the better TRP. Proposal 15-1: For the UE configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis (i.e. Option 1), - Alt 1: X+1 CRIs are reported, whereas X CRIs are for single-TRP measurement hypotheses and one CRI is for NCJT measurement hypothesis. Each CRI bit size depends on the corresponding number of either valid CMR pairs for NCJT measurement hypothesis or valid CMRs for single-TRP measurement hypotheses. - Alt 2: X+1 CRIs are reported, whereas X CRIs are for single-TRP measurement hypotheses and one CRI is for NCJT measurement hypothesis. CRI bit size depends on the corresponding number of valid CMRs in the CSI-RS resource set, i.e., as legacy CRI bit size definition. - Alt 23: if any. |
| LG | We are fine with Alt1. |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt1. |
| Nokia/NSB | Support Alt 1 |
| Futurewei | We support Alt. 1. |
| NEC | Support the proposal with Alt. 1. |
| Samsung | Support the proposal. |
| InterDigital | Support Alt.1 |

Proposal 15-2:  For the UE be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses (i.e. Option 2),

- Alt 1: Single CRI is reported whereas CRI bit size depends on total number of valid CMR pairs for NCJT measurement hypothesis and valid CMRs for single-TRP measurement hypotheses.

  - FFS further mapping mechanism between each CRI codepoint and  Single-TRP/NCJT measurement hypothesis.

- Alt 2: additional UCI field is needed on top of CRI reporting. The gNB is informed that CRI reporting may correspond to either Single-TRP or NCJT measurement hypothesis.

- Alt 3: reuse RI reporting implicitly, e.g. depending on the number of reported RIs. The gNB is informed that CRI reporting may correspond to either Single-TRP or NCJT measurement hypothesis.

| Company | Comments |
| --- | --- |
| QC | Support Alt1. |
| ZTE | Support Alt1 |
| Lenovo/MotM | Support Alt3 |
| Intel | Alt. 1 |
| Apple | Alt. 1 |
| MediaTek | Support Alt. 3. |
| OPPO | Support Alt 3. |
| Ericsson | Alt. 1 |
| DOCOMO | Fine to further study Alt.1 and Alt.3. |
| vivo | Support Alt2. Same reasoning as Proposal 15-1, i.e., no effort on redefining CRI and able to tell the network the better TRP for NCJT hypothesis. |
| LG | Support Alt. 1 |
| Fraunhofer IIS, Fraunhofer HHI | Support Alt1. |
| Spreadtrum | Support the proposal, and down-selection could be discussed later. |
| Nokia/NSB | Alt 1 |
| Futurewei | We support Alt. 1. |
| NEC | Fine with the proposal. While we think for option 2, CRI reporting should be jointly discussed with other CSIs, as for different hypothesis, the number of RI, CQI, PMI and LI is different. |
| Samsung | Support Alt1. |
| InterDigital | Support Alt. 1. |

- RI and PMI restriction

The RI pairs allowed to report for an NCJT measurement hypothesis should correspond to the layer combinations in DMRS entries [6][8][14]. For example, RI pair of (1, 3) or (3,1) should not be reported because the current DMRS entries do not support layer combination of 1+3 or 3+1.

Considering the companies’ views, the following proposal is suggested:

Proposal 16: Support the indication of following RI combinations by a joint RI field for a NCJT measurement hypothesis in CSI part 1, when the maximal transmission layers is less than or equal to 4:

- {1, 1}, {1, 2}, {2,1}, {2,2}

- FFS: CBSR and/or RI restrictions per TRP or across TRPs

| Company | Comments |
| --- | --- |
| QC | Support. |
| Lenovo/MotM | Support |
| Apple | We are fine |
| MediaTek | Support. |
| OPPO | Support |
| Ericsson | Support |
| DOCOMO | Support. |
| vivo | We are OK. |
| Fraunhofer IIS, Fraunhofer HHI | Support. |
| Nokia/NSB | Support |
| Futurewei | Support. |
| NEC | Support. |
| Samsung | Support |
| InterDigital | Support the proposal. |

### 3.1.3 CSI processing criteria

In the CSI calculation for one NCJT measurement hypothesis, UE needs to measurement CSI based on 2 CMRs and the corresponding IMR. More calculation complexity is required comparing the CSI calculation for one single-TRP measurement hypothesis. Companies (OPPO [2], Spreadtrum [4], ZTE [7], Nokia, Nokia Shanghai Bell [17], Qualcomm [14],) have considered the occupation of CPU and active CSI-RS resource/ports for NCJT hypothesis. OPPO, Nokia, Nokia Shanghai Bell, ZTE and Qualcomm propose that for a NCJT hypothesis, the number of occupied CPUs should be correspond to the number of PMI calculations, i.e., 2. Spreadtrum propose the number of occupied CPUs depends on the CSI report configuration i.e., CSI report option 1 with X = 0, or 1, or 2 or CSI report option 2.

In addition, Qualcomm proposes that two active NZP CSI-RS resources and total number of active ports are occupied for a NCJT hypothesis. ZTE propose that if a CMR appears twice no matter in the same or different settings, for MTRP or STRP, it should be calculated twice to count active resources and active ports.

Based on the majority view, the following proposal is suggested:

Proposal 17:  An NCJT CSI hypothesis is assuming to occupy two CPUs, two active NZP CSI-RS resources, and a number of active ports corresponding to both CMRs.

- If a NZP CSI-RS resource is referred X times by CMR pairs for NCJT measurement hypothesis and CMR for Single-TRP measurement hypothesis, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times for active resources and active ports.

| Company | Comments |
| --- | --- |
| QC | Support. Since the discussion is for single-DCI, we suggest to say “At least for single-DCI based multi-TRP”. For multi-DCI (if agreed), resource/port counting may require further discussions as two PDSCHs are assumed for the purpose of CSI computation. |
| ZTE | Support. To QC’ comment, we think it is better not to tie CSI measurement with PDSCH transmission schemes. |
| Lenovo/MotM | Should be deferred until more clarity is provided on Proposals 13-1, 13-2, on whether/how a CMR for NCJT hypothesis can be reused for another NCJT or single-TRP hypotheses |
| Intel | OK |
| Apple | Support |
| MediaTek | Agree with Lenovo/MotM that it depends on whether a CMR for an NCJT hypothesis can be reused for another NCJT or single-TRP hypotheses. |
| OPPO | Support |
| Ericsson | Ok |
| DOCOMO | OK |
| vivo | Support |
| LG | We are fine with the proposal. |
| Spreadtrum | Fine |
| Nokia/NSB | Ok |
| Futurewei | We support FL’s proposal. |
| NEC | Fine with the proposal. |
| Samsung | Support |
| InterDigital | Support the proposal. |

## CSI Reporting Enhancements for Multi-TRP

### 3.2.2 UCI design

With respect to UCI of CSI report for NCJT enhancement, companies (Huawei, HiSilicon, China Unicom [1], OPPO [2], ZTE [7], Fraunhofer IIS, Fraunhofer HHI [10], Ericsson [20] and DoCoMo [21]) provide the design based on 2-part CSI report. CSI part 1 is with the fixed payload determined by pre-configured CSI parameters. The payload of CSI part 2 can vary depending on reported content of CSI part 1.

Aim to the large CSI feedback overhead under CSI report option 1 with X=2, Ericsson proposes that RI/PMI of the NCJT CSI and single-TRP CSIs are shared.

Based on the majority view, the following proposal is suggested:

Proposal 18:  A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications:

- Within CSI part 1

  - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details

- Within CSI part 2:

  - FFS further compression/omission  for PMI reporting among Single-TRP and NCJT hypotheses

| Company | Comments |
| --- | --- |
| QC | Support in principle. Seems that the second bullet is not needed unless if we are a bit more specific. |
| ZTE | We are OK for this proposal. For CSI part 2, PMI should be included |
| Lenovo/MotM | Support. Agree with ZTE’s comment |
| Intel | Support the proposal |
| Apple | We need more discussion. |
| MediaTek | Support in principle. We need to consider jointly CSI for single-TRP hypothesis at least for Option 2. |
| OPPO | Agree with the proposal with PMI and CQI in part 2. |
| Ericsson | Although the last FFS includes compression for PMI reporting among single-TRP and NC-JT CSI reports, it doesn’t capture our proposal of also sharing RI among single-TRP and NC-JT CSI measurement hypotheses. We suggest to include this as a separate FFS bullet as suggested below: Revised Proposal 18: A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications: - Within CSI part 1 - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details - Within CSI part 2: - FFS further compression/omission for PMI reporting among Single-TRP and NCJT hypotheses - FFS whether RI/PMI can be shared between NCJT CSI and single-TRP CSIs to reduce CSI feedback overhead. |
| DOCOMO | Support in principle. Do not understand the FFS for CSI part 2. Do not support the newly added FFS by E///. |
| vivo | We think that Option1 and Option2 may have different design according to Proposal 15. Option1 does not need to distinguish the STRP hypothesis from the NCJT hypothesis, while Option2 does. Therefore, Option1 and Option2 can be designed separately to optimize their respective UCI payloads. Proposal 18: A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications: - Within CSI part 1 - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details - Note: additional field if it is used to distinguish STRP and NCJT for Option 2. - Within CSI part 2: - FFS further compression/omission for PMI reporting among Single-TRP and NCJT hypotheses |
| Fraunhofer IIS, Fraunhofer HHI | Support. Second bullet is too generic and not needed. |
| Spreadtrum | In principle, fine with the proposal |
| Nokia/NSB | Support |
| Futurewei | Support FL’s proposal in principle. |
| NEC | Fine with the proposal. |
| Samsung | Support the proposal. We also agree with PMI and CQI in part 2. |
| InterDigital | Support the proposal. |

### 3.2.3 Priority and omission rules for CSI

Companies (ZTE [7], MediaTek [8], Qualcomm [14], and Ericsson [20]) have considered the CSI omission rule, which is summarized as following:

ZTE [7] propose to reuse the rule in Rel-15/16 to omit a portion of Part 2 CSI.

MediaTek[8] and Qualcomm [14] propose that for CSI Option 1 with X=1 and X=2, the CSI(s) for single-TRP measurement hypothesis and the CSI for NCJT measurement hypothesis have the different priority. Specifically, the CSI for single-TRP measurement hypothesis has the higher priority for CSI Option 1 with X=1. For CSI Option 1 with X=2, Qualcomm proposes that 2 CSIs for single-TRP measurement hypothesis have the higher priority while MediaTek proposes to adopt the following priority order: best single-TRP → best NCJT → 2nd best single-TRP

Ericsson [20] proposes that CSI for NCJT measurement hypothesis might be omitted by UE when the rank associated with at least one TRP is greater than 2.

Considering the diverse views, the following proposal is suggested:

Proposal 19: For the UE configured to report X CSIs (at least when X>0) associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis, study following alternatives for potential CSI omission/priority:

- Alt 1: Prioritize CSI with different measurement hypotheses within the single CSI report, when the UE is configured with CSI Option 1 with X=1 or 2.

- Alt 2: Prioritize single-TRP CSI than NCJT CSI within the single CSI report, if the rank associated with a single-TRP CSI is greater than 2.

| Company | Comments |
| --- | --- |
| QC | Support Alt1. Is Alt2 even feasible? gNB does not know the RI. Hence, it cannot know which / how many CSIs are removed. Should the gNB try all combinations by blind decoding? |
| ZTE | Study of the options are fine for us. However, what is the motivation if we adopt the priority. It is better to list the referring section of 38.214, or 38.212. |
| Lenovo/MotM | Support Alt1. Agree with QC’s comment. RI is reported by UE, Alt 2 may cause ambiguity in UCI |
| Intel | In our tdoc we proposed to support omission of NCJT CSI in part 2 depending indicated via CRI or RI or CQI in part 1. It is needed for the cases where NCJT provides poor performance (i.e. CQI=0) or NCJT is worse comparing to the sTRP on the same CMR. Thus, we propose to add the following FFS to the proposal. - FFS: omission of NCJT CSI in CSI part 2 depending on the corresponding CRI or RI or CQI in CSI part 1 |
| Apple | UCI omission rule should be discussed at much later stage. |
| MediaTek | Support Alt. 1. |
| OPPO | It can be discussed later. |
| Ericsson | To clarify, what we have in mind is similar to that mentioned by Intel. In our proposal, the omission of NC-JT CSI in part 2, depends in the RIs indicated in part 1. In our tdoc, we showed results that when the rank of single TRP is 4, NC-JT performs poorly compared to single-TRP PDSCH scheduling. Hence, we suggest to replace the current Alt 2 with the FFS added by Intel: Revised Proposal 19: For the UE configured to report X CSIs (at least when X>0) associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis, study following alternatives for potential CSI omission/priority: - Alt 1: Prioritize CSI with different measurement hypotheses within the single CSI report, when the UE is configured with CSI Option 1 with X=1 or 2. - Alt 2: Prioritize single-TRP CSI than NCJT CSI within the single CSI report, if the rank associated with a single-TRP CSI is greater than 2 Omission of NCJT CSI in CSI part 2 depending on the corresponding CRI or RI or CQI in CSI part 1. |
| DOCOMO | More discussion is needed. |
| vivo | Support the FL’s proposal. |
| LG | Support Alt1. |
| Spreadtrum | In Rel-15/Rel-16, for one single CSI report, priority is only considered for part 2 when omission occurs. Thus, we prefer to firstly discuss the content of part 2, then later discuss the priority issue. |
| Nokia/NSB | Prefer Alt 1. We don’t think Alt 2 can work in case of CPU count. CSI priority rules are also used to determine which CSIs are not updated in case of CPU overbooking, in which case it’s not possible to have priority rules that are conditional on CRI, RI or CQI in part 1. Alt 1 is preferred because the same priority function can be used for both omissions and CPU overbooking |
| Futurewei | Support Alt. 1. |
| NEC | Support the proposal. |
| Samsung | Support the proposal in principle. We agree with QC’s concern’s on Alt2. Hence, we prefer Ericsson’s revised proposal 19. |
| InterDigital | Support the proposal. |

## Others

Companies are also proposing other enhancements/issues related to Multi-TRP CSI, which can be discussed further once basic CSI measurement enhancement is more or less clarified and agreed by RAN1. So far following views are not converged too much, based on tdoc review.

| Issues | Companies | Views |
| --- | --- | --- |
| The scope of CSI enhancement for MTRP transmission schemes | Vivo[5] | Support CSI enhancement for different single-DCI-based MTRP transmission schemes, including HST-SFN schemes specified in Rel-17. |
|  | CATT [6] | In Rel-16, in addition to 1a, transmission schemes such as 2a/2b/3 and 4 are also supported, and more new transmission schemes are under discussion in Rel-17 for HST scenario. to reflect the actual channel condition in a specific transmission scheme, indication/configuration/report on the transmission scheme assumed for CSI calculation can be considered. |
| Confirming the working assumption on Category 2. | Samsung [15] | On CSI enhancement for multi-DCI based NC-JT, support confirming the working assumption with from RAN1#103-e. |
|  | OPPO [2] | The benefit of CSI enhancement for multi-DCI based M-TRP transmission should be justified. |
| CPU occupation rule | Samsung [15] | Design new CPU occupation rule for dynamic NC-JT CSI report |
| Non-PMI based port selection | CATT [6] | Non-PMI based feedback can be supported for CSI enhancement for M-TRP. |
|  | MediaTek[8] | Non-PMI based port-selection is supported for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting. |

| Company | Comments |
| --- | --- |
| QC | There seem to be enough number of proposals to discuss in this meeting. These additional issues can be discussed in the future if needed. |
| vivo | - Support CSI enhancement for different single-DCI-based MTRP transmission schemes, including HST-SFN schemes specified in Rel-17. - On CSI enhancement for multi-DCI based NC-JT, support confirming the working assumption with from RAN1#103-e. |

# Proposals for Online/Offline Discussion

TBD

# Work Plan

TBD

# References

- 3GPP R1-2102339, Discussion on CSI Enhancements for Rel-17, Huawei, HiSilicon, China Unicom, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2102384, CSI enhancement for M-TRP and FDD reciprocity, OPPO, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2102438, Further Discussion on CSI Enhancements for NCJT MTRP, InterDigital, Inc., RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2102447, Discussion on CSI enhancement for multi-TRP and FR1 FDD reciprocity, Spreadtrum Communications, Inc., RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2102512, Further discussion and evaluation on MTRP CSI and partial reciprocity, vivo, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-R1-2102604, Discussion on CSI enhancements for mTRP and FDD reciprocity, CATT, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-R1-2102666, CSI enhancements for Multi-TRP and FR1 FDD reciprocity, ZTE, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-R1-2102679, CSI enhancement for NCJT and FR1 FDD reciprocity, MediaTek Inc., RAN1#104bis-e, E-meeting, April 12th – 20th, 2021

- 3GPP R1-R1-2102766, CSI enhancement for multi-TRP and FDD, FUTUREWEI, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021

- 3GPP R1-R1-2102810, CSI enhancements on Type II PS codebook and multi-TRP, Fraunhofer IIS, Fraunhofer HHI, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021

- 3GPP R1-2102883, Enhancements on CSI reporting for Multi-TRP, CMCC, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103020, On CSI enhancements for MTRP and FDD, Intel Corporation, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103094, Views on Rel-17 CSI enhancement, Apple, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103156, CSI enhancements: MTRP and FR1 FDD reciprocity, Qualcomm Incorporated, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103227, Views on Rel. 17 CSI enhancements, Samsung, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103293, Additional considerations on CSI enhancements, Sony, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103371, Enhancement on CSI measurement and reporting, Nokia, Nokia Shanghai Bell, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103510, CSI enhancements for Rel-17, LG Electronics, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103526, Discussion on CSI enhancement for multi-TRP, NEC, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103543, CSI enhancements for Multi-TRP and FR1 FDD reciprocity, Ericsson, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103565, Discussion on CSI enhancements, NTT DOCOMO, INC., RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

- 3GPP R1-2103607, CSI enhancements for multi-TRP and FDD reciprocity, Lenovo, Motorola Mobility, RAN1#104bis-e, E-meeting, April 12th – 20th, 2021.

# Appendix

- Companies’ proposals on CSI enhancements for FDD

Table A-1 	Companies’ proposals on CSI enhancements for FDD

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon, China Unicom | Proposal 1: Binomial coefficient can be used for port-selection for W1 in R17 PS CB. Proposal 2: Polarization-common based free-selection should be supported for W1. Moreover, 4, 8, 12, 16, 24 and 32 can be considered as candidate values of K1. Proposal 3: FD bases used for Wf quantitation are limited within a DFT window with size of N, which can be configured/indicated by gNB. Proposal 4: N=2, Mv=1 and 2 should be supported for R17 PS CB. When N=2 and Mv=1, UE can report selected FD components within the set configured by gNB. Proposal 5: Larger R should be supported for R17 PS CB, with R= 2, 4, 8, 16. Proposal 6: Polarization-common based bitmap for W2 should be supported for R17 PS CB. Considering that gNB can implement compression implicitly, β = 1 should be supported for R17 and other candidate values can be considered as well. Proposal 7: Option 1 + Option 2 or Option 1 + Option 3 should be supported to improve utilization of CSI-RS. |
| OPPO | Proposal 1: Support Polarization-common selection for Rel-17 PS. Proposal 2: For Wf, consider - For M = 1, Wf is fixed all-one vector and not reported - For M> 1, N=Mv , i.e., Wf comprises M consecutive FD vectors Proposal 3: Support 0.25 density CSI-RS for Rel-17 codebook |
| Spreadtrum Communications | Proposal 14: For the port selection matrix W1, support polarization-common design. Proposal 15: Confirm the working assumption that Mv>1 is a UE optional feature. Proposal 16: For FD basis selection, support configuring a single window to the UE by RRC signaling. Proposal 17: For FD basis selection, support selecting Mν components within a window if Mν<N. Proposal 18: Support Option 0: No further CSI-RS enhancement as the baseline. |
| vivo | - Proposal 10: - gNB indicates one FD set with K starting tap indexes and a common length M, where K=1,2,4,8 and M=1,2,4. K=1, M=1. There is no tap indication, i.e., tap 0 for measurement, and all the SD-FD pairs are precoded on CSI-RS ports. K=1, M>1. Partial SD-FD pairs are precoded on CSI-RS ports and other FD bases are indicated by gNB. K>1, M=1. More than one SD-FD pair is precoded on one CSI-RS port and the delay location of each SD-FD pair per CSI-RS port is indicated by the tap indication. K>1, M≥1. Only SD bases are precoded on CSI-RS ports and all the FD bases are indicated by gNB. - Proposal 11: More values of Mv can be supported and larger value of Mv can be considered. Mv=1,2,4 should be supported and Mv=6 can be further studied. - Proposal 12: Polarization-common CSI-RS port selection is preferred. - Proposal 13: To reduce the complexity of UE to counteract timing mismatch, gNB can map CSI-RS ports with a predetermined order or provide information on which ports can be used for timing calibration. Based on the order or the information, UE can process timing calibration with much less complexity without loss of timing calibration accuracy. |
| CATT | Proposal-1: For M_v=1, the location of non-zero coefficients are indicated by the port selection indication. Proposal-2: Port selection should be polarization-common. Proposal-3: If DFT basis vectors are to be configured to UE, they are limited within a window of size N. Proposal4: Dynamic indication of M_initial should be considered in order to flexibly shift the delay position. Proposa-5: The number of FD components and the selected FD components are to be reported by UE. Proposal-6: In addition to M_v=1, only M_v=2 is supported. Proposal-7: At least 48 SD-FD pairs shall be supported in specification. Proposal-8: Opiton3 or the combination of both Option1 and Option3 is supported. |
| ZTE | Proposal 10: Support polarization-common W1 in Rel-17 PS codebook. Proposal 11: Reuse the Rel-16 non-zero coefficient quantization approach for the amplitudes and phases in W2 of Rel-17 PS codebook. Proposal 12: - - The set of candidate vectors of Wf is a window configured by gNB, where both the window size and M_initial are configured. - - UE selects and reports the Wf vectors within the window configured by gNB. Proposal 13: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook. |
| MediaTek Inc. | Proposal 14: Polarization specific port selection codebook utilizing UL/DL reciprocity of angle and delay should be supported in Rel-17. Proposal 15: M_v=1 implies that W_f has been turned off by the gNB and the UE reports a frequency independent precoder W=W_1 W_2. In this case, W_f∈C^(N_3× 1) is a column vector of all 1s. Proposal 16: Support the configuration of M_v>1 by means of RRC signaling, and indicate the M_v candidate DFT FD basis vectors to the UE via MAC-CE signaling. Support candidate values of M_v between 2 to 4. |
| Fraunhofer IIS, Fraunhofer HHI | Proposal: Support at least one value of M in the range of [2,4] Proposal: For the Rel. 17 PS CB, the size of the window is equal to the number of delays M_v. Proposal: Support fixing or configuring the starting index of window M_init to be zero. Proposal: Support R=2 for Rel. 17 PS codebook. Proposal: Support polarization common port selection for W1. Proposal: Support option 0 – no further CSI-RS enhancements. |
| Intel | Proposal 4: - Support Mv = 2 frequency-domain (FD) vectors in Wf matrix - Wf is the same across all layers (layer-common) - Reporting of FD vectors used in Wf should be considered Proposal 5: - Support PMI subband size equal to the 1/D PRB, where D is CSI-RS density (measured in 1/PRB) Proposal 6: - Support polarization-common spatial domain (SD) vectors selection for W1 matrix Proposal 7: - Support reporting of selected CSI-RS ports by using index of combination - Support the same coefficient quantization design as for Rel. 16 Type II codebooks - Support the same partial CSI omission on PUSCH design as for Rel. 16 Type II codebooks - Consider reporting of combination index for selection of K0 coefficients Proposal 8: - Support 0.25 CSI-RS density in Rel. 17 - New CSI-RS pattern with CSI-RS ports distributed over several PRB can be considered |
| Qualcomm | Proposal 8: For Rel-17 FDD CSI, confirm the WA made in RAN1 #103e to an agreement that supporting M > 1 as optional feature with additional capability signalling. Proposal 9: For Rel-17 FDD CSI, when M > 1, support the following for Wf: - M=2 and Wf is comprised of FD basis 0 and 1. - No UE reporting Proposal 10: For Rel-17 FDD CSI, support R=1 only. Proposal 11: For Rel-17 FDD CSI, support polarization-specific port-selection. Proposal 12: For Rel-17 FDD CSI, support following for linear combination coefficient reporting. - Configuration of max ratio of non-zero coefficients per layer, β, max number of non-zero coefficients per layer is K1Mβmax number of non-zero coefficients across all layers is 2K1Mβ. - UE reporting of actual number of non-zero coefficients - UE reporting of location of non-zero coefficients per layer via bitmap - Individual amplitude and phase quantization for non-zero each coefficient Proposal 13: For Rel-17 FDD CSI, no CSI-RS enhancement is needed. |
| Sumsung | Proposal 6: Regarding Rel. 17 codebook components, - Polarization-common port selection is baseline; - Polarization-specific port selection can be considered if it can achieve superior performance-overhead tradeoff - The Mv value(s) should be determined based on the tradeoff among UE complexity, CSI overhead, potential-specification impact, and the UPT performance for small as well as large number of CSI-RS ports. - The supported Mv value(s) should decrease as number of CSI-RS ports increases. - When Mv>1, one of the columns of the Wf matrix should be an all-one vector. - Confirm working assumption on Wf being UE optional - Support R=1, and additional value(s) of R should be studied based on tradeoff among UE complexity, CSI overhead, potential-specification impact, and the UPT performance. - The Wf component is configured to the UE. - The length of the all-one vector (when Wf is turned OFF) depends on the reporting format (e.g. WB/SB, SB size etc.), and hence should be discussed with the reporting format discussion. - Regarding codebook parameters - The value of P (number of rows in W1) can be discussed together with the CSI-RS related study. - Study the value(s) of L from {1,2,3,4,6,8} - Study the value(s) of beta from {1,3/4,1/2} Proposal 7: Regarding CSI-RS related enhancement, - Support option 0 (no enhancement) as baseline - Down-select to Option 3 for further study, and the study should consider aspects in Observation 1 |
| Sony | Proposal 1. If further CSI-RS enhancements are needed for CSI-RS associated with Rel-17 PS codebook, then prefer Option 1. If necessary, Option 1 can be combined with Option 4. Proposal 2. For minimum specification impact, maintain the polarization-common base selection and reporting mechanism of Rel-15/Rel-16. A polarization-specific mechanism should only be introduced if it can be shown that, at least for some scenarios of interest, it provides substantial advantage over polarization-common. Proposal 3: Based on UL CSI, further restrict the set of CSI-RS ports eligible by the UE to those compatible with UL signal angles. Proposal 4: Introduce an FD sampling size parameter d'. Based on UL CSI, further restrictions to d' can be applied in order to limit the set of FD DFT vectors eligible by the UE. |
| Nokia, Nokia Shanghai Bell | Proposal 1. Support Option 1 and Option 2 with a new single pattern for 32-port CSI-RS resources occupying 4 subcarriers and 8 symbols per RB carrying the RS, such that the maximum number of 32-port resources that the network can accommodate per slot is increased to 3/d, for density d=1,12,14. Proposal 2. Support a minimum configurable CSI-RS density depending on the value of R, given by dmin=maxRNPRBSB,14, for R=1,2,4,8. Proposal 3. Regarding the network configuration of Wf, support RRC configuration of a single window of N FD components, i.e., a UE is configured to measure the N FD components in the window for the selected ports. Proposal 4. Regarding candidate values of Mν and N, consider the combinations N=2, Mν=2 and N=1,2,4 for Mν=1. Proposal 5. Support polarisation-common port selection of K1/2 out of P/2 CSI-RS ports. Proposal 6. Support combinatorial indication of the K1/2 out of P/2 selected ports for W1. Proposal 7. Regarding the candidate values for K1, consider the same value set as for P, i.e., P,K1∈{4,8,12,16,24,32} with K1≤P. Proposal 8. Regarding the indication of the W2 bitmap, further evaluate the two alternatives: polarisation-common bitmap, of size K12×Mν⋅ν, and polarisation specific bitmap, of size K1×Mν⋅ν. Proposal 9. Regarding the maximum number of NZC per layer, K0, reuse the definition from Rel-16, such that K0=βK1M1. Further study additional candidate values for β.Regarding the maximum number of NZC per layer, K0, reuse the definition from Rel-16, such that K0=βK1M1. Further study additional candidate values for β. |
| LG Electronics | Proposal #5: If enhancement is needed, consider option 1(a lower CSI-RS density per CSI-RS resource) for lower CSI-RS overhead in Rel-17. Proposal #6: Confirm the working assumption. - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters |
| Ericsson | Proposal 1 Support polarization-specific port selection. Proposal 2 Support Mv=1 and 2, larger value of Mv is not needed. Proposal 3 RAN1 should consider supporting large R values, for example R=NSBCQI, where NSBCQI is the CQI subband size. Proposal 4 Do not support lower CSI-RS density, e.g., ρ=0.25. Proposal 5 Support Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook |
| NTT，DOCOMO， INC | Proposal 7: - It is sufficient to report Mv= 1, 2 FD bases with the agreed Rel. 17 Type II PS codebook structure Proposal 8: - Allow UE to select and report Mv number of FD bases from a DFT bases window identified by the UE (not from a pre-configured set of DFT bases by the NW) Proposal 9: - Consider dynamic configuration of turning on/off Wf using DCI. Proposal 10: - If Mv is small, then there is no need of reporting a bitmap. All the non-zero LC coefficients within W2 can be reported. Further, this helps achieving a unified solution for PMI reporting when Wf is off, to a certain extent, since that requires reporting all the LC coefficients. |
| Lenovo，Motorola Mobility | - No further modification is necessary on the Rel. 17 codebook structure. - Support two frequency compression parameter values for Rel. 17 Type-II port-selection codebook, with M configured to either of the values {1,2}. - A layer-common, window-based approach is used to configure the FD basis indices for all layers, where the set of FD basis indices is contiguous, and whose location and size are higher-layer configured. - Support UE-assisted FD basis indices selection for each layer from a layer-common network-configured window. - Polarization-common port selection and polarization-specific coefficient quantization should be supported for Rel. 17 Reciprocity-based Port Selection codebook. - Support Option 0: No further CSI-RS enhancement as the baseline. - Configure the UE with two frequency compression parameter values for both strong and weak uplink/downlink channel reciprocity, where the UE can select the appropriate parameter value based on the strength of the channel reciprocity. |

- Companies’ proposals on CSI enhancements for Multi-TRP

Table A-2 	Companies’ proposals on CSI enhancements for Multi-TRP

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon, China Unicom | Proposal 8: The maximal value of N is 2 for NCJT measurement hypothesis, when the UE is configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR. Proposal 9: A NZP CSI-RS resource set configured with Ks CMRs for single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses is associated to a CSI-IM resource set configured with Ks+N CSI-IM resources, - Whereas the first Ks CSI-IM resources are associated with Ks CMRs one-by-one and remaining N CSI-IM resources are associated with N selected CMR pairs one-by-one. Proposal 10: If a CSI-IM resource is associated with a CMR pair, UE shall assume that the CSI-IM resource is QCLed with two CMRs in that pair with respect to ‘QCL-typeD’. Proposal 11: The report quantity is either ‘cri-RI-PMI-CQI’ or ‘cri-RI-LI-PMI-CQI’ if a CSI-ReportConfig is associated with NCJT measurement hypothesis in Rel-17. Proposal 12: For a CSI-RS resource set with Ks NZP CSI-RS resources for sTRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, each of Ks+N CRI codepoints corresponds to one measurement hypothesis and total bitwidth for CRI in UCI is log2(Ks+N). Proposal 13: 1-part CSI report is not supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis. Proposal 14: For a 2-part CSI report associated with NCJT measurement hypothesis: - For Option 1, CSI report part 1 comprises - One CRI, two RIs, one WB CQI, and SB CQIs for NCJT hypothesis and - X CRI, X RI, X WB CQI, and SB CQIs corresponding to each WB CQI, for X sTRP hypotheses respectively (if applicable) - For Option 2, CSI report part 1 comprises - One CRI, one or two RIs, one WB CQI, SB CQIs - The remaining CSI are in CSI report part 2. |
| OPPO | Proposal 4: For CSI measurement associated to single CSI reporting setting for NCJT, - It is preferred to support only Ks=2 CMRs and N=1 for FR1 and FR2. Whether CRI is needed depends on the configured reporting mechanism. - If Ks>2 CMRs is supported in a CSI-RS resource set, the maximal value of N and Ks are Nmax= 2 and Ks,max= 4. - If Ks>2 CMRs is supported in a CSI-RS resource set, support K1=K2 only and implicit CMR pairing between CMR groups for NC-JT measurement. - If Ks CMRs are configured in a CSI-RS resource set, (2N+Ks) CPUs are counted for the CSI report (Ks CPUs for single-TRP and 2N CPUs for NCJT measurement hypothesis). Proposal 5: For Option 2, the recommended measurement hypothesis can be reported via one of - Alt.1: individual one bit, to indicate S-TRP or NC-JT measurement hypothesis - Alt.2: CRI, to select one CMR from Ks CMRs or CMR pair from N CMR pairs - Alt 3: RI(s), to indicate one or two rank values Proposal 6: For Option 2 and type 1 codebook, - The recommended measurement hypothesis is reported via CSI part 1 - CRI, two RIs and CQI for first CW are reported via CSI part 1. - One or two PMI(s) corresponding to the reported hypothesis, possible CQI for second CW are reported in CSI part 2. Proposal 7: For CSI for NC-JT hypothesis in the CSI report of Option 1 and type 1 codebook, - CRI, two RIs and CQI for first CW are reported via CSI part 1 similar to Option 2. - Two PMIs, possible CQI for second CW are reported in CSI part 2. Proposal 8: The benefit of CSI enhancement for multi-DCI based M-TRP transmission should be justified. |
| InterDigital | Proposal 1: Support N=1 and Ks=2; no other values are considered. Proposal 2: For NCJT CSI measurement configured with single reporting setting, support interference measurement based on NZP CSI-RS. Proposal 3: Study two-step CSI-RS measurement reporting for NCJT where - NZP CSI-RS is configured per TRP, - in the first step, a PMI corresponding to the first TRP, and in the second step a PMI corresponding to the second TRP is determined and reported. Proposal 4: Study a two-step SRS plus CSI-RS measurement/reporting for NCJT where - NZP CSI-RS is configured per TRP, - in the first step UE transmits an SRS, and in the second step based on the received precoded CSI-RS from each TRP, UE estimates and report the CSI |
| Spreadtrum | Proposal 1: Not support N> 1 and Ks > 2. Proposal 2: Not support interference measurement based on NZP CSI-RS given by nzp-CSI-RS-ResourcesForInterference for a CSI report associated with NCJT measurement hypothesis. Proposal 3: Support interference measurement based on CSI-IM given by csi-IM-ResourcesForInterference for a CSI report associated with NCJT measurement hypothesis. Proposal 4: For option 1 with X=0, the occupied CPUs could be OCPU,1, - OCPU,1∈[Ks,Ks] , where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement - FFS: the specific value of OCPU,1 Proposal 5: For option 1 with X=1, the occupied CPUs could be OCPU,2, - OCPU,2∈[1.5Ks, 2.5Ks] ,where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement - FFS: the specific value of OCPU,2 Proposal 6: For option 1 with X=2, the occupied CPUs could be OCPU,3, - OCPU,3∈[2Ks, 3Ks] ,where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement - FFS: the specific value of OCPU,3 Proposal 7: For option 2, the occupied CPUs could be OCPU,4, - OCPU,4∈[2Ks, 3Ks] ,where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement - FFS: the specific value of OCPU,4 Proposal 8: Support to introduce new CSI computation delay requirement for NC-JT CSI. Proposal 9: Support to reuse Rel-15 two-part UCI structure for NCJT Proposal 10: For option 1 with X=0, for UCI composition and structure, - 2RI, 1 or 2 CQI(s), 2 LI should be include into Part1; - 2 PMIs (if required) should be include into Part2; Proposal 11: For option 1 with X=1 or X=2, for UCI composition and structure, - Some CSI information for single TRP, e.g., CRI/RI/CQI for the first CW, should be placed into Part 1; - Some CSI information for single TRP, e.g., PMI, CQI for the second CW(if reported), and CSI information for NCJT should be placed into Part 2; Proposal 12: For option 2, Study how to demonstrate the validity of CSI parameters for joint reporting in NC-JT. Proposal 13: Support option 2, i.e., for a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report two RIs, two PMIs, two LIs and two CQIs. |
| vivo | RAN1 should prioritize the CSI feedback mechanism with N=1 and Ks =2 and other values of N>1 and Ks>2 can be further studied. CMRs in each CMR group can be used for both NC-JT and STRP measurement hypotheses for FR1 and FR2. For N=1 and Ks=2, support configuring one CSI-IM resource for a MTRP CSI measurement. Support the CSI reporting format/UCI is optimized separately for Option1 and Option2. Support adding a separate field in a CSI report to report recommended hypothesis. Support to minimize payload of part 1 of MTRP CSI report for both wideband and subband reporting considering performance when part2 is completely omitted. Consider different configurations of RI restrictions, codebook subset restriction across TRPs. Support total layers of NC-JT reception no more than 4. Support CSI enhancement for different single-DCI-based MTRP transmission schemes, including HST-SFN schemes specified in Rel-17. |
| CATT | Proposal-10: Non-PMI based feedback can be supported for CSI enhancement for M-TRP. Proposal-11: For CSI reporting based on single report setting, two associated CMR resources in the same resource set are used for channel measurement of two TRPs. In CSI calculation, the UE assumes that in PDSCH transmission, PMI-1/RI-1 and PMI-2/RI-2 are applied to the channel of TRP 1 and 2 respectively. By doing so, inter-TRP interference measurement can be achieved without introducing non-precoded IMR. Proposal-12: Considering the impacts of the two options on spec, option 1 is slightly preferred. - Option 1 (Explicit): CMRs corresponding to different TRPs can be associated with different reporting settings respectively, with the same configurations between two settings except for PUCCH/PUSCH resources and CMR/IMR resources setting(s) Proposal-13: One CQI per codeword is reported even if the reported rank is more than 5 in CSI for NC-JT. Proposal-14: In CSI reporting for NC-JT, the possible combinations of rank reported to each of the TRPs should follow the rule of DMRS allocation. Proposal-15: Indication/configuration/report on the transmission scheme assumed for CSI calculation can be considered. Proposal-16: Further discuss the following alternatives for CSI reporting of M-DCI based NC-JT. - Alt-1: Two independent reports, for different TRPs respectively - Alt-2: One set of report quantities can be reported to any of the two TRPs - Alt-3: Separate reports (i.e., Alt-1) can be used if the resources for CSI reporting towards different TRPs are different. If resources for CSI reporting towards different TRPs are overlapped, joint CSI reporting (i.e., Alt-2) can be used. |
| ZTE | Proposal 1: In FR2, one of the following two options should be adopted - Option 1: a CMR configured for a MTRP CMR pair cannot be used for another MTRP CMR pair in FR2. - Option 2: if a CMR x is paired with a CMR m, and also paired with another CMR n, CMR m and CMR n should be quasi co-located with QCL-TypeD. Proposal 2: In FR2, a CMR configured for a MTRP CSI should not be used for a STRP CSI. Proposal 3: Two bitmaps should be configured by RRC signaling for MTRP CMR pair selection and STRP CMR selection respectively. Proposal 4: For CSI report option 1, X+1 CRI should be reported, where X CRIs are for STRP and 1 CRI is for MTRP CSI, each CRI bit size depends on the corresponding number of valid CMR pairs for MTRP or valid CMRs for STRP. Proposal 5: For CSI report option 2, 1 CRI should be reported, where the CRI bit size depends on the total number of the valid CMR pairs for MTRP and valid CMRs for STRP. Proposal 6: The number of occupied CPU should be OCPU=2N+T, where N is the number of valid CMR pairs for MTRP and T is the number of valid CMRs for STRP. Proposal 7: If a CMR appears twice no matter in the same or different report settings, for MTRP or STRP, it should be calculated twice to count active resources and active ports. Proposal 8: For CSI report option 1, each of X+1 CSI should be divided into CSI part 1 and CSI part 2 following Rel-15/16 mechanism. Based on the above proposal, the mapping order of CSI field should be specified accordingly. Proposal 9: Reuse Rel-15/16 CSI omission rule for CSI part 2 as follows, where smaller number for priority level has higher priority. - Priority level 0 includes all part 2 wideband CSI for X STRP and one MTRP; - Priority level 1 includes all part 2 subband CSI of even subbands for X STRP and one MTRP; - Priority level 2 includes all part 2 subband CSI of odd subbands for X STRP and one MTRP. |
| MediaTek | Proposal 1: Support that the number of CMRs in two groups can be different, i.e., K1≠K2. Proposal 2: In addition to (1, 2), support (N, Ks)= (2, 3), (3, 4), (4, 4) up to UE capability. For Ks=4, both K1,K2=3, 1 and K1,K2=(2, 2) can be supported. Proposal 3: Ks>4 is configurable and MAC-CE indicates which of M≤4 single-TRP measurement hypotheses should be performed by the UE and which of N≤4 NCJT measurement hypotheses should be considered by the UE. Proposal 4: For CSI reporting for NCJT, FR2 optimization is not further considered in R17. Proposal 5: For NCJT CSI measurement configured with single reporting setting, only support interference measurement based on CSI-IM. Proposal 6: How to interpret the two CMRs configured for an NCJT measurement hypothesis can be up to UE implementation. Proposal 7: For an NCJT interference hypothesis, the corresponding CRI is associated with two CMRs, whereas the mapping from CRI to IMR remains one-to-one. Proposal 8: NZP IMR can be configured for single-TRP measurement hypotheses in a CSI reporting setting with NCJT CSI measurement. Proposal 9: For Option 1 with X = 1, the CSI associated with the best NCJT measurement hypothesis has a lower reporting priority than the CSI associated with the best single-TRP measurement hypothesis. Proposal 10: For Option 1 with X = 2, the following priority order should be adopted: best single-TRP → best NCJT → 2nd best single-TRP. Proposal 11: For Option 2, the second RI can be reported as 0 to signal the best single-TRP measurement hypothesis. Proposal 12: The allowed RI pairs for an NCJT measurement hypothesis assuming the maximal transmission layers less than or equal to 4 should be (1, 1), (1, 2), (2, 1), (2, 2). Proposal 13: Non-PMI based port-selection is supported for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting. |
| Futurewei | Proposal 1: FeMIMO supports that CMR for a S-TRP measurement hypothesis is selected by UE from the CMRs belonging to the CMR pairs configured for NCJT measurement hypotheses. Each of the NZP CSI-RS resources in one of the two CMR groups that has been configured for NCJT measurement hypotheses can be selected by the UE as CMR for the S-TRP measurement hypotheses, whereas each of the NZP CSI-RS resources in the other CMR groups that has been configured for NCJT measurement hypotheses will then be selected as the NZP CSI-RS for interference measurement (IM). Proposal 2: FeMIMO supports that a CSI-IM resource is associated with a CMR pair configured for a NCJT measurement hypothesis. This CSI-IM resource can also be associated with the S-TRP measurement hypothesis defined based on the CMRs belonging to the CMR pairs configured for NCJT measurement hypotheses. Proposal 3: Regarding UE reporting mechanism, FeMIMO supports Option1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 |
| Fraunhofer IIS, Fraunhofer HHI | Proposal: Support option 0 – no further CSI-RS enhancements.Proposal: A bitmap of size K1K2 is used to indicate the CMR pairs associated with the N NCJT measurement hypotheses. Proposal: A one-to-one mapping between CRI codepoints and CMRs/CMR pairs is used. Each CRI codepoint is associated with a single-TRP or NCJT measurement hypothesis. Proposal: A two-part UCI structure is adopted for option 1 and option 2. - For option 2, UCI part 1 comprises one CRI, two RIs, a wideband CQI and sub-band CQIs, - For option 1, UCI part 1 comprises X CRIs, X RIs and X wideband CQIs and the sub-band CQIs for the X selected single-TRP measurement hypotheses (if configured), and one CRI, two RIs, one wideband CQI and the sub-band CQIs for the selected NCJT measurement hypothesis. |
| CMCC | Proposal 1: The two NZP CSI-RS resources in one CMR pair shall be determined from two different CMR groups. Proposal 2: Some CMRs in the CMR group can’t be used for both NCJT and Single-TRP measurement hypotheses in FR2. Proposal 3: One CMR can’t be configured within different CMR pairs in FR2. Proposal 4: Support Option 1 (the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis) for single CSI report setting. |
| Intel | Proposal 1: - Support number of CMR Ks > 2 and number of CMR pairs N > 1 for a CSI report - Detailed design of RRC signaling for CMR configuration can be discussed in RAN2 considering functionality agreed in RAN1 - Selection of CMR or CMR pairs at the UE is indicated via CRI Proposal 2: - Strive to minimize standardization efforts to support for CSI for multi-DCI based NCJT Proposal 3: - Support omission of CSI for NCJT measurement hypothesis in CSI part 2 - There is no need to explicitly specify criterion for NCJT measurement hypothesis omission in RAN1 - Omission of NCJT measurement hypothesis is indicated in CSI part 1 by using CRI/RI or CQI fields |
| Apple | Proposal 1 For a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report - Two RIs, two PMIs, two LIs and two CQIs Proposal 2 For interference measurement under NCJT, CMR, including RI/PMI decision, from one TRP should be considered as the interference, i.e. IMR, to the other TRP. Proposal 3 For reporting mechanism, regarding the following three cases - Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked - Single TRP operation: Report each TRP under the assumption that the other TRP is blanked - Multiple TRP operation: Report both TRP under NCJT operation gNB can configure one or multiple of them - UE can independently indicate whether UE supports each reporting or not - The CPU and active RS counting rule needs to be further discussed and clarified |
| Qualcomm | Proposal 1: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT: - K=K1=K2 is assumed with the maximum value of K equal to 4. The maximum value of N that spec supports is equal to 2 or 4, and is further based on UE capability. - Within the reporting setting CSI-ReportConfig, at least support RRC configuration of - A bitmap to indicate N CMR pairs out of K2 possible CMR pairs - A flag to enable/disable all CMRs that appear in a configured CMR pair as indicated by the bitmap - UE can indicate whether it supports reusing a CMR between a single-TRP hypothesis and a NCJT hypothesis or between two NCJT hypotheses in a reporting setting. Proposal 2: In a CSI report config, CRI codepoint mapping to CSI hypotheses is based on - CRI codepoints are first mapped to single-TRP hypotheses. The number of such codepoints is determined based on the number of CMRs across both CMR groups and whether the flag enables/disables individual CMRs that are used in a CMR pair. - The additional CRI codepoints are mapped to N CMR pairs corresponding to N NCJT hypotheses. Proposal 3: In a given CSI-ReportConfig, N CSI-IM resources should be configured with a one-to-one mapping to the N CMR pairs (corresponding to N NCJT CSI hypotheses / last N CRI codepoint). - QCL-Type D of the CMRs associated with a NCJT hypothesis are applied to the corresponding CSI-IM resource. - FFS: For a given NCJT hypothesis corresponding to two CMRs, whether 2 additional CSI-IM resources are needed or whether the 2 CSI-IM resources associated with the two CMRs can be reused. Proposal 4: An NCJT CSI hypothesis occupies two CPUs, two active resources, and a number of active ports corresponding to both CMRs. Proposal 5: For Option 1 with X=1 or 2, the order of CSI reports in the UCI as well as CSI priority for CSI omission is based on an order between the two CSI reports associated with the CSI-ReportConfig. CSI priority can be expressed as PriiCSIy,k,c,s,i, where i=0,1,2 corresponds to single-TRP CSI(s) and NCJT CSI. Proposal 6: For a NCJT CSI, the two RI’s and LI’s are based on - One joint RI field in CSI part 1 indicates RI pair from {1+1,1+2,2+1,2+2} rank hypotheses - Introduce a separate RRC configuration for NCJT rank restriction with 4-bit bitmap, which determines the number of allowed rank pairs nRI, NCJT. - The size of the RI field is - When Option 1 is configured: log2nRI, NCJT bits. - When Option 2 is configured: max(log2nRI,,log2nRI, NCJT) bits. - The two LI’s are reported in CSI part 2, which require 2 / 1 / 0 bits depending on the indicated rank pair. Proposal 7: In the NCJT CSI, for subband part of CSI part 2, adopt one of the following alternatives for the order between even/odd subbands versus first/second PMIs: - Alt1: Even and odd subbands of the first PMI are placed first followed by even and odd subbands of the second PMI. - Alt2: Even subbands of the first and second PMIs are placed first followed by the odd subbands of the first and second PMIs. |
| Samsung | Proposal 1: On CSI enhancements for multi-DCI based NC-JT, support both Category 1 and 2, and allow UE to be configured one of two Categories. Proposal 2: On CSI enhancement for multi-DCI based NC-JT, support confirming the working assumption with from RAN1#103-e. Proposal 3: On CSI enhancements for multi-TRP, support CMR to be re-used as IMR for both non pre-coded and pre-coded CSI-RS Proposal 4: For NC-JT CSI reporting enhancement, support and study followings: - Support CRI-based dynamic reporting between NC-JT and non-NC-JT CSI - Support non-PMI based port-selection - Support restrictions among reported RIs or PMIs - Study UCI structure optimized for dynamic NC-JT CSI report Proposal 5: Design new CPU occupation rule for dynamic NC-JT CSI report |
| Nokia, Nokia Shanghai Bell | Proposal 10. Support RRC and MAC-CE indication of N NCJT pairs by means of a bitmap of size K1K2. Proposal 11. Support RRC and MAC-CE indication of the number of CMR resources configured for single-TRP measurements, KsC, with 0≤KsC≤Ks, such that the number of CPUs occupied by the M-TRP report is OCPU=2N+KsC. Proposal 12. Support RRC and MAC-CE indication of the number of single-TRP CSIs, KsR∈{0,1,2}, to be reported in addition to the NCJT CSI. In particular, KsR=KsC=0 configures a UE to report only the best of N NCJT measurements. KsR=0<KsC configures a UE to report a single CSI, which is the best between the N NCJT measurements and KsC single-TRP measurements. Proposal 13. Support extending the CRI with bit width log2(N+KsC), where KsC is the number of CMR resources configured for single-TRP measurement. KsC/2 codepoints are associated to CMR resources from group 1 and KsC/2 from group 2. Proposal 14. Support configurations with the same number of CMR resources in the two groups, i.e., K1=K2. Proposal 15. Support different priority levels for CSIs with single-TRP and NCJT transmission hypothesis, respectively. Study whether priority levels should be fixed or flexible. |
| LG Electronics | Proposal #1: Support up to 8 CMRs for a CSI-RS resource set for NCJT. Proposal #2: For the purpose of NCJT CSI calculation, support an independent ZP IMR corresponding to a CMR pair, in addition to two ZP IMRs for STRP CSI corresponding to two CMRs of the CMR pair, respectively. Proposal #3: UE may assume that the IMR configured for NCJT CSI is QCLed with respect to TypeD with both CMRs. Proposal #4: For a UE that requires a single PTRS port, support single LI report across both TRPs in addition to the agreed per-TRP LI report. |
| NEC | Proposal 1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis, support Option 1 (the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis). Proposal 2: Payload for CRI reporting can be reduced based on the structure of two CMR groups. And for single-TRP measurement hypothesis, CRI payload size should be separately defined in case of different number of CSI (i.e. different values of X). Proposal 3: TRP specific CBSR and RI restriction can reduce the UE complexity considerably, which should be introduced at least for NCJT measurement hypothesis. Proposal 4: Restriction on possible values of the two reported RIs should be considered to reduce the overhead. |
| Ericsson | Proposal 6 For the number of CMR pairs indicated for NC-JT CSI hypotheses, support up to N=4. Proposal 7 Support up to Ks=8 CMR resources in the CSI-RS resource set for a multi-TRP CSI report setting. Proposal 8 In NR Rel-17, the number of CMRs in the two CMR groups, K1 and K2, can be either equal or different. Proposal 9 Support dynamic update of CMR pairs for NC-JT CSI measurement hypothesis using MAC-CE and/or RRC signalling Proposal 10 For NC-JT CSI reports that include single-TRP CSIs (i.e., Option 1 with X = 2), study the CSI overhead-performance tradeoff among the following schemes: -> A scheme where RI/PMI of NC-JT CSI and single-TRP CSIs are reported independently -> A scheme where RI/PMI of the NC-JT CSI and single-TRP CSIs are shared Proposal 11 If the rank associated with at least one TRP is greater than 2, then the UE may omit CSI associated with NCJT measurement hypothesis. Proposal 12 For NC-JT CSI with a single CSI reporting setting , if the NZP CSI-RS resources for channel measurement are configured without QCL-type D or with the same QCL-type D, a UE assumes that the interference on the CSI-IM resources represents two observations of a same interference. Proposal 13 For NC-JT CSI with a single CSI reporting setting, if the NZP CSI-RS resources for channel measurement are configured with different QCL-type D source RS, a UE assumes that the interferences on different CSI-IM resources correspond to different interference sources. Proposal 14 For NC-JT CSI with a single CSI reporting setting, a UE assumes that an NZP CSI-RS or CSI-IM resource for interference measurement is QCLed with respect to “QCL-type D” with the associated NZP CSI-RS resource for channel measurement. |
| DOCOMO | Proposal 1: - Support K1=K2. Support maximal values of Ks>2 and N>1. - At least the maximal number of CMR resources supported in Rel-16 should be supported for Ks. - At least N=Ks/2 can be supported. Fine to support N=K1*K2 for FR1. - A K1*K2 bitmap can be configured by RRC signaling to indicate the CMR pairs for CSI measurement. If the bitmap is not configured, UE assumes one-to-one mapping between the two CMR groups as CMR pairs for CSI measurement. - For FR2, UE does not expect a CMR resource to be indicated in more than one CMR pair via the bitmap. Proposal 2: - Support to configure a CSI-IM resource for each CMR for single-TRP measurement hypotheses, and a CSI-IM resource for each indicated CMR pair for NCJT measurement hypotheses, i.e., (K1+K2+N) CSI-IM resources. - If configured by a new RRC parameter, each CMR resource within a CMR pair should be assumed as NZP interference when deriving CSI based on the other CMR resource within the CMR pair. Proposal 3: - UE reports one CRI to indicate a beam pair for NCJT measurement hypotheses. - If UE is configured to report CSIs for both single-TRP measurement hypotheses and NCJT measurement hypothesis in a CSI report, the CRI reporting index from 0 to K1+K2-1 corresponds to single-TRP measurement hypotheses, and the CRI reporting index from K1+K2 to K1+K2+N-1 corresponds to the CMR pairs for NCJT measurement hypotheses. - If UE is configured to report CSI for NCJT measurement hypothesis only in a CSI report, the CRI reporting index from 0 to N-1 corresponds to the CMR pairs for NCJT measurement hypotheses. Proposal 4: - Consider new index indicating single-TRP and NCJT measurement hypotheses, as well as TRP index information (e.g., CMR group index) for single-TRP measurement hypotheses in CSI priority rule. Proposal 5: - Support to configure reporting a new indicator in a CSI to indicate the measurement hypothesis of the reported CSI. Proposal 6: - For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting for single-DCI based NCJT, the UE is expected to report, - two RIs, two PMIs, two LIs and one CQI per codeword, when the maximal transmission layer is larger than 4; - one RI, two PMIs, one LI and one CQI, for HST-SFN. |
| Lenovo, Motorola Mobility | - For FR1, support CMR grouping with one CMR group size restricted to one. - For FR1, no CMR pairing restrictions across CMR groups should be imposed. - Further discuss CMR group pairing for FR2. - The maximum Ks should depend on the frequency range and UE capability. - Support multi-DCI multi-TRP CSI enhancements along with single-DCI multi-TRP. - Support explicit CSI Reporting Configuration from each TRP for multi-DCI based NCJT. - Enhancements on CSI feedback structure should be considered for Option 1 with X=1,2. - Discuss updating the CSI report priority function that takes the multi-TRP CSI reporting framework into consideration. - CSI Report for HST-SFN should include 2 PMI/CRI and 1 RI/LI/CQI. - TRP-specific CSI-RS Resources should be used in HST-SFN deployment. - Discuss the CSI reporting framework for HST-SFN to ensure the usefulness of the CSI feedback in case the network switches to single-TRP transmission mode. |