3GPP TSG RAN WG1 #104b-e	   R1-2103901

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.12.1

Source: 		Moderator (CMCC)

Title: 	Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS

Document for:	Discussion/decision

# Introduction

The WI NR_MBS was approved in RAN plenary #86 meeting [1], and the WID was revised in RAN plenary #88 e-meeting [2]. One of the objectives is to specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service, and this objective also includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

The following email thread for group scheduling is announced by chairman in RAN1#104b-e:

[104b-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on Apr-15, Apr-20 – Fei (CMCC)

In this contribution, we summarize the related issues and proposals based on the contributions submitted in RAN1#104b-e under the agenda item 8.12.1 [3]-[25]. The following sections are structured as follows.

From section 2 to 7, we categorized the key issues raised by contributions into 6 kinds and each section covers one kind of issues. In each section, we first provide the background and related proposals submitted in this meeting in sub-section X.1, then one or several initial proposals related to this issue are recommended by moderator in sub-section X.2, and then in sub-section X.3 one or more tables are provided to collect company views for the initial proposals in the 1st round email discussion, and then in sub-section X.4 the proposals will be updated based on companies’ inputs. As email discussion goes on, we may add more sub-sections for companies to provide views for the next round email discussion and for moderator to provide further updated proposals.

In section 8, some proposals will be selected for discussion in the GTW session.

In this meeting, the following was agreed:

Agreement:

For group-common PDCCH of Rel-17 MBS, support at least two DCI formats.

- DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI.

- DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI

  - FFS: Which of DCI format 1_1 or 1_2 is used as the baseline

- FFS: Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2 fields

If possible, please try to provide your replies within 24h. Moderator will try to update the proposals based on companies’ inputs on a daily basis.

# Issue #1: CFR for MBS

## Background and submitted proposals

Background

In RAN1#104-e, the following agreements were achieved for CFR for multicast of RRC-CONNECTED UEs.

Agreement:

For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

    - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

  - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the group-common PDCCH / PDSCH configuration

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

- FFS whether the use of a common frequency resource for multicast is optional or not

- FFS whether the common frequency resource is applicable for PTM scheme 2 (if supported) or not

Agreement:

- If Option 2B is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the starting PRB and the length of PRBs of the MBS frequency region within a dedicated unicast BWP are configured via UE-specific RRC signaling.

  - The starting PRB is referenced to one of the two options:

    - Option 1: Point A

    - Option 2: the starting PRB of the dedicated unicast BWP

  - FFS the detailed signaling

- If Option 2A is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the configurations of the starting PRB and the length of PRBs of the MBS frequency resource reuse the legacy BWP configuration.

Agreement:

From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations:

- Starting PRB and the number of PRBs

- One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP)

- One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP)

- SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP)

- FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP

- FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED

- FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed

- Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2

- Note: This agreement does not negate any previous agreements made on CFR

Submitted Proposals

- Huawei, HiSilicon

  - Proposal 3: For CFR confined within a dedicated unicast BWP,

    - It is up to gNB to schedule unicast or multicast within the CFR, and

    - Additional CORESETS could be configured if the CFR does not fully contain the CORESETS configured for unicast scheduling with the total number not exceeding UE capability.

- OPPO

  - Observation 1: Even though CFR is configured as MBS specific BWP, it is not necessarily to activate the BWP for MBS reception.

  - Proposal 1: Option 2A should be agreed for CFR configuration, where MBS specific BWP should not occupy BWP ID 0~4 and should not be activated.

  - Proposal 2: Configuring a UE with no unicast reception in the common frequency resource is not supported.

  - Proposal 3: Support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities.

- Spreadtrum

  - Proposal 1: UE can be configured with or without unicast reception in the common frequency resource.

  - Proposal 2: Support only one common frequency resource per dedicated unicast BWP per UE.

- ZTE

  - Proposal 1: A unified CFR design is used for broadcast reception in RRC_IDLE/RRC_INACTIVE states, broadcast reception in RRC_CONNECTED state, multicast reception in RRC_INACTIVE state and multicast reception in RRC_CONNECTED state.

  - Proposal 2: Regarding the CFR configuration,

    - Starting PRB and the number of PRBs is needed no matter whether the CFR equal to the dedicated unicast BWP or not

    - One subcarrierSpacing and one cyclicPrefix separate from those of the dedicated unicast BWP should be included

  - Proposal 3: The CFR configuration for broadcast further includes:

    - One PDSCH-ConfigCommon for MBS (i.e., separate from the PDSCH-ConfigCommon of the dedicated unicast BWP)

    - One PDCCH-ConfigCommon for MBS (i.e., separate from the PDCCH-ConfigCommon of the dedicated unicast BWP)

  - Proposal 6: Up to 3 CORESETs in addition to existing CORESETs in the dedicated unicast BWP can be configured for a CFR.

- vivo

  - Proposal 1: For RRC_CONNECTED UEs, when defining/configuring common frequency resource for group-common PDCCH/PDSCH, Option 2B is preferred.

    - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

- CATT

  - Observation 1: BWP switching is needed between MBS specific BWP and dedicated unicast BWP according to Rel-15/16 principle because they are two independent BWPs and configurations.

  - Observation 2: MBS specific BWP may not be feasible when a UE can support to be configured with only one BWP.

  - Proposal 3: Option 2B, MBS frequency region, is supported to define MBS common frequency resource for RRC_CONNECTED UEs.

  - Proposal 4: For MBS frequency region, the starting PRB is referenced to the starting PRB of the dedicated unicast BWP.

  - Proposal 5: RIV indication mechanism in Rel-15 NR can be reused to indicate MBS frequency region, and one field can be added to BWP-DownlinkDedicated IE.

  - Proposal 6: It is supported that a UE can receive unicast services in the common frequency resource.

  - Proposal 7: PDSCH-Config for CFR can share the common fields in PDSCH-Config for unicast to reduce signaling payload size for MBS.

  - Proposal 8: PDCCH-Config for CFR can share the common fields in PDCCH-Config for unicast to reduce signaling payload size for MBS.

  - Proposal 9: Common frequency resource for multicast is optional.

  - Proposal 10: If configured, at most one MBS common frequency resource is supported per UE/per dedicated unicast BWP based on UE capability.

  - Proposal 11: Common frequency resource configuration can be used for PTM scheme 2. The bounding information between UE-specific PDCCH and group-common PDSCH should be further studied.

  - Proposal 19: When MBS frequency region (Option 2B) is supported, up to one CORESET can be configured specifically for MBS service on a dedicated unicast BWP.

  - Proposal 20: When MBS frequency region (Option 2B) is supported, shared CORESET by MBS service and unicast service can be supported on a dedicated unicast BWP.

- Nokia

  - Proposal-1: The key requirement for receiving multicast data using group common PDCCH is to signal the starting PRB relative to the UE-dedicated BWP as a frequency resource / PRB offset parameter, and the length of PRBs or CFR size for the MBS CFR.

    - Note: The signaling details of these parameters could be RAN2 decision.

  - Proposal-2: The starting PRB should be referenced to the starting PRB of the dedicated unicast BWP (option 2 for MBS CFR option 2B).

  - Observation-1: The UE would have two different PDCCH / PDSCH-config parameters for MBS CFR and unicast within a single UE-dedicated BWP.

  - Proposal-3: Mechanisms for applying different PDCCH / PDSCH-config parameters within the same UE-dedicated BWP needs to be further studied and clarified, especially considering shared CORESETs, search spaces and other similar parameters between multicast and unicast.

  - Observation-2: The key difference between option 2A and 2B is related to the RRC signaling of the common frequency resources:

    - Option 2A requires the signaling of MBS specific BWP with parameters possibly taken from current BWP configurations and would possibly require BWP switching based on center frequency alignment.

    - Option 2B requires the signaling of the MBS frequency region – in terms of the starting PRB and length of PRBs within each UE’s dedicated unicast BWP, and PDCCH/PDSCH-config parameters.

    - The impact of option 2A on the number of BWPs that can be configured for a UE needs to be studied and clarified.

  - Observation-3: Currently it is not clear whether simultaneous reception of unicast and multicast traffic within the same slot is possible with option 2A.

  - Proposal-4: Agree on selecting option 2B for configuring multicast common frequency resources, due to the additional complexities involved in the use of option 2A related to BWP switching.

  - Proposal-5: Further study and agree on the commonalities identified between the multicast and broadcast CFR design.

  - Observation-4: It would be beneficial to maintain currently defined limits for the number of CORESETs, in order to minimize UE and gNB complexity and to ensure backward compatibility.

  - Proposal-6: The existing limits on the number of CORESETs for UE-specific BWPs are also applied to those BWPs with MBS CFR, and the number of CORESETs configured within the MBS CFR should be left to gNB implementation.

  - Observation-5: The motivation for configuring RRC_CONNECTED UEs with no unicast reception within the MBS CFR needs to be further clarified.

  - Observation-6: Multiple common frequency resources can be configured per UE based on gNB implementation – even though the motivations for doing so are not clear, with the maximum limit dependent on UE capabilities and available system resources.

  - Observation-7: For multicast traffic, the motivation for configuring multiple CFRs per UE requires further clarification, and for broadcast traffic, there are potential benefits in terms of power savings from having multiple overlapping CFRs configured per UE, depending on UE capabilities and traffic characteristics.

  - Proposal-7: Agree to limit CFRs to one per UE per BWP, considering factors such as additional signaling required for configuring multiple CFRs, and that a single CFR could be utilized to configure multiple MBS services

  - Proposal-8: Agree that CFR for multicast defaults to the UE-dedicated unicast BWP, and when there is no explicit unicast traffic scheduled within the BWP.

  - Proposal-9: Agree that CFR concept and related configurations for starting PRB and length of PRBs are only applicable for PTM scheme 1.

- MediaTek

  - Proposal 4: Send LS to RAN4 to confirm whether Option 2A needs BWP switching delay when simultaneous receiving unicast and multicast services if Option 2A is used to configure the MBS CFR.

  - Proposal 5: Network implementation guarantee the allocation of common frequency resource for UEs in connected mode to receive the PTM transmission.

  - Proposal 8: Not increase the total existing number of CORESET and search space for NR MBS scheduling.

  - Proposal 6: Not support more than one common frequency resources for NR MBS.

- FUTUREWEI

  - Proposal 1: The starting PRB and the number of PRBs of the CFR within the unicast BWP is signaled in the SIB as a baseline. Additional configuration using RRC can also be considered.

  - Proposal 2: Both starting location and the length can be jointly encoded to reduce overhead in the signaling.

  - Proposal 3: Baseline of 1 CFR per unicast BWP per UE is supported. Additional CFR greater than 1 is FFS.

  - Proposal 4: CFR configuration for UEs in Idle state should be supported.

  - Observation 1: The number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH should be considered during UE capability discussions for MBS-enabled UE.

- ETRI

  - Proposal1: The option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP should be supported for the common frequency resource.

  - Proposal2: The starting PRB of the dedicated unicast BWP is used as a reference point to indicate the starting PRB of the MBS frequency region.

- CMCC

  - Proposal 1. UE cannot be configured with no unicast reception in the common frequency resource.

  - Proposal 2. Support only one common frequency resource for multicast service per dedicated unicast BWP.

  - Proposal 3. For RRC_CONNECTED UEs, PDSCH-config/PDCCH-config/SPS-config(s) for MBS can be optional configured for CFR. If the PDSCH-config/PDCCH-config for MBS is not configured, the PDSCH-Config/PDCCH-config of the dedicated unicast BWP can be re-used for group-common PDCCH/PDSCH.

  - Proposal 4. If the CFR is equal to the unicast BWP, the signalling of starting PRB and the length of PRBs is not needed, which the configuration of starting PRB and the length of PRBs of unicast BWP is re-used.

  - Proposal 7. The mandatary maximum number limit of CORESETs per BWP (i.e., 3 for single-TRP or 5 for multi-TRP) is kept for Rel-17 MBS. Additional CORESETs for MBS can be optionally supported.

  - Proposal 8. The CORESET(s) confined within the CFR can be used for group-common PDCCH for multicast service and UE-specific PDCCH for unicast service.

  - Proposal 16. For RRC_CONNECTED UEs, the same CFR with RRC_IDLE/INACTIVE UEs is used for broadcast reception when the same group-common PDCCH and the corresponding scheduled group-common PDSCH are received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs, but can be different from the CFR used for multicast reception.

  - Proposal 17. For RRC_CONNECTED UEs, the group-common PDCCH and the corresponding scheduled group-common PDSCH for broadcast reception are transmitted in UE-specific active BWP, which can be different from the group-common PDCCH/PDSCH received by RRC_IDLE/RRC_INACTIVE UEs when UE-specific active BWP of RRC_CONNECTED UE does not totally contain the common frequency resource of RRC_IDLE/INACTIVE UEs.

  - Proposal 18. For RRC_CONNECTED UEs, the same CFR is used for broadcast reception and multicast reception, when UE-specific active BWP of RRC_CONNECTED UE does not totally contain the common frequency resource of RRC_IDLE/INACTIVE UEs.

- Intel

  - Proposal 1: A default CFR identical to active unicast BWP can be defined for UEs when no CFR configuration is provided

  - Proposal 2: RAN1 should strive for unified CFR for CONNECTED and IDLE mode UEs

  - Proposal 3: MBS capable UEs may support an additional CFR CORESET for monitoring MBS PDCCH and the support of the additional CORESET can be a UE capability.

  - Proposal 4: The UE expects no restriction on unicast reception within the CFR since it is contained within the active DL BWP of the UE.

  - Proposal 5: One CFR per dedicated BWP is sufficient for scheduling MBS transmissions.

  - Proposal 12: For PTP or PTM scheme 2, the CORESET scheduling MBS (re)transmission can be configured outside the MBS frequency region.

- Apple

  - Proposal 1: The starting PRB of CFR is referenced to the Point A.

- Qualcomm

  - Proposal 1: For RRC_CONNECTED UEs, a common frequency resource is defined as an MBS specific BWP (Option 2A).

    - UE can monitor a CFR if it is full within the associated unicast BWP and with same numerology, where no BWP switching when receiving unicast and multicast.

    - One or more CFRs can be configured per UE subject to UE capability.

    - One or more CFRs can be configured per dedicated BWP subject to UE capability.

  - Proposal 2: For RRC_CONNECTED UEs, the LBRM for GC-PDSCH TBS is determined per CFR.

  - Proposal 3: For RRC_CONNECTED UEs, the xOverhead for GC-PDSCH TBS determination is configured per CFR.

  - Proposal 4: For RRC_CONNECTED UEs, the MAC-CE over GC-PDSCH can be used to active SPS ZP CSI-RS configured per CFR.

  - Proposal 5: For RRC_CONNECTED UEs, more than one CORESET for GC-PDCCH can be configured per MBS CFR.

    - Keep the maximum total number of CORESETs per MBS CFR same as that of unicast BWP.

    - Keep the maximum total number of CORESETs per UE unchanged.

- Samsung

  - Proposal 1: An MBS frequency region within the active DL BWP is configured to a UE for MBS operation.

  - Observation 1:

    - (a) It is a gNB implementation aspect whether or not to configure unicast receptions for a UE in the CFR.

    - (b) Group-common PDCCH/PDSCH configuration can follow UE-specific PDCCH/PDSCH configuration.

    - (c) There is no need to support more than one CFR for a UE within an active DL BWP.

    - (d) Configuration of a CFR is not necessary (if not configured, the CFR is the active DL BWP).

    - (e) There is no need to introduce PTM scheme 2.

    - (f) An active BWP change needs to be common for unicast and multicast.

  - Observation 2: There is no need to increase the number of CORESETs for a UE configured with MBS and unicast PDSCH.

- LGE

  - Proposal 1: Support both Option 2A and 2B for both multicast and broadcast regardless of RRC states for compromise as follows:

    - Option 2A for a wider MBS specific BWP than the initial DL BWP or UE’s active DL BWP, and

    - Option 2B for a frequency region within or equal to the initial DL BWP or UE’s active DL BWP.

    - Which option is used is up to gNB configuration

    - The existing term ‘BWP’ is re-used to specify CFR in specifications.

  - Proposal 2: Connected UE should maintain at least one UE’s active BWP as specified in REL-15/16.

  - Proposal 3: BWP switching between MBS specific BWP and UE’s active BWP is NOT supported. UE is allowed to simultaneously activate one MBS specific BWP and one UE’s active BWP.

  - Proposal 4: MBS capable UE activates only one MBS DL BWP at a time for REL-17.

- Convida

  - Proposal 4: Unified CFR design should be used for MBS for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs.

  - Proposal 5: Dedicated MBS BWP (option 2A) should be supported for RRC_CONNECTED UEs and RRC_IDLE/INACTIVE UEs in NR MBS.

  - Proposal 6: Dedicated CORESET(s) should be configured for NR MBS in addition to the existing CORESETs in UE dedicated BWP.

- Lenovo

  - Proposal 1: An MBS frequency region with contiguous PRBs confined within the dedicated unicast BWP is configured for MBS, i.e., Option 2B is supported.

  - Proposal 2: The starting PRB index and the number of contiguous PRBs of the MBS frequency region are configured within the dedicated unicast BWP via RRC signaling.

  - Proposal 3: The starting PRB of the MBS frequency region is configured with reference to the starting PRB of the dedicated unicast BWP.

  - Proposal 6: RB numbering within the common frequency region is with reference to the lowest RB of the common frequency region.

- NTT Dococmo

  - Proposal 1: Support Option 2A to define a common frequency resource for group-common PDCCH/PDSCH.

    - BWP switching is not supported.

  - Proposal 2: Support at most one common frequency resource per dedicated unicast BWP.

  - Observation 1: CORESET(s) for CFR is needed when any existing CORESETs for unicast are not included in the CFR.

- ASUSTeK

  - Proposal 3: If a UE’s active BWP is switched from an MBS-capable BWP to an MBS-incapable BWP, it needs some studies for the UE to resume multicast PDCCH/PDSCH receptions.

- Ericsson

  - Observation 4: With Option 2A, the UE would need to have two simultaneously active BWPs, which is preferable to BWP switching.

  - Observation 5: The use of two active BWPs will have significant MAC layer impact and require significant RAN2 work

  - Proposal 5: If 2A is selected, when the unicast and multicast BWs are the same, it should be possible to configure MBS on the unicast BWP alone, i.e. without using an MBS BWP.

  - Proposal 6: If 2A is selected, when MBS is configured on the unicast BWP, it should be possible to use MBS-specific RRC configurations of PDCCH, PDSCH and SPS on the unicast BWP.

  - Proposal 7: If 2A is selected, when MBS is configured on the unicast BWP, it should be possible to reuse the unicast RRC configurations of PDCCH, PDSCH and SPS for MBS by not using any MBS-specific RRC configurations for MBS.

  - Observation 6: When multicast uses the same frequency range as unicast, there is no need to explicitly RRC configure a CFR (frequency range), since the unicast frequency range, as given by the unicast BWP, then applies for both unicast and MBS.

  - Proposal 8: If 2B is selected, explicit configuration of the CFR (frequency range) is optional when the CFR is the same as the unicast BWP.

  - Proposal 9: If 2B is selected, it should be possible, as a special case, to reuse the unicast RRC configurations of PDCCH, PDSCH and SPS for MBS, in which case MBS-specific configurations are not used.

  - Proposal 10:  If 2A is selected, and no MBS BWP has been configured (i.e. for multicast), the UE should receive the broadcast BWP like an MBS BWP, which should allow parallel reception of unicast and broadcast.

  - Proposal 11: If 2A is selected, when the UE is RRC configured with an MBS BWP and MCCH configured with a broadcast BWP, these BWPs need to be aligned in the sense of using the same frequency range. In this case the MBS BWP “inherits” the broadcast BWP, so that the broadcast BWP is not counted as an additional BWP in the overall number of BWPs that the UE is configured with.

  - Proposal 12: The frequency range and broadcast configurations of PDCCH, PDSCH and SPS, as indicated by MCCH, are inherited by the unicast BWP and become additional configurations of the unicast BWP, which allows parallel reception of unicast, multicast and broadcast using a single active BWP.

  - Proposal 13: Downselect to Option 2B for the unicast & MBS multicast use case.

  - Proposal 14: Extend Option 2B to support also broadcast, to allow UEs in RRC Connected to receive in parallel unicast, multicast and broadcast transmissions using the unicast BWP as the active BWP. The broadcast CFR and PDCCH, PDSCH and SPS configurations are inherited by the unicast BWP as additional configurations.

  - Proposal 15: If 2B is selected, the reference point for the starting PRB of the CFR is Point A.

  - Proposal 24: The CORESET for group common PDCCH is counted toward of the already existing CORESET capability of the UE. No additional  number of CORESET capability is defined for MBS only.

## Initial Proposals based on contributions

Summary

We have agreed in the last meeting that the CFR for multicast of RRC-CONNECTED UEs includes the frequency resources, PDCCH-config/PDSCH-config/SPS-config(s) for MBS, although based on this we can continue discussing other detailed issues, it would be better that we make a decision on the down-selection of Option 2A and 2B as soon as possible. Regarding Option 2A, based on contributions submitted in this meeting, companies still have different understandings on whether BWP switching is needed between MBS specific BWP and dedicated unicast BWP if only one active BWP at a time is allowed for the UE in the specification according to Rel-15/16 principle, and whether the specification needs to be enhanced to allow the UE to have two active BWPs. As pointed out in [25], with Option 2A, the UE would need to have two simultaneously active BWPs, which is preferable to BWP switching, and the use of two active BWPs will have significant MAC layer impact and require significant RAN2 work (see	TS 38.321 - 5.15 Bandwidth Part (BWP) operation). In [4], it is suggested to send LS to RAN4 to confirm whether Option 2A needs BWP switching delay when simultaneous receiving unicast and multicast services if Option 2A is used. It seems the proponents of Option 2A did not provide clear explanation regarding whether the UE need to support two simultaneously active BWPs or not. Regarding Option 2B, companies have no doubt that the simultaneous reception of unicast and multicast in the same slot can be supported without BWP switching. From moderator point of view, one way to move forward is that RAN1 continues discussing more details of Option 2A (e.g., whether the UE need to support two simultaneously active BWPs or not, etc.) to see if RAN1 can reach a common understanding on Option 2A in this meeting, and RAN1 will try to make a down-selection in RAN1#105-e. If RAN1 cannot make the decision before the end of RAN1#105-e, RAN1 will send an LS to RAN2 and the down-selection will be up to RAN2. I hope proponents of Option 2A can provide more details on this option during the email discussion so that we can try to have a common understanding on it.

Regarding the reference point of the starting PRB of Option 2B for CFR, 6 companies have explicit proposals, 2 [Ericsson, Apple] of them propose to take Point A as the reference for the starting PRB of the CFR (i.e., option 1), others propose to take the starting PRB of the dedicated unicast BWP as the reference point (i.e., option 2). Although option 2 seems a straightforward way and implies less overhead since the CFR is within the dedicated unicast BWP, Ericsson provided a reason for using option 1 in [25] which seems reasonable, i.e., with option 1 there is no need to reconfigure the CFR because of a dedicated unicast BWP reconfiguration. Therefore, moderator suggest companies further consider whether option 1 is acceptable taking into account the reason raised by Ericsson.

Regarding the FFS whether UE can be configured with no unicast reception in the common frequency resource, at least eight companies think such kind of restriction is not necessary, and it is up to gNB implementation to schedule unicast or multicast within the CFR.

Regarding the FFS whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed, companies have different views. In a summary, there are two options as below. It seems Option 1 has more supporters, and one company thinks that this issue should be considered during UE feature discussions for MBS-enabled UE. Moderator tends to mark this issue as medium priority, and companies are encouraged to express their views on whether we should defer discussion on this issue.

- Option 1: The maximum number of CORESETs per BWP is not increased for support of MBS if a CFR is confined within the BWP. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

- Option 2: The maximum number of CORESETs per serving cell is not increased for support of MBS, but the maximum number of CORESETs per BWP can be increased to up to N based on UE capability for support of MBS if a CFR is confined within the BWP. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

Regarding whether the CORESETs can be shared for unicast and multicast, there are basically 4 options in a summary as below. Since no company proposes specific restriction for this issue, moderator is trying to check if Option 1 can be acceptable for companies, and hope companies can express their views on this issue.

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP,

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Regarding the FFS whether to support more than one CFR per UE / per dedicated unicast BWP subjected to UE capabilities, 9 companies [Spreadtrum, CATT, Nokia, MTK, FutureWei, CMCC, Intel, Samsung, NTT Docomo] explicitly propose that one CFR per BWP is sufficient. 2 companies [OPPO, Qualcomm] propose to support more than one CFR per BWP subject to UE capability. Based on majority view, moderator propose that zero or one CFR per BWP is supported, and more than one CFR per BWP is FFS. However, moderator wants to check with companies whether such a scenario need to be supported as illustrated in the following figure, in which UE1 and UE2 receive MBS service 1 in group 1 and UE2 and UE3 receive MBS service 2 in another group 2. If it needs to be supported, then UE2 may need to be configured with more than 2 CFRs in a BWP. Even this scenario is not supported, UE2 can still receive MBS service 2 in a PTP manner but it cannot be grouped together with UE3.

Regarding whether the use of a CFR for multicast is optional or not, 5 companies [CATT, Nokia, Intel, Samsung, Ericsson] propose that the CFR for multicast defaults to the dedicated unicast BWP if no CFR configuration is provided in this dedicated unicast BWP. However, it is unclear in this case how the UE identifies whether it should receive multicast in this dedicated unicast BWP assuming that the RRC configurations of PDCCH/PDSCH/SPS for unicast can be reused for multicast, or it should not receive multicast in this dedicated unicast BWP. Therefore, it seems that the key point here is not whether a CFR for multicast is optional or not, but whether it is possible to reuse the frequency resources and RRC configurations of PDCCH/PDSCH/SPS for unicast in the dedicated unicast BWP for multicast transmission or not.

Initial Proposals

The following moderator recommendations are made.

[Moderator’s recommendation]

[High] Initial Proposal 1-1:

The down-selection of Option 2A and Option 2B for CFR will be made before the end of RAN1#105-e.

- If RAN1 cannot make the decision before RAN1#105-e, RAN1 will send an LS to RAN2 and the down-selection will be up to RAN2.

[High] Initial Proposal 1-2:

For Option 2A of CFR for multicast of RRC-CONNECTED UE, UE needs to support two simultaneously active BWPs.

[High] Initial Proposal 1-3:

For Option 2B of CFR for multicast of RRC-CONNECTED UE, the starting PRB of CFR is referenced to PointA.

[High] Initial Proposal 1-4:

It is based on gNB implementation to schedule unicast or multicast within the CFR.

[Medium] Initial Proposal 1-5:

Regarding the CORESETs for support of MBS, Option 1 is supported.

- Option 1: The maximum number of CORESETs per BWP is not increased for support of MBS if a CFR is confined within the BWP. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

- Option 2: The maximum number of CORESETs per serving cell is not increased for support of MBS, but the maximum number of CORESETs per BWP can be increased to up to N based on UE capability for support of MBS if a CFR is confined within the BWP. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

  - FFS: the value of N

[High] Initial Proposal 1-6:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, Option 1 is supported.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

[High] Initial Proposal 1-7:

Zero or one CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

- FFS: whether more than one CFR is supported per dedicated unicast BWP

[High] Initial Proposal 1-8:

For RRC_CONNECTED UEs, it is possible to reuse the frequency resources and RRC configurations of PDCCH/PDSCH/SPS for unicast in the dedicated unicast BWP for multicast transmission.

- FFS: how to reuse the frequency resources and RRC configurations of PDCCH/PDSCH/SPS for unicast in the dedicated unicast BWP for multicast transmission

## Company Views (1st round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 1-1: We support Option 2B and the reason is well summarized by FL. If RAN1 can’t made the down-selection, we are afraid that RAN2 could not made same thing. Meanwhile, we think RAN1 is a better place to make such decision than RAN2. 1-2: This seems like observations instead of proposals. 1-3: We prefer the starting PRB of CFR is reference to the starting PRB of the associated unicast BWP for the sake of RRC signaling overhead reduction. Regarding the benefit of no need to reconfigure the CFR when the associated BWP is reconfigured if the starting PRB of the CFR is with reference to Point A, it is not clear to us. The CFR will be reconfigured when the associated dedicated BWP is reconfigured. With the starting PRB of the CFR is reference to the Point A, seemingly, the configuration of the CFR is not associated to the dedicated unicast BWP. 1-4: generally agree. 1-5: agree. 1-6: agree. 1-7: agree. 1-8: We don’t fully understand the point. Maybe it is possible to delay it. |
| Spreadtrum | Fine with Proposal 1-1, 1-2, 1-3, 1-4, 1-5, 1-7. For Proposal 1-8, we think the issue should be discussed later, depending on other proposals, e.g., proposal 2-1. |
| Apple | For Proposal1-2, it’s not exactly two active BPWs, if the MBS BWP is configured within the UE dedicated BWP. This proposal is more related to Proposal 1-5 whether the CORESET can be configured larger than 3 in a BWP. We support Proposal 1-3, the starting PRB of CFR is referenced to Point A, this configuration can apply to group users to save the signaling overhead, it don’t require the gNB to calculate/configure the PRB offset to dedicated BWP for each UE. For Proposal 1-8, we are ok with proposal for PTP transmission. |
| CATT | Proposal 1-1: Option 2B is supported. - It is better for RAN1 rather than RAN2 making the decision. Currently, the most critical concern is whether BWP switching is needed or not, which cannot be fully/clearly demonstrated by companies. Even the decision of down-selection is forwarding to RAN2, the same situation/argument may also happen. Proposal 1-2: Not support. Proposal 1-3: Instead of point A, the starting PRB is referenced to the starting PRB of the dedicated unicast BWP. Similar view of Lenovo. Proposal 1-4: Generally OK. Proposal 1-5: Generally OK. Proposal 1-6: Generally OK. Proposal 1-7: ONLY support the main bullet. The motivation and benefit of the FFS are not so clear and persuasive. Proposal 1-8: Generally OK. |
| OPPO | Proposal 1-1: RAN1 should strive to make the down-selection within this meeting, we prefer not to make such a conclusion at the beginning of the meeting. Proposal 1-2: In our understanding, the MBS dedicated BWP is NOT necessarily activated as normal dedicated unicast BWPs. As the frequency range indicated in the MBS dedicated BWP is confined within the activated unicast BWP, UE can receive group common PDCCH/PDSCH within the frequency range with the MBS specific parameters (e.g.， PDCCH-config, PDSCH-config, SPS-config, etc.) even the MBS dedicated BWP is not explicitly activated. Proposal 1-3: As we are going to make down-selection, and seems this proposal does not impact other high priority issues in this summary, we suggest to postpone this issue. Proposal 1-4: Agree. Proposal 1-5: Fine to defer this issue. Proposal 1-6: As CFR configuration includes PDCCH-config, CORESET usable for MBS can be indicated explicitly, there is no need to define implicit rule to indicate the CORESET applicable for MBS. Proposal 1-7: Agree. Proposal 1-8: In our understanding this proposal is not needed. As CFR configuration includes frequency resources and PDCCH/PDSCH/SPS, gNB is allowed to configure all these parameters identical as those for unicast dedicated BWP. |
| ZTE | For Proposal 1-1: It seems difficult to make down-selection in RAN1 based on previous discussion. It seems better if we can wait for the CFR outcome for UEs in IDLE state and see if we can select one solution with the most commonality between IDLE UEs and CONNECTED UEs. If we want to have such an agreement, we would prefer the following. RAN1 strives to make down-selection between Option 2A and Option 2B for CFR before the end of RAN1#105-e, considering the commonality between IDLE UEs and CONNECTED UEs. - If RAN1 cannot make the decision before RAN1#105-e, RAN1 will send an LS to RAN2 and the down-selection will be up to RAN2. For Proposal 1-2: From our perspective, UE can support two simultaneously active BWPs for Option 2A. However, this doesn’t introduce any additional UE complexity if MBS BWP is fully confined with unicast BWP and share the same SCS. For Proposal 1-3: It seems not necessary to use PointA as the reference. We would propose to use the starting PRB of carrier bandwidth as the reference. Anyway, the CFR can’t be configured outside the carrier bandwidth. Meanwhile, the CFR configuration also doesn’t need to update when BWP switches if the SCS keeps the same. For Proposal 1-4: Ok with this proposal. For Proposal 1-5: From our perspective, it would be too restrictive to keep 3 CORESETs for all UEs receiving MBS. Currently, with one specific CORESET#0 for CSS and another CORESET for beam recovery, there is only one CORESET specific for unicast scheduling. If MBS and unicast have to share the same CORESET, the flexibility would be too limited. We would prefer Option2 to allow UE to report a larger UE capability. For Proposal 1-6: For Option1, if the CORESET configured within CFR can be used to schedule unicast transmission, then it seems not necessary to configure CORESET in PDCCH-config that is fully contained in the CFR. Because, anyway, you can configure this CORESET in the PDCCH-config for MBS. In this sense, we prefer Option4 though we are also ok with Option1. For Proposal 1-7: From our perspective, the use case for two CFRs are valid and we can also consider supporting more than one CFR. Thus, the following is proposed from us. Zero or one CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs. - FFS: whether more than one CFR is supported per dedicated unicast BWP subject to UE capability For Proposal 1-8: We fail to see the benefit of this proposal. If no specific RRC configurations of PDCCH/PDSCH/SPS has been configured for MBS, then how can a UE determine whether to receive MBS service or not. It may be ambiguous whether there is no MBS service at all or network just wants the UE to reuse the same unicast configuration for MBS. |
| CMCC | 1-1: Agree，slightly prefer option 2B 1-2: Agree, in current BWP framework, one UE can only support one active BWP at a time, if option 2A is supported, which means UE should support two active BWPs simultaneously. 1-3: Not agree, it is much straightforward to use the dedicated BWP as the reference RB allocation signalling. 1-4: Agree. 1-5: Prefer option 2 1-6: Prefer option 1, it can give gNB more scheduling flexibility. 1-7: Support zero or one CFR, not support multiple. If multiple CFRs are configured, there will be multiple G-RNTI DCI sizes, which causes more complex on DCI size alignment. 1-8: Agree. |
| NTT DOCOMO | Proposal 1-1,Proposal 1-2: We are generally fine with FL’s proposal. We prefer Option 2A. We think that it is possible to support two active BWPs without significant spec impact. Proposal 1-4: We are fine with FL’s proposal. Proposal 1-5: We are fine with FL’s proposal. To minimize implementation impact Option 1 is preferred. Proposal 1-6: We are fine with FL’s proposal. We don’t see any motivation to make the CORESET for MBS unusable for unicast, and vice versa. Proposal 1-7: We are fine with FL’s proposal. Proposal 1-8: We are fine with FL’s proposal. When configurations of PDCCH/PDSCH/SPS for multicast is not provided in the CFR, the corresponding configurations for unicast should be reused for multicast. |
| Huawei/HiSilicon | If defer the decision on option2A/2B, any discussion for next-step issues for each option should be deterred as well given the limited time for this meeting. Therefore, proposals 1-1, 1-2, and 1-3 can wait. Proposal 1-4 ok. If proposal 1-5 depends on option2A/2B and is medium, discussion can wait as well. “if a CFR is confined within the BWP” could be misleading to mean CFR could be outside of BWP. Proposal 1-6, I am thinking whether we need an explicit proposal/agreement for this issue because the CORESET/search space configuration in PDCCH-config is up to network. Proposal 1-7, ok. Proposal 1-8, the phrasing is more like one open question for discussion instead of proposal for agreement. Not sure about the intention behind. |
| LG | P1-1: We think that harmonization between Option 2A and Option 2B could be also possible, e.g. as proposed in LG’s contribution. In addition, we also think that commonality between IDLE UEs and CONNECTED UEs for both broadcast and multicast. Thus, we could slightly modify ZTE’s suggestion as follows: RAN1 strives to make down-selection or harmonization between Option 2A and Option 2B for CFR before the end of RAN1#105-e, considering the commonality between IDLE UEs and CONNECTED UEs for broadcast and multicast:. - If RAN1 cannot make the decision before RAN1#105-e, RAN1 will send an LS to RAN2 and the down-selection will be up to RAN2. P1-2: Agree. We do not support CFR switching at least while keeping the associated UE’s active BWP. P1-3: We are fine with the initial proposal. P1-4: Disagree. Unicast is scheduled within UE’s active BWP (i.e. as in REL-15/16) which can associated to the CFR. We do not think that unicast can be scheduled within the CFR. CFR has no configuration of PDCCH/PDSCH for unicast. We wonder if it is really important to agree 1-4. P1-5: We prefer Option 2. P1-6: We prefer Option 3. We do not need to change unicast for multicast. P1-7: We are fine with the initial proposal without FFS. We do not need more than one CFR. P1-8: It seems not clear at this moment whether we can reuse some of the parameters from unicast BWP configuration. We could possibly discuss this proposal later. |
| MTK | Proposal 1-1: Support Option 2B. The controversial key point about this issue is whether Option 2A can incur BWP switching delay due to MBS specific BWP. RAN1 has discussed it in several last meeting but didn’t reach a consensus. If delivering this issue to RAN2, maybe the similar situation will emerge. Considering the RAN4 working scope, we suggest send an LS to RAN4 to confirm whether Option 2A can cause BWP switching delay if RAN1 cannot make the decision in this meeting. The down-selection of Option 2A and Option 2B for CFR will be made before the end of RAN1#105-e. - If RAN1 cannot make the decision before RAN1#105-e, RAN1 will send an LS to RAN4 and confirm whether Option 2A can cause BWP switching delay. Proposal 1-2: The purpose of the proposal is not clear from our side. Does it meaning to support two simultaneously actives BWPs in order to support Option 2A or only state the situation that two simultaneously active BWPs are needed if Option 2A is supported? Not support Option 2A. In current spec, a UE only has one active BWP, there is no need to support two simultaneously active BWPs in order to support Option 2A. Proposal 1-3: Not support the proposal. The starting PRB of the dedicated unicast BWP will have less signaling overhead than Point A. By the way, when unicast BWP is changed, reconfiguring the MBS CFR is natural, especially if some MBS parameter also need to be changed. Proposal 1-4: We are generally fine with the proposal. Proposal 1-5: we are fine with option 1. Proposal 1-6: we are fine with the proposal. Proposal 1-7: We are fine with the main bullet |
| Samsung | Proposal 1-1: OK to wait a month until May if a decision cannot be made now – the two choices are simple and the issue is rather trivial, does not prevent overall progress. Prefer to keep it in RAN1 as it is more appropriate that RAN2 for making that decision and as similar discussion (with possibly more confusion) is likely to happen in RAN2. Depending on specifics of Option 2A, we can be OK with either Option 2A or Option 2B. Proposal 1-2: The proposal is problematic and can wait outcome for Option 2A/2B – in Rel-16, BWPs can have several independent parameters that would require new UE capabilities or be impossible to support. Also, no apparent need for an agreement now on proposal 1-2. Proposal 1-3: Reference either to point A or to the unicast BWP are possible choices. The issue is trivial either way but proposal can wait for decision on Option 2A or Option 2B. Proposal 1-4: Agree Proposal 1-5: Support Option 1. Option 2 is not in scope as it requires new UE hardware and complexity. Proposal 1-6: Agree with option 1. Also no need to have any specific agreement as this is a gNB implementation issue through the mapping of search space sets to CORESETs. Proposal 1-7: Agree Proposal 1-8: It is better to postpone and discuss together with the FFS. |
| Qualcomm | 1-1: One of the key issues on the down-selection of Option 2A and 2B for CFR is to consider whether to have the commonality between IDLE/INACTIVE and CONNECTED UEs. Since we will not discuss broadcast for IDLE/INACTIVE UEs this meeting, we are fine to delay the down-selection for now. 1-2: The MBS specific BWP is a virtual BWP, not in the same level as a dedicated BWP. The UE is still monitoring only one active dedicated BWP. Only if the MBS specific BWP is within the active BWP, the UE can monitor the MBS and unicast at the same time. 1-3: ok 1-4: ok in principle. But it seems like a conclusion rather than agreement if no spec impact. 1-5: We prefer Option 2. 1-6: Not agree with Option 1, which is contradictory to RAN1 agreement that the PDCCH-config in CFR is used for multicast GC-PDCCH. We are fine with Option 4 to allow the CORESET in CFR PDCCH-Config can be used for unicast. 1-7: Agree with ZTE’s proposal. 1-8: The proposal is not clear to us. Does it mean no CFR is configured or some of the parameters of PDCCH/PDSCH/SPS in the CFR are not configured? Besides, we would like to have some discussion on the rate matching parameters for GC-PDSCH, which are not fully covered by PDSCH-Config, such as LBRM and xOverhead. |
| Futurewei | 1-1: We should downselect this meeting and option to defer need not to be offered at the start of the mtg. Support Option 2B. 1-2: Agree with this observation. This does not need to be an agreement. 1-3: Support 1-4: Support 1-5: Support Option 1 1-7: Support 1-8: Support |
| Nokia, NSB | Proposal 1-1: We agree with this proposal in general since agreeing on the CFR option is important to define some of the other concepts such as CORESETs within CFR. However, we do not think that the down-selection between the options could be left to RAN2 and should be a RAN1 decision. Our preference is option 2B, since we believe that this is the simplest option and requires minimal specification enhancements. Proposal 1-2: We agree with this proposal and the need to support two active BWP is one of the key drawbacks of option 2B. Having multiple active BWPs with center frequencies aligned would impose significant restrictions on the gNB scheduling and we would prefer to avoid it. Proposal 1-3: We are fine with this proposal if this is in line with the views of majority of the companies. However, as we had presented in our contribution, we believe starting PRB referenced to the UE-dedicated BWP would be a relatively simpler option. Proposal 1-4: Agree Proposal 1-5: We agree with this proposal and option 1 is preferred due to lower implementation complexity. We also agree with Huawei’s comment that: “if a CFR is confined within the BWP” is redundant since it was agreed that CFR would be always confined within the UE’s active dedicated BWP. Proposal 1-6: We agree with FL’s proposal. Proposal 1-7: We agree with FL’s proposal, and agree with CMCC’s views that the number of CFRs could be limited to one in order to minimize complexity. Proposal 1-8: We agree with FL’s proposal. |
| Ericsson | P1-1: We agree with the aim that “The down-selection of Option 2A and Option 2B for CFR will be made before the end of RAN1#105-e”. However, should RAN1 be unable to reach that aim, we still think RAN1 alone should decide on this. The CFR is a PHY concept and belongs to RAN1. Since the decision may have RAN2 repercussions with respect to MAC layer, we think however RAN2 should be asked to comment on the solution if we, from a PHY/RAN1 perspective, would prefer 2A. If we anyway prefer 2B, we do not need to ask RAN2 for such comments. For this reason, we think a decision in favor of 2B could be achieved with less delay than 2A. The impact on standardization delay should therefore be considered. P1-2: We agree P1-3: We agree P1-4: We agree but would like to propose a reformulation: “It is up to gNodeB implementation to decide whether and how to schedule unicast and/or multicast within the CFR.” Since this agreement would not have any specification impact, we suggest considering the alternate solution of making this a Conclusion. P1-5: We agree with Medium priority and are also fine with Option 1 but are OK to leave Option 2 as FFS. P1-6: We agree P1-7: We agree. This applies for both 2A and 2B, i.e. both 2A and 2B can be used for MBS without a configured CFR. P1-8: We agree |
| Convida | For Proposal 1-1: We support option 2A. We agree with ZTE that we should strive to choose the solution that has the most commonality between the IDLE/INACTIVE UEs and CONNECTED UEs. We don’t see how option 2B can work for IDLE/INACTIVE UEs when a CFR that is wider than the initial BWP need to be used. It will be good for companies who support option 2B to elaborate how this problem can be solved before we make any decision. For Proposal 1-2: From our view, we think the dedicated MBS BWP does NOT necessarily need to be activated as long as it is confined within the unicast BWP and has the same SCS. However, we are also fine to mandate activating two active BWPs simultaneously if companies are OK with it. For Proposal 1-3: Not agree. For option 2B, we support the starting PRB of CFR is referenced to starting PRB of the associated unicast BWP. For Proposal 1-4: Agree. For Proposal 1-5: We are fine with FL’s proposal. For Proposal 1-6: We slightly prefer option 3. For Proposal 1-7: We are fine with FL’s proposal. For Proposal 1-8: We fail to understand the intention of this proposal. Maybe more clarifications are needed. |
| ETRI | 1-1: Agree 1-2: It seems conclusion rather than proposal for agreement. 1-3: We prefer the starting PRB of the dedicated unicast BWP. 1-4: It seems conclusion rather than proposal for agreement. 1-5: Agree 1-6: Agree 1-7: Agree 1-8: Agree |
| vivo | 1-1: We support Option 2B and the reason is well summarized by FL. According to RAN2’s discussion, it seems that RAN2 want RAN1 to make this decision. We are afraid that RAN2 can make this decision. Fine with Proposal 1-2, 1-3, 1-4, 1-5, For Proposal 1-6, not sure option 1 can work well. pdcch-DMRS-ScramblingID is configured in IE of ControlResourceSet in PDCCH-config. Even if the CORESET configured in the dedicated unicast BWP is fully confined within CFR, the pdcch-DMRS-ScramblingID is UE specific. For Proposal 1-7, we are just wondering how UE knows to receive multicast if zero CFR is configured. In our view, some parameters configured in CFR, such as starting PRB and length of PRBs, can be optional, but UE has to be configured with a CFR if UE wants to receive multicast. For Proposal 1-8, we think it is related with Proposal 1-7. In addition, one issue is that, for multicast SPS, PDSCH scrambling ID is different from that for unicast SPS, i.e., CS-RNTI, if SPS for unicast for unicast in the dedicated unicast BWP is reused for multicast, how to distinguish one SPS is for unicast and multicast? For the case shown in the above figure, we think it should be supported. |
| Moderator | Proposal 1-1: The proposal was updated to accommodate most companies’ comments. Proposal 1-2: It’s apparently companies, even the ones in the 2A camp, have different understandings of how Option 2A works. This proposal just aims to reach a common understanding of 2A. Otherwise, I do not know how it is possible for the 2A camp to persuade 2B camp to accept 2A. I suggest the proponents of 2A try to reach a consensus on this issue. Proposal 1-3: The proposal was updated based on majority view. Proposal 1-4/1-5: The proposal was updated to accommodate companies’ comments. Proposal 1-6: It seems companies need more time to further study. Proposal 1-7: Some companies prefer to delete the FFS bullet, some companies prefer to support more CFR, I think the current wording could be a compromise, so I did not update it. Proposal 1-8: The discussion for this proposal is postponed based on companies’ comments. |

## Updated Proposals (after 1st round of inputs)

[High] Updated Proposal 1-1:

The down-selection of Option 2A and Option 2B for CFR will be made before the end of RAN1#105-e, considering the commonality between RRC_IDLE and RRC_CONNECTED for broadcast and multicast, specification impact, etc.

[High] Updated Proposal 1-2:

If Option 2A of CFR is supported for multicast of RRC-CONNECTED UE, UE needs to support two simultaneously active BWPs.

[High] Updated Proposal 1-3:

If Option 2B of CFR is supported for multicast of RRC-CONNECTED UE, the starting PRB of CFR is referenced to the starting PRB of the dedicated unicast BWP.

[High] Updated Proposal 1-4(for conclusion):

It is based on gNB implementation to decide whether and how to schedule unicast and/or multicast within the CFR.

[Medium] Updated Proposal 1-5:

Regarding the CORESETs for support of MBS, at least support Option 1, FFS whether Option 2 is optionally supported.

- Option 1: The maximum number of CORESETs per BWP is not increased for support of MBS. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

- Option 2: The maximum number of CORESETs per serving cell is not increased for support of MBS, but the maximum number of CORESETs per BWP can be increased to up to N based on UE capability for support of MBS. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

  - FFS: the value of N

[High] Updated Proposal 1-6:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

[High] Initial Proposal 1-7:

Zero or one CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

- FFS: whether more than one CFR is supported per dedicated unicast BWP

## Company Views (2nd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 1-1: OK. 1-2: Agree. 1-3: Agree. 1-4: Agree. 1-5: Agree. 1-6: The current proposal does not make any progress. Is it possible to agree at least Option 1 then FFS other options? |
| Samsung | Agree with all proposals except 1-2. Clarifications/restrictions are needed for proposal 1-2 (e.g. same numerology for the BWPs) as BWPs can have independent configurations and it is not generally possible for a UE, without introducing new capabilities, to simultaneously support 2 active DL BWPs. |
| Nokia, NSB | 1-1: OK 1-2: We support this proposal. However, we also agree with Samsung here, that there are potentially numerous conditions for this to be met without added UE capabilities, e.g. same SCS, same center frequency, new UE capabilities, etc., which needs to be clarified in the proposal. 1-3: OK 1-4: OK 1-5: OK 1-6: OK 1-7: OK |
| Qualcomm | 1-2: not agree with the current proposal. As explained before, we think is CFR in Opt2A is not in same level as that of the active dedicated BWP. 1-3: for multicast UEs with a dedicated BWP, either reference to Point A or a dedicated BWP can work. But if a broadcast CFR configured for IDLE/INACTIVE/CONNECTED UEs, it seems straightforward to use reference to Point A, where no dedicated BWP is available for IDLE/INACTIVE UEs and the dedicated BWP per CONNECTED UE may be different. Therefore, we prefer original 1-3. 1-6, we don’t agree ‘at least Option 1’. Whether the unicast pdcch-config can be used for multicast is not only dependent on the CORESET frequency range. Other parameters for unicast PDCCH may be different and cannot be used for GC-PDCCH. We prefer Option 4. If a pdcch-config in CFR can be shared by multicast and unicast, it means the parameters aligned among multicast UEs can be applied to the unicast transmission per UE. 1-7, it is still not clear how to let UE know multicast is in the dedicated BWP if zero CFR is configured. We think CFR is needed. But it is possible that some parameters (e.g., start PRB/range if frequency range is same as dedicated BWP) are not configured and use unicast parameters as a default value. |
| Convida | For Updated Proposal 1-1: Fine with the proposal. For Updated Proposal 1-2: From our view, we think the dedicated MBS BWP does NOT necessarily need to be activated as long as it is confined within the unicast BWP and has the same numerology/SCS. However, we are also fine to mandate activating two active BWPs simultaneously if companies are OK with it. For Updated Proposal 1-3: Fine with the proposal. For Updated Proposal 1-4: Fine with the proposal. For Updated Proposal 1-5: Fine with the proposal. For Updated Proposal 1-6: Fine with the proposal. For Updated Proposal 1-7: Fine with the proposal. |
| LG | P1-1: We can live with the updated proposal. P1-2: We should clarify that one of the two simultaneously active BWPs is the CFR and we do not support switching between unicast BWP and the CFR. [High] Updated Proposal 1-2: If Option 2A of CFR is supported for multicast of RRC-CONNECTED UE, UE needs to support at most two simultaneously active BWPs, at most one active unicast BWP and at most one active CFR within the frequency resource of the unicast BWP. Switching between the unicast BWP and the CFR is not supported. P1-3: We are fine with the updated proposal. P1-4: The proposal is misleading because we would not have two simultaneous active BWPs for unicast. Unicast is scheduled within UE’s active BWP (i.e. as in REL-15/16) which can associated to the CFR. CFR has no configuration of PDCCH/PDSCH for unicast. We do not think that unicast is scheduled for both UE’s active BWP and the associated CFR. We propose to revise P1-4 as follows: [High] Updated Proposal 1-4(for conclusion): It is based on gNB implementation to decide whether and how to schedule unicast on a unicast BWP and/or multicast on a CFR within the frequency resource of CFR. P1-5: We prefer Option 2. P1-6: OK for further study. We prefer Option 3. We do not need to change unicast for multicast. P1-7: We are fine with the initial proposal without FFS. We do not need more than one CFR. |
| Huawei/HiSilicon | 1-1, unclear about “commonality between RRC….”, so far, to my knowledge, RAN2 has only agreed broadcast to be received in RRC CONNECTED but the configuration comes from MCCH/MTCH different from the configurations from RRC dedicated signaling for multicast. RAN2 has not agreed multicast to be received in IDLE/INACTIVE states. It’s unclear to me what commonality we actually mean to be considered. 1-2, 1-3, hesitate to spend time on discussing the specific issues for option2A/2B, prefer to focus on the common issues for these two options which would be more time efficient for the discussion. 1-4, ok for conclusion 1-5, ok to at least support option 1 and FFS option2. 1-6, as commented in the last round which is also echoed by some companies, we don’t see the need to discuss these options. We have agreed to have a separate PDCCH-config for multicast (probably optional), then all the things could be left to network to have proper configuration, for example, configuring the same CORESETS/seach space in different configurations if intends to share the common frequency resources for scheduling unicast and multicast. 1-7, ok in principle. As QC commented, it would be better to clarify how multicast config is indicated if no CFR. |
| CATT | For the updated proposals: Proposal 1-1: It seems like that it is very difficult to down select from the two options. There is no necessary to agree such an agreement only about down selection of two options. According to the discussion by now, more technical details discussion/agreements of both options can facilitate our discussion and decision. We can also live with the updated proposal 1-1 even we think it might not be necessary. Proposal 1-2: OK. Proposal 1-3: Support. Proposal 1-4: Support. Proposal 1-5: Generally OK. Proposal 1-6: Generally OK. Proposal 1-7: ONLY support the main bullet without FFS in the sub-bullet. |
| NTT DOCOMO | Proposal 1-1: We are fine with the proposal. Proposal 1-2: We are fine with the proposal. Proposal 1-4: We are fine with the proposal. Proposal 1-5: We are fine with the proposal. Proposal 1-6: We are fine with the proposal. We prefer Option 1 and 3. We don’t see clear motivation to make the CORESET which configured in PDCCH-Config for unicast and fully contained in the CFR unusable for multicast. Whether the CORESET configured in PDCCH-Config for multicast is used for unicast transmission can be up to configuration. Proposal 1-7: We are fine with the proposal. |
| Google | P1-2: From our view, introducing multiple active BWPs is out of scope for MBS in RAN1. Down selecting from Option 2A and 2B is the matter RRC structure and naming, and it should up to RAN2’s decision. What RAN1 can do is to inform RAN2 that if they want to reuse the BWP RRC structure for CFR, BWP or BWP-to-CFR switching is not needed as long as the CFR is configured under a dedicated BWP with the same numerology/SCS. |
| vivo | 1-1: OK 1-2: OK 1-3: Same view with QC 1-4: OK 1-5: OK 1-6: OK 1-7: same concern as QC |
| ZTE | We are ok with Proposal 1-1, 1-4 and 1-6. For Proposal 1-2, we share the view of FL. That is, it is necessary to activate two BWPs simultaneously, but only a MBS BWP with a specific relationship with the unicast BWP can be activated simultaneously. For example, the MBS BWP should have the same SCS and CP configurations and are included within the unicast BWP, etc. In this sense, it is consistent no matter whether the CFR is defined as a physical BWP or a virtual BWP under option 2A. Further, we fail to find the essential difference on UE's complexity/capability among receiving two physical BWPs simultaneously, receiving one physical BWP and one virtual BWP simultaneously, or even receiving one physical BWP and one CFR simultaneously. What's more, the main motivation of option 2A is to reuse the existing working mechanism and signaling configuration structure of the BWP, so as to save the standardized work for both RAN1 and RAN2 on define functions and signaling configuration details of the CFR, and to deliver it to the gNB's configuration. This motivation is also common for both physical BWP and virtual BWP. Therefore, we do not think the two implementation modes under option 2A are mutually exclusive, but believe that there are many ways to form a unified solution. For example, for reusing the existing working mechanism of the physical BWP, a separated BWP ID can be configured for the MBS BWP no matter it is defined as a physical BWP or a virtual BWP. For Proposal 1-3, the CFR is one common frequency resource range for a group of users, which requires a common start for reference. Point A and start of the carrier bandwidth are both OK for reference. Our preference is the start of carrier bandwidth. But we can also compromised to use Point A as the reference for progress. However, we don’t see reason to apply the start of BWP as the reference, it requires UE to recalculate the CFR whenever BWP switching happens even though the CFR location is not changed. Besides, if CFR is allowed to be larger than the initial BWP for IDLE UE, it is not possible to use the start of initial BWP as the reference in this case. In other words, using start of BWP as the reference is not a common solution for both IDLE and CONNECTED. For Proposal 1-5: One compromised solution to agree that Option1 is the baseline and Option2 is optionally supported, e.g., Regarding the CORESETs for support of MBS, at least support Option 1 as baseline, FFS whether Option 2 is optionally supported. For Proposal 1-7: Similar view as other companies, we are not clear how to receive MBS with zero CFR configured. More clarification is needed. |
| Spreadtrum | Proposal 1-2: the proposal extends beyond MBS, where two simultaneously active BWPs could be supported, which is beyond Rel-15/16 UE capability. Fine with other proposal other than Proposal 1-2. |
| MTK | Proposal 1-1: Generally OK. Proposal 1-2: OK Proposal 1-3: OK Proposal 1-4: OK Proposal 1-5: OK with updated proposal. Proposal 1-6: OK Proposal 1-7: we are ok with the main bullet. |
| OPPO | Proposal 1-1: Share similar view as Huawei, the “commonality” is unclear for us. For RRC_CONNECTED UE the CFR is configured within UE dedicated unicast BWP, whereas RRC_IDLE UE does not have dedicated BWP, definitely there is no common solution for both. If we are not trying to make the down-selection this meeting rather to do that next meeting, we suggest to agree on some criteria for down-selection this meeting, otherwise, it may not be easy to down-select within one meeting. From our perspective, the following criterion could be considered: - Option 2A is selected if this option does not introduce additional active BWP or BWP switching, otherwise Option 2B is selected. Proposal 1-2: Even CFR is configured as MBS dedicated BWP, as it is confined within unicast dedicated BWP and has same SCS/CP, UE can receiver group common PSCCH/PDSCH without activating the MBS dedicated BWP. Proposal 1-3: This issue should not be discussed until Option 2B is down-selected. Proposal 1-4: ok Proposal 1-5: Seems this proposal does not make any progress, if Option 2 is agreed later, Option 1 is not applicable anymore, do we need to agree on it given Option 2 is still open. Proposal 1-6: Fine to further study, but we think the COREST for unicast and multicast should be indicated explicitly by respective PDCCH-config. |
| CMCC | 1-1: OK 1-2: OK, in our understanding, for Option 2A, two active BWPs contains one active BWP configured for unicast and one active BWP configured for multicast. 1-3: OK, as the concern about the common design about IDLE/INACTIVE UE, we think we can also extend the same method for IDLE/INACTIVE UE, that is for IDLE/INACTIVE UEs, the starting PRB of CFR is referenced to the starting PRB of the initial DL BWP, i.e., CORESET #0. 1-4: OK 1-5: OK 1-6: OK 1-7: OK There is another question about this proposal, that is how UE derive the multicast service configuration？ If the multicast service is configured bundling with the CFR configuration, there may be some problem when no CFR is configured. If gNB first transmits some multicast service related configuration, e.g., TMGI, UE will know it is configured with multicast service, but the detailed physical layer parameters configuration have not been configured, e.g., CFR, PDSCH/PDCCH config, UE can still work with zero CFR. |
| Ericsson | P1-1: We agree P1-2: We agree with observation of the proposal. We dislike the need to use two active BWPs but see this as a logical implication of 2A, which is one reason why we prefer 2B. P1-3: We realize that this is the majority view, but we fail to see the technical benefit of using the starting PRB. We have explained why using point A has a technical merit. P1-4: We agree P1-5: We agree P1-6: We agree but would like to express our preference for Option 1. P1-7: We agree |
| Apple | 1-2: For option 2A, the MBS BWP has the restriction that is confined within dedicated BWP and has the same CP/SCS as the dedicated BWP, don’t see any issue of BWP switching, there is no switching at all. Checking the definition of CFR defined in last meeting, it’s almost exactly same as defining a BWP. If using the MBS BWP, there is no issue of additional CORESET configuration. 1-3: don’t see any technical merits to using the starting PRB as the reference point. Point A is common and always there, the offset can be broadcast; but starting PRB is UE dedicated BWP specific, which need UE dedicated signaling, and it’s not friendly to gNB implementation. 1-5: make it clear that this proposal is only applied to Option 2B or CFR. |
| Moderator | Proposal 1-1: Huawei and OPPO still have some concern on the commonality, so I deleted the second part to just set a deadline for the down-selection. Basically we should consider what we have to consider when we do the down-selection. @OPPO, thanks for your advice to try to agree some principles for down-selection, but I feel it is hard for companies to agree the condition you suggested at the moment. We can first to have some common understanding on option 2A in proposal 1-2, then we can consider whether we can add some principles for the down-selection. Proposal 1-2: Based on the comments, I tried to list the key points for Option 2A. Companies are encouraged to further refine the proposal, and hope we can have a common understanding on it. Proposal 1-3: Based on the comments, I think both point A and the starting PRB of the dedicated BWP can work, the two options can also be taken into account when we do the down-selection. I think this issue may be easier to discuss after we have a conclusion on the down-selection, so I deferred the proposal. @QC, here we focus on CFR for multicast of RRC_CONNECTED state, for a broadcast CFR configured for IDLE / INACTIVE / CONNECTED UEs, it depends on the CFR design for IDLE/INACTIVE. Although we do not discuss the CFR design for IDLE/INACTIVE in this meeting, but at least in my understanding, it is possible that the CFR for IDLE/INACTIVE is also confined within the initial BWP (CORESET 0 or the initial BWP configured by SIB). Then the starting PRB of CFR for IDLE/INACTIVE can also be referenced to the initial BWP (CORESET 0 or the initial BWP configured by SIB), which is similar as updated proposal 1-3. Proposal 1-4: I reworded it based on LG’s comment. Proposal 1-5: We can try again to see if it is agreeable to have option 1 as baseline and Option 2 is optionally supported. If it is not agreeable, we will defer it. @Aplle, this proposal can be generally applied for both 2A and 2B. Proposal 1-6: It seems companies’ views diverge at this stage. I will leave it for further study. Proposal 1-7: Several companies propose to delete the FFS, but some prefer not, I think it is anyway FFS. Some prefer to clarify zero, and I explained the intention and deleted it and added an FFS to clarify. @QC, the intention of zero in this proposal is that, a UE is not mandated to be configured with CFR in every BWP. It is possible that some BWPs are configured with CFRs, and some are not. For a BWP that is not configured with CFR, whether UE can receive multicast or not needs to be discussed further. That relates to the discussion whether the use of CFR for multicast is optional or not, which is originally intentioned to be discussed in proposal 1-8, but deferred based on the comments in the 1st round. Anyway, we can delete zero at the moment. |

## Updated Proposals (after 2nd round of inputs)

[High] Updated Proposal 1-1:

The down-selection of Option 2A and Option 2B for CFR will be made before the end of RAN1#105-e.

[High] Updated Proposal 1-2:

If Option 2A of CFR is supported for multicast of RRC-CONNECTED UE, UE needs to support one active dedicated unicast BWP and one active MBS specific BWP simultaneously.

- (Same as in the agreement in RAN1#104) The MBS specific BWP is confined within the frequency resource of the dedicated unicast BWP. The MBS specific BWP is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

- The MBS specific BWP cannot be activated/deactivated independently.

  - the MBS specific BWP is active only when its associated dedicated unicast BWP is active.

- no BWP switching between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

- The MBS specific BWP has its own bwp-Id which is different from the bwp-Id(s) of dedicated unicast BWP(s).

  - FFS whether the MBS specific BWP is counted in the maximum 4 BWPs that can be configured per serving cell or not

[High] Updated Proposal 1-4(for conclusion):

It is based on gNB implementation to use the frequency resources in a CFR for unicast and/or multicast.

[Medium] Updated Proposal 1-5:

Regarding the CORESETs for support of MBS, take Option 1 as baseline, and Option 2 is optionally supported.

- Option 1: The maximum number of CORESETs per BWP is not increased for support of MBS. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

- Option 2: The maximum number of CORESETs per serving cell is not increased for support of MBS, but the maximum number of CORESETs per BWP can be increased to up to N based on UE capability for support of MBS. The CORESETs include the CORESETs configured in the dedicated unicast BWP and the CORESETs configured in the CFR which is confined within the dedicated unicast BWP.

  - FFS: the value of N

[High] Updated Proposal 1-6:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

[High] Initial Proposal 1-7:

One CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

- FFS: whether more than one CFR is supported per dedicated unicast BWP

- FFS: whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP

## Company Views (3rd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## Updated Proposals (after 3rd round of inputs)

# Issue #2: PDCCH configuration for MBS

## Background and submitted proposals

Background

In RAN1#104-e, the following agreements were achieved.

Agreement:

The maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- FFS whether the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

Working Assumption:

Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

Agreement:

For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, at least support CSS

- FFS: reuse existing CSS type(s) in Rel-15/16 or define a new Type CSS

- FFS: Two options for monitoring priority:

  - Option 1: the monitoring priority is the same as existing Rel-15/16 CSS

  - Option 2: the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

Submitted Proposals

- Huawei, HiSilicon

  - Proposal 4: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state,

    - Reuse Type3-CSS with monitoring priority kept the same as the current specification defined.

  - Proposal 5: DCI formats 1_0, 1_1 and 1_2 can be used for scheduling multicast with necessary modifications, and new DCI format is not needed:

    - For a common multicast frequency region for multicast configured within dedicated unicast BWP and a group-common PDCCH based scheduling, the FDRA field in DCI is dimensioned per the common multicast frequency region.

  - Proposal 6: The existing “3+1” DCI size budget should be kept for multicast, and DCI size for multicast at least should be aligned with DCI format 1_0 if used for scheduling.

  - Proposal 7: Re-distributing the BD/CCE limit among serving cells can be supported subject to UE capability.

- OPPO

  - Proposal 7: A new DL DCI format should be defined for the scheduling of group-common PDSCH.

  - Proposal 8: The G-RNTI is counted as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

  - Proposal 9: The size of the group common DCI is configurable up to 126 bits.

  - Proposal 10: For a UE receiving group-common PDSCH transmitted with PTM scheme 1, a TPC-PUCCH-RNTI different from that for unicast should be configured.

  - Proposal 11: The budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration.

  - Proposal 12: A new CSS type is defined for group-common PSCCH transmission, the monitoring priority of the new CSS is determined based on the index associated with the CSS.

- Spreadtrum

  - Proposal 5: For search space type for Rel-17 MBS, support to define a new search space type for multicast.

  - Proposal 6: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS.

  - Proposal 7: For group-common DCI for MBS, both DCI 1_0, and DCI1_1/1_2 could be considered as the starting point.

- ZTE

  - Proposal 4: For MBS group PDCCH,

    - DCI format 1_0 can be defined as a baseline DCI format.

    - A DCI format based on either DCI format 1_1 or DCI format 1_2 can be further supported for capacity improvement.

  - Proposal 5: Regarding Rel-17 NR MBS,

    - Define a new Type x-PDCCH CSS set for the group common PDCCH.

    - The monitoring priority of the group-common PDCCH with DCI format 1_0 is the same as existing Rel-15/16 CSS.

    - The monitoring priority of the group-common PDCCH with DCI format 1_x is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

  - Proposal 7: For MBS group-common PDCCH, the budget of BDs/CCEs of an unused CC for group-common PDCCH can be used for UEs supporting CA capability in Rel-17 MBS.

  - Proposal 8: Regarding DCI size alignment used for group-common PDCCH,

    - DCI format 1_0: it is counted as “C-RNTI”, and current mechanism can be reused for determining the size of DCI format 1_0 for group-common PDCCH and unicast PDCCH.

    - DCI format 1_x: it is counted as “other RNTI”, and gNB will ensure that the number of DCI sizes does not exceed budget.

- vivo

  - Proposal 11: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state,

    - Reuse the existing CSS type(s) in Rel-15/16 with modification or define a new type CSS

    - Option 2: the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

- CATT

  - Proposal 21: For search space set of group-common PDCCH of PTM scheme 1, existing CSS type(s) in Rel-15/16 is reused.

  - Proposal 22: The monitoring priority is the same as existing Rel-15/16 CSS.

  - Proposal 23: USS can also be considered for group-common PDCCH of PTM scheme 1.

  - Proposal 24: Whether the budget sharing of DBs/CCEs of an unused CC can be supported is based on per UE capability.

  - Proposal 25: G-RNTI is counted as one of the “other RNTIs” in 3+1 DCI size budget rule for group-common PDCCH.

- Nokia

  - Proposal-21: Agree that the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration.

  - Proposal-24: Define a new CSS type for multicast rather than reusing an existing Rel-15/16 CSS type.

  - Proposal-25: Agree that the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

- MediaTek

  - Proposal 7: Type 3-PDCCH CSS with little modification (e.g., support G-RNTI) can be reused for multicast group common PDCCH monitoring and corresponding initialization value of  Yp,ns,fμ can be defined as G-RNTI value.

  - Proposal 9: Keep the “3+1” DCI size defined in Rel-15 for Rel-17 MBS and “G-RNTI” is counted as “C-RNTI”.

  - Proposal 10: DCI format 1_X can be as a baseline for multicast group-common PDSCH scheduling.

- FUTUREWEI

  - Proposal 5: The number of BDs/CCEs follow Rel-15 limits for UEs without CA capability and for UEs with CA capability, the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

  - Proposal 6: Reuse the existing CSS type(s) in Rel-15/16 for the search space of group-common PDCCH.

- CMCC

  - Proposal 5. Define a new CSS type for group-common PDCCH of PTM transmission scheme 1 for multicast in RRC_CONNECTED state.

  - Proposal 6. For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, support Option 2: the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

  - Proposal 9. For PTM transmission scheme 1, both fallback DCI format 1_0 and non-fallback DCI format 1_1/1_2 could be considered, and some fields in DCI format 1_1/1_2 are not needed.

  - Proposal 10. Confirm the working assumption of keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

    - The G-RNTI is counted as “C-RNTI”.

  - Proposal 11. Align the DCI size of DCI format 0_2/1_2 or DCI format 0_1/1_1 with the G-RNTI DCI size after current steps in Rel-16 DCI size alignment procedure.

    - The G-RNTI DCI size can be configured by gNB, which is larger than the original calculation of bitlength of DCI fields according to configurations.

- Intel

  - Proposal 13: For determining BD/CEE limits for NR MBS in Rel-17, Option 1 should be supported for UEs without CA capability and Option 2 should be supported for UEs with CA capability. Down-selection is not necessary.

  - Proposal 14: Search space set configuration for monitoring DCI scheduling multicast PDSCH can re-use NR Type 3 CSS configuration while additionally supporting monitoring of DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI. Alternately, a new NR CSS Type can be defined for monitoring multicast DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI

  - Proposal 15: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS and USS (if supported)

  - Proposal 16: DCI Format for scheduling NR MBS transmissions:

    - Delivery Mode 1 (high QoS): DCI formats 1_1, 1_2 can be used. If needed, a compact DCI format for multicast scheduling can be defined

    - Delivery Mode 2 (low QoS): DCI format 1_0 can be used since the group of UEs can also include RRC_IDLE/INACTIVE mode UEs

  - Proposal 17: Working assumption on keeping “3+1” DCI size budget should be confirmed and G-RNTI should be counted together as one of the “3” C-RNTIs and not as “Other RNTI”.

- Apple

  - Proposal 3: Define a new common search space type for multicast. The monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

  - Proposal 4: Confirm the working assumption: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- Qualcomm

  - Proposal 6: For RRC_CONNECTED UEs, define a new type of CSS for MBS GC-PDCCH.

    - The monitoring priority of GC-PDCCH in case of overbooking is determined based on the search space set indexes.

  - Proposal 7: For RRC_CONNECTED UEs, DCI format 1_0/1_1/1_2 can be used for GC-PDCCH.

    - Confirm the WA: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS

    - DCI size if over the size budget is aligned between GC-PDCCH and unicast PDCCH using the same DCI format (G-RNTI is counted as C-RNTI).

  - Proposal 8: For RRC_CONNECTED UEs, support reusing BDs/CCEs of an unused CC for group-common PDCCH for MBS.

    - NOTE: This is similar to the method used for multi-DCI based multi-TRP in Rel-16.

    - FFS details of UE capability.

- Samsung

  - Observation 3: Determining number of PDCCH candidates/CCEs based on activated/non-dormant BWP SCells instead of configured cells can be useful but it relates to CA enhancements.

  - Proposal 2: The size of the DCI format scheduling MBS PDSCH is counted together with the sizes of unicast DCI formats.

  - Proposal 3: Revisit the WA on keeping the “3+1” DCI size budget after concluding the DCI format for MBS PDSCH.

  - Proposal 4: The DCI format for scheduling MBS PDSCH is based on DCI format 1_2 or is a new DCI format.

  - Proposal 5: The monitoring priorities of search space sets for MBS PDCCH are determined according to the corresponding search space set indexes as for USS sets in Rel-16.

- LGE

  - Proposal 5: support CSS Type 3 for group common PDCCH for connected UEs as well as idle/inactive UEs.

  - Proposal 6: support additional new CSS type 4 for multicast of which monitoring priority is handled like USS.

  - Proposal 7: the G-RNTI is “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH for less impact. If necessary, the total number of different DCI sizes configured to monitor could be increased up to 5 for the cell where CFR is configured, while the total number of different DCI sizes with C-RNTI configured to monitor is kept as 3.

- Lenovo

  - Proposal 7: The number of bits for frequency domain resource assignment indicator in DCI is determined based on the bandwidth of the common frequency region.

  - Proposal 8: A common CORESET is configured within the common frequency region for MBS for the group of UEs.

  - Proposal 9: A common search space is configured associated with the common CORESET for MBS for the group of UEs.

  - Proposal 10: DCI format 1-0 with CRC scrambled by G-RNTI is used as the group-common DCI.

  - Proposal 11: For DCI size alignment, DCI format with CRC scrambled by G-RNTI is counted as the DCI format with CRC scrambled by C-RNTI.

- NTT Dococmo

  - Proposal 3: For scheduling group-common PDSCH of PTM scheme 1, use DCI format 1_0, remove fields not used for MBS and introduce new fields for other purposes (e.g., 2 layer MIMO).

    - Note: maintain the same DCI size as existing DCI format 1_0

  - Proposal 4: Reuse Type3-PDCCH CSS for search space set of group-common PDCCH of PTM scheme 1.

  - Proposal 5: Support Option 1 for the monitoring priority of search space set of group-common PDCCH in PTM scheme 1.

  - Observation 2: The benefit of using the budget of BDs/CCEs of an unused CC is small.

- Ericsson

  - Proposal 25: Group common PDCCH and unicast PDCCH can be configured within the same CORESET

  - Proposal 26: Extend the  existing type3 CSS from Rel-15/16 to support

    - Additional DCIs for scheduling via group common PDCCH

    - Support of G-RNTI(s)

  - Proposal 27: The priority of search space for multicast is higher than UE specific search space but lower than the existing common search space defined in R15/R16.

  - Proposal 28: Based on UE capability, a UE may use the budget of BDs/CCEs of an unused CC for a group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration.

  - Proposal 29: Specify one fall-back and one non-fallback DCI for group scheduling of PDSCH via group-PDCCH.

  - Proposal 30: The  G-RNTI is counted as   “C-RNTI”  when considering the “3+1” DCI size budget rule for group-common PDCCH.

  - Proposal 31: A new, non-fallback DCI format for MBS downlink scheduling is introduced e.g. DCI 1_3, present in the common search space and based on DCI 1_1

    - FFS: details of the fields in DCI 1_3

  - Proposal 32: The determination of DCI 1_3, monitored in the common search space  is inserted as step ”2B” in the DCI alignment procedure

  - Observation 12: For MBS Fallback CI format, legacy DCI format 1_0, can be reused in the CSS without requiring additional Blind decoding and without requiring DCI size alignment between unicast and multicast.

  - Proposal 33: Reuse DCI 1_0 as fallback DCI format for MBS, with CRC scrambled with G-RNTI.

    - FFS: details of the MBS fallback DCI format fields.

  - Proposal 34: When scheduling with non-fallback DCI, Scrambling parameters n_ID and n_RNTI for group PDCCH DMRS in the CSS is given by pdcch-DMRS-ScramblingID and the group PDCCH G-RNTI, respectively.

  - Proposal 35: Scrambling parameters n_ID and n_RNTI for group PDSCH schedule by the multicast non-fallback DCI in CSS is given by

    - N_RNTI is given by G-RNTI

    - n_ID =  the higher-layer parameter dataScramblingIdentityPDSCH  if CORESETPoolIndex is not configured

    - if the higher-layer parameters dataScramblingIdentityPDSCH and dataScramblingIdentityPDSCH2 are configured together with the higher-layer parameter CORESETPoolIndex containing two different values

    - n_ID =  the higher-layer parameter dataScramblingIdentityPDSCH if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 0

    - n_ID = the higher-layer parameter dataScramblingIdentityPDSCH2 if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 1;

## Initial Proposals based on contributions

Summary

Regarding the search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state,

- 8 companies [Huawei, CATT, MTK, Futurewei, Intel, LGE, Docomo, Ericsson] propose to reuse existing Type-3 CSS, and most of them propose that the monitoring priority for multicast is kept the same as existing CSS defined in R15/R16 (i.e., option 1), except that 1 company [Ericsson] proposes the monitoring priority for multicast is higher than USS but lower than the existing CSS defined in R15/R16.

- 9 companies [OPPO, Spreadtrum, ZTE, vivo, Nokia, CMCC, Apple, Qualcomm, LGE] propose to define a new type CSS, and most companies propose the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets (i.e., option 2), except 1 company [ZTE] proposes a compromised solution that the monitoring priority of the group-common PDCCH with DCI format 1_0 is the same as existing Rel-15/16 CSS, but the monitoring priority of the group-common PDCCH with non-fallback DCI format is determined based on the search space set indexes of search space set(s) for multicast and USS sets. 1 company [Samsung] also proposes option 2 for monitoring priority, but thinks whether the search space set of group-common PDCCH is considered a new CSS type or a Type-3 CSS is not a design issue and need not be discussed. Regardless whether it is a new CSS type or a Type-3 CSS, it seems there is not a majority view for the monitoring priority.

- From moderator point of view, there may be two way forwards. One is what ZTE suggested, i.e., define a new Type-x CSS set for the group common PDCCH, the monitoring priority of the group-common PDCCH with DCI format 1_0 is the same as existing Rel-15/16 CSS, and the monitoring priority of the group-common PDCCH with non-fallback DCI format is determined based on the search space set indexes of search space set(s) for multicast and USS sets. However, this way forward is based on a prerequisite that both fallback DCI format and non-fallback DCI format are supported for multicast. The other way forward is that both Type-3 CSS and a new Type-x CSS are supported for multicast, and when Type-3 CSS is used for group-common PDCCH of PTM scheme 1, option 1 is supported for the monitoring priority, and when new Type-x CSS is used for group-common PDCCH of PTM scheme 1, option 2 is supported for the monitoring priority.

Regarding DCI format of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state,

- 10 companies think at least DCI format 1_0 can be supported with possible modifications (e.g., remove fields not used for MBS and introduce new fields for other purposes 2 layer MIMO proposed in [23]) and maintain the same DCI size as existing DCI format 1_0 defined in Rel-15/16.

- 7 of them think DCI format 1_1 or 1_2 can be additionally supported with possible modifications.

- 2 companies propose to define a new DCI format, and 1 company propose to base on DCI format 1_2 or define a new DCI format.

Regarding the FFS of maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell,

- 9 companies explicitly propose to support that the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability.

- 1 company thinks “unused CC” is interpreted to be deactivated SCells, or SCells having a dormant BWP as active DL BWP, and determining number of PDCCH candidates/CCEs based on activated/non-dormant BWP SCells instead of configured cells can be useful but it relates to CA enhancements.

- 1 company thinks the benefit of using the budget of BDs/CCEs of an unused CC is small.

Regarding the working assumption for DCI size budge, 4 companies propose to confirm it, while 1 company propose to revisit it after concluding the DCI format. Regarding whether the G-RNTI is counted as “C-RNTI” or as “other RNTI”, it seems companies’ views diverge. 7 companies propose to count the G-RNTI as “C-RNTI”, 3 companies propose to count the G-RNTI as “other RNTI”, 1 company proposes that for fallback DCI the G-RNTI is counted as “C-RNTI”, while for non-fallback DCI the G-RNTI is counted as “other-RNTI”. Moderator think maybe we can revisit this working assumption after we have more progress on DCI format and give companies time to converge.

Initial Proposals

The following moderator recommendations are made.

[Moderator’s recommendation]

[High] Initial Proposal 2-1:

For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following two alternatives:

- Alt 1: support a new Type-x CSS,

  - the monitoring priority of the group-common PDCCH with DCI format 1_0 (if supported) is the same as existing Rel-15/16 CSS,

  - the monitoring priority of the group-common PDCCH with non-fallback DCI format (if supported) is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

- Alt 2: support both Type-3 CSS and a new Type-x CSS,

  - if Type-3 CSS is used for group-common PDCCH, the monitoring priority is the same as existing Rel-15/16 CSS

  - if new Type-x CSS is used for group-common PDCCH, the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

[High] Initial Proposal 2-2:

For group-common PDCCH of Rel-17 MBS, support one fallback DCI and one non-fallback DCI.

- reuse DCI format 1_0 as fallback DCI format with CRC scrambled with G-RNTI.

  - FFS details of the fallback DCI format fields.

- FFS non-fallback DCI format is based on DCI format 1_1 or 1_2

  - FFS details of the non-fallback DCI format fields

[High] Initial Proposal 2-3:

In Rel-17 MBS, for RRC_CONNECTED UEs supporting CA capability, support to determine the number of PDCCH candidates/CCEs based on activated/non-dormant BWP SCells instead of configured cells.

## Company Views (1st round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 2-1: agree. 2-2: We support using DCI format 1-0 with G-RNTI as the group-common DCI. Regarding using the non-fallback DCI as the group-common DCI, does it imply the non-fallback DCI will be transmitted in the CSS? 2-3: agree. |
| Spreadtrum | For proposal 2-1, in Rel-15/Rel-16, the PDCCH monitoring priority is based on SS, not DCI format, thus we would like to add one alternative as bellow: [High] Initial Proposal 2-1: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following two alternatives: - Alt 1: support a new Type-x CSS, - Alt 1-1: - the monitoring priority of the group-common PDCCH with DCI format 1_0 (if supported) is the same as existing Rel-15/16 CSS, - the monitoring priority of the group-common PDCCH with non-fallback DCI format (if supported) is determined based on the search space set indexes of search space set(s) for multicast and USS sets. - Alt 1-2: the monitoring priority of the group-common PDCCH is determined based on the search space set indexes of search space set(s) for multicast and USS sets. - Alt 2: support both Type-3 CSS and a new Type-x CSS, - if Type-3 CSS is used for group-common PDCCH, the monitoring priority is the same as existing Rel-15/16 CSS - if new Type-x CSS is used for group-common PDCCH, the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets. For proposal 2-2, fine with FL’s proposal. |
| Apple | For Proposal 2-1, it could be better to move forward step by step. First, to agree on whether Alt. 1 or Alt. 2 is supported, then to determine the monitor priority. We support main bullet of Alt.1. We are ok with Proposal 2-2 For Proposal 2-3, agree the intention of this proposal. for us, it’s not clear the difference between activated Scell and unused CC. But anyway, to make the proposal clearer, it can be updated as below. In Rel-17 MBS, for RRC_CONNECTED UEs supporting CA capability, the budget of BDs/CCEs of an activated/non-dormant BWP SCell (or unused CC) can be used for group-common PDCCH to count the number of BDs/CCEs |
| CATT | Proposal 2-1: OK. Proposal 2-2: OK. Proposal 2-3: Generally OK with the proposal. Besides, when some of the UEs in the same group do not support CA, how to deal with this case of UEs with different capabilities? |
| OPPO | Proposal 2-1: We support a new Type-x CSS, and the monitoring priority of this CSS is determined based on its search space index. Seems the 2 alternatives now do not cover this option. For the first sub-bullet of Alt.1, does it mean the fallback DCI if supported is transmitted in Rel-15/16 CSS? Proposal 2-2: We failed to understand the motivation of introducing fallback DCI, it is for a compromised solution of search space set design, or the fallback DCI has other indispensable functionalities for MBS? Proposal 2-3: Agree. |
| ZTE | For Proposal 2-1: We support Alt.1. For Alt.2, we have to define two search space types for MBS, which will unnecessary complicate the standardization and implementation potentially. For Proposal 2-2: We support one fallback DCI plus one non-fallback DCI for MBS scheduling. It would be too restrictive if we can only use fallback DCI to schedule MBS, which means all advanced features of unicast can’t be used for MBS. We would propose the following updated proposal. For group-common PDCCH of Rel-17 MBS, support one fallback DCI and one non-fallback DCI. - reuse DCI format 1_0 as fallback DCI format with CRC scrambled with G-RNTI. - FFS details of the fallback DCI format fields. - FFS non-fallback DCI format is based on DCI format 1_1 or 1_2 - FFS details of the DCI format and non-fallback DCI format fields For proposal 2-3: We think the current wording is a bit unclear. In fact, based on the current spec, determination of the number of PDCCH candidates/CCEs is based on all cells, including the deactivated SCell and SCell under dormant BWP. The current proposal seems to change this legacy behavior, which seems not to be the intention of this proposal. From our perspective, the intention is to reuse the MTRP mechanism by counting Cell receiving MBS as 2 cells. We can still use the term “virtual CC”, and editor will translate it in the specification language. |
| CMCC | 2-1: For Alt1, as the comment by Spreadtrum, the current PDCCH monitoring priority is based on SS not DCI format. In addition, whether to support both configure fallback DCI and non-fallback DCI should be clarified in Alt 1. If one new type CSS supports configuring two DCI formats, the current PDCCH overbooking rule needs some modification, because only non-fallback DCI formats can be dropped but not the whole SS. If one new type CSS only supports configuring one DCI format, i.e., either fallback DCI or non-fallback DCI, the function of Alt 1 equals to Alt 2. 2-2: Agree. 2-3: Agree. |
| NTT DOCOMO | Proposal 2-1: We think that proper CSS type depends on DCI format. We prefer to deprioritize CSS type until which DCI format is used for MBS is concluded. Proposal 2-2: We think that one fallback DCI is enough. It may be difficult to keep “3+1” DCI size budget if non-fallback DCI is supported. DCI formats to be used and DCI size alignment procedure should be considered together. Proposal 2-3: We are fine with FL’s proposal. |
| Huawei/HiSilicon | Proposal 2-1, we see either defining a new type CSS or reusign type-3 CSS, no need to support both type 3 and new type CSS. Proposal 2-2, not sure it is a good idea at this moment to preclude one of 1_1 or 1_2 especially when that one is used for unicast scheduling. Proposal 2-3, not sure what spec change is expected. If the configured number of cells does not exceed 4, is there spec impact? What if exceeding 4? |
| LG | P2-1: We are fine the initial proposal. P2-2: We are fine the initial proposal. P2-3: The initial proposal seems different than the current behavior. We do not understand the intention of change of the current behavior. |
| MTK | Proposal 2-1: Support Type-3 CSS with some modification, but we are open to discuss whether a new CSS is needed or how to understanding the “new” CSS. Proposal 2-2: The motivation is not clear to simultaneously define one fallback DCI and one non-fallback DCI. Reusing DCI format 1-0 as fallback DCI format can be as a baseline, whether to define a non-fallback can be FFS, we suggest the proposal can be modified as following: [High] Initial Proposal 2-2: For group-common PDCCH of Rel-17 MBS: - reuse DCI format 1_0 as fallback DCI format with CRC scrambled with G-RNTI. - FFS details of the fallback DCI format fields. - FFS non-fallback DCI format is based on DCI format 1_1 or 1_2 - FFS details of the non-fallback DCI format fields Proposal 2-3: We have the similar question as CATT noted. |
| Samsung | Proposal 2-1: Do not support. Needs further discussion. A simpler proposal for a decision on SS set index based prioritization (as for USS) or for Rel-16 CSS prioritization (multicast always prioritized over unicast) may first be considered. - For Alt. 1, there is no need to consider DCI format 1_0 – SS set prioritization is neither based on DCI format nor DCI format 1_0 has been agreed to use for multicast. Also note that PDCCHs in Type-3 CSS are infrequent or are optional UE features, and there won’t be another DCI 2_x for DCI 1_0 to be received together with in every slot. - Alt.2 seems to propose two different approaches to be supported. That is unnecessary and not agreeable. - We agree that a new Type-3 CSS is needed but it is only a partial solution because PDCCH candidates for all multicast services would overlap (and also overlap with Rel-16 CSS). Then, the question is how many candidates there should be to avoid permanent overlapping for all multicast services and whether there are any CCEs left. USS should also be considered as it accomplishes everything without spec impact. Proposal 2-2: Do not support. The proposal again includes DCI format 1_0 which requires discussion (we do not think DCI 1_0 is particularly meaningful for MBS although the size of DCI 1_0 can be discussed). Discussion on DCI formats for multicast and on achieving the “3+1” constraint is first needed. Discussion on whether or not to have fallback DCI and non-fallback DCI can be part of it. Proposal 2-3: Needs further discussion The intention of the proposal is beneficial but it relates to CA and not to MBS. The “In Rel-17 MBS” in the beginning of the proposal should be removed. The proposal does not increase the BD/CCE budget per slot on a scheduled cell – there is no difference between UEs that support CA and UEs that do not support CA for monitoring unicast PDCCH and GC-PDCCH – the maximum number of BDs/CCEs on a scheduled cell per slot is same, and same as in Rel-15. The proposal is beneficial in case there are multiple scheduling cells and more scheduling cells than scheduled cells (i.e. cross-carrier scheduling) – it does not relate to MBS. |
| Qualcomm | 2-1: We think the reason to define a new type of CSS is to make the monitoring priority configurable. If new type of CSS is defined, no need to use Type-3 CSS since the use case of Alt 2 (different priority) can now be covered by using the configured "SS index". Therefore, we prefer to the following modifications: [High] Initial Proposal 2-1: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, support a new Type-x CSS, - the monitoring priority of the group-common PDCCH is determined based on the search space set indexes. 2-2: Not necessary to limit only one non-fallback DCI for MBS. The DCI format 1_1 or 1_2 can be used for MBS, up to the gNB selection. 2-3: The current proposal is confusing. The PDCCH BDs/CCEs are based on the configured cells, which is not changed for mTRP case, neither to be changed for MBS case. [High] Initial Proposal 2-3: In Rel-17 MBS, for RRC_CONNECTED UEs supporting CA capability, support to count the number of PDCCH candidates/CCEs as the unused CC in the configured cells. NOTE: This is similar to the method used for multi-DCI based multi-TRP in Rel-16. |
| Futurewei | 2-3: Support |
| Nokia, NSB | Proposal 2-1: We agree with the down-selection proposal and support Alt-1. Proposal 2-2: We are fine with this proposal. Proposal 2-3: We are fine with this proposal. |
| Ericsson | P2-1: We prefer Alt2. Regarding Type-x CSS, we suggest adding an FFS for the exact list of supported DCIs. P2-2: We agree P2-3: We agree |
| Convida | For Proposal 2-1: We are fine with FL’s proposal. For Proposal 2-2: For the non-fallback DCI, we wonder if a new DCI format should be also considered and added in the FFS point? For Proposal 2-3: We are fine with FL’s proposal. |
| vivo | For Proposal 2-1, for Alt 1, one SS can be configured with multiple DCI formats, if the new type-x CSS is configured with both DCI format 1_0 and other non-fallback DCI format. Then how to determine the monitoring priority? For Alt 2, if the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets, then gNB can configure a type-x CSS with an index smaller than that for all USS, then what’s the motivation to support type-3 CSS? |
| Moderator | The following agreement has been agreed in the 1st GTW session. Agreement: For group-common PDCCH of Rel-17 MBS, support at least two DCI formats. - DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI. - DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI - FFS: Which of DCI format 1_1 or 1_2 is used as the baseline - FFS: Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2 fields Proposal 2-1: I reformulated the proposal based on the new agreement and companies’ comments. The original Alt 1 is updated as new Alt4, and original Alt2 is updated as new Alt 3, new Alt 1 and new Alt 2 are newly added but they are proposals from different companies as explained in the summary part of section 3.2. @vivo, regarding your first question, my understanding is different DCI formats of group-common PDCCH cannot be configured in the same new Type-x CSS. Regarding the second question, it is up to gNB to configure a type-x CSS with an index smaller or larger than that for all or some USS sets. Proposal 2-2: closed. Proposal 2-3: The proposal was updated to accommodate some comments, but some companies think this is a CA enhancement but not for MBS. My understanding is that the intention of this proposal is to borrow the budget of BD/CCEs of an unused CC for the BD/CCEs for group-common PDCCH in the active CC, so that the impact of introducing MBS on BD/CCEs for unicast can be alleviated. |

## Updated Proposals (after 1st round of inputs)

[High] Updated Proposal 2-1:

For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following two alternatives:

- Alt 1: support Type-3 CSS

  - The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

- Alt 2: support a new Type-x CSS

  - The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

- Alt 3: support both Alt 1 and Alt 2

  - If Type-3 CSS is used for group-common PDCCH, the monitoring priority is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

  - If new Type-x CSS is used for group-common PDCCH, the monitoring priority is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

- Alt 4: support a new Type-x CSS

  - If the first DCI format of group-common PDCCH is configured in new Type-x CSS, the monitoring priority of the new Type-x CSS is the same as existing Rel-15/16 CSS

  - If the second DCI format of group-common PDCCH is configured in new Type-x CSS, the monitoring priority of the new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets.

  - Different DCI formats of group-common PDCCH cannot be configured in the same new Type-x CSS.

[High] Updated Proposal 2-3:

For RRC_CONNECTED UEs supporting CA capability, the budget of BDs/CCEs of an unused CC (deactivated SCells or SCells having a dormant BWP as active DL BWP) can be used for group-common PDCCH to count the number of BDs/CCEs.

- FFS: how to deal with the case that UEs in the same MBS group have different CA capabilities

- NOTE: This is similar to the method used for multi-DCI based multi-TRP in Rel-16.

## Company Views (2nd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 2-1: Agree. 2-3: It seems this issue is a normal CA issue. Maybe it is better to focus on the MBS-specific issues. |
| Samsung | 2-1: OK but it would be good to down-select (else, the status is practically same as before). All proposals will result to fixed collisions among all PDCCH candidates and with Rel-16 CSS. Proponents should describe how that is resolved. USS should be supported as it avoids all problems and has no specification impact. From the listed alternatives, only Alt.2 can potentially work. Note that Rel-16 Type-3 CSS is optional for most DCI 2_x and even when it is present, it is infrequent (e.g. 10 msec or more periodicity) – same for non-Type-3 CSS. Alt.1 therefore does not work (i.e. multicast PDCCH needs to be in every slot and always has higher priority than unicast, even for URLLC - that is not because there is Rel-16 CSS). Alt.3 is unnecessary - combinations should not be supported unless it is critical to do so. Alt.4 has both the problems of Alt.1 and Alt.3. 2-3: There are still two problems with the proposal; one major, one minor. The major problem is that the proposal requires a new UE capability although it does not say so (and, of course, it does not say what such UE capability should be). The number of BDs/CCEs per slot per scheduled cell is at most MPDCCHmax,slot,μ/CPDCCHmax,slot,μ – and that number is same for UEs that support CA and for UEs that don’t. There seems to be a misunderstanding by many of the NR specifications. To see this differently, don’t even consider deactivated/dormant cells. Just assume a UE that declared a CA capability for 4 cells and is configured for single-cell operation. The available BDs/CCEs are not used in R15/R16 simply because the UE capability per slot per scheduled cell is at most MPDCCHmax,slot,μ/CPDCCHmax,slot,μ. The reference for the additional BDs/CCEs that a UE can use is not related to deactivated/dormant cells but on UE capability Ncellscap. Using the additional UE capability can increase MPDCCHtotal,slot,μ/CPDCCHtotal,slot,μ, not MPDCCHmax,slot,μ/CPDCCHmax,slot,μ. That is why the proposal is related to CA – it does not relate to supporting multicast on a scheduled cell. The minor problem is that the proposal limits the use of the leftover UE capability to group-common PDCCH. Why can it not be used for non-group-common PDCCH? |
| Nokia, NSB | 2-1: Agree – with “two” deleted, “following two alternatives:” 2-3: Agree with intention – would like wording improved. For example, are these additional BDs/CCEs only available for the MBS service (group-common PDCCH)? Could they also be used for the dedicated unicast associated with the MBS service/CFR? |
| Qualcomm | 2-1: we prefer Alt2. Alt1 is too restricted. Alt3 is not needed since Alt2 can enable a configuration of SS index to cover Alt1. Alt4 is over-complicated than the legacy way (i.e., based on SS index only). 2-3: We agree with Samsung and Lenovo that 2-3, as written right now, should not be done in MBS. Our interpretation of “unused CC” was completely different. Let us try to explain. Our proposal is to introduce a new UE capability, similar to blindDetectFactor-r16 (R) for mTRP. Both MPDCCHtotal,slot,μ/CPDCCHtotal,slot,μ and MPDCCHmax,slot,μ/CPDCCHmax,slot,μare increased based on γ=R scaling on a subset of the configured CCs, Ncells,1DL. Note that the total BDs/CCEs are still based on the configured CCs (not active CCs). For mTRP, the CC(s) with mTRP will be counted by scaling γ times. Here, what we want is to use the same mechanism for MBS. Instead of TRP1 / TRP2, do unicast / multicast. So, for the CC with multicast in a CFR, it will be counted by scaling γ times. It is a specific issue for multicast instead of a general CA problem. In the proposal, some companies think ‘unused CC’ is not clear, which is not used/defined in spec. Our understanding of the "unused CC" is related to UE capability signaling as follows: - UE indicates that it supports {CC1, CC2, CC3} with unicast only - UE indicates that it supports {CC1, CC2} with multicast in CC2 and scaling of γ=2 In the spec, the "unused CC" is not explicitly mentioned, is just that the UE would say "I support scaling of γ=R" in the bands that I support two CCs instead of 3CCs. When the UE reports the capabilities, the "unused CC" is implicit in the band combination. If the UE has multicast on CC1 or CC2, the CC3 cannot be configured (of course not be able to be activated). |
| Convida | For Updated Proposal 2-1: Fine with the proposal. For Updated Proposal 2-3: Agree with Lenovo, prefer to postpone the discussion and focus on other fundamental issues. |
| LG | P2-1: We are fine the updated proposal. |
| Huawei/HiSilicon | 2-1, ok to have Alt1 and Alt2 kept, others not necessary. Note main bullet is saying “two”. 2-3, Similar views as Samsung and that is also why I asked the spec impact for 4 cells or more than 4 cells in the last Round. I can understand QC’s intention better now after QC clarified. However, I suggest rewording the proposals more directly understandable, e.g., reflect how spec is going to change roughly, because spec is the common ground we could understand what the solution is. |
| CATT | For updated proposals: Proposal 2-1: Generally OK with it. For Alt 3, the intention is to support both Type-3 and new Type of CSS. According to current wording, the two sub-bullets under Alt 3 are the same with Alt 1 and Alt 2, respectively. I would like to suggest deleting the two sub-bullets under Alt 3. I understand the intention of adding them in Alt3 in case it is agreed and Alt 1 and Alt 2 will be deleted. If so, the main bullet can say “support following CSS types” and then keep Alt1 and Alt 2. Proposal 2-3: We can live with it, even we think it is based on UE capability and similar with Rel-16 multi-TRP mechanism and there is no necessary to make such an agreement. |
| NTT DOCOMO | Proposal 2-1: We are generally fine with the proposal. We prefer Alt 1 or Alt 2. New Type-x CSS should be supported to use DCI 1_1/1_2. In current specification DCI 1_1/1_2 can be transmitted only in USS. If DCI 1_1/1_2 is added to Type-3 CSS, it might unnecessarily increase UE processing to monitor CSS. Proposal 2-3: We are fine with the proposal. |
| vivo | 2-1: Agree – with “two” deleted, Alt 2 is preferred. 2-3: Same view with Lenovo |
| ZTE | For Proposal 2-1: Support. One minor update for the main bullet, “the following four two alternatives”. Besides, one minor change to the last bullet of Alt.4. “one” is added to clarify that the DCI formats are not allowed to be configured in the same type-x CSS, but they can be configured in different type-x CSS. - The first DCI format and the second DCI format of group-common PDCCH cannot be configured in the same one new Type-x CSS. For Proposal 2-2: It seems that companies still have different understanding on this. At least there are the following two interpretations. Interpretation#1: If UE is configured with 5 physical CCs and UE is configured to receive MBS on PCell, if UE reports its CA capability as 4, if one carrier is deactivated, then the BD budget for PCell would be 44*2*(4/5)=70 (let’s say SCS=15). Is this the correct understanding? Interpretation#2: If UE is configured with 5 physical CCs and UE is configured to receive MBS on PCell, if UE reports its CA capability as 4, for all cases even if all the CCs are activated, when calculating the BD budget for PCell, we count PCell as 2 carriers. In this case, the BD for PCell would be 44*2*(4/6)= 58 (let’s say SCS=15). Our understanding is that Interpretation#2 is used in Rel-16 MTRP. However, the current proposal seems to be Interpretation#1. Which interpretation is the common understanding? |
| Spreadtrum | Proposal 2-1: fine. Prefer Alt.2. Proposal 2-3: The current proposal is difficulty to understand, e.g., what is unused CC. If possible, please draft it in specification language. If the proposal is to state like QC’s clarification, we suggest to clearly capture it in the proposal. |
| OPPO | Proposal 2-1: support the proposal, Alt 2 is preferable for us. Proposal 2-2: given the situation, we also prefer to postpone this proposal. |
| CMCC | 2-1: Prefer alt 2 2-3: OK |
| Ericsson | P2-1: We agree and have a preference for Alt3. P2-3: We agree |
| Apple | 2-1: Alt2 is preferred 2-3: agree with Lenovo. |
| Moderator | Proposal 2-1: It seems most companies prefer to keep Alt1 or Alt2, let’s try to make a little progress and at least agree to down-select one from Alt 1 and Alt 2, that means we will not support both. Proposal 2-3: I tried to reword the proposal according to the comments although I know some companies prefer to defer, but I hope at least we make it more clear what this proposal want to do. |

## Updated Proposals (after 2nd round of inputs)

[High] Updated Proposal 2-1:

For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from Alt 1 and Alt2:

- Alt 1: support Type-3 CSS

  - The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

- Alt 2: support a new Type-x CSS

  - The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

- Alt 3: support both Alt 1 and Alt 2

  - If Type-3 CSS is used for group-common PDCCH, the monitoring priority is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

  - If new Type-x CSS is used for group-common PDCCH, the monitoring priority is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

- Alt 4: support a new Type-x CSS

  - If the first DCI format of group-common PDCCH is configured in new Type-x CSS, the monitoring priority of the new Type-x CSS is the same as existing Rel-15/16 CSS

  - If the second DCI format of group-common PDCCH is configured in new Type-x CSS, the monitoring priority of the new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets.

  - The first DCI format and the second DCI format of group-common PDCCH cannot be configured in the same one new Type-x CSS.

[High] Updated Proposal 2-3:

For RRC_CONNECTED multicast UEs supporting CA capability, support the following principles for determining MPDCCHtotal,slot,μ / CPDCCHtotal,slot,μ  and the maximum numbers of BD/CCE UE is required to monitor per slot for a serving cell supporting multicast reception:

- When determining  MPDCCHtotal,slot,μ / CPDCCHtotal,slot,μ  defined in 38.213, the number of DL serving cell(s) supporting multicast reception is increased as R times.

- The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

- R is a value reported by the UE

## Company Views (3rd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## Updated Proposals (after 3rd round of inputs)

# Issue #3: Transmission scheme and HARQ process management

## Background and submitted proposals

Background

In RAN1#104-e, the following agreements were achieved.

Agreement:

For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission.

- The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

Submitted Proposals

- Huawei, HiSilicon

  - Proposal 1: The configurable number of maximum HARQ process number is kept unchanged for UE supporting multicast reception, and

    - The HPN for PTM scheme 1 initial transmission is different from that for unicast initial transmission for each UE within the group at a given time.

  - Proposal 2: Define UE’s behavior when UE is scheduled to receive a multicast TB#2 initial transmission and a PTP TB#1 retransmission with the same HPN.

- OPPO

  - Proposal 4: HARQ process ID assignment between unicast and MBS is fully up to gNB.

  - Proposal 5: PTM transmission scheme 2 is not supported.

- Spreadtrum

  - Proposal 3: For RRC_CONNECTED UEs for NR MBS, not support PTM2 transmission scheme.

  - Proposal 4: If initial transmission for multicast is based on PTM transmission scheme 1, not simultaneously support PTM1 and PTP together as the retransmission scheme.

- ZTE

  - Proposal 10: Regarding HARQ process management for NR multicast, HPNs are separated for unicast and each multicast service, and a multicast service specific HPN entity is required for each multicast service.

  - Proposal 11: A distinguishing indication among unicast and different multicast services should be introduced into DCI of PTP transmission for associating the PTM scheme 1 and PTP transmitting the same TB.

- vivo

  - Proposal 3: A UE can be configured with multiple common RNTIs for PDSCH scrambling for different Broadcast/Multicast services.

  - Proposal 4: For RRC_CONNECTED UEs, support PTM transmission scheme 2 for multicast.

  - Proposal 5: For the retransmission of group-common PDSCH for MBS service, the retransmission scheme(s) is configured:

    - Only PTM scheme 1 is supported, or

    - Only PTP is supported, or

    - Both PTM scheme 1 and PTP are supported

- CATT

  - Proposal 1: PTM transmission scheme 2 group scheduling is supported in NR MBS when the group is small.

  - Proposal 2: Multi-group-common PDCCH scheduling mechanism is supported in NR MBS to reduce PDCCH overhead when the group is large.

  - Observation 3: From UE’s perspective, PTM transmission scheme 2 used as retransmission is considered as initial transmission, if the DCI for initial transmission using PTM scheme 1 is missed by the UE.

  - Proposal 12: When PTM transmission scheme 1 is used for initial transmission, PTM scheme 2 can be supported for retransmission(s) for the whole group of UEs.

  - Proposal 13: PTM scheme 2 and PTP can be combined as retransmission schemes for all the UEs in the same group for a TB.

  - Proposal 14: When supporting both MBS service and unicast service receptions by a UE, the total number of HARQ processes is not supposed to be increased.

  - Proposal 15: MBS services can be configured with specific HPNs, e.g. HPN#0~3.

- Nokia

  - Proposal-10: For RRC_CONNECTED UEs, support UE-specific PDCCH with CRC scrambled by a C-RNTI for dynamic scheduling and CS-RNTI for SPS, to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on a common RNTI.

  - Proposal-11: The same group-common PDSCH for PTM transmission can be accessed either by:

    - A set of UEs using the same group-common PDCCH with CRC scrambled by a common RNTI, or

    - A set of UEs, where each UE uses a UE-specific PDCCH with CRC scrambled by a C-RNTI or CS-RNTI

  - Proposal-12: The network can dynamically modify the signaling used to configure a UE to access a group-common PDSCH.

  - Proposal-18: The transmission of a new TB associated with the multicast service sent using PTM scheme 1 cannot be initiated until either the maximum number of retransmissions for the previous TB is completed or HARQ ACK message is received from all the UEs which received the TB – assuming HARC ACK/NACK scheme is utilized.

  - Proposal-19: Retain existing maximum HARQ process limits for UEs with MBS capability

  - Proposal-20: The total number of HARQ processes are dynamically split between initial transmissions for unicast and initial transmissions for multicast in an implementation-specific manner by the gNB.

- MediaTek

  - Proposal 1: The total HARQ process number defined in Rel-15/16 (e.g., 16) is unchanged for UE receiving unicast and multicast service.

  - Proposal 2: The maximum HARQ process used for MBS is up to network implementation.

  - Proposal 3: The HARQ process ID indicated in DCI for PTP retransmission is kept the same value with corresponding PTM scheme 1 initial transmission.

- FUTUREWEI

  - Proposal 7: If ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if PTM scheme 1 is used for initial transmission, PTM scheme 1 retransmission and PTP retransmission can be used by UEs in the same MBS group.

  - Proposal 8: A UE receiving its PTP retransmission is allowed to receive a simultaneous PTM scheme 1 retransmission for the same TB for decoding. This support should be left to UE implementation.

  - Observation 2:

    - The UE’s HARQ processes are divided into two sets, the set for unicast and another set for MBS PTM. This allows the simplest association, as in Fig 1, where the same HARQ ID and NDI is used for PTM (re)transmissions and PTP retransmissions for a TB.

    - PTM initial tx, PTM retx, and the simultaneous PTP (HARQ) retx for the UE(s) need to share the same HARQ process ID to support soft combining.

    - If there are PTM retx or PTP (HARQ) retx that is still active/pending on the HARQ process, the gNB starts the initial transmission of a new data in a new HARQ process.

    - Either additional MBS PTM HARQ processes are allowed in addition to the existing number of HARQ processes (preferred), or the number of HARQ processes is kept unchanged in the UE and some number (possibly up to a maximum) are allowed for MBS PTM.

  - Proposal 9: For Connected UEs, PTM transmission scheme 2 for retransmission(s) should also be supported.

  - Proposal 10: PTM transmission scheme 2 for initial transmission of MBS is not supported for Connected UEs.

- CMCC

  - Proposal 21. The maximum number of HARQ processes per cell, currently supported for unicast PDSCH, is kept unchanged for UEs receiving unicast and multicast.

  - Proposal 22. For broadcast, dedicated HARQ process(es) are assigned and the HARQ process number is not indicated in the group-common PDCCH.

  - Proposal 23. It is up to gNB’s implementation to dynamically indicated the HARQ process for unicast and multicast without semi-static split or allocation between them.

- Intel

  - Proposal 6: PTM Scheme 2 should be supported when ACK/NACK based HARQ feedback is configured or enabled for the UEs within a group.

  - Proposal 7: Only one among PTP or PTM Scheme 2 can be supported for UE specific retransmission when the initial transmission was based on PTM Scheme 1. The support of PTP or PTM Scheme 2 can be configured by UE-specific RRC signaling. Different UEs in a group can potentially support different retransmission schemes but not both simultaneously.

  - Proposal 8: The HARQ process ID is used to associate PTM Scheme 2 based retransmission with the initial transmission using PTM Scheme 1. The UE does not expect to receive a unicast transmission using the same HARQ process ID as the ongoing MBS transmission

  - Proposal 9: Different group RNTIs corresponding to high and low QoS delivery modes are configured for RRC_CONNECTED UEs

- Qualcomm

  - Proposal 9: Retransmission schemes based on PTP and PTM-1 can be supported for different UEs in the same group.

  - Proposal 10: For HARQ process management, the maximum number of HARQ process IDs is kept unchanged.

    - Support dynamic split of HARQ processes between unicast and multicast.

    - A DCI field is used to differentiate the HARQ process ID used for PTP unicast data or for PTP multicast retransmission.

- Samsung

  - Observation 4: There is no reason to increase the maximum number of HARQ processes per cell relative to Rel-16.

  - Observation 5: There is no reason to separate HARQ processes between unicast and MBS.

- Chengdu TD Tech

  - Proposal 2: For the ACK/NACK based HARQ-ACK feedback, both the PTP bearer and the PTM bearer with the PTM scheme 1 can be used for the retransmission of the associated TB.

  - Proposal 3: For the NACK-ONLY based HARQ-ACK feedback, the PTM bearer with the PTM scheme 1 can be used for the retransmission of the associated TB, where the PTM scheme 1 can use beam sweeping or the partial beam sweeping.

    - FFS：Whether or not the different PDSCH sources for retransmitting the same TB in the different beam coverage areas can be used.

- Convida

  - Proposal 1: PTP transmission and PTM transmission scheme 2 should be supported for initial transmission for MBS.

  - Proposal 2: PTM transmission scheme 2 should be supported for retransmission for MBS.

  - Proposal 3: Mechanism needs to be introduced for the UE to distinguish between the UE-specific PDCCH scheduling the MBS PDSCH and scheduling the unicast PDSCH.

- Lenovo

  - Proposal 4: The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UEs receiving unicast and/or multicast.

  - Proposal 5: It is up to gNB to determine a HARQ process number used for initial unicast transmission or initial multicast transmission.

- NTT Dococmo

  - Proposal 6: The maximum number of HARQ processes is kept unchanged from Rel-16.

  - Observation 3: If simultaneous transmissions of retransmission using PTM scheme 1 and retransmission using PTP are supported, there are several issues that need to be considered.

  - Proposal 7: Not support PTM scheme 2 as retransmission scheme for PTM scheme 1.

- Ericsson

  - Proposal 1: The UE may receive PTP initial transmission within the PTM leg of a split MRB. For a given UE, the gNB may choose between PTM and/or PTP dynamically and independently for each TB.

  - Observation 1: In the current specification, the UE is not expected to receive another PDSCH associated with the same HARQ process before it has decoded that process and responded with HARQ-ACK if configured to do so.

  - Proposal 2: Based on UE capability, a UE in a G-RNTI-based scheduling group may receive both PTM and PTP with same HARQ process and NDI, within the same HARQ-ACK feedback bundling window determined via dlDataToUL-ACK.

  - PTM-2 based initial transmission is not supported.

  - PTM-2 based retransmission is not supported.

## Initial Proposals based on contributions

Summary

Regarding the HARQ process management,

- It was agreed in last meeting that the HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB, but some companies further propose that the HARQ process ID indicated in DCI for PTP retransmission is kept the same value with corresponding PTM scheme 1 initial transmission. Maybe it would be better that we make it more clear.

- 9 companies [Huawei, CATT, Nokia, MTK, CMCC, Qualcomm, Samsung, Lenovo, NTT Docomo] propose to keep the maximum number of HARQ processes per cell unchanged for UE to support multicast reception, while 1 company [Futurewei] prefers either additional MBS PTM HARQ processes are allowed in addition to the existing number of HARQ processes, or the number of HARQ processes is kept unchanged in the UE and some number (possibly up to a maximum) are allowed for MBS PTM.

- It seems 7 companies [OPPO, Nokia, MTK, CMCC, Qualcomm, Samsung, Lenovo] share a similar view that the HARQ processes are dynamically split between initial transmissions for unicast and initial transmissions for multicast in an implementation-specific manner by the gNB. 1 company [ZTE] proposes that the HPNs are separated for unicast and each multicast service, and a multicast service specific HPN entity is required for each multicast service. 1 company [Futurewei] prefers that the UE’s HARQ processes are divided into two sets, the set for unicast and another set for MBS PTM. 1 company [CATT] proposes that MBS services can be configured with specific HPNs, e.g. HPN#0~3.

- It seems 2 companies [Nokia, Futurewei] share a similar view that, for a HARQ process ID, if the PTM retransmission or PTP retransmission of the previous TB, which is initially transmitted with PTM scheme 1, with the HARQ process ID is not completed (e.g., the maximum number of retransmissions is not reached or HARQ ACK is not received from all the UEs in the MBS group assuming ACK/NACK based HARQ scheme is used), gNB cannot start the PTM scheme 1 initial transmission of a new TB with the same HARQ process ID. However, 1 company [Huawei] proposes to define UE’s behavior when UE is scheduled to receive a multicast TB#2 initial transmission and a PTP TB#1 retransmission with the same HPN.

- It seems 2 companies [ZTE, Qualcomm] share a similar view that a DCI field is used to differentiate the HARQ process ID used for unicast (re)transmission or PTP retransmission for multicast. Moderator think we can further discuss this issue after we have a common understanding on how the HARQ processes are split between unicast and multicast.

Regarding whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group, 4 companies [vivo, Futurewei, Qualcomm, Ericsson] propose to support this.

Regarding PTM scheme 2, 5 companies explicitly propose to support PTM scheme 2 for initial transmission and retransmission, while 4 companies explicitly propose to not support it. It seems the situation does not change much compared with the last meeting. Moderator propose to first focus on the previous issues.

1 company proposes that a UE can be configured with multiple G-RNTIs for PDSCH scrambling for different MBS services. I think this should be a common understanding, but I’m not sure whether we need to make such an agreement in RAN1.

Initial Proposals

The following moderator recommendations are made.

[Moderator’s recommendation]

[High] Initial Proposal 3-1:

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

[High] Initial Proposal 3-2:

The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception.

- The HARQ processes are dynamically split between initial transmissions for unicast and initial transmissions for multicast in an implementation-specific manner by the gNB.

- FFS whether to specify a maximum number of HARQ processes specifically used for multicast.

[High] Initial Proposal 3-3:

For a HARQ process ID, if the PTM retransmission or PTP retransmission of the previous TB, which is initially transmitted with PTM scheme 1, with the HARQ process ID is not completed (e.g., the maximum number of retransmissions is not reached or HARQ ACK is not sent assuming ACK/NACK based HARQ scheme is used), down-select one of the following two alternatives:

- Alt 1: UE is not expected to receive a PTM scheme 1 initial transmission of a new TB with the same HARQ process ID.

- Alt 2: it is possible that UE receives a PTM scheme 1 initial transmission of a new TB with the same HARQ process ID.

[High] Initial Proposal 3-4:

For RRC_CONNECTED UEs, if PTM scheme 1 is used for initial transmission, PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group.

[High] Initial Proposal 3-5:

A UE can be configured with multiple G-RNTIs for different MBS services.

## Company Views (1st round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 3-1: no need since the agreement made in previous RAN1 meeting is clear: Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB. 3-2: We are OK with the main bullet. Regarding the first sub-bullet, it may be better to remove the word of “dynamically” since it is pure implementation issue of gNB. Regarding the second bullet, it is not necessary to set the maximum number as it is also gNB implementation issue. 3-3: The main bullet is not clear to us. What does it mean “if the PTM retransmission or PTP retransmission of the previous TB,”? It seems no verb in this clause. 3-4: We are not OK with it. Now that PTM scheme 1 is adopted as retransmission, why PTP retransmission is sent simultaneously? Those UEs which PTP retransmission is sent to will receive different retransmissions for a given TB. 3-5: Is it better to discuss this issue in RAN2? |
| Spreadtrum | For Proposal 3-1, in principle we are fine; For Proposal 3-2, fine with FL’s proposal; For Proposal 3-3, support Alt 1. In-order operation should be kept across unicast and multi-cast, considering UE’s implementation complexity. For Proposal 3-4, not support. We have not seen the necessary and benefit. For proposal 3-5, fine. |
| Apple | For Proposal 3-4, doesn’t it mean the UE would detect both PTM scheme 1 and PTP re-transmission. |
| CATT | Proposal 3-1: This proposal is more like an observation based on current mechanism/agreements, unless there is another meaning for the proposal. Based on the agreements by now, initial transmission and retransmission can be as 1) PTM 1 (initial Tx) + PTM 1 (Re-Tx); 2) PTM 1 (initial Tx) + PTP (Re-Tx). Whenever the same TB is (re-)transmitted, the same HPN and non-toggled NDI should be used. If not, the previous buffered data has to be flushed based on current mechanism. Proposal 3-2: The main bullet is supported. For the two sub-bullets, if the allocation of HPN between unicast and multicast is up to gNB based on current mechanism, there is no necessary to explicitly agree about it, or a simple wording can be used as follows: [High] Initial Proposal 3-2: The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception. - The HARQ processes are dynamically split between initial transmissions for unicast and initial transmissions for multicast in an implementation-specific manner by the gNB. - FFS whether to specify a maximum number of HARQ processes specifically used for multicast. - How to allocate HARQ processes between unicast and multicast is up to gNB. Proposal 3-3: Based on current Rel-15/16 mechanism, only alternative 1 can be supported. Whenever the same HPN is used and NDI is toggled, the buffer should be flushed. The gNB is supposed to avoid such case happen by proper scheduling. For alternative 2, it is more likely a UE implementation or new description scheme from UE side. UE can differentiate and storage different TBs but sharing the same HPN. The detailed design is not clear, and it may require new capability for UE on buffer. Therefore, Alt 2 is not supported. Proposal 3-4: NOT support. Similar view with Lenovo, a UE will receive PTM 1 and PTP that scheduling the same PDSCH as retransmission, which is meaningless. Proposal 3-5: Generally OK. |
| OPPO | Proposal 3-1~ Proposal 3-5: Agree. |
| ZTE | For Proposal 3-1: Support. For Proposal 3-2: We have some technical concerns for this proposal. If the HARQ processes are dynamically split between unicast and multicast, then it means network has to consider all the HARQ processes for unicast used by all the UEs within the same group to determine a HARQ process for multicast. If the number of UEs within one group is large, it would definitely complicate the network scheduling. One simpler solution is to support two separate HARQ entities for unicast and MBS. Note that the total HARQ processes per cell can keep the same. As long as there are two separate HARQ entities for MBS and unicast, network decouple the HARQ process for unicast and MBS. For Proposal 3-3: Based on our understanding, if separate HARQ entities are configured for unicast and MBS, then the issue in this proposal can already be resolved. Thus, we would propose to finalize Proposal 3-2 first and defer the discussion of this proposal. For Proposal 3-4: We support this proposal. For Proposal 3-5: We are ok with this proposal but it seems better to let RAN2 to decide this. |
| CMCC | 3-1 ~3-5: Agree |
| NTT DOCOMO | Proposal 3-1: We are fine with FL’s proposal. Proposal 3-2: We are fine with FL’s proposal. Proposal 3-3: We prefer Alt.1 to simplify UE processing. Proposal 3-4: A UE which receives PTP retransmission will also receive PTM scheme 1 retransmission at the same slot. It will be necessary to specify the behavior for such a case. We propose to add sub-bullet. “FFS : Handling of two PDSCHs associated with the same TB that are received simultaneously” Proposal 3-5: We are fine with FL’s proposal. |
| Huawei/HiSilicon | 3-1: ok 3-2: ok. 3-3: ok |
| LG | P3-1: Support the initial proposal. P3-2: We are fine with the initial proposal. We could add ‘FFS for broadcast’. P3-3: We prefer Alt 1 for simplicity. Alt 2 would increase complexity in some aspects. P3-4: We support the initial proposal. Simultaneous PTM/PTP retransmissions would be beneficial in some cases e.g. different UE-dedicated BWPs for different UEs, different TCIs states for different UEs. P3-5: We are fine with the initial proposal. RAN2 could further discuss whether one or multiple services can be mapped to a G-RNTI. |
| MTK | Proposal 3-1: support the proposal. Proposal 3-2: support the proposal. Proposal 3-3: support Alt 1. If Alt 2 is supported, it will increase the UE HARQ buffer capability. Proposal 3-4: The motivation of supporting PTM schem1 and PTP simultaneously retransmission is not clear. Proposal 3-5: we are ok with the proposal. |
| Samsung | Proposal 3-1: Agree (although this additional agreement is not really needed). Proposal 3-2: Agree (although this additional agreement is not really needed – gNB implementation issue). Proposal 3-3: Support Alt. 1. It is also our opinion that nothing needs to be agreed/specified (gNB misconfiguration – same as in Rel-16) Proposal 3-4: Do not agree. Introduces unnecessary specification and UE complexity without meaningful benefit. Proposal 3-5: Agree. |
| Qualcomm | 3-2: Fine in principle but prefer to change the second subbullet more general as “FFS the detailed signaling”. 3-3: Alt2 does not work. If the UE still receive PTP retransmission of TB1, the PTM-1 transmission of TB2 comes in will flush the buffer of the HARQ process. The legacy way should apply here as well: “The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6]”. Agree with other proposals. |
| Futurewei | 3-1. 3-2, 3-4: Support 3-3: Alt 1 3-5: It is not clear what ‘services’ here mean from RAN1 specs perspective. |
| Nokia, NSB | Proposal 3-1: Agree with the proposal Proposal 3-2: Agree with the proposal Proposal 3-3: Agree with the proposal and we support Alt 1. Proposal 3-4: We agree with Lenovo’s views and do not support this proposal. For simplicity, it would be preferred to use only one mode of retransmission. The motivation of having both PTM and PTP for retransmissions simultaneously is not clear, and is also not resource efficient. Proposal 3-5: Agree with the proposal |
| Ericsson | P3-1: We agree. P3-2: We agree but wish to point out that unicast and MBS PTP both use C-RNTI and belongs to the same category with no need for differentiation between these. Regarding the FFS we do not see the need for that – this can be left to gNB implementation. We also note that without the FFS, the agreement does not have spec impact. P3-3: We prefer Alt2, since with the DCI NDI bit toggled, the UE will understand that the data is new. This will always work when the same RNTI is used for the new transmission. Furthermore, the gNB may also have reasons to initiate the transmission of a new TB before an earlier TB is completed by all UEs. We note that there is an issue with respect to potential conflicts of NDI and propose that how to resolve that is FFS. P3-4: We agree P3-5: We agree |
| Convida | For Proposal 3-1: Agree. For Proposal 3-2: Agree. For Proposal 3-3: Prefer to defer the discussion of this proposal. For Proposal 3-4: Support. For Proposal 3-5: Support. |
| Vivo | For Proposal 3-1, 3-2, 3-3, 3-5, we are fine in principle; For Proposal 3-2, fine with FL’s proposal; For Proposal 3-4, one question if PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group, for a UE within the MBS group, whether PTM scheme 1 retransmission and PTP retransmission can be simultaneously supported |
| Moderator | Proposal 3-1: Some companies think it is necessary or more like an observation, so I updated it for conclusion. Proposal 3-2: I updated the proposal based on companies’ comments, but it seems ZTE still has technical concern on this proposal. @LG, I think we can focus on multicast first. Proposal 3-3: As ZTE pointed out, this proposal may depend on the outcome of proposal 3-2. So I deferred the discussion for the moment. Proposal 3-4: I think the motivation of this proposal is that, when multiple UEs in the same group received the PTM 1 initial transmission incorrectly, then gNB can use PTM1 retransmission for some of the UEs and use PTP retransmission for the other UEs (e.g., cell edge UEs). As pointed out in [25] (copied below) and some other papers, this implies that some UEs (e.g., cell edge UEs) may receive PTM 1 retransmission and PTP retransmission with same HARQ process and NDI, within the same HARQ-ACK feedback bundling window determined via dl-DataToUL-ACK. I’m not sure the companies who do not support this proposal (Lenovo/ Spreadtrum/ Apple/ CATT/ MTK/ Samsung/ Nokia) have the same understanding on this proposal, so I did not update it at the moment. It may be updated based on more discussions. “If, for the same group of UEs, utilization of multiple retransmission (PTM and PTP) schemes is allowed, the UEs receiving the PTP-based retransmission will also “see” the PTM retransmission, as they also monitor group PDCCH. Therefore, the UEs will have to deal with two PDSCHs with the same HARQ process, within the same HARQ-ACK feedback bundling window determined via dlDataToUL-ACK. The PDSCHs of the PTP and PTM can be either scheduled in the same slot or in different slots. In either case, the UE can process these two PDSCHs, respectively, scrambled via C-RNTI and G-RNTI, either separately (one or both PDSCHs) or jointly via soft-combining according to the capability. In practice, there can be a situation where the soft-combined PDSCHs may result in a HARQ-ACK for the decoding, even if both individual PDSCH decoding would have resulted in HARQ-NACK. In such case, the UE can directly provide the feedback corresponding to the soft-combining of the two PDSCHs, instead of providing feedback corresponding to each individual PDSCH. The existing type-1 or semi-static HARQ codebook construction supports this operation of the HARQ-ACK feedback for different PDSCHs. “ @vivo, regarding your question, my understanding is that there seems no motivation to simultaneously support PTM scheme 1 retransmission and PTP retransmission for the same UE. @Docomo, regarding the FFS suggested by you“FFS : Handling of two PDSCHs associated with the same TB that are received simultaneously”, I think the two PDSCHs may not always be received simultaneously. Proposal 3-5: Some companies think it would be better to discuss this proposal in RAN2. Futurewei thinks it is not clear what ‘services’ here mean from RAN1 specs perspective. So I want to ask companies if you are OK to leave it to RAN2. I did not update this proposal at this moment, it may be updated or delete based on further discussion. |

## Updated Proposals (after 1st round of inputs)

[High] Initial Proposal 3-1 (for conclusion):

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

[High] Updated Proposal 3-2:

The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception.

- How to allocate HARQ processes between unicast and multicast is up to gNB.

[High] Initial Proposal 3-4:

For RRC_CONNECTED UEs, if PTM scheme 1 is used for initial transmission, PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group.

[High] Initial Proposal 3-5:

A UE can be configured with multiple G-RNTIs for different MBS services.

## Company Views (2nd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 3-1: OK to make it as conclusion. 3-2: Agree. 3-4: We don’t support this proposal since it does not make sense to retransmit to a subset of UEs via PTM scheme 1 and simultaneously retransmit to other UEs in same group via PTP. Why not only use PTM scheme 1 as retransmission to the group? How can gNB retransmit to those UEs via PTM scheme 1 and not retransmit to other UEs? To us, after the initial transmission via PTM scheme 1, gNB adopts PTM scheme 1 if majority UEs respond with NACK (we assume ACK/NACK based feedback as the main bullet talks about PTP retransmission here); otherwise, gNB adopts PTP to few UEs which respond with NACK. We don’t understand why both retransmission schemes are adopted simultaneously, which is not resource efficient way for gNB. 3-5: Still, we think it is up to RAN2. Meanwhile, if we discuss it in RAN1, is the number of G-RNTIs equal to the number of supported MBS services by the UE? |
| Samsung | Support 3-1, 3-2, 3-5. Do not support 3-4. It is unnecessary (as Lenovo explained above) and will only lead to additional complexity. |
| Nokia, NSB | 3-1: Support 3-2: Support 3-4: Do not support – adds complexity for gains (cell-edge UEs?) that could be gained by additional PTM retransmissions? Also, does this depend on the FFS agreement of the revised [High] Updated Proposal 4-2? 3-5 Support |
| Qualcomm | Agree 3-1, 3-2, 3-4, 3-5. For 3-4, we support the proposal. We think PTM-1 with a wide beam can be used for retransmission for most of UEs in the same group who requires retransmission. The PTM-1 wide beam can be adjusted to the subset of UEs. In the meanwhile, the PTP with unicast narrow beamforming is beneficial for a cell-edge UE. - If only PTP is used for retransmission, the beam is flexible for each UE but PTP PDSCH result in high overhead. - If only PTM-1 is used for retransmission, the beam is less flexible in order to cover all the UEs who requires retransmission. |
| Convida | For Updated Proposal 3-1: Fine with the proposal. For Updated Proposal 3-2: Fine with the proposal. For Updated Proposal 3-4: Fine with the proposal. For Updated Proposal 3-5: Fine with the proposal. |
| LG | P3-1: Support the initial proposal. P3-2: We are fine with the updated proposal. We could add ‘FFS for broadcast’. P3-3: We prefer Alt 1 for simplicity. Alt 2 would increase complexity in some aspects. P3-4: We support the initial proposal. P3-5: We are fine with the initial proposal. RAN2 could further discuss whether one or multiple services can be mapped to a G-RNTI. |
| Huawei/HiSilicon | 3-1, it is the sole understanding per the agreement made in the last meeting. If conclusion/clarification can help, then fine. 3-2, ok 3-3, per Round-1 discussion, it is important to clarify Alt2 in 3-3 is not intended to increase UE capability to receive the PTM1 new TB with the same HPN as PTP retransmission of an old TB. Rather, the point is that it is too difficult for network to coordinate one HPN which is not used by any UE for unicast (assuming we agree the HPN is split between unicast and multicast up to gNB), so as long as a UE within the group not feedback ACK, multicast will get stuck not able to use the same HPN for new TB transmission initial, which will make multicast not practically workable, so it has to be discussed as high priority issue. 3-4, question for clarification, maybe for QC, any spec impact you are imagining? It might be up to gNB. 3-5，We prefer to focus on one G-RNTI discussion first, when all the discussion/solution is clear, extending one to multiple would be more time efficient for discussion. |
| CATT | Updated proposals: Proposal 3-1: OK. Proposal 3-2: OK. Proposal 3-3: It needs more discussion on the details and issues of this proposal. Proposal 3-4: NOT support. The key issue, as mentioned above by several companies, is how the UEs deal with the receptions of PTM1 and PTP with the same PDSCH’s retransmission? Proposal 3-5: Generally OK. |
| NTT DOCOMO | Proposal 3-1: We are fine with the proposal. Proposal 3-2: We are fine with the proposal. Proposal 3-4: We are generally fine with the proposal. We think simultaneous transmissions of two retransmission schemes would be useful. But we are not sure the handling of simultaneously received 2 PDSCHs can be up to implementation or additional specification work is needed. Proposal 3-5: We are fine with the proposal. |
| Google | P3-1: Support the proposal P3-2: Support the proposal P3-4: Don’t support the proposal. If a UE can receive initial transmission from PTM scheme 1, the UE should able to receive retransmission from PTM scheme 1 regardless whether the UE is configured with PTP retransmission or not. Proposal 3-4: For RRC_CONNECTED UEs, if PTM scheme 1 is used for initial transmission, PTP retransmission can be configured to a UE in addition to the PTM scheme 1 retransmission. - Different UEs in the same MBS group can be configured with different retransmission schemes simultaneously, e.g. PTM scheme 1 and PTP retransmission, or PTM scheme 1 retransmission only. P3-5: Support the proposal |
| vivo | 3-1: OK to make it as conclusion. 3-2: OK. 3-4: With this proposal, we need to of two PDSCHs associated with the same TB that are received simultaneously, which may be too complexed. “If, for the same group of UEs, utilization of multiple retransmission (PTM and PTP) schemes is allowed, the UEs receiving the PTP-based retransmission will also “see” the PTM retransmission, as they also monitor group PDCCH. Therefore, the UEs will have to deal with two PDSCHs with the same HARQ process, within the same HARQ-ACK feedback bundling window determined via dlDataToUL-ACK.” That means, for a UE support PTP retransmission, the UE also supports PTM retransmission. That is, for the same UE, it is needed to simultaneously support PTM scheme 1 retransmission and PTP retransmission. If different retransmission schemes are supported within an MBS group for different UEs, but for a certain within the group UE, it only supports one retransmission scheme, then the UE supports PTP retransmission would not detect the PDSCH based on PTM scheme. 3-5: OK |
| ZTE | Support for Proposal 3-1, 3-4 and 3-5. We are also ok to leave Proposal 3-5 to RAN2. |
| Spreadtrum | Proposal 3-1: Although we think it is unnecessary in light of the agreement in last meeting, but we are fine with it. Proposal 3-2: fine Not support 3-4: We have not seen the necessary. Proposal 3-5: Prefer it is up to RAN2, but we are fine |
| MTK | Proposal 3-1: Support the proposal. In the last meeting, we achieved an agreement that the HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB. But it doesn’t give specific solution how to associate the relationship b/w PTM scheme 1 and PTP using HPID. We think this proposal can make it clearer. Proposal 3-2: support. Proposal 3-4: The wording is not clear for us. As the concept of PTM scheme 1 whose CRC is scrambled by G-RNTI and is transmitted for all UEs in the same MBS group, how to understand PTM scheme 1 is used for subset of UE within in one MBS group? We think the PTM scheme 1 with retransmission is also used for all UE. If the UE is cell edge UE, it can be dynamic changed from PTM to PTP based on RAN2’s discussion. Proposal 3-5: basically support, if majority views think it is up to RAN2, we are also OK. |
| OPPO | Fine with Proposal 3-1, 3-2, 3-4 and 3-5. |
| CMCC | Support |
| Ericsson | P3-1: Support P3-2: Support P3-4: Support. We see a spec impact to this proposal when the UE will monitor both group PDCCH scrambled by G-RNTI and PTP retransmission in the same “PDSCH to HARQ window”, if these two transmissions have the same HARQ ID/NDI. P3-5: Support in principle. In our understanding, although reception of multiple servicers may exploit multiple G-RNTIs, the concept of “service” is not a PHY concept, so should be used in the agreement. RAN1 should not decide on services. We therefore propose the following reformulation: “A UE can be configured with multiple G-RNTIs. NOTE: This can e.g. be used for different MBS services” |
| Apple | 3-4: If the same TB is re-transmitted by PTM and PTP at the same time, UE just can handle one PDSCH for the same HARQ process, another one could be discarded. NACK could be sent always. For this proposal, implementation impacts need to check. |
| Moderator | Proposal 3-1: It seems all companies are fine with this, so I marked it as stable. Proposal 3-2: It seems all companies can accept this. I added an FFS for broadcast since I received sustained request from LG Proposal 3-4: At least 9 companies do not support this proposal, so I suggest to postpone it. Google provided a suggestion, and I added a new proposal 3-4a for it. @Huawei, the spec impact is mentioned by Ericsson, I think at least spec should be enhanced to allow the case that UE may receive PTM1 retransmission and PTP retransmission of the same TB with the same HARQ ID/NDI in the same “PDSCH to HARQ window”. Regarding proposal 3-3, as some companies pointed out it may depend on the outcome of proposal 3-2, so I we can discuss it after we finalize 3-2. Proposal 3-5: Most companies are fine with this proposal, although some companies are also fine to leave it to RAN2. Based on Ericsson’s comment, I updated it a little bit. I think such an agreement may be helpful since some other discissions also need such an assumption in RAN1. |

## Updated Proposals (after 2nd round of inputs)

[High](stable) Initial Proposal 3-1 (for conclusion):

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

[High] Updated Proposal 3-2:

The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception.

- How to allocate HARQ processes between unicast and multicast is up to gNB.

- FFS for broadcast

[High] Initial Proposal 3-4a:

For RRC_CONNECTED UEs, if PTM scheme 1 is used for initial transmission, PTP retransmission can be configured to a UE in addition to the PTM scheme 1 retransmission.

- E.g., PTM 1(initial tx) ->PTM 1(retx) ->PTP(retx)->PTM1(retx)

- Note: PTM scheme 1 retransmission is supported by default

[High] Updated Proposal 3-5:

RAN1 assume that a UE can be configured with multiple G-RNTIs

- Note: This can, e.g., be for different MBS services.

## Company Views (3rd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## Updated Proposals (after 3rd round of inputs)

# Issue #4: SPS for MBS

## Background and submitted proposals

Background

In RAN1#104-e, the following agreements were achieved.

Agreement:

For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability

- The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS.

- FFS: How to allocate the total SPS configurations between MBS and unicast.

Agreement:

For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS

- FFS: The retransmission scheme(s)

- FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2

Working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Submitted Proposals

- Huawei, HiSilicon

  - Proposal 8: The multicast SPS configuration index is different from unicast SPS configuration index for each UE within the group.

  - Proposal 9: Support PTM scheme 1 and PTP for multicast SPS retransmission.

- OPPO

  - Proposal 6:

    - Allocation of total SPS configurations between MBS and unicast is up to gNB;

    - A SPS configuration for MBS should be activated/deactivated by group-common PDCCH scrambled by the corresponding CS-RNTI specifically for MBS.

    - SPS configuration for MBS is only activated/deactivated by group common DCI, HARQ feedback mode for the group common DCI is indicated by the DCI.

- Spreadtrum

  - Proposal 8: Confirm the working assumption, i.e., Support group-common PDCCH for SPS group-common PDSCH activation/deactivation.

  - Proposal 9: It is up to gNB’s implementation for SPS group-common PDSCH retransmission, e.g.,  either by unicast dynamic scheduling or group-common scheduling can be considered. However, simultaneously scheduling unicast and group-common retransmission shall be avoided.

- ZTE

  - Proposal 12: For SPS-based MBS transmission, the following features are supported,

    - UE-specific activation/deactivation

    - Up to 8 configured SPS configurations can be supported for unicast and MBS respectively per BWP

    - Retransmission of SPS group-common PDSCH, the design for the retransmission for PTM transmission scheme 1 can be reused for it

- vivo

  - Proposal 6: When a UE is configured with multiple SPS group-common PDSCHs, it should be supported to configure group-common RNTI for each SPS group-common PDSCH per SPS configuration.

  - Proposal 7: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, UE-specific PDCCH is supported

  - Proposal 8: A group-common configured scheduling RNTI which is different with the group-common RNTI for group-common dynamic scheduled PDSCH is used for this SPS PDSCH

  - Proposal 9: For MBS for RRC_CONNECTED UEs, HARQ-ACK for SPS group-common PDSCH is configurable.

  - Proposal 10: For MBS in RRC_CONNECTED state, for SPS group-common PDSCH, support retransmission with PTM scheme 1.

- CATT

  - Proposal 16: PTM scheme 1 and PTP can be used as retransmission scheme(s) for SPS group-common PDSCH for MBS.

  - Proposal 17: Both group-common PDCCH and UE-specific PDCCH (if supported) can be used for SPS activation for MBS for RRC_CONNECTED UEs.

  - Proposal 18: Group-common PDCCH is used for SPS deactivation for MBS for RRC_CONNECTED UEs.

- Nokia

  - Proposal-10: For RRC_CONNECTED UEs, support UE-specific PDCCH with CRC scrambled by a C-RNTI for dynamic scheduling and CS-RNTI for SPS, to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on a common RNTI.

  - Proposal-13: Maintain the existing SPS limits, for e.g. eight SPS configurations per BWP for group-common PDSCH SPS configurations per UE.

  - Proposal-14: The allocation of total available SPS configurations between unicast and multicast should be left to gNB implementation, with signaling details of such configurations left to RAN2.

  - Proposal-15: Inherit uplink HARQ feedback configuration for SPS-based MBS from SPS for unicast in combination with uplink feedback configuration for non-SPS-based MBS, augmented by mechanisms for reliable SPS activation / deactivation.

  - Proposal-16: Support HARQ retransmissions on SPS-allocated resources.

  - Proposal-17: Possibilities to add in-band control signaling on PDSCH to facilitate retransmissions on SPS-allocated PDSCH resources should be studied.

- MediaTek

  - Proposal 11: The network can flexible allocate the SPS number for MBS and unicast based on services requirement.

  - Proposal 12: A GC-CS-RNTI scrambling the CRC of corresponding DCI format is defined for MBS SPS activation/deactivation and retransmission.

  - Proposal 13: Group common PDCCH is supported for activation/deactivation of MBS group common PDSCH and the UE specific PDCCH with GC-CS-RNTI is optional supported for activation of MBS group common PDSCH.

  - Proposal 14: MBS SPS activation/deactivation’s feedback mechanism only support ACK/NACK based HARQ feedback mode.

  - Proposal 15: MBS SPS PDSCH without PDCCH scheduling can support ACK/NACK or common NACK only feedback mode.

- FUTUREWEI

  - Proposal 11: At least UE-specific PDCCH is supported for deactivation of SPS group-common PDSCH.

  - Proposal 12: Support of more than one SPS group-common PDSCH configuration.

- CMCC

  - Proposal 12. Both group-common PDCCH and UE-specific PDCCH can be used for activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state.

  - Proposal 13. HARQ-ACK feedback for SPS group-common PDSCH deactivation PDCCH can be supported.

  - Proposal 14. PTM transmission scheme 1 and PTP can be used as retransmission for SPS group-common PDSCH.

  - Proposal 15. It is up to gNB’s implementation to allocate the SPS configurations between MBS and unicast.

    - The SPS group-common PDSCH for MBS can be configured per MBS service.

- Intel

  - Proposal 19: For DL SPS configuration for NR MBS

    - Group common PDCCH is used for SPS activation with HARQ ID field set to all 0’s and RV field set to 00 for the TB being scheduled

    - PUCCH resource for HARQ feedback may be configured via RMSI, OSI or RRC

    - For SPS release, similar group common PDCCH can be used with HARQ ID set to all 0s, MCS and FDRA set all 1’s and RV set 0. For SPS release DCI, UE can be configured with PUCCH resource via RRC

    - The PUCCH resources for HARQ feedback for SPS PDSCH as well as the SPS release DCI can be UE-specific if ACK/NACK based feedback is supported or configured or a common PUCCH resource can be configured for the case when NACK-only feedback is supported or configured.

- Apple

  - Proposal 2: It’s up to gNB scheduling SPS group-common PDSCH re-transmission is via PTM transmission or PTP transmission.

- Qualcomm

  - Proposal 11: Define group CS-RNTI (G-CS-RNTI) for SPS GC-PDSCH, different from CS-RNTI for unicast SPS PDSCH.

    - FFS: number of G-CS-RNTI.

  - Proposal 12: Support one or more activated SPS GC-PDSCH configurations per CFR subject to UE capability.

  - Proposal 13: For SPS GC-PDSCH activation/release,

    - Confirm the WA: At least group-common PDCCH is supported.

    - Support UE-specific PDCCH in addition to GC-PDCCH.

    - Support separate activation of SPS GC-PDSCH by using GC-PDCCH or UE-specific PDCCH.

    - Support joint and separate release of SPS GC-PDSCH by using GC-PDCCH or UE-specific PDCCH.

  - Proposal 14: Support PTM-1 and PTP for the retransmission of SPS GC-PDSCH, similar as dynamic GC-PDSCH.

- Samsung

  - Proposal 6: A DCI format with CS-RNTI can be used for activation of SPS MBS PDSCH receptions.

- LGE

  - Proposal 8: If new CS-RNTI is introduced for group common SPS, the new CS-RNTI is “other RNTI”.

  - Proposal 13: For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

  - Proposal 14: Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

  - Proposal 15: For group common SPS, both group common NACK only based HARQ-ACK and UE specific ACK/NACK based HARQ-ACK are supported.

  - Proposal 16: For group common SPS, at least UE specific confirmation to group common SPS release can be supported by PUCCH A/N.

  - Proposal 17: For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

- Chengdu TD Tech

  - Proposal 1: Support the following HARQ-ACK feedback methods for each SPS MRB of the PTM bearer for an MBS session:

    - ACK/NACK based HARQ-ACK feedback with the different SPS PUCCH resources for the different RRC_CONNECTE UEs.

      - FFS: details for the ACK/NACK based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

    - NACK-ONLY based HARQ-ACK feedback with the different RRC_CONNECTED UEs sharing the same SPS PUCCH resources

      - FFS: details for the NACK-ONLY based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

      - FFS: whether or not other information can be fed back with the NACK-ONLY information.

- Convida

  - Proposal 7: Both group-common PDCCH and UE-specific PDCCH should be supported for activation/deactivation DCI for MBS SPS.

  - Proposal 8: Support dynamic rescheduling for MBS SPS retransmission.

  - Proposal 9: Further study whether to support SPS rescheduling for MBS SPS retransmission.

  - Proposal 10: PTM transmission scheme 1, PTM transmission scheme 2, and PTP transmission should be considered for dynamic scheduling of the MBS SPS retransmission.

- Lenovo

  - Proposal 12: It is up to gNB to allocate the total SPS configurations between multicast and unicast.

  - Proposal 13: For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

  - Proposal 14: For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is used for the UE to transmit ACK for the SPS PDSCH.

  - Proposal 15: For SPS PDSCH transmission in PTM manner in initial transmission, PTP based retransmission is supported.

- NTT Dococmo

  - Proposal 8: Use ACK/NACK based feedback for HARQ-ACK feedback for activation/deactivation regardless of feedback configuration for SPS group-common PDSCH.

  - Proposal 9: Support activation/deactivation on a subgroup basis for SPS group-common PDSCH.

- ASUSTeK

  - Proposal 4: The transmission scheme of the SPS group-common PDSCHs supports at least PTM transmission scheme 1.

  - Proposal 5: An SPS group-common transmission identifier, e.g. a G(C)-CS-RNTI, can be configured for activation, deactivation/release, or retransmissions of SPS group-common PDSCHs.

  - Proposal 6: PTP transmission is supported for retransmissions of SPS group-common PDSCHs.

  - Proposal 7: A UE monitors PDCCHs with CRC-scrambled by the G(C)-CS-RNTI within the UE-dedicated CORESET when the UE expects to receive retransmission(s) of SPS group-common PDSCH(s).

- Ericsson

  - Proposal 17: Confirm that group common PDCCH is used to activate/deactivate SPS group common PDSCH

  - Proposal 18: For group based SPS, UEs missing the PDCCH activation message are sent an activation recovery message via MAC-CE containing the original PDCCH information and the slot number where it was transmitted.   For deactivation,  UE specific PDCCH deactivation order can be sent to UEs not responding to the group de-activation PDCCH.

  - Proposal 19: Multiple group-based SPS configuration are supported, conditioned to UE capability

  - Proposal 20: The UE is expected to provide feedback via HARQ for all PDCCH associated with a PDCCH activation or deactivation order for SPS

  - Proposal 21: The network can RRC configures each UE in the group an additional PDSCH-to-HARQ time offset so that when UEs receive group common PDCCH activate/deactivate command, they can acknowledge this command in different slots to avoid PUCCH resource congestion.

  - Proposal 22: The UE can be configured to either transmit or not transmit HARQ for the SPS PDSCH not corresponding to a SPS PDCCH activation or deactivation.

  - Proposal 23: The SPS UL feedback framework for the SPS scheduled (i.e. PDCCH-less) PDSCH is the same as for non-SPS MBS PDSCH scheduling.

## Initial Proposals based on contributions

Summary

Regarding SPS configuration for MBS, 5 companies [OPPO, Nokia, MTK, CMCC, Lenovo] propose that it is up to gNB implementation to allocate the total SPS configurations between unicast and MBS. 1 company [Nokia] proposes to maintain the existing SPS limits, i.e., eight SPS configurations per BWP, for group-common PDSCH SPS configurations per UE. 1 company [ZTE] proposes that up to 8 configured SPS configurations can be supported for unicast and MBS respectively per BWP. 1 company [Huawei] proposes that the multicast SPS configuration index is different from unicast SPS configuration index for each UE within the group.

Regarding the retransmission scheme(s) for SPS group-common PDSCH, 10 companies propose to reuse the retransmission schemes of dynamic group-common PDSCH, i.e., PTM scheme 1 and PTP can be used for retransmission of SPS group-common PDSCH.

Regarding the working assumption for activation/deactivation of SPS group-common PDSCH, most companies support to at least use group-common PDCCH for activation/deactivation, 1 company [vivo] raises an issue for activation with group-common PDCCH when more than one SPS PDSCH is overlapped in a slot. Moderator suggests companies to further consider the issue raised by vivo in [7]. 5 companies [ZTE, vivo, CMCC, Qualcomm, Convida] propose to also support UE-specific PDCCH for both activation and deactivation. 3 companies [Samsung, MTK, CATT] propose to support UE-specific PDCCH for activation, and 2 companies [Futurewei, Ericsson] propose to support UE-specific PDCCH for deactivation. 1 company [NTT Docomo] proposes to support activation/deactivation on a subgroup basis.

Regarding the FFS whether and how to address the missed activation and deactivation, 1 company [Ericsson] proposes that UEs missing the PDCCH activation message are sent an activation recovery message via MAC-CE containing the original PDCCH information and the slot number where it was transmitted.

6 companies [OPPO, vivo, LGE, ASUSTek, MTK, Qualcomm] propose to define group CS-RNTI (G-CS-RNTI) for SPS GC-PDSCH, different from CS-RNTI for unicast SPS PDSCH.

Initial Proposals

The following moderator recommendations are made.

[Moderator’s recommendation]

[High] Initial Proposal 4-1:

For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total.

- It is up to gNB implementation to allocate the total SPS configurations between unicast and MBS.

- The SPS configuration index for MBS is different from the SPS configuration index for unicast in a BWP

[High] Initial Proposal 4-2:

Support PTM scheme 1 and PTP for the retransmission of SPS group-common PDSCH.

[High] Initial Proposal 4-3:

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

[High] Initial Proposal 4-4:

Define G-CS-RNTI for SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

- FFS: number of G-CS-RNTI.

## Company Views (1st round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 4-1: Generally OK with us. BTW, the second bullet may be not necessary as it has to be defined as that. 4-2: agree. 4-3: agree. 4-4: We are OK with it although we are not sure this issue should be discussed in RAN2 or RAN1. |
| Spreadtrum | Fine with Proposal 4-1; Fine with Proposal 4-3; Fine with Proposal 4-4; For Proposal 4-2, does it mean that simultaneously PTM scheme 1 and PTP retransmission for one initial SPS group-common PDSCH is supported? If yes, we not support it. |
| Apple | For Proposal 4-2, similar comments as Proposal 3-4, does it require the UE to detect PTM and PTP re-transmission at the same time? |
| CATT | Proposal 4-1: Generally OK. Proposal 4-2: Generally OK. One clarification about it: the intention of this proposal is to support both schemes as retransmission schemes for SPS, but not to support simultaneously using PTM1 and PTP for the same transmission of a TB. If not, it seems like that same issue with proposal 3-3 (Alt 2) will happen. Maybe a modification as follows could make it clear. [High] Initial Proposal 4-2: Support PTM scheme 1 or PTP for the retransmission of SPS group-common PDSCH. Proposal 4-3: OK. Proposal 4-4: OK. |
| OPPO | Proposal 4-1~Proposal 4-4: Agree. |
| ZTE | For Proposal 4-1: From our perspective, to avoid the impact on unicast, it would be better to allow configuring more than 8 SPS configurations in a BWP of a serving cell for unicast and MBS in total subject to UE capability. 8 can be the baseline, but it doesn’t hurt to allow UE to report a larger number. For Proposal 4-2: Support For Proposal 4-3: Support For Proposal 4-4: Support |
| CMCC | 4-1~4-4: Agree. |
| NTT DOCOMO | Proposal 4-1: We are fine with FL’s proposal. Proposal 4-2: We are fine with FL’s proposal. Proposal 4-3: We are fine with FL’s proposal. In order to support individual UE joining/leaving SPS, UE-specific PDCCH should also be supported. Proposal 4-4: We are fine with FL’s proposal. |
| Huawei/HiSilicon | 4-1: It is up to gNB implementation to allocate the total SPS configurations or configure the SPS index for unicast or multicast?? 4-2: ok 4-3: ok 4-4: can you clarify the proposal intends for activation/deactivation of SPS group-common PDSCH or for retransmission as well? At least, the RNTI for PTM 1 retransmission and PTP retransmission should be different which should be addressed as well. |
| LG | P4-1: We are fine with the initial proposal. P4-2: We are fine with the initial proposal. P4-3: We are fine with the initial proposal. P4-4: We are fine with the initial proposal. |
| MTK | Proposal 4-1~Proposal 4-4: we are fine with these proposals. |
| Samsung | Agree with all proposals. |
| Qualcomm | 4-1: We prefer to change the second subbullet to “FFS the SPS configuration index for MBS”. Agree with other proposals |
| Futurewei | 4-1, 4-2, 4-3, 4-4: Support |
| Nokia, NSB | Proposal 4-1: Agree with the proposal Proposal 4-2: Agree with the proposal Proposal 4-3: Agree with confirming the working assumption. We would also prefer to support UE-specific PDCCH for activation/deactivation. Proposal 4-4: Agree with Huawei regarding further clarifications for the proposal. We understand that G-CS-RNTI is a special group-common RNTI for SPS, but it would be good to clarify that it is meant to activate / deactivate / modify the SPS group-common PDSCH. |
| Ericsson | P4-1: We agree P4-2: We agree P4-3: We agree P4-4: We agree |
| Convida | For Proposal 4-1: Agree. For Proposal 4-2: We are fine with proposal. We also think PTM scheme 2 should also be supported. For Proposal 4-3: We are fine with confirming the WA. We think UE-specific PDCCH should also be supported. For Proposal 4-4: Support. |
| vivo | Fine with Proposal 4-1, 4-3,4-4; For Proposal 4-2, similar as retransmission for DG group-common PDSCH, do we need to discuss whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group. And whether a UE can support both retransmission schemes simultaneously. For Proposal 4-3, we prefer to confirm the WA after solving our concern in our contribution and copied below for convenience. When a UE is configured with multiple SPS PDSCHs, it is possible that more than one SPS PDSCH are time domian overlapped in a slot. In NR Rel-16, the UE is expected to detect the SPS PDSCH with lowest configuration index and generate HARQ-ACK for this SPS PDSCH only (UE does not generate HARQ-ACK for the other overlapped SPS PDSCHs). This makes gNB configure a SPS PDSCH of higher priority with lower configuration index. Then, if UE is configured with SPS unicast PDSCH and SPS group-common PDSCH, it is also possible that more than one SPS PDSCH is overlapped in a slot. Then one simple way to handle this case is to resue the machanism in NR Rel-16, i.e. UE is exepcted to detect the SPS PDSCH with lowest configuration index. However, this would be not suitable for SPS group-common PDSCH and SPS unicast PDSCH if group-common PDCCH is used for the activation/deactivation of SPS group-common PDSCH. That is because, if group-common PDCCH is used for activation, that means the SPS group-common PDSCH configuration index among a group UE is the same. Taking an example as following: - UE 1 is configured with 7 SPS unicast PDSCHs and 1 SPS group-common PDSCH, where the SPS group-common with the lowest priority. - Follwoing the reception priority principle in NR Rel-16, gNB will configure the SPS group-common PDSCH with index 7 - UE 2 is configured with 7 SPS unicast PDSCHs and 1 SPS group-common PDSCH, where the SPS common PDSCH with the highest priority - Follwoing the reception priority rinciple in NR Rel-16, gNB will configure the SPS group-common PDSCH with index 0 In this example, if UE 1 and UE 2 in the same MBS group, and the SPS group-common PDSCH is for the same MBS service, then it wouldn’t be able to configure the SPS group-common PDSCH with a suitable index for both UE 1 and UE 2. For example, if the SPS group-common PDSCH is configured with a index of 7, it can satisfy UE’s service requirement by it would totally confict with UE 2’s service requirement. To satisfy both UE’s requirement, SPS configuration index should take value from [0,15], that is for UE 2, its SPS unicast PDSCH configuration index is lager than 7. However, if there are more UEs in the MBS group share the same SPS PDSCH, the case would be more complex and value range of SPS configuration index would be very larger, which is not practical to enlarge the value range of SPS configuration index.. |
| Moderator | Proposal 4-1/4-2/4-4: Updated based on comments. Proposal 4-3: Vivo has concern on group-common PDCCH for activation of SPS group-common PDSCH, hope companies can further think and try to address vivo’s concern. I keep this proposal unchanged, it may be updated based on more discuss. |

## Updated Proposals (after 1st round of inputs)

[High] Updated Proposal 4-1:

For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total.

- It is up to gNB implementation to allocate the total SPS configurations between unicast and MBS.

- FFS the SPS configuration index for MBS

[High] Updated Proposal 4-2:

Support PTM scheme 1 or PTP for the retransmission of SPS group-common PDSCH.

- FFS: whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

[High] Initial Proposal 4-3:

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

[High] Updated Proposal 4-4:

Define G-CS-RNTI at least for activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

- FFS: number of G-CS-RNTI.

## Company Views (2nd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 4-1: Generally OK to us. For the second bullet, we think it is not necessary for the FFS as it is still up to gNB implementation. 4-2: We are OK with the main bullet and not OK with the sub-bullet. The reason is similar to Proposal 3-4. 4-3: Agree. 4-4: Agree with the main bullet. For the FFS, does it imply multiple G-CS-RNTIs for multiple SPS MBS services? |
| Samsung | Agree with the proposals (don’t agree with some FFS … but they are FFS). |
| Nokia, NSB | 4-1: Support 4-2: Support 4-3: Support – note we support using UE-specific PDCCH for activation/deactivation 4-4: Support |
| Qualcomm | Agree. In addition, in 4-4, it is also possible to use G-CS-RNTI for dynamic PTM-1 retransmission of the SPS GC-PDSCH without PDCCH. |
| Convida | For Updated Proposal 4-1: Fine with the proposal. For Updated Proposal 4-2: For the main bullet, we prefer the previous version ‘Support PTM scheme 1 and PTP for the retransmission of SPS group-common PDSCH’. The current version with ‘or’ may be interpreted as only one scheme will be supported which is not aligned with the intention that both of the two schemes are supported. For Updated Proposal 4-3: Fine with the proposal. We think UE-specific PDCCH should also be supported. For Updated Proposal 4-4: Fine with the proposal. |
| LG | P4-1: We are fine with the updated proposal. P4-2: We can live with the updated proposal. Simultaneous PTM/PTP retransmissions would be beneficial in some cases e.g. different UE-dedicated BWPs for different UEs, different TCIs states for different UEs in the same MBS group. P4-3: We are fine with the initial proposal. P4-4: We are fine with the updated proposal. We could add FFS: use of G-CS-RNTI for retransmission of SPS group common PDSCH. |
| Huawei/HiSilicon | 4-1, with the main bullet as it, the sub-bullet can be reworded into: - It is up to gNB implementation to configure the SPS configurations index for unicast and multicast, respectively. 4-2, clarification question, the point is to specify one of PTM1 and PTP for SPS retransmission or the point is to specify two but support one at a given time? As we have agreed PTM1 and PTP for non-SPS, I don’t see the reason to preclude one for SPS because both of them have their use cases. 4-4, add FFS RNTI for retransmission of SPS multicast. |
| CATT | Proposal 4-1: OK with the main bullet and first sub-bullet. The second sub-bullet of FFS is not necessary, or we prefer the previous wording if the SPS index is supposed to be defined. Proposal 4-2: ONLY Support the main bullet. Proposal 4-3: OK. Proposal 4-4: OK. |
| NTT DOCOMO | Proposal 4-1: We are fine with the proposal. Proposal 4-2: For the main bullet, we prefer the original proposal. The reason is as Convida says. Proposal 4-3: We are fine with the proposal. Proposal 4-4: We are fine with the proposal. |
| Google | P4-1: Support the proposal P4-2: Similar to the P3-4, we prefer following proposal Proposal 4-2: For SPS group-common PDSCH, if PTM scheme 1 is used for initial transmission, PTP retransmission can be configured to a UE in addition to the PTM scheme 1 retransmission. - Different UEs in the same MBS group can be configured with different retransmission schemes simultaneously, e.g. PTM scheme 1 and PTP retransmission, or PTM scheme 1 retransmission only. P4-3: Support the proposal P4-4: Support the proposal |
| vivo | 4-1: ok 4-2: OK 4-3: we still have concern to confirm the WA. We hope to discuss the issue raised above before confirming the WA. 4-4: can’t understand why G-CSI-RNTI is only for activation/deactivation. For SPS unicast, CS-RNTI is used for activation/deactivation, SPS PDSCH scrambling and retransmission. For SPS group-common, all companies support PTM scheme 1 for retransmission, G-CSI-RNTI is also needed for SPS PDSCH scrambling and retransmission. |
| ZTE | For Proposal 4-4, if G-CS-RNTI is used for at least activation/deactivation of SPS group-common PDSCH, then what about the RNTI for SPS PDSCH for group common PDSCH? Maybe we can update it as following. Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.  FFS: number of G-CS-RNTI. FFS: RNTI for the retransmission of SPS group-common PDSCH |
| Spreadtrum | Proposal 4-1: generally fine. FFS item could be deleted, for the first bullet has said that it is up to gNB implementation to allocate SPS configuration. Proposal 4-2: generally fine. FFS item could be deleted, we have not seen the necessary Proposal 4-3:fine Proposal 4-4: fine |
| MTK | Proposal 4-1: We are fine with the updated proposal. Proposal 4-2: We are fine with the updated proposal. Proposal 4-3: OK. Proposal 4-4: We are fine with the updated proposal. |
| OPPO | Support the proposals. |
| CMCC | Support |
| Ericsson | P4-1: We agree P4-2: Support in principle, but we agree with Huawei that the proposal could lead to confusion by changing “and” to “or” in the sentence. In our understanding, the “and” in the sentence meant that the standard supports both options, i.e. the network could choose to configure PTP or group scheduling for retransmission. We also support the FFS. P4-3: We agree P4-4: We agree |
| Apple | We are ok with the proposals. |
| Moderator | Proposal 4-1: Updated based on Huawei’s comment. Proposal 4-2: Some companies prefer to delete the FFS, some prefer to keep it. Considering it is FFS anyway, I think we can keep it. @Huawei, regarding your question, the intention is the later one, i.e., to specify both but support one at a given time. Proposal 4-3: Vivo still has concern on group-common PDCCH for activation of SPS group-common PDSCH, hope companies can try to address vivo’s concern copied below. When a UE is configured with multiple SPS PDSCHs, it is possible that more than one SPS PDSCH are time domian overlapped in a slot. In NR Rel-16, the UE is expected to detect the SPS PDSCH with lowest configuration index and generate HARQ-ACK for this SPS PDSCH only (UE does not generate HARQ-ACK for the other overlapped SPS PDSCHs). This makes gNB configure a SPS PDSCH of higher priority with lower configuration index. Then, if UE is configured with SPS unicast PDSCH and SPS group-common PDSCH, it is also possible that more than one SPS PDSCH is overlapped in a slot. Then one simple way to handle this case is to resue the machanism in NR Rel-16, i.e. UE is exepcted to detect the SPS PDSCH with lowest configuration index. However, this would be not suitable for SPS group-common PDSCH and SPS unicast PDSCH if group-common PDCCH is used for the activation/deactivation of SPS group-common PDSCH. That is because, if group-common PDCCH is used for activation, that means the SPS group-common PDSCH configuration index among a group UE is the same. Taking an example as following: - UE 1 is configured with 7 SPS unicast PDSCHs and 1 SPS group-common PDSCH, where the SPS group-common with the lowest priority. - Follwoing the reception priority principle in NR Rel-16, gNB will configure the SPS group-common PDSCH with index 7 - UE 2 is configured with 7 SPS unicast PDSCHs and 1 SPS group-common PDSCH, where the SPS common PDSCH with the highest priority - Follwoing the reception priority rinciple in NR Rel-16, gNB will configure the SPS group-common PDSCH with index 0 In this example, if UE 1 and UE 2 in the same MBS group, and the SPS group-common PDSCH is for the same MBS service, then it wouldn’t be able to configure the SPS group-common PDSCH with a suitable index for both UE 1 and UE 2. For example, if the SPS group-common PDSCH is configured with a index of 7, it can satisfy UE’s service requirement by it would totally confict with UE 2’s service requirement. To satisfy both UE’s requirement, SPS configuration index should take value from [0,15], that is for UE 2, its SPS unicast PDSCH configuration index is lager than 7. However, if there are more UEs in the MBS group share the same SPS PDSCH, the case would be more complex and value range of SPS configuration index would be very larger, which is not practical to enlarge the value range of SPS configuration index. Proposal 4-4: Updated based on comments. @Lenovo, it could be further studied whether multiple G-CS-RNTIs for multiple SPS MBS services can be supported. |

## Updated Proposals (after 2nd round of inputs)

[High] Updated Proposal 4-1:

For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total.

- It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively.

[High] Updated Proposal 4-2:

Both PTM scheme 1 and PTP can be used for the retransmission of SPS group-common PDSCH.

- FFS: whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

[High] Initial Proposal 4-3: (Hope companies try to address vivo’s concern)

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

[High] Updated Proposal 4-4:

Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

- G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH if PTM scheme 1 is supported as the retransmission scheme

- FFS: number of G-CS-RNTI.

## Company Views (3rd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## Updated Proposals (after 3rd round of inputs)

# Issue #5: Simultaneous operation with unicast reception

## Background and submitted proposals

Background

In RAN1#103-e, the following agreement was achieved.

Agreements: Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

- Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

- Case 2: support TDM among multiple group-common PDSCHs in a slot

- Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

In RAN1#104-e, the following proposals were discussed but not agreed.

Proposal:

For Rel-17 MBS UE, the maximum number of TDMed PDSCH receptions, including unicast PDSCH(s) and group-common PDSCH(s), that can be supported in a slot per CC is N, where

- N=1 as mandatory

- N=2/4/7 subject to UE capability

Proposal:

At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs

- Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC

  - FFS: the value(s) of M

- Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of N

- Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC

  - FFS: the value(s) of K and L

- FFS Case 5: support FDM among T (T=2) group-common PDSCHs in a slot per CC

  - FFS whether T>2 is supported or not

- Note: This proposal does not negate the previous agreement to support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability.

Submitted Proposals

- Spreadtrum

  - Proposal 10: For simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs,

    - The capability signaling is optional;

    - Support TDM between M TDMed unicast PDSCHs and one group-common PDSCH in a slot

      - FFS: the value of M

      - FFS: per CC, or across CC

    - Support TDM among K group-common PDSCHs in a slot

      - FFS: the value of K

      - FFS: per CC, or across CC

    - Support TDM between L TDMed unicast PDSCHs and T TDMed group-common PDSCHs in a slot

      - FFS: the value of L, T

      - FFS: per CC, or across CC

- vivo

  - Proposal 2: For simultaneous reception of unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs, support the following cases.

    - Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

    - Case 2: support TDM among multiple group-common PDSCHs in a slot

    - Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

    - Case 5: support FDM among multiple group-common PDSCHs in a slot

- CATT

  - Proposal 26: When the simultaneous reception of unicast and multicast is out of a UE’s capability, a dropping principle should be considered.

- Nokia

  - Proposal-22: Agree to maintain existing limitations and requirements in terms of the maximum number of PDSCHs in a slot simultaneously received per UE.

  - Proposal-23: Prioritize the support for TDM between one or more unicast and group-common PDSCHs over the FDM options.

- CMCC

  - Proposal 24. Support the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

    - Case 1: TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

    - Case 2: TDM among multiple group-common PDSCHs in a slot

    - Case 3: TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

  - Proposal 25. The maximum number of PDSCHs in a slot simultaneous received per UE can be 2, 4, or 7 based on UE capability, and regardless that the PDSCH is unicast PDSCH or group-common PDSCH.

  - Proposal 26. Not support the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

    - Case 4: FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

    - Case 5: FDM among multiple group-common PDSCHs in a slot

- Intel

  - Observation 1: The use case for multiple simultaneous MBS PDSCH reception should be clarified further before choosing supported cases for multiplexing. The total number of PDSCHs that can be simultaneously received may be subject to UE capability.

  - Proposal 18: The reception of MBS and unicast in FDM mode should be a UE capability

- Samsung

  - Proposal 7: The number of TDM (MBS or unicast) PDSCH receptions in a slot is same as for the corresponding Rel-16 UE capability.

  - Proposal 8: Support of FDMed PDSCH receptions is for one unicast PDSCH and one group-common PDSCH.

- Ericsson

  - Observation 7: The support of case 1-5 depends on the UE capabilities to monitor multiple PDCCH candidates with different G-RNTI and C-RNTI

  - Observation 8: The support of interslot TDM only requires the UE to monitor 1 PDSCH per slot, which is supported by the basic legacy UE capability (N=1).

  - Proposal 16: Inter-slot TDM is supported with exisiting UE capability. The support of intra-slot TDM cases for MBS are up to UE capability.

## Initial Proposals based on contributions

Summary

Regarding the simultaneous operation with unicast reception, it seems 3 companies [Samsung, Nokia, CMCC] share similar view that the number of TDMed PDSCH receptions (including unicast PDSCH and group-common PDSCH) in a slot is the same as for the corresponding Rel-16 UE capability. However, in last meeting discussion, some companies suggested to discuss this issue in the UE feature discussion phase. Regarding the case 1~5, it seems 6 companies [Spreadtrum, vivo, Nokia, CMCC, Samsung, Ericsson] support the intra-slot TDM cases (case 1~3) subject to UE capability.

Initial Proposals

The following moderator recommendations are made.

[Moderator’s recommendation]

[Medium] Initial Proposal 5-1:

For Rel-17 MBS UE, the maximum number of TDMed PDSCH receptions, including unicast PDSCH(s) and group-common PDSCH(s), that can be supported in a slot per CC is N, where

- N=1 as mandatory

- N=2/4/7 subject to UE capability

[High] Initial Proposal 5-2:

At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs

- Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC

  - FFS: the value(s) of M

- Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of N

- Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC

  - FFS: the value(s) of K and L

## Company Views (1st round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | 5-1: N=4 can’t occupy all the 14 symbols in one slot. |
| Spreadtrum | Fine with proposal 5-1; Fine with proposal 5-2; |
| Apple | For Proposal 5-2, if case 3 is agreed, case 1 and 2 are not necessary. UE capability could be discussed later. |
| CATT | Proposal 5-1: Generally OK. The maximum number is up to UE capability, so there is no necessary to specify the max. number. Proposal 5-2: Generally OK. Similar view with 5-1, it is up to UE capability supporting the maximum number. Furthermore, one question about FDM: the current proposal is only about TDM, are we going to discuss the cases of FDM later or will not be discussed? |
| OPPO | Proposal 5-1: can be discussed in UE feature phase. Proposal 5-2: agree. |
| ZTE | For Proposal 5-1: Support For Proposal 5-2: Support We also support FDMed multicast PDSCHs. |
| CMCC | 5-1~5-2: Agree. |
| NTT DOCOMO | Proposal 5-1: We are fine with FL’s proposal. Proposal 5-2: We are fine with FL’s proposal. |
| LG | P5-1: We are fine with the initial proposal. P5-2: We are fine with the initial proposal. |
| MTK | We suggest that these issues are discussed in the UE feature phase. |
| Samsung | Agree with both proposals. |
| Qualcomm | Not urgent to discuss UE capability for now. |
| Nokia, NSB | Proposal 5-1: Agree with the proposal Proposal 5-2: Agree with the proposal We agree with ZTE and also support FDM options, if there is wider support. |
| Ericsson | P5-1: We agree P5-2: We agree |
| vivo | Fine with Proposal 5-1; For Proposal 5-2, we support case 1, case 2 and case 3. We also support unicast PDSCH and group-common PDSCH multiplexed in FDMed manner. |
| Moderator | Based companies’ input, I will postpone the discussion for proposal 5-1, but I think have an agreement on proposal 5-2 is helpful for the discussion for HARQ-ACK feedback in AI 8.12.2. I keep proposal 5-2 unchanged to see if any company have concern on it. @CATT, I don’t plan to discuss FDM case in this meeting, since it seems not possible to have an agreement in this meeting for the FDM case based on companies’ proposals. |

## Updated Proposals (after 1st round of inputs)

[High] Initial Proposal 5-2:

At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs

- Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC

  - FFS: the value(s) of M

- Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of N

- Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC

  - FFS: the value(s) of K and L

## Company Views (2nd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | Agree. |
| Samsung | OK |
| Nokia, NSB | OK we also support FDM options, if there is wider support |
| Qualcomm | Ok We also support FDMed multicast and multicast, similar as FDMed unicast and multicast. |
| LG | P5-1: We are fine with the initial proposal. |
| CATT | OK with the intention. But we think it is more valuable to discuss the cases how to deal with the case when simultaneous receptions is out of UE’s capability. The rules of priority / dropping is necessary to define since UEs in the same group may not have the same capability. |
| vivo | Ok. we also support FDM options |
| ZTE | OK, we also support FDM options. |
| Spreadtrum | Fine. Regarding FDM option, we don’t support. On one hand, the benefit is not clear, but huge effort is needed, e.g., type-1/type-2 codebook enhancement; on the other hand, it would obviously increase UE’s complexity. |
| MTK | We have the similar view as Spradtrum, there is no clear motivation to support FDMed multiple multicast. FDMed 1 unicast PDSCH and 1 groupcast PDSCH are sufficient in R17 MBS. |
| OPPO | ok |
| CMCC | Support |
| Ericsson | P5-2: We agree |
| Apple | Fine with proposal. |
| Moderator | Proposal 5-2: It seems all companies are fine with this proposal, so I marked it as stable. Based on CATT’s comment, I added proposal 5-3. |

## Updated Proposals (after 2nd round of inputs)

[High](stable) Initial Proposal 5-2:

At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs

- Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC

  - FFS: the value(s) of M

- Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of N

- Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC

  - FFS: the value(s) of K and L

[High] Initial Proposal 5-3:

When the simultaneous reception of unicast and multicast in a slot is out of a UE’s capability, a dropping principle should be considered.

## Company Views (3rd round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## Updated Proposals (after 3rd round of inputs)

# Issue #6: Other issues (multi-beam, broadcast for RRC_CONNECTED UEs, etc.)

## Background and submitted proposals

Submitted Proposals

- OPPO

  - Proposal 13: A separate TCI states space is activated by MAC CE for group common PDSCH.

- ZTE

  - Proposal 9: Association between MOs of group-common PDCCH and SSBs or CSI-RSs should be defined for beam sweeping transmission of NR MBS.

    - Considering full beam sweep for broadcast transmission.

    - Considering partial beam sweep for multicast transmission.

  - Proposal 13: RAN1 further studies whether to support HARQ-ACK feedback for broadcast service for UEs under RRC_CONNECTED state.

- CMCC

  - Proposal 19. For RRC_CONNECTED UEs, only the group-common PDCCHs belong to broadcast service reported in MBS Interest Indication procedure are counted in the monitored CSS PDCCH candidates  and non-overlapping CCEs   in a slot or span.

  - Proposal 20. Define a new CSS type for group-common PDCCH of PTM transmission scheme 1 for broadcast in RRC_CONNECTED state.

- Intel

  - Proposal 10: For delivery mode 2, MBS configuration is provided via a MCCH carried over a PDSCH scheduled by a PDCCH carrying a DCI format 1_0 with CRC scrambled with a new SC-RNTI. Change in configuration can be notified through a PDCCH carrying another DCI 1_0 with CRC scrambled with SC-N-RNTI.

  - Proposal 11: The PDCCH scheduling the MCCH is monitored in Type0 CSS set configure by searchSpaceZero or Type0A CSS set or alternately on new mcch-searchSpace which is a CSS configured by the MBS specific PDCCH-ConfigCommon.

  - Proposal 20: NR MBS uses PDSCH Mapping Type A with DM-RS Type 1 as a baseline. PDSCH Mapping Type B and use of Type 2 DM-RS are not precluded.

  - Proposal 21: For NR MBS support of multi-layer MIMO transmission with rank adaptation (from UE perspective) is not precluded.

  - Proposal 22: For groupcast transmission, all UEs within the group share the same DM-RS port(s). Additionally, UEs receiving unicast transmission are multiplexed on remaining orthogonal DM-RS ports.

  - Proposal 23: Advanced transmission schemes like multiuser superposition transmission (MUST) for improving group spectral efficiency are not precluded

- LGE

  - Proposal 9: support transmission of multiple TDMed group-common PDSCHs carrying a same TB with selectively different RSs for both broadcast and multicast. Different UE in the group selectively receive same or different PDSCHs among TDMed PDSCHs carrying the TB.

  - Proposal 10: Multiple TCI states can be configured in PDSCH-config for group common PDSCH for the CFR.

  - Proposal 11: From gNB perspective, gNB may configure multiple CORESETs and transmit group common PDCCHs to multiple UEs in a group. The DCI can be repeated on multiple CORESETs with same or different TCI states

  - Proposal 12: Multiple TCI states can be configured for a CORESET ID for a Search Space of group common PDCCH by RRC.

- Sony

  - Proposal 1: Support dedicated beam configuration for MBS beam report to identify suitable beams for group-common PDSCH/PDCCH in addition to unicast.

  - Proposal 2: The network shall configure time/frequency resources of the beam sweeping for the  group common PDCCH/PDSCH.

- NTT Dococmo

  - Proposal 10: The default QCL assumption of group-common PDSCH should be specified for the case that the time offset between the group-common PDCCH and the corresponding PDSCH is less than the threshold timeDurationForQCL.

- ASUSTeK

  - Proposal 1: For NR MBS group-scheduling, a reference TDRA table for mapping the group-common PDSCH transmission occasion in time domain needs to be identified and known to a corresponding group of UEs.

  - Proposal 2: A “group-common TDRA table” is configured per MBS group for NR MBS group-scheduling.

## Initial Proposals based on contributions

Summary

Moderator does not plan to discuss these issues in this meeting currently, if more companies propose to discuss some of the proposals, moderator will take that into account in the next round discussion.

## Company Views (1st round of inputs)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LG | We could possibly discuss configuration of TCI states for group common PDCCH/PDSCH during this meeting. |
| Samsung | Agree. Regarding multi-beam proposals, please also note the following from the WID. FR2: we assume that there are no issues to provide Multicast / Broadcast transmissions in FR2. If any enhancements is needed it should be treated with lower priority compared to the minimum set of objectives above. |

# Proposals for GTW session

# References

- RP-193248	New WID proposal: NR Multicast and Broadcast Services

- RP-201038	Revised WID: Core part: NR multicast and broadcast services

- R1-2102318	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2102414	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2102469	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2102501	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2102542	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2102609	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2102655	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

- R1-2102702	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2102782	Discussion on aspects for group scheduling	FUTUREWEI

- R1-2102844	Discussion on common frequency resource configuration for MBS	ETRI

- R1-2102900	Discussion on group scheduling mechanisms	CMCC

- R1-2103050	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

- R1-2103124	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

- R1-2103186	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2103260	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

- R1-2103316	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2103357	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2103362	Further discussion on group scheduling for RRC_CONNECTED UEs	Chengdu TD Tech, TD Tech

- R1-2103419	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

- R1-2103546	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

- R1-2103594	Discussion on group scheduling mechanism for RRC_CONNECTED Ues	NTT DOCOMO, INC.

- R1-2103658	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

- R1-2103738	Mechanisms to support MBS group scheduling for RRC_CONNECTED UEs	Ericsson

# Appendix 1: Agreements in #102 e-meetings

RAN1#102-e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

o   FFS: whether to support UE-specific PDCCH to schedule a PDSCH for MBS.

Agreements:

- For RRC_CONNECTED UEs, define/configure common frequency resource for group-common PDSCH.

  - FFS: whether to reuse the BWP framework or not

  - FFS: the relation between the common frequency resource and UE dedicated BWP, e.g., the common frequency resource is a MBS specific BWP, or the common frequency resource is confined within UE’s dedicated BWP, etc.

  - FFS: whether more than one common frequency resource can be configured per UE

Agreements:

- For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability.

  - FFS: TDM or SDM in a slot.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

# Appendix 2: Agreements in #103 e-meetings

RAN1#103-e

Mechanisms to support group scheduling for RRC_CONNECTED UEs

Agreements: For convenience of discussion, consider the following clarification as RAN1 common understanding.

- PTP transmission: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme.

- PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme.

- Note: The ‘UE-specific PDCCH / PDSCH’ here means the PDCCH / PDSCH can only be identified by the target UE but cannot be identified by the other UEs in the same MBS group with the target UE.

- Note: The ‘group-common PDCCH / PDSCH’ here means the PDCCH / PDSCH are transmitted in the same time/frequency resources and can be identified by all the UEs in the same MBS group.

- FFS whether or not to have additional definition of transmission scheme(s)

Agreements: For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support retransmission(s) can use PTM transmission scheme 1.

- FFS: whether to support PTP transmission for retransmission(s).

- FFS: whether to support PTM transmission scheme 2 for retransmission(s).

- FFS: How to indicate the association between PTM scheme 1 and PTP transmitting the same TB.

- FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different UEs in the same group?

Working assumption:

For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

    - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

  - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the group-common PDCCH / PDSCH configuration

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

Agreements: Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

Agreements: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

- FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation

- FFS: whether to support more than one SPS group-common PDSCH configuration per UE

- FFS: whether and how uplink feedback could be configured

- FFS: retransmission of SPS group-common PDSCH

Agreements: For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH.

- FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, the CCE indexes are common for different UEs in the same MBS group.

Agreements: Down select from the two options for BDs/CCEs limit for Rel-17 MBS

- Option 1: the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- Option 2: For UEs supporting CA capability, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

Agreements:For RRC_CONNECTED UEs, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots (mandatory for the UE supporting MBS).

Agreements:Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

- Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

- Case 2: support TDM among multiple group-common PDSCHs in a slot

- Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

Agreements:For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options.

- Option 1: Define a new search space type specific for multicast

- Option 2: Reuse the existing CSS type(s) in Rel-15/16

  - FFS: whether modifications are needed for multicast

- Option 3: Reuse the existing USS in Rel-15/16 with necessary modifications for MBS

  - FFS: detailed modifications

Agreements:No specification enhancement in Rel-17 to support SDM between unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs.

Agreements: For PTM transmission scheme 1, if Option 2A or Option 2B for common frequency resource for group-common PDCCH/PDSCH is agreed, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource.

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options for the monitoring priority of search space set

- Option 1: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS

- Option 2: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS

- Other options are not precluded

- The monitoring priority is used at least for PDCCH overbooking case

  - FFS for other cases (e.g., to prune PDCCH in terms of whether it’s unicast or multicast, etc.)

Mechanisms to improve reliability for RRC_CONNECTED UEs

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

- FFS details

Agreements:

- For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

  - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of the common frequency resource(s) (if configured) and initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Agreements: From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs.

- FFS details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS type, define a new CSS type, etc.

- FFS other details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH

# Appendix 3: Agreements in #104 e-meetings

RAN1#104-e

Mechanisms to support group scheduling for RRC_CONNECTED UEs

Agreement:

For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

    - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

  - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the group-common PDCCH / PDSCH configuration

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

- FFS whether the use of a common frequency resource for multicast is optional or not

- FFS whether the common frequency resource is applicable for PTM scheme 2 (if supported) or not

Agreement:

- If Option 2B is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the starting PRB and the length of PRBs of the MBS frequency region within a dedicated unicast BWP are configured via UE-specific RRC signaling.

  - The starting PRB is referenced to one of the two options:

    - Option 1: Point A

    - Option 2: the starting PRB of the dedicated unicast BWP

  - FFS the detailed signaling

- If Option 2A is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the configurations of the starting PRB and the length of PRBs of the MBS frequency resource reuse the legacy BWP configuration.

Agreement:

For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission.

- The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

Agreement:

The maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- FFS whether the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

Working Assumption:

Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

Agreement:

For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability

- The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS.

- FFS: How to allocate the total SPS configurations between MBS and unicast.

Agreement:

For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS

- FFS: The retransmission scheme(s)

- FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2

Agreement:

From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations:

- Starting PRB and the number of PRBs

- One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP)

- One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP)

- SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP)

- FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP

- FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED

- FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed

- Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2

- Note: This agreement does not negate any previous agreements made on CFR

Agreement:

For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, at least support CSS

- FFS: reuse existing CSS type(s) in Rel-15/16 or define a new Type CSS

- FFS: Two options for monitoring priority:

  - Option 1: the monitoring priority is the same as existing Rel-15/16 CSS

  - Option 2: the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

Working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Mechanisms to improve reliability for RRC_CONNECTED UEs

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on concatenating the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, one common frequency resource for group-common PDCCH/PDSCH can be defined/configured.

- FFS: whether to define/configure more than one common frequency resources

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB.

- It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes.

- FFS: association rules between SSB indexes and UE monitoring occasions.

- FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured

Agreement:

For broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE UEs and the SCS and CP are the same.

- FFS: the case when UE-specific active BWP of RRC_CONNECTED UE does not contain the common frequency resource of RRC_IDLE/INACTIVE UEs.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported:

- [Case E] the case where a CFR is defined based on a configured BWP.

  - In particular, study the following:

    - whether a configured BWP for MBS is needed or not.

    - whether BWP switching is needed or not.

  - In this study, the configured BWP has the following properties:

    - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth.

    - The CFR has the frequency resources identical to the configured BWP.

    - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

  - Note: The configured BWP is not larger than the carrier bandwidth

- the case where the initial BWP fully contains the CFR in the frequency domain.

  - In this study the following sub-cases are considered:

    - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

    - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS.

- the case where the initial BWP has same size as the CFR in the frequency domain.

  - In this study the following two sub-cases are considered:

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS.