3GPP TSG RAN WG1 #104b-e-	R1-2104082

e-Meeting, April 12th – 20th, 2021

Source:	moderator (vivo)

Title:	Feature lead summary#4 on Enhancements on Multi-TRP inter-cell operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Proposals

Proposed Conclusion 1-1

For discussion purposes only in Rel-17 FeMIMO on inter-cell multi-TRP (AI 8.1.2.2), the term “non-serving cell” is used in the following context:

- A UE receiving DL RS which are QCLed to an SSB with a PCI different from a serving cell PCI reusing R15 QCL rules as concluded in RAN1#104-e can be said to be receiving the DL RS from a non-serving cell

- A UE receiving PDSCH/ UE-specific PDCCH QCLed to an SSB with a PCI different from a serving cell PCI reusing R15 QCL rules as concluded in RAN1#104-e can be said to be receiving the UE-specific PDCCH/PDSCH from a non-serving cell

Supported by: Lenovo/MotM, Samsung, Ericsson, OPPO, Nokia, IDC

Opposed by:

Updated Proposal 2

For intercell MTRP operation, with the additional PCI different from that of the serving cell,

- Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with one CORESETPoolIndex.

Supported by: Lenovo/MotM

- Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex, and more than one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with one CORESETPoolIndex

Supported by:

Note: signaling details on association of PCI, TCI states, CORESETpoolindex is separate discussion

Possible Agreement

Clarify that “SSB time domain position” for non-serving cell SSB consists of “halfFrameIndex” and “ssb-PositionsInBurst”

Supported by: QC, Apple, Nokia, Lenovo/MotM, Ericsson, OPPO, Nokia, CATT

Opposed by: LG, Huawei, HiSilicon, (ZTE)

Reference

Previous agreements

In RAN1 #102e meeting, the following agreements were made:

Agreement

Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation.

- Details on configuration of non-serving cell RS;

- Allowed source and target RS types for RS transmitted from the non-serving cell TRP ;

- Allowed QCL types for RS transmitted from the non-serving cell TRP ;

- Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any;

- Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels;

- Other details not precluded.

In RAN1#103e meeting, further agreements were made as below:

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

For future meetings

Consider rate matching behavior related to non-serving cell SSB.

In RAN1#104e meeting, further agreements were made as below:

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.