3GPP TSG RAN WG1 Meeting #105-e	R1-2104193

e-Meeting, May 19 – May 27, 2021

Agenda Item:	8.11.1.2

Source:	FUTUREWEI

Title:	Discussion on techniques for inter-UE coordination

Document for:	Discussion and decision

# Introduction

At RAN#86, a WI on sidelink enhancements was agreed for Rel-17 [1] and modified in [2]. In this WI, there is an objective on resource allocation enhancements to enhanced reliability:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

In RAN1#104e, an LS was sent to RAN [8] informing the status the study, with a companion document [9] capturing the detailed observations from the evaluation results for inter-UE coordination in Mode 2.

Conclusion: - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature. - The detailed observations can be found in the attachment of the LS

It is concluded in RAN#91e that no WID update is necessary. WGs continue specifying inter-UE coordination.

Based on the discussions and conclusions from RAN1#104b-e [11], this contribution continues the discussions on inter-UE coordination techniques and schemes.

# UE coordination

## Background

In RAN1#102, a discussion paper [5] discussed several views that was later captured in a summary document [4]. There were several proposals in [4].

- Companies are encouraged to consider at least the following aspects when studying the feasibility and benefit of the enhancement(s) in mode 2

- Hidden-node problem

- Exposed-node problem

- Half duplex problem

- Consecutive packet loss (as described in WID)

- [Resource collision (i.e., Time-frequency resource overlapping [and/or Time resource overlapping] caused by the reason other than hidden-node problem]

In RAN1#103-e meeting, RAN1 listed up three types of “A set of resources” for inter-UE coordination in Mode 2:

- Type A: UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

  - e.g., based on its sensing result

- Type B: UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

  - e.g., based on its sensing result and/or expected/potential resource conflict

- Type C: UE-A sends to UE-B the set of resource where the resource conflict is detected

In [6] we stated significant benefits for UE coordination:

- Better system performance, since exchanging resources can enable low-level interference coordination, thus reducing collisions

- Reduced complexity UEs that do not need to perform sensing

- Reduced power consumption since one UE can provide the resource allocation for other UEs, which then do not need to listen all the time

- Deployment of hierarchical systems, which are particularly useful for public safety.

We also provided some scenarios in [6] showing that UE coordination can be used for relaying and hierarchical structure. It can be especially useful for public safety services where a central controller can allocate resources for other UEs. It can also be useful for pedestrian UEs, where a RSU can allocate resources for the pedestrian UEs [7].

In [10], we discussed the simulation results for broadcast, groupcast, and unicast and examined some UE coordination schemes, as well as reiterated the obvious benefits of UE coordination with a use case.

## Discussions on Inter-UE Coordination

### UE-B Usages of Coordination Resources

In RAN1#103-e meeting, three types of resources for inter-UE coordination are defined, namely, type A preferred resources, type B not-preferred resources, and type C resources with conflicts. Based on these three types of resources, in RAN1#104b-e meeting, two coordination schemes have been agreed where scheme 1 is for coordination A and B, scheme 2 is for coordination C.

In sidelink inter-UE coordination, UE A forms the set of resources based on the one or more of the three types and sends that information to UE B. Upon receiving the set of resources as coordination information from UE A, UE B can perform resource selection purely based on the set of resources in the coordination message from UE A without performing sensing itself. For example, UE B can select a resource from the received type A resource set or select a resource that is not in the received set of type B or type C resources. UE B can also perform resource selection based on the coordination resources and its own sensing results. In some cases, there is no conflict, e.g., UE A sends the information of type B or type C coordination resources to UE B, UE B selects the resource from the available resources from its own sensing results of which the Type B or Type C coordination resources are excluded. However, sometimes a conflict may occur. For example, none of type A preferred resources is among the available resources at UE B according to UE B’s own sensing output. It is then a question what UE B should choose: perform resource selection based on the coordination message from UE A or based on its own sensing results. It can be a general question whether the set of UE A’s preferred resources is strictly followed at UE B for UE B’s resource selection or just serve as a “recommendation”, even if there is no conflict. This general question is also applicable to type B and type C coordination resources.

We propose to support both usages of coordination resources in the specification and one of them can be configured based on some attributes of UE A and UE B, e.g., roles, importance, capabilities, etc. For example, in one use case of V2X sidelink, truck platooning, the leading truck is certainly more important than the trailing trucks as the leading truck makes decisions on the driving actions. If UE A is the leading truck and UE B is the trailing truck, UE B should strictly follow UE A’s coordination message to perform resource selection for communication within the platoon and outside the platoon. When UE A is the trailing truck and UE B is the leading truck, UE B can use UE A’s coordination message as recommendations and may not follow it particularly when there is a conflict. Similarly, in the use case of public safety, when fire engines are dispatched to a fire scene, the incident commander organizes and monitors the whole process. If UE A is the incident commander and UE B is the firefighter, UE B should follow UE A’s coordination message and select the resource accordingly. If UE A is a firefighter and UE B is the incident commander, UE B may use the coordination resources as recommendations and may not strictly follow it. Therefore, due to the difference of attributes of UE A and UE B, the use of inter-UE coordination information at UE B can change accordingly.

Proposal 1: Support both UE B’s usages on coordination resources, i.e., either strictly follow it OR use them as recommendations for UE B’s resource selection. Which of UE B’s usages is based on the attributes or (pre-)configuration of UE A and UE B.

As shown in Appendix, three candidate options for coordination scheme 1 were agreed in the RAN1#104b-e meeting for UE-B’s to take into account the coordination information in the resource (re)-selection for its own transmission copied below and whether

  - For scheme 1:

    - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

    - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

Based on above discussions, both options 1-1 and 1-2 should be supported without needing to down-select from them. Since both option 1-1 and 1-2 cover the resource reselection, option 1-3 may not be needed unless further clarifications are provided.

Proposal 2: Support both option 1-1 and 1-2 for coordination scheme 1 for UE-B’s to take into account the coordination information in the resource (re)-selection for its own transmissions.

### Determination of UE-A and UE-B

Since the inter-UE coordination can only be performed after the link between UE-A and UE-B is established and the sidelink links are established at higher layer, UE-A or UE-B can be determined by higher layers can be based on roles, attributes, and capabilities of the UEs during the link establishment procedure. Determining UE-A and UE-B at the higher layer can reduce the design complexity reduced and the impact to specification.

It is clear that as the intended receiver, a UE can be UE A to provide coordination information to the transmitting UE, UE B, for resource (re)-selection. Also, in the groupcast scenario, as one of the receivers, a UE can provide the coordination information to UE B. In this scenario, it is not necessary that all intended receivers in the groupcast need to provide coordination information. For example, in truck platooning, the leading truck may only request one or several furthest trailing trucks to provide coordination information for the leading truck to select resources for the groupcast. Therefore, UE A can be one of the intended receivers.

On the other hand, the inter-UE coordination process can be used either for pedestrian UEs or public safety UEs. For instance, if RSUs are deployed, a RSU can be located at an area where pedestrians are likely to be present (e.g., intersection, traffic, light, pedestrian crossway, etc.) and use inter-UE coordination as follows: after sensing, the RSU reserves some resources for pedestrian usage for UEs in its vicinity. V2P UEs with SCI detection or data reception capability can obtain the configuration of the reserved resources from the RSU. The V2P UEs can then select the resource from the resources reserved for the V2Ps by the RSU (UE B takes this into account in the resource selection for its own transmission). For the public safety case, a similar solution can be used, with the incident commander as the leader UE, reserving resources for other UEs. When indicating the reserved resource, the signaling from Rel-16 (SCI 1) can be reused to mark resources are reserved. This way, even Rel-16 UEs can benefit from resource reservation. Therefore, UE A may not be any of intended receivers.

Proposal 3: One of the intended receivers can be UE A but it is not necessary that UE A should be one of any intended receivers.

### Inter-UE Coordination without Explicit Triggering

As described in 2.2.2, UE-A and UE-B can be determined by higher layer. Inter-UE coordination can also be configured by higher layer based on roles, attributes, and capabilities of the UEs without explicit triggering. In some use cases, e.g., truck platooning and public safety cases described in Section 2.2.1, it can be beneficial that UE A, e.g., the leading truck or incident commander, provides coordination information to UE B directly without explicit triggering. The coordination can start by UE A when a certain condition is met, and the coordination period is also determined at UE A, possibly with a higher layer signaling. UE A may not need any inputs from UE B to form the coordination information or obtain updated information periodically from UE B through MAC CE without triggering the coordination. During the coordination period, UE A forms the coordination information, e.g., type A, B, or C-like resource sets, and sends it to UE B. One example of such coordination is that for the half-duplex issue, UE A keeps updating the information of its transmitting slot to UE B. UE B then takes the coordination information into account in the resource selection for its own transmission.

Proposal 4: Support inter-UE coordination configured by the high layer without explicit triggering where UE A starts the coordination and sends the coordination information to UE B, and UE B takes the information into account in the resource selection for its own transmission.

### Information needed for forming coordination information

As shown in [11], the information that is needed for UE A to form the coordination information was discussed. A lengthy list was provided. To understand the information needed for UE A to obtain the coordination information, it is better to consider what UE A’s actions or behaviors are needed.

First, UE A can form the coordination information based on its sensing results. Therefore, any information related to UE A’s sensing is needed which include the sensing parameters, configuration, and sensing outcome. Clearly the sensing outcome is the information needed for obtaining the coordination.  When we consider that sensing is one of necessary processes to obtain coordination information, we do not need to put the outcome as a candidate information now. We can then focus on the detail information that is needed for UE A’s sensing process, which will also be helpful to form coordination triggering message and configurations through the upper layer. For UE A’s sensing, some sensing related parameters or requirement may be needed from UE B, e.g., the transmission periodicity, PDB, and resource selection window etc.

Second, UE A’s own transmission and reception information are needed, which is mainly related to half duplex issue. This includes the UE A’s scheduled resources for UL transmissions, NR SL transmissions and receptions for UE A, and UE A’s PSFCH transmission and reception.

The coordination information can be obtained by UE A from other UE’s. Then any related information or process for UE A to receive such coordination information can be further discussed if it is necessary.

When forming the coordination information, e.g., the preferred or non-preferred resource set, based on above three categories of information, i.e., a resource exclusion/selection procedure, additional information is required for such process, e.g., UE B’s resource selection window and UE A’s own candidate resource. Some information such as UE B’s resource selection window requires additional signaling from UE B.

We can see that UE A’s sensing and forming coordination require certain signaling from UE B or other UEs, e.g., incident commander, platoon leader, RSU, which shall be the focus.

Proposal 5: The coordination information can be obtained based on UE A’s sensing process, UA’s own transmission/reception, and the coordination information from others. The coordination information may be derived via resource exclusion/selection process.

Proposal 6: Focus on the UE A’s sensing and forming the coordination information process that requires additional signaling from UE B and/or other UEs

### Container of Inter-UE Coordination Triggering Information

Inter-UE coordination can also be explicitly triggered by UE-B when UE-B needs some coordination information from UE A. The coordination can be explicitly triggered with a trigger message sent to UE A. Besides the coordination triggering, some information for coordination is also needed for UE-A to obtain and send the coordination message to UE B e.g., parameters for UE A sensing procedures and coordination resource pool, etc. The explicit coordination triggering is suitable for both periodic traffic and aperiodic traffic at UE B.

Similar to the coordination message, the delay requirement for coordination triggering is stringent. Therefore, the SCI is a preferred option for delivering the triggering message. We can either modify the existing SCI format or specify a new SCI format to carry the triggering message.

Proposal 7: For triggering UE coordination, the triggering message is delivered by SCI. Modify an existing SCI format or specify a new SCI format to carry the triggering message.

### Container for Inter-UE Coordination Message

The payload size of inter-UE coordination information depends on the size of coordination resource sets. If the payload size is large, using RRC signaling and/or MAC data are good options. However, if the information for inter-UE coordination is based on UE sensing output, this information will be outdated after a certain time period, particularly for detected aperiodic traffic. High layer signaling incurs a large delay, which is thus not suitable for such a coordination message. On the other hand, using a MAC CE can have a smaller delay. However, the transmission of data on PSSCH itself requires UE A to perform the sensing and resource selection processes in order to deliver the message to UE B, and the reliability is less than the control channel information. Therefore, both higher layer signaling and MAC CE are generally not suitable either for delivering real-time coordination information.

As stated in [6][10], the easiest and more appropriate way to have the set of resources determined at UE-A communicated is using the SCI. SCI format 1-A contains the following fields: priority, frequency resource assignment, time resource assignment, and resource reservation period. It seems that these fields could be used as the baseline to indicate the set of resources determined at UE A. Thus, UE coordination can be supported by using SCI format 1-A as the baseline. Any other needed information can be transmitted in a second stage SCI. Due to potentially large set of the resources, efficient design of the 2nd stage SCI to reduce the size of the SCI is desired.

Proposal 8:

- For UE coordination, the set of resources are sent using SCI 1-A as the baseline

  - The resource allocation fields are reused with minor modification

  - Any other information needed can be sent in a second stage SCI

  - Consider efficient design of the second stage SCI to reduce the SCI message size

### UE A Sensing and Reporting Procedures for Inter-UE Coordination

The contents for the three types of resources can be obtained from the sensing process at UE A. Therefore, if sensing is needed at UE A, the sensing process at UE A should be tied to UE B’s resource selection, which needs to be specified. Besides the sensing process at UE A, information exchange between UE B and UE A needs to be determined. Here we discuss the sensing and reporting procedures for periodic and aperiodic traffic at UE B separately.

For periodic traffic, once coordination is triggered, the transmission slot and periodicity can be forwarded to the UE A in the trigger message. The effective coordination time can also be included in the message. UE A can then perform sensing and resource selection procedure similar to that of UE B without coordination. However, for inter-UE coordination, the timing for UE A sensing and processing are different as UE A needs to send the coordination information to UE B in time for UE B to reserve the resources in the resource selection window.

Fig. 1. Timing for UE B sensing and sending coordination message for periodic traffic at UE B.

As illustrated in Fig. 1, when periodic traffic is triggered at slot n for UE B, UE B’s resource selection window is on [n+T1, n+T2]. Without inter-UE coordination, the sensing window for UE B is on slots [n-T0, n-Tproc,0] and T2 is up to UE B’s implementation. With coordination, in order to form any type of resource set at UE A, UE B’s resource selection window has to be known at UE A, including T2. The value of T2 can be included in the triggering message as well as T1. Note T1 can be omitted if UE A sets T1= Tproc,1 by default as the range of T1 is small. Since sensing is performed at UE A, time is needed for UE A to send a coordination message to UE B. Therefore, the timing requirement for UE A’s sensing process is different from UE B’s. For example, as shown in Fig. 1, UE A monitors the slots [n-T0, n-Tr-Tproc,0], where Tr is the time requirement for UE A to send the coordination message to UE B. The coordination report window is then [n-Tr, n] which may include the processing time for UE A transmitting the message.

The aforementioned sensing is mainly for detecting periodic traffic from other UEs. It is also important to detect aperiodic traffic. Contiguous based partial sensing for detecting aperiodic traffic is still under discussion. Since the SCI can only inform resource reservations within a window of 32 slots, the entire UE coordination procedure for one transmission should be done within 32 slots in order to have benefit from the coordination. On the other hand, sensing reliability depends on the sensing window size. Therefore, latency of contiguous sensing for coordination needs to be discussed. Detailed timing needs to be specified to satisfy latency requirements.

Fig. 2. Timing for UE B sensing and sending coordination message for aperiodic traffic at UE B.

For aperiodic traffic at UE B, as shown in Fig. 2, the packet arrives at slot n. The coordination is then triggered at n>n. The earliest time is on slot n+1. After UE A receives the trigger message, it starts sensing on slots [n+TC,A, n+TC,B], then reporting one or more coordination messages to UE B within slots [n+TC,B +Tproc,0 , n+T1,C -Tproc,1] where T1,C is the first slot in the resource selection window. Again, the coordination procedure needs to be completed within 32 slots. To benefit from inter-UE coordination, the duration of the sensing and reporting procedures should allow a sufficient size for the resource selection window at UE B.

Due to different timing requirements for periodic and aperiodic traffic at UE B, sensing and coordination reporting both traffic types have to considered at UE B. A unified design can be considered on sensing and reporting procedures at UE-A for inter-UE coordination, e.g., a triggering process with different parameters in the triggering message for periodic and aperiodic reporting.

Proposal 9: Timing requirements for sensing and reporting procedures at UE A needs to be specified.

Proposal 10: Discuss sensing and reporting procedures for both periodic and aperiodic traffic. Specify triggering process, triggering message, sensing and reporting procedures at UE-A for both periodic and aperiodic traffic.

### Consecutive packet loss

As discussed in [5] and [10], the problem arises when two UEs with the same traffic periodicity or when a periodicity is a multiple of the other periodicity happen to select the same resources. The initial collision is then replicated until resource reselection.

Rel-16 NR V2X provides significant benefits in terms of performance, compared with LTE-V. It is known that LTE-V UEs can suffer of consecutive packet loss [3]. Note that the problem for LTE-V was known, but overall performance was sufficient for Basic Safety Messages (BSM). An example of consecutive collisions as happening for LTE-V is shown in Fig. 3. The initial collision is then replicated until resource reselection because the traffic periodicities are integer multiples of each other, and the same resource is selected.

Fig. 3. Example of recurring consecutive collisions. An '!' indicates a collision.

NR improves the performance and can alleviate consecutive packet loss by e.g., the use of HARQ. Note however that there can still be improvements: for instance, for periodic transmission as shown in Fig. 3, the use of HARQ results in systematic retransmissions until resource reselection, thereby resulting in an inefficient use of the resources.

To solve this issue the receiving UE can send a message to the transmitting UE indicating recurring collisions: in such a case, when receiving packets from the same sensing process (indicated by the same reservation field). Note that if HARQ is used, the transmitting UE knows about consecutive failures based on the NACK feedback. However, given that the consecutive NACKs may be due to other reasons than consecutive collisions (e.g., blockage), so additional feedback can be useful. In addition, if a UE transmits without requiring HARQ feedback, notification of consecutive collision is needed.

The solution can provide better performance since when a UE is notified of consecutive collisions, it can immediately take corrective actions. Thus, this solution is appropriate for unicast or groupcast.

Proposal 11: The receiving UE can signal to the transmitting UE that consecutive collisions occur or based on a collision detection from the sensing outcome.

# Conclusion

Inter-UE coordination techniques were discussed. We observe and propose the following:

Proposal 1: Support both UE B’s usages on coordination resources, i.e., either strictly follow it OR use them as recommendations for UE B’s resource selection. Which of UE B’s usages is based on the attributes or (pre-)configuration of UE A and UE B.

Proposal 2: Support both option 1-1 and 1-2 for coordination scheme 1 for UE-B’s to take into account the coordination information in the resource (re)-selection for its own transmissions.

Proposal 3: One of the intended receivers can be UE A but it is not necessary that UE A should be one of any intended receivers.

Proposal 4: Support inter-UE coordination configured by the high layer without explicit triggering where UE A starts the coordination and sends the coordination information to UE B, and UE B takes the information into account in the resource selection for its own transmission.

Proposal 5: The coordination information can be obtained based on UE A’s sensing process, UA’s own transmission/reception, and the coordination information from others. The coordination information may be derived via resource exclusion/selection process.

Proposal 6: Focus on the UE A’s sensing and forming the coordination information process that requires additional signaling from UE B and/or other UEs

Proposal 7: For triggering UE coordination, the triggering message is delivered by SCI. Modify an existing SCI format or specify a new SCI format to carry the triggering message.

Proposal 8:

- For UE coordination, the set of resources are sent using SCI 1-A as the baseline

  - The resource allocation fields are reused with minor modification

  - Any other information needed can be sent in a second stage SCI

  - Consider efficient design of the second stage SCI to reduce the SCI message size

Proposal 9: Timing requirements for sensing and reporting procedures at UE A needs to be specified.

Proposal 10: Discuss sensing and reporting procedures for both periodic and aperiodic traffic. Specify triggering process, triggering message, sensing and reporting procedures at UE-A for both periodic and aperiodic traffic.

Proposal 11: The receiving UE can signal to the transmitting UE that consecutive collisions occur or based on a collision detection from the sensing outcome.

# References

RP-193257, NR sidelink enhancement, RAN#86-e, Dec 2019.

RP-201385, “WID revision: NR sidelink enhancement,” LG Electronics, RAN#88-e, June 29-July 3, 2020.

R1-1611130, “Discussion on reselection counter in sensing,” Huawei, HiSilicon, RAN1#87, Reno, USA, November 14-18, 2016.

R1-2007412, “Summary for AI 8.11.2.2 Feasibility and benefits for mode 2 enhancements”, LGE, RAN1#102-e, August 17-28, 2020.

R1-2005296, “Views on resource allocation enhancements for sidelink communication”, FUTUREWEI, RAN1#102-e, August 17-28, 2020.

R1-2007554, “Views on resource allocation enhancements for sidelink communication”, FUTUREWEI, RAN1#103, Oct. 26 – Nov. 13, 2020.

R1-2100486, “Power consumption reduction for sidelink resource allocation”, FUTUREWEI, RAN1#104e, Jan. 25 – Feb. 5, 2021.

R1-2102168, “LS on Mode 2 enhancements in NR sidelink”, LGE, RAN1#104e, Jan. 25-Feb. 5, 2021

R1-2102166, “Detailed observations from evaluation results”, LGE, RAN1#104e, Jan. 25-Feb. 5, 2021

R1-2100047, “Views on resource allocation enhancements for sidelink communication”, FUTUREWEI, RAN1#104e, Jan. 25 – Feb. 5, 2021

R1-2104103, “Feature lead summary for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements”, LGE, RAN1#104b-e, Apr. 12-Apr. 20, 2021

# Appendix

The agreements from RAN1#104b-e are listed below.

Agreement:

- Support the following schemes of inter-UE coordination in Mode 2:

  - Inter-UE Coordination Scheme 1:

    - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

      - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

    - FFS condition(s) in which Scheme 1 is used

  - Inter-UE Coordination Scheme 2:

    - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

      - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

    - FFS condition(s) in which Scheme 2 is used

Agreements:

- Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

  - Details include applicable scenario(s)/inter-UE coordination scheme(s)

  - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

    - Including the possibility of being subject to certain conditions and/or capability

Agreement:

- When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

  - For scheme 1:

    - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

    - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information