3GPP TSG RAN WG1 Meeting #104b-e	R1-2104324

e-Meeting, May 10th – 27th, 2021

Source: 	ZTE

Title: 	Remaining Issues of Single UL Tx

Agenda item:	7.2.10

Document for:	Discussion/Decision

Introduction

In this contribution, one remaining issue for single UL Tx is discussed.

Discussion

In the latest RAN2 CR R2-2006349, it has been agreed to use tdm-PatternConfig2-r16 as the new RRC name for Rel-16 EN-DC single UL Tx. Meanwhile, tdm-PatternConfig2-r16 refers to the same structure as tdm-PatternConfig-r15.

Currently, RRC parameter subframeAssignment-r15 is used in LTE spec for Rel-15 EN-DC/NE-DC single UL Tx. RRC parameter subframeAssignment-r16 is used in LTE spec for Rel-16 EN-DC single UL Tx. However, subframeAssignment-r16 has already been deleted by RAN2 CR R2-2006349. Thus, subframeAssignment-r16 should be updated to tdm-PatternConfig2-r16.

Furthermore, subframeAssignment-r15cannot be used to differentiate Rel-15 and Rel-16 single UL Tx anymore since both Rel-15 and Rel-16 will use subframeAssignment-r15. Thus, subframeAssignment-r15the in LTE spec should be updated to tdm-PatternConfig-r15 and tdm-PatternConfigNE-DC-r15. Note that tdm-PatternConfig-r15 and tdm-PatternConfigNE-DC-r15 are for EN-DC and NE-DC, respectively.

The Draft CR for LTE spec 36.212 and 36.213 are enclosed under the same tdoc number.

Proposal 1:

Approve TP-1 enclosed in R1-2104324 for LTE TS36.212.

Approve TP-2 enclosed in R1-2104324 for LTE TS36.213.

tdm-PatternConfig-r15 TDM-PatternConfig-r15 OPTIONAL, -- Cond FDD-PCell ------------- tdm-PatternConfig2-r16 TDM-PatternConfig-r15 OPTIONAL, -- Need ON ------------- tdm-PatternConfigNE-DC-r15 TDM-PatternConfig-r15 OPTIONAL -- Cond FDD-PSCell tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].. tdm-PatternConfig2 This field is used for dual UL transmission in EN-DC with LTE FDD PCell and for single UL transmission in EN-DC with LTE FDD/TDD PCell, as specified in TS 38.101-3 [101] and TS 38.213 [88]. The network sets at most one of tdm-PatternConfig and tdm-PatternConfig2 to setup. When this field is configured in EN-DC with LTE TDD PCell, it is not applicable if TDD configuration is sa0 or sa6 in SIB1.. tdm-PatternConfigNE-DC This field is used when power control or IMD issues require single UL transmission in NE-DC as specified in TS 38.101-3 [101] and TS 38.213 [88]..

-- ASN1START TDM-PatternConfig-r15 ::= CHOICE { release NULL, setup SEQUENCE { subframeAssignment-r15 SubframeAssignment-r15, harq-Offset-r15 INTEGER (0.. 9) } } SubframeAssignment-r15 ::= ENUMERATED {sa0, sa1, sa2, sa3, sa4, sa5, sa6} -- ASN1STOP

Conclusion

In this contribution, we provide CRs for LTE spec for single UL Tx.

Proposal 1:

Approve TP-1 enclosed in R1-2104324 for LTE TS36.212.

Approve TP-2 enclosed in R1-2104324 for LTE TS36.213.