5	Power control

Downlink power control determines the Energy Per Resource Element (EPRE). The term resource element energy denotes the energy prior to CP insertion. The term resource element energy also denotes the average energy taken over all constellation points for the modulation scheme applied. Uplink power control determines the average power over a SC-FDMA symbol in which the physical channel is transmitted.

5.1	Uplink power control

If the UE is configured with shortTTI, PUCCH in this clause refers to SPUCCH defined in [3] if the HARQ-ACK is sent in response to PDSCH scheduled by DCI format 7-1A/1B/1C/1D/1E/1F/1G or if the scheduling request is sent on resources configured by higher layer parameter sr-SlotSPUCCH-IndexFH or sr-SlotSPUCCH-IndexNoFH or sr-subSlotSPUCCH-Resource for slot/subslot-based transmissions, unless otherwise noted.

If the UE is not configured with shortTTI or the UE is configured with shortTTI, and UCI is to be transmitted in a subframe, the term 'subframe/slot/subslot' or 'subframe/slot' refers to a subframe in this clause.

If the UE is configured with shortTTI, and UCI is to be transmitted in a slot, the term 'subframe/slot/subslot' or 'slot/subslot' or 'subframe/slot' refers to a slot in this clause.

If the UE is configured with shortTTI, and UCI is to be transmitted in a subslot, the term 'subframe/slot/subslot' or 'slot/subslot' refers to a subslot in this clause.

Throughout this clause,

-	if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, , otherwise .

-	if the UE is configured with higher layer parameter shortTTI and the corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G is detected in a subslot, if the UE is configured for subslot uplink transmissions, is given by higher layer parameter proc-TimeAdv from , otherwise.

Uplink power control controls the transmit power of the different uplink physical channels.

If a UE is configured with a LAA SCell for uplink transmissions, the UE shall apply the procedures described for PUSCH and SRS in this clause assuming frame structure type 1 for the LAA SCell unless stated otherwise.

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with tdm-PatternConfig/tdm-PatternConfigNE-DC for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration tdm-PatternConfig/tdm-PatternConfigNE-DC.

For a UE configured with EN-DC with primary cell frame structure type 1, if the UE is configured with tdm-PatternConfig2 for a serving cell, and if the UE indicates a capability fdd-PCellUL-TX-AllUL-Subframe-r16 (as specified in [11]), the UE transmits any uplink physical channel or signal without associated DCI if configured, in the serving cell on any uplink subframes. Otherwise, if the UE is configured with tdm-PatternConfig2 for the serving cell and if the UE does not indicate a capability fdd-PCellUL-TX-AllUL-Subframe-r16, the UE is not expected to transmit any uplink physical channel or signal without associated DCI except for PRACH in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration tdm-PatternConfig2.

For a UE configured with EN-DC with primary cell frame structure type 2, if the UE is configured with tdm-PatternConfig2 for a serving cell, and if the UE indicates a capability tdd-PCellUL-TX-AllUL-Subframe-r16 (as specified in [11]), the UE transmits any uplink physical channel or signal without associated DCI if configured, in the serving cell on any uplink subframes. Otherwise, if the UE is configured with tdm-PatternConfig2 for the serving cell and if the UE does not indicate a capability tdd-PCellUL-TX-AllUL-Subframe-r16, the UE is not expected to transmit any uplink physical channel or signal without associated DCI except for PRACH in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration tdm-PatternConfig2.

For PUSCH, the transmit power  defined in Clause 5.1.1, is first scaled by the ratio of the number of antennas ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme. The resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted.

For PUCCH or SRS, the transmit power , defined in Clause 5.1.1.1, or is split equally across the configured antenna ports for PUCCH or SRS.  is the linear value ofdefined in Clause 5.1.3.

A cell wide overload indicator (OI) and a High Interference Indicator (HII) to control UL interference are defined in [9].

For a serving cell with frame structure type 1, a UE is not expected to be configured with UplinkPowerControlDedicated-v12x0.