3GPP TSG RAN WG1 #105-e                                                R1-2104330

e-Meeting, May 10th – May 27th, 2021

Source: 	ZTE Corporation

Title: 	Discussion on enhanced PUSCH repetition type A

Agenda item:	8.8.1.1

Document for:	Discussion/Decision

Introduction

In the RAN1#104-e meeting [1], the following agreements and conclusion related to enhancements on PUSCH repetition type A were reached.

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

In this contribution, we provide our views on the details for enhancements on PUSCH repetition type A.

Increasing the number of repetitions for PUSCH repetition type A

Maximum number of repetitions

Increasing the number of repetitions is a simple way for coverage enhancement for VoIP service. Currently, NR supports PUSCH repetition type A with maximum repetition number up to 16. Larger number of repetitions could be considered to extend the PUSCH coverage. Based on evaluation results in SI phase, PUSCH with eMBB service is the worst channel in most cases while increasing the number of repetitions cannot help to improve its performance. Thus, a moderate increase could be sufficient for PUSCH with VoIP service, e.g., increasing to 32 repetitions.

Proposal 1: For PUSCH repetition type A, the maximum number of repetitions is extended to 32.

Applicable use cases

As discussed in FL summary [2], the applicable use cases for increasing the maximum number of repetitions are FFS. There are three cases as listed below:

- Case 1: FDD or SUL

- Case 2: TDD with the contiguous-slot-based counting

- Case 3: TDD with the available-slot-based counting

- From NW perspective, we don’t see a need to limit the enhancements to TDD or FDD only, and it’s up to NW whether/how to enable the features that supported by UE. In other words, if UE supports both two enhancements in this agenda, gNB can certainly enable either one or both enhancements. There is no need to limit the maximum number of repetitions differently based on whether it is counted by available slots or not if they are all supported by UE. From UE capability perspective, we don’t see any complexity difference for different bands or a need for TDD/FDD differentiation. So, we think all the three cases listed above are valid.

- Proposal 2: The increased maximum number of repetitions is applicable to all frequency bands.

Additional candidate values for repetition numbers

In Rel-16, 8 candidates for repetition factors are supported. It could be increased to 16 candidates in Rel-17. The detailed values could be further discussed.

numberOfRepetitions-r16 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

Proposal 3: When increasing the number of repetitions for PUSCH repetition type A, the number of candidate repetition factors could be increased from 8 to 16 in Rel-17. FFS the detailed candidate values.

RRC parameters to be extended to support more repetitions

- In RAN1#104-e meeting, the increase of the maximum number of repetitions with repetition factors configured in a TDRA list has been agreed to be supported. But, whether it can be configured in PUSCH-Config and/or ConfiguredGrantConfig is still FFS. We think the enhancement of PUSCH repetition type A should focus on Rel-16 repetition factors, and introducing only one new parameter for numberofrepetitions with Rel-17 suffix included in TDRA table is sufficient. If this new parameter is not configured, then the number of repetitions is determined by legacy rules.

Proposal 4: For Rel-17 PUSCH repetition Type A, increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig is not supported.

The number of repetitions counted on the basis of available UL slots

Definition of available slots for PUSCH repetitions

Based on the agreement in RAN1 #104-e, in terms of how to define a slot is available for UL transmissions, there are two alternatives listed as shown below :

- Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

- Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

- According to the above listed, semi-static slot configuration, RRC configuration, dynamic SFI, PUSCH priority for URLLC and UL cancellation could impact on PUSCH transmission. In the following, we provide our views on these aspects for PUSCH repetitions.

    - PUCCH overlapping

In Clause 9 of TS 38.213, a UE does not transmit a PUSCH if it would overlap with PUCCH in some cases, e.g.,

- The A-CSI PUSCH is without UL-SCH, and it overlaps with a PUCCH transmission on a serving cell that includes positive SR information.

- The PUSCH is low priority, and it overlaps with PUCCH with high priority.

- The PUSCH overlaps in one or more slots with PUCCH repetition.

- In above case of overlapping PUCCH, there would be no ambiguity on the number of repetitions between gNB and UE. Thus, the dropped PUSCH due to overlapping with PUCCH should not be counted in the total number of repetitions.

- Proposal 5: Whether or not a slot is determined as available for UL transmissions depends on PUCCH overlapping.

- If a PUSCH repetition is dropped due to overlapping with PUCCH, it will not be counted as one repetition in the  total number of repetitions for PUSCH repetition.

    - Semi-static configurations without SFI configured

- In Clause 11.1 of TS 38.213, a UE does not transmit a PUSCH repetition if it conflicts with semi-static DL symbols or SSB symbols summarized as follows:

For DG PUSCH with repetition, if dynamic SFI is not configured, it conflicts with a semi-static DL symbol: - If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot. - For PUSCH with or without repetition, it conflicts with SS/PBCH blocks: For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. For CG PUSCH with and without repetition, if dynamic SFI is not configured, it conflicts with a semi-static DL symbol: For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.

- When SFI is not configured, in case of conflicting with semi-static DL symbols or SSB symbols, there would be no ambiguity on the number of repetitions between gNB and UE. Thus, it is proposed that the dropped PUSCH due to such collision should not be counted in the total number of repetitions.

- Proposal 6: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations including semi-static DL symbols and SSB symbols.

- If a PUSCH repetition is dropped due to overlapping with semi-static DL symbols or SSB symbols, it will not be counted as one repetition in the total number of repetitions for PUSCH repetition.

    - Dynamic SFI

In Rel-15, even dynamic SFI is configured, there would be no ambiguity on the number of repetitions between gNB and UE. Because, for DG PUSCH, it can be transmitted on UL symbols or semi-static flexible symbols, and no conflict between dynamic grant and SFI is expected in Rel-15. For CG PUSCH, the UE will not transmit on flexible symbols, and therefore there is no conflicts with SFI. The detailed UE behavior is summarized as follows.

- DG PUSCH

For DG PUSCH, if dynamic SFI is configured and received, it does not expect to conflict with dynamic DL symbols: For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot For DG PUSCH, if dynamic SFI is configured and not received, it conflicts with semi-static DL symbols (it can be transmitted on semi-static flexible symbols ) For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR. ...

- CG PUSCH

For CG PUSCH, if dynamic SFI is configured and received, it conflicts with a dynamic DL/flexible symbol: If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 or the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6. TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols. For CG PUSCH, if dynamic SFI is configured and not received, it conflicts with semi-static flexible symbols: For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided enableConfiguredUL, the UE - does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is after PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0

Based on the above listed, Rel-15 solutions for the collision with dynamic SFI transmission for DG PUSCH or CG PUSCH can be reused. As a result, we have the following proposal.

Proposal 7: When a SFI is configured, whether or not a slot is determined as available for UL transmissions depends on dynamic SFI:

- For a DG PUSCH repetition, if it is dropped due to overlapping with semi-static DL symbols, it will not be counted as one repetition in the total number of repetitions for DG PUSCH repetition.

- For a CG PUSCH repetition, if it is dropped due to overlapping with dynamic DL/flexible symbols or semi-static flexible symbols, it will not be counted as one repetition in the total number of repetitions for CG PUSCH repetition.

    - UL cancellation

- In Clause 11.2A of TS 38.213, if a UE detects a DCI format 2_4 for a serving cell to cancel a PUSCH transmission, the UE will cancel the PUSCH. There would be no ambiguity on the number of repetitions between gNB and UE.

- An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=doffset⋅2-μUL2-μ where doffset is provided by delta_Offset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

Proposal 8: Whether or not a slot is determined as available for UL transmissions depends on UL cancellation.

- If a PUSCH repetition is dropped due to receiving UL cancellation, it will not be counted as one repetition in the  total number of repetitions for PUSCH repetition.

Details of the determination of available slots

Based on the conclusion in RAN1 #104-e, whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetition should be further analyzed. There are two alternative should be considered.

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

As analyzed in section 2.2.1, whether or not a slot is determined as available for UL transmissions depends on semi-static slot configuration, RRC configuration, dynamic SFI, PUSCH priority for URLLC and UL cancellation. There is no ambiguity between gNB and UE on the understanding of which slot is an available slot. So, it seems unnecessary to limit all the available slots to be prior to the first actual transmission. Similar as UCI multiplexing on PUSCH repetition, the multiplexing timeline is per repetition basis. In our view, Alt-b is more preferred.

Proposal 9: Whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetition, Alt-b is supported.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Proposal 1: For PUSCH repetition type A, the maximum number of repetitions is extended to 32.

Proposal 2: The increased maximum number of repetitions is applicable to all frequency bands.

Proposal 3:When increasing the number of repetitions for PUSCH repetition type A, the number of candidate repetition factors could be increased from 8 to 16 in Rel-17. FFS the detailed candidate values.

Proposal 4: For Rel-17 PUSCH repetition Type A, increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig is not supported.

- Proposal 5: Whether or not a slot is determined as available for UL transmissions depends on PUCCH overlapping.

- If a PUSCH repetition is dropped due to overlapping with PUCCH, it will not be counted as one repetition in the  total number of repetitions for PUSCH repetition.

- Proposal 6: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations including semi-static DL symbols and SSB symbols.

- If a PUSCH repetition is dropped due to overlapping with semi-static DL symbols or SSB symbols, it will not be counted as one repetition in the total number of repetitions for PUSCH repetition.

Proposal 7: When a SFI is configured, whether or not a slot is determined as available for UL transmissions depends on dynamic SFI:

- For a DG PUSCH repetition, if it is dropped due to overlapping with semi-static DL symbols, it will not be counted as one repetition in the total number of repetitions for DG PUSCH repetition.

- For a CG PUSCH repetition, if it is dropped due to overlapping with dynamic DL/flexible symbols or semi-static flexible symbols, it will not be counted as one repetition in the total number of repetitions for CG PUSCH repetition.

Proposal 8: Whether or not a slot is determined as available for UL transmissions depends on UL cancellation.

- If a PUSCH repetition is dropped due to receiving UL cancellation, it will not be counted as one repetition in the  total number of repetitions for PUSCH repetition.

Proposal 9: Whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetition, Alt-b is supported.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Reference

3GPP RAN1#104-e, Chairman notes.

3GPP RAN1#104-e, R1-2102113, FL Summary on Enhancements on PUSCH repetition type A, Moderate (Sharp).