3GPP TSG RAN WG1 #105-e                                                 R1-2104443

e-Meeting, May 19th – May 27th, 2021

Source: 	ZTE Corporation

Title: 	Discussion on PUCCH coverage enhancement

Agenda item:	8.8.2

Document for:	Discussion/Decision

Introduction

According to the revised new WID on NR coverage enhancements was approved [1], one objective of the WID is to specify enhancements for PUCCH for both FR1 and FR2 as well as TDD and FDD. The detail objectives for PUCCH enhancements are as follows:

- Specification of PUCCH enhancements [RAN1, RAN4] - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1] - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4] - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

In this contribution, we provide our views on corresponding enhancements for PUCCH.

Dynamic PUCCH repetition factor indication

In RAN1 #104-e meeting, the following were agreed for dynamic indication of PUCCH repetition.

Agreements: Down select from the following two options to support dynamic PUCCH repetition factor indication. - Option 1 (without DCI enhancement): Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI. - FFS details, e.g., via reusing the “PUCCH resource indicator” field (without increase # bits of it), starting CCE index (when applicable) of DCI, by PDCCH aggregation level, etc. - FFS: RRC signaling enhancement details - Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI - e.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication - FFS whether there is a need for RRC update

In Rel-15/16, the repetition factor for PUCCH is RRC configured by nrofSlots in PUCCH-FormatConfig as shown below. It is applied to all PUCCH resources with the same PUCCH format. For instance, if the PUCCH repetition factor is configured by nrofSlots as 2 for PUCCH format 1, all PUCCH format 1 resources are of the same repetition number.

PUCCH-FormatConfig ::=                  SEQUENCE {

interslotFrequencyHopping               ENUMERATED {enabled}         OPTIONAL, -- Need R

additionalDMRS                          ENUMERATED {true}            OPTIONAL, -- Need R

maxCodeRate                             PUCCH-MaxCodeRate            OPTIONAL, -- Need R

nrofSlots                               ENUMERATED {n2,n4,n8}        OPTIONAL, -- Need S

pi2BPSK                                 ENUMERATED {enabled}         OPTIONAL, -- Need R

simultaneousHARQ-ACK-CSI                ENUMERATED {true}            OPTIONAL  -- Need R

}

Observation 1: In Rel-15/Rel-16, PUCCH repetition factor is semi-statically configured per PUCCH format.

- For dynamic PUCCH repetition factor indication, a new PUCCH repetition signaling mechanism needs to be specified.  A most straightforward way is to introduce a PUCCH repetition factor field in DL scheduling DCI. The number of bit field will depend on the number of repetition factor to be supported. But it will result in larger DCI size, which would result in degraded PDCCH performance.

- Another alternative is to configure the PUCCH repetition factor per PUCCH resource instead of per PUCCH format. More specifically, it can add a new IE indicating the PUCCH repetition factor to each PUCCH resource. For different PUCCH resources, different PUCCH repetition factors can be configured. In this way, dynamic PUCCH repetition factor can be supported by indicating the PUCCH resource through PRI.

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                       PUCCH-ResourceId,

startingPRB                             PRB-Id,

intraSlotFrequencyHopping            ENUMERATED { enabled }       OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                       OPTIONAL, -- Need R

repetitionNum                          ENUMERATED {n1,n2,n4,n8}        OPTIONAL, -- Need S

format                                   CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

Observation 2: Dynamic PUCCH repetition factor can be achieved by configuring the repetition factor per PUCCH resource which can be dynamically indicated by PRI.

In summary, from our point of view, the most promising solution is that dynamic PUCCH repetition factor is supported by indicating the PUCCH resource through PRI with configuring the repetition factor per PUCCH resource. In order to have a fair comparison with Option 2, it’s better not to preclude the possibility of enhancing the DCI (e.g., enlarging the PRI bit field) for Option 1 now.

Proposal 1: Dynamic PUCCH repetition factor is supported by configuring the repetition factor per PUCCH resource and indicating the PUCCH resource dynamically through PRI.

- FFS whether to enhance the PRI bit field.

DMRS bundling

Use case

In the LS sent from RAN1 to RAN4 in[2], there are the following use cases for joint channel estimation for PUCCH repetitions.

For PUCCH repetitions, the following use cases are considered in RAN1. Among the following cases, RAN1 suggest RAN4 to prioritize the study on use case 3, 4a, 4b, and 5b for PUCCH repetitions.  Use case 1: back-to-back PUCCH repetitions within one slot.  Use case 2: non-back-to-back PUCCH repetitions within one slot. ‐ Use case 2a: no uplink transmission in the middle of two PUCCH repetitions ‐ Use case 2b: other uplink transmissions in the middle of two PUCCH repetitions  Use case 3: back-to-back PUCCH repetitions across consecutive slots.  Use case 4: non-back-to-back PUCCH repetitions across consecutive slots. ‐ Use 4a: no uplink transmission in the middle of two PUCCH repetitions ‐ Use 4b: other uplink transmissions in the middle of two PUCCH repetitions  Use case 5: PUCCH repetitions across non-consecutive slots. ‐ Use case 5a: no uplink transmission in the middle of two PUCCH repetitions ‐ Use case 5b: other uplink transmissions in the middle of two PUCCH repetitions Note: RAN1 assumes “back-to-back PUCCH repetitions” has zero gap in-between adjacent PUCCH repetitions. Note: intervening “other uplink transmissions” can be either on the same component carrier or a different component carrier.

Regarding Use case 1, short PUCCH repetition within one slot has been supported in Rel-17 URLLC WI. From the perspective of the requirements to keep phase continuity, there is no difference compared to Use case 3 according to  RAN4 reply LS in [3]. Thus, Use case 1 can be supported. In addition of PUCCH format 2, we want to highlight that Use case 1 should also include PUCCH format 0. Though no DMRS is used for PUCCH format 0, joint sequence detection/decoding can be applied among multiple PUCCH repetitions for better performance. This is similar as joint channel estimation operation to improve the performance.

Regarding Use case 3, it should be supported as similar use case has been supported for joint channel estimation of PUSCH transmissions.

Proposal 2: Support Use case 1 and Use case 3 for joint channel estimation or joint detection of PUCCH repetitions.

- Clarify that Use case 1 includes both PUCCH format 0 and PUCCH format 2.

As for use cases with no back-to-back PUCCH repetitions (Use case 2/4/5), RAN4 has replied further information in [4], as also copied below.

RAN4 confirms the feasibility of phase continuity and power consistency for non-zero un-scheduled gap case for a gap less than 14 symbols when UE is not required to meet the existing off power requirements. Whether new or existing off power requirements for shorter duration than 1 msec as well as the maximum value of X un-scheduled symbols will be introduced are pending on further RAN4 discussions. For the case with other UL channels in between repetitions, at least if the other scheduled signals/channels during the non-zero gap have the same settings in antenna port, occupied PRBs and UL power than the repeated transmission signals/channels, it is feasible to maintain the phase continuity and power consistency across the repetitions.

Based on above RAN4 reply, RAN4 only confirms the feasibility of phase continuity and power consistency for non-zero un-scheduled gap case for a gap less than 14 symbols when UE is not required to meet the existing off power requirements. However, whether or how to define new off power requirements are still not decided yet. In other words, if no new off power requirements are defined, the phase continuity cannot be kept. So, whether Use case 2a/4a/5a should be supported in RAN1 depends on RAN4 further decision. As for Use case 2b/4b/5b, it is very difficult to satisfy the requirements defined in RAN4 for the other signals transmitted in the middle of two PUCCH repetitions from RAN1 perspective. Thus, we prefer not to support Use case 2b/4b/5b in RAN1.

Proposal 3: Decide whether to support Use case 2a/4a/5a for PUCCH repetitions depending on RAN4 further discussion.

Proposal 4: Do not support joint channel estimation for Use case 2b/4b/5b for PUCCH repetitions.

Time domain window

In RAN plenary #91-e meeting, it was concluded that joint channel estimation for PUCCH should be based on similar mechanism(s) for enabling joint channel estimation for PUSCH when applicable. According to the requirements defined in RAN4, we don’t identify any difference to keep phase continuity between PUCCH and PUSCH transmissions. Therefore, the mechanisms defined for PUSCH can be reused for PUCCH in general. As discussed in our companion contribution in [5], a time domain window should be specified for joint channel estimation PUSCH. In our view, the time domain window size should be reported per UE level and applies to both PUSCH and PUCCH. gNB can configure a time domain window based on UE capability reporting, and the configured time domain window size should be also the same between PUSCH and PUCCH. However, how to define the start of time domain window may or may not be the same for PUSCH and PUCCH. For instance, if the time domain window is fixed in certain time domain occasions based on the frame boundary, the start of time domain window can be also the same for both PUSCH and PUCCH. Otherwise, it could be different if start of the time domain window is related to the scheduling DCI.

Proposal 5: Specify a time domain window for PUCCH repetition.

- A UE reports a same time domain window size for PUSCH and PUCCH.

- gNB configures a same time domain window for PUSCH and PUCCH.

- FFS the start of time domain window for PUCCH.

Inter-slot frequency hopping with inter-slot DMRS bundling

Regarding inter-slot frequency hopping with inter-slot DMRS bundling, the following agreements and conclusion were reached in RAN1#104-e. In this section, the performance evaluation and some preliminary analysis for inter-slot frequency hopping with inter-slot DMRS bundling are provided.

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling. - FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16. - Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible Conclusion: For the study of enhancing inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, at least the following aspects can be considered: - Performance tradeoff between maximizing # consecutive UL slots in one frequency hop (to achieve more DMRS bundling gain) and maximizing # hops (to achieve more diversity gain) - Note: the maximum # frequency hopping positions is still 2 as in Rel-15/16., which is signaled by startingPRB and secondHopPRB - Interaction between hopping boundary determination and TDD configuration Conclusion: For the simulations to study the enhancement of inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, simulation assumptions in 38.830 are reused as a starting point. Note: Additional simulation scenarios/assumptions are not precluded. Conclusion: In Rel-17, deprioritize the study of DMRS pattern/location/granularity optimization for PUCCH coverage enhancement in AI 8.8.2. This conclusion could be revisited after the progress on the study of DMRS pattern/location/granularity optimization for PUSCH coverage enhancement in AI 8.8.1.3.

In Figure 1, three DMRS bundling patterns are provided for PUCCH format 3 with 4 repetitions. For Cases-1, two sub-cases are evaluated with or without enabling cross-slot joint channel estimation. The detailed simulation assumptions are provided in Table A-1 in the appendix.

- Case 1: No frequency hopping is enabled.

- Case 1-1: No cross-slot joint channel estimation. (Baseline-1)

- Case 1-2: Cross-slot joint channel estimation is enabled. (Enhancement-1)

- Case 2: Legacy inter-slot frequency hopping is enabled. (Baseline-2)

- Case 3: Inter-slot frequency hopping with inter-slot bundling, bundling size is 2.

- Case 1: No frequency hopping for 4 PUCCH repetitions

- Case 2: Legacy inter-slot frequency hopping for 4 PUCCH repetitions

- Case 3: Inter-slot frequency hopping with inter-slot bundling

- Figure 1. Three DMRS patterns for PUCCH repetitions

- Figure 2. Simulation results for different DMRS patterns with 4 repetitions.

According to the results in Figure 2, we can find the following observations.

- For DMRS bundling only (Case 1-2) vs inter-slot FH only (Case 2):

- In low SNR region, DMRS bundling only has better performance than inter-slot FH only.

- In high SNR region, inter-slot FH only is expected to have better performance than DMRS bundling only.

- For the target BLER@0.01, similar performance is achieved for both cases.

- DMRS bundling + inter-slot FH (Case 3) vs inter-slot FH only (Case 2) or DMRS bundling only (Case 1-2)

- Around 1dB performance gain of DMRS bundling + inter-slot FH is observed compared to inter-slot FH only or DMRS bundling only at target BLER@0.01.

Observation 3: For PUCCH with four repetitions, enabling both inter-slot frequency hopping and DMRS bundling can provide 1 dB gain compared with inter-slot frequency hopping only or DMRS bundling only.

Therefore, inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation among repetitions per bundle should be supported.

Proposal 6: Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation among repetitions per bundle is supported.

In our companion contribution [5], some other aspects for DMRS bundling for PUSCH are discussed, including how to determine the inter-slot bundling size, how to determine the inter-slot bundling pattern for TDD operation, and how to support optimization of DMRS granularity in the time domain. Given it has agreed that RAN1 shall strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible, we prefer to further discuss the details for PUCCH after a better progress made for PUSCH.

Proposal 7: RAN1 further discusses the details for PUCCH after a better progress made for PUSCH regarding the following aspects.

- Determination of the inter-slot bundling size.

- Determination of the inter-slot bundling pattern for TDD operation.

- Support optimization of DMRS granularity in the time domain.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: In Rel-15/Rel-16, PUCCH repetition factor is semi-statically configured per PUCCH format.

Observation 2: Dynamic PUCCH repetition factor can be achieved by configuring the repetition factor per PUCCH resource which can be dynamically indicated by PRI.

Proposal 1: Dynamic PUCCH repetition factor is supported by configuring the repetition factor per PUCCH resource and indicating the PUCCH resource dynamically through PRI.

- FFS whether to enhance the PRI bit field.

Proposal 2: Support Use case 1 and Use case 3 for joint channel estimation or joint detection of PUCCH repetitions.

- Clarify that Use case 1 includes both PUCCH format 0 and PUCCH format 2.

Proposal 3: Decide whether to support Use case 2a/4a/5a for PUCCH repetitions depending on RAN4 further discussion.

Proposal 4: Do not support joint channel estimation for Use case 2b/4b/5b for PUCCH repetitions.

Proposal 5: Specify a time domain window for PUCCH repetition.

- A UE reports a same time domain window size for PUSCH and PUCCH.

- gNB configures a same time domain window for PUSCH and PUCCH.

- FFS the start of time domain window for PUCCH.

Observation 3: For PUCCH with four repetitions, enabling both inter-slot frequency hopping and DMRS bundling can provide 1 dB gain compared with inter-slot frequency hopping only or DMRS bundling only.

Proposal 6: Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation among repetitions per bundle is supported.

Proposal 7: RAN1 further discusses the details for PUCCH after a better progress made for PUSCH regarding the following aspects.

- Determination of the inter-slot bundling size.

- Determination of the inter-slot bundling pattern for TDD operation.

- Support optimization of DMRS granularity in the time domain.

Reference

3GPP RAN#91-e, RP-210855, Revised WID on NR coverage enhancements, China Telecom.

3GPP RAN1#104b-e, R1-2104119, Reply LS on PUCCH and PUSCH repetition, RAN1.

3GPP RAN1#104b-e, R1-2102298, Reply on LS on PUCCH and PUSCH repetition, RAN4.

3GPP RAN1#105-e, R1-2104168, Reply LS on PUCCH and PUSCH repetition, RAN4, Qualcomm.

3GPP RAN1#105-e, R1-2104332, “Discussion on joint channel estimation for PUSCH”, ZTE.

Appendix

- Table A-1 Simulation assumption for DMRS bundling for PUCCH

| - Parameters | - Value |
| --- | --- |
| - Carrier frequency | - 4 GHz |
| - Channel | - TDL-C , DS = 300ns |
| - UE speed | - 3 km/h |
| - Antenna configuration | - 1Tx, 4Rx |
| - Number of symbols | - 14 |
| - Number of RBs | - 1 |
| - Number of repetition | - 4 |
| - DMRS location | - The 3rd ,10th symbol |
| - Payload | - 22 bits |
| - Metric | - 1% BLER |
| - Receiver | Receiver for Rel-15/16 PUCCH: ML coherent receiver, w/o cross-slot channel estimation Receiver for PUCCH enhancement scheme: ML coherent receiver, w/ cross-slot channel estimation |