3GPP TSG RAN WG1 #105-e	R1-2104340

e-Meeting, May 10th – 27th, 2021

Source: 	ZTE

Title: 	Discussion on Cross-Carrier Scheduling from SCell to PCell

Agenda item:	8.13.1

Document for:	Discussion/Decision

Introduction

During RAN#86 meeting, a new work item on DSS (dynamic spectrum sharing) was approved for Rel-17 [1]. One of the objectives of this WI is to support PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell. In RAN1 #103-e, 104-e and 104b-e meeting, some progress were made for SCell scheduling PCell [2, 3, 4].

In this contribution, we provide our analysis on how to support PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

General issue

Configuration of SCell scheduling PCell

With current NR spec listed below, P(S)Cell cannot be scheduled by SCell as schedulingCellInfo can only be configured as own in the CrossCarrierSchedulingConfig for P(S)Cell. Furthermore, if cif-Presence is set to true, the default CIF value for indicating a grant or assignment for this P(S)Cell is 0. When schedulingCellInfo for a SCell is configured as other in the CrossCarrierSchedulingConfig, that means this SCell is a scheduled cell and also it will be configured with cif-InSchedulingCell and schedulingCellId, wherein the candidate value for cif-InSchedulingCell is [1,...,7].

CrossCarrierSchedulingConfig ::= SEQUENCE { schedulingCellInfo CHOICE { own SEQUENCE { -- Cross carrier scheduling: scheduling cell cif-Presence BOOLEAN }, other SEQUENCE { -- Cross carrier scheduling: scheduled cell schedulingCellId ServCellIndex, cif-InSchedulingCell INTEGER (1..7) } }, ... }

In case that PDCCH on sSCell can be used to schedule PDSCH or PUSCH on P(S)Cell, P(S)Cell should be additional configured as a scheduled cell of the sSCell. CIF for PCell scheduled by sSCell is explicitly configured according to the agreements reached in RAN1 #104b-e meeting as listed below.

Agreement - When CCS from sSCell to PCell/PSCell is configured - CIF=0 used for sSCell self-scheduling, and CIF for sSCell to PCell cross-carrier scheduling is explicitly configured using RRC signalling

With current structure of schedulingCellInfo, only own or other can be configured for a serving cell and only own can be configured for P(S)Cell. As a result, schedulingCellInfo for P(S)Cell should be updated and configured as a scheduling cell and scheduled cell simultaneously. Two options can be considered. One is adding the third choice of “own and other”, and cif-Presence, schedulingCellId and cif-InSchedulingCell are all available for this new choice, and CIF=0 for PCell self-scheduling by set TRUE for cif-Presence and CIF for PCell scheduled by sSCell is configured by cif-InSchedulingCell. The other is reusing the choice of other with re-interpretation of this configuration that PCell self-scheduling is also supported with CIF=0 for PCell self-scheduling. The details of configuration can be determined in RAN2.

Proposal 1: For SCell scheduling PCell, PCell should be configured as the scheduling cell and the scheduled cell simultaneously by RRC parameter CrossCarrierSchedulingConfig.

- CIF=0 used for PCell self-scheduling

USS configuration of SCell scheduling PCell

In Rel-15/Rel-16, the search space ID is applied to associate the search space in scheduling cell and search space in scheduled cell. And for a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates in the IE SearchSpace, all the optional fields are absent (regardless of their presence conditions). In other words, in case that Cell #1 cross-carrier schedules Cell #2 with search space #n (SS #n), UE monitors the corresponding PDCCH on Cell #1 based on the MO configurations for SS #n of Cell #1 and acquires the candidate number based on SS #n of Cell #2.

For SCell scheduling PCell, the same approach can be reused. For USS configuration on PCell, at least two options can be considered.

      - Option 1. A USS set configured on PCell can only be used for either self-scheduling or CCS from SCell.

      - Option 2. A USS set configured on PCell can be used for both self-scheduling and CCS from SCell.

For Option 1, if a USS set on PCell is only configured with searchSpaceId and nrofCandidates, this USS set can only be used for CCS from sSCell. If a USS set on PCell is configured with other optional fields besides nrofCandidates, this USS set can only be used for self-scheduling from PCell.

For Option 2, a USS set configured with other optional fields besides nrofCandidates can be used for both self-scheduling and CCS from SCell. Then the description of the IE SearchSpace for CCS from SCell should be updated such that except for nrofCandidates in the IE SearchSpace, all the optional fields are ignored (regardless of their presence conditions).

Option 1 may need more search space configuration compared with Option 2. However, since up to 10 USS sets can be configured in a BWP, the number of search space sets are sufficient. It is preferred to apply the straightforward option, i.e., Option1.

Proposal 2: For SCell scheduling PCell and for USS configuration, search space ID is used to associate the search space configuration for PCell and search space configuration for SCell.

- A USS set configured on PCell can only be used for either self-scheduling or CCS from SCell.

- If the USS #n on PCell is used for cross-carrier scheduling from SCell, UE acquires the MO configuration from USS #n on SCell and acquire the candidate number from USS #n on PCell, which is the same as in Rel-15/Rel-16.

Another issue of extending the SS configuration on the sSCell for scheduling PCell was proposed in RAN1 #104b-e meeting as listed below.

- When CCS from sSCell to Pcell/PSCell is configured - UE is configured with search space set Ssa on P(S)Cell with search space ID = X1 - If UE is configured with a search space set SSb on sSCell with same search space ID = X1 (i.e., ‘linked SS sets’ approach as in Rel16) - DCI formats monitored by UE on SSb with n_CI corresponding to CIF P(S)Cell are used for P(S)Cell PUSCH/PDSCH scheduling - For monitoring the DCI formats on SSb used for P(S)Cell PUSCH/PDSCH scheduling - Alt 1 - nrofCandidates is derived from Ssa configuration - monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration are derived from SSb configuration and same parameters are also used for self-scheduling on sSCell (i.e., same approach as in Rel16) - Alt 2 - nrofCandidates is derived from Ssa configuration - monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration are configured independently from the parameters used for self-scheduling on sSCell - FFS Details

Based on our understanding, Alt.1 is the legacy mechanism, which is preferred by us. Alt.2 seems to offer more flexibility which had been discussed during Rel-15/Rel-16 for legacy cross-carrier scheduling, we can also accept Alt.2 if this is the majority view. But we may also need to clarify whether Alt.2 is only for sSCell scheduling PCell, or can also be applied to legacy cross-carrier scheduling, e.g., sSCell scheduling other SCell.

BD/CCE limit handling

In the scope of Rel-17 DSS, there is a restriction that the total PDCCH blind decoding budget should not be changed as a result of this work. There are two PDCCH blind decoding budget, one is  or , the other is  or . For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

In Rel-16 CA, there is only one scheduling cell for a scheduled cell. And the BD/CCE limit on the scheduling cell for the scheduled cell is determined based on the numerology of the scheduling Cell. In Rel-17 DSS with support of SCell scheduling PCell, there are two scheduling cells (PCell and sSCell) for a scheduled cell (PCell), how to handle the BD/CCE limit is analyzed below.

Some guidance were proposed in RAN1 #104b-e meeting as listed below.

For RAN1#105-e, companies are encouraged to consider: - Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options - Option A - At least when P(S)Cell SCS is not higher than sSCell SCS, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1 - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16) - FFS signalling details on how the limit Z1 is realized, e.g. - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m)) - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - separate BdfactorR for P(S)Cell and sSCell - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m)) - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))+ max(y(m)) - Counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured - Option B - At least when P(S)Cell SCS is not higher than sSCell SCS, For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2 - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16) - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2 - FFS signalling details on how the limit Z2 is realized - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured - Option C - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3 - Z3 is derived by the PDCCH monitoring capability of PCell - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4 - Z4 is derived by the PDCCH monitoring capability of sSCell - FFS details to define Z3 and Z4, e.g. - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than Z4 PDCCH candidates per slot of sSCell - Note - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

M_max/C_max

As shown in Figure 1 for the scheduled cell (PCell), there are M1 candidates on the first scheduling cell (PCell) with self-scheduling and M2 candidates on the second scheduling cell (sSCell) with CCS. Since the SCS on the two scheduling cells are same and equal to 15kHz, the M_max of the scheduled cell (PCell) should be same as =44 with μ=0 and (M1 + M2) for monitoring should not be larger than M_max. Note, same mechanism can be used for C_max.

Figure 1 SCell-schedule-PCell with same SCS

In case the SCS of two scheduling cells are different, as shown in Figure 2, there are several options as listed below to determine the M_max of the scheduled cell (PCell) with two scheduling cells (PCell and sSCell). Assume that the SCS of the first scheduling cell (PCell) is μ1 and the SCS of the second scheduling cell (sSCell) is μ2. The same mechanism can be used for determination of C_max.

Figure 2 SCell-schedule-PCell with different SCS

Take left schematic in Figure 2 to compare Option A/B/C above in Table 1. With same total BD limits, Option B has more BD opportunities due to shared BD between two scheduling cells per PCell slot, while Option A and C are similar but Option A is more flexible than option C due to shared BD between two consecutive slots overlapped with one PCell slot.

Table 1 Examples of Option A/B/C

| Option A | Option B | Option C |
| --- | --- | --- |
| Z1=44 per slot with μ1=0 | Z2=44 per slot with μ1=0 | Z3=22 per slot with μ1=0 and Z4=11 per slot with μ2=1 |

There are two separate BD limits of each scheduling cell for the same scheduled cell for Option A/C, while one total BD limit for Option B. For Option B, the M_max of the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) can be determined by the following options. Note, the M_max determination can be also used for Option A.

- Option 1: Determine the M_max by the SCS of the first scheduling cell (PCell), that is M_max = ;

- Option 2: Determine the M_max by the SCS of the second scheduling cell (sSCell), that is M_max = ;

- Option 3: Determine the M_max by the maximum SCS of the two scheduling cells, that is M_max = ;

- Option 4: Determine the M_max by the minimum SCS of the two scheduling cells, that is M_max = ;

- Option 5: Determine the M_max by the SCS of the two scheduling cells with scaling factor for each scheduling cell, that is M_max =  +  per slot with SCS = μ1;

Comparing the above options, Option 1 – Option 4 can be classified as only using one of the scheduling cells to determine the M_max of the scheduled cell (PCell). Option 5 can be classified as using both of the scheduling cells to determine the M_max of the scheduled cell (PCell). For all the five options, the SCS of the scheduling cell(s) is/are taken into account for determining the M_max of the scheduled cell (PCell) which is aligned with the intention in Rel-16 CA. While different options will lead to different value of M_max of the scheduled cell (PCell). Option 3 is slightly preferred with more candidates than other options which is beneficial in terms of flexible scheduling for network implementation. Considering the issue on PDCCH overbooking/dropping discussed in section 2.4, Option 5 can be another preferred choice.

For Option A/C, the maximum candidate number (M1_max) on the first scheduling cell (PCell) for the scheduled cell (PCell) and the maximum candidate number (M2_max) on the second scheduling cell (sSCell) for the scheduled cell (PCell) can be predefined or configured by high layer signaling. For example, M1_max = , M2_max =  for Option C or  for Option A, wherein, P1+P2 = 1. Then M1 candidates for PCell self-scheduling should not be larger than M1_max and M2 candidates for SCell scheduling PCell should not be larger than M2_max. The difference between Option A and Option C is that define BD limit for sSCell-schedule-PCell per two sSCell slots overlapped with one PCell slot or only per sSCell slot. Option C is preferred because it is similar as normal BD limit definition and can be used for the SCS of scheduling cell is larger or smaller than the SCS of the scheduled cell.

Proposal 3: For SCell scheduling PCell, Option B is our first preferred option and Option C is our second preferred option compared with Option A.

- M_max/C_max can be determined by the maximum SCS of PCell and sSCell, or be determined by the SCSs of the two scheduling cells with scaling factor for each scheduling cell.

- M1_max/C1_max and M2_max/C2_max for PCell self-scheduling and sSCell scheduling PCell, can be predefined/configured respectively.

M_total/C_total

If a UE is configured with  downlink cells (Note, assume without M-TRP, that is = and =0, and also assume without span based monitoring) with DL BWPs of the scheduling cells using SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

In case that PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is supported, PDCCH blind decoding will be performed on two cells for this P(S)Cell. In this case, how to calculate the number of P(S)Cell in parameter  is a new issue. For example, assume 6 cells are configured for a UE shown in Figure 3, wherein the SCS = 15kHz for PCell #1 and the SCS = 30kHz for sSCell #2. In Rel-15/Rel-16 case PCell cannot be scheduled by sSCell (the red arrow in Figure 3 is not presented), M_total_15khz = floor(4*44*1/6) = 29; M_total_30khz = floor(4*36*5/6) = 120. In case PCell can be scheduled by sSCell (the red arrow in Figure 3 is presented), whether PCell is counted as one or two cells for the two scheduling cells are analyzed below.

Figure 3 CA with SCell-schedule-PCell

- Case 1: Counted as one cell for the two scheduling cells

      - Option 1: The number of PCell is still count as 1 for only one of the scheduling cells, i.e., PCell. This can be used combined with Option B.

      - Option 2: The number of PCell is counted as N1 < 1 and N2 < 1 for the two scheduling cells (PCell and sSCell) respectively, wherein, N1 + N2 = 1. For example, N1=N2=0.5, M_total_15khz = floor(4*44*0.5/6) = 14;  M_total_30khz = floor(4*36*5.5/6) = 132. This can be used combined with Option A/C.

Option 2 seems more accurate with consideration on both scheduling cells, while option 1 is simple for BD/CCE splitting among numerologies. Note, if the SCS of the two scheduling cells (PCell and sSCell) are same, then the result of M_total based on option 1 and option 2 are same, regardless the specific value of N1 and N2.

- Case 2: Counted as two cells for the two scheduling cells

      - The number of PCell is counted as 1 for the first scheduling cell (PCell) and 1 for the second scheduling cell (sSCell) respectively. Then, M_total_15khz = floor(4*44*1/7) = 25; M_total_30khz = floor(4*36*6/7) = 123.

Comparing the two cases above in Table 2, the common part of the two cases is M_total_15khz is reduced and M_total_30khz is increased compared with the Rel-15/Rel-16 case. The difference of two cases is that the total blind detection capability for the PCell may be increased and it may occupy the blind detection capability of other SCells in case 2, otherwise the blind detection capability of the PCell scheduled by sSCell is very limited if keep the blind detection capability for other SCells unchanged. While only partial blind detection capability of the PCell is transferred to the sSCell in case 1 and the blind detection capability for other SCells is not impacted. Since there is a restriction that the total PDCCH blind decoding budget should not be changed as a result of this work, it is preferred to count PCell as 1 cell for the two scheduling cells.

Table 2 blind detection capability for different cases

| Cases | M_total | PCell self-scheduling | PCell scheduled by sSCell with equal division with other SCells scheduled by sSCell {BD for PCell, BD for SCells} | PCell scheduled by sSCell with other SCells scheduled by sSCell non-impacted {BD for PCell, BD for SCells} |
| --- | --- | --- | --- | --- |
| Rel-15/Rel-16 case | M_total_15khz = floor(4*44*1/6)=29; M_total_30khz = floor(4*36*5/6)=120 | 29 | {-,120} | {-,120} |
| Case 1 | M_total_15khz = floor(4*44*0.5/6)=14; M_total_30khz = floor(4*36*5.5/6)=132 | 14 | {132/5.5*0.5=12,132/5.5*5=120} | {12,120} |
| Case 2 | M_total_15khz = floor(4*44*1/7)=25; M_total_30khz = floor(4*36*6/7)=123 | 25 | {123/6=20,123/6*5=103} | {3,120} |

Proposal 4: For SCell scheduling PCell, the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) should be counted as one cell in CA scaling calculation.

PDCCH overbooking/dropping

In Rel-15/Rel-16, overbooking is only allowed on PCell/PSCell to guarantee the usage of candidates in USS sets with high-efficiency regardless of the non-dense CSS configuration. It should be noted that the overbooking is only for PDCCH scheduling PCell, i.e., overbooking is not allowed for PDCCH on PCell scheduling a SCell if the SCell is CCS by the PCell.

In Rel-17 DSS with support of SCell scheduling PCell, there are two scheduling cells (PCell and sSCell) for a scheduled cell (PCell). No matter USS sets are partially or fully moved to sSCell, overbooking should be also supported for PDCCH scheduling the PCell. As a result, overbooking should be supported on the two scheduling cells (PCell and sSCell) for the same scheduled cell (PCell). In RAN1 #104b-e meeting, it is agreed that PDCCH overbooking on sSCell USS set(s) is not allowed. That means PDCCH overbooking on PCell for self-scheduling only or overbooking across PCell and sSCell for scheduling PCell is supported.

Proposal 5: For SCell scheduling PCell, overbooking is supported for PCell self-scheduling only or across PCell and sSCell for scheduling the PCell.

For Option B with only M_max is defined and M1_max/M2_max is not defined, it may be enough for the Alt.1 (i.e., all USS with non-fallback DCI are moved to SCell) in section 3.3 and USS with fallback DCI is also absent on PCell. In this case, M1 candidates in CSS sets are all monitored and candidates dropping will only occur among M2 candidates in USS sets. The candidates dropping across PCell and sSCell can be performed on the second scheduling cell by USS index with the budget determined by a function of M_max minus M1. Moreover, M1_max and M2_max are useless in this case, because there is no need to define a budget only for the candidates in CSS sets. Meanwhile, usage of candidates in USS sets will be low-efficiency if M1_max and M2_max are defined, because the remaining candidates of M1_max minus the candidates in CSS sets cannot be utilized by the USS sets beyond the budget of M2_max.

In case of Alt.1 in section 3.3 and USS with fallback DCI present on PCell, or Alt.2 (i.e., parts of USS with non-fallback DCI are moved to SCell) in section 3.3, similar mechanism as above can be used for both scheduling cells simultaneously. That is, candidates dropping across PCell and sSCell can be performed on both scheduling cell by USS index with the budget determined by a function of M_max minus candidates of CSS in M1. For simplification, this case may be more applicable with M1_max and M2_max defined, especially for the case of different SCSs for PCell and sSCell. Then candidates dropping on PCell for self-scheduling can be performed according to M1_max.

At this stage, both Alt.1 and Alt.2 in section 3.3 are available now. PDCCH overbooking only on PCell for self-scheduling can be combined with Opion A/C, while PDCCH overbooking across PCell and sSCell for scheduling PCell can be combined with Option B. Maybe both can be workable with different UE capabilities for USS alternatives.

Proposal 6: For SCell scheduling PCell, candidates dropping can be performed across PCell and sSCell for scheduling PCell combined with Option B, or candidates dropping can be performed only on PCell for self-scheduling combined with Option A/C.

sSCell activation/deactivation and dormancy

In Rel-16, any SCell can be deactivated or switched to dormant BWP. In Rel-17 DSS with support of SCell scheduling PCell, sSCell activation/deactivation operation is agreed in RAN1 #104-e meeting listed below.

Agreement - When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported

The remaining issues on sSCell activation/deactivation and whether sSCell can be switched to dormant BWP are analyzed below.

The motivation of Rel-17 DSS with SCell scheduling PCell is offloading PDCCH to sSCell to alleviate PDCCH blocking on PCell. PCell scheduled by sSCell is a configurable function which should not impact PCell baseline scheduling, especially when sSCell is deactivated or switched to dormant BWP. Since sSCell activation/deactivation is already supported, sSCell dormancy can also be supported as in Rel-15/Rel-16. If sSCell cannot be deactivated or dormant, UE will always keep monitoring at least on two cells.

Proposal 7: When CCS from sSCell to PCell/PSCell is configured, Rel-16 CA dormancy operation for the sSCell is supported.

For Alt.1 in section 3.3, if USS with fallback DCI is absent on PCell, once sSCell is deactivated or switched to dormant BWP, there is no DCI in USS scheduling PDSCH/PUSCH on PCell. If USS with fallback DCI is present on PCell, sSCell can be deactivated or dormant at the sacrifice of system performance because only fallback DCI can be used to schedule PDSCH/PUSCH on PCell. Meanwhile, sSCell cannot be indicated to dormancy by non-fallback DCI because there is no non-fallback DCI on PCell. Because CA activation/deactivation operation for the sSCell is already supported and sSCell dormancy is recommended to be also supported, maybe a supplement mechanism is needed, i.e. USS with non-fallback DCI on PCell for self-scheduling is implicitly activated when sSCell is deactivated/dormant.

For Alt.2 in section 3.3, sSCell can be deactivated or switched to dormant BWP flexibly as non-fallback DCI formats can be configured on both PCell and sSCell.

Proposal 8: For SCell scheduling PCell, further study whether automatic recovery of USS with non-fallback DCI scheduling on PCell is needed once sSCell is deactivated or switched to dormant BWP.

Activation/deactivation of sSCell scheduling PCell/PSCell

In section 2.5, in case of sSCell activation or non-dormancy, sSCell scheduling PCell is always enabled if configured. And in case of sSCell deactivation or dormancy, sSCell scheduling PCell is disabled too. However, the motivation of this two functions are different, one is for power saving while another is for PDCCH offloading. Considering PDCCH capacity is sufficient or not on PCell is dynamic, activation/deactivation of scheduling from sSCell to PCell/PSCell can be supported instead of RRC reconfiguration.

For Alt.1 in section 3.3, if USS with fallback DCI is also absent on PCell, scheduling from sSCell to PCell/PSCell cannot be deactivated due to lack of DCI for scheduling PDSCH/PUSCH on PCell. If USS with fallback DCI is present on PCell, scheduling from sSCell to PCell/PSCell can be deactivated at the sacrifice of system performance because only fallback DCI can be used to schedule PDSCH/PUSCH on PCell.

For Alt.2 in section 3.3, scheduling from sSCell to PCell/PSCell can be activated/deactivated flexibly depending on PDCCH capacity variation as non-fallback DCI formats can be configured on both PCell and sSCell.

Above all, except for all USS sets are allocated on sSCell, activation/deactivation of CCS from sSCell to PCell/PSCell can be supported instead of RRC reconfiguration.

Proposal 9: For SCell scheduling PCell, dynamic activation/deactivation of scheduling from sSCell to PCell/PSCell can be supported to avoid frequent RRC reconfiguration.

Search space and DCI formats handling

Type 3 CSS

In RAN1 #103-e meeting, the following conclusion was achieved. It was agreed that the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

Conclusion - When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16 - FFS: DCI format 2_5 and DCI Format 2_6 handling - Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

In RAN1 #104-e meeting, the following agreement was achieved. It was agreed that UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell. That means Type 3 CSS with fallback DCI can be reused with no changes compared to Rel-15/Rel-16. In case Type 3 CSS is configured on SCell, fallback DCI in Type 3 CSS cannot be configured which is aligned with the above agreement. In case Type 3 CSS is configured on PCell, whether fallback DCI in Type 3 CSS can be configured or not depends on network implementation. More specifically, fallback DCI in Type 3 CSS can be kept on PCell/PSCell if configured.

Agreement - When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell

Except for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and fallback DCI (DCI format 1_0/0_0), DCI format 2_5 and 2_6 can also be associated with Type 3 CSS. The handling of remaining DCI formats in Type 3 CSS is analyzed below.

- Type 3 CSS with DCI format 2_5

DCI format 2_5 is used for notifying the availability of soft resources for IAB-DU. Group common DCI format 2_5 in Type 3 CSS can be configured on PCell/PSCell or SCell, which is similar as DCI format 2_0/2_1/2_2/2_3/2_4. So there is no additional issues to decide regarding the location of Type 3 CSS with DCI format 2_5.

- Type 3 CSS with DCI format 2_6

Group common DCI format 2_6 in Type 3 CSS can only be configured on PCell/PSCell because this format is used for SCell dormancy indication. If DCI format 2_6 in Type 3 CSS can be configured on the SCell used for scheduling PCell/PSCell, whether this SCell should be always active like PCell/PSCell and whether this SCell can be indicated as dormancy should be further discussed. To reduce the specification impact and standarization effort, it is preferred to keep the DCI format 2_6 on Pcell/PSCell only.

Proposal 10: For SCell scheduling PCell, Type 3 CSS with DCI format 2_5/2_6 can be reused with no changes compared to Rel-15/Rel-16, i.e.,

- Type 3 CSS with DCI format 2_5 can be configured on PCell/PSCell or SCell.

- Type 3 CSS with DCI format 2_6 can only be configured on PCell/PSCell.

USS with fall-back DCI

To ensure that RRC_IDLE UEs can still camp on this PCell successfully, network has to configure type 0/0A/1/2 CSS with fall-back DCI in the PCell. While for the USS with fallback DCI for scheduling PCell, three alternatives can be considered.

- Alt.1 Leave it on PCell

- Alt.2 Move it to sSCell

- Alt.3 Don’t support the USS with fallback DCI for scheduling PCell

Firstly, considering the spec impact and fallback DCI in CSS for PCell’s PDSCH/PUSCH scheduling can be only on PCell, Alt.2 is not preferred because fall-back DCI doesn’t support cross-carrier scheduling from the first NR release. Meanwhile, UE needs to monitor fall-back DCI for PCell’s PDSCH/PUSCH scheduling on PCell for robust scheduling. In case Alt.3 is supported, fallback DCI with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI for unicast scheduling can only be transmitted in the Type 0/0A/1/2/3 CSS, but there are also many restrictions. For example, only aggregation level 4/8/16 can be used for Type 0/0A/2 CSS; Type 1 CSS can be only used if the UE has not been provided a Type 3 CSS or a USS and the UE has received a C-RNTI; Type 3 CSS can be used if this optional search space is configured and only configured on PCell. From this perspective, Alt.3 would be too restrictive. As a result, Alt.1 is preferred due to flexible scheduling.

Proposal 11: For SCell scheduling PCell, fallback DCI in USS scheduling PCell’s PDSCH/PUSCH is restricted on PCell/PSCell only.

USS with non-fall-back DCI

During RAN1 #103-e meeting, the following agreements were achieved.

Agreements: - Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded) - Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s) - Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded) - Alt 2-1: - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-2: - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported - FFS: Details of switching mechanism - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-3: - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously - Alt 2-4: - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously - FFS following aspects - Impact of sSCell activation/deactivation and sSCell dormancy - Impact on BD/CCE limit handling - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a USS set configured for scheduling of PCell/PSCell)

During RAN1 #104-e meeting, the following working assumption was achieved.

Working Assumption: - When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s) - The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell - Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17 - FFS: Whether the UE can monitor PDCCH from both cells in the same slot.

Regarding USS with non-fall-back DCI (DCI format 0_1/1_1/0_2/1_2), Alt.1 is to move all USS sets with DCI format 0_1/1_1/0_2/1_2 to sSCell, while Alt.2 is to move parts of the USS sets with DCI format 0_1/1_1/0_2/1_2 to sSCell and leave the remaining still on PCell.

Alt.1 may be simpler while Alt.2 can offer higher flexibility for network implementation. Alt.1 is benefit to offload PDCCH to sSCell as much as possible, while Alt.2 is robust in case of worse coverage on sSCell.

Both Alt.1 and Alt.2 are available now, in accordance with some related issued analyzed in section 2, a summary is listed below.

Summary

For Alt.1, and if USS with fallback DCI is also absent on PCell, Option B with only define M_max for the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) may be enough (i.e., M1_max and M2_max are not needed), the candidates dropping across PCell and sSCell can be performed on the second scheduling (sSCell) cell by USS index with the budget determined by a function of M_max minus M1. sSCell can be deactivated or switch to dormant BWP if mechanism of automatic recovery of USS with non-fallback DCI scheduling on PCell is introduced, which is similar when scheduling from sSCell to PCell/PSCell is deactivated.

For Alt.1, and if USS with fallback DCI is present on PCell, Option A/C with M1_max and M2_max defined on each of the two scheduling cells (PCell and sSCell) for the same scheduled cell (PCell) is slightly preferred for candidates dropping on PCell for self-scheduling. sSCell can be deactivated or switch to dormant BWP at the sacrifice of system performance if no mechanism of automatic recovery of USS with non-fallback DCI scheduling on PCell is introduced, which is the same reason of that scheduling from sSCell to PCell/PSCell can be deactivated.

For Alt.2, Option A/C with M1_max and M2_max defined on each of the two scheduling cells (PCell and sSCell) for the same scheduled cell (PCell) is slightly preferred for candidates dropping on PCell for self-scheduling. Since non-fallback DCI on both scheduling cells, sSCell can be activated/deactivated or switch to dormant BWP flexibly without impact the scheduling PDSCH/PUSCH on PCell, and scheduling from sSCell to PCell/PSCell can be activated/deactivated flexibly depend on PDCCH capacity variation.

Above all, for the most important aspect on PDCCH BD/CCE handling and PDCCH overbooking handling, PDCCH overbooking/dropping only on PCell for self-scheduling can be combined with Opion A/C, while PDCCH overbooking/dropping across PCell and sSCell for scheduling PCell can be combined with Option B. Maybe both can be workable with different UE capabilities for USS alternatives.

For the Alt.2-1 to Alt.2-4, the Alt.2-1 is a baseline function which is assumed to represent Alt.2 in the analysis in section 2, and there is no reason to restrict only monitoring one scheduling cell at one time. As a result, UE can monitor PDCCH from both scheduling cells in the same slot. Note that the maximum number of DCI that UE can decoded is another issue. Alt.2-2 to Alt.2-4 can be used to reduce or distribute monitoring on two scheduling cells and can be implemented by gNB configuration.

Proposal 12: For SCell scheduling PCell and for USS with non-fallback DCI, UE can monitor PDCCH from both cells in the same slot.

Maximum number of unicast DCI

In Rel-15/Rel-16, each scheduled cell has only one scheduling cell. Take FDD as an example, the maximum number of unicast DCI that UE can decode for the scheduled cell across the set of monitoring occasions within a slot (FG 3-1) or a span (FG 3-5b/11-2) on the scheduling cell is one unicast DCI scheduling DL and one unicast DCI scheduling UL, except FG 18-5c/d which can process X=1, 2, 4 DCI scheduling DL/UL.

In Rel-17 DSS, in case a scheduled cell (PCell) has two scheduling cells (PCell and sSCell), how to define the maximum number of unicast DCI that UE can decode for the scheduled cell should be resolved.

Firstly, the value of the maximum number of unicast DCI that UE can decode for the scheduled cell is better to keep unchanged due to R17 DSS, that means the maximum number of unicast DCI that UE can decode for the scheduled cell is one unicast DCI scheduling DL and one unicast DCI scheduling UL for FDD except FG18-5c/d no matter the scheduled cell has one scheduling cell or two scheduling cells. As a result, the UE complexity will not be increased.

Secondly, because there are two scheduling cells (PCell and sSCell) for one scheduled cell (PCell), the set of monitoring occasions within a slot or a span on which scheduling cell should be determined.

- Per slot processed FG. In case the SCSs of the two scheduling cells (PCell and sSCell) are same, the selected slot can be referred to any one of two scheduling cells. In case the SCS of the two scheduling cells (PCell and sSCell) are different, the selected slot can be referred to one of the two scheduling cells with larger SCS. As shown in Figure 4, in case PCell can be scheduled by sSCell, the maximum number of unicast DCI that UE can decode for the scheduled cell(PCell) is one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot of sSCell for FDD.

Figure 4 Two scheduling cells with different SCSs

- Per span processed FG. No matter the SCS of the two scheduling cells (PCell and sSCell) are same or not, the span pattern on the two scheduling cells may be different, i.e. as shown in Figure 5, span location on the two scheduling cells may be different due to different combination (X,Y) and monitoring occasions of each scheduling cell, more discussion is needed for this scenario to determine the selected span referred to which scheduling cell.

Figure 5 Span pattern on the two scheduling cells

Proposal 13: For SCell scheduling PCell, regarding the maximum number of unicast DCIs that UE can decode for the scheduled cell (PCell),

- The number is the same as Rel-15/Rel-16 within a slot corresponding to the larger SCS between PCell and sSCell.

- FFS how to determine the maximum number within a span.

Conclusion

In this contribution, the high-level consideration and detailed consideration of SCell-schedule-PCell from our side are provided with the following proposals.

Proposal 1: For SCell scheduling PCell, PCell should be configured as the scheduling cell and the scheduled cell simultaneously by RRC parameter CrossCarrierSchedulingConfig.

- CIF=0 used for PCell self-scheduling

Proposal 2: For SCell scheduling PCell and for USS configuration, search space ID is used to associate the search space configuration for PCell and search space configuration for SCell.

- A USS set configured on PCell can only be used for either self-scheduling or CCS from SCell.

- If the USS #n on PCell is used for cross-carrier scheduling from SCell, UE acquires the MO configuration from USS #n on SCell and acquire the candidate number from USS #n on PCell, which is the same as in Rel-15/Rel-16.

Proposal 3: For SCell scheduling PCell, Option B is our first preferred option and Option C is our second preferred option compared with Option A.

- M_max/C_max can be determined by the maximum SCS of PCell and sSCell, or be determined by the SCSs of the two scheduling cells with scaling factor for each scheduling cell.

- M1_max/C1_max and M2_max/C2_max for PCell self-scheduling and sSCell scheduling PCell, can be predefined/configured respectively.

Proposal 4: For SCell scheduling PCell, the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) should be counted as one cell in CA scaling calculation.

Proposal 5: For SCell scheduling PCell, overbooking is supported for PCell self-scheduling only or across PCell and sSCell for scheduling the PCell.

Proposal 6: For SCell scheduling PCell, candidates dropping can be performed across PCell and sSCell for scheduling PCell combined with Option B, or candidates dropping can be performed only on PCell for self-scheduling combined with Option A/C.

Proposal 7: When CCS from sSCell to PCell/PSCell is configured, Rel-16 CA dormancy operation for the sSCell is supported.

Proposal 8: For SCell scheduling PCell, further study whether automatic recovery of USS with non-fallback DCI scheduling on PCell is needed once sSCell is deactivated or switched to dormant BWP.

Proposal 9: For SCell scheduling PCell, dynamic activation/deactivation of scheduling from sSCell to PCell/PSCell can be supported to avoid frequent RRC reconfiguration.

Proposal 10: For SCell scheduling PCell, Type 3 CSS with DCI format 2_5/2_6 can be reused with no changes compared to Rel-15/Rel-16, i.e.,

- Type 3 CSS with DCI format 2_5 can be configured on PCell/PSCell or SCell.

- Type 3 CSS with DCI format 2_6 can only be configured on PCell/PSCell.

Proposal 11: For SCell scheduling PCell, fallback DCI in USS scheduling PCell’s PDSCH/PUSCH is restricted on PCell/PSCell only.

Proposal 12: For SCell scheduling PCell and for USS with non-fallback DCI, UE can monitor PDCCH from both cells in the same slot.

Proposal 13: For SCell scheduling PCell, regarding the maximum number of unicast DCIs that UE can decode for the scheduled cell (PCell),

- The number is the same as Rel-15/Rel-16 within a slot corresponding to the larger SCS between PCell and sSCell.

- FFS how to determine the maximum number within a span.

Reference

RP-193260, New WID on NR Dynamic spectrum sharing (DSS), RAN #86.

RAN1 #103-e meeting, chairman notes.

RAN1 #104-e meeting, chairman notes.

RAN1 #104b-e meeting, chairman notes.