3GPP TSG RAN WG1 #105-e		R1-2104349

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

The following objective is included in this work item [1]:

- Physical layer aspects including [RAN1]:

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

According to the agreements in RAN1#104-e[2], the link level simulations and the methodology of link budget calculations are listed in appendix. In RAN1#104b-e[3], some agreements have made, including the maximum PRB number of enhanced PF 0/1/4, pre-DFT block-wise spreading and OCC length, DMRS sequence type of PF4.

In this contribution, we provide some evaluations for further discussion or down-selecting for the alternatives of enhanced PUCCH format 0/1/4, including the RE mapping for SCS 120kHz, the sequence type of PUCCH format 0/1, additionally, the specific PRB number and whether the UE multiplexing with misalignment or not. Moreover, we also discuss the cyclic shift for a single long sequence, the indication of number of RBs, and the shortage of common PUCCH resource.

- Discussion

Simulation evaluation for PUCCH

In this section, we provide details of our PUCCH evaluations, including the evaluation assumptions and reporting metrics. Then we provide some evaluation results for down-selecting or further discussion.

Evaluation assumption and methodology

Our evaluation assumptions and reporting metrics are based on the baseline as agreed in RAN1#104-e. There are some additional details we provided in the following Table 1. Furthermore, we list all the evaluation cases in Table 3 to express more clearly.

Table 1 The additional evaluation assumptions and reporting metrics

| Parameter | Notes | Values |
| --- | --- | --- |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) Note: If alternative RE allocation per PRB is used, companies to report details | For case 0/1/2, all REs are allocated per PRB. For case 3/4, there are 6 REs allocated per PRB. |
| Frequency hopping details | Frequency offset between hops, | If the PRB number of BWP is N_RB, then the frequency offset between the minimum PRB index in the first hop and the maximum PRB index in the second hop are N_RB – 1. |
| Number of RBs used per hop (N_RB) | N_RB contiguous RBs per hop | The contiguous RBs per hop is {1,2,3,4,5, 6….12} |
| Sequence construction details | Sequence type for PF0/1 Sequence type for DMRS of PF4 | For case 0/1/3, the sequence type is alt.1. For case 2/4, the sequence type is alt.2. |
| OCC configuration details | Applicable for PF1, PF4 | 1 user is configured by OCC index 0. OCC length is 2. |
| PUCCH payload encoder type | Reed Muller or Polar for PF4 | Reed Muller when payload is 4 bits or 11 bits. Polar when payload is 22 bits. |
| PUCCH encoding rate(s) | Applicable for PF4 If multiple payload sizes evaluated, encoding rates to be reported for each payload size | The encoding rates are listed in Table 2 for different payloads and different cases. |
| Number of multiplexed users, e.g., by code division, if applicable | 1 user Note: Companies to report if other cases if evaluated | Most cases are evaluated with 1 user while we also provide the evaluation with 2 users for PUCCH format 0. |
| Pmax (dBm) | Maximum allowed conducted power considering combined limit per region (from Table 3). Note: Companies should report if Pmax is considered per region or a combined limit is considered across multiple regions | Pmax is considered per region. |

Table 2 The coding rates for PUCCH format 4

|  | Case 0 | Case 1 (2 PRB) | Case 1 (4 PRB) | Case 1 (8 PRB) | Case 1 (16 PRB) |
| --- | --- | --- | --- | --- | --- |
| 4 bits | 0.0278 | 0.0139 | 0.0069 | 0.0035 | 0.0017 |
| 11 bits | 0.0764 | 0.0382 | 0.0191 | 0.0095 | 0.0048 |
| 22 bits | 0.2292 | 0.1146 | 0.0573 | 0.0286 | 0.0143 |

Table 3 The cases of PUCCH evaluation

| Case | RE mapping: (X, Y) is used to state X contiguous PRBs per hop and Y REs are allocated per PRB for one user. | Sequence type and sequence length |
| --- | --- | --- |
| Case 0 | (1, 12) | Alt 1, 12 |
| Case 1 | (X≥2, 12) | Alt 1, X*12 |
| Case 2 | (X≥2, 12) | Alt 2, 12 |
| Case 3 | (X≥2, 6) with sub-PRB interlace | Alt 1, X*6 |
| Case 4 | (X≥2, 6) with sub-PRB interlace | Alt 2, 6 |

For example, when X =2, the RE mapping and sequence type for evaluation cases mentioned above are as illustrated in Figure 1, the different colours show the sequence with cyclic shifts across RBs, the same colour means a single sequence.

Figure 1 the RE mapping and sequence type for evaluation cases when X = 2

Evaluation results and analysis

There are some evaluation results provided according to the evaluation assumptions in section 2.1.1. We also provide some analysis for down-selecting or further discussion with the RE mapping for SCS 120kHz, sequence type for PUCCH format 0/1.

Sequence Construction for Enhanced PF 0/1

Considering the sequence type for enhanced PUCCH format 0/1, there are two alternatives agreed in RAN1 104 e-meeting as the following agreement. Moreover, the maximum values for the NRB for enhanced PF 0/1/4 have been agreed in RAN1 104b-e meeting according to the simulation assumption in appendix. Firstly, the 95-th percentile CM is provided in Table 4 and Figure 2 to compare the performance of the two sequence types.

Agreement #RAN1 104-e: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Agreement #RAN1 104b-e: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS - FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Table 4 The 95-th percentile cubic metric [dB] for different number of RBs

| NRB | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alt1 (Case1) | 0.796 | 0.8147 | 1.309 | 2.111 | 2.145 | 2.202 | 2.249 | 1.601 | 2.205 | 1.771 | 2.295 | 2.031 |
| Alt2 (Case2) | 0.796 | 2.852 | 3.352 | 3.524 | 3.372 | 3.17 | 2.76 | 2.387 | 2.204 | 1.81 | 1.652 | 1.476 |
| Alt1 (Case3) | 1.444 | 0.7667 | 0.768 | 0.8029 | 2.394 | 1.305 | 2.089 | 2.123 | 2.202 | 2.144 | 1.61 | 2.211 |
| Alt2 (Case4) | 1.444 | 2.948 | 3.132 | 2.532 | 1.952 | 2.032 | 0.9752 | 1.364 | 1.956 | 2.178 | 2.276 | 2.54 |

Figure 2 the comparison of two sequence types of 95th percentile CM for different number of RBs

As we see from Table 4 and Figure 2, for case 1 and case 2, the CM of alt 2 is much larger than that of alt 1 when PRB number is not more than 9, and a bit lower when PRB number is more than 10. For case 3 and case 4, the CM of alt 2 is higher than that of alt 1 when PRB number is less than 5 or larger than 9 or equal to 6, while lower when PRB number is equal to 5/7/8/9. Additionally, the trend of alt 1 is steadier than alt 2 with varying PRB numbers. In general, alt 1 has better CM performance than alt 2 if we support only one sequence type for enhanced PF 0/1/4.

Observation 1: Alt 1 has better CM performance than alt 2 if we support only one sequence type for enhanced PF 0/1/4.

Secondly, for single user case, the Figure 3 illustrates the MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is 2 for the cases 1/2/3/4.

Figure 3 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is 2 for case 0/1/2/3/4 for single user

As we see from Figure 3, for enhanced PUCCH format 0, the MIL of case 1 is higher than that of case 2 in all regions. Moreover, in Europe, the case 2 does not have MIL gain compared with case 0, while case 1 has slight MIL gain compared with case 0. For case 3 and case 4, the case 3 & 4 both have MIL gain compared with case 0 in US and South Korea. Moreover, the MIL of case 3 is higher than that of case 4 in Europe while lower in other regions. Similarly, in Europe, the case 4 does not have MIL gain compared with case 0, while case 3 has slight MIL gain compared with case 0.

Observation 2: For enhanced PUCCH format 0, when SCS is 120kHz and PRB number is 2, the MIL of case 1 is higher than that of case 2 in all regions. The MIL of case 3 is higher than that of case 4 in Europe while lower in other regions. The case 3 & 4 both have MIL gain compared with case 0 in US and South Korea. The case 4 does not have MIL gain compared with case 0, while case 3 has slight MIL gain compared with case 0 in Europe.

Thirdly, for multiplexing of users with aligned or misaligned RB allocations shown in Figure 4, the Figure 5 and Figure 6 illustrate the MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is 2 for the cases 1/2/3/4.

Figure 4 The PRB allocation of aligned/misaligned user multiplexing

Figure 5 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is 2 for case 1/2/3/4 for multiplexing user with aligned PRB allocation

Figure 6  The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is 2 for case 1/2/3/4 for multiplexing user with misaligned PRB allocation

As we see from Figure 5, when user multiplexing with aligned RB allocation, for UE 1 and UE 2, the MIL of case 1 and 3 is higher than that of case 2 and 4 in Europe, and have minor difference in other regions.  As we see from Figure 6, when user multiplexing with misaligned RB allocation, for UE 1, the MIL of case 3 is lower than that of case 4 in all regions, while the MIL of case 1 is lower than that of case 2 in US and South Korea, similar in other regions. For UE 2, the MIL of case 1 is higher than that of case 2 in Europe and have minor difference in other regions, and the MIL of case 3 is higher than case 4.

In term of the capacity of user multiplexing, the maximum supported number of multiplexing users of alt 1 is more than that of alt 2 for user multiplexing with aligned or misaligned RB allocation. In current Rel-15/16 specification, the alt 1 is applied to PF3-DMRS, and applied to enhanced PF4-DRMS as agreed in RAN1 104b-e meeting. The alt 2 is applied only to UCI when interlace is configured. So, the specification impact of the two alternatives are almost similar.

Observation 3: When user multiplexing with aligned RB allocation, for UE 1 and UE 2, the MIL of case 1 and 3 are higher than those of case 2 and 4 in Europe, and have minor difference in other regions.

Observation 4: The capacity of user multiplexing of alt 1 is more than that of alt 2. Moreover, the specification impacts of the two alternatives are almost similar.

Proposal 1: For enhanced PUCCH format 0/1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred.

RE Mapping for Enhanced PF 0/1/4 for 120 kHz SCS

The RE mapping for enhanced PUCCH format 0/1

Considering the RE mapping for enhanced PUCCH format 0/1 of SCS 120kHz, there are two alternatives agreed in RAN1 104 e-meeting as the following agreement.

Agreement #RAN1 104-e: For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs - FFS: Values of N_RB for each SCS - For 480/960 kHz SCS, all REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - For 120 kHz SCS, further discuss the following two alternatives: - Alt-1: All REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping)

We have discussed the sequence type of alt 1 (long sequence) should be supported in section 2.1.2.1. As illustrated in Figure 3, for single user case, when PRB number equals to 2, the RE mapping alt 1(case 1) has similar MIL performance with alt 2(case 2). Furthermore, we evaluate the user multiplexing cases shown in Figure 7. The Figure 8

Figure 8 and Figure 9 illustrate the MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns} for the cases 1&3, when PRB number equals to 2. In Figure 8, the power between the two UEs are the same called “power balance”, owing to the power control tolerance in [4], 3dB is considered to model power imbalance in Figure 9 called “power imbalance”. For alt 1-1 and alt 2-1, the power imbalance does not have impact because the two users are multiplexed in different subcarriers. While for alt 1-2, the two users are multiplexed in the same RE, so, the power imbalance impact should be considered.

Figure 7 the user multiplexing cases for the two RE mapping alternatives

Figure 8 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, when power is balance

Figure 9 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, when power is imbalance

As we see from Figure 8, when power is balanced, the MIL of alt 1-1 is worse than those of alt 1-2 and 2-1 in US and South Korea, better in Europe. In Europe, the PSD is 23dBm/MHz, so the power is limited by the conduct power 21dBm. As we see from Figure 9, when power is imbalance, the MIL of alt 2-1 is better than those of alt 1-1 and 1-2 in US and South Korea as well as when delay spread is 10ns in Europe.

Observation 5: The alt 1-1 has MIL performance than those of alt 1-2 and 2-1. The alt 2-1 has better MIL performance when power is imbalanced.

Proposal 2: For enhanced PUCCH format 0/1, for 120 kHz SCS, we support alt 2 for RE mapping.

The RE mapping for enhanced PUCCH format 4

For UCI of PUCCH format 4, the Fourier basis can be used for the OCC spreading code {[1,1,1,1], [1, j, -1, -j], [1, -1, 1, -1], [1, -j, -1, j]} to multiplex 4 UEs, and {[1,1], [1, -1]} to multiplex 2 UEs, according to [4], we can conclude that the pre-DFT-OCC user multiplexing is equivalent to the comb-based user multiplexing. So, we think it is not necessary to use additional sub-PRB interlace RE mapping for UCI of enhanced PUCCH format 4.

Figure 10 Example of Rel.15 PUCCH format 4 for one PRBs using a single DFT-precoder.

Proposal 3: For UCI of enhanced PUCCH format 4, all REs within each RB are mapped is preferred for 120kHz SCS.

Figure 11 the comparison of two alternative of RE mapping for DMRS of PF4, SCS is120Khz, delay spread is {5ns, 10ns, 20ns}, payload = 4 bits/11 bits/ 22 bits

For DMRS of PUCCH format 4, the two alternatives of RE mapping are evaluated illustrated in Figure 11. As we see from Figure 11, alt 2 has better MIL performance when payload is 11 bits and 22 bits, as well as payload is 4 bits when delay spread is 10ns or 20ns.

Observation 6: Alt 2 has better MIL performance when payload is 11 bits and 22 bits, as well as payload is 4 bits when delay spread is 10ns or 20ns.

Proposal 4: For DMRS of PUCCH format 4, the sub-PRB interlaced mapping should be supported.

The other issues of enhanced PUCCH format 0/1/4

The indication of PRB number

Agreement #RAN1 104-e: - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4) Agreement #RAN1 104b-e: Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4: - Alt-1: - For enhanced PF0/1 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS - For enhanced PF4 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. - Alt-2: - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)] Agreement #RAN1 104b-e: Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4: - Alt-1: - For enhanced PF0/1 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS - For enhanced PF4 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. - Alt-2: - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

The specific value of NRB

As shown in Figure 12, the MIL of PF 0 for case 1/3 varies from PRB 1 to 12. It is more flexible to indicate every value in the allowed range. So, we support alt-1.

Figure 12 the MIL of PUCCH format 0, SCS 120kHz, 10ns, case1/3, 1~12 PRB

Proposal 5: For the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4, the alternative 1 is supported.

Indication of NRB by cell-specific (for PF0/1)

The indication of NRB for common PUCCH format 0/1 can be predefined in Table 5-1 or configured by SIB1. In Rel.15, the PUCCH frequency resource allocation is decided once the first PRB index is decided, because there is constant one PRB allocated for PUCCH 0/1. However, the PUCCH frequency resource for enhanced PUCCH 0/1 with multi-PRB should be dependent on not only the first PRB index but also the NRB. Moreover, the first PRB index is also dependent on the NRB.

For example, when the NRB indicated by SIB1 is 4, if rPUCCH8=0 ,	the UE determines the PRB index of the PUCCH transmission in the first hop as (RBBWPoffset+rPUCCHNCS)*4 and the PRB index of the PUCCH transmission in the second hop as NBWPsize-1-(RBBWPoffset-rPUCCHNCS)*4, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes. If rPUCCH8=1 ,	the UE determines the PRB index of the PUCCH transmission in the first hop as NBWPsize-1-(RBBWPoffset-rPUCCHNCS)*4 and the PRB index of the PUCCH transmission in the second hop as (RBBWPoffset+rPUCCHNCS)*4.

Moreover, for sub-PRB interlace RE mapping when SCS is 120kHz, the RE pattern candidates should be indicated by SIB1. Then the specific sub-PRB mapping pattern is dynamically indicated to UE by DCI format.

Proposal 6: The indication of NRB for common PUCCH format 0/1 can be configured in the predefined table of PUCCH resource sets before dedicated PUCCH resource configuration or configured by SIB1.

Proposal 7: The PUCCH frequency resource and the first PRB index are dependent on the NRB.

Proposal 8: The sub-PRB interlace RE mapping pattern candidates should be configured by SIB1 and then dynamically indicated to UE by DCI format.

Indication of NRB by dedicated signalling (PF0/1/4)

For dedicated PUCCH resource, the frequency resource is RRC configured in Rel.15&Rel.16, so the NRB should be indicated by UE-specific RRC configuration. Similarity, the sub-PRB interlace RE mapping pattern can be also indicated by UE-specific RRC configuration. For example, the following IE is listed:

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

comboffset                                    {2}

comb                                          {4}

PRBnumofmultiPRB                              {2}

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

Proposal 9: The indication of NRB (PF0/1/4) and sub-PRB interlace RE mapping pattern are by dedicated RRC signalling.

The cyclic shift for a single long sequence

The cyclic shift α varies as a function of the symbol and slot number according to [5]

αl=2πNscRBm0+mcs+mint+ncsns,fμ,l+l' mod NscRB

For PUCCH format 0/1, where m0 is RRC configured (the value ranges from 0 to 11) for dedicated PUCCH resource and predefined for common PUCCH resource as in Table 5-1. NscRB is 12 which is the total RE number within one PRB.  except for PUCCH format 0 when it depends on the information to be transmitted as Table 5-2 to Table 5-5 below.

As we discussed in section 2.1.2.1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred for PUCCH format 0/1. For example, the sequence length is 24 when PRB number is 2 for SCS 480kHz. To support more and better UE multiplexing, NscRB should be change to 24. Addtionally, the value range of m0 should change from 0～11 to 0～23 for dedicated PUCCH resource, the value candicates of m0 in Table 5-1 should also change for common PUCCH resource. Similarily, and the value of mcs in Table 5-2 to Table 5-5 should be changed dependent on the length of sequence.

Table 5 Copy of Table 9.2.1-1 and Table 9.2.3-3&4 and Table 9.2.5-1&2 from TS 38.213[6]

Table 5-1 Copy of Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration Index; PUCCH format; First symbol; Number of symbols; PRB offset; Set of initial CS indexes. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 10; 4; 4; {0, 3, 6, 9}. 7; 1; 4; 10; 0; {0, 6}. 8; 1; 4; 10; 0; {0, 3, 6, 9}. 9; 1; 4; 10; 2; {0, 3, 6, 9}. 10; 1; 4; 10; 4; {0, 3, 6, 9}. 11; 1; 0; 14; 0; {0, 6}. 12; 1; 0; 14; 0; {0, 3, 6, 9}. 13; 1; 0; 14; 2; {0, 3, 6, 9}. 14; 1; 0; 14; 4; {0, 3, 6, 9}. 15; 1; 0; 14; {0, 3, 6, 9}. Table 5-2 Copy of Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0 HARQ-ACK Value; 0; 1. Sequence cyclic shift. Table 5-3 Copy of Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0 HARQ-ACK Value; {0, 0}; {0, 1}; {1, 1}; {1, 0}. Sequence cyclic shift. Table 5-4 Copy of Table 9.2.5-1: Mapping of values for one HARQ-ACK information bit and positive SR to sequences for PUCCH format 0 HARQ-ACK Value; 0; 1. Sequence cyclic shift. Table 5-5 Copy of Table 9.2.5-2: Mapping of values for two HARQ-ACK information bits and positive SR to sequences for PUCCH format 0 HARQ-ACK Value; {0, 0}; {0, 1}; {1, 1}; {1, 0}. Sequence cyclic shift.

Proposal 10：For a single sequence of length equal to the total number of mapped REs of the PUCCH resource, the cyclic shift should be adapted with the length of the sequence at least for PUCCH format 0/1.

The shortage of common PUCCH resource

According to section 2.2.1,  the value of NRB can be 8 when SCS is 120kHz for 400 MHz carrier. As shown in Table 5, for row index 0, 3,7,11, the PUCCH resource set is allocated 8 PRBs when one PRB is allocated for PUCCH format 0/1. Therefore, there are at least 64 PRBs requirement for the BWP so that there should be further design for the PUCCH resource sets before dedicated PUCCH resource configuration when the bandwidth of BWP is not enough. Table 6 is an example of additional SLIV for row index 0, and the PUCCH resource are listed below. The frequency resource is saved owing to the additional SLIV.

Table 6 The example of additional SLIV for index 0

| Index | PUCCH format | First symbol, Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- |
| 0 | 0 | (12,2) (10,2) | 0 | {0, 3} |

| rPUCCH | PRB index for the first hop | CS | SLIV |  | rPUCCH | PRB index for the first hop | CS | SLIV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | （12,2） |  | 0 | 0 | 0 | （12,2） |
| 1 | 0 | 3 | （12,2） |  | 1 | 0 | 3 | （12,2） |
| 2 | 1 | 0 | （12,2） |  | 2 | 1 | 0 | （12,2） |
| 3 | 1 | 3 | （12,2） |  | 3 | 1 | 3 | （12,2） |
| 4 | 2 | 0 | （12,2） |  | 4 | 2 | 0 | （12,2） |
| 5 | 2 | 3 | （12,2） |  | 5 | 2 | 3 | （12,2） |
| 6 | 3 | 0 | （12,2） |  | 6 | 3 | 0 | （12,2） |
| 7 | 3 | 3 | （12,2） |  | 7 | 3 | 3 | （12,2） |
| 8 | NBWPsize-1 | 0 | （12,2） |  | 8 | NBWPsize-1 | 0 | （10,2） |
| 9 | NBWPsize-1 | 3 | （12,2） |  | 9 | NBWPsize-1 | 3 | （10,2） |
| 10 | NBWPsize-2 | 0 | （12,2） |  | 10 | NBWPsize-2 | 0 | （10,2） |
| 11 | NBWPsize- 2 | 3 | （12,2） |  | 11 | NBWPsize- 2 | 3 | （10,2） |
| 12 | NBWPsize- 3 | 0 | （12,2） |  | 12 | NBWPsize- 3 | 0 | （10,2） |
| 13 | NBWPsize- 3 | 3 | （12,2） |  | 13 | NBWPsize- 3 | 3 | （10,2） |
| 14 | NBWPsize- 4 | 0 | （12,2） |  | 14 | NBWPsize- 4 | 0 | （10,2） |
| 15 | NBWPsize- 4 | 3 | （12,2） |  | 15 | NBWPsize- 4 | 3 | （10,2） |

Proposal 11: The additional SLIV or OCC should be included in the PUCCH resource sets before dedicated PUCCH resource configuration.

- Conclusion

In this contribution, we focus on the evaluation methodology, result analysis and the enhancement methods on the increasing of the number of RBs of PUCCH format 0/1/4, and have the following observations and proposals:

Observation 1: Alt 1 has better CM performance than alt 2 if we support only one sequence type for enhanced PF 0/1/4.

Observation 2: For enhanced PUCCH format 0, when SCS is 120kHz and PRB number is 2, the MIL of case 1 is higher than that of case 2 in all regions. The MIL of case 3 is higher than that of case 4 in Europe while lower in other regions. The case 3 & 4 both have MIL gain compared with case 0 in US and South Korea. The case 4 does not have MIL gain compared with case 0, while case 3 has slight MIL gain compared with case 0 in Europe.

Observation 3: When user multiplexing with aligned RB allocation, for UE 1 and UE 2, the MIL of case 1 and 3 are higher than those of case 2 and 4 in Europe, and have minor difference in other regions.

Observation 4: The capacity of user multiplexing of alt 1 is more than that of alt 2. Moreover, the specification impacts of the two alternatives are almost similar.

Observation 5: The alt 1-1 has MIL performance than those of alt 1-2 and 2-1. The alt 2-1 has better MIL performance when power is imbalanced.

Observation 6: Alt 2 has better MIL performance when payload is 11 bits and 22 bits, as well as payload is 4 bits when delay spread is 10ns or 20ns.

Proposal 1: For enhanced PUCCH format 0/1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred.

Proposal 2: For enhanced PUCCH format 0/1, for 120 kHz SCS, we support alt 2 for RE mapping.

Proposal 3: For UCI of enhanced PUCCH format 4, all REs within each RB are mapped is preferred for 120kHz SCS.

Proposal 4: For DMRS of PUCCH format 4, the sub-PRB interlaced mapping should be supported.

Proposal 5: For the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4, the alternative 1 is supported.

Proposal 6: The indication of NRB for common PUCCH format 0/1 can be configured in the predefined table of PUCCH resource sets before dedicated PUCCH resource configuration or configured by SIB1.

Proposal 7: The PUCCH frequency resource and the first PRB index are dependent on the NRB.

Proposal 8: The sub-PRB interlace RE mapping pattern candidates should be configured by SIB1 and then dynamically indicated to UE by DCI format.

Proposal 9: The indication of NRB (PF0/1/4) and sub-PRB interlace RE mapping pattern are by dedicated RRC signalling.

Proposal 10：For a single sequence of length equal to the total number of mapped REs of the PUCCH resource, the cyclic shift should be adapted with the length of the sequence at least for PUCCH format 0/1.

Proposal 11: The additional SLIV or OCC should be included in the PUCCH resource sets before dedicated PUCCH resource configuration.

- Appendix

Agreement #RAN1 104-e:

Tables 1, 2, and 3 in Section 2.3 of R1-2102127 are agreed as a common set of assumptions for link level simulations and link budget calculations for evaluating enhancements to PUCCH formats 0/1/4

Note: Other parameters can be additionally considered in the evaluations

Table 1: Simplified Evaluation Assumptions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier Spacing [kHz] | 120, 480, 960 kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) Note: If other values used, companies to report values |
| PUCCH Frequency Hopping | On |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) Note: If alternative RE allocation per PRB is used, companies to report details |
| Waveform | CP-OFDM for PF0/1 DFT-s-OFDM for PF4 |
| CP Type | Normal CP |
| Channel Model | TDL-A model as defined in of TR38.901 Section 7.7.2: - Delay spread (DS) = {5ns, 10ns, 20ns} - Optional: DS = 40ns |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| PA Model | None |
| gNB TRP PN Model | Zero phase noise |
| UE PN Model | Zero phase noise |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |

Table 2: Reporting metrics

| Parameter | Value | Notes |
| --- | --- | --- |
| PUCCH Format |  | PF0, PF1, PF4 |
| Subcarrier spacing, SCS [kHz] |  | SCS = {120, 480, 960} kHz |
| Frequency hopping details |  | Frequency offset between hops, |
| Number of RBs used per hop (N_RB) |  | N_RB contiguous RBs per hop |
| PUCCH bandwidth per hop, BW [MHz] |  | BW = N_RB * 12 * SCS / 1e6 |
| Number of OFDM symbols used for PUCCH resource |  | 1 or 2 for PF0 {4 .. 14} for PF1/4 |
| Sequence construction details |  | Sequence type for PF0/1 Sequence type for DMRS of PF4 |
| OCC configuration details |  | Applicable for PF1, PF4 |
| Cyclic shift configuration details |  | For PF0/1 For DMRS of PF4 |
| Number of multiplexed users, e.g., by code division, if applicable |  | 1 user Note: Companies to report if other cases if evaluated |
| PUCCH payload encoder type |  | Reed Muller or Polar for PF4 |
| PUCCH payload size(s) (bits) |  | For PF4, at least the following values should be considered: - Low: 4 bits - Moderate: 11 bits - High: 22 bits Maximum isotropic loss (see calculation below) to be reported for each PUCCH payload size |
| PUCCH encoding rate(s) |  | Applicable for PF4 If multiple payload sizes evaluated, encoding rates to be reported for each payload size |
| Required SNR (dB) |  | Required SNR needed to fulfil detection criterion, from link level simulations based on Table 1 (see Notes (1) and (2) at bottom of table for definition of detection criteria for PF 0/1/4). |
| Cubic Metric, CM (dB) |  | Reported value is the 95th percentile, i.e., the CM for which 95% of all sequences of the design fall below |
| UE Tx Beamforming gain (dBi) |  | TxBF = 6 dBi Notes: - TxBF includes antenna element gain - If other TxBF value(s) used, companies to report value(s) |
| BS Rx Beamforming gain (dBi) |  | RxBF = 20 dBi Notes: - RxBF includes antenna element gain - If other RxBF value(s) used, companies to report value(s) |
| UE Power Limitations |  | Maximum EIRP: UE_EIRP = 25 dBm Maximum conduced power (prior to consideration of backoff): UE_P = 21 dBm Optional: - UE_EIRP = 40dBm - UE_P = 21 dBm Note: Companies to report if other cases evaluated |
| Pmax (dBm) |  | Maximum allowed conducted power considering combined limit per region (from Table 3). Note:Companies should report if Pmax is considered per region or a combined limit is considered across multiple regions |
| Backoff (dB) |  | Power backoff is equal to the cubic metric, CM Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| Transmit power, P_TX (dBm) |  | Maximum allowed transmit power including UE power limitation and backoff P_TX = min(Pmax, UE_EIRP – TxBF, UE_P – Backoff) |
| Noise power, P_N (dBm) |  | BS Noise Figure, NF = 7 dB Noise PSD = -174 dBm/Hz P_N = Noise PSD + 10*log10(BW * 1e6) + NF Note: BW is the PUCCH bandwidth per hop in MHz |
| Maximum Isotropic Loss, MIL (dB) |  | MIL = P_TX – P_N – Required SNR + TxBF + RxBF |
| Definition of detection criteria for PF0/1/4: (1) For PF0/1 (payload of 1 or 2 bits) the detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and the criterion is defined as the SNR for which P(ACK to Error) ≤ 1% AND P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only. (2) For PF4 (payload greater than 2 bits): the detection criterion is the UCI block error probability BLER ≤ 1% (as in TS38.104 Section 8.3.6) |  |  |

Table 3: Regulatory Power Limits by Region

| Region | Maximum Conducted Power, Pmax (dBm) |
| --- | --- |
| US | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm - TxBF Conducted power limit as a function of PUCCH BW per hop: Pmax_P = 27 dBm – max(0, 10*log10(100 / BW)) Combined limit: Pmax = min(Pmax_P, Pmax_EIRP) |
| Europe | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm – TxBF Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| South Korea | Conducted power limit due to EIRP limit: Pmax_EIRP = 43 dBm – TxBF when an equipment is >=300m from an astronomical antenna Pmax_EIRP = 27 dBm – TxBF when an equipment is <300m from an astronomical antenna Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| Other regions | … |
| Note: BW is the PUCCH bandwidth per hop in MHz |  |

Agreement #RAN1 104-e:

For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs

- FFS: Values of N_RB for each SCS

- For 480/960 kHz SCS, all REs within each RB are mapped

  - Note: PRB and sub-PRB interlaced mapping is not considered further

- For 120 kHz SCS, further discuss the following two alternatives:

  - Alt-1: All REs within each RB are mapped

    - Note: PRB and sub-PRB interlaced mapping is not considered further

  - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping)

Agreement #RAN1 104-e:

- The configured number of RBs for enhanced PF 0/1/4 is denoted NRB

  - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings

  - The maximum value of NRB depends on subcarrier spacing

    - FFS: maximum value for each SCS and each of PF0/1/4

  - FFS: Allowed values of NRB within the [min/max] range

  - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4)

  - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs.

  - For PF4:

    - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload

    - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers

- Note: if frequency hopping is enabled, NRB is the number of RBs per hop

- Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets based at least on the agreed evaluation assumptions

Agreement #RAN1 104-e:

- For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives:

  - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured.

  - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction:

    - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

Agreement #RAN1 104-e:

- For DMRS of enhanced PF4, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives for sequence construction:

  - Alt-1: A single sequence of length equal to the total number of mapped Res of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4.

  - Alt-2: A single sequence of length equal to the number of mapped Res per PRB of the PUCCH resource is used, and the sequence is repeated in each PRB. At least the following scheme is considered for PAPR/CM reduction:

    - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

Agreement #RAN1 104-e:

- For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 as defined for Rel-16 PF4

- Further study the following and decide in RAN1#104-b:

  - Whether or not additional OCC lengths are supported

  - Down-select to one of the following alternatives for blockwise spreading

    - Alt-1: Blockwise spreading is performed across all allocated RBs

    - Alt-2: Blockwise spreading and DFT is performed per-RB followed by per-RB PAPR/CM reduction mechanism.

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

Agreement #RAN1 104b-e:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Agreement #RAN1 104b-e:

Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured

  - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

Agreement #RAN1 104b-e:

For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 only, as in Rel-15/16.

Agreement #RAN1 104b-e:

For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

Agreement #RAN1 104b-e:

For UCI of enhanced PF4, support pre-DFT blockwise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e).

Agreement #RAN1 104b-e:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz.

Agreement #RAN1 104b-e:

User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

- References

- RP-202925, 3GPP TSG RAN Meeting #90-e, Electronic Meeting, 7-11 December 2020.

- Chairman Notes, 3GPP TSG RAN WG1 #104-e meeting.

- Chairman Notes, 3GPP TSG RAN WG1 #104b-e meeting.

- 3GPP TS 38.101-2 V17.1.0 (2021-03), User Equipment (UE) radio transmission and reception, Part 2: Range 2 StandaloneR1-1912937, 3GPP TSG RAN WG1 Meeting #99, November 18th – November 22nd, 2019

- 3GPP TS 38.211 V16.5.0 (2021-03), Physical channels and modulation.

- 3GPP TS 38.213 V16.5.0 (2021-03), Physical layer procedures for control.