3GPP TSG RAN WG1 #105-e			                     R1-2104364

e-Meeting, May 10th –  27th, 2021

Source:	vivo

Title:	Discussion on inactive state positioning and on-demand PRS

Agenda Item:	8.5.6

Document for:	Discussion and Decision

Introduction

At the RAN#91e meeting, the WID on NR Positioning Enhancements RP-210903[1] was approved including the following objective related to inactive positioning and on-demand PRS:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state - Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3] - UE-initiated request of on-demand DL PRS transmission; - LMF (network)-initiated request of on-demand DL PRS transmission;

In this contribution, we will present our views for inactive positioning and on-demand PRS.

Positioning in inactive state

At RAN#86 meeting, the study item on NR Positioning Enhancements was approved [2]. From RAN1’s perspective, the SI includes device efficiency (power consumption, complexity, etc.), low latency, etc. Nowadays, many applications provide trajectory tracking functions and require periodic positioning, such as childcare tracking, etc. Without supporting idle/inactive state positioning, UE needs to switch between connected and idle/inactive state continuously, which leads to lots of signaling overhead. Moreover, in some scenarios where flow of people needs to count or lots of user locations need to obtain, all UEs have to go back to connected mode to perform positioning which would cause large UE power consumption. In addition, from the perspective of latency, additional latency will be introduced if the UE switches to connected state from idle/inactive state for positioning measurement and report. If positioning can be performed without entering the connected mode, the UE power consumption, latency and the signaling overhead could be reduced.

Furthermore, SI has reached the below conclusion in [3, TR 38.875] that power saving gains are observed by supporting idle/inactive positioning.

TR 38.875 11 Conclusions

The UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were also analysed, and power saving gains are observed with detailed observations related to power savings are outlined in Section 8.4.

Finally, the normative work for inactive state positioning was supported in WID.

DL positioning in inactive state

DL PRS configuration

In RAN2#meeting 113e, it was concluded that PRS configuration in inactive state can be delivered to the UE by two ways: posSIB and LPP message in connected state. For the latter case, the UE can obtain UE-specific PRS configuration. However, due to the mobility of the UE in inactive state, part or all of the PRS configuration may become invalid. For instance, when the UE moves out of a certain area, some previous PRS configurations may no longer be applicable, such as priority indication of TRPs, expected-RSTD, etc. The priority of the TRP below may be related to the location of the UE. For example, in order to ensure positioning accuracy, some TRPs around the cell where the UE is located can be indicated as high priority. However, as the UE moves in inactive state, the TRP with the previous high priority may be far away from the UE, but the TRP currently around the UE is indicated with a low priority. If the UE continues to use the previous priority rules for measurement and reporting, the positioning accuracy and efficiency may be affected. Therefore, the validity criteria for PRS configuration in inactive delivered by LPP message in connected state should be considered.

TS38.214 [4] Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 dl-PRS-IDs of the frequency layer are sorted according to priority; - Up to 2 DL PRS resource sets per dl-PRS-ID of the frequency layer are sorted according to priority.

- Validity criteria for PRS configuration in inactive state delivered by LPP message in connected state should be considered.

In real deployment, the PRS can be considered as a cell-level or regional-level public signal. For UEs in connected state and UEs in inactive state, they can share the same PRS in the same area. For example, the LMF can select the PRS resources, configured for quite a large number of connected UEs, to be shared to inactive UEs. Furthermore, in Rel-17, LMF Initiated on-demand control of DL-PRS transmission is recommended for normative work. According to some UEs’ demand or LMF’s own demand, the LMF can adjust the configuration of PRS, e.g., request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc. In a simplest case, PRS transmission can be ‘turned off’ when not needed and turned on when needed according some default set of PRS configuration parameters. However, ‘turning on/off’ some set of PRS configuration affect the PRS reception in the inactive state. For instance, one UE may obtain assistance data for DL positioning during RRC_CONNECTED state, and then it enters RRC_INACTIVE state. To perform positioning measurement, the UE will "wake up" at the periodical time window configured for the PRS to search and monitor the PRS. Subsequently, the LMF turn off the set of PRS configuration on demand. Once the inactive UE is not notified in time, it still ‘wake up’ at the pre-configured periodical time window to search and monitor PRS, which will lead to wasteful power consumption. Therefore, it is proposed to inform the inactive UEs the minor change of PRS configuration for on-demand control without entering connected state, when the size of this information is small, e.g., ‘turn on/off’ some set of PRS configuration.

- Informing the inactive UEs the minor change of PRS configuration without entering connected state can be considered.

- e.g., minor change of PRS configuration for on-demand control, such as ‘turn on/off’ some set of PRS configuration

Physical layer procedure for DL PRS measurement

- QCL information

In connected state, PRS can be configured with QCL-D relationship with SSB or other RS. On the UE side, it can assume that PRS and SSB or other RS have a similar spatial relationship. For PRS reception in inactive state, regarding QCL-D relationship configured for PRS, we don’t see that it needs to be changed compared to the connected state. Therefore, we support reusing QCL configuration in connected state for PRS reception for inactive UEs.

- Support reusing QCL configuration in connected state for PRS reception for inactive UEs.

- Relationship between PRS measurement and initial DL BWP

Another issue should be considered is the relationship between PRS measurement and initial DL BWP. In general, there is no measurement gap configured for inactive state PRS measurement, since in inactive state, it can be considered that there is no data transmission, so that when UE performs inter-frequency measurement (e.g. RRM measurement, PRS measurement), it is no need to configure measurement gap to interrupt data transmission. However, when the UE performs cell search and initial access, the initial DL BWP is configured for the UE to receive coreset 0, RMSI and other important messages. If the UE needs to measure the PRS during the effective time of the initial BWP, the related UE behavior needs to be clarified. For example, whether to support UE to process PRS outside the initial DL BWP and/or PRS whose SCS is different with the initial DL BWP.

- The relationship between PRS measurement and initial DL BWP should be further studied, e.g. including

- how to support UE to process PRS outside the initial DL BWP and/or PRS whose SCS is different with the initial DL BWP

- PRS collision with other DL signals

In inactive state, in addition to measuring the PRS, the UE would also perform cell search and initial access related procedures, therefore, collisions between the PRS and these signals are also a problem. Due to no measurement gap is configured for PRS measurement in inactive state, hence, when PRS and other DL signals (e.g. SSB, SIB1, COREST0, MSG2/MSGB, paging, etc.) are in the same symbol, how the UE processes the PRS needs to be considered.

- In inactive state, when PRS and other DL signals (e.g. SSB, SIB1, COREST0, MSG2/MSGB, paging, etc.) are in the same symbol, how the UE processes PRS should be considered.

DL positioning report size optimization

Considering positioning report in inactive state is mainly the scope of RAN2, in this section, we briefly introduce some content related to RAN1, e.g. DL positioning report size optimization.

To reduce resource overhead and improve latency performance for some applications involving relatively infrequent exchange of small amounts of UL data (e.g., one uplink packet), the UE is allowed to transmit UL small data in idle or inactive state. In LTE, for EDT, only data size less than 1000 bits can be delivered using EDT, otherwise the connected mode procedure is used. In NR, data volume threshold is used for the UE to decide whether to do SDT or not. Only if the data size is small enough, it makes sense to transmit in inactive state. In previous RAN2 meeting, it was agreed that positioning report in inactive state is supported by enhancing small data transmission. Therefore, the data size for positioning report in inactive state also needs to meet the requirements of small data transmission.

Positioning report in inactive state may include positioning measurement or location estimate. For positioning measurements results, it would not be categorized as small data. For example, according to the structure of DL-TDOA measurement, if we only consider 3 TRPs’ measurements to be report, the bit size would be more than 1000 bits. Furthermore, if we include all the TRP measurement results, after rough calculation, the size of data for positioning report will even reach the order of 100,000 bits, which is far from acceptable. The size of the measurement result depends on the number of neighboring cells and the report format.

Therefore, how to optimize the data size of positioning report especially for positioning measurements in inactive state but not to reduce the positioning performance too much should be considered. There may be 2 options for DL positioning report size optimization.

- One is to compress positioning report size. As mentioned earlier, the size of the measurement result is related to the number of neighboring cells, so if we limit the number of neighboring cells in the measurement result, the size of the measurement result can be greatly reduced. Therefore, priority indication for TRPs/PRSs to be measured and reported can be considered for inactive positioning report. In addition, differential report can also be considered. Differential report can represent the change/difference of the latter report compared to the previous report in 2 adjacent reports. For instance, if the difference between the two measurement results is less than a certain threshold, the UE reports a small number of bits (e.g., 1 bit) to indicate that the measurement result is almost unchanged; if the difference between the two measurement results exceeds a certain threshold, the UE reports the differential measurement result. This method is meaningful for inactive UEs, since inactive UEs may have lower mobility, and the PRS measurement results (e.g., first path in DL-TDOA) between the 2 adjacent reports may not change much.

- Another option is to allocate suitable size for the positioning report. Since the size of the positioning measurement may be larger than the general SDT, it can be considered to allocate dedicated/separate RACH resource (e.g., RO+preamble combination) for the positioning report to distinguish it from the general SDT. Then the network can allocate suitable uplink resources for positioning data transmission in inactive state.

- The data size optimization of positioning report especially for positioning measurements in inactive state should be considered. Two options can be considered:

- Option1: compressing positioning report size

- Option2: allocating suitable size for the positioning report

DL positioning capability

Potential new UE capabilities need to be defined for inactive state positioning, which may include the following at least:

- Positioning measurement in inactive state should be supported. Hence, the applicable state of UE measurement for positioning in [5, 38.215 5.1] needs to be extended to RRC_CONNECTED and RRC_INACTIVE.

- PRS processing capability in inactive state should be defined. In inactive state, in order to save power, the UE may turn off some functions and modules, so the PRS processing capability may be limited. For example, the maximum supported PRS bandwidth per frequency layer may be smaller than connected state; the maximum number of supported frequency layers may be smaller than connected state; the durationOfPRS-Processing capability (N, T) for any P(≥T) time window may also be different with connected state, etc. In addition, considering PRS processing capability in connected state is defined assuming measurement gap is configured, while in inactive state, there may be no measurement gap is assumed, so that the PRS processing capability without measurement gap should be considered in inactive state.

- Other capabilities such as NR-DL-PRS-ResourcesCapability should also be considered, which may be different with connected state.

- DL positioning capability should be defined in inactive state, e.g. including at least

- Positioning measurement supporting in inactive state

- DL PRS processing capability in inactive state

- DL PRS resource capability in inactive state

UL positioning in inactive state

UL positioning configuration

UL positioning in inactive states should also be supported in Rel-17. In order to reduce unnecessary workload, we propose to study the uplink positioning in the inactive state based on the uplink positioning framework in the connected state. Therefore, in the inactive state, we should prioritize research on SRS for positioning. So, we propose

- Support SRS for positioning as UL positioning RS in inactive state.

According to the NR design, dedicated SRS is only allocated to UE in connected state. Hence, in current specification, UL positioning is only applicable to UE in connected state. In inactive state, UL positioning can be applied to UE if the UE’s dedicated SRS configuration is not released after the UE leaving connected mode but still in an area that is valid predefined by the network. With this enhancement, an inactive state UE could transmit dedicated SRS, and then the network could measure SRS and obtain the position of the UE. The dedicated SRS configuration can be transmitted to the UE via RRC release with suspend config when UE is in connected state. The predefined area can be the cell in which RRC Release is received or a cell list, where the SRS configuration is relatively stable without large update and does not need to be updated frequently. When the UE leaves this area, the UE should initiate random access to enter the connected state. Correspondingly, the UE and gNB need to release the previously configured SRS applied in inactive state.

We think there may be some concerns about this enhancement. For each inactive UE requiring positioning, SRS configuration needs to be reserved in both the camping and neighbouring cells. This may cause a lot of resource occupation, especially when there are many UEs performing UL positioning in the inactive state. Will this enhancement lead to a very limited capability for positioning? In our understanding, the number of positioning UE can be supported is related to the period of SRS allocated to each UE. The longer the period is, the more UEs can be supported. In some popular applications, the interval between consecutive positioning requests is very long, e.g. 1 minute. Accordingly, the period of positioning SRS resource for each UE can be very long. Therefore, the number of positioning UEs can be supported seems not a big issue. We believe these applications are the main potential motivation for UL positioning inactive state. So, long-period SRS can be applied for uplink positioning in inactive state, e.g. period>=160ms.

- Support to transmit SRS configuration via RRC release when UE is in connected state for UL positioning in inactive states.

- SRS configuration is valid in a predefined area, where the SRS configuration is relatively stable without large update and does not need to be updated frequently

- The predefined area can be the cell in which RRC release is received or a cell list

Due to the mobility of the UE in inactive state, the part or all of the SRS configuration may become invalid. In addition to the predefined area, the other validity criteria should also be considered for SRS configuration in inactive state. For example, the RSRP of the pathloss RS or the spatial relation RS measured by the UE has changed a lot, causing the pathloss RS or the spatial relation RS previously configured to be no longer available; or, the timing of UE transmitting SRS changes a lot, e.g., due to camping cell change or TA command update; or, some other event causing the pre-configured SRS configuration is no longer valid. Therefore, we think the validity criteria for SRS configuration in inactive state should be further studied.

In addition, the fallback behavior should also be considered when the validity criteria for SRS configuration in inactive state is not met. One way is to enable the UE to enter the connected state and perform positioning or update the SRS configuration in the connected state. Moreover, considering that positioning report in the inactive state has been supported, when the validity criteria for SRS configuration is not met, the UE can remain in the inactive state and use the similar method as the positioning report (e.g., via SDT) to indicate the event causing validity criteria not met and require updated SRS configuration; and then the gNB can deliver the updated SRS configuration to the UE without entering connected state.

- The validity criteria for SRS configuration in inactive state should be considered.

- The fallback behavior should be considered when the validity criteria for SRS configuration in inactive state is not met, e.g., including:

- Entering connected state to perform UL positioning or update the SRS configuration

- Remaining in inactive state to perform UL positioning and receive updated SRS configuration

Moreover, in order to reduce UE power consumption, we hope that the UE enters the connected state as little as possible during the UL positioning procedure. For example, when a small part of the SRS configuration becomes invalid or a small part of the SRS configuration needs to be updated, the UE may not enter the connected state to receive the updated SRS configuration. A simple way to update small SRS configuration in inactive state is through the paging message. Thus, we think transmission of the SRS configuration while retaining the UE in inactive state can be considered.

- Transmission of the SRS configuration while retaining the UE in inactive state can be considered.

Physical layer procedure for UL positioning

In this section, we focus on transmission procedure of SRS in inactive state, including: SRS spatial relation, SRS power control and SRS transmission timing.

- SRS spatial relationship

Regarding SRS spatial relation in inactive state, there may be two ways that can be applied to SRS transmission. One way is to pre-configure SRS spatial relation with another RS, such as SSB or PRS. However, this way needs to consider the problem of spatial relationship failure caused by UE movement. Another way is to enable SRS beam sweeping in inactive state. In this way, there is no need to consider maintaining the spatial relationship between the UE and gNB. In addition, considering that in UL positioning, all gNBs around the UE need to measure SRS, SRS beam sweeping is a suitable way for SRS transmission. At the same time, in order to improve coverage, SRS repetition during beam sweeping may also be considered.

- SRS power control

For SRS power control, the UE can be configured with power control related parameters when it leaving connected state. For example, for one SRS resource set, in order to increase the probability of successful pathloss RS detection, the UE can be indicated with a set of SSB, and then it selects a certain SSB whose measurement result is larger than the threshold as the pathloss RS. If the UE determines that the UE is not able to accurately measure the pre-configured pathloss RS, the UE calculates pathloss using a RS resource obtained from the SS/PBCH block of the cell that the UE uses to obtain MIB, e.g. MIB of the camping cell.

- SRS transmission timing

For UL positioning in inactive state, the SRS transmission timing can be changed depending on the change of DL timing of the camping cell and related TA adjustment. However, in inactive state, SRS transmission timing cannot be adjusted in time as the connected state. This makes the inactive UE not synchronized in uplink, causing interference to other UEs which are synchronized in connected mode. In our understanding, the positioning SRS is monitored by more than one gNB. However, one UE can at most maintain uplink synchronization with one of the gNBs (i.e. the serving gNB of a connected UE). It implies even connected UE maybe unsynchronized with some gNBs monitoring its positioning SRS according the current UL positioning design. Hence, uplink un-synchronization seems not a new issue introduced by allowing inactive mode UE to send dedicated positioning SRS. Considering SRS transmission timing in connected state is based on the timing and TA configuration of  the serving cell, similarly, we think SRS transmission timing in inactive state can be based on the camping cell.

- Regarding UL SRS for positioning in inactive state, potential specification impact on SRS spatial relationship, SRS power control and SRS transmission timing should be further studied by RAN1.

DL+UL positioning in inactive state

For DL and UL positioning method, the UE not only needs to measure downlink signals from TRPs and report Rx-Tx measurements, but also transmits an uplink signal to the same TRPs. It can be considered as a combination of downlink only positioning method and uplink only positioning method. So that if DL only positioning method and UL only positioning method are supported, DL and UL positioning method is naturally supported. Therefore, we propose to give priority to the work on separate DL positioning and separate UL positioning, and then work on the DL and UL positioning.

- Prioritize the work on separate DL positioning and separate UL positioning in inactive state, and then work on the DL and UL positioning in inactive state.

On-demand PRS

In RAN2#113e[6], the following agreements were achieved for on-demand PRS.

Agreements: UE-initiated on-demand PRS request is enabled by enhancing LPP RequestAssistanceData. FFS how much control the network has over the UE request. The UE-initiated mechanism is enabled by the UE request triggering a request from the LMF, and the actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated. Put the stage 2 description for UE-initiated and LMF-initiated PRS request under the same framework.

In this section, some issues related to on-demand PRS are discussed.

UE-initiated request of on-demand PRS

The procedure of UE-initiated request of on-demand DL-PRS is as following figure, where UE transmits the request to LMF through LPP, and LMF determine the final request of on-demand DL-PRS and transmit to gNBs.

Figure 1 UE-initiated request of on-demand PRS

In this procedure, LMF can collect requests from more than one UEs, and determine the final requests that should be required to guarantee the performance of those requesting UE. This is useful for determining the final requests according to UEs from multiple cells (including both serving gNBs and neighbor gNBs).

It is noted that the on-demand PRS is about UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc. For UE-initiated request of on-demand PRS, UE can request specific PRS parameters based on the QoS request (e.g. accuracy and latency requirement) of the location service, its own preferences (e.g. low power requirement) and/or previous measurements/location estimates. For these specific parameters such as PRS pattern, periodicity or BW, we think they belong to RAN1-related parameters and should be determined by RAN1.

LMF-initiated request of on-demand PRS

The procedure of LMF-initiated request of on-demand PRS is as Figure 2, where LMF transmits the request to gNBs, and also provides the new configuration of on-demand DL-PRS to UEs.

Figure 2 LMF-initiated request of on-demand PRS

It is a good solution to support LMF-initiated request of on-demand DL-PRS, since no matter whether it is a MO-LR, NI-LR or MT-LR Service, LMF would know the location service requirement and further decide which on-demand PRS should be requested taken all the requirements into consideration. Besides, except for the UE-based positioning, LMF can acquire all measurement results/location estimates of each target device, so that it is also reasonable for LMF to initiate on-demand PRS according to various measurement results/location estimates. For UE-based positioning, it is also helpful to determine on-demand PRS by LMF if LMF can request the UE to report measurements and location results together, such as via the signaling of requesting location information to request measurements and location results together and get a corresponding response, then, the requirement of UE-based positioning can be taken into account for the new configuration of on-demand PRS.

- To support LMF-initiated request of on-demand DL-PRS, the measurement results and location can be reported and/or requested to be reported for UE-based mode.

For LMF-initiated request of on-demand PRS, LMF can request specific PRS parameters based on the QoS request (e.g. accuracy and latency requirement) of the location service, UE-initiated on-demand request and/or previous measurements/location estimates. As for specific parameters that can be dynamically changed, we think the detailed parameters should also be defined in different levels and decided by RAN1.

- It is up to RAN1 to decide specific parameters (e.g. PRS pattern, periodicity, BW, etc) for LMF-initiated and UE-initiated request of on-demand PRS.

Interpretions of ON/OFF request

In our view, the interpretations of ON/OFF request can be the following aspects:

The most basic interpretation of ON/OFF request of on-demand DL PRS transmission is the simplest way to enable on-demand PRS. The ON request is a message to start the on-demand PRS transmission and the OFF request is a message to fallback to the PRS transmission with basic configurations. The basic ON request is easy to understand, for example, the gNBs have been configured with normal PRS and on-demand PRS, and in general they only transmit normal PRS. Only when the gNBs receive the ON request they begin to transmit the on-demand PRS. The basic OFF request, for us, is more reasonable to be a message to fallback to the PRS transmission  with the configuration to meet basic positioning requirements rather than turn off the PRS completely. One reason is that there is rarely a situation when the location service is not needed at all, another reason is that for idle/inactive UEs which cannot request for their requirement in time, or R16 UEs which don’t support on-demand PRS request, the network cannot assume these UEs do not need location service and turn off PRS transmission completely. Therefore, as far as we are concerned, the basic OFF request should be intended to turn off on-demand PRS transmission and fallback to PRS transmission with a basic PRS configuration such as configuration of small bandwidth, long periodicity, low power or low resource overhead to meet the basic positioning requirement.

A finer level of interpretation of ON/OFF request of on-demand DL PRS transmission is to ON/OFF which level of PRS configuration. The intention of on-demand PRS is mainly for improving accuracy, reducing latency, improving network efficiency, and device efficiency. From these perspectives, for the ON/OFF request, at least the ON/OFF request in frequency layer, resource set and resource level should be supported. The ON request in frequency layer typically includes frequency layer and bandwidth, which is requested for improving positioning accuracy. The ON request in resource set should include periodicity to meet the lower latency requirement, repetition factor to guarantee the coverage or muting pattern to reduce the interference. The ON request in resource level may be aimed to request a finer beam to achieve high accuracy. For OFF request, when the high accuracy or low latency or high network/device efficiency is not needed, the OFF request can request each PRS configuration parameter to fallback to a basic configuration.

Another interpretation is that when the PRS pre-configuration is numbered, such as a list of PRS configurations each consists of associated DL-PRS parameters (bandwidth, periodicity, repetition factor, frequency layer, muting etc.). In this case, UE/LMF can request to ON one PRS from the preconfigured PRS configuration list. When UE/LMF request OFF message, it means the PRS should be fallbacked to a basic configuration.

From the above discussion, we think at least the basic ON/OFF request and the finer level ON/OFF request should be supported. The basic ON/OFF request is for the simplest on-demand requirement, while the finer level ON/OFF request can meet different on-demand requirement flexibly. To support the above ways of ON/OFF request, firstly basic PRS configurations of different levels should be predefined, for example, a basic PRS configuration consisting of all parameters, or a basic PRS configuration of bandwidth. Secondly multiple on-demand PRS in different levels should be configured, for example, several configured bandwidths and periodicity, then UE/LMF can request to ON one PRS configuration from the configured multiple PRS.

- For ON/OFF request of on-demand PRS, at least support the following two interpretations:

- A basic ON/OFF request of on-demand PRS

- A finer level ON/OFF request of on-demand PRS in frequency layer, resource set and resource level

- ON request of on-demand PRS means to start the transmission of on-demand PRS.

- OFF request of on-demand PRS means to turn off the transmission of on-demand PRS and  fallback to the transmission of PRS with basic configurations.

- Support the following configurations of on-demand PRS:

- Basic PRS configurations in different configuration levels

- Multiple on-demand PRS configurations in different configuration levels

Other related issues

On-demand PRS capability

On-demand DL-PRS is not allowed for UEs in Rel-15 or Rel-16 or earlier versions. In Rel-17, for those UEs support on-demand DL-PRS, they need report their capabilities to network, so that network can distinguish which UE is supportive of on-demand DL-PRS. And these UEs can be configured and take measurements with on-demand DL-PRS. For on-demand DL-PRS, UE capability is an important parameter that needs consideration.

- On-demand PRS capability needs to be defined and reported to network.

On-demand measurement gap

In current specification, when the UE is expected to measure the PRS resource outside the active DL BWP it may request a measurement gap via higher layer parameter NR-PRS-MeasurementInfoList. If on-demand PRS is introduced, the corresponding on-demand measurement gap also need to be introduced.

For example, UE is configured by on-demand PRS with a small periodicity for small latency requirement, but the other PRS configuration remain unchanged, in this case the UE will not require a new measurement gap but still measure and process PRS according to the previous measurement gap configuration, thus failing to achieve the purpose of ‘on-demand’.

To support on-demand measurement gap, both the UE and LMF request procedure should be considered. The current specification only support UE requesting a measurement gap, but for on-demand measurement gap, we think LMF request procedure will have more benefit, the reasons are mainly as following:

- Based on the conclusion of Rel-16 ‘UE is not expected to process DL PRS without configuration of measurement gap’, if this behaviour does not change during Rel-17 stage, regardless of whether the PRS is within the scope of the active BWP, MG is always requested.  Therefore, even if the LMF does not know the BWP configuration, it can also request the MG on-demand.

- LMF request MG can reduce latency. For example, this request can be sent by LMF before ‘LPP Provide Assistance Data message’, so it is no need for the UE to spend extra time requesting the MG and even waiting for MG configuration after receiving assistance data, which can reduce physical layer latency nearly 20ms.

- Support to introduce on-demand measurement gap for on-demand PRS in Rel-17.

- LMF requests measurement gap should be supported.

Interference caused by on-demand PRS

The interference of on-demand PRS are mainly caused by the different configurations. For example, in a certain area, TRPs can transmit the on-demand PRS with 100M bandwidth as the high accuracy demand of some UEs; however, other UEs adjacent to this area still receive regular PRS with a small bandwidth like 20M and use the additional 80MHz to receive data, the ‘80MHz bandwidth data’ of these UEs will be interfered by the on-demand PRS. Another example is shown in figure 3, where TRP transmits the on-demand PRS with small periodicity as the request of some UEs, other UEs with a large PRS periodicity will be interfered by the small periodicity PRS.

Figure 3 Interference caused by on-demand PRS

- Interference problems caused by on-demand PRS should be considered and solved by RAN1.

Conclusion

In this contribution, we discuss issues for inactive positioning and on-demand PRS with the following proposals.

- Validity criteria for PRS configuration in inactive state delivered by LPP message in connected state should be considered.

- Informing the inactive UEs the minor change of PRS configuration without entering connected state can be considered.

- e.g., minor change of PRS configuration for on-demand control, such as ‘turn on/off’ some set of PRS configuration

- Support reusing QCL configuration in connected state for PRS reception for inactive UEs.

- The relationship between PRS measurement and initial DL BWP should be further studied, e.g. including

- how to support UE to process PRS outside the initial DL BWP and/or PRS whose SCS is different with the initial DL BWP

- In inactive state, when PRS and other DL signals (e.g. SSB, SIB1, COREST0, MSG2/MSGB, paging, etc.) are in the same symbol, how the UE processes PRS should be considered.

- The data size optimization of positioning report especially for positioning measurements in inactive state should be considered. Two options can be considered:

- Option1: compressing positioning report size

- Option2: allocating suitable size for the positioning report

- DL positioning capability should be defined in inactive state, e.g. including at least

- Positioning measurement supporting in inactive state

- DL PRS processing capability in inactive state

- DL PRS resource capability in inactive state

- Support SRS for positioning as UL positioning RS in inactive state.

- Support to transmit SRS configuration via RRC release when UE is in connected state for UL positioning in inactive states.

- SRS configuration is valid in a predefined area, where the SRS configuration is relatively stable without large update and does not need to be updated frequently

- The predefined area can be the cell in which RRC release is received or a cell list

- The validity criteria for SRS configuration in inactive state should be considered.

- The fallback behavior should be considered when the validity criteria for SRS configuration in inactive state is not met, e.g., including:

- Entering connected state to perform UL positioning or update the SRS configuration

- Remaining in inactive state to perform UL positioning and receive updated SRS configuration

- Transmission of the SRS configuration while retaining the UE in inactive state can be considered.

- Regarding UL SRS for positioning in inactive state, potential specification impact on SRS spatial relationship, SRS power control and SRS transmission timing should be further studied by RAN1.

- Prioritize the work on separate DL positioning and separate UL positioning in inactive state, and then work on the DL and UL positioning in inactive state.

- To support LMF-initiated request of on-demand DL-PRS, the measurement results and location can be reported and/or requested to be reported for UE-based mode.

- It is up to RAN1 to decide specific parameters (e.g. PRS pattern, periodicity, BW, etc) for LMF-initiated and UE-initiated request of on-demand PRS.

- For ON/OFF request of on-demand PRS, at least support the following two interpretations:

- A basic ON/OFF request of on-demand PRS

- A finer level ON/OFF request of on-demand PRS in frequency layer, resource set and resource level

- ON request of on-demand PRS means to start the transmission of on-demand PRS.

- OFF request of on-demand PRS means to turn off the transmission of on-demand PRS and  fallback to the transmission of PRS with basic configurations.

- Support the following configurations of on-demand PRS:

- Basic PRS configurations in different configuration levels

- Multiple on-demand PRS configurations in different configuration levels

- On-demand PRS capability needs to be defined and reported to network.

- Support to introduce on-demand measurement gap for on-demand PRS in Rel-17.

- LMF requests measurement gap should be supported.

- Interference problems caused by on-demand PRS should be considered and solved by RAN1.

- References

- RP-210903-(rev.fr.RP-210897)-clean, Revised WID on NR Positioning Enhancements

- 3GPP RP-193237, New SID on NR Positioning Enhancements

- TR 38.875, “Study on NR Positioning Enhancements”

- TS 38.214, “Physical layer procedures”

- TS 38.215, “Physical layer measurements”

- Report of session on positioning and sidelink relay, RAN2#113bis-e

- Report of session on positioning and sidelink relay, RAN2#113-e