3GPP TSG RAN WG1 #105-e		R1-2104383

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Other enhancements for simultaneous operation of child and parent links

Agenda Item:	8.10.2

Document for:	Discussion and Decision

- Introduction

New WID on Enhancements to Integrated Access and Backhaul has been approved as following [1].

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]: - Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including: - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx). - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing. - Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we provide our view on IAB-node timing mode(s), power control and interference management to support simultaneous transmission and/or reception by child and parent links.

- Discussion

  - Timing mode

In the previous RAN1 e-meetings, the following were agreed.

Agreement An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode - FFS whether the Rel-16 OTA synchronization mechanism is sufficient or enhancements are required - If required, details of enhancements including the uplink timing(s) required to support different timing alignment cases Agreement Case 6 timing mode operation at an IAB-node is controlled by the parent node to which the UL transmission is intended for. Agreement An IAB-node, when operating in Case 7 timing mode, can enable a child node to set its DL Tx timing based on Rel-16 OTA timing synchronization mechanism. - FFS whether Rel-16 OTA synchronization mechanism enhancements are required - FFS details of enhancements, if required Agreement Case 7 timing is supported with symbol level alignment without explicit support for slot level alignment Agreement Switching between Case 1, Case 6, and Case 7 timing is supported. - FFS whether Case 6 and Case 7 timing shall be restricted to certain resources, e.g. excluding resources used for access or TDM backhaul - FFS details on switching including the switching conditions - FFS relationship between switching timing modes with the usage/indication of different resource multiplexing modes - FFS whether Rel-16 OTA synchronization shall be enhanced to support switching timing modes

In Rel-16, it was specified a mechanism to fulfill Case 1 timing, where the timing of DU DL is derived from TA and T_delta. TA is the parameter defined for MT UL timing advance adjustment, and T_delta is signaled for adjustment of DU DL timing. In our opinion, the derivation of DU DL timing of Case 6 and Case 7 timing mode can reuse the Rel-16 OTA synchronization mechanism, assuming that the legacy TA and T_delta would always be signaled to IAB node even when Case 6 or Case 7 timing mode is enabled (i.e., according to TA+T_delta).

Proposal 1: The derivation of DU DL TX timing of Case 6 and Case 7 timing mode is based on Rel-16 OTA synchronization mechanism.

In Case 6 timing mode, the transmission timing of MT (i.e., MT UL timing) should be aligned with the transmission timing of DU (i.e., DU DL timing). This means that the IAB-MT can ignore the UL timing according to Rel-16 TA adjustment mechanism when Case 6 timing is applied. In this case, the DU DL timing can reuse the Rel-16 OTA synchronization mechanism, meanwhile the MT UL timing can align with the DU DL timing of the IAB node. According to the above, the signaling defined in Rel-16 is enough and no additional signaling is needed. Besides, for the parent node, it should simultaneously maintain multiple UL reception timing if it has been configured with multiple timing modes with one or several IAB child nodes. Therefore, we have the following proposal.

Proposal 2: To support Case 6 timing mode, IAB node should set its MT TX timing in alignment with the collocated DU TX timing.

In Case 7 timing mode, the reception timing of DU (i.e., DU UL timing) should be aligned with the reception timing of MT (i.e., MT DL timing). Thus, the child MT should maintain 2 TAs for UL transmission to support simultaneous reception of IAB node, one TA is used for transmission based on the case 7 timing mode, the other TA is used for transmission based on legacy case 1 timing mode. In our opinion, the Rel-15 UE timing advance adjustment can be baseline for UL timing determination of child MT, and the IAB node can either indicate a timing offset in addition to Rel-15 TA or indicate a new timing advance (e.g., TA_new) beside the Rel-15 TA to maintain the 2 TAs.

Proposal 3: To support Case 7 timing mode, the DU should indicate timing offset in addition to TA or indicate a new timing advance (e.g., TA_new) to the child node.

Furthermore, in order to avoid the impact of the access UEs, the new timing advance adjustment should only applied to child IAB node.

Proposal 4: The enhanced UL timing adjustment should not be applied to access UEs.

Based on the agreement made in RAN1#104e e-meeting, relationship between switching timing modes with the usage/indication of different resource multiplexing modes should be further studied.  In our opinion, Rel-17 timing modes are associated with corresponding multiplexing cases, i.e., Case 6 is applied to multiplexing case A (MT TX, DU TX); Case 7 is applied to multiplexing case B (MT RX, DU RX). Therefore, if the multiplexing cases is changed, the timing mode should be changed accordingly. Some details of multiplexing case adaptation can be found in our companion contribution [4].

Proposal 5: The switching between different timing modes is associated with the switching of the associated multiplexing cases.

  - Power control

Agreement RAN1 to further study whether the legacy UL power control mechanism (including PHR) is sufficient for an IAB-node operating in an enhanced multiplexing mode. - FFS: if not (i.e., the legacy mechanism is not sufficient), support an IAB-node indicating information to assist with its UL power control. Agreement Support an IAB-node indicating information to assist with the DL power control of its parent-node towards the IAB-node without mandating an expected behavior at the parent node. - Note: At least the assistance information is for supporting the simultaneous operation within the IAB-node to avoid power imbalance - FFS: type of assistance information (e.g., desired received power, power adjustment, preferred CSI-RS resource) - FFS: whether this information is provided to the parent-node, the CU, or both. - FFS: applicability of the assistance information (e.g. relation to beams or multiplexing modes) - FFS: the channel carrying this assistance information

DL power control

In RAN1#104e e-meeting, DL power control enhancement of parent node has been agreed for supporting the simultaneous Rx/Rx within the IAB-node. For balancing reception PSD between DU and MT, MT reception PSD should be pre-determined based on intended reception PSD of IAB DU. It is noted that the situation of the enhanced DL power control is analogous to UL power control, where the PSD of the intended receiver is considered to calculate the transmission power. With such assumption, the DL transmission of parent DU can be calculated at least according to the pre-determined reception PSD of IAB MT, scheduled bandwidth and pathloss between parent DU and IAB MT. Thus, the following proposal is made.

Proposal 6: To assist DL power control of parent node, IAB MT reports its desired reception power to parent node.

The channel condition between parent DU and IAB MT will impact the DL transmission/reception power. Hence, it is beneficial for IAB MT to dynamically feedback power adjustment command to assist DL power control of parent node, thus to maintain relative stable reception power at IAB MT. Moreover, channel condition varies per beam, if the power adjustment command is reported per beam, parent DU can maintain multiple redundant beams for DL transmission which increases system robustness.

However, it is noted that feedback of power adjustment command is beneficial only when parent can adjust its transmission power. If parent DU is not capable to change its transmission power (e.g., EPRE) due to its RF constraint, the parent node should have the freedom not to follow the power adjustment command.

Proposal 7: Support IAB MT to report DL power adjustment command to parent node.

- FFS per beam report of DL power adjustment command.

- Parent node is not mandated to follow the power adjustment command.

In RAN1#104e e-meeting, it has agreed to further study the signaling to report DL power control information from IAB to parent node. Since the power control information is delivered to parent node, both MAC CE and UCI can be used. If MAC CE is used to convey the report, the reporting procedure is up to RAN2; if UCI is used, RAN1 should reuse existing UCI reporting procedure for simplicity, e.g., reuse CSI report framework.

Proposal 8: Either MAC CE or UCI is used for reporting DL power control information to parent node.

UL power control

In RAN1#104e e-meeting, it has been agreed to further study the enhancement for UL power control procedure. In legacy UL power control procedure, the UE transmission power is determined based on power control formula with an upper bound restricted by Pcmax. However, in case of simultaneous transmission of IAB DU and IAB MT, the transmission EPRE of IAB MT is restricted by a pre-determined DU EPRE, then the maximum transmission power of IAB MT is determined by the pre-determined EPRE and scheduled bandwidth. Therefore, the upper bound of IAB MT transmission power should be determined based on PSD restriction of IAB MT.

Proposal 9: The maximum UL TX power of IAB MT is determined based on TX PSD restriction of IAB MT.

  - Interference management

Agreement Use the Rel-16 interference management frameworks (e.g. CLI, RIM) to handle IAB interference scenarios, and discuss if any of the following enhancements are needed (not an exhaustive list): - FFS: extend the information exchange (e.g. the resource configuration, result of CLI measurements, etc.) among different entities (e.g. between parent-child nodes, adjacent IAB nodes, between network and IAB-node, etc.) - FFS: required enhancements on CLI measurement accuracy (e.g. via timing adjustment, etc.) - FFS: required enhancements on CLI measurements (e.g. introducing short-term measurements, multi-beam measurements, etc.) Agreement RAN1 to select among the following options to support DU-to-DU measurement and report. - For DU-to-DU CLI measurement: - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques) - Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM) - Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements) - For DU-to-DU CLI report: - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques) - Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM) - Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements) Agreement RAN1 to decide whether to enhance interference mitigation through information exchange to support beam-management at the parent or child node in RAN1#104bis-e - FFS: reporting of desired beams for reception in DL or desired beams for transmission in UL by the IAB node for a given multiplexing mode - FFS: indicating applicable beams in DL or beams in UL for a given multiplexing mode.

DU-to-DU CLI management

In RAN1#104e e-meeting, several options for DU-to-DU CLI measurement and report have been identified for down-selection. For the CLI measurement, it is believed that DU implementation can handle it. For the CLI report, there is no need to exchange measured CLI between different gNBs following assumption made in Rel-16 CLI WI. However, if the measured CLI is to be reported to CU, the related signaling can be specified in RAN3.

Proposal 10: For DU-to-DU CLI measurement and report, no RAN1 impact is expected.

UE-to-UE CLI management

In RAN1#103e e-meeting, it has been agreed to reuse Rel-16 CLI framework as starting point to handle IAB interference. In Rel-16 CLI management framework, the intended TDD configurations are exchanged for the TDD configuration coordination between gNBs; besides, CLI measurement and report schemes were specified to handle the case where TDD configurations are not aligned.

Regarding the CLI measurement, it is believed that the existing RSRP/RSSI based measurements specified in Rel-16 are sufficient. From the CLI report perspective, it is beneficial to report the CLI to both IAB node and CU to assist resource scheduling/configuration, thus the related signaling should be enhanced.

Proposal 11: UE-to-UE CLI is reported to both parent node and CU.

- FFS related signaling.

Regarding the exchange of resource configuration (e.g., at least including TDD configuration) between gNBs, high layer signaling is assumed. However, RAN1 can discuss whether additional resource configuration parameter(s) besides TDD configuration should be exchanged between IAB nodes or not. It is suggested to discuss the exchange of H/S/NA property between IAB nodes, because coordination of H/S/NA property between IAB nodes can efficiently avoid interference, e.g., if IAB node A suffers interference from IAB node B in a given slot, it can simply set ‘H’ to IAB node A and set ‘NA’ to IAB node B in the slot.

Proposal 12: For UE-to-UE CLI mitigation, exchange of resource configuration between IAB nodes should be specified, including TDD configuration and/or resource type configuration. Related signaling is up to RAN3.

Beam management

Agreement RAN1 to decide whether to enhance interference mitigation through information exchange to support beam-management at the parent or child node. - FFS: reporting of desired beams for reception in DL or desired beams for transmission in UL by the IAB node for a given multiplexing mode - FFS: indicating applicable beams in DL or beams in UL for a given multiplexing mode.

In RAN1#104e e-meeting, it has been agreed to further study beam management enhancement to mitigate interference. To control the IAB MT reception interference, either IAB DU implementation can decide not to schedule UL/DL beams which incur severe interference to MT reception, or IAB MT implementation can report proper CRIs to parent node for which IAB MT reception suffers less from interference. To control the DU reception interference, either IAB DU implementation can schedule proper UL beams which suffer less interference, or IAB MT can report proper SRIs or train proper UL beams which incur less interference to DU reception. The only problem is that the afore-mentioned proper beam direction is changing per multiplexing mode, thus the legacy beam management needs to be enhanced to support per multiplexing case beam training/beam indication, i.e., CRI report is per multiplexing case, UL beam training is per multiplexing case.

Proposal 13: For the BH link between parent DU and IAB MT, enhance the associated DL/UL beam management as following.

- CRI report is per multiplexing case.

- UL beam training is per multiplexing case.

- Conclusion

This contribution focus on IAB-node timing mode(s), power control and interference management to support simultaneous transmission and/or reception by child and parent links and the following are proposed,

Proposal 1: The derivation of DU DL TX timing of Case 6 and Case 7 timing mode is based on Rel-16 OTA synchronization mechanism.

Proposal 2: To support Case 6 timing mode, IAB node should set its MT TX timing in alignment with the collocated DU TX timing.

Proposal 3: To support Case 7 timing mode, the DU should indicate timing offset in addition to TA or indicate a new timing advance (e.g., TA_new) to the child node.

Proposal 4: The enhanced UL timing adjustment should not be applied to access UEs.

Proposal 5: The switching between different timing modes is associated with the switching of the associated multiplexing cases.

Proposal 6: To assist DL power control of parent node, IAB MT reports its desired reception power to parent node.

Proposal 7: Support IAB MT to report DL power adjustment command to parent node.

- FFS per beam report of DL power adjustment command.

- Parent node is not mandated to follow the power adjustment command.

Proposal 9: The maximum UL TX power of IAB MT is determined based on TX PSD restriction of IAB MT.

Proposal 10: For DU-to-DU CLI measurement and report, no RAN1 impact is expected.

Proposal 11: UE-to-UE CLI is reported to both parent node and CU.

- FFS related signaling.

Proposal 12: For UE-to-UE CLI mitigation, exchange of resource configuration between IAB nodes should be specified, including TDD configuration and/or resource type configuration. Related signaling is up to RAN3.

Proposal 13: For the BH link between parent DU and IAB MT, enhance the associated DL/UL beam management as following.

- CRI report is per multiplexing case.

- UL beam training is per multiplexing case.

- Reference

- RP-193251, “New WID on Enhancements to Integrated Access and Backhaul”, RAN#86, Spain, December 2019.

- TR38.874, “Study on Integrated Access and Backhaul”, RAN1, 3GPP.

- R1-2100465, “Other enhancements to Rel-17 IAB nodes”, vivo, RAN1#104-e, e-Meeting, Jan, 2021.

R1-2104382, “Enhancement to resource multiplexing between child and parent links”, vivo, RAN1 #105-e, e-Meeting, May, 2021.