3GPP TSG RAN WG1 #105-e                                             R1-2104392

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Discussion on joint scheduling

Agenda Item:	8.13.2

Document for:	Discussion and Decision

- Introduction

Regarding the study of scheduling PDSCH on multiple cells using a single DCI in R17 DSS WID [1]:

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

In the contribution, we provide our views on scheduling PDSCH on two cells with a single DCI.

- Discussion

We have evaluated the performance of joint scheduling for the following scenarios, and provide the results in Appendix:

- Baseline for comparison: self-scheduling+cross-carrier-scheduling

  - Scheduling the scheduling cell and another cell by separate single-cell-DCIs

  - 100% UE are scheduled by single-cell-DCIs

- Scenario of multi-cell scheduling for evaluation:

  - Scheduling the scheduling cell and another cell simultaneously by a joint-DCI

  - 100% UE are scheduled by joint-DCI

- Combination 1

  - Scheduling cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs

  - Scheduled cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW

- Combination 2

  - Scheduling cell: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs

  - Scheduled cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW

- Combination 3

  - Scheduling cell: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs

  - Scheduled cell: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW

- Combination 4

  - Scheduling cell: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs

  - Scheduled cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW

Figure 1. scenarios for comparison between single scheduling and multi-cell scheduling

Several aspects including CCE saving, PDCCH blocking, throughput, and scheduling flexibility are analysed. In the LLS simulation, we assume that the payload size of a single-cell-DCI that schedules a single serving cell is 60bits (excluding 24bits CRC), and the payload size of joint-DCI is 60/72/84/96/108 bits (excluding 24bits CRC) respectively, corresponding to a 50/40/30/20/10% DCI information bits compression rate compared with using 2 single-cell-DCIs. According to the simulation results in the appendix, we have the following observations

Observation 1. Compared with using single-cell-DCI, a joint-DCI scheduling two PDSCHs on two cells brings more than 
-  33.09% CCE saving for combination 1, 
-  28.13% CCE saving for combination 2,
-  32.59% CCE saving for combination 3, 
-  18.14% CCE saving for combination 4, 
and the CCE saving gain becomes more significant if the compression rate of joint-DCI size increases.

Observation 2. With the same CORESET bandwidth, a joint-DCI with size (excluding CRC) =108 bits brings around
-  8.22%~8.84% reduction in PDCCH blocking rate for combination1, 
-  5.2%~8.05% reduction in PDCCH blocking rate for combination2,
-  6.49%~9.07% reduction in PDCCH blocking rate for combination3,
-  2.37%~5.66% reduction in PDCCH blocking rate for combination4
compared with using single-cell-DCI, and the reduction in PDCCH blocking rate becomes more significant if the joint-DCI size decreases.

Observation 3. Compared with using single-cell-DCI, joint-DCI brings around
-  24~27 RB reduction in CORESET BW and <=3.24% theoretical throughput gain for combination1, 
-  42~54 RB reduction in CORESET BW and <=3.32% theoretical throughput gain for combination2,
-  12~16 RB reduction in CORESET BW and <=3.66% theoretical throughput gain for combination3,
-  6~12 RB reduction in CORESET BW and <=4.79% theoretical throughput gain for combination4.

If the joint DCI size is large enough (i.e., 108bit) to provide independent resource allocation fields for each carrier as well as high scheduling flexibility, the corresponding throughput gain approximates the theoretical throughput gain. However, further compression of resource allocation fields in joint-DCI may be unavoidable for the small DCI size, consequently, the scheduling flexibility would be inevitably restricted – for instance, the granularity of frequency-domain scheduling resources (i.e., RBG size) becomes coarser in joint-DCI compared to single-cell-DCI, which may affect the spectrum efficiency adversely. For example, in combination 4, assuming that the RBG size for single-cell DCI scheduling is 8 RB, to compress the joint DCI size to 96 bits, the RBG size for joint-DCI scheduling needs to be increased to be greater than 8 RB (i.e., 9 RB or larger). While for the intra-band cases having the same numerology and same bandwidth (i.e., combination 1/3), the RBG size can remain the same as the single-cell DCI scheduling. Based on this assumption and results in Figure 5~Figure 7, we have the following observations:

Table 1. RBG size assumption for evaluation

|  | Single-cell-DCI | Scheme4(96bits) | Scheme5(108bits) |
| --- | --- | --- | --- |
| Combiantion1 | 8RB | 8RB | 8RB |
| Combiantion2 | 16RB | 16RB | 16RB |
| Combiantion3 | 4RB | 4RB | 4RB |
| Combiantion4 | 8RB | 9RB | 8RB |

Observation 4. Joint-DCI with size=96bits~108 bits can bring
-  <=2.44% throughput gain compared with single-cell-DCI for combiantion1 in practical scenarios
-  <=2.32% throughput gain compared with single-cell-DCI for combiantion2 in practical scenarios
-  <=3.12% throughput gain compared with single-cell-DCI for combiantion3 in practical scenarios
compared with single-cell-DCI.

Observation 5. When the number of UE is 10, joint-DCI with size=96bits~108 bits can bring <=1.42% throughput gain compared with single-cell-DCI for combiantion4 in practical scenarios. However, it can also lead to 0.2%~0.31% throughput loss if the number of UE increases to 15/20.

In summary, multi-cell scheduling could bring throughput gain due to PDCCH overhead reduction and PDCCH blocking rate reduction. However, the throughput gain may not be significant due to inevitable scheduling restrictions, especially for the inter-band cases. Considering the simulations results and the time budget for Rel-17 DSS, we suggest one of the following options as a way forward:

- Alt1. Support multi-cell PDSCH scheduling with a single DCI in Rel-17 DSS WID only for simplified scenarios, e.g., scheduled cells with same SCS in intra-band CA

- Alt2. To stop working on this feature in Rel-17 DSS, while concluding that the multi-cell scheduling PDSCH with a single DCI will be addressed in Rel-18

Proposal 1. Consider adopting one of the following alternatives for the way forward on joint scheduling:
	Alt1. Support multi-cell scheduling PDSCH with single DCI in Rel-17 DSS WID for simplified scenarios, e.g., scheduled cells with same SCS in intra-band CA

	Alt2. To stop working on this feature in Rel-17 DSS, while concluding that the multi-cell scheduling PDSCH with a single DCI will be addressed in Rel-18.

- Potential issues

As shown in the above evaluation, DCI size compression is important to achieve resource-saving and thus system throughput improvement. However, how to compress the DCI payload size for multi-cell scheduling of different TBs is a challenging task. On one side, in R15 and R16, most of the information fields in a DCI depend on the configurations of the scheduled cell, so an information field cannot be directly shared between the cells, especially if there is a big difference between the configurations of the two cells. It is preferable not to share the fields that heavily depend on the configuration of the scheduled cell and the fields that directly affect the transmission mechanism such as MCS. The fields that do not depend on the scheduled cells, such as MIMO-related information and PUCCH-related information that is mainly associated with PUCCH Pcell, can be considered to be shared between two cells. In the following table, we listed some major information fields and provides a preliminary analysis of the field type for combination1. The legacy-DCI is 60 bits while the joint-DCI based on Table 2 is 96 bits.

Table 2. Example of DCI information field type

| Information field | Separate field or shared field | Number of bits |
| --- | --- | --- |
| Identifier for DCI formats – 1 bit | Shared field | 1 |
| Carrier indicator – 0 or 3 bits | Shared field | 3 |
| Bandwidth part indicator – 0, 1 or 2 bits | Separate field | 2 |
| FDRA – 13 bits for 106 RB | Separate field | 26 |
| TDRA – 0, 1, 2, 3 or 4 bits | Separate field | 6 |
| VRB-to-PRB mapping – 1 bit | Shared field | 1 |
| PRB bundling size indicator – 0 or 1 bit | Separate field | 2 |
| Rate matching indicator – 0, 1, or 2 bits | Separate field | 2 |
| ZP CSI-RS trigger – 0, 1, or 2 bits | Separate field | 2 |
| Modulation and coding scheme – 5 bits | Separate field | 10 |
| New data indicator – 1 bit | Separate field | 2 |
| Redundancy version – 2 bits | Separate field | 4 |
| HARQ process number – 4 bits | Separate field | 8 |
| Downlink assignment index – 0,2,4 bits | Shared field | 4 |
| TPC command for scheduled PUCCH – 2 bits | Shared field | 2 |
| PUCCH resource indicator – 3 bits | Shared field | 3 |
| PDSCH-to-HARQ feedback timing indicator – 3 bits | Shared field | 3 |
| Antenna port(s) – 4, 5, or 6 bits | Separate field | 10 |
| Transmission configuration indication 0 or 3 bits | Shared field | 0 |
| SRS request – 2, 3 bits | Separate field | 4 |
| DMRS sequence initialization – 1 bit. | Shared field | 1 |
| Joint-DCI size |  | 96 bits (excluding CRC) |

Proposal 2. Field type (i.e., shared or cell-specific) of each information field in joint-DCI needs to be investigated.

In addition to the field design of joint-DCI mentioned above, the following issues need to be resolved as well.

- The framework of multi-cell scheduling

In cross-carrier scheduling, UE is indicated by RRC IE CrossCarrierSchedulingConfig that the data on this cell is scheduled either by PDCCH in its own cell (i.e. ‘own’ in schedulingCellInfo) or PDCCH on another cell (i.e. ‘other’ in schedulingCellInfo). It is not clear if a cell that can be jointly scheduled is allowed to perform self-scheduling, e.g., UL scheduling. If yes, the design of multi-cell scheduling to some extent is similar to type3 CSS. If not, it should be further clarified that whether the UL grant for the jointly scheduled cell can be located on a cell other than the one on which joint-DCI is received. We need to clarify whether the support of multi-cell scheduling may reuse the current cross-carrier scheduling framework with some changes.

- Whether multi-cell scheduling reuses the framework of type3 CSS or cross-carrier scheduling

- Whether a cell jointly scheduled by another cell can be configured with PDCCH resources and can perform self-scheduling, e.g., for UL transmission

- Whether a joint-DCI monitored for a cell and a legacy-DCI monitored for the same cell should be located on the same serving cell

- Configuration of cell pairs for multi-cell scheduling

It is necessary to further clarify that how to configure the cell pairs for multi-cell scheduling and whether there are any restrictions on choosing cell pairs for multi-cell scheduling. For example, the following aspects need to be clarified:

- Whether there can be more than one cell configured with a joint-DCI

- Whether a Scell can be configured with a joint-DCI

- Whether a cell can jointly schedule other two cells with a joint-DCI

- Whether Pcell can be scheduled by a Scell with a joint-DCI

- SS configuration and DCI format for multi-cell scheduling

Another aspect that needs to study is the DCI format for multi-cell scheduling. If an existing DCI format such as DCI format 1_1 is used for multi-cell scheduling purpose, the DCI for single scheduling case should always perform zero-padding to keep alignment with the DCI size of the case where multi-cell scheduling is enabled, which implies that a large number of redundant bits will be inserted into the DCI format even for single scheduling. In this case, it not only leads to a higher DCI code rate but also leads to worse coverage and worse performance for single scheduling. By contrast, introducing a new DCI format for multi-cell scheduling can ensure a high degree of scheduling flexibility while avoiding negative impacts on the performance of the existing DCI. However, it raises some other issues, such as how to maintain the DCI size budget.

- PDCCH BD budget maintenance for multi-cell scheduling

In R15, the PDCCH BD budget is maintained for each scheduled cell and there is only one scheduled cell corresponding to a legacy-DCI. However, two scheduled cells are corresponding to a joint-DCI in the multi-cell scheduling case, it should be discussed how to maintain the PDCCH BD budget across the scheduled cell pair. For example, whether the two scheduled cells share the same budget, or if they should be handled individually.

- HARQ-ACK reporting for multi-cell scheduling

For single TB on every PDSCH occasion involved in a HARQ-ACK codebook in R15, there is only 1 HARQ-ACK bit generated in response to the corresponding DCI with an associated PUCCH. However, if multi-cell scheduling is supported, there would be more than 1 TB scheduled by a DCI, which may have different outcomes of PDSCH decoding. Thus, some discussions such as whether HARQ-ACK bits corresponding to the jointly scheduled PDSCHs should be multiplexed in the same codebook and how to determine the HARQ-ACK in response to a joint-DCI, as well as some enhancements for the HARQ-ACK codebook construction, i.e., how to guarantee the reliability of HARQ codebook in case of missed joint-DCI, are needed.

Observation 6. To support multi-cell scheduling, the following issues need to be resolved
-  DCI field design
-  Any restrictions on the scheduled cells to be paired for multi-cell scheduling
-  Framework of multi-cell scheduling
-  Whether to introduce a new DCI format 
-  PDCCH BD budget maintenance if multi-cell scheduling is enabled
-  HARQ-ACK codebook determination if multi-cell scheduling is enabled

- Conclusion

In this contribution, we discuss some issues on multi-cell scheduling and have the following observations and proposals:

Observation 1. Compared with using single-cell-DCI, a joint-DCI scheduling two PDSCHs on two cells brings more than 
-  33.09% CCE saving for combination 1, 
-  28.13% CCE saving for combination 2,
-  32.59% CCE saving for combination 3, 
-  18.14% CCE saving for combination 4, 
and the CCE saving gain becomes more significant if the compression rate of joint-DCI size increases.

Observation 2. With the same CORESET bandwidth, a joint-DCI with size (excluding CRC) =108 bits brings around
-  8.22%~8.84% reduction in PDCCH blocking rate for combination1, 
-  5.2%~8.05% reduction in PDCCH blocking rate for combination2,
-  6.49%~9.07% reduction in PDCCH blocking rate for combination3,
-  2.37%~5.66% reduction in PDCCH blocking rate for combination4
compared with using single-cell-DCI, and the reduction in PDCCH blocking rate becomes more significant if the joint-DCI size decreases.

Observation 3. Compared with using single-cell-DCI, joint-DCI brings around
-  24~27 RB reduction in CORESET BW and <=3.24% theoretical throughput gain for combination1, 
-  42~54 RB reduction in CORESET BW and <=3.32% theoretical throughput gain for combination2,
-  12~16 RB reduction in CORESET BW and <=3.66% theoretical throughput gain for combination3,
-  6~12 RB reduction in CORESET BW and <=4.79% theoretical throughput gain for combination4.

Observation 4. Joint-DCI with size=96bits~108 bits can bring
-  <=2.44% throughput gain compared with single-cell-DCI for combiantion1 in practical scenarios
-  <=2.32% throughput gain compared with single-cell-DCI for combiantion2 in practical scenarios
-  <=3.12% throughput gain compared with single-cell-DCI for combiantion3 in practical scenarios
compared with single-cell-DCI.

Observation 5. When the number of UE is 10, joint-DCI with size=96bits~108 bits can bring <=1.42% throughput gain compared with single-cell-DCI for combiantion4 in practical scenarios. However, it can also lead to 0.2%~0.31% throughput loss if the number of UE increases to 15/20.

Observation 6. To support multi-cell scheduling, the following issues need to be resolved
-  DCI field design
-  Any restrictions on the scheduled cells to be paired for multi-cell scheduling
-  Framework of multi-cell scheduling
-  Whether to introduce a new DCI format 
-  PDCCH BD budget maintenance if multi-cell scheduling is enabled
-  HARQ-ACK codebook determination if multi-cell scheduling is enabled

Proposal 1. Consider adopting one of the following alternatives for the way forward on joint scheduling:
	Alt1. Support multi-cell scheduling PDSCH with single DCI in Rel-17 DSS WID for simplified scenarios, e.g., scheduled cells with same SCS in intra-band CA

	Alt2. To stop working on this feature in Rel-17 DSS, while concluding that the multi-cell scheduling PDSCH with a single DCI will be addressed in Rel-18.

Proposal 2. Field type (i.e., shared or cell-specific) of each information field in joint-DCI needs to be investigated.

- Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN #86, Dec 2019.

- Appendix

- Evaluation: CCE saving

Table 3. Required SNR and the distribution of each AL @ combination1

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 12.64 | 23 | 12.64 | 23 | 14.35 | 20 | 17.96 | 15 | / | / | / | / |
| AL=2 | 4.96 | 27 | 4.96 | 27 | 6.23 | 24 | 7.22 | 25 | 8.109 | 37 | 8.856 | 34 |
| AL=4 | 0.4998 | 24 | 0.4998 | 24 | 1.032 | 27 | 1.515 | 28 | 2.068 | 28 | 2.547 | 29 |
| AL=8 | -2.873 | 19 | -2.873 | 19 | -2.425 | 20 | -1.889 | 21 | -1.526 | 22 | -1.191 | 22 |
| AL=16 | -5.816 | 6 | -5.816 | 6 | -5.384 | 7 | -5.029 | 9 | -4.694 | 10 | -4.335 | 12 |
| Average No of CCEs | 4.25*2 |  | 4.25 |  | 4.57 |  | 4.99 |  | 5.38 |  | 5.69 |  |
| Saving ratio | / |  | 50% |  | 46.25% |  | 41.33% |  | 36.73% |  | 33.09% |  |

Table 4. Required SNR and the distribution of each AL @ combination2

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 6.169 | 43 | 6.169 | 43 | 7.662 | 38 | 10.73 | 30 | / | / | / | / |
| AL=2 | 1.258 | 23 | 1.258 | 23 | 1.969 | 23 | 2.341 | 29 | 3.012 | 56 | 3.594 | 53 |
| AL=4 | -2.123 | 19 | -2.123 | 19 | -1.695 | 22 | -1.26 | 23 | -0.8911 | 24 | -0.4904 | 24 |
| AL=8 | -4.927 | 8 | -4.927 | 8 | -4.57 | 10 | -4.139 | 10 | -3.845 | 11 | -3.531 | 12 |
| AL=16 | -7.14 | 3 | -7.14 | 3 | -6.798 | 2 | -6.442 | 3 | -6.177 | 4 | -5.882 | 6 |
| Average No of CCEs | 2.89*2 |  | 2.89 |  | 2.99 |  | 3.24 |  | 3.79 |  | 4.15 |  |  |
| Saving ratio | / |  | 50% |  | 48.20% |  | 43.82% |  | 34.33% |  | 28.13% |  |  |

Table 5. Required SNR and the distribution of each AL @ combination3

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 12.26 | 20 | 12.26 | 20 | 14.02 | 17 | 17.05 | 13 | / | / | / | / |
| AL=2 | 5.064 | 21 | 5.064 | 21 | 6.337 | 20 | 7.097 | 22 | 7.822 | 32 | 8.929 | 28 |
| AL=4 | 0.1091 | 30 | 0.1091 | 30 | 0.6573 | 31 | 1.27 | 28 | 1.777 | 27 | 2.357 | 27 |
| AL=8 | -3.042 | 21 | -3.042 | 21 | -2.522 | 21 | -2.05 | 23 | -1.663 | 25 | -1.276 | 27 |
| AL=16 | -5.872 | 7 | -5.872 | 7 | -5.405 | 10 | -4.964 | 12 | -4.642 | 13 | -4.253 | 14 |
| Average No of CCEs | 4.67*2 |  | 4.67 |  | 5.14 |  | 5.56 |  | 5.98 |  | 6.29 |  |
| Saving ratio | / |  | 50% |  | 44.91% |  | 40.42% |  | 35.94% |  | 32.59% |  |

Table 6. Required SNR and the distribution of each AL @ combination4

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 5.785 | 44 | 5.785 | 44 | 7.176 | 39 | 10.08 | 31 | / | 0 | / | 0 |
| AL=2 | 0.4598 | 25 | 0.4598 | 25 | 1.294 | 25 | 1.87 | 29 | 2.5 | 57 | 3.003 | 55 |
| AL=4 | -2.662 | 17 | -2.662 | 17 | -2.22 | 20 | -1.791 | 22 | -1.388 | 23 | 0.9966 | 11 |
| AL=8 | -5.62 | 8 | -5.62 | 8 | -5.258 | 9 | -4.819 | 10 | -4.528 | 12 | -4.217 | 25 |
| AL=16 | -7.973 | 2 | -7.973 | 2 | -7.607 | 3 | -7.262 | 3 | -6.961 | 3 | -6.686 | 4 |
| Average No of CCEs | 2.69*2 |  | 2.69 |  | 3.01 |  | 3.21 |  | 3.68 |  | 4.40 |  |  |
| Saving ratio | / |  | 50% |  | 43.99% |  | 40.27% |  | 31.46% |  | 18.14% |  |  |

- Evaluation: PDCCH blocking

| Figure 2. PDCCH blocking rate when No of scheduled UEs=10 |  |
| --- | --- |
| Figure 3. PDCCH blocking rate when No of scheduled UEs=15 |  |
| Figure 4. PDCCH blocking rate when No of scheduled UEs=20 |  |

- Evaluation: Throughput

Table 7. CORESET BW required for different schemes with the same blocking rate@ combination1

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 66 | 72 |
| Throughput gain | / | 3.24% | 3.03% | 2.59% | 2.16% | 1.73% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 66 | 72 |
| Throughput gain | / | 3.24% | 3.03% | 2.59% | 2.16% | 1.73% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 66 | 69 |
| Throughput gain | / | 3.24% | 3.03% | 2.59% | 2.16% | 1.95% |

Table 8. CORESET BW required for different schemes with the same blocking rate@ combination2

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 270 | 138 | 156 | 168 | 192 | 216 |
| Throughput gain | / | 3.09% | 2.67% | 2.39% | 1.83% | 1.27% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 270 | 132 | 156 | 168 | 198 | 228 |
| Throughput gain | / | 3.23% | 2.67% | 2.39% | 1.69% | 0.98% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 270 | 138 | 150 | 168 | 204 | 228 |
| Throughput gain | / | 3.09% | 2.81% | 2.39% | 1.55% | 0.98% |

Table 9. CORESET BW required for different schemes with the same blocking rate@ combination3

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 48 | 32 | 32 | 32 | 36 | 36 |
| Throughput gain | / | 3.66% | 3.66% | 3.66% | 2.74% | 2.74% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 48 | 32 | 32 | 32 | 36 | 36 |
| Throughput gain | / | 3.66% | 3.66% | 3.66% | 2.74% | 2.74% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 48 | 32 | 32 | 32 | 36 | 36 |
| Throughput gain | / | 3.66% | 3.66% | 3.66% | 2.74% | 2.74% |

Table 10. CORESET BW required for different schemes with the same blocking rate@ combination4

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 96 | 48 | 57 | 60 | 72 | 90 |
| Throughput gain | / | 4.79% | 3.89% | 3.59% | 2.39% | 0.60% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 78 | 84 |
| Throughput gain | / | 4.49% | 4.19% | 3.59% | 1.79% | 1.20% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 78 | 84 |
| Throughput gain | / | 4.49% | 4.19% | 3.59% | 1.79% | 1.20% |

Figure 5. Throughput gain achieved by scheme4/5(96 bits/108bits) when No of UE=10

Figure 6. Throughput gain achieved by scheme4/5(96 bits/108bits) when No of UE=15

Figure 7. Throughput gain achieved by scheme4/5(96 bits/108bits) when No of UE=20

Table 11. LLS Simulation assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Carrier frequency(scheduling cell) Subcarrier spacing Simulation bandwidth/No. of PRBs Antenna configuration at BS/UE CORESET configuration | • Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz(106RB) carrier BW, 2-symbol CORESET with 96RBs • Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz(273RB) carrier BW, 1-symbol CORESET with 270RBs • [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz(52RB) carrier BW, 3-symbol CORESET with 48RBs] • [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz(106RB) carrier BW, 2-symbol CORESET with 96RBs] |
| Link-level Channel model | TDL-C |
| TDL correlation level | Low |
| Delay Spread | 300ns |
| UE speed | 3 km/h |
| payload | 60/72/84/96/108 |
| Aggregation level | 1/2/4/8/16 |
| CCE-to-REG mapping | interleaved |
| REG bundle size | 6 |
| Interleaver size | 2 |
| Tx Diversity | One port precoder cycling |
| Target BLER | 1% |
| Modulation | QPSK |
| Channel coding | Polar code |

Table 12. SNR CDF table @ combination1

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -10.2546 | 20 | -0.36453 | 40 | 3.22835 | 60 | 7.45343 | 80 | 14.7788 |
| 1 | -5.53642 | 21 | -0.13111 | 41 | 3.40787 | 61 | 7.69348 | 81 | 15.6094 |
| 2 | -4.74039 | 22 | -0.00364 | 42 | 3.55026 | 62 | 7.89926 | 82 | 16.0546 |
| 3 | -4.16097 | 23 | 0.154634 | 43 | 3.71084 | 63 | 8.15225 | 83 | 16.5229 |
| 4 | -3.6152 | 24 | 0.316127 | 44 | 3.86172 | 64 | 8.39473 | 84 | 17.4413 |
| 5 | -3.33601 | 25 | 0.465781 | 45 | 4.10461 | 65 | 8.71262 | 85 | 18.0229 |
| 6 | -2.94179 | 26 | 0.687305 | 46 | 4.34906 | 66 | 8.93064 | 86 | 18.7217 |
| 7 | -2.68084 | 27 | 0.790504 | 47 | 4.46795 | 67 | 9.20377 | 87 | 19.0812 |
| 8 | -2.46978 | 28 | 1.02582 | 48 | 4.63613 | 68 | 9.46371 | 88 | 20.0456 |
| 9 | -2.23169 | 29 | 1.1432 | 49 | 4.8972 | 69 | 9.76022 | 89 | 21.0523 |
| 10 | -1.9535 | 30 | 1.33747 | 50 | 5.10365 | 70 | 10.0948 | 90 | 21.7823 |
| 11 | -1.77057 | 31 | 1.51264 | 51 | 5.35755 | 71 | 10.3657 | 91 | 22.9945 |
| 12 | -1.63586 | 32 | 1.72767 | 52 | 5.61563 | 72 | 10.7204 | 92 | 23.8569 |
| 13 | -1.5181 | 33 | 1.86436 | 53 | 5.76374 | 73 | 11.0177 | 93 | 25.4977 |
| 14 | -1.32022 | 34 | 2.04097 | 54 | 5.89568 | 74 | 11.3839 | 94 | 26.4652 |
| 15 | -1.1529 | 35 | 2.29597 | 55 | 6.14152 | 75 | 11.8047 | 95 | 28.0576 |
| 16 | -0.9492 | 36 | 2.47988 | 56 | 6.32072 | 76 | 12.2188 | 96 | 29.2536 |
| 17 | -0.81625 | 37 | 2.63678 | 57 | 6.62136 | 77 | 12.9898 | 97 | 30.9672 |
| 18 | -0.67884 | 38 | 2.84109 | 58 | 6.90661 | 78 | 13.577 | 98 | 34.7759 |
| 19 | -0.49287 | 39 | 3.05602 | 59 | 7.14573 | 79 | 14.1688 | 99 | 38.0076 |
|  |  |  |  |  |  |  |  | 100 | 49.618 |

Figure 8. SNR CDF@combination1

Table 13. SNR CDF table @ combination2

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -24.1586 | 20 | -0.86392 | 40 | 2.20429 | 60 | 7.05718 | 80 | 16.4319 |
| 1 | -14.0176 | 21 | -0.73406 | 41 | 2.34103 | 61 | 7.56236 | 81 | 16.9294 |
| 2 | -10.3672 | 22 | -0.56698 | 42 | 2.63156 | 62 | 7.76868 | 82 | 17.6157 |
| 3 | -8.10383 | 23 | -0.44329 | 43 | 2.8635 | 63 | 8.24649 | 83 | 18.0561 |
| 4 | -6.85355 | 24 | -0.28876 | 44 | 3.07717 | 64 | 8.75504 | 84 | 18.8066 |
| 5 | -5.72503 | 25 | -0.13847 | 45 | 3.27446 | 65 | 9.02377 | 85 | 19.5125 |
| 6 | -5.12569 | 26 | 0.002199 | 46 | 3.42531 | 66 | 9.39361 | 86 | 20.1874 |
| 7 | -4.43667 | 27 | 0.135032 | 47 | 3.66266 | 67 | 9.79873 | 87 | 21.2498 |
| 8 | -4.0066 | 28 | 0.298252 | 48 | 3.8878 | 68 | 10.1475 | 88 | 22.0417 |
| 9 | -3.75121 | 29 | 0.425629 | 49 | 4.2176 | 69 | 10.6638 | 89 | 23.1602 |
| 10 | -3.53802 | 30 | 0.580691 | 50 | 4.48816 | 70 | 11.2491 | 90 | 23.9371 |
| 11 | -3.23432 | 31 | 0.842471 | 51 | 4.72583 | 71 | 11.6533 | 91 | 24.9571 |
| 12 | -2.8392 | 32 | 0.995557 | 52 | 5.04191 | 72 | 12.208 | 92 | 25.9982 |
| 13 | -2.46033 | 33 | 1.17571 | 53 | 5.35037 | 73 | 12.7977 | 93 | 26.9958 |
| 14 | -2.15962 | 34 | 1.27262 | 54 | 5.54458 | 74 | 13.2294 | 94 | 29.1328 |
| 15 | -1.87732 | 35 | 1.45418 | 55 | 5.84028 | 75 | 13.7596 | 95 | 30.5847 |
| 16 | -1.74311 | 36 | 1.66068 | 56 | 6.0695 | 76 | 14.1825 | 96 | 32.435 |
| 17 | -1.53056 | 37 | 1.79714 | 57 | 6.22072 | 77 | 14.6769 | 97 | 34.1081 |
| 18 | -1.25115 | 38 | 1.91417 | 58 | 6.47333 | 78 | 15.3013 | 98 | 36.8946 |
| 19 | -1.07078 | 39 | 2.06022 | 59 | 6.78597 | 79 | 15.7433 | 99 | 41.3785 |
|  |  |  |  |  |  |  |  | 100 | 53.7262 |

Figure 9. SNR CDF@combination2

Table 14. SNR CDF table @ combination3

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -7.26419 | 20 | -0.96545 | 40 | 1.7748 | 60 | 5.56674 | 80 | 12.4162 |
| 1 | -5.21172 | 21 | -0.88688 | 41 | 1.88603 | 61 | 5.92351 | 81 | 12.9676 |
| 2 | -4.80452 | 22 | -0.75159 | 42 | 2.03053 | 62 | 6.15983 | 82 | 13.6152 |
| 3 | -4.38764 | 23 | -0.61685 | 43 | 2.17083 | 63 | 6.57576 | 83 | 14.146 |
| 4 | -3.95765 | 24 | -0.47036 | 44 | 2.32176 | 64 | 6.75301 | 84 | 15.442 |
| 5 | -3.61514 | 25 | -0.33375 | 45 | 2.50363 | 65 | 7.11825 | 85 | 15.9526 |
| 6 | -3.35388 | 26 | -0.17279 | 46 | 2.64933 | 66 | 7.3838 | 86 | 16.6827 |
| 7 | -3.18269 | 27 | -0.0451 | 47 | 2.81553 | 67 | 7.65885 | 87 | 17.2967 |
| 8 | -2.91377 | 28 | 0.093423 | 48 | 3.01089 | 68 | 7.98236 | 88 | 18.028 |
| 9 | -2.74461 | 29 | 0.248865 | 49 | 3.24202 | 69 | 8.24703 | 89 | 18.9256 |
| 10 | -2.60492 | 30 | 0.438283 | 50 | 3.40196 | 70 | 8.50787 | 90 | 19.8244 |
| 11 | -2.36578 | 31 | 0.576086 | 51 | 3.61096 | 71 | 8.80913 | 91 | 21.1362 |
| 12 | -2.28163 | 32 | 0.680843 | 52 | 3.76803 | 72 | 9.2037 | 92 | 21.8695 |
| 13 | -2.11585 | 33 | 0.839814 | 53 | 3.9496 | 73 | 9.48494 | 93 | 22.4685 |
| 14 | -1.89804 | 34 | 0.917518 | 54 | 4.14849 | 74 | 9.77827 | 94 | 23.6129 |
| 15 | -1.66357 | 35 | 1.06479 | 55 | 4.40149 | 75 | 10.4052 | 95 | 25.2505 |
| 16 | -1.53982 | 36 | 1.23596 | 56 | 4.63591 | 76 | 10.8369 | 96 | 26.9431 |
| 17 | -1.36078 | 37 | 1.36786 | 57 | 4.81894 | 77 | 11.2731 | 97 | 28.5205 |
| 18 | -1.24942 | 38 | 1.5008 | 58 | 4.97215 | 78 | 11.5699 | 98 | 30.1764 |
| 19 | -1.07945 | 39 | 1.66537 | 59 | 5.2814 | 79 | 12.0173 | 99 | 32.1415 |
|  |  |  |  |  |  |  |  | 100 | 36.8842 |

Figure 10. SNR CDF@combination3

Table 15. SNR CDF table @ combination4

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -22.0262 | 20 | -1.28806 | 40 | 1.99085 | 60 | 6.88581 | 80 | 16.481 |
| 1 | -14.7467 | 21 | -1.13791 | 41 | 2.21929 | 61 | 7.34555 | 81 | 17.2132 |
| 2 | -12.4635 | 22 | -0.88186 | 42 | 2.43963 | 62 | 7.73864 | 82 | 18.1 |
| 3 | -10.6342 | 23 | -0.64076 | 43 | 2.7346 | 63 | 8.08924 | 83 | 18.8629 |
| 4 | -7.57208 | 24 | -0.47889 | 44 | 2.93743 | 64 | 8.42503 | 84 | 19.7797 |
| 5 | -6.36859 | 25 | -0.35376 | 45 | 3.16592 | 65 | 8.79445 | 85 | 20.6463 |
| 6 | -5.56474 | 26 | -0.25832 | 46 | 3.39976 | 66 | 9.25276 | 86 | 21.2617 |
| 7 | -4.98859 | 27 | -0.09675 | 47 | 3.57166 | 67 | 9.6758 | 87 | 22.2406 |
| 8 | -4.35556 | 28 | 0.021877 | 48 | 3.75616 | 68 | 10.0586 | 88 | 23.2867 |
| 9 | -3.87205 | 29 | 0.256459 | 49 | 3.92262 | 69 | 10.4647 | 89 | 23.97 |
| 10 | -3.52634 | 30 | 0.429175 | 50 | 4.18057 | 70 | 10.9435 | 90 | 24.9883 |
| 11 | -3.25525 | 31 | 0.522494 | 51 | 4.4416 | 71 | 11.609 | 91 | 25.7142 |
| 12 | -2.96183 | 32 | 0.708476 | 52 | 4.60271 | 72 | 12.3138 | 92 | 27.0815 |
| 13 | -2.67992 | 33 | 0.898988 | 53 | 5.14542 | 73 | 12.9802 | 93 | 28.4339 |
| 14 | -2.429 | 34 | 1.02653 | 54 | 5.38973 | 74 | 13.3398 | 94 | 29.6644 |
| 15 | -2.24673 | 35 | 1.22581 | 55 | 5.59945 | 75 | 13.8092 | 95 | 31.6005 |
| 16 | -2.08099 | 36 | 1.3598 | 56 | 5.90199 | 76 | 14.4038 | 96 | 33.7328 |
| 17 | -1.87038 | 37 | 1.48601 | 57 | 6.09438 | 77 | 15.0527 | 97 | 36.3763 |
| 18 | -1.66034 | 38 | 1.63309 | 58 | 6.44936 | 78 | 15.6939 | 98 | 39.0046 |
| 19 | -1.49349 | 39 | 1.81223 | 59 | 6.60945 | 79 | 16.0457 | 99 | 41.9799 |
|  |  |  |  |  |  |  |  | 100 | 53.4473 |

Figure 11. SNR CDF@combination4