3GPP TSG RAN WG1 #105-e			R1-2104447

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.13.3

Source: 	Spreadtrum Communications

Title: 	Discussion on efficient activation/de-activation mechanism for SCells in NR CA

Document for:	Discussion and decision

Introduction

In WID of further Multi-RAT Dual-Connectivity enhancements [1], one objective is efficient activation/de-activation mechanism for SCells in NR CA. The following agreement and wording assumption were achieved during RAN1 104e meeting [2],[3].

Agreement

For efficient activation of SCells

- Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including timeline design for receiving temporary RS

Note: Separate from the support of Option 1a, it is up to RAN4 whether or not to consider an activation time enhancement for Option 2 without requiring further RAN1 work

- Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding Rel-15/16 A-TRS(s)

Send an LS to RAN4. The LS is endorsed in R1-2104110.

This contribution provides some considerations on efficient activation/de-activation mechanism for SCells.

Discussion

NR Rel-15 SCell activation/deactivation timeline is specified in [4]. In general, when receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where THARQ is the timing between PDSCH and its HARQ-ACK; Tactivation_time is the SCell activation delay; TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Triggering command

For the triggering command of efficient SCell activation, Option 1a was agreed to be supported. In order to elaborate option 1a and compare their pros and cons, Option 1a is divided into 3 potential options.

- Option 1a-1: A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers

- Option 1a-2: A PDSCH TB, e.g. containing one new MAC-CE for both triggers

- Option 1a-3: Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well

Regarding these potential methods of single triggering signalling, we gives some considerations in the following Table 1.

First of all, Option 1a-1 and 1a-2 are not good choices. New MAC-CE should be defined in RAN2 with insignificant advantage compared with Option 1a-3. Because 1a-1/1a-2 can provide more TRS configuration or information for a specified SCell. But considering this TRS only used for AGC adjustment and fine time/frequency tracking, the TRS pattern is more stable. Since the TRS pattern depends on which frequency range the SCell is and whether it is a known cell or unknown cell. Thus, a TRS configuration can be enough for expediting SCell activation. Multiple TRS resources or configurations do not needed for efficient SCell activation. Therefore extra triggering or information for TRS in MAC-CE do not have obvious benefit.

Thus, we suggest Option 1a-3 can be supported for further study.

Table 1: Potential methods of triggering signalling

| Potential Options | SCell activation command and TRS triggering command | Pros | Cons |
| --- | --- | --- | --- |
| Option 1a-1: A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers | Separate MAC-CE in one PDSCH TB for SCell activation and TRS triggering | Reuse the Rel-15/16 SCell activation MAC-CE, | One new MAC-CE for TRS trigger |
| Option 1a-2: A PDSCH TB, e.g. containing one new MAC-CE for both triggers | one MAC-CE in one PDSCH TB for both and TRS triggering | NA | One new MAC-CE for SCell activation and TRS triggering. |
| Option 1a-3: Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well | Rel-15/16 SCell activation MAC-CE for SCell activation and implicitly triggered for TRS. | Reuse the Rel-15/16 SCell activation MAC-CE No additional triggering mechanism for TRS. | Only one TRS can be implicitly triggered |

- For SCell activation command, at least Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well, and a DCI for both triggers can be supported.

TRS pattern

From the reply LS [5] on temporary RS for efficient SCell activation, the following Table 2 is a summary of the cases. It is clear that only one case can reuse Rel-15 TRS pattern, which is the SCell to be activated belongs to FR1 is known cell and SCell measurement cycle is equal to or smaller than 160ms. When SCell measurement cycle is larger than 160ms, 2 bursts are needed, one for AGC and the other for time/frequency tracking. One issue is there is a gap between these two bursts, so these two bursts may not be continuous. Similar as this case, the other scenarios also need some combined TRS patterns.

Regarding 2 bursts of TRS, it needs further study, like if there is a gap between these two bursts, how to guarantee the gap period. One method is reuse the Rel-15 TRS resource configuration and two separate triggering offsets. But it is can be decided after RAN4 decision.

Considering the other undecided TRS pattern, a good forward compatibility configuration structure which use the Rel-15 TRS resource as the starting point can be considered. In this TRS configuration structure, number of bursts or symbols together with gap length can be flexibly configured. Of course the detailed structure can be decided in RAN2.

Table 2: Temporary RS pattern for efficient SCell activation

| Cases |  |  | RAN4 conclusion or status | TRS pattern |
| --- | --- | --- | --- | --- |
| FR1 | known | MC <= 160ms | 1 burst for AGC and T/F tracking | Reuse Rel-15 TRS pattern |
|  |  | MC > 160ms | 1 burst for AGC, and separate 1 burst for T/F tracking Gaps between RS symbols for AGC and T/F tracking: - Option 1: 2 slots - Option 2: 2 ms | Cannot reuse Rel-15 TRS pattern. |
|  | unknown | Intra-band continuous CA | 1 burst (4 samples) for AGC and 1 burst (4 samples) for T/F tracking |  |
|  |  | Other cases | TBD |  |
| FR2 | known | at least one active serving cell on that FR2 band | 1 burst (4 samples) can be used for T/F tracking |  |
|  | unknown |  |  |  |
|  | known | no active serving cell on that FR2 band | 1 burst (4 samples) can be used for T tracking FFS: number of symbols |  |
|  | unknown | no active serving cell on that FR2 band | Temporary RS can not be used for AGC |  |

- Further study the combined of TRS patterns used for SCell activation.

Timeline design for receiving temporary RS

If MAC-CE is used as triggering command, the point after THARQ (timing between PDSCH and its HARQ-ACK) can be set to the earliest temporary RS transmission occasion. So one offset can be enough for temporary RS and configured by RRC singling, which starting point is the HARQ-ACK feedback slot of triggering command. Such as the offset can be equal or later than [3ms + 1].

- When MAC-CE is used as triggering command, the offset between the SCell activation triggering and temporary RS can be configured by RRC singling, and starting point of the offset is the HARQ-ACK feedback slot of triggering command.

Conclusion

In this contribution, we made the following proposals.

- For SCell activation command, at least Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well, and a DCI for both triggers can be supported.

- Further study the combined of TRS patterns used for SCell activation.

- When MAC-CE is used as triggering command, the offset between the SCell activation triggering and temporary RS can be configured by RRC singling, and starting point of the offset is the HARQ-ACK feedback slot of triggering command.

References

RP-193260, 3GPP RAN, New WID on NR Dynamic spectrum sharing (DSS)

RAN1 104b-e Chairman’s Notes

R1-2103886	Summary#1 of efficient SCell activation/de-activation mechanism of NR CA	Moderator (Huawei)

3GPP TS 38.133: NR; Requirements for support of radio resource management

R1-2104170	Reply LS on temporary RS for efficient SCell activation in NR CA