3GPP TSG-RAN WG1 Meeting #105 -e	Tdoc R1-2104463

Online, 19th May– 27th May 2021

Agenda Item:	8.2.6

Source:	Ericsson

Title:	Channel access mechanisms

Document for:	Discussion, Decision

# Table of Contents

Table of Contents	1

1	Introduction	1

2	Baseline listen before talk (LBT) design	2

2.1	Energy detection threshold enhancement	2

2.1.1	Pout in the EDT equation	3

2.1.2	Channel bandwidth and LBT bandwidth	4

2.1.2.1	Definitions in EN 302 567	4

2.1.2.2	Definitions in 3GPP	4

2.1.2.3	Definition of LBT bandwidth	6

2.1.3	Multi-channel/carrier LBT operation	7

2.2	Short control signaling transmissions	8

3	Further enhancements to LBT mechanisms	9

3.1	Omni-directional LBT vs Directional LBT	10

3.1.1         Multi-beam COT	12

3.2	Receiver assisted channel access and interference management	13

3.2.1	Receiver assistance mechanisms inherently integrated with the data transmission procedure	14

3.2.2	Receiver assistance mechanisms decoupled from data transmission procedure	16

3.3	 COT sharing aspects	18

3.3.1	CAT2 LBT	18

3.3.2	 Gaps in Shared COT	19

3.4	CAPC and CWS enhancements	20

3.5	 Sensing slot structures	21

3.6	No LBT/LBT mode switching	23

4	Conclusion	24

5	References	28

# 1	Introduction

A revised WI [1] was approved in RAN #90-e to study and extend NR support in the frequency range of 52.6 GHz to 71 GHz [2]. The objectives for the WI, according to the outcome of the study item and leveraging FR2 design to the extent possible, are to extend NR operation up to 71 GHz considering both licensed and unlicensed operation. The WI description included the following updates:

Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to license-exempt spectrum for frequencies between 52.6 GHz and 71 GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

    - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

    - Study, and if needed specify, energy detection threshold enhancement

In this contribution, we first outline the changes to baseline LBT design in ETSI HS EN 302 567 v2.2.0 and the corresponding impact to the work in 3GPP. We then examine further enhancements to LBT mechanisms. Finally, we provide conclusions on the potential enhancement beneficial to NR operation in 52.6 GHz to 71 GHz.

# 2	Baseline listen before talk (LBT) design

## 2.1	Energy detection threshold enhancement

RAN1#104-e Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

The following agreement was made in RAN1#104-e[10] regarding the baseline Energy detection threshold according to EN 302 567 v2.2.0 [4].

RAN1#104b-e Working assumption:

For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

Furthermore, the following was agreed as a working assumption in RAN1#104b-e[16] regarding Pout.

The next sub-sections deal with the following FFSs in the above agreement.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS: definition of Operating Channel BW

- FFS: EDT when the COT has time varying transmission beams and varying EIRPs.

### 2.1.1	Pout in the EDT equation

The Pout and Pmax used in the equation is defined as follows in EN 302 567 v2.2.0 [4]

EN 302 567 Clause 4.2.5.3 
The LBT mechanism is as follows:

1) Before a single transmission or a burst of transmissions on an Operating Channel, the equipment that initiates

transmission shall perform a Clear Channel Assessment (CCA) Check in the Operating Channel.

…

7)The energy detection threshold for the CCA Check shall be -80 dBm + 10 × log10 (Operating Channel

Bandwidth (in MHz)) + 10 × log10 (Pmax / Pout) (Pmax and Pout in W EIRP) where Pout is the RF output

power (EIRP) and Pmax is the RF output power limit defined in clause 4.2.2.1.”

EN 302 567 Clause 4.2.2.1

4.2.2.1	Definition

The RF output power is the mean equivalent isotropically radiated power (EIRP) for the equipment during a transmission burst.”

4.2.2.2	Limit

The maximum RF output power is applicable to the system as a whole when operated at the highest stated power level. For a smart antenna system, the limit applies to the configuration that results in the highest EIRP. In case of multiple (adjacent or non-adjacent) channels the total RF output power of all channels shall be less than or equal to the limits in table 3.”

It is also worthy to note that the LBT mechanism is performed by the transmitter before a transmission or a burst of transmissions that could last a duration of 5ms (MCOT) according to the regulation. However, 5ms is a long time in the 60 GHz regime and the products in this band may transmit multiple bursts within a COT. This is evident since the use cases in the band involves serving multiple users with different power requirements, beam directions, link adaptation and/or different QoS requirements. In addition, most devices using the band may configure a fixed transmit power for a transmission burst (for a certain beam direction or a user). The devices then can estimate the EIRP per transmission burst depending on the beams in the burst.

Poutb3

eCCA

MCOT

Time

Tx EIRP

Transmission burst #1

Transmission burst #2

Transmission burst #3

Poutb1

Poutb2

gap

gap

From the above, it could be noted that while RF output power in clause 4.2.2.1 is defined as the mean EIRP during a transmission burst, there may be multiple transmission bursts within a COT with varying EIRPs, transmission beams. A typical example for such a COT is depicted in Figure 1 below.

Figure 1:	A typical COT with multiple transmission bursts using varying EIRPs

Therefore, from our understanding, the Pout used in the EDT equation should correspond to the highest mean EIRP in the COT if only one eCCA is performed for such multiple transmission bursts. Using the highest mean EIRP in the EDT equation results in the most conservative EDT (lowest EDT), which provides fair coexistence.

Confirm the working assumption that Pout corresponds to the maximum of the mean output power EIRPs of the transmissions or transmission bursts in a COT that may contain varying transmission beams and EIRPs.

The above principle is also applicable, for example, if omni-directional sensing is used to perform LBT followed by directional transmissions. In such a case, the Pout generated by the directional transmission needs to be used in the equation to determine EDT (to perform omnidirectional sensing), and not the EIRP assuming omnidirectional transmission or omnidirectional sensing. Therefore, regardless of the sensing gain, it is the transmission direction and the corresponding transmission gain that is reflected in Pout used in the EDT equation mandated by the regulations.

As evident from the above discussion, ED threshold defined in EN 302 567 is a function of the transmission beam’s EIRP, the maximum EIRP and the operating channel BW. If it is agreed to include sensing gain in the determination of EDT in 3GPP, e.g., increase the ED threshold based on the sensing BF gain, the resulting ED threshold would be higher than the one in HS, thus is in violation of the HS’s requirements. It is also noted that the HS does not preclude to use a stricter, lower EDT than what is required. Therefore, decreasing the ED threshold based on sensing BF gain is allowed in HS and could be up to the implementation if the network see the benefit of lower the ED threshold based on sensing BF gain.

ED threshold defined in EN 302 567 v2.2.0 is a function of the transmission’s EIRP Pout, which includes the transmission beamforming gain. It does not include the sensing beamforming gain.

Further adjustment on ED threshold based on the transmission and sensing beamforming gains could be up to implementation while not violating EDT requirements as per regulations.

### 2.1.2	Channel bandwidth and LBT bandwidth

#### 2.1.2.1	Definitions in EN 302 567

In the previous sub-section, we highlighted the changes to the EDT threshold in EN 302 567 that RAN1 needs to incorporate regardless of the discussions surrounding LBT bandwidth and the EDT threshold. EDT thresholds scales with the LBT BW but it is still not clear how the LBT BW (Operating channel BW) and the channel bandwidths are related. There are also multiple terms for the same word in different standard bodies (ETSI, IEEE 802.11, 3GPP RAN1 etc.) which may initiate unnecessary ambiguity and can potentially delay progress in the RAN1 WI.

For this purpose, we want to highlight the relationship between various terms pertaining to a “channel” in different domains and connect it to the requirements in the HS EN 302 567. A presumption of conformity is provided if a radio device complies with the requirements of HS EN 302 567. Consequently, a manufacturer may put such compliant product on the European market. In its “Declaration of conformity” a manufacturer may declare one or more supported channel bandwidths. These supported channel bandwidths are the nominal channel bandwidths (NBWs) of the equipment. A device may declare multiple nominal channel bandwidths. It can be noted that the nominal channel bandwidths in ETSI domain corresponds to the carrier bandwidth in RAN1/RAN2 and RAN4 domain.

In addition, for every declared channel bandwidth, the device is required to support at least one mode of transmission where the transmissions occupy at least 70% of the declared nominal channel bandwidth. Nominal channel BW and a transmission mode capable of achieving 70% of the NBW is defined for the purpose of defining out-of-band emissions and meeting the spectral emission mask. This has no impact on the LBT bandwidth or the bandwidth on which a device intends to transmit. That is, any declared channel bandwidth is allowed and the actual transmission bandwidth at any point in time may be different from the declared channel bandwidth. Therefore, EN 302 567 provides flexibility for any technology to define their own operating channel bandwidths i.e. LBT bandwidths (and the corresponding EDT thresholds) thus making it technology neutral.

In EN 302 567, the nominal channel bandwidth and at least one transmission mode with occupied channel BW 70% of NBW is defined for spurious out-of-band emissions and not for LBT purposes.

The relationship between the LBT bandwidth and the channel bandwidth is not specified in EN 302 567 for the sake of technology-neutrality and flexibility.

#### 2.1.2.2	Definitions in 3GPP

RAN4

The UE channel bandwidth in RAN4 is defined with respect to a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth. The relationship between the channel bandwidth, the guard band and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1 in TS 38.101-1 [5].

Figure 2: Relationship between channel bandwidth, transmission bandwidth configuration and transmission bandwidth

As shown in the Figure 2, once a UE is configured/scheduled with active resource blocks for a transmission burst, the gNB/UE will transmit within a part of the full transmission bandwidth configuration for a specific channel occupancy time (COT) (a slot, for example). In EN 302 567 the maximum COT (MCOT) allowed for a transmission burst is 5 ms.

RAN1

TS 37.213 defines the following:

A channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum.

We observe that the definition of channel in 37.213 and RAN4 differs in the sense that the channel as defined in 37.213 is the LBT BW and the channel as defined in RAN4 corresponds to how the term carrier is used in 37.213.

#### 2.1.2.3	Definition of LBT bandwidth

RAN1#104b-e Agreement:

For LBT for single carrier transmission, continue down selection between

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth

For LBT for multi-carrier transmission in intra-band CA, continue down selection between

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

The following agreement was made in RAN1#104b-e[16] to further down-select between different options for LBT bandwidth:

There is a need to first agree on the principle of LBT over a single carrier before discussing the multi-channel case.

For the SC case, as observed in Section 2.1.2.1, EN 302 567 does not mandate a specific LBT BW or a relation between the LBT BW and the channel bandwidth. In addition, a device can use any of the declared channel bandwidths and operating channel BWs. Thus, from an EN 302 567 perspective at least Alt SC1 alternatives above are already allowed. RAN1 does not define the LBT BW even in Rel-16. Instead 37.213 uses the following generic definition as discussed in Section 2.2.2:

37.213: A channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum.

We further observe that the generic channel definition in 37.213 covers both the case in 5/6 GHz where the LBT BW is equal to the RAN4 channel BW (i.e. carrier BW) and the case where the LBT BW is smaller than the RAN4 channel BW.

Comparing the channel definition in 37.213 with the definition of operating channel in the ETSI BRAN 60 GHz harmonized standard, EN 302 567:

EN 302 567: operating channel: channel on which the RLAN equipment has started the Adaptivity mechanism to start transmissions

We conclude that the LBT BW definition in 37.213 is already compliant with EN 302 567. The only modifications needed are to adapt the ED threshold scaling rules for this frequency range.

Operating channel BW defined in EN 302 567 is the LBT BW in RAN1 which is already defined in 37.213 as a “channel”

The above definition in 37.213 and EN 302 567 at least covers Alt SC1 for single carrier transmissions. Furthermore, Alt SC3 in our opinion poses an artificial restriction on the LBT Bandwidth and forces the channel BW to be exact multiples of the LBT BW which is not necessary because of the lack of a nominal channel BW requirement in the 60 GHz regulations. This is different from the 5/6 GHz bands where the LBT BW is specified to be 20 MHz in the harmonized standards with supporting channel raster that the technologies must follow. It is to be noted that there is no fixed channelization and importantly, no channel raster defined in the EN 302 567 v2.2.0. It is not clear to us how Alt SC3 and corresponding CA5 can be supported when the channels are not aligned. It also poses another issue in determining guard bands, if an LBT BW unit is defined as per SC3, which is complex. In addition, the behavior of the node when the LBT in a LBT BW unit fails is also not discussed. For example, if 100 MHz is defined as the LBT BW unit, for a SC of 1.6 GHz channel bandwidth, there are a total of 16 logical LBTs. The behavior of the node when LBT fails in some of these LBT units are not discussed nor specified in ETSI TC BRAN.

Alt SC3/CA5 poses an artificial restriction to insert guard bands at the end of the LBT units

For SC3, LBT failure for a node within a LBT unit is complex and not discussed.

Therefore, we support Alt SC1 for single carrier case and do not support SC3. It can be left for implementation for the reasons outlined above.

Consequently, for the CA case, we think it is pre-mature to discuss LBT in multi-carrier before the single carrier case is agreed. However, if we follow the HS EN 302 567 and 37.213, Alt CA1 is the preferred option from the above agreement and is already possible within the scope of the current regulation. The other options are not precluded by implementation. In the next sub-section, we further discuss the issues of multi-channel LBT operation.

Definitions in EN 302 567 and TS 37.213 at least covers Alt SC1.

Support Alt SC1/Alt CA1 for LBT in single carrier and multi-carrier operation. Other options are not precluded by implementation.

### 2.1.3	Multi-channel/carrier LBT operation

Prior to discussing the agreements in RAN1, let us discuss the multi-carrier LBT operation in 5 GHz specifications.

In the first method, LBT was carried out per carrier. In the second method, a channel bonding scheme was employed where LBT was performed on the primary channel/carrier and then a short CAT2 LBT was performed on the secondary channels.

This was captured in 37.213 as follows.

“An eNB/gNB can access multiple channels on which transmission(s) are performed, according to one of the Type A or Type B procedures”

Agreement:

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

In RAN1 104-e, the following was agreed:

Type A multi-channel/carrier case allows multi-carrier LBT by performing channel access independently on each channel/carrier, which is compliant with the HS EN 302 567. We think that this could be re-used for the 60 GHz multi-channel/carrier LBT.

Support Alt1 in the agreement that allows only Type A multi-channel access from 37.213.

Type B multi-channel case corresponds to the channel bonding case as described above.  ETSI BRAN neither specifies CAT2 LBT nor channel bonding based multi-channel access in the HS EN 302 567.

The Type B channel access is further complicated in 60 GHz, as there are no nominal channel BW units that all the devices sharing the band would use. For example, this was 20 MHz in the 5 GHz domain. According to the regulation, a manufacturer can state through declaration of conformance the bandwidths they support. By adopting Type B multi-channel access, we are forcing an unnecessary restriction on what these BWs need to be and where the sensing needs to be done. Moreover, the channel bonding scheme worked well in 5 GHz because it was based on following the same channelization and the same unit of 20 MHz for LBT.

We should not consider Type B multi-channel access from 37.213 in this WI for three reasons. Firstly, there is no fixed channelization or nominal channel BW in 60 GHz. Secondly, any channel bandwidth, including multiple carriers with different carrier BWs for each carrier, is allowed according to the regulation if it is declared by the manufacturer. Thirdly, it is not straight-forward how to select the primary carrier for type B channel access as the HS EN 302 567 does not specify CAT2 LBT nor multi-channel/carrier operation. Consequently, a device may randomly choose a carrier with narrower bandwidth as its primary channel and perform CAT2 LBT on the wider bandwidth secondary channels. Therefore, there is too much specification effort in 3GPP RAN1 to allow Type B channel access.

Alternatively, ETSI HS EN 302 567 adequately captures the LBT BW to be used as the operating channel BW. Therefore, it is enough to define LBT bandwidth for a single carrier and allow Type A multi-channel option from 37.213 for multi-carrier operations.

ETSI regulation for 60 GHz bands do not support Type B multi-channel access.

Do not support Type B multi-channel access for NR operation in 52.6 GHz to 71 GHz.

## 2.2	Short control signaling transmissions

The use of Short Control Signalling Transmissions shall be constrained as follows:

within an observation period of 100 ms,

the total duration of the equipment's Short Control Signalling Transmissions shall be less than 10 ms within said observation period.

Recently, ETSI TC BRAN added a separate clause for Short control signaling transmissions to HS EN 302 567. The details of short control signaling are described below.

Unlike EN 301 893 (5 GHz) and EN 303 687 (6 GHz), EN 302 567 specifies no limitation on the number of control transmissions but only on the total duration of short control transmissions during an observation period. From an HS compliance perspective, a margin of up to 10% control frame transmissions without performing an LBT is allowed. This permission should be taken into consideration when designing any LBT related changes for control channels/signals.

In HS EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period.

In RAN1#104b-e [16], there were two agreements on SCS transmissions.

Agreement:

- Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH.

  - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule

  - FFS: Whether this can be applied to all supported SCS or specific SCS.

  - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH

  - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

- FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc

Agreement:

For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission.

- RMSI PDCCH and RMSI PDSCH

- Other broadcast PDSCH

- PDSCH without user-plane data

- PDCCH

- CSI-RS

- PRS

- Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst)

Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval.

FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

It has been agreed in the last meeting RAN1#104b-e that Discovery Burst (including SS/PBCH), defined as in Rel-16 NR, is supported for NR Rel-17[14]. Therefore, we think that it is reasonable to extend the short control signaling transmissions exemption for SS/PBCH to the Discovery Burst.

Support extending the Short control signalling transmissions exemption to Discovery Burst.

Moreover, based on EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period. Therefore, we have the following proposal.

Consistent with EN 302 567, a node can access the channel without LBT for control signal/channel transmissions, the total duration of which shall not exceed 10 ms within an observation period of 100ms. The following signals/channels shall be classified as short control signaling transmissions:

  - msg1 and msg3 for the 4 step RACH and MsgA for the 2-step RACH

  - FFS: Other control transmissions not multiplexed with user data (subject to gNB configuration)

# 3	Further enhancements to LBT mechanisms

LBT has been used as a medium access mechanism for license-exempt spectrum in lower frequency ranges, e.g., 2.4 GHz and 5 GHz bands. However, since the millimeter wave frequency range is characterized by high radio propagation loss that require the use of directional transmission and reception via large antenna arrays, LBT does not operate satisfactory. Interference conditions in the 60 GHz band are considerably different compared to lower frequency bands. The following aspects dominate in the 60 GHz band:

- The transmission power limitation imposed by spectrum regulations and the attenuation characteristics around the 60 GHz range prevents radio signals to cause strong interference to other nodes located tens of meters away.

- In practical deployments, the signal from other transmitters are often blocked by objects further reducing the interference.

- Highly directional signal transmission is less likely to interfere other nodes even in the close vicinity, except for the nodes that lie directly in the transmission beam coverage. The probability of interference is further reduced for the nodes that employ directional reception.

- Highly directional transmission also makes it very difficult for a transmitter to correctly detect the interference level at intended receiver, and hence the fundamental assumption in classical LBT for interference avoidance no longer holds. Moreover, we demonstrated in [6] that the interference level at both transmitter and receiver is well below the LBT threshold, regardless of any difference between the sensed level at either node.

Therefore, the effectiveness and necessity of employing LBT to mitigate interference in the 60 GHz band is questionable.

The effectiveness of LBT itself as medium access mechanism for co-existence in unlicensed spectrum in 60 GHz band is questionable. Therefore, any further enhancement on LBT baseline from the HS need to be justified both on the performance gain and the required complexity.

## 3.1	Omni-directional LBT vs Directional LBT

In HS EN 302 567 there is no mention or reference to directional LBT. In IEEE 802.11ad and IEEE 802.11ay the common understanding is that a device performs omni-directional LBT or quasi-omni-directional LBT.  This is simpler for beam switching or sector sweeping. We propose to keep the same for 3GPP spec as well.

Common understanding in ETSI and IEEE 802.11ad and IEEE 802.11ay specs are omni-directional LBT or quasi-omnidirectional LBT

Directional LBT where the transmitter listens to the channel only in the direction(s) that it intends to transmit, has been discussed in the 60 GHz unlicensed SI. One common understanding is that directional LBT could increase the spatial reuse by reducing the exposed node problem. However, the transmitter already rarely defers against interferences due to high directional beamforming and pathloss. Thus, it seems to be unnecessary to optimize LBT in 60 GHz band by enabling directional LBT. To confirm that hypothesis, the performance of directional LBT compared to omni-directional LBT for different scenarios, i.e., indoor scenarios A, B and C were evaluated in [6]. The results revealed no benefits for using directional LBT as compared to operating an omni-directional LBT. Furthermore, operating without LBT still has the best throughput performance.

Simulation studies in general indicate no significant gain from using directional LBT.

Based on the above observations, we have the following proposal

Support omni-directional LBT or quasi-omni-directional LBT as the baseline LBT procedure for 60 GHz band.

In RAN1-104b-e[16], there was support from companies to study directional LBT. It was discussed whether and how to define the relationship between sensing and transmission beams, but we did not arrive at a consensus. The following proposal was discussed in detail:

##### Proposal 2.9.2-1 (closed):

- 3GPP specification defines the relative relationship between all applicable sensing beams and the transmission beam, at least sensing beam “covers” the transmission beam,

  - Alt 1. To define “cover”, the angle included in the [3]dB beamwidth of the transmission beam is included in the [3]dB beamwidth of the sensing beam

  - Alt 2. Extending the beam correspondence framework and/or QCL/TCI framework to define “cover”

  - Alt 3. Leave RAN4 to define cover

The main motivation to introduce the relationship between sensing and transmission beams is the concern that without this relationship the transmitter could sense in a direction which is different from the transmit direction resulting in miss detection of the interference in the intended transmit direction. However, in our view, it is intuitive that the sensing beam should be in the direction of the transmission beam. A device will be tested with this assumption in HS EN 302 567 and hence there is no need to define this relationship. In addition, it is worthy to note that the directional sensing/ LBT itself is not precluded by the regulations. If the sensing beam is the same as the directional transmission beam, the device can perform LBT using the beam and use the intended EIRP for the transmission to calculate EDT. If the transmission is performed in a direction different to sensing it will fail the tests in EN 302 567, where the interference signal shall be aligned to the direction of the transmission as described in Clause 5.3.8.2 of the HS.

Directional LBT is currently not precluded in the existing regulations. EN 302 567¨s tests intrinsically ensure sensing beam is in the direction of the transmission beam for devices equipped with directional antenna systems.

Secondly, there is no notion of directions/beams for sensing described in the TS 37.213 and it will take a lot of specification effort to introduce them. Furthermore, it becomes highly complex to specify various beam directions in RAN1.

Notion of “beams” for sensing/LBT is non-existent in 37.213.

Regarding Alt 1, it is not clear to us how much of this alternative could be specified in RAN1 as measuring 3 dB beamwidths of the sensing beam and transmission beams seems to be more relevant to the requirements and testing in RAN4. From our view, if this alternative is to be supported, some descriptions on the definition of 3dB beamwidth covering could be considered in RAN1 (e.g., include some descriptions in 37.213), then leave it to RAN4 to develop the necessary requirements and testing. Therefore, we think that  at a high level Alt1 and Alt 3 are similar and could be considered together.

Alt1 and Alt3 have more RAN4 spec impact and can be considered together under a single alternative.

On the other hand, Alt2 would require a lot of specification efforts and has several issues as follows.

- Alt2 relies on beam correspondence framework. However, at the UE side, beam correspondence is not a mandatory feature, i.e., there could be some UEs, especially cheap UEs in unlicensed band, that could not meet the beam correspondence requirement at all, depending on the capability. Furthermore, the gNB does not have a beam correspondence requirement and hence will not be tested in RAN4. Therefore, it is not clear how this alternative could work if the beam correspondence is not supported at the UEs and/or gNB unless Rel-17 NR support beam correspondence as mandatory feature at both the UEs and gNB.

- Even if the UEs and gNB could support beam correspondences, it is still not clear how to use this feature for scenarios with multiple beams. The current beam correspondence is defined per DL-UL beam pair (one-to-one relationship), it could not be used to define the relationship “cover” between a single sensing beam and multiple transmission beams (one-to-many relationship).

- The current beam correspondence testing requirement in RAN4 is very loose. It could result in scenarios that sensing beam based on beam correspondence requirement is not well aligned or sufficiently “cover” the transmission beam. Therefore, it is not guaranteed that a device using directional LBT based on beam correspondence framework could pass the regulatory test. Omni LBT or quasi-omni LBT would be a safer option on this aspect.

- It is not clear how QCL/TCI framework can be used to indicate “cover”. In current NR, the QCL/TCI framework is used to define the relationship between two DL transmissions (i.e., between two transmission beams). Therefore, it is not clear how to use it to define the relationship between a sensing beam and a transmission beam (i.e., between a received beam and a transmission beam).

Based on the above observations we propose the following.

Do not support Alt.2 on extending the beam correspondence framework and/or QCL/TCI framework to define “cover”.

“A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. The basic unit for sensing is a sensing slot with a duration Tsl=5us. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration, and determines that the detected power in the intended transmission directions for at least X us within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy.”

Moreover, we think it is reasonable to leave it to RAN4 to define requirements to ensure the sensing beam includes the intended transmission direction. From RAN1 point of view this can be specified to capture this requirement as follows, or consider the 3 dB beamwidth description (based on proposal from Alt.1). The proposed highlighted text below modifies the existing text in 37.213 and cater it for 60 GHz regime. We could agree to have this or similar text in RAN1 while leaving it to RAN4 for the test.

### 3.1.1 	Multi-beam COT

RAN 1- 104-e Agreements:

Agreement:

For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both)

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Agreement:

Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover"

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

Notwithstanding the discussions and potential proposals on directional LBT, the following agreement on supporting TDM and SDM transmissions was agreed in RAN1-104e.

However, we note that Alt 1 with “wide beam” as an omni-directional beam based LBT at the beginning of COT and no LBT for the following beams is the better option for the following reasons:

- Alt 1 with omni-directional LBT is simpler and compliant with current HS (no need for LBT from the responding device(s)) in a friendly manner by performing omni-directional LBT (instead of wide sensing beam) at the beginning of COT;

- Alt 2 or by allowing directional LBT, would require the device(s) to perform LBT several times within a COT whenever the transmit direction changes, which adds unnecessary overhead leading to degraded performance. During the SI, many companies showed that LBT (either directional or omni-directional) degrade performance in most scenarios compared to no LBT;

- No further specification impact as there is no need to define “cover” for multi-beam case.

Therefore, to summarize, to enable time domain or spatial domain multiplexing of DL/UL in multiple beams, the LBT sensing beam needs to cover all the sector areas or the beam regions. We think it is easier to perform an omni-directional or quasi-omni directional “umbrella LBT” that covers all the possible beam directions at the beginning of the COT. Furthermore, there is no need to complicate channel access mechanism by having multiple LBT procedures for multiple beam directions. EN 302 567 does not define it, neither should 3GPP do that. In addition, there is also no need to define the relationship between the LBT sensing beam and the transmission beam if the LBT beam is omni-directional or quasi-omni-directional. As stated earlier, quasi-omni-directional LBT is also used by IEEE 802.11ad and IEE 802.11ay devices for operation in the 60 GHz band.

If any enhancements to better enable multiple beam transmissions within a COT when LBT mode is used can be agreed now, it is to support Alt 1 in principle for TDM and SDM case where a single LBT at the beginning of the COT is performed with the definition of “cover” meaning omni-directional or quasi-omni-directional.

RAN 1- 104b-e Agreements:

Agreement:

For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed one after another in time domain

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

In addition, further agreements regarding Alt 2 and Alt 3 above were agreed in RAN1-104b-e.

We note that, if the above proposal 10 were to be agreed, the details of the above agreements in RAN1- 104b-e becomes invalid. However, if multiple alternatives are supported from the above agreement from RAN1 -104e, then we would like to highlight the following complexity involved with the approach. Since the alternatives in the above agreements are considered based on the assumption that directional LBT for each beam is supported, there needs to be an agreement on  whether to support directional LBT as described in section 3.1 before we discuss the multi-beam COT. As discussed in the previous section, there is no notion of LBT/sensing related “beams” in the current 37.213 specification, nor have we agreed on how to specify directional LBT for a single beam case in RAN1. However, we have already agreed to consider performing independent per-beam LBT (Alt 2) for multi-beam COTs in RAN1 104-e. Therefore, in our opinion, we cannot make further agreements on the multi-beam case without agreeing on how to specify the single beam case.

All alternatives agreed to be considered for a COT with TDM and SDM of beams, depends solely on how directional LBT for a single beam would be specified.

RAN1 needs to decide on whether and how to specify directional LBT for single sensing beam case before further discussing multiple sensing beams.

## 3.2	Receiver assisted channel access and interference management

In TR 38.808, it was agreed to further investigated when specifications are developed, receiver assistance mechanism where receiver provides assistance information (signaling) to transmitter.

- Class A) Receiver provides assistance information (signalling) to transmitter only. The following aspects of Class A can be further discussed when specifications are developed. - Applicability in the following potential channel access modes: - LBT is performed prior to transmission, - No LBT is performed prior to transmission. - Details of assistance information (e.g., type, timing, content, how the assistance information is obtained etc.). - Whether the assistance information can be obtained by LBT performed at the receiver prior to transmission. - Whether the assistance information can be obtained by existing layer 1 and layer 3 measurements with enhancements if needed. - If any specification changes are needed to support Class A.

The above conclusion captured in TR 38.808 clearly suggests that the receiver assistance mechanism may or may not be directly connected to LBT that occurs prior to every data transmission. More specifically, receiver assistance mechanisms to be studied can be integrated inherently to the conventional LBT for every data transmission or can be completely decoupled from data transmission. An example for the former scenario is RTS/CTS handshaking as employed by Wi-Fi where channel clearance assessment is done at both transmitter and receiver prior to every data transmission. An example for the latter usage can be the legacy CSI or RSSI reporting in NR, where UE measures the radio channel periodically or aperiodically according to the instruction from gNB and report the measurement results back to the gNB via L1 or L3 signaling.

Agreement:

For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion

- Alt 1. Legacy RSSI measurement and reporting with possible enhancements

- Alt 2. AP-CSI report with possible enhancements

- Alt 3. LBT at receiver

  - Alt 3.1 eCCA

  - Alt 3.2 Cat2 LBT

In RAN1 104-e, the following was agreed:

It is clear that Alt-3 maps to receiver assistance mechanisms inherently integrated to the data transmission procedure (former case in the previous paragraph), while Alt-1 and Alt-2 map to receiver assistance mechanisms completely decoupled from data transmission (latter usage in the previous paragraph). In the remaining of this section we will analyze  these two different categories of receiver assistance mechanisms separately.

### 3.2.1	Receiver assistance mechanisms inherently integrated with the data transmission procedure

In RAN1 103-e we presented the performance evaluation for ideal receiver assistance LBT which shows an upper bound of what the receiver assistance LBT procedure would provide under an unrealistic assumption [6]. Even under the unrealistic assumption of instantaneous feedback (i.e., LBT result at the receiver can be instantly accessed by the transmitter without considering any signaling overhead), and with the minimum Contention Window size (3 observation slots), the evaluation result doesn’t show consistent performance improvement as compared to no LBT operation. On the contrary, aaccording to the results in [6] the ideal receiver assistance LBT reduces the system throughput in terms of both mean and 5th percentile throughput. Similar observation was also captured in the evaluation reports from some other companies.

The fact that receiver assisted LBT doesn’t prove consistent system performance improvement is mainly due to high pathloss and directive transmission in the spectrum above 52.6 GHz. For these reasons, it was observed in our evaluation that nodes rarely experience interference from other nodes in the same network or from another network in the vicinity. That is to say, the whole system pays for the significant overhead for performing LBT prior to every data transmission while doesn’t get much reward by doing so. Hence, it was observed in most cases that the system performance degrades with receiver assisted LBT compared to no LBT operation.

We notice some company claimed there was increased throughput for 5th percentile UEs under high load ([7]). However, from the email discussions in a previous meeting [8], we note that the gains obtained in the scenario does not seem to stem from LBT at the receiver, instead it seems to be achieved by optimal scheduling based on accurate and timely feedback information from the UEs. More specifically, the gNB obtains accurate and timely interference measurements from the UEs, based on which the gNB selects the UE with the lowest interference level to schedule data to. We agree that accurate and timely feedback information from the UEs can help the gNB to better schedule radio resources and improve system performance. However, this technical advantage is NOT inherently connected to LBT at the receiver. It can also be achieved with the receiver assistance mechanisms decoupled from the data transmission procedure, as discussed in the next section.

Receiver assisted LBT does not show consistent performance improvement as compared to no LBT operation.

According to our understanding, receiver assisted LBT involves an RTS/CTS-like handshaking mechanism in every data transfer procedure, which significantly increases data transfer latency and reduces spectrum efficiency and system capacity, which is illustrated in Figure 3 below.

Figure 3 demonstrates two exemplary DL data transmission scenarios, the first with the conventional transmitter LBT and the second with receiver assisted LBT. Multi-PDSCH is used in both examples, where 8 PDSCHs are scheduled by a single DCI. The receiver assisted LBT mechanism employed in the second example is according to the explanation from some companies during the last meeting, assuming very optimistic timing for the handshaking procedure. It can be seen from the examples that, even with the very optimistic timing assumption, the DL data transmission procedure with receiver assisted LBT takes 4 extra slots of overhead, which accounts for 40% latency increment compared to the conventional transmitter LBT.

One may claim that the overhead due to LBT at the receiver can decrease in a gNB initiated COT where the handshaking only occurs once at the very beginning of the COT. However, even in such scenario, the extra overhead due to LBT at the receiver is still not trivial. For example, even if the PDSCH transmission duration is increased 10 times to 100 slots, the handshaking overhead still accounts for 4% of the total data transfer latency.

Receiver assistance LBT involves RTS/CTS-like handshaking in every data transfer procedure, which significantly increases data transfer latency, reduces spectrum efficiency and system capacity.

Figure 3: Extra latency with Receiver Assistance LBT

According to some proponents of receiver assistance LBT, the RTS indication can be included in a DL DCI that schedules the PDSCH(s), and the CTS indication can be transmitted via PUCCH, which implies extension of the current DL DCI formats and introduction of new UCI type. Furthermore, the gNB might need to schedule the radio resource, in terms of OFDM symbols and PRBs, for the UE to perform LBT. The LBT procedure performed by the UE, either eCCA or CAT2 LBT or anything else, also needs to be specified in the specification. In case the UE responses with negative CTS (i.e., LBT fails at the UE), the gNB needs to cancel the planned PDSCH transmission immediately and reschedule the radio resource for another purpose. All these technical complexity and cost need to be evaluated and the standardization and implementation effort should not be under-estimated.

The standardization and implementation technical complexity and cost for receiver assistance LBT should not be under-estimated.

From the above analysis, it can be concluded that receiver assistance LBT under current consensus requires a RTS/CTS-like handshaking mechanism in every data transmission, which increases the data transfer latency and reduces spectrum efficiency, while doesn’t show much consistent system performance improvement. Moreover, the standardization and implementation technical complexity and cost for such receiver assistance LBT is not trivial. Therefore, we propose not to pursuit receiver assisted LBT in Rel-17.

Do not support receiver assisted LBT (Alt-3) in Rel-17.

### 3.2.2	Receiver assistance mechanisms decoupled from data transmission procedure

As discussed in the previous section, accurate and timely UE feedback is always beneficial for the scheduler to schedule radio resource more efficiently at the gNB. This is true regardless of whether the NR operation is on licensed or unlicensed spectrum. For NR operation on unlicensed spectrum, it is of particular interest for the gNB to obtain knowledge about the interference level at the UEs to avoid transmitting data to UEs that experience persistent interference, for example, due to hidden node problem. Hence, we agree that receiver assistance feedback of interference level from UEs is beneficial for system performance improvement in unlicensed operation.

With the receiver assistance mechanism, whenever the gNB wants to probe the interference level at the UE side, it can configure or trigger UE feedback, via the existing L3 RSSI reporting (periodic report) or L1 CSI reporting (periodic, semi-persistent and triggered aperiodic report). Different from the receiver assisted LBT as discussed in the previous section, which is an inherent part of a data transmission procedure, the receiver assistance mechanisms in this category are not directly connected to an individual data transmission. That is to say, the receiver assistance mechanisms should operate independently from the data transmission, and the data transmission procedure should operate as if there is no such a thing as receiver assistance.

NR specification has already provided a complete set of radio channel measurement and reporting mechanisms. In our view, receiver assistance information could potentially include various measurement results collected at the receiver, such as L1-RSRP, L1-SINR and RSSI (Note that L1-RSSI reporting is not defined in the current specifications but can be included). Measurement and reporting of receiver assistance information could be incorporated into the existing CSI reporting framework. In the UL, gNB can perform channel and interference measurement towards the targeted UE before UL data transmission is scheduled, hence there is no need for communicating receiver assistance information over the air interface.

Alt 1 and Alt 2 can reuse the measurement and reporting mechanisms already supported in 3GPP with possible enhancements. This means that receiver assistance can already be supported with these existing mechanisms and only enhancements (if need) to these need to be considered.

Support Alt-1 and Alt-2 for receiver assistance mechanisms that are based on the existing RSSI or CSI reporting and decoupled from data transmission procedure.

The current NR specification supports various CSI reporting mechanisms, such as periodic CSI reporting, semi-persistent CSI reporting and aperiodic CSI reporting, for various purposes in different application scenarios. Periodic CSI reporting is suitable for transmitting a small amount of information bits on PUCCH; semi-persistent CSI reporting can be used to report a bit more information bits either on PUCCH or PUSCH; aperiodic CSI reporting is developed for a large channel report on PUSCH. The aperiodic CSI reporting mechanism could potentially be adopted for receiver assistance information reporting due to its scheduling flexibility.

The current NR specification supports aperiodic CSI reporting on PUSCH. The gNB pre-configures the UE with a list of aperiodic trigger states, with each trigger states linked to one or multiple associated report configurations. Each associated report configuration contains a CSI report configuration ID and specifies a set of CSI-RS resource sets (NZP CSI-RS, SSB and CSI-IM resource sets) for channel and optionally for interference measurement. In the current NR specification, an aperiodic CSI is triggered by an UL grant DCI (DCI Format 0_1 or 0_2). The CSI request value in triggering DCI points to one of the trigger states in the pre-configured aperiodic trigger state list. When the UE detects a UL grant DCI with a valid CSI Request value, the UE should perform channel and optional interference measurement based on the CSI resource sets specified in the pre-configured associated report configuration, compute CSI-related, L1-RSRP-related or L1-SINR-related quantities as specified in the CSI report configuration, and transmit CSI report on the PUSCH resource scheduled by the UL grant DCI.

In the discussion below we will analyse the issues with the legacy CSI reporting and how CSI reporting can be enhanced to better support receiver assistance information reporting.

L1 report quantity

NR in Rel-15/16 only specifies L1-RSRP and L1-SINR in CSI measurement to be reported on PUCCH/PUSCH. For receiver assistance feedback in unlicensed operation, it seems beneficial to introduce a new Layer 1 report quantity of L1-RSSI so that the UE can measure and report interference level in terms of received signal strength. The measurement of L1-RSSI can be based on ‘empty’ radio resource (OFDMs and PRBs) that are reserved for RSSI measurement or based on CSI-IM.

A new L1 report quantity of L1-RSSI can be introduced for UE to report interference level to gNB.

Aperiodic CSI reporting on PUCCH

In NR Rel-15 and Rel-16, aperiodic CSI reporting can only be triggered by UL grant DCIs and the CSI report can only be transmitted on PUSCH. An enhancement is currently being discussed in Rel-17 in the URLLC WI to support triggering Aperiodic CSI by a PDSCH scheduling DCI, i.e., DCI format 1_1 and 1_2. Since there is no associated PUSCH, the triggered Aperiodic CSI report is carried by PUCCH. This new Aperiodic CSI mechanism can be considered as some sort of aperiodic CSI reporting on PUCCH, which is built on the Aperiodic CSI reporting framework as specified in the current NR specification, mainly with the following enhancement:

- The aperiodic CSI is triggered by a DL DCI (DCI Format 1_1 or 1_2). The DCI carries a CSI request, pointing to a pre-configured aperiodic CSI trigger state.

- The CSI report is transmitted on PUCCH. The PUCCH resource for CSI report transmission can be specified by higher layer configuration (CSI-ReportConfig).

In our view, the aperiodic CSI-Reporting mechanism in the current specification is a suitable tool to communicate receiver assistance information to the transmitter, i.e., the gNB.

Enhancement to enable aperiodic CSI reporting to be triggered by DL DCIs and to be transmitted on PUCCH as being discussed in the URLLC WI can be reused to communicate receiver assistance information to gNB.

CSI processing delay

The primary goal of receiver assistance mechanisms is to identify the situation where a UE experiences persistent interference, for example, due to the hidden node problem. Since the receiver assistance mechanisms of this category is not directly connected to any individual data transmission, the requirement on UE feedback latency can be quite relaxed. But still, it might be beneficial to optimize the current CSI reporting delay, to make the receiver assistance mechanisms more efficient.

In the current NR specification, the CSI reporting delays are quite long mainly due to the excessive CSI processing latency. For instance, it is up to 11 slots for 120 kHz SCS and could be up to 33 slots for 960 kHz SCS based on our estimation. Considering L1-RSSI measurement at the UE can most likely be done within a much shorter time compared to a complete CSI calculation, the CSI processing delay can be tightened at least for L1-RSSI measurement. In our view, the latency for UE to calculate and report L1-RSSI to gNB could be confined within one or two slots, on-par with the latency for LBT at the receiver as shown in Figure 3 in the previous section.

Current processing delay requirement for CSI reporting in NR can be reduced for L1-RSSI reporting, to make the receiver assistance mechanisms more efficient.

Based on above analysis, we propose the following:

The following enhancements on the current AP-CSI reporting can be considered to better support receiver assistance information reporting:

- Introduce new report quantity of L1-RSSI

- Support AP-CSI reporting triggered by DL DCIs and transmitted on PUCCH as being discussed in the URLLC WI

- Reduce CSI processing latency requirement for L1-RSSI reporting.

## 3.3	 COT sharing aspects

### 3.3.1	CAT2 LBT

Agreement:

For Cat 2 LBT, down-select from the following alternatives

- Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation

- Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation

Agreement:

If Cat 2 LBT is introduced, the following use cases can be further studied:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

- COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling

Other use cases not precluded.

FFS if Cat 2 LBT is mandated for each use case or not.

Regarding the CAT2 LBT the following agreement was made in RAN1#104-e

CAT2 LBT has been touted as useful mechanism for various use cases in RAN1 104-e for 60 GHz unlicensed operation.

However, it should be noted that the evaluations done during the SI phase do not encourage the use of CAT3 LBT/eCCA (8us +5us*rand(0,3) = 23us max, the contention window is fixed with minimum value of 3) as it results in aggregated throughput degradation when LBT is used. Adding further complexity and overhead in terms of CAT2 LBT is not only necessary but futile. Furthermore, EN 302 567 does not mandate any CAT2 LBT.

CAT2 LBT is not specified in HS EN 302 567

There are no studies done to show that doing CAT2 LBT at the UE (when LBT-mode or no-LBT mode is used at the transmitter) is good for coexistence in the use cases described above in the 60 GHz band considering the fact that CAT3 LBT itself did not have a significant impact in the simulation studies.

In RAN1 103-e we presented the performance evaluation for ideal receiver assisted LBT (RAL) [6]. In the evaluated procedure, the CAT3 LBT procedure is performed at the receiver. It is also worthy to note that the LBT result at the receiver is assumed to be available instantly at the transmitter without accounting for any overhead for exchanging this information between the transmitter and the receiver. According to the results, the CAT3 LBT at the receiver reduces the system throughput (both mean and 5th%ile) even for the most ideal scenario. We postulated that if a more conservative CAT3 LBT (23 µs) did not yield any performance improvement for receiver-assistance in channel access, CAT2 LBT (8 µs) will likely not.

Furthermore, in RAN1 104b-e [15], we presented the performance evaluation for COT sharing using CAT2 LBT without any receiver-assistance, in addition to CAT3 LBT at the initiating device. In the simulations, CAT2 LBT was performed at the responding device when the gap between the previous transmission and the current transmission is more than 8us using Scenario A, which has the largest number of nodes among the indoor scenarios (A, B and C).

It was observed in [15] that, there are no gains obtained due to performing CAT2 LBT at the responding device for COT sharing when the initiating device performs both omni-directional and directional CAT3 LBT. The mean and 5th percentile throughput of the system either remains unchanged or is slightly lowered when performing CAT2 LBT at the responding device, regardless of whether it was directional or omni-directional. It can also be noted that the type of LBT (omni-directional or directional CAT3) at the initiating device also seem to have no significant impact on the system throughput.

Simulations study show that there is no consistent gain using CAT2 LBT compared to no LBT for COT sharing.

It is not precluded to do CAT2 LBT in addition to the CAT3 LBT requirements. There is no motivation to specify it in the 3GPP RAN1 standard.

Do not support CAT2 LBT for NR operation in 52.6 GHz to 71 GHz.

### 3.3.2		Gaps in Shared COT

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select from

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 2. Define a maximum gap X, such that a later transmission can share the COT without LBT only if the later transmission starts within X from the end of the earlier transmission

  - FFS: Value for X

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

  - FFS: Value for Y

  - FFS:  How to define the one-shot LBT

The following was agreed in RAN1- 104-e

ETSI HS EN 302 567 specifies that an equipment (initiating or not initiating transmission), upon correct reception of a packet which was intended for that equipment, can skip the CCA Check, and immediately proceed with the transmission in response to received frames. This does not clearly explain what “immediately” means but it also does not mandate CAT2 LBT. Furthermore, the evaluations done so far do not encourage the use of LBT as it results in aggregated throughput degradation when LBT is used. Although, we see the need to clarify what “immediately” is, there is no need to include additional mechanisms to the specification without evaluating them.

Gaps defined in the system does not have a direct bearing on the interference caused to the system. It is only in the best interest for a system to have minimum gaps so that the channel access by other neighboring systems do not conclude that the channel is idle. We understand the generic concern that a long gap within a COT may cause other devices to access the channel and cause collisions. However, this makes more sense only in 5/6 GHz and not in the 60 GHz regime where the interference is very low and the transmissions, highly directional. Simulations during the SI showed that there is no significant benefit of LBT in general. Our recent simulations also confirmed that with a gap of 8us or more, CAT2 LBT does not have any consistent impact on the system throughput. Therefore, we do not think it is an issue. In fact,existing IEEE 802.11ad and IEEE 802.11ay systems may use a maximum of 40 µs gap while performing Beamforming refinement protocol, see aBRPIFS in Figure 5 [12][13].

ETSI BRAN regulations do not specify a minimum or maximum gap in the 60 GHz HS.

Support Alt 1 for gaps in COT sharing.

## 3.4	CAPC and CWS enhancements

The below statement was included in TR 38.808 v1.0.0 [3],

“Use the CCA check procedure in EN 302 567 as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work: - whether CAPC and contention window adjustment mechanisms are introduced, - whether contention window range needs to be adjusted.”

There is a justification for having channel access priority class (CAPC), contention window size (CWS) adjustment in 5 GHz because the propagation characteristics and coverage of this frequency range might result in interference issues. So, it was important to make sure that high priority data is prioritized in this case (via CAPC) and collisions are resolved via CWS adjustment.

The situation is very different in 60 GHz. Most companies have shown that the LBT is inducing unnecessary deferral that reduces throughput performance. Differentiating between traffic types would mean inducing even larger unnecessary latencies.

In general, LBT in 60 GHz may or may not bring gains for the 5th perc. users, but what all companies agree on is that it has a negative impact on the aggregated system performance. Therefore, there is no justification to increase the LBT overhead by further introducing CAPC and CW adjustment.

Another point that strengthens this argument is that CWS adjustment and CAPC are not specified by the regulations in the HS EN 302 567 which was decided to be used as baseline for channel access in 60 GHz when LBT is applied.

The work should be focused on what is needed to enhance the performance and not to re-specify the 5 GHz LBT aspects in 60 GHz without a strong motivation.

Do not support CAPC and CWS adjustment for NR operation in 52.6 GHz to 71 GHz.

## 3.5	 Sensing slot structures

Agreement:

For energy measurement in 8us deferral period, down-select from the following:

- Alt 1. Two energy measurements are required

- Alt 2. One measurement is required

- Alt 3. Extend the 8us to 10us and perform two measurements, one in each 5us segment

For energy measurement in 5us observation slot, perform single measurement

- FFS minimum duration of the measurement

- FFS location of the measurement

In RAN1 104-e, the following was agreed regarding the sensing structures for LBT.

Regarding the sensing periods of 8us deferral period and 5us sensing slot for eCCA, EN 302 567 does not specify any sensing structure but these parameters were adopted from 802.11ad specifications.

5us slot is definition of aSlotTime, and 8 µs is sum of aSIFSTime (3 µs) + aSlotTime (5 µs). Furthermore, in IEEE 802.11ad amendment (incorporated in IEEE 802.11-2020), a SIFSTime and aSlotTime is calculated as

IEEE 802.11ad and IEEE 802.11ay do not perform two energy measurements in the 8 µs deferral period

Based on the above, there is no need to perform two energy measurements within an 8 µs period as 802.11ad also do not. ETSI HS requirements were developed from IEEE 802.11ad. Moreover, there is no study to show that having an 8 µs deferral period with one energy measurement is detrimental to coexistence.

ETSI HS does not require two energy measurements in 8 µs deferral period.

No simulation studies to suggest that two energy measurements are needed in an 8us deferral period for good coexistence.

For energy measurement in 8 µs deferral period, Alt2 is preferred.

Regarding 5 µs measurement periods, aSlotTime (5 µs) is defined as including aCCAtime. aCCATime is defined as “the maximum time (in microseconds) the CCA mechanism has available to assess the medium to determine whether the medium is busy or idle.” This is specified as a maximum duration of 3us in the 802.11ad-2012 specifications as shown in Figure 4. However, this is changed to “implementation dependent” in the recently released 802.11-2020 specifications, Table 20-30 Figure 5. Therefore, we think these values should be the same in 3GPP specifications as well. However, in 3GPP, we need to define only a minimum duration and not a maximum duration.

The minimum duration of energy measurement within 5 µs can be left for implementation.

Working assumption:

For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

The location of the measurement can be implementation dependent. It was agreed in RAN 104b-3 as a working assumption to leave the location of energy measurement within 5us for implementation.

Therefore, we propose the following.

Confirm the working assumption that for the location of the energy measurement in 5us,it can be left for implementation.

Figure 4:	 802.11ad-2012 specification: aCCATime value specified as less than 3 us

Figure 5:	Recent 802.11-2020 specification showing aCCATime value for 802.11ad specified as “Implementation dependent”.

## 3.6	No LBT/LBT mode switching

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between

- Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication

- Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

- FFS: Whether the indication of the decision on applying LBT mode or no-LBT  mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation)

- FFS: Whether a gNB and its UE(s) can have different mode

- FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication

Both LBT and no LBT channel access modes has been agreed to support for NR operation in 52.6 GHz to 71 GHz. The mechanism to indicate which LBT mode to use by the UEs has been discussed last meetings. The following alternatives for LBT mode indication have been agreed in the last meeting.

Indicating LBT mode via cell specific signal is necessary for the UE to determine which LBT mode to use before entering to the RRC connected mode. Similar mechanism to indicate LBT mode (whether dynamic LBT or semi statistic LBT) is also used in rel-16. Therefore, it is reasonable to support cell specific indication of LBT mode in rel-17. In our companion paper [14] we discuss some further details on how to signal the LBT during the initial access procedure.

For the other option, LBT mode is also indicated as part of UE-specific RRC configuration, which implies that different UEs could be indicated different LBT modes in the same cell. This mechanism could be useful for the scenario that channel access is supported by long-term interference measurement, in which gNB could switch some UEs that get consistent strong interference to the LBT mode.

Support Atl.2 where both cell specific and UE specific signals could be used for indicating LBT mode.

Regarding to the first FFS of the above agreement, three different scenarios are considered:

- Different UEs with different beams in the same cell.

- Different beam pairs for the same gNB-UE pair.

- Different beams from different cells for the same UE.

For scenario i), we think it is covered by Alt.2, where the LBT mode is indicated per UE. Therefore, if Alt.2 is supported, it is possible to indicate different LBT modes for different UEs with different beams in the same cell.

For scenario ii), the typical use case will be that the UE transmits in different beams instantaneously in UL, where different LBT modes are used for different beams. Then, the LBT procedure for multiple beams could be similar to multi-channel LBT, where different LBT engines will be aligned so that all LBTs could be finished in the same time for UL transmission alignment. However, the main benefit of no LBT mode is to reduce LBT overhead, if the beam with no LBT mode need to wait for the beam with LBT mode finishing LBT, then the benefit of no LBT is diminished. Therefore, we do not see the benefit of indicating separate LBT mode for different beams for the same UE.

For scenario iii) Similar to the discussion on the benefit of different LBT modes for different beams of the same UEs in scenario ii), we do not see the benefit to indicate different LBT modes for different beams from different cells. However, since LBT mode is already indicated independently per cell (if Alt.1 or Alt.2 is supported), no more complexity is required for indicate different LBT modes for different cells. It could be up to gNB implementation to whether to indicate the same or different LBT modes for different beams from different cells for the same UE.

Regarding to the second FFS, if Alt.2 is supported, it already covers the case that gNB could have different LBT modes from some of its UE(s). For instance, when different LBT modes are configured for different UEs in the same cell, the serving gNB could have different LBT mode from at least one of the UEs in the cell. Therefore, we propose the following.

For NR operation in 52.6GHz to 71 GHz, gNB and UE(s) could have different LBT modes.

# 4	Conclusion

In this contribution, we first discuss the baseline changes to LBT design that need to be addressed due to the change in the harmonized standard HS EN 302 567. Our analysis shows that there is no significant gain from doing LBT but we propose enhancements to different LBT mechanisms, its usage and design, when used. Finally, we discuss a few potential enhancements that are beneficial to NR operation in 52.6 GHz to 71 GHz. The following observations can be drawn:

Observation 1	ED threshold defined in EN 302 567 v2.2.0 is a function of the transmission’s EIRP Pout, which includes the transmission beamforming gain. It does not include the sensing beamforming gain.

Observation 2	In EN 302 567, the nominal channel bandwidth and at least one transmission mode with occupied channel BW 70% of NBW is defined for spurious out-of-band emissions and not for LBT purposes.

Observation 3	The relationship between the LBT bandwidth and the channel bandwidth is not specified in EN 302 567 for the sake of technology-neutrality and flexibility.

Observation 4	Operating channel BW defined in EN 302 567 is the LBT BW in RAN1 which is already defined in 37.213 as a “channel”

Observation 5	Alt SC3/CA5 poses an artificial restriction to insert guard bands at the end of the LBT units

Observation 6	For SC3, LBT failure for a node within a LBT unit is complex and not discussed.

Observation 7	Definitions in EN 302 567 and TS 37.213 at least covers Alt SC1.

Observation 8	ETSI regulation for 60 GHz bands do not support Type B multi-channel access.

Observation 9	In HS EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period.

Observation 10	The effectiveness of LBT itself as medium access mechanism for co-existence in unlicensed spectrum in 60 GHz band is questionable. Therefore, any further enhancement on LBT baseline from the HS need to be justified both on the performance gain and the required complexity.

Observation 11	Common understanding in ETSI and IEEE 802.11ad and IEEE 802.11ay specs are omni-directional LBT or quasi-omnidirectional LBT

Observation 12	Simulation studies in general indicate no significant gain from using directional LBT.

Observation 13	Directional LBT is currently not precluded in the existing regulations. EN 302 567¨s tests intrinsically ensure sensing beam is in the direction of the transmission beam for devices equipped with directional antenna systems.

Observation 14	Notion of “beams” for sensing/LBT is non-existent in 37.213.

Observation 15	Alt1 and Alt3 have more RAN4 spec impact and can be considered together under a single alternative.

Observation 16	All alternatives agreed to be considered for a COT with TDM and SDM of beams, depends solely on how directional LBT for a single beam would be specified.

Observation 17	Receiver assisted LBT does not show consistent performance improvement as compared to no LBT operation.

Observation 18	Receiver assistance LBT involves RTS/CTS-like handshaking in every data transfer procedure, which significantly increases data transfer latency, reduces spectrum efficiency and system capacity.

Observation 19	The standardization and implementation technical complexity and cost for receiver assistance LBT should not be under-estimated.

Observation 20	A new L1 report quantity of L1-RSSI can be introduced for UE to report interference level to gNB.

Observation 21	Enhancement to enable aperiodic CSI reporting to be triggered by DL DCIs and to be transmitted on PUCCH as being discussed in the URLLC WI can be reused to communicate receiver assistance information to gNB.

Observation 22	Current processing delay requirement for CSI reporting in NR can be reduced for L1-RSSI reporting, to make the receiver assistance mechanisms more efficient.

Observation 23	CAT2 LBT is not specified in HS EN 302 567

Observation 24	Simulations study show that there is no consistent gain using CAT2 LBT compared to no LBT for COT sharing.

Observation 25	It is not precluded to do CAT2 LBT in addition to the CAT3 LBT requirements. There is no motivation to specify it in the 3GPP RAN1 standard.

Observation 26	ETSI BRAN regulations do not specify a minimum or maximum gap in the 60 GHz HS.

Observation 27	IEEE 802.11ad and IEEE 802.11ay do not perform two energy measurements in the 8 µs deferral period

Observation 28	ETSI HS does not require two energy measurements in 8 µs deferral period.

Observation 29	No simulation studies to suggest that two energy measurements are needed in an 8us deferral period for good coexistence.

Based on the extensive analysis and observations provide in this contribution, we propose

Proposal 1	Confirm the working assumption that Pout corresponds to the maximum of the mean output power EIRPs of the transmissions or transmission bursts in a COT that may contain varying transmission beams and EIRPs.

Proposal 2	Further adjustment on ED threshold based on the transmission and sensing beamforming gains could be up to implementation while not violating EDT requirements as per regulations.

Proposal 3	Support Alt SC1/Alt CA1 for LBT in single carrier and multi-carrier operation. Other options are not precluded by implementation.

Proposal 4	Support Alt1 in the agreement that allows only Type A multi-channel access from 37.213.

Proposal 5	Do not support Type B multi-channel access for NR operation in 52.6 GHz to 71 GHz.

Proposal 6	Support extending the Short control signalling transmissions exemption to Discovery Burst.

Proposal 7	Consistent with EN 302 567, a node can access the channel without LBT for control signal/channel transmissions, the total duration of which shall not exceed 10 ms within an observation period of 100ms. The following signals/channels shall be classified as short control signaling transmissions:

1	msg1 and msg3 for the 4 step RACH and MsgA for the 2-step RACH

2	FFS: Other control transmissions not multiplexed with user data (subject to gNB configuration)

Proposal 8	Support omni-directional LBT or quasi-omni-directional LBT as the baseline LBT procedure for 60 GHz band.

Proposal 9	Do not support Alt.2 on extending the beam correspondence framework and/or QCL/TCI framework to define “cover”.

Proposal 10	If any enhancements to better enable multiple beam transmissions within a COT when LBT mode is used can be agreed now, it is to support Alt 1 in principle for TDM and SDM case where a single LBT at the beginning of the COT is performed with the definition of “cover” meaning omni-directional or quasi-omni-directional.

Proposal 11	RAN1 needs to decide on whether and how to specify directional LBT for single sensing beam case before further discussing multiple sensing beams.

Proposal 12	Do not support receiver assisted LBT (Alt-3) in Rel-17.

Proposal 13	Support Alt-1 and Alt-2 for receiver assistance mechanisms that are based on the existing RSSI or CSI reporting and decoupled from data transmission procedure.

Proposal 14	The following enhancements on the current AP-CSI reporting can be considered to better support receiver assistance information reporting:

Proposal 15	Do not support CAT2 LBT for NR operation in 52.6 GHz to 71 GHz.

Proposal 16	Support Alt 1 for gaps in COT sharing.

Proposal 17	Do not support CAPC and CWS adjustment for NR operation in 52.6 GHz to 71 GHz.

Proposal 18	For energy measurement in 8 µs deferral period, Alt2 is preferred.

Proposal 19	The minimum duration of energy measurement within 5 µs can be left for implementation.

Proposal 20	Confirm the working assumption that for the location of the energy measurement in 5us,it can be left for implementation.

Proposal 21	Support Atl.2 where both cell specific and UE specific signals could be used for indicating LBT mode.

Proposal 22	For NR operation in 52.6GHz to 71 GHz, gNB and UE(s) could have different LBT modes.

# 5	References

RP-202925, Revised WID: Extending current NR operation to 71 GHz

RP-193229, New WID on Extending current NR operation to 71 GHz

3GPP TR 38.808 Technical Report, "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)" v1.0.0, December 2020.

EN 302 567 v2.2.0, Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum, Accessed on 2021-1-18 https://www.etsi.org/deliver/etsi_en/302500_302599/302567/02.02.00_20/en_302567v020200a.pdf,

3GPP TS 38.101-1 Technical Specification, “5G; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (3GPP TS 38.101-1 version 16.5.0 Release 16)”, Accessed on 2021-1-18  https://www.etsi.org/deliver/etsi_ts/138100_138199/13810101/16.05.00_60/ts_13810101v160500p.pdf

R1-2007983 Channel access mechanism; 3GPP TSG-RAN WG1 Meeting #103-e, 26th October to 13th November 2020

R1-2008976, Channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon; 3GPP TSG-RAN WG1 Meeting #103-e, 26th October to 13th November 2020

R1-2009760, FL summary#10 for channel access of 52.6GHz to 71GHz band, Moderator (Qualcomm); 3GPP TSG-RAN WG1 Meeting #103-e, 26th October to 13th November 2020

R1-2007984, Evaluation results for NR in 52.6 - 71 GHz, Ericsson

R1-2102081 Email discussion summary#3 for channel access for 52.6-71GHz, Moderator (Qualcomm);3GPP TSG-RAN WG1 Meeting 104-e, January 25th – February 5th, 2021

ERC REC (70)03 Relating to the use of Short-Range Devices (SRD) https://docdb.cept.org/download/25c41779-cd6e/Rec7003e.pdf

IEEE Std 802.11ad™-2012: IEEE Standard for Information Technology—Telecommunications and Information Exchange between Systems; Local and Metropolitan Area Networks—Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications; Amendment 3: Enhancements for Very High Throughput in the 60 GHz Band

IEEE Std 802.11™‐2020: IEEE Standard for Information Technology—Telecommunications and Information Exchange between Systems; Local and Metropolitan Area Networks—Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

R1-2104460, "Initial Access Aspects," Ericsson, RAN1#105-e, May 2021.

R1-2102793, “Channel Access Mechanisms”, Ericsson, RAN1#104b-e, April 2021.

R1-2104040, FL summary#3 for channel access mechanism for 52.6GHz to 71GHz, Moderator (Qualcomm), RAN1_104b, April 2021.