3GPP TSG RAN1 Meeting #105-e                                              R1-2104466

Electronic Meeting, 19th – 27th May, 2021

Source:	CATT

Title:	Discussion on the EN-DC/NE-DC type applicable to those UE capabilities

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

RAN2 has discussed UE capability of intra-band and inter-band EN-DC/NE-DC in the following and send an LS [1] to RAN4/1 with two questions regarding the band combination associated with these capabilities.

| R1: 6-24 | Applying the same UL timing between NR and LTE | ul-TimingAlignmentEUTRA-NR Indicates whether to apply the same UL timing between NR and LTE for dynamic power sharing capable UE operating in a synchronous intra-band contiguous (NG)EN-DC. If this field is absent, UE shall be capable of handling a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous (NG)EN-DC network, as specified in TS 38.133 [5]. If this capability is included in an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part, this capability is used to indicate the restriction to the intra-band (NG)EN-DC BC part. |
| --- | --- | --- |
| R1: 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | pa-PhaseDiscontinuityImpacts Indicates incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band (NG)EN-DC/NE-DC, intra-band CA and FDM based ULSUP. |
| R4: 2-16 | PA architectures for intra-band EN-DC | dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. |
| R4:2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. |
| R4:2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. |

This paper discussed the applied band combination for RAN1 EN-DC/NE-DC capability 6-24 and 6-23.

# Discussion

RAN2 had discussed band combination of the 5 (NG)EN-DC/NE-DC BC types below were for the purpose of analyzing the applicability of the UE capability parameters with  the boldface bands representing UL band.

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78.

For UE capability 6-24 applying the same UL timing between NR and LTE, it indicates whether to apply the same UL timing between NR and LTE for dynamic power sharing capable UE operating in a synchronous intra-band contiguous (NG)EN-DC.   This capability is generally for intra-band synchronous EN-DC/NE-DC or restriction of intra-band EN-DC for inter-band EN-DC with intra-band EN-DC.  Thus, capability 6-24 applies to Types 1 and 2.   Type 3 does not fit to UE capability 6-24 because it does not support UL in both bands of intra-band EN-DC/NE-DC.   Types 4 and 5 are inter-band EN-DC/NE-DC.  Types 4 and 5 do not apply UE capability 6-24.

For UE capability 6-23, UE indicates that it is not able to support PA phase discontinuity with CA/DC when two UL carriers are not fully aligned in overlapping transmissions.   The UE incapability of support PA phase discontinuity applies to the UE design with sharing PA for intra-band CA or EN-DC/NE-DC.    Thus, the capability 6-23 applies to Types 1 and 2.    Type 3 does not support UL in both bands of intra-band EN-DC/NE-DC.  Types 4 and 5 are inter-band EN-DC/NE-DC and do not have issue of phase discontinuity since most likely they could not share the same PA.

In summary, the proposed answer to RAN2 Questions 1 and 2 are as follows,

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

Answer: Types 1 and 2 for UE capabilities ul-TimingAlignmentEUTRA-NR and pa-PhaseDiscontinuityImpacts.

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

Answer: Yes.

# Conclusion

In this contribution, we discussed RAN2 question on the band combination applied to RAN1 UE capabilities for EN-DC/NE-DC 6-23 and 6-24.   We have the following proposed answers to RAN2’s two questions regarding RAN1 UE capabilities,

- Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

Answer: Types 1 and 2 for UE capabilities ul-TimingAlignmentEUTRA-NR and pa-PhaseDiscontinuityImpacts.

- Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

Answer: Yes.

# Reference

- R1-2104162, “LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities”, RAN2, ZTE