3GPP TSG RAN WG1 #105-e		R1-2104487

e-Meeting, May 19th – 27th, 2021

Source:	CATT

Title:	Discussion on HST-SFN transmission schemes

Agenda Item:	8.1.2.4

Document for:	Discussion and Decision

Introduction

HST-SFN deployment scenario is one of the most important scenarios for high speed train deployment. In order to have better performance in HST-SFN deployment scenario, the following objective was approved in the WID for Rel-17 NR FeMIMO [1]:

Enhancement to support HST-SFN deployment scenario: i. Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission ii. Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In RAN1 #104bis e-meeting, the following agreements and working assumptions on DL transmission schemes and TRP-based frequency offset pre-compensation schemes for HST-SFN deployment were achieved:

Agreement Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission - The corresponding MAC CE includes at least the following fields - Serving cell ID - CORESET ID - Two TCI state IDs - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE - FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex Agreement Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - FFS: Details including UL RS enhancement - DL RS based Doppler feedback by UE - FFS: Details - FFS: Whether UE capability needs to be introduced - Whether to support one or both will be decided later Agreement - Support dynamic (DCI-based) switching of scheme 1 (PDSCH) with single-TRP scheme by TCI state field in DCI format 1_1/1_2 - This feature is UE optional - FFS all other details including RRC signalling, possible RAN4 impact (if any), etc. Working Assumption All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1 Agreement Support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a - FFS: Whether dynamic switching is additionally supported Agreement Scheme 1 for PDSCH is identified by - New RRC parameter and the number of TCI states indicated by DCI - FFS RRC configuration details, e.g., per BWP or per CC - FFS whether or not restriction to a single CDM group for DM-RS is also supported

In this document, we share our views on the enhancements for HST-SFN deployment scenario.

In HST-SFN deployment scenario, DL signals can be transmitted in SFN manner from multiple TRPs that deployed along the railway. When a signal is SFN-ed transmitted, the propagation channel of the signal comprises multiple paths with different Doppler shifts and the channel cannot be well estimated by a single SFN-ed transmitted reference signal. Hence, TRP-specific/non-SFN-ed reference signals for Doppler profile estimation are expected.

Definition of QCL

In Rel-15, the definition of QCL is specified as follows:

Two antenna ports are said to be quasi co-located if the large-scale properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

For the enhanced scheme, the large-scale properties of the channel that convey DMRS port(s) can be inferred from the combination of channels of the RS ports that indicated by multiple TCI states. However, the large-scale properties of the channel of each RS port that indicated by the multiple TCI states cannot be inferred from the channel of the DMRS port. Hence, a definition on QCL association relationship of an antenna port and an antenna port group is needed. One example of the definition is as follows:

Antenna port x and antenna port group Y are said to be quasi co-located if the large-scale properties of the channel over which a symbol on antenna port x is conveyed can be inferred from the combination of channels over which the symbols on the antenna ports of the antenna port group Y are conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Observation 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group is needed in Rel-17.

Proposal 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group can be introduced.

  - E.g. Antenna port x and antenna port group Y are said to be quasi co-located if the large-scale properties of the channel over which a symbol on antenna port x is conveyed can be inferred from the combination of channels over which the symbols on the antenna ports of the antenna port group y are conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Identification of SFN-ed PDSCH transmission scheme

In RAN1#104bis e-meeting, the following agreement for identification of scheme 1 is shown below:

- Scheme 1 for PDSCH is identified by

- New RRC parameter and the number of TCI states indicated by DCI

  - FFS RRC configuration details, e.g., per BWP or per CC

  - FFS whether or not restriction to a single CDM group for DM-RS is also supported

The first issue is the frequency granularity for this RRC configuration. Since the RRC parameter for R16 M-TRP ‘RepetitionScheme’ is configured per BWP, the RRC parameter for scheme 1 (PDSCH) can also be configured per BWP.

The second issue for identification of scheme 1 (PDSCH) is whether or not restrict the DMRS ports for scheme 1 to a single CDM group.

If the restriction the number of CDM groups is used for scheme 1, dynamic switching of scheme 1 with SDM 1a can be achieved additionally according to the indicated numbers of CDM groups. For example, if a TCI codepoint associated with 2 TCI states is indicated and the DMRS port(s) of PDSCH are within one CDM group, the PDSCH is SFN-ed transmitted with the scheme 1 of HST scenario in Rel-17; If a TCI codepoint associated with 2 TCI states is indicated and DMRS port(s) of PDSCH are in two CDM groups, the PDSCH is transmitted with SDM 1a.

If dynamic switching between scheme 1 and 1a is not supported, restricting the DMRS ports of the PDSCH with scheme 1 to be in a single CDM group is benefit for reducing the DCI overhead for DMRS ports indication. If a UE is configured with scheme 1, the new table including only states for DMRS ports within one CDM group can be introduced for antenna port indication. For example, if DMRS type is ‘1’, the max length of front-load DMRS is one symbol and all DMRS ports are within CDM group 0, Table 1 can be used for SFN-ed PDSCH transmission scheme and 1 bit DCI overhead for DMRS ports indication can be reduced compared with Rel-16. Therefore, restricting the number of CDM groups to one is preferred for scheme 1.

Table 1: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 0,1 |
| 6-7 | Reserved | Reserved |

Dynamic switching of scheme 1 (PDSCH) with S-TRP

In RAN1 #104bis e-meeting, it is agreed that dynamic (DCI based) switching of scheme 1 (PDSCH) with S-TRP based UE optional feature is supported. If UE reports the capability for dynamic switching of scheme 1 with S-TRP, the following alternatives are shown below:

- Alt 1: A new RRC parameter on indicating whether dynamic switching is enabled is supported.

- Alt 2: No RRC parameter on indicating whether dynamic switching is enabled.

For Alt 1, UE could be configured by a RRC signaling for whether dynamic switching would be enabled after UE reporting capability for dynamic switching of scheme 1 with S-TRP. If UE reports capability of supporting dynamic switching of scheme 1 with S-TRP, gNB may still not configure dynamic switching for UE according to the current scheduling and channel condition.

For Alt 2, whether dynamic switching is enabled or not only depends on the reported capability. It’s not reasonable to always follow the capability reporting.

Compared with Alt2, Alt 1 is more flexible because that gNB can flexibly configure dynamic switching for UE according to the current scheduling and channel information. Hence Alt 1 is preferred for dynamic (DCI based) switching of scheme 1 (PDSCH) with S-TRP based on UE’s capability.

Proposal 2:

- The new RRC parameter identifying Scheme 1 for PDSCH is configured per BWP.

Proposal 3:

- For scheme 1,

  - restricting DMRS ports within one CDM group;

  - new tables for antenna port indication are introduced to reduce DCI overhead.

Proposal 4:

- Dynamic switching of scheme 1 and S-TRP, whether dynamic switching is enabled or not is configured by RRC.

Issues related to SFN-ed PDCCH transmission scheme

Default beam for PDSCH

In Rel-16, a PDSCH can be indicated with a TCI codepoint associated to two TCI states for M-TRP transmission schemes of SDM 1a, FDM 2a, FDM 2b, TDM 3, and TDM 4. After a Rel-16 UE reports its capability on supporting two default beams for PDSCH, gNB can configure RRC parameter enableTwoDefaultTCI-States for the UE to let UE use two default TCI states for M-TRP PDSCH reception. Then, UE determines TCI states for PDSCH based on configuration of enableTwoDefaultTCI-States and the value of the time offset between the reception of the DL DCI and the corresponding PDSCH and TCI field in DL DCI.

On the default TCI states for PDSCH, the following three cases have to be considered:

- Case 1: After a UE receives an initial higher layer configuration of TCI states and before reception of the MAC CE activation command

- Case 2: The time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD'

- Case 3: The time offset between the reception of the DL DCI and the corresponding PDSCH is equal or greater than the threshold timeDurationForQCL, and there is no TCI field in the DCI scheduling PDSCH

For case 1, in Rel-16, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.The scheme can be reused for in Rel-17.

For case 2, in Rel-16, one CORESET is indicated with only one TCI states. If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint containing two different TCI states.

For the Rel-17 enhanced PDCCH transmission, a PDCCH can be indicated with a TCI codepoint with two TCI states for M-TRP transmission schemes of SFN, FDM and TDM. For SFN scheme, one CORESET can be associated with two TCI states. For PDCCH with FDM or TDM M-TRP repetition scheme, two search space sets from two CORESETs can be linked by RRC signaling for M-TRP transmission. Compared to selecting the lowest TCI codepoint corresponding to two TCI states, the channel property of a SFN-ed PDCCH containing two TCI states is more likely to be closer to SFN-ed PDSCH. So if a UE is configured with enableTwoDefaultTCI-States, the following schemes can be considered:

- Option 1: Reuse R16 rules for M-TRP SDM 1a.

For option 1, UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint containing two different TCI states.

- Option 2: Introduce new rules for SFN-ed PDSCH, the detail rules is shown below:

  - If there is any CORESET associated with two TCI states, UE applies the QCL assumption of the CORESET with the lowest ID that associated with two TCI states in the latest slot when receiving the PDSCH.

  - Else if there is any CORESET that is associated with another CORESET for PDCCH repetition transmission, UE applies the QCL assumption of the CORESET with lowest ID among the CORESETs that have associated CORESET, and the associated CORESET of the CORESET with lowest ID in the latest slot.

  - Else the UE may assume that TCI states of PDSCH are associated with the lowest TCI codepoint among the TCI codepoints containing two different TCI states.

In Rel-16, if a UE is not configured with enableTwoDefaultTCI-States, the UE assumes that PDSCH is QCL-ed with TCI states of the CORESET with the lowest ID in the latest slot. Since in Rel-17 enhanced PDCCH transmission, the CORESET with the lowest ID may be associated with two TCI states, UE has to know which TCI state the PDSCH is associated to. The following four candidate alternatives can be considered for this case:

- Alt 1：UE applies the QCL assumption of the PDSCH according to the TCI state of the CORESET with the lowest ID, and the CORESET with the lowest ID in the latest slot is expected to be only associated with one TCI state.

- Alt 2: UE applies the QCL assumption of the CORESET with lowest ID that has only one TCI state in the latest slot, if any; otherwise, UE applies the QCL assumption corresponding to the first TCI state of the CORESET with lowest ID.

- Alt 3: UE applies the QCL assumption of the CORESET with lowest ID in the latest slot. If the CORESET with the lowest ID in the latest slot is associated to two TCI state, UE applies the QCL assumption corresponding to the first TCI state of the CORESET.

Since Alt 1 makes a big restriction on gNB scheduling, Alt 2 and Alt 3 are more preferred for Rel-17.

Proposal 5: For the default beams of a PDSCH with scheme 1,

- When the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’, and a UE is configured with ‘enableTwoDefaultTCI-States’, the following scheme is supported:

  - Option1: Reuse R16 rules for M-TRP SDM 1a;

  - Option2: Introduce new rules for Rel-17 SFN-ed PDSCH

    - If there is any CORESET associated with two TCI states, UE applies the QCL assumption of the CORESET with the lowest ID that associated with two TCI states in the latest slot when receiving the PDSCH.

    - Else if there is any CORESET that is associated with another CORESET for PDCCH repetition transmission, UE applies the QCL assumption of the CORESET with lowest ID among the CORESETs that have associated CORESET, and the associated CORESET of the CORESET with lowest ID in the latest slot.

    - Else the UE may assume that TCI states of PDSCH are associated with the lowest TCI codepoint among the TCI codepoints containing two different TCI states.

Proposal 6:

- When the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’, and a UE is not configured with ‘enableTwoDefaultTCI-States’, one of the following schemes is supported:

- Option 1: UE applies the QCL assumption of the CORESET with lowest ID that has only one TCI state in the latest slot, if any; otherwise, UE applies the QCL assumption corresponding to the first TCI state of the CORESET with lowest ID.

- Option 2: UE applies the QCL assumption of the CORESET with lowest ID in the latest slot. If the CORESET with the lowest ID in the latest slot is associated to two TCI state, UE applies the QCL assumption corresponding to the first TCI state of the CORESET.

For case 3 in Rel-16, the PDSCH follows the TCI state of the scheduling PDCCH. In Rel-17, it may not work when the CORESET scheduling PDSCH is associated with two TCI states, but UE is not capable of two default beam for PDSCH. For the default TCI states for PDSCH in case 3, the following scheme can be considered:

- If a UE is configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with two TCI states, UE applies the QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH the CORESET.

- If a UE is not configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with two TCI states, UE applies the first TCI state of the CORESET when receiving the PDSCH.

- If a UE is not configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with only one TCI states, UE applies the QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH the CORESET.

Proposal 7:

- When the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold ’timeDurationForQCL’ but there is no TCI field in the DCI scheduling PDSCH, the following scheme on QCL assumption for PDSCH with scheme 1 is considered:

  - If a UE is configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with two TCI states, UE applies the QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH.

  - If a UE is not configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with two TCI states, UE applies the first TCI state of the CORESET when receiving the PDSCH.

  - If a UE is not configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with only one TCI states, UE applies the QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH the CORESET.

Default beam for A-CSI RS

In Rel-16, if UE reports the capability for two default reception beams, UE determines the QCL assumption of an A-CSI RS based on RRC configuration of enableTwoDefaultTCI-States and the time offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured. When the time offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is less than the threshold beamSwitchTiming that UE reported, the default TCI state for A-CSI RS can be determined with the following scheme:

- If the UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint is mapped to two TCI states,

- If there is any other DL signal in the same symbols as the A CSI-RS, UE applies the QCL assumption of this DL signal when receiving the A CSI-RS. The other DL signal refers to PDSCH or other aperiodic CSI-RS scheduled with offset equal or larger than the respective corresponding threshold, periodic CSI-RS, and semi-persistent CSI-RS. If that PDSCH is indicated with two TCI states, the UE applies the first TCI state of the two TCI states when receiving the aperiodic CSI-RS.

- Else if there is any PDSCH in the same symbols as the A CSI-RS and the time offset between PDSCH and DL DCI scheduling PDSCH is less than timeDurationForQCL, UE applies the first TCI state of the lowest TCI codepoint that containing two TCI states.

- Else if there is any other DL signal in the same symbols as the A CSI-RS, UE applies the QCL assumption of other DL signal when receiving the A CSI-RS. The other DL signal refers to PDSCH or other aperiodic CSI-RS scheduled with offset equal or larger than the respective corresponding threshold, periodic CSI-RS, and semi-persistent CSI-RS.

- Else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

In Rel-17, a PDSCH or PDCCH can be associated with one or two TCI states. Hence, the default beam for A-CSI RS need to be enhanced according the corresponding enhancement for default beam(s) for PDSCH and PDCCH.

Proposal 8:

- The default beam for A-CSI RS in Rel-17 needs to be enhanced according to the corresponding enhancement for PDSCH and PDCCH.

Enhancement on PDCCH monitoring

In Rel-15/16, if multiple CORERSETs in overlapping PDCCH monitoring occasions have different QCL-TypeD properties, UE monitors PDCCH only in the CORESET(s) having the same QCL-TypeD property as the CORESET corresponding to the CSS with lower index, if any; otherwise, to the USS set with the lowest index.

In Rel-17, one CORESET can be activated with two TCI states corresponding to two QCL-TypeD properties by MAC-CE command. In such case, how to monitor multiple CORERSETs with different QCL-TypeD properties in overlapping monitoring occasion needs to be studied. A new RRC parameter should be introduced for indication that multiple CORERSETs with different QCL-TypeD properties in overlapping monitoring occasion can be monitored. And different enhancement scheme can be considered according to this RRC parameter.

If a UE is configured by this RRC parameter, UE can monitor the CORESET with up to two TCI states associated to two QCL-typeD properties. The following enhancement scheme can be considered:

- UE monitors PDCCHs in a CORESET that corresponding to the CSS with lowest index, if any; otherwise, to the USS set with the lowest index, and other CORESETs that to be monitored are determined by the following cases:

  - If the CORESET is associated with only one TCI states of QCL-TypeD, another one SFN-ed CORESET including TCI state of this CORESET can be monitored too. And other CORESET that associated with the same TCI state(s) as these two CORESETs can also be monitored.

  - If the CORESET is associated with two TCI states corresponding to two QCL-TypeD properties, any other CORESETs that have been configured with qcl-Type set to same 'typeD' properties or a subset of these two QCL-TypeD as the CORESET can also be monitored.

If UE is not configured by this RRC parameter, UE can monitor only one TCI state corresponding to QCL-TypeD property. The following enhancement scheme can be considered:

- UE monitors PDCCHs only in a CORESET that corresponding to the CSS with lowest index, if any; otherwise, to the USS set with the lowest index, and other CORESETs monitored should be determined by the following cases:

  - If the CORESET is indicated by only one TCI states corresponding to QCL-TypeD property, and any other CORESETs that have been configured with qcl-Type set to same 'typeD' properties as the CORESET can also be monitored.

  - If the CORESET is indicated by two TCI states corresponding to two QCL-TypeD properties, this CORESET could not be monitored by UE. And UE can only monitor other CORESETs that be indicated only one TCI state same as the first TCI states associated QCL-TypeD property in this CORESET.

Proposal 9:

- Enhancements on PDCCH monitoring of multiple CORESETs in overlapping monitoring occasion with different QCL-TypeD is needed in Rel-17,

  - Introducing a new RRC parameter for indicating whether multiple QCL-TypeD properties are monitored is supported.

Enhancements with frequency offset pre-compensation

It was agreed in RAN1 #102 e-meeting that the following three steps can be assumed for frequency offset pre-compensation schemes:

- 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation

- 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step

- 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step

In RAN1 #103 e-meeting, the following agreement on QCL assumption for HST-SFN scenario was achieved:

When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations

- Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI state can be associated with {delay spread}  and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties

- Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD)

- Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting

- Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation.

- This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

In RAN1 #104bis e-meeting, the following agreement on QCL assumption for HST-SFN scenario was achieved:

Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both:

- UL RS based Doppler estimation by gNB

  - FFS: Details including UL RS enhancement

- DL RS based Doppler feedback by UE

  - FFS: Details

  - FFS: Whether UE capability needs to be introduced

- Whether to support one or both will be decided later

In this section, we provide our views on the transmission schemes with frequency offset pre-compensation. For simplicity, we present the transmission schemes with PDSCH under the scenario with 2 TRPs as example. Note that the schemes discussed in this section apply to both PDCCH and PDSCH, and the transmission schemes can be extended to the scenarios with multiple TRPs.

Based on the agreement in RAN1 #104bis e-meeting, the following two options were listed for indication of the carrier frequency for UL, and whether to support one or both will be decided later:

- Option 1: DL RS based Doppler feedback by UE

- Option 2: UL RS based Doppler estimation by gNB

For option 1, explicit reporting of the Doppler shift(s) is acquired in the 1st step using CSI framework. Although the Doppler spread for PDSCH/DMRS can be eliminated by option 1, the overhead and Doppler adjustment delay for CSI reporting and the design of CSI reporting are needed and UE-specific TRS resource(s) with frequency offset pre-compensation may be needed in the 3rd step for PDSCH/DMRS channel estimation. For option 2, implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency is acquired in the 1st step.

There are 2 alternatives for option 2:

  - Alt 1: In the 2nd step, multiple SRS resources are transmitted and each SRS resource is transmitted on the carrier frequency acquired by a corresponding TRS in the 1st step. The TRS is transmitted in TRP-specific/non-SFN manner.

  - Alt 2: In the 2nd step, one SRS is transmitted on a carrier frequency acquired by any TRS transmitted in the 1st step.

The procedure of Alt 1 is provided in Fig. 1, with

- 1st Step: 2 TRP-specific TRS resources are transmitted at fc without frequency offset pre-compensation, where fc is the DL center frequency determined by gNB;

2nd Step: 2 SRS resources are transmitted according to the carrier frequency acquired in the 1st step respectively, i.e. SRS 1 is transmitted at fc+f1+fUL-DL and SRS 2 is transmitted at fc+f2+fUL-DL, where f1, and f2 are Doppler shifts from TRP 1 and TRP 2 respectively, and fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS/TRS is SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of f1 and f2 respectively.

In the 3rd step, the frequency offset pre-compensation for each TRP is determined as a half of frequency difference of the SRS corresponding to the TRP and the configured UL carrier frequency.

Figure 1 Procedure of Alt 1, option 2

Although the above procedure can eliminate the Doppler spread for PDSCH/DMRS, it has the following drawbacks:

  - UE-specific & TRP-specific SRS resources are needed;

  - Multiple UL signals with different carrier frequency may cause inter-subcarrier interference among UL signals, which would damage the UL performance;

  - UE-specific TRS resource(s) with frequency offset pre-compensation may be needed in the 3rd step for PDSCH/DMRS channel estimation;

  - A QCL-like association relationship between UL RS in the 2nd step and DL RS in the 1st step is needed.

Based on whether the TRS resource(s) transmitted in the 3rd step is frequency offset pre-compensated or not, the following two schemes can be considered for Alt 2, Option 2:

- Scheme 1: The TRS resources transmitted in the 3rd step are frequency offset pre-compensated;

- Scheme 2: The TRS resources transmitted in the 3rd step are not frequency offset pre-compensated;

For scheme 1, we consider the following schemes:

- Scheme 1-1: SFN-ed DMRS /TRS transmission with UE-specific frequency pre-compensation applied in each TRP in the 3rd step.

Figure 2 Procedure of scheme 1-1

Scheme 1-1 can be seen as Rel-15 SFN-ed transmission scheme + UE-specific frequency offset pre-compensation. A possible transmission procedure for scheme 1-1 is shown in Fig. 2, with

- 1st Step: TRS resource(s) are transmitted at fc without frequency offset pre-compensation, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS/TRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+f1-f2 respectively, where ∆f' is a frequency determined by gNB.

For this scheme, gNB estimates Doppler shifts for each TRP with the SRS transmission, and determines the pre-compensation frequency values for each TRP accordingly. The transmission frequency of the SRS can be the UL carrier frequency that determined by UE according to any downlink signals. In particular, if ∆f'=0, i.e. there is no frequency offset pre-compensation for TRP1, the reception frequency of PDSCH/DMRS/TRS would be fc+f1.

Scheme 1-1 can eliminate the Doppler spread for PDSCH/DMRS. However, it requires UE-specific TRS resources and UE-specific SRS resource(s), which cause large overhead and is not expected.

- Scheme 1-2: SFN-ed DMRS/TRS transmission with UE-group common frequency pre-compensation applied in each TRP in the 3rd step.

The procedure of scheme 1-2 is the same as scheme 1-1 except that UE-group common frequency offset pre-compensation is used instead of the UE-specific frequency offset pre-compensation. So scheme 1-2 can be seen as Rel-15 SFN-ed transmission scheme + UE-group common frequency offset pre-compensation.

A possible transmission procedure for scheme 1-2 is shown in Fig. 3:

- 1st Step: TRS resource(s) are transmitted at fc without pre-compensation, where fc is the DL center frequency determined by gNB;

- 2nd Step: An SRS resource is transmitted according to the carrier frequency determine by a reference UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆fRUE + fUL-DL, where ∆fRUE is the frequency difference of the DL center frequency determined by the reference UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS/TRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+ f1,RUE-f2,RUE respectively, where ∆f' is a frequency determined by gNB.

Figure 3 Procedure of scheme 1-2

For scheme 1-2, gNB estimates Doppler shifts for each TRP with the SRS of a reference UE and determines the pre-compensation frequency values for each TRP accordingly. Assuming that for the reference UE, the frequency difference of the DL center frequency determined by the UE and fc is ∆fRUE, the Doppler shifts of TRP 1 and TRP 2 without Doppler pre-compensation are f1,RUE and f2,RUE respectively; and for a target UE, the frequency difference of the DL center frequency determined by the UE and fc is ∆f, the Doppler shifts of TRP 1 and TRP 2 without Doppler pre-compensation are f1 and f2 respectively. Then the reception frequency of the SRS transmitted by the reference UE is fc+∆f+ fUL-DL +f1,RUE in TRP 1 and fc+∆f+fUL-DL+f2,RUE in TRP 2. If the pre-compensation frequency value is ∆f' in TRP 1 and ∆f'+f1,RUE-f2,RUE in TRP 2, the reception frequency of PDSCH/DMRS/TRS transmitted to the target UE would be fc+∆f'+f1 for TRP 1 and fc+∆f'+f1+f1,RUE-f2,RUE for TRP 2 respectively. In particular, if ∆f'=∆fRUE, the reception frequency of PDSCH/DMRS/TRS transmitted to the target UE would be fc+∆fRUE+f1 for TRP 1 and fc+∆fRUE+f2+f1,RUE-f2,RUE for TRP 2 respectively. Then compared to the DL frequency fc+∆f that locked by the target UE, the Doppler shift of TRP 1 is ∆fRUE+f1-∆f and the Doppler shift of TRP 2 is ∆fRUE-∆f +f2+f1,RUE-f2,RUE, i.e. there is still Doppler spread of the target UE. gNB estimates Doppler shifts for each TRP with the SRS resource, and determines the pre-compensation frequency values for each TRP accordingly. In particular, if ∆f'=0, the reception frequency of PDSCH/DMRS/TRS from TRP 1 would be fc+f1, and the reception frequency of PDSCH/DMRS/TRS from TRP 2 would be fc+f2+f1,RUE-f2,RUE.

For scheme 1-2, the differences of the Doppler shifts among different TRPs are reduced and UE-specific TRS resource is not needed. However, since the pre-compensation frequency value is determined by the SRS of a reference UE, the Doppler spread for PDSCH cannot be well eliminated.

- Scheme 1-3: SFN-ed DMRS transmission and TRP-specific TRS transmission in the 3rd step, with UE-specific frequency pre-compensation applied in each TRP.

The procedure of scheme 1-3 is the same as scheme 1-1, except that TRP-specific TRS resources are transmitted in the 3rd step in scheme 1-3.

Scheme 1-3 can be seen as scheme 1 in section 2 + UE-specific frequency offset pre-compensation. Fig. 4 shows a possible transmission procedure for scheme 1-3, where:

- 1st Step: TRS resource(s) are transmitted at fc without pre-compensation, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where fc is the DL center frequency determined by gNB, ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+f1-f2 respectively, 2 TRS resources are transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+f1-f2 respectively, where ∆f' is a frequency determined by gNB.

Figure 4 Procedure of scheme 1-3

For scheme 1-3, gNB estimates Doppler shifts for each TRP with the SRS resource, and determines the pre-compensation frequency values for each TRP accordingly. In particular, if ∆f'=0, the reception frequency of PDSCH/DMRS/TRS would be fc+f1.

Similar as scheme 1-1, the Doppler spread for PDSCH/DMRS can be eliminated by scheme 1-3, and UE-specific TRS resources is required in the 3rd step.

- Scheme 1-4: SFN-ed DMRS transmission and TRP-specific TRS transmission in the 3rd step, with UE-group common frequency pre-compensation applied in each TRP.

The procedure of scheme 1-4 is the same as scheme 1-2 except that TRP-specific TRS is transmitted in the 3rd step in scheme 1-4. Compared to scheme 1-3, in the 3rd step of scheme 1-4, UE-group common frequency offset pre-compensation is used instead of UE-specific frequency offset pre-compensation, so scheme 1-4 can be seen as scheme 1 in section 2 + UE-group common frequency offset pre-compensation.

In scheme 1-3 and scheme 1-4, two TRP-specific TRS resources are transmitted and they are frequency pre-compensated with same frequencies as that for PDSCH/DMRS, therefore the Doppler profile and delay profile of the channel of PDSCH can be estimated by the TRS resources, and QCL assumption variant E can be used, i.e., the same DMRS port(s) are associated with two TCI states, where both TCI states can be associated with QCL-TypeA.

UE-specific TRS requires large TRS overhead in the system, so scheme 1-1 and scheme 1-3 might not be good solutions in practice. Since scheme 1-2 can be seen as Rel-15 SFN-ed transmission scheme + UE-group common frequency pre-compensation and scheme 1-4 can be seen as Rel-17 enhanced transmission scheme in section 2 + UE-group common frequency pre-compensation, whether frequency pre-compensation is used is up to gNB’s implementation.

In scheme 2, TRS resources are not frequency pre-compensated, therefore UE-specific TRS transmission is not needed. The following alternatives can be considered for scheme 2:

Scheme 2-1: DMRS is SFN-ed transmitted from all TRPs with UE-specific frequency pre-compensation in each TRP; one TRS is transmitted from a reference TRP for Doppler properties estimation, and another TRS is SFN-ed transmitted for delay properties estimation, both TRS resources are transmitted without frequency offset pre-compensation.

Fig. 5 shows a possible transmission procedure for frequency offset pre-compensation scheme 2-1:

- 1st Step: One TRS resource is transmitted from a reference TRP, and another TRS is SFN-ed transmitted from all TRPs. all TRS resources are transmitted without frequency offset pre-compensation at fc, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where fc is the DL center frequency determined by gNB, ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of 0 and f1-f2, respectively.

For this scheme, one TRS for Doppler properties estimation and one TRS for delay properties estimation are configured for PDSCH/PDCCH.

QCL assumption variant B can be considered for scheme 2-1, i.e. the same DMRS port(s) are associated with two TCI states, where one of the TCI state is associated with {average delay, delay spread} and another TCI state is associated with QCL-TypeB.

Figure 5 Procedure of scheme 2-1

- Scheme 2-2: DMRS is SFN-ed transmitted from all TRPs with UE-specific frequency offset pre-compensation in each TRP; one TRS is transmitted from a reference TRP for Doppler properties estimation and another TRS is transmitted from the other TRP, both TRS resources are transmitted without frequency offset pre-compensation and are used for delay properties estimation.

Fig. 6 shows a possible transmission procedure for frequency offset pre-compensation scheme 2-2:

- 1st Step: One TRS resource is transmitted from a reference TRP, and another TRS is transmitted from other TRP, all TRS are transmitted without frequency offset pre-compensation at fc, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where fc is the DL center frequency determined by gNB, ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of 0 and f1-f2 respectively.

Figure 6 Procedure of scheme 2-2

The difference of scheme 2-2 and scheme 2-1 is that 2 TRP-specific TRS resources are used for delay properties estimation in the 3rd step for scheme 2-2. For scheme 2-2, if the transmit frequency of UL RS is up to UE’s implementation, a residual Doppler shift may be observed by UE. The advantages and disadvantages of scheme 2-2 are similar as scheme 2-1.

QCL assumption variant A can be considered for scheme 2-2, i.e. the same DMRS port(s) are associated with two TCI states, where one of the TCI state is associated with {average delay, delay spread} and another TCI state with QCL-TypeA. QCL assumption variant E also can be considered for scheme 2-2, i.e. the same DMRS port(s) are associated with two TCI states, where both TCI states are associated with QCL-TypeA. If Variant E is adopted, Doppler shift and Doppler spread in one TCI state should be omitted according to a rule. For example, Doppler shift and Doppler spread in the second TCI state is omitted.

Fig. 7 and Fig. 8 provide the performance comparison of different schemes when UE is located in the middle of 4 TRPs. And in Fig. 8, the simulation results with CFO = 0.05 ppm in each TRP and Doppler shift adjustment delay of 128ms due to the periodicity of SRS transmission in each pre-compensation scheme are provided. In the evaluations, 4 TRPs are assumed and PDSCH/DMRS is SFN-ed transmitted from 4 TRPs. Since up to 2 TCI states are supported, we assume that for the enhanced scheme 1 in section 2 without frequency offset pre-compensation, pre-compensation scheme 1-3, 1-4 and 2-2, TRS 1 is SFN-ed transmitted from the 2 TRPs in front of the train and TRS 2 is SFN-ed transmitted from the 2 TRPs behind the train; in pre-compensation scheme 2-1, TRS 1 is SFN-ed transmitted from the 2 TRPs in front of the train, TRS 2 is SFN-ed transmitted from the 4 TRPs. More simulation assumptions can be found in Table I in the Appendix, and more simulation results can be found in [2].

The following observations can be obtained from the simulation results that:

- Compared to Rel-15 SFN-ed PDSCH transmission scheme, obvious performance gain can be achieved by the solutions with frequency offset pre-compensation;

- The performance of the schemes of scheme 1 in section 2 + frequency offset pre-compensation (pre-compensation scheme 1-3 and 1-4) outperforms the performance of the scheme 1 in section 2 without frequency offset pre-compensation;

- The performance of pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensated (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2) outperforms the performance of pre-compensation schemes with PDSCH transmitted with UE-group common frequency offset pre-compensated (pre-compensation scheme 1-2 and 1-4).

When CFO and Doppler shift adjustment delay are not considered,

- Similar performance can be achieved by pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensation schemes (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2);

- All the UE-specific frequency offset pre-compensation schemes (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2) outperforms scheme 1 in section 2.

When CFO and Doppler shift adjustment delay in pre-compensation schemes are considered,

- Similar performance can be achieved by pre-compensation scheme 1-1, 2-1 and 2-2, and their performance is worse than that of pre-compensation scheme 1-3 and 1-4 and scheme 1 in section 2. That’s because when CFO in each TRP and Doppler shift adjustment delay are considered, the Doppler profile cannot be well eliminated for schemes with frequency offset pre-compensation. The performance of schemes with one TRS for Doppler profile estimation would be worse than the schemes with multiple TRS resources for Doppler profile estimation.

Figure 7 Performance comparisons of different schemes without CFO and delay in each TRP

Figure 8 Performance comparisons of different schemes with Delay = 128 ms and CFO = 0.05 ppm in each TRP

Based the above observations, frequency pre-compensation schemes are preferred to be supported for HST-SFN deployment. Delay of the Doppler shift adjustment doesn’t impact the performance of Alt 1-3 and Alt 1-4. In our simulation with Doppler shift adjustment delay of 128 ms, scheme 1-3 outperforms option 2-1 and option 2-2. If smaller delay is considered (i.e. more frequently SRS transmission), the performance of scheme 2-1 and 2-2 would be improved and the gap between the performances of scheme 1-3 and scheme 2-/2-1 would be reduced. Besides, scheme 1-3 requires UE-specific frequency pre-compensated TRS resources. This may lead to large TRS overhead in the system. Although compared to scheme 1-3, less overhead of TRS is needed in scheme 1-4, UE-group common frequency pre-compensation is needed in scheme 1-4, UE’s performance may not be guaranteed, especially when there are multiple trains to be served by same TRPs. Since scheme 2-1 and 2-2 can achieve significantly performance gain compared to Rel-15 SFN-ed transmission scheme and they don’t require UE-specific TRS transmission or UE-group common frequency offset pre-compensation, scheme 2-1 and 2-2 are more preferred.

For the proposed scheme 2-1 and 2-2, the indication of QCL assumption can be mainly through the two following methods:

- Option 1: Introduce new QCL types

- Option 2: The existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type

For option 1, QCL assumption variant B is supported for scheme 2-1, QCL assumption variant A is supported for scheme 2-2. In addition, if the new QCL type can be introduced for scheme 2-1 and 2-2, the new QCL type in this TCI codepoint also can be used for the identification of TRP pre-compensation schemes.

For option 2, QCL assumption variant E also can be considered for scheme 2-1 and 2-2, i.e. the same DMRS port(s) are associated with two TCI states, where both TCI states are associated with QCL-TypeA. If Variant E is adopted, Doppler shift and Doppler spread in the first one TCI state should be omitted for scheme 2-1 or 2-2 according to a rule; Average delay and Delay spread in the second TCI state should be omitted for scheme 2-1.

Option 1 is preferred for TRP pre-compensation scheme 2-1 and 2-2, since it is clearer than option 2 and the new QCL type can be used to identify whether TRP pre-compensation scheme is applied.

According our simulation results, it can be seen that significant performance gain can be achieved by using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step. Moreover, the overhead for CSI reporting and overhead of TRS of implicit Doppler shift reporting may be less than explicit Doppler shift reporting. Hence implicit Doppler shift reporting is preferred.

Proposal 10:

- For the enhanced transmission scheme with frequency pre-compensation,

  - Explicit reporting of the Doppler shift(s) acquired in the 1st step is not supported;

  - Scheme 2-1 or scheme 2-2 is supported:

    - Variant A or Variant B of TCI states indication for PDCCH/PDSCH is supported.

  - If new QCL type for variant A or B is supported, the QCL type in TCI codepoint can be used to identify whether TRP pre-compensation scheme is applied.

For the proposed schemes, gNB would choose one TRP as a reference TRP for frequency pre-compensation. It is obvious that best performance would be achieved if UE tracks the DL frequency of the carrier based on the signal from the reference TRP. In order to have the best performance, UE can be indicated with which TRS is used for DL frequency tracking/which TRP is used as the reference TRP explicitly or implicitly. In Rel-16, it’s up to UE’s implementation on how to track DL frequency. In general, for the initial access procedure, UE would track DL frequency by SSB. Then UE can track DL frequency by TRS or DMRS up to its implementation. Indicating UE which DL signal is used for DL frequency tracking is a big restriction on UE’s behavior, and its influence is not clear. We prefer not to indicate UE which DL signal is used for DL frequency tracking. UE can track DL frequency based on the TRS used for Doppler shift estimation.

Since PDSCH/PDCCH and its associated DMRS is not frequency offset pre-compensated in the reference TRP, and the pre-compensated frequency in the other TRP(s) is determined based on the difference of Doppler shift that estimated by the TRP(s) and the reference TRP. The SRS for Doppler shift estimation is transmitted in which frequency is not important. Association of DL and UL signals is not needed for HST-SFN scenario with frequency pre-compensation.

Proposal 11:

- For the enhanced transmission scheme with frequency pre-compensation,

  - The indication on which RS is used for DL frequency tracking is not needed;

  - Association of DL and UL signals is not needed.

Conclusions

In this contribution, we provide our views on the enhancement for Rel-17 HST-SFN deployment scenarios. We have the following observations and proposals:

Observation 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group is needed in Rel-17.

Proposal 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group can be introduced.

  - E.g. Antenna port x and antenna port group Y are said to be quasi co-located if the large-scale properties of the channel over which a symbol on antenna port x is conveyed can be inferred from the combination of channels over which the symbols on the antenna ports of the antenna port group y are conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Proposal 2:

- The new RRC parameter identifying Scheme 1 for PDSCH is configured per BWP.

Proposal 3:

- For scheme 1,

  - restricting DMRS ports within one CDM group;

  - new tables for antenna port indication are introduced to reduce DCI overhead.

Proposal 4:

- Dynamic switching of scheme 1 and S-TRP, whether dynamic switching is enabled or not is configured by RRC.

Proposal 5: For the default beams of a PDSCH with scheme 1,

- When the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’, and a UE is configured with ‘enableTwoDefaultTCI-States’, the following scheme is supported:

  - Option1: Reuse R16 rules for M-TRP SDM 1a;

  - Option2: Introduce new rules for Rel-17 SFN-ed PDSCH

    - If there is any CORESET associated with two TCI states, UE applies the QCL assumption of the CORESET with the lowest ID that associated with two TCI states in the latest slot when receiving the PDSCH.

    - Else if there is any CORESET that is associated with another CORESET for PDCCH repetition transmission, UE applies the QCL assumption of the CORESET with lowest ID among the CORESETs that have associated CORESET, and the associated CORESET of the CORESET with lowest ID in the latest slot.

    - Else the UE may assume that TCI states of PDSCH are associated with the lowest TCI codepoint among the TCI codepoints containing two different TCI states.

Proposal 6:

- When the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold ‘timeDurationForQCL’, and a UE is not configured with ‘enableTwoDefaultTCI-States’, one of the following schemes is supported:

- Option 1: UE applies the QCL assumption of the CORESET with lowest ID that has only one TCI state in the latest slot, if any; otherwise, UE applies the QCL assumption corresponding to the first TCI state of the CORESET with lowest ID.

- Option 2: UE applies the QCL assumption of the CORESET with lowest ID in the latest slot. If the CORESET with the lowest ID in the latest slot is associated to two TCI state, UE applies the QCL assumption corresponding to the first TCI state of the CORESET.

Proposal 7:

- When the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold ’timeDurationForQCL’ but there is no TCI field in the DCI scheduling PDSCH, the following scheme on QCL assumption for PDSCH with scheme 1 is considered:

  - If a UE is configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with two TCI states, UE applies the QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH.

  - If a UE is not configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with two TCI states, UE applies the first TCI state of the CORESET when receiving the PDSCH.

  - If a UE is not configured with ‘enableTwoDefaultTCI-States’ and the CORESET that schedules the PDSCH is associated with only one TCI states, UE applies the QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH the CORESET.

Proposal 8:

- The default beam for A-CSI RS in Rel-17 needs to be enhanced according to the corresponding enhancement for PDSCH and PDCCH.

Proposal 9:

- Enhancements on PDCCH monitoring of multiple CORESETs in overlapping monitoring occasion with different QCL-TypeD is needed in Rel-17,

  - Introducing a new RRC parameter for indicating whether multiple QCL-TypeD properties are monitored is supported.

Proposal 10:

- For the enhanced transmission scheme with frequency pre-compensation,

  - Explicit reporting of the Doppler shift(s) acquired in the 1st step is not supported;

  - Scheme 2-1 or scheme 2-2 is supported:

    - Variant A or Variant B of TCI states indication for PDCCH/PDSCH is supported.

  - If new QCL type for variant A or B is supported, the QCL type in TCI codepoint can be used to identify whether TRP pre-compensation scheme is applied.

Proposal 11:

- For the enhanced transmission scheme with frequency pre-compensation,

  - The indication on which RS is used for DL frequency tracking is not needed;

  - Association of DL and UL signals is not needed.

References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, RAN #86.

[2] R1-2102605, Evaluation for HST-SFN deployment, CATT, RAN1 #104bis-e.

Appendix

Table-I Link level simulation assumptions for HST-SFN

| Parameter | Value |
| --- | --- |
| Propagation condition | 4-tap channel model |
| TRP layout | Ds=700m, Dmin=150m |
| Carrier frequency | 3.5 GHz |
| Speed | 500 km/h |
| BW | 20 MHz |
| SCS | 30 KHz |
| Number of RBs for PDSCH | 50 PRBs |
| Duplexing | TDD |
| TDD pattern | DDDDDDDSUU |
| gNB antenna configuration | 2Tx |
| UE antenna configuration | 2Tx |
| DMRS configuration | DMRS type 1, 1+1+1 |
| TRS configuration | Periodic of 10ms, 2-slot pattern |
| MCS | MCS 13 based on 64QAM table |
| Rank | Rank 1 |